IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Draft Amendment to IEEE Standard fors |  |
| Date Submitted | [18 January 2007,Revised on 12 March 2007] |  |
| Source | [Ismail Lakkis1, Jisung Oh2] [(1) Tensorcom, (2) Samsung Electronics] [ ] | Voice: [+1-858-231-9753, ] Fax: [+1-858-678-0300] E-mail: [ilakkis@tensorcom.com, Jisung0714@samsung.com] |
| Re: | [P802-15-3c-DF2 HSI-OFDM Comments Resolution] |  |
| Abstract | [Proposed resolution for comments related to HIS-OFDM PHY] |  |
| Purpose | [Notice of Request for formal CFP submissions to the P802.15.3c Millimeter-wave based Alternative PHY Task Group] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment 23: Specify the base rate

Resolution: The HSI-OFDM base rate shall be 59 Mbps (MCS #0)

The following s MCSs are mandatory:

0, 1, 2, 3, and 4.

Table 1 – Data field rate-dependent parameters

| M C S | Data Rate (Mbps) | Modulation scheme | Coding mode | Outer FEC rate (RO) | Inner FEC rate (RI) |  | Spreading factor (Lf) | Spread & coded bits/sym. (NSCBPS) | Coded bits/sym. (NCBPS) | Data info* bits/sym. (NIBPS) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MSB | LSB |  |  |  | MSB | LSB |
| 0 | 59 | QPSK | EEP | 0.94 | 1/2 |  | 24 | 672 | 28 | 14 |  |
| 1 | 708 | QPSK |  | 0.94 | 1/2 |  | 2 | 672 | 336 | 168 |  |
| 2 | 1416 | QPSK |  | 0.94 | 1/2 |  | 1 | 672 | 672 | 336 |  |
| 3 | 2124 | QPSK |  | 0.94 | 3/4 |  | 1 | 672 | 672 | 504 |  |
| 4 | 2478 | QPSK |  | 0.94 | 7/8 |  | 1 | 672 | 672 | 588 |  |
| 5 | 2832 | 16-QAM |  | 0.94 | 1/2 |  | 1 | 1344 | 1344 | 672 |  |
| 6 | 4248 | 16-QAM |  | 0.94 | 3/4 |  | 1 | 1344 | 1344 | 1008 |  |
| 7 | 4956 | 16-QAM |  | 0.94 | 7/8 |  | 1 | 1344 | 1344 | 1176 |  |
| 8 | 6372 | 64-QAM |  | 0.94 | 3/4 |  | 1 | 2016 | 2016 | 1512 |  |
| 9 | 1512 | QPSK | EEP | 1 | 1/2 |  | 1 | 672 | 672 | 336 |  |
| 10 | 2664 | QPSK |  | 1 | 7/8 |  | 1 | 672 | 672 | 588 |  |
| 11 | 4536 | 16-QAM |  | 1 | 3/4 |  | 1 | 1344 | 1344 | 1008 |  |
| 12 | 1770 | QPSK | UEP | 0.94 | 1/2 | 3/4 | 1 | 672 | 672 | 336 | 504 |
| 13 | 2301 | QPSK |  | 0.94 | 3/4 | 7/8 | 1 | 672 | 672 | 504 | 588 |
| 14 | 3540 | 16QAM |  | 0.94 | 1/2 | 3/4 | 1 | 1344 | 1344 | 672 | 1008 |
| 15 | 4602 | 16QAM |  | 0.94 | 3/4 | 7/8 | 1 | 1344 | 1344 | 1008 | 1176 |

* Reed Solomon encoded bits when RS is used.

Comment 71: Tables 142, 143 & 144 contain the same value.

Resolution: correct Tables are provided below:

Table 142– Length 128 Golay codes a128 in hexadecimal format

| Re[a128,1] | FA3663AF5063C9FA05C99C505063C9FA |
| --- | --- |
| Im[a128,1] | 509CC90505369CAFAF6336FA05369CAF |
| Re[a128,2] | C60A5F93390AA093C60A5F93C6F55F6C |
| Im[a128,2] | F5396CA00A3993A0F5396CA0F5C66C5F |
| Re[a128,3] | 6CF55F39C65F0A6C930AA0C6C65F0A6C |
| Im[a128,3] | C6A00A9393F5A039395FF56C93F5A039 |
| Re[a128,4] | F066960055C333A5553C335AF09996FF |
| Im[a128,4] | AAC3CCA5F09996FFF0669600AA3CCC5A |

Table 143 – Length 512 Golay complementary codes a512 in hexadecimal format

| Re[a512,1] | 5F6CF5C6A06C0AC693A0C6F5935FC60A5F6CF5C6A06C0AC66C5F390A6CA039F55F6CF5C6A06C0AC693A0C6F5935FC60AA0930A395F93F53993A0C6F5935FC60A |
| --- | --- |
| Im[a512,1] | 6C5F390A935FC60A5F6CF5C65F93F5396C5F390A935FC60AA0930A39A06C0AC66C5F390A935FC60A5F6CF5C65F93F53993A0C6F56CA039F55F6CF5C65F93F539 |
| Re[a512,2] | 0096C3AA5ACC660F5A3366F00069C3550096C3AA5ACC660FA5CC990FFF963CAAFF693C55A53399F0A5CC990FFF963CAA0096C3AA5ACC660FA5CC990FFF963CAA |
| Im[a512,2] | A5CC990F0069C3550096C3AAA53399F0A5CC990F0069C355FF693C555ACC660F5A3366F0FF963CAAFF693C555ACC660FA5CC990F0069C355FF693C555ACC660F |
| Re[a512,3] | 39F5AF6336055F6C39F5AF63C9FAA093C60A509CC9FAA09339F5AF63C9FAA09339F5AF6336055F6C39F5AF63C9FAA09339F5AF6336055F6CC60A509C36055F6C |
| Im[a512,3] | C905A06C390AAF9CC905A06CC6F5506336FA5F93C6F55063C905A06CC6F55063C905A06C390AAF9CC905A06CC6F55063C905A06C390AAF9C36FA5F93390AAF9C |
| Re[a512,4] | AF9CC9FAAF63C905AF9CC9FA509C36FA9CAFFAC99C50FA36635005369C50FA36AF9CC9FAAF63C905AF9CC9FA509C36FA6350053663AF05C99CAFFAC963AF05C9 |
| Im[a512,4] | 6350053663AF05C9635005369C50FA36AF9CC9FAAF63C90550633605AF63C9056350053663AF05C9635005369C50FA3650633605509C36FAAF9CC9FA509C36FA |

Table 144 – Length 512 Golay complementary codes b512 in hexadecimal format

| Re[b512,1] | AF9C0536509CFA366350360563AF36FAAF9C0536509CFA369CAFC9FA9C50C905AF9C0536509CFA366350360563AF36FA5063FAC9AF6305C96350360563AF36FA |
| --- | --- |
| Im[b512,1] | 9CAFC9FA63AF36FAAF9C0536AF6305C99CAFC9FA63AF36FA5063FAC9509CFA369CAFC9FA63AF36FAAF9C0536AF6305C9635036059C50C905AF9C0536AF6305C9 |
| Re[b512,2] | F066335AAA3C96FFAAC39600F09933A5F066335AAA3C96FF553C69FF0F66CC5A0F99CCA555C36900553C69FF0F66CC5AF066335AAA3C96FF553C69FF0F66CC5A |
| Im[b512,2] | 553C69FFF09933A5F066335A55C36900553C69FFF09933A50F99CCA5AA3C96FFAAC396000F66CC5A0F99CCA5AA3C96FF553C69FFF09933A50F99CCA5AA3C96FF |
| Re[b512,3] | C9055F93C6F5AF9CC9055F93390A506336FAA06C390A5063C9055F93390A5063C9055F93C6F5AF9CC9055F93390A5063C9055F93C6F5AF9C36FAA06CC6F5AF9C |
| Im[b512,3] | 39F5509CC9FA5F6C39F5509C3605A093C60AAF633605A09339F5509C3605A09339F5509CC9FA5F6C39F5509C3605A09339F5509CC9FA5F6CC60AAF63C9FA5F6C |
| Re[b512,4] | 5F6C390A5F9339F55F6C390AA06CC60A6C5F0A396CA00AC693A0F5C66CA00AC65F6C390A5F9339F55F6C390AA06CC60A93A0F5C6935FF5396C5F0A39935FF539 |
| Im[b512,4] | 93A0F5C6935FF53993A0F5C66CA00AC65F6C390A5F9339F5A093C6F55F9339F593A0F5C6935FF53993A0F5C66CA00AC6A093C6F5A06CC60A5F6C390AA06CC60A |

### Comment 62: SFD figure 146 is wrong (remove) and table 145 has the wrong description

### Resolution: Remove figure 146 and replace table 145 by the following table.

Table 145 – SFD patterns

| Piconet number | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Header rate of 39 Mbps & short CES | [-1 -1 -1 -1] | [-1 –j +1 +j] | [-1 +1 -1 +1] | [-1 +j +1 –j] |
| Header rate of 936 Mbps & short CES | [-1 -1 +1 +1] | [-1 –j -1 –j] | [-1 +1 +1 -1] | [-1 +j -1 +j] |
| Header rate of 39 Mbps & long CES | [-1 -1 +1 -1] | [-1 –j -1 +j] | [-1 +1 +1 +1] | [-1 +j -1 -j] |
| Header rate of 936 Mbps & long CES | [-1 -1 -1 +1] | [-1 –j +1 –j] | [-1 +1 -1 -1] | [-1 +j +1 +j] |

Comment 61: Spreading Step is missing from HSI-OFDM

Resolution: Add the following step to the data field (step 6) and the Text shown below for the spreader description:

- Compute the FCS (4 octets) over frame payload,

- Form the unscrambled data field by appending the frame payload with 4-octet FCS and a sufficient number of pad bits (see 12.4.1) to ensure that the frame payload is aligned on the OFDM symbol boundary,

- Scramble the resulting combination according to 12.5,

- Encode and interleave the scrambled data field as specified in 12.6,

- Map the data field onto the appropriate constellation as described in 12.7,

- Spread the QPSK and QAM modulated signal according to 12.8,

- Interleave the spread data with the tone interleaver as described in 12.9,

- Modulate the resulting complex values with the OFDM modulator described in 12.10,

- Insert PCES periodically as described in 12.11.

## 12.8 The spreader

The spreading factor in relationship to the data rate in the data field, base frame header field and optional frame header field is specified in Tables 1, 2, and 3 specifically. The spreading rules shall be the same for all three fields.

### 12.8.1 Spreader for spreading factor of 1

For a spreading factor of 1, the modulated QPSK and QAM complex values a(k), where k = 0, 1, 2, … at the output of the constellation mapper shall be grouped into sets of 336 complex numbers. Each group shall be assigned to an OFDM symbol. We shall denote this by writing the complex number bk,n, as

where n is the OFDM symbol number. Each group shall be passed to the tone interleaver before being modulated by the OFDM modulator into an OFDM symbol.

### 12.8.2 Spreader for spreading factor of 2

For a spreading factor of 2, the modulated QPSK complex values a(k), where k = 0, 1, 2, … at the output of the constellation mapper shall be grouped into sets of 168 complex numbers. We shall denote this by writing the complex number ak,n, as

where n is the group number. Each group shall be spread by a factor of two to generate a block of 336 complex numbers as follows:

Each spread group shall be passed to the tone interleaver before being modulated by the OFDM modulator into an OFDM symbol.

### 12.8.3 Spreader for spreading factor of 24

For a spreading factor of 24, the modulated QPSK complex values a(k), where k = 0, 1, 2, … at the output of the constellation mapper shall be grouped into sets of 14 complex numbers. We shall denote this by writing the complex number ak,n, as

where n is the group number. Each group shall be spread by a factor of 24 to generate a block of 336 complex numbers as follows:

where q is a length 12 sequence given by:

and the function floor() rounds its argument to the nearest integer toward minus infinity. Each spread group shall be passed to the tone interleaver before being modulated into an OFDM symbol.

Comment 65: Constellation normalization factor is wrong incorrect in Table 153.

Resolution: Replace Table 153 by the following:

Table 153 - Modulation-dependent normalization factor

| Modulation | KMOD |
| --- | --- |
| QPSK | 1/√1+d2 |
| 16-QAM | 1/√5(1+d2) |
| 64-QAM | 1/√21(1+d2) |

## Comments 64 & 66: Data field change and FFT figure

Resolution: Replace the OFDM modulator text by the following

## 12.10 The OFDM modulator

The stream of interleaved complex numbers is divided into groups of ND = 336 data complex numbers. We shall denote this by writing the complex number dk,n, which corresponds to data subcarrier k of OFDM symbol n, as

Where  is the number of OFDM symbols occupied by the header and data field.

The discrete-time signal during the nth OFDM symbol is given by:

Where , ND is the number of data subcarriers, NP is the number of pilot subcarriers, NR is the number of reserved subcarriers, NG is the number of guard subcarriers, NFFT is the number of total subcarriers, and dm,n, pm,n, gm,n are the complex numbers placed on the mth data, pilot, and guard subcarriers of the nth OFDM symbol, respectively.

The functions MD(m), MP(m), and MG(m) define a mapping between the indices [0:ND-1], [0:NP-1], and [0:NG-1] into logical frequency offset index [-NFFT/2:NFFT/2-1]. The exact definition for the mapping functions are given below:

where the function round, rounds the input argument to the nearest integer.

The mapping of data and pilot subcarriers within an OFDM symbol is illustrated in Figure 15. The mapping is further summarized in Table 25. As shown in Figure 15, there are 16 groups of subcarriers where each group is constituted of 21 data subcarriers and one pilot subcarrier.

Figure 1 – Subcarrier frequency allocation

Table 2 – Subcarrier frequency allocation

| Subcarriers type | Number of subcarriers | Logical subcarriers indices |
| --- | --- | --- |
| Null subcarriers | 141 | −256:−186∪186:255 |
| DC subcarriers | 3 | -1, 0, 1 |
| Pilot subcarriers | 16 | −166:22:−12∪12:22:166 |
| Guard subcarriers | 16 | −185:−178∪178:185 |
| Data subcarriers | 336 | All others |

The common way to implement the inverse Fourier transform, as shown in Equation (3), is by an inverse Fast Fourier Transform (IFFT) algorithm. If, for example, a 512-point IFFT is used, the logical subcarriers 2 to 185 are mapped to the same numbered IFFT inputs, while the logical frequency subcarriers –185 to –2 are copied into IFFT inputs 327 to 510. The rest of the inputs, 186 to 326 and the 0 (dc) input, 1, and 511 are set to zero. The subcarriers 0, 1, and 511 are set to zero to avoid difficulties in D/A and A/D converter offsets and carrier feedthrough in the RF system. This mapping is illustrated in Figure 16. After performing an IFFT, the output is cyclically extended to the desired length.

Figure 2 – Inputs and outputs relationship of the IFFT

In the following subclauses, the pilot, and guard subcarriers are detailed.

### 12.10.1 Pilot subcarriers

In all OFDM symbols following the packet preamble, sixteen of the subcarriers shall be dedicated to pilot signals in order to allow for coherent detection and to provide robustness against frequency offsets and phase noise. These pilot signals shall be placed into logical frequency subcarriers -166, -144, -122, -100, -78, -56, -34, -12, 12, 34, 56, 78, 100, 122, 144, and 166.  The information for the mth pilot subcarrier of the nth OFDM symbol shall be defined as follows:

The polarity of the pilot subcarriers is controlled by the sequence, xn, generated by the linear feedback shift register (LFSR) illustrated in Figure 17.  The LFSR operates in the same way as described in section 1.2.2.

The 15 bit seed value chosen is as follows: . The first 16 output values should be: .

### 12.10.2 Guard subcarriers

In all OFDM symbols following the packet preamble, there shall be 16 guard subcarriers, 8 on each edge of the occupied frequency band, at logical frequency subcarriers -185, -184, …, -178 and 178, 179, …, 185. The data on these subcarriers shall be left to the implementer. Individual implementations may exploit these guard subcarriers for various purposes, including relaxing the specifications on analog transmit and analog receive filters, and possibly peak to average power ratio reduction.

Comment: PMD operation specifications, Tx specifications & Rx specifications are missing

Resolution: Add the following at the end of the HSI-PHY clause:

## 12.12 PMD operating specifications

### 12.12.1 Operating frequency bands

The set of operating channels is defined in Table x. A compliant 802.15.3c implementation that implements the HSI PHY shall support at least channel number 2 or channel number 3.

Table 3 – PHY channel allocation

| Channel index | Start frequency | Center frequency | Stop frequency | Chip rate |
| --- | --- | --- | --- | --- |
| 1 | 57.240 GHz | 58.320 GHz | 59.400 GHz | 2592 MHz |
| 2 | 59.400 GHz | 64.480 GHz | 61.560 GHz | 2592 MHz |
| 3 | 61.560 GHz | 62.640 GHz | 63.720 GHz | 2592 MHz |
| 4 | 63.720 GHz | 64.800 GHz | 65.880 GHz | 2592 MHz |

### 12.12.2 Channel assignments

A total of 16 logical channels are assigned for operation, 4 logical channels in each of the 4 defined operating frequency channels. A compliant implementation shall support at least one logical channel for channel number 2 or channel number 3.

### 12.12.3 Regulatory compliance

Please comply.

### 12.12.4 Operating temperature range

A conformant implementation shall meet all of the specifications in this standard for ambient temperatures from 0 to 40 C.

## 12.13 Transmitter Specifications

### 12.13.1 Transmit PSD mask

The transmitted spectrum shall have a 0 dBr (dB relative to the maximum spectral density of the signal) bandwidth not exceeding 1.88 GHz, –20 dBr at 1.1 GHz frequency offset, –28 dBr at 1.6 GHz frequency offset and –40 dBr at 2.2 GHz frequency offset and above. The transmit spectrum mask is shown in Figure 3. The measurements shall be made according to the FCC rules as specified in part 15.

Figure 3 – Transmit spectral mask

### 12.13.2 Constellation error

The relative constellation RMS error, averaged over all data and pilot subcarriers of the OFDM symbols and over all of the frames, shall not exceed the values given in Table 4.

Table 4 – Allowed relative constellation error versus data rate

| Data rate | Relative constellation RMS error |
| --- | --- |
| 59 Mbps to 1.5Gbps | -7 dB |
| 2.1 Gbps to 2.7 Gbps | -14 dB |
| 2.8 Gbps to 5 Gbps | -21 dB |
| Above 6Gbps | -25 dB |

The relative constellation RMS error calculation shall be performed using a device capable of converting the transmitted signal into a stream of complex samples at 2592 Msamples/s or more, with sufficient accuracy in the I/Q imbalance, DC offset, phase noise, etc. The sampled signal shall then be processed in a manner similar to that of an ideal receiver including adding the 64 samples of the cyclic prefix to the received OFDM symbol.

### 12.13.3 Chip rate and clock alignment

The transmitted center frequency and chip clock frequency tolerances shall be 20 ppm maximum.

## 12.14 Receiver specifications

### 12.14.1 Receiver sensitivity

For a bit error rate (BER) of 10-6, the minimum receiver sensitivity shall be -50 dBm for MCS index 2. The BER shall be measured at the MAC/PHY interface after PHY level FEC. A PN sequence defined by x55 + x24 + 1 shall be used for the BER measurement.

### 12.14.2 Receiver CCA performance

The start of a valid transmission at a receiver level equal to or greater than the minimum sensitivity of -50 dBm for MCS number 2 shall cause CCA to indicate busy with a probability > 90% within 2.5 microseconds. If the preamble portion was missed, the receiver shall hold the carrier sense (CS) signal busy for any signal 20 dB above the minimum sensitivity of -50 dBm.

### 12.14.3 Receiver maximum input level

The receiver maximum input level is the maximum power level of the incoming signal, in dBm, present at the input of the receiver for which the error rate criterion is met. A compliant receiver shall have a receiver maximum input level of at least -25 dBm for each of the modulation formats that the device supports.

### 12.14.4 PHY layer timing

The default values for the PHY layer timing parameters are defined in Table 5Error: Reference source not found.

Table 5 – PHY layer timing parameters

| PHY parameter | Value | Definition |
| --- | --- | --- |
| pPHYMIFSTime | 0.0 s | 11.2.7.4 |
| pPHYSIFSTime | 2.5 s | 11.2.7.2 |
| pCCADetectTime | 2.5 s | 11.6.5 |
| pPHYChannelSwitchTime | 100 s | 11.2.7.5 |

### 12.14.4 Interframe spacing

The interframe spacing parameters are given in Table 6.

Table 6 – Interframe spacing parameters

| 802.15.3 MAC parameter | PHY parameter | Definition |
| --- | --- | --- |
| MIFS | pPHYMIFSTime | 11.2.7.4 |
| SIFS | pPHYSIFSTime | 11.2.7.2 |
| pBackoffSlot | pPHYSIFSTime + pCCADetectTime | 11.2.7.1 |
| BIFS | pPHYSIFSTime + pCCADetectTime | 11.2.7.1 |
| RIFS | 2×pPHYSIFSTime + pCCADetectTime | 11.2.7.1 |

### 12.14.5 Receive-to-transmit turnaround time

The RX-to-TX turnaround time shall be less than pPHYSIFSTime. The RX-to-TX turnaround time shall be measured at the air interface from the trailing edge of the last symbol received until the first symbol of the PHY preamble is present at the air interface.

### 12.14.6 Transmit-to-receive turnaround time

The TX-to-RX turnaround time shall be less than pPHYSIFSTime. The TX-to-RX turnaround time shall be measured at the air interface from the trailing edge of the last transmitted symbol until the receiver is ready to begin the reception of the next PHY packet.

### 12.15.7 Time between successive transmissions

The time between successive transmissions shall be pPHYMIFSTime, including the power-up ramp specified in 11.5.7. The pPHYMIFSTime shall be measured at the air interface from the trailing edge of the last symbol transmitted until the first symbol of the PHY preamble is present at the air interface.

### 12.15.8 Channel switch time

The channel switch time is defined as the time from when the last valid bit is received at the antenna on one channel until the DEV is ready to transmit or receive on a new channel. The channel switch time shall be less than pPHYChannelSwitchTime.

## 12.16 HSI PHY management

The PHY PIB comprises the managed objects, attributes and notifications required to manage the PHY layer of a DEV.

### 12.16.1 PHY supported data rate encoding

The encoding of the PHY data rates used in the Supported Data Rates field in the Capability IE, as described in 7.4.11, is given in Table 7.

Table 7 – PHY supported data rates encoding

| Supported MCSs | b0 | b1 | b2 | b3 |
| --- | --- | --- | --- | --- |
| 0:4 | 0 | 0 | 0 | 0 |
| 0:4 and 9:10 | 0 | 0 | 0 | 1 |
| 0:7 | 0 | 0 | 1 | 0 |
| 0:7 and 9:11 | 0 | 0 | 1 | 1 |
| 0:4 &12:13 | 0 | 1 | 0 | 0 |
| 0:7 & 12:15 | 0 | 1 | 0 | 1 |
| 0:7 and 9:15 | 0 | 1 | 1 | 0 |
| All | 0 | 1 | 1 | 1 |
| All | 1 | 0 | 0 | 0 |

### 12.16.2 HSI PHY fragment size encoding

The encoding of the preferred fragment size used in the Capability IE, as described in 7.4.11, is given in Table 8.

The PHY definitions create restrictions on the maximum frame size, maximum transfer unit size and minimum fragmentation size that will be supported. These parameters are defined in this subclause.

Table 8 – HIS PHY preferred fragment size encoding

| Field value | Preferred fragment size (Octets) |
| --- | --- |
| 0 | pMaxFrameBodySize |
| 1 | TBD |
| 2 | TBD |
| 3 | TBD |
| 4 | TBD |
| 5 | TBD |
| 6 | TBD |
| 7 | pMinFragmentSize |

### 12.16.3 Maximum frame length

The maximum frame length allowed, pMaxFrameBodySize, shall be 16232 octets. This total includes the frame body and FCS but not the PHY preamble, PHY header or MAC header.

### 12.16.4 Maximum transfer unit size

The maximum size data frame passed from the upper layers, pMaxTransferUnitSize, shall be 16232 octets. If security is enabled for the data connection, the upper layers should limit data frames to 16232 octets minus the security overhead.

### 12.16.5 Minimum fragment size

The minimum fragment size, pMinFragmentSize, shall be xxx octets.

### 12.16.6 HSI PHY PIB

The PHY dependent PIB values are given in Table 9

Table 9 - HSI PHY supported modes

| Bits | Content | Description |
| --- | --- | --- |