IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Comment to issue number 81 in rev 11 |  |
| Date Submitted | [25 March 2008] |  |
| Source | [Abbie Mathew] [NewLANS] [43 Nagog Park, Suite 200.] [Acton, MA 01720] | Voice: [] Fax: [+978-849-8005] E-mail: [amathew@newlans.com] |
| Re: | [IEEE 802.15.3c Channel Model] |  |
| Abstract | [Comment to issue number 81 in [15-08-0020-11-003c-df0-comments] |  |
| Purpose | The purpose of this document is to address the outstanding comments in [15-08-0020-11-003c-df0-comments] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

12.2.2.1.7 Dual AMI (DAMI)

In DAMI modulation, the coded and interleaved binary serial input data, bk, where k= 0, 1, 2, …, shall be first split into two sub-sequences defined as follows:

b(o)k=b[k]if k is odd0if k is even and b(e)k=0if k is oddb[k]if k is even,

where the superscripts ‘o’ and ‘e’ stand for ‘odd’ and ‘even’, respectively. AMI coding scheme is then applied to both sub-sequences to form two coded sub-sequences,d(m)k (‘m’ = ‘o’ or ‘e’), which can be mathematically described as

d(m)k=a(m)k−a(m)k−2

with a(m)k being formed by

a(m)k=a(m)[k−2]⊕b(m)[k],

where ⊕ denotes the modulo-2 addition and the initial values for a(m)k are a(m)−2=a(m)[−1]=0. The output values, dk, are finally formed by

dk=d(o)k+d(e)k.

The resulting constellation is illustrated in Figure 149(g). Note that the constellation points are not labeled by the input bits due to the complicated relationship between them. The transmitted RF signal for DAMI system is an SSB modulated signal accompanied by two pilot carriers:

sRF(t)=s(t)cos2πfct+s(t)sin2πfct+Apcos2πfct+Apcos2πfc−12Tbt,

where fc is the centre frequency, Tb is the bit interval, and s(t) is the Hilbert transform of the baseband signal s(t). The amplitude of the pilot carriers, Ap, is chosen such that the power density of each pilot must not exceed that of the SSB signal and the integrated power of each pilot is at least 25 dB below that of the SSB signal. The baseband signal s(t) can be represented by

s(t)=k=0Npacket−1d[k]g(t−kTb),

where g(t) is the baseband pulse shape, Npacket is the number of bits in the packet, and the coefficient d[k] is a real number generated according to the description above.

Figure 149(g): Constellation for DAMI