IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Resolution of comment 145 in df3 issue tracking |  |
| Date Submitted | [12 May 2008] |  |
| Source | [Zhiguo Lai] [NewLANS, Inc.] [43 Nagog Park, Suite 200.] [Acton, MA 01720] | Voice: [] Fax: [+978-849-8005] E-mail: [zlai@newlans.com] |
| Re: | [] |  |
| Abstract | [Resolution of comment 145 in [15-08-0255-02-003c-df3-issue-tracking] |  |
| Purpose | The purpose of this document is to address the outstanding comments in [15-08-0255-02-003c-df3-issue-tracking] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

12.2.2.1.7 Dual AMI (DAMI)

In DAMI modulation scheme, the coded binary serial input data, bk, where k= 0, 1, 2, …, shall be first precoded to form an intermediate data, bk, defined as follows:

bk=b[k−2]⊕b[k]

where ⊕ denotes the modulo-2 addition and two initial values b−2=b[−1]=0 shall be used for the precoding. The output values, dk, are formed by

dk=KmodI[k]+jQ[k]

where I[k] and Q[k] are given by Table xx. The resulting constellation is illustrated in Figure xx. The normalization factor Kmod=1 for DAMI constellation.

Table xx. DAMI encoding table

| Precoded input bits bk−2bk | Ik | Qk |
| --- | --- | --- |
| 00 | 0 | 0 |
| 01 | 1 |  |
| 10 | -1 |  |
| 11 | 0 |  |

Figure xx. DAMI constellation

The transmitted RF signal for a DAMI system is a single-sideband (SSB) modulated signal accompanied by two low-power pilot tones. The SSB signal can be written as

sSSB(t)=s(t)cos2πfct+s(t)sin2πfct

where fc is the center frequency, s(t) is the baseband signal, and s(t) is the Hilbert transform of s(t). The baseband signal s(t) can be represented by

s(t)=k=0Npacket−1d[k]gt−kTsym

where Npacket is the number of symbols in the packet, Tsym is the symbol length, and g(t) is the baseband pulse shape. It is noted that one symbol corresponds to one bit for a DAMI system, meaning that the symbol length is the same as the bit length.

The baseband pulse g(t) shall be chosen such that the power spectral density (PSD) of the baseband signal, that is, sin2(2πfTsym)|G(f)|2, where G(f) is the Fourier transform of g(t), complies with the transmit PSD mask given in Clause 12.2.8.4.

The two pilot tones shall have frequencies fc and fc−1(2Tsym), respectively. Both of them shall be in phase with the SSB signal. Their amplitudes shall be chosen such that the integrated power of each pilot is 25 dB (with ±1 dB tolerance) below the integrated power of the SSB signal.