IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Comment Resolutions related to MAC (UEP, UEP/EEP, Retransmission) |  |
| Date Submitted | July, 2008 |  |
| Source | [Chang woo Pyo, Zhou Lan, Fumihide Kojima, Hiroyuki Nakase, Hirosh Harada, Shuzo Kato] | Voice: [+81-46-847-5092] Fax: [+81-46-847-5440] E-mail: [cwpyo@nict.go.jp] |
| Re: | 802.15.3c Teleconference Meeting |  |
| Abstract | IEEE 802.15 Task Group TG3c Comment Resolution |  |
| Purpose | Comment Resolutions related to MAC (UEP, UEP/EEP, Retransmission) |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Note that blue colored sentences are newly added or modified

5.3.16 Unequal error protection (UEP)

For flexible error protection that accounts for data that has more emphasis on the msbs than the lsbs, e.g.,

video, UEP is provided as an option. This UEP has three different types as described in 8.17 and may be applied for three different PHY modes described in 5.5.1.

7.2.8 MAC subheader for standard aggregation

Figure 10a—MAC subheader for standard aggregation format

Figure 10a—MAC subheader for standard aggregation format

The Retransmission Indication field shall be set to one if retransmission is needed and shall be set to zero otherwise.

8.17 UEP

Three types of UEP may be used for flexible error protection of a frame consisting of MSB and LSB subframes. Type 1 is used to protect each subframe by FEC only, Type 2 is to protect each subframe by both MCS and FEC, and Type 3 is to divide a frame into MSB subframes and LSB subframes, each of which are aggregated into separated frames and protected by both MCS and FCS. UEP Type 1 protects an aggregated frame which consists of msb subframes, lsb subframes or both msb and lsb subframes by using the different FECs.

UEP Type 2 protects an aggregated frame which consists of msb subframes, lsb subframes or both msb and lsb subframes by using different MCSs. UEP Specific IE, defined in clause 7.4.24, is only used in UEP Type 2 in order for the originating DEV to inform the recipient DEV of the separating position of msbs and lsbs. If necessary, UEP Type 2 can transmit announce command frame with UEP Specific IE to dynamically change the separating position.

UEP Type 3 protects MSB and LSB in a subframe unequally by either applying different FEC coding rates to msbs and lsbs or mapping msbs and lsbs to a skewed constellation. Capability of supporting UEP by FEC coding and/or by the skewed constellation is defined in 7.4.7.

All UEP types, UEP Type 1, UEP Type 2 and UEP Type 3, can be used in SC PHY as described in 12.2.2.4. UEP Type 3 is only used in HSI OFDM and AV OFDM as described in 12.3.2.5 and 12.4, respectively. UEP only supports color spaces with 8 bits per color.

It is required for originating and target DEVs to understand the UEP type and MCS information of each other before sending frames that use UEP. This is accomplished by the DEVs reporting their UEP capability in the DEV Capability IE in the association process. If the source and destination do not support the same UEP type, then only EEP frames shall be exchanged between the DEVs.

12.2.2.4 Unequal Error Protection

The MCS types, which are used for unequal error protection, are shown in Table 109. The UEP MCSs for SC PHY mode are shown in Table 109.

Table 109—MCS types used for UEP UEP MCS for SC PHY mode

In UEP Type 3, when the transmitter chooses one of the UEP MCS types, the transmitted data are divided into two groups, msb and lsb, as shown in Figure 195. The msb and lsb groups are processed in parallel to apply adopt different LDPC coding rates. The outputs of the two LDPC encoders are multiplexed to generate a single bit stream for the symbol mapping.

LDPC(576, 228) LDPC (672,336), LDBPC(576, 432) LDPC (672,504), 3:2 alternating

LDPC(576, 432) LDPC (672,504), LDBPC(576, 504) LDPC (672,588), 7:6 alternating

Figure 196—UEP bit multiplexing/interleaving