IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Comment Resolutions for TX switched diversity |  |
| Date Submitted | Sep, 2008 |  |
| Source | [Chang woo Pyo, Zhou Lan, Fumihide Kojima, Hiroyuki Nakase, Hirosh Harada, Shuzo Kato] | Voice: [+81-46-847-5092] Fax: [+81-46-847-5440] E-mail: [cwpyo@nict.go.jp] |
| Re: | 802.15.3c Teleconference Meeting |  |
| Abstract | IEEE 802.15 Task Group TG3c Comment Resolution |  |
| Purpose | Comment Resolutions for TX switched diversity |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

This document resolves the comments related to TX diversity

Note that blue colored sentences are newly added or modified

Modify 6.3.19 (comment 23)

6.3.19 Transmit switched diversity

Table 3z—MLME-TXDIV primitive parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| TrgtID | Integer | Any valid DEVID, as defined in 7.2.3 | Specifies the DEVID of the target DEV for exchanging transmit Switched Diversity information. |
| OrigID | Integer | Any valid DEVID, as defined in 7.2.3 | The DEVID of the DEV that initiated the MLME request. |
| SupportedNumAntennas | Integer | 0-15 | The number of antennas supported by the DEV. |
| Timeout | Integer | 0-65535 | The time in milliseconds allowed for the primitive to complete. |
| TxDiversityThresholdType | Enumeration | SINR, RSSIR, FER_EXPONENT, BER_EXPONENT LQI as defined in 12.x.x.x | Specifies the type of threshold touse to trigger antenna switching. |
| TxDiversityTheshold | Integer | 0-255 | Specifies the value of the threshold based on the TxDiversityThresholdType. |
| AntennaSwitchingStatus | Enumeration | NOT_SWITCH, RESERVED, SWITCH, BEST_SWITCH | Indication of antenna switching status. |
| AntennaIndex | Integer | 0-15 | Specifies antenna index to be used. |
| ResultCode | Enumeration | SUCCESS, FAILURE | Indicates the result of the MLME request. |
| ReasonCode | Enumeration | NOT_SUPORTED, OTHER | Indicates the reason for a ResultCode of FAILURE. |

The TXDiversityThreshold is encoded based on the value of TXDiversityThresholdType parameter. The

encodings are:

— SINR: 0 dB to 30 dB in 1 dB steps.

— RSSIR: 0 dB to 30 dB in 1 dB steps.

— FER_EXPONENT: The negative of the exponent, from 1 to 10 in 0.25 step size.

— BER_EXPONENT: The negative of the BER exponent from 1 to 10 in 0.25 step size.

For example, an SINR or RSSIr of 14 dB would be encoded as 14 where as a BER or FER exponent of -3.25 representing 10-3.25 would be encoded as 9 (the negative of the exponent, minus one and multiplied by four).

Comment 57, 536, 583

Remove Transmit switched diversity IE (7.2.23)

Modify Feedback IE (7.4.36)

7.4 Information elements

Table 48—Information elements

| Element ID hex value | Element | Subclause | Present in beacon |
| --- | --- | --- | --- |
| 0x15 | Transmit switched diversity | 7.4.23 | As needed |

7.4.23 Transmit switched diversity

Figure 48g—Transmit switched diversity information element format

Add the following fields on Feedback IE

7.4.27 Feedback

| Bits: 2 | 4 | 4 | 4 | 4 | 4 | 1 | 1 | 8 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Transmit beam index | Second best sector SINR | Second best sector | Best sector SNIR | Best sector | Beam switch utility | Mode | Length | Element ID |

Figure 48m—Feedback information element format

Mode shall be set to 0 for feedback mode of beamforming, otherwise shall be set to 1 for TX switch mode of the originating DEV beam.

Beam Switch Utility shall be set to 1 to request the originating DEV to switch the transmit beam, otherwise shall be set to 0 to response the switching result of the originating DEV.

Transmit Beam Index shall be set to the index of the transmit beam of the originating DEV.

Remove 7.5.11, 7.5.11.1and 7.5.11.2

Remove Figure 90a, Figure 90b and Figure 90c

Modify Table 50 and Remove Table 54a

7.5 MAC command types

Table 50—Command types

| Command type hex value b15-b0 | Command name | Subclause | Associated | Secure membership (if required) |
| --- | --- | --- | --- | --- |
| 0x0025 | Transmit switched diversity request | 7.5.11.1 | X | X |
| 0x0026 | Transmit switched diversity response | 7.5.11.2 | X | X |
| 0x0025-0x00FF | Reserved |  |  |  |

7.5.11 Transmit switched diversity

7.5.11.1 Transmit switched diversity request

Figure 90a—Transmit switched diversity request command format

Figure 90b—Switching status field

Table 54a—Indication of antenna switching

7.5.11.2 Transmit switched diversity response

Figure 90c—Transmit switched diversity response command format

Remove 8.16.1 and figure 147c and 147d

8.1 Introduction

The aggregation is described in 8.7a. Channel probing is described in 8.9.7. The use of switched diversity is described in 8.16.1. UEP is described in 8.17.

8.16 Directional antennas

Directional communications enable a DEV to increase the antenna gain to support higher data rates and/or

longer distances. Support for two types of directional antennas are provided, beam forming and transmit

switched diversity.

8.16.1 Transmit switched diversity

Transmit switched diversity is used to achieve diversity gain from shadowing or blockage. The process is

performed in CAP by using the base rate. To perform transmit switched diversity, The PNC needs to have

multiple antennas that share one common RF chain, as illustrated in Figure 147c.

The PNC first informs the DEV of the number of transmit antennas. In the first transmission, the PNC arbitrarily selects one antenna from multiple transmit antennas. When the DEV receives a packet from the PNC, the DEV check the received SNR and compares it to a predetermined threshold. According to the relationship between the received SNR and the threshold, the DEV sends an MLME-TXDIV.request to the MAC/MLME. Then, DEV sends the Transmit Switched Diversity Request command, as described in 7.5.11.1, to the PNC. If the received SNR is larger than the threshold, the DEV will feedback ‘no-switch’ information to PNC. If the received SNR is smaller than the threshold, the DEV will feedback ‘switch’ information to PNC. If the SNR for all transmit antennas are less than the predetermined threshold, DEV will feedback the index of the transmit antenna with the largest received SNR. According to the information in the command frame, the PNC performs the switching operation. PNC then sends Transmit Switched Diversity Response command as defined in 7.5.11.2, containing the switching results to DEV. A fter that, PNC MAC/MLME s ends MLME-TXDIV.indication including antenna i ndex information to PNC DME.

The originating DEV after receiving Transmit Switched Diversity Response command shall be able to terminate transmit switched diversity process according to the result.

Figure 147d illustrates the message sequence for transmit switched diversity request process.

Figure 147c—The structure of transmit switched diversity system

Figure 147d—MSC for transmit switched diversity

13.4.2 Beam tracking and switching

Add the following sentences on the end of 13.4.2

During a data transmission from DEV1 to DEV2, when tracking is not able, DEV2 may request DEV1 to switch the transmitting beam according to LQI as defined in 12.x.x.x. To switch the transmitting beam of DEV1, DEV2 shall send the announce command frame to DEV1, which contains Feedback IE, as described in 7.4.27, where the mode field and the beam switch utility field shall be set to 1 and the transmit beam field indicates the next transmitting beam of DEV1.

According to the feedback information in the announce command frame from DEV2, DEV1 may switch the current transmitting beam to the next transmitting beam indicated in Feedback IE, as described in 7.4.27. DEV1 shall send the announce command containing Feedback IE of the switching result to DEV2, where the mode field shall be set to 1, the beam switch utility field shall be set to 0 and the transmit beam field indicates the switched beam of DEV

Modify Table D.3and Table D.4 in Annex D

Annex D

Table D.3—MAC frames

| Item Number | Item Description | Reference | Transmitter |  | Receiver |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Support N/A Yes No | Status | Support N/A Yes No |
| MF4.8 | PNC handover request | 7.5.3.1 | O |  | O |  |
| MF4.9 | PNC handover response | 7.5.3.2 | O |  | O |  |
| MF4.10 | PNC handover information | 7.5.3.3 | O |  | O |  |
| MF4.33 | Block ACK | 7.3.2.3 | O |  | O |  |
| MF4.34 | Beam forming request | 7.5.11.1 | O |  | O |  |
| MF4.35 | Beam forming response | 7.5.11.2 | O |  | O |  |
| MF4.36 | Channel probing response | 7.5.12.2 | O |  | O |  |
| MF4.37 | UEP information request | 7.5.12.1 | O |  | O |  |
| MF4.38 | UEP information response | 7.5.13.2 | O |  | O |  |
| MF4.39 | UEP path request | 7.5.13.3 | O |  | O |  |
| MF4.40 | UEP path response | 7.5.13.4 | O |  | O |  |
| MF4.41 | Transmit switched diversity request | 7.5.14.1 | O |  | O |  |
| MF4.42 | Transmit switched diversity response | 7.5.14.2 | O |  | O |  |

Table D.4—MAC sublayer functions

| Item number | Item Description | Reference | Status | Support |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | N/A | Yes | No |
| MLF3 | PNC handover capable | 8.2.3 | O |  |  |  |
| MLF23 | Aggregation | 8.7a | O |  |  |  |
| MLF24 | Block ACK | 8.8.3b | O |  |  |  |
| MLF25 | Beam forming | 13 | O |  |  |  |
| MLF26 | Channel probing | 8.16.1 8.9.7 | O |  |  |  |
| MLF27 | UEP | 8.17 | O |  |  |  |
| MLF28 | Transmit switched diversity | 8.16.2 | O |  |  |  |