IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Edits to TG6 Technical Requirements Document (TRD) |  |
| Date Submitted | [9 September, 2008] |  |
| Source | [Maulin Patel, Philips] [Srinath Hosur, TI] [Anuj Batra, TI] [Okundu Omeni, Toumaz] [Rob Davies, Philips] [Jeremy Moss, Philips] [Sthanu Ramakrishnan, TI] | E-mail: [maulin.patel@philips.com] [hosur@ti.com] [batra@ti.com] [okundu.omeni@toumaz.com] [rob.j.davies@philips.com] [j.moss@philips.com] [sthanu@ti.com] |
| Re: | [Body Area Network (BAN) Technical Requirements document] |  |
| Abstract | [Edits to BAN Technical Requirements] |  |
| Purpose | [This document illustrates edits to the BAN Technical Requirements documentation] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

1.0 General	3

2.0 Introduction	5

3.0 BAN Technical Characteristics Summary	5

4.0 Topology	7

5.0 Bit Rates	7

6.0 Transmission range	10

7.0 Security	10

8.0 Quality of Service (QoS)	10

9.0 Power Consumption	11

10.0 Coexistence and Interference Resistance	11

11.0 Form Factor	12

12.0 Antenna	12

13.0 Complexity	12

14. Mobility	12

15. Specific Absorption Rate (SAR)	13

16. Regulatory Matters	13

17. References	13

# 1.0 General

This technical requirement document (TRD) describes the technical aspects that TG6 standard must fulfill, such as performance-related issues, reliability issues and availability issues. These types of requirements are often called quality of service (QoS) requirements; other requirements are usually maintenance-level requirements or external constraints, sometimes called compliance. Technical requirements are summarized as any other specifications; they have a name and a unique identifier. Technical requirements are documented in the same manner as any specifications, including a description, an example, a source or references to related technical requirements and a revision history.

TG6 needs to effectively define and manage requirements to ensure they are meeting needs of the BAN users, while proving compliance.

Ideally, requirements are:

• Correct (technically and legally possible)

• Complete (express a whole idea or statement)

• Clear (unambiguous and not confusing)

• Consistent (not in conflict with other requirements)

• Verifiable (it can be determined that the system meets the requirement)

• Traceable (uniquely identified and trackable)

• Feasible (can be accomplished within cost and schedule)

• Modular (can be changed without excessive impact)

• Design-independent (does not pose a specific solution on design)

Each requirement must first form a complete sentence, containing a subject and a predicate. These sentences must consistently use the verb “shall”, “will” or “must” to show the requirement's mandatory nature, and “should” or “may” to show that the requirement is optional. The whole requirement specifies a desired end goal or result and contains a success criterion or other measurable indication of the quality.

TRD needs to capture these levels of user requirements, maintaining intelligent traceability and change impact analysis between them.

Typical constraint requirements can specify:

• Performance

• Interfaces

• Security

• Safety

• Reliability

• Availability

• Maintainability

An efficient way of writing better requirements is to ensure they are clearly mapped to test cases. Making sure each requirement is clearly verifiable from the start, not only helps prepare later phases of the project, it also puts the developer in the correct state of mind. Requirements and their associated tests must also indicate what the system should not do, and what happens at the limits (degraded mode).

This rule also applies for compliance requirements: indicating how they shall be tested is a good way to write better requirements.

TRD need to implement a reliable and repeatable change control process that helps turn this challenge into an opportunity.

By providing examples and counter-examples of good requirements and documents, IEEE can enhance the quality, consistency, and completeness of the requirements. These can originally be templates, industry standards and rules inside a repository, such as the IEEE server.

Requirement typical sentence construction

Defects to avoid:

- Vagueness

- Weakness

- Over specification

- Subjectivity

- Multiplicity

- Unclear meaning

- Implicit meaning

Some words to be used with caution:

“adequate”, “applicable”, “appropriate”, “approximate”, “bad”, “best practice”, “between”, “clearly”, “compatible”, “completely”, “consider”, “could”, “down to”, “easy/easily”, “effective”, “efficient”, “equivalent”, “excellent”, “good”, “his/her”, “however”, “ideal”, “etc”, “in order to”, “include but shall not be limited to”, “least”, “like”, “low”, “maximise”, “may”, “most”, “minimum/mal”, “must”, “nearly”, “necessary”, “needed”, “normal”, “or”, “possible/bly“, “practicable”, “provide”, “quality”, “readily”, “relevant”, “safe/ly“, “same”, “should”, “significant”, “similar”, “so as”, “subject to”, “substantial”, “sufficient”, “suitable”, “support”, “target”, “typical”, “up to”, “user friendly”, “whether”, “will”, “with”, “worse”.

# 2.0 Introduction

This document provides the technical contents of the project to develop PHY and MAC protocols for Body Area Network. This document will provide guidance on how to respond to a call for proposals. As for any communication protocol, the reference model used for this standard is the following:

MAC_SAP: MAC Service Access Point

PHY_SAP: PHY Service Access Point

PMD: Physical Medium Dependent (radio)

PLCP: PHY Layer Convergence Protocol,

Figure 1, Reference partitioning

This document serves two purposes.  First, it summarizes the applications presented in response to BAN Study group and TG6 Call for Applications. Second, it describes and defines the fundamental requirements implied by applications but not necessarily stated explicitly.

# 3.0 BAN Technical Characteristics Summary

The intended standard will define the PHY and MAC layers for short range, wireless communication in and around the body area. The standard aims to support a low complexity, low cost, ultra-low power and highly reliable wireless communication for use in close proximity to, or inside, a human body (but not limited to humans) to satisfy an evolutionary set of entertainment and healthcare products and services. The project will also address the coexistence.

3.1 High level description

The standard intends to address both medical/healthcare applications and other non-medical applications with diverse requirements. The medical applications cover continuous waveform sampling of biomedical signals, monitoring of vital signal information, and low rate remote control of medical devices. The non-medical applications include video and audio, bulk and small data transfer, and command and control for interactive gaming etc. Dependent on the application, the BAN may require a network of anywhere from a few sensor or actuator devices communicating to a portable handset or PDA, to potentially hundreds of sensors and actuators (e.g. EEG) communicating to a gateway device through which is connected to the Internet or a local or wide area network.

Devices for the above applications are usually highly constrained in terms of resource such as CPU processing power, battery capacity and memory size and operate in unstable environments. At the same time, medical sensors and actuators have to be physically small to be wearable or implantable. The wearable gateway device may also have some form of resource constraint. However, it is relatively more powerful than the medical sensor and actuator.

The devices would operate indoor, outdoor in home, hospital, small clinic, fitness center etc. There may be interference from and to the other devices in the environment. Patients may simultaneously have both medical application and non-medical application, and both wearable and implantable applications on/in its body.

Because of the space limitation and location dependent characteristics of medical information, it is unlikely to deploy redundant medical sensors for vital information collection. As a result, there is little redundancy in the traffic. Depending on the philosophy of medical application, the major traffic tends to be point-to-multipoint (e.g. stimuli) and multipoint-to-point (e.g. ECG). Therefore the traffic flow can be asymmetric. During diagnosis, doctor may investigate a parameter in a command/response mode. The “downstream” traffic (commands) is coming from the gateway to a particular sensor or actuator.

Most of biomedical and vital signals tend to be periodic and of low frequency. The packet generation interval can vary from 1ms to 1000s. Other applications, such as motion detection and fall detection for the elderly or infirm, can be event-based/bursty. Some applications may involve transmitting a log file once a day, with typically Kbytes of data. Some medical sensors may detect alarm conditions. Time crucial alarm packets are expected to have higher priority than sensing data.

Remote medical monitoring and control applications can be “open loop” or “closed loop”. In the former case, sensor data makes its way through the gateway to the caregiver who may decides to take an action, and control information is sent out to the actuator in the network. It is envisioned that the “closed loop” control is the future trend, in which packets will flow over local loops without intervention from the caregiver. Close loop control may have a latency requirement that can be 100ms to seconds. In many of these applications, if the packets do not arrive within the specified interval, the system will enter an emergency alarm state, often with live or dead indication.

Non-medical applications are generally point-to-point. The real time video and audio traffic are particular sensitive to the end-to-end delay and delay variant. For example, the end-to-end delay in interactive game should be less than 250 msec.

3.2 Overall requirements

Given the broad range of possible applications space, the key feature for this standard must be to address the issue of scalability in terms of data rates, power consumption, network size, and security. In some cases the tradeoff for speed or security may increase power consumption or indeed for improved quality of service. Other tradeoffs will need to be addressed in either or both the MAC or Physical layers. This project should formulate and propose the methods that this can be achieved, whilst demonstrating the key goals of improved energy efficiency is indeed achieved.

Anticipated high-level characteristics of the MAC and PHY layers are summarized as follows.

- The BAN should be self-forming, (good management) secure, robust and reliable, shall be able recover from link & node failures.

- The data rate requirements of Section 5.0 shall be met.

- The power consumption should allow for self-powered operating time without intervention from several hours to several years, depending on applications.

- The QoS support shall be provided as defined in Section 8.0.

- Security should be lightweight, scalable and energy efficient;

- Co-existence between wearable and implant BANs, coexistence between BAN and other wireless technologies, and coexistence of BAN in medical environments (EMC/EMI) should be addressed;

- Minimization of SAR into the body to satisfy the local regulatory requirements.

Specific detailed requirements associated with these characteristics are described in the sections that follow.

# 4.0 Topology

The network components may be in close proximity to, or inside, a human body. Bi-directional links are required;

The network should be workable without requiring complex set up procedure and shall tolerate dynamic insertion and de-insertion of nodes into a network.

The network configuration should be efficiently scalable upto 64 nodes .

Typical applications imply data collection by a unique or a set of coordinated data collectors. The data collectors should be able to transmit multicast messages to a set of nodes.

# 5.0 Spectral Efficiency

There shall be at least one mandatory PHY data rate that has a spectral efficiency between 1/6 and 1/3 bps/Hz. Of course, this constraint does not preclude other data rate with differing spectral efficiencies as options or additional mandatory PHY data rates.

# 6.0 Transmission range

A 95% outage transmission range of at least 3 meters shall be supported for the lowest mandatory PHY data rate under the approved channel models.

7.0 Security

Medical BAN applications have substantial financial, privacy and human safety implications. Security and privacy are key concerns of patients, doctors and medical service providers. During the transmission of encrypted physiological data, patient identifiers should be particularly protected from overhearing.

The level of security should be equal to or stronger than those specified by IEEE 802.15.4-2006 standard.

Because security and privacy protection mechanism require a significant amount of computational resource and storage resource, the mechanism should be energy efficient and lightweight.

# 8.0 Quality of Service (QoS)

QoS is an important part in the framework of risk management for BAN applications. QoS support should be a major focus of PHY and MAC. The critical factor is the reliability of the transmission, meaning that appropriate error detection and correction methods, interference avoidance methods, or any other suitable techniques should be provided at PHY and MAC level. Other QoS measurements include point-to-point delay and delay variation. QoS provisions should be flexible such that they can be tailored to suit application needs.

Other QoS parameters have a strong impact on MAC and PHY layers:

- QOS mechanisms shall support the latency requirements as defined in the applications summary document [5].

- Capability of providing fast (<? sec) and reliable (99.9999%) reaction in emergency situations and alarm message, which have higher priority than others, shall be provided; (TB Checked)

- Power saving mechanism (such as duty cycling) shall be provided, whilst not impacting application latency requirement.

- Mechanisms for Channel migration shall be provided.

# 9.0 Power Consumption

All proposals shall include support for mechanisms that minimize the power consumption for devices. For example, some possible mechanisms that can be used to minimize power consumption are duty cycling, power save modes and coordination of nodes to limit frequent wake-up.

# 10.0 Coexistence and Interference Resistance

BAN devices shall co-exist with other BAN devices and other wireless devices using the same spectrum.

The devices must be able to operate in high noise, high multipath and dynamic environment. The PHY layer must be able to sustain an appropriate level of co-channel and out-of-band interference. Both medical application in hospital, small clinic, healthcare center and home have to be considered, along with wearable entertainment applications.

MAC should support simultaneous co-located operation of multiple BANs in crowded places such as subways, hospital ward, music concert etc. In particular, implantable BAN and wearable BAN should gracefully coexist in-and-around the body.

# 12.0 Antenna

No assumption is made about antenna radiation pattern. The antenna(s) should be specifically designed to operate in a body centric environment and handle the issues resulting therein, such as propagation through and around the body.

# 13.0 Complexity

Complexity should be minimized to enable mass commercial adoption for a variety of cost sensitive products. Complexity (gate count estimates, die size estimates) and bill-of-material (BOM) should be provided as part of the proposal. Note that in a number of applications, the final solutions may be disposable.

# 15. Specific Absorption Rate (SAR)

At the frequencies of operation of most wireless devices, the known health effects that centre around tissue is heating. BAN devices are quite low power and there is not enough power available for whole-body SAR to be a concern. However, since the device may be in close proximity to, or inside, a human body, the localized SAR could be quite large if all the available power is deposited in a small volume. The localized SAR into the body must be minimized.  BAN devices shall comply with international or local SAR regulations. SAR is regulated, with limits for local exposure (Head) of: in US: 1.6 W/kg in 1 gram and in EU: 2 W/kg in 10 gram. This limits the TX power in US < 1.6 mW and in EU < 20 mW.

# 16. Regulatory Matters

The transmit spectrum mask (measured by EIRP or ERP) at PHY layer shall comply with necessary geopolitical or regional regulations. This is defined in greater detail in 15-08-0034-05-0006-ieee-802-15-6-regulation-subcommittee-report. Ref  [6].

Devices shall comply with the regulatory requirements specified for the chosen frequency band by relevant authorities. If possible, a common worldwide frequency band is desirable for harmonization worldwide.

# 17. References

- IEEE 802.15.4-006 standard

- BAN application matrix (07-0735-08)

- 15-07-0735-08-0ban-application-requirement-analysis.xls

- 15-08-0406-00-0006-tg6-applications-matrix.xls

- 15-08-0407-00-0006-tg6-applications-summary.doc

- 15-08-0034-05-0006-ieee-802-15-6-regulation-subcommittee-report.doc