Letter to IEC TC 76

Dear Mr. Dennis,

LEDs emitting in the visible spectral range have been used for signalling in the past and are expected to dominate illumination and display applications in the near feature. Besides their well-known attractive properties (e.g., high energy efficiency) they offer the opportunity for simultaneous free-room data transmission – due to their unique high-speed modulation capabilities via their driving current. Data transmission rates of up to 100 Mbit/s have already been realised with test lamps and 1 Gbit/s in display applications. The combination of illumination or signalling with (in-door) free-space data transmission offers a series of new and promising applications, for example in mobile-to-mobile communication, as well as building and industrial automation. The potential of this emerging technology has been studied by academic institutions around the world, and its technological feasibility has been assessed by numerous companies, many of them affiliated with the Visible-Light Communications Consortium in Japan, and, currently, within OMEGA, an EU R&D project within the seventh frame programme.

The maturity of the technology and its approaching commercialisation recently triggered the formation of a new standardisation study group, which is affiliated with IEEE 802.15. The current time line is to draft a standard for communication with light by the end of 2010.

It is obvious that such a standard will have to address the radiation safety requirements. Also, LEDs used as lamps would have to be assessed and classified according to the new lamp safety standard IEC 62471. In terms of this standard, even the relaxed requirements for GLS-sources (General Lighting Service) can be applied. Thus, the lighting part of the technique will definitely be covered by these requirements.

However, we are concerned and unsure about the data transmission part, since LEDs used for free-space data transmission are currently covered by the scope of the application-related laser safety standard IEC 60825-12. We feel that the latter standard is, due to the underlying laser safety philosophy, much more restrictive compared to the lamp standard. However, in our opinion, due to the high-speed modulation used for optical wireless data transmission, the sources would still have to be considered as CW sources in terms of eye safety (i.e., the average pulse criterion applies). Therefore, there should be no added hazards arising just from modulating the LED. Additionally, especially in lighting scenarios, the emitted light would only be modulated when there is data to be sent. So, in packet-based data transmission, one would experience periods with and without modulation. Hereby, it is worth mentioning that the modulation-free periods readily would exceed seconds if not even minutes. According to existing safety standards the durations without modulation would have to be addressed by IEC 62471, while the very same LED would have to adhere to IEC 60825-12 during data transmission. So, in essence, one would end up with a device that repeatedly transits from one standard to the other during operation.

It is understood, that there is a consensus in principle within TC 76 to remove LEDs from the scope of IEC 60825-12. We, the IEEE 802.15 study group for visible-light consideration would support such a move because

- a simplified but still appropriate safety assessment of communication LEDs with one standards than rather two will simplify and ease the development and market-introduction of safety-compliant products , and

- as already expressed, we fear that due to the unique transitions between pure illumination and illumination in concert with data transmission the very same device would cycle through both safety standards during operation, which, in our eyes is a highly undesirable  and confusing situation.

We would therefore kindly ask you to discuss this issue at the next meeting IEC in November in Beijing.

With best regards,

(Eun Tae Won, chair)

on behalf of the IEEE 802.15 study group on visible-light communications.