IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Comment Resolutions for TX switched diversity |  |
| Date Submitted | Sep, 2008 |  |
| Source | [Chang woo Pyo1, Zhou Lan1, Fumihide Kojima1, Hiroyuki Nakase1, Hirosh Harada1, Shuzo Kato1,Yongsun Kim2, Kyeongpyo Kim2, Seung-Eun Hong2 ,Hyoungjn Kwon2, Wooyong Lee2] | Company [NICT1, ETRI2] Voice: [+81-46-847-5092] Fax: [+81-46-847-5440] E-mail: [cwpyo@nict.go.jp] |
| Re: | 802.15.3c Teleconference Meeting |  |
| Abstract | IEEE 802.15 Task Group TG3c Comment Resolution |  |
| Purpose | Comment Resolutions for TX switched diversity |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

This document resolves the comments related to TX diversity

Note that blue colored sentences are newly added or modified

Modify 6.3.19

6.3.19 Transmit switched diversity

Table 3z—MLME-TXDIV primitive parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| TrgtID | Integer | Any valid DEVID, as defined in 7.2.3 | Specifies the DEVID of the target DEV for exchanging transmit Switched Diversity information. |
| OrigID | Integer | Any valid DEVID, as defined in 7.2.3 | The DEVID of the DEV that initiated the MLME request. |
| SupportedNumAntennas | Integer | 0-15 | The number of antennas supported by the DEV. |
| Timeout | Integer | 0-65535 | The time in milliseconds allowed for the primitive to complete. |
| TxDiversityThresholdType | Enumeration | SINR, RSSIR, FER_EXPONENT, BER_EXPONENT LQI as defined in 12.x.x.x | Specifies the type of threshold to use to trigger antenna switching. |
| TxDiversityTheshold | Integer | 0-255 | Specifies the value of the threshold based on the TxDiversityThresholdType. |
| AntennaSwitchingStatus | Enumeration | NOT_SWITCH, RESERVED, SWITCH, BEST_SWITCH | Indication of antenna switching status. |
| AntennaIndex | Integer | 0-15 | Specifies antenna index to be used. |
| ResultCode | Enumeration | SUCCESS, FAILURE | Indicates the result of the MLME request. |
| ReasonCode | Enumeration | NOT_SUPORTED, OTHER | Indicates the reason for a ResultCode of FAILURE. |

The TXDiversityThreshold is encoded based on the value of TXDiversityThresholdType parameter. The

encodings are:

— SINR: 0 dB to 30 dB in 1 dB steps.

— RSSIR: 0 dB to 30 dB in 1 dB steps.

— FER_EXPONENT: The negative of the exponent, from 1 to 10 in 0.25 step size.

— BER_EXPONENT: The negative of the BER exponent from 1 to 10 in 0.25 step size.

For example, an SINR or RSSIr of 14 dB would be encoded as 14 where as a BER or FER exponent of -3.25 representing 10-3.25 would be encoded as 9 (the negative of the exponent, minus one and multiplied by four).

Add TSD support field on Capability IE

Modify Transmit switched diversity IE

7.4 Information elements

7.4.7 Capability

The TSD support bit shall be set to one if the DEV is capable of transmit switched diversity (TSD) as defined in 13.7. Otherwise the bit shall be set to zero.

7.4.23 Transmit switched diversity (TSD)

The Transmit Switched Diversity IE shall be formatted as illustrated in Figure 48g.

| octets: 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- |
| Transmit direction index | TSD switching request time | Number of transmit directions | mode | Length | Element ID |

Figure 48g—Transmit switched diversity information element format

The Mode field shall be encoded as shown in Table 54a.

Table 54a—mode field value

| Value | Mode | descriptions |
| --- | --- | --- |
| 0 | Announce | Announce TSD IE as needed |
| 1 | Request | Request to switch the next transmit direction |
| 2 | Request unknown | Request the current transmit direction |
| 3 | Response | Response to the result of the switching request |
| 4~255 | Reserved |  |

The number of transmit directions field indicates the number of TX directions supported by.PNC or DEV

The TSD switching request time field indicates the time at which the DEV shall transmit TSD IE to the PNC periodically.

Transmit direction index field indicates the index of the transmit direction of the PNC. If transmit direction index is set to 0, the PNC selects one of transmit directions which are previously not in use.

Remove 7.5.11, 7.5.11.1and 7.5.11.2

Remove Figure 90a, Figure 90b and Figure 90c

Modify Table 50 and Remove Table 54a

7.5 MAC command types

Table 50—Command types

| Command type hex value b15-b0 | Command name | Subclause | Associated | Secure membership (if required) |
| --- | --- | --- | --- | --- |
| 0x0025 | Transmit switched diversity request | 7.5.11.1 | X | X |
| 0x0026 | Transmit switched diversity response | 7.5.11.2 | X | X |
| 0x0025-0x00FF | Reserved |  |  |  |

7.5.11 Transmit switched diversity

7.5.11.1 Transmit switched diversity request

Figure 90a—Transmit switched diversity request command format

Figure 90b—Switching status field

Table 54a—Indication of antenna switching

7.5.11.2 Transmit switched diversity response

Figure 90c—Transmit switched diversity response command format

Remove 8.16.1 and figure 147c and 147d

8.1 Introduction

The aggregation is described in 8.7a. Channel probing is described in 8.9.7. The use of switched diversity is described in 8.16.1. UEP is described in 8.17.

8.16 Directional antennas

8.16.1 Transmit switched diversity

Transmit switched diversity is used to achieve diversity gain from shadowing or blockage. The process is

performed in CAP by using the base rate. To perform transmit switched diversity, The PNC needs to have

multiple antennas that share one common RF chain, as illustrated in Figure 147c.

The PNC first informs the DEV of the number of transmit antennas. In the first transmission, the PNC arbitrarily selects one antenna from multiple transmit antennas. When the DEV receives a packet from the PNC, the DEV check the received SNR and compares it to a predetermined threshold. According to the relationship between the received SNR and the threshold, the DEV sends an MLME-TXDIV.request to the MAC/MLME. Then, DEV sends the Transmit Switched Diversity Request command, as described in 7.5.11.1, to the PNC. If the received SNR is larger than the threshold, the DEV will feedback ‘no-switch’ information to PNC. If the received SNR is smaller than the threshold, the DEV will feedback ‘switch’ information to PNC. If the SNR for all transmit antennas are less than the predetermined threshold, DEV will feedback the index of the transmit antenna with the largest received SNR. According to the information in the command frame, the PNC performs the switching operation. PNC then sends Transmit Switched Diversity Response command as defined in 7.5.11.2, containing the switching results to DEV. A fter that, PNC MAC/MLME s ends MLME-TXDIV.indication including antenna i ndex information to PNC DME.

The originating DEV after receiving Transmit Switched Diversity Response command shall be able to terminate transmit switched diversity process according to the result.

Figure 147d illustrates the message sequence for transmit switched diversity request process.

Figure 147c—The structure of transmit switched diversity system

Figure 147d—MSC for transmit switched diversity

Modified the following sentences

P. 159, replace lines 29:30 by:
Support for beam forming is optional. However, when the beam forming in this clause is implemented, the sector level training shall be supported for switched/sectored antennas and the two-level
training shall be supported for all other antenna configurations. The tracking phase is optional.

P.164, line 50, replace
"For example, for sectored ..." by "For example, for switched/sectored ..."

Add the following sub section 13.7

13.7 Transmit switched diversity

Transmit switched diversity may be used in case that the PNC is capable of multiple directional antennas that are switched one to another and the DEV is capable of the antenna with omni. The TSD capability is indicated in the TSD supported field in Capability IE, as described in 7.4.7.

In case that PNC sends directional beacons, the association procedure of TSD shall follow the directional association as described in 8.6.6.3. During data transmission from PNC to DEV in CTA, TSD may be used. When TSD is used, the DEV shall check the LQI from the data frame and shall compare the LQI with TxDiversityTheshold as described in 6.3.19. If the LQI is lower than TxDiversityTheshold, the DEV shall send the announce command frame to the PNC with TSD IE, which shall set the mode field to 1 and set the transmit direction index field to the next transmit antenna of the PNC. The PNC shall respond to the announce command frame to the DEV with TSD IE, which shall set the mode field to 3 and the transmit direction index field to the corresponding transmit direction of the PNC. The PNC may transmit data frames after switching the transmit direction to the new transmit direction.

The PNC may announce TSD switching request time to the DEV by using the announce command frame with TSD IE which shall set the mode field to 0 and set TSD switching request time field. The DEV shall feedback to the PNC periodically by using the announce command frame according to TSD switching request time whether the current transmit direction of the PNC is changed or not, where the announce command frame includes TSD IE that shall set the mode field to 1 and set the transmit direction index field to the corresponding transmit direction of the PNC.

If the DEV may not know the transmit direction index, the DEV may send the announce command frame with TSD IE to the PNC which shall set the mode field to 2. The PNC shall send the announce command frame with TSD IE to the PNC, which shall set the mode field to 3 and set the transmit direction index filed set to the current antenna index of the PNC.

Modify Table D.3and Table D.4 in Annex D

Annex D

Table D.3—MAC frames

| Item Number | Item Description | Reference | Transmitter |  | Receiver |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Support N/A Yes No | Status | Support N/A Yes No |
| MF4.8 | PNC handover request | 7.5.3.1 | O |  | O |  |
| MF4.9 | PNC handover response | 7.5.3.2 | O |  | O |  |
| MF4.10 | PNC handover information | 7.5.3.3 | O |  | O |  |
| MF4.33 | Block ACK | 7.3.2.3 | O |  | O |  |
| MF4.34 | Beam forming request | 7.5.11.1 | O |  | O |  |
| MF4.35 | Beam forming response | 7.5.11.2 | O |  | O |  |
| MF4.36 | Channel probing response | 7.5.12.2 | O |  | O |  |
| MF4.37 | UEP information request | 7.5.12.1 | O |  | O |  |
| MF4.38 | UEP information response | 7.5.13.2 | O |  | O |  |
| MF4.39 | UEP path request | 7.5.13.3 | O |  | O |  |
| MF4.40 | UEP path response | 7.5.13.4 | O |  | O |  |
| MF4.41 | Transmit switched diversity request | 7.5.14.1 | O |  | O |  |
| MF4.42 | Transmit switched diversity response | 7.5.14.2 | O |  | O |  |

Table D.4—MAC sublayer functions

| Item number | Item Description | Reference | Status | Support |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | N/A | Yes | No |
| MLF3 | PNC handover capable | 8.2.3 | O |  |  |  |
| MLF23 | Aggregation | 8.7a | O |  |  |  |
| MLF24 | Block ACK | 8.8.3b | O |  |  |  |
| MLF25 | Beam forming | 13 | O |  |  |  |
| MLF26 | Channel probing | 8.16.1 8.9.7 | O |  |  |  |
| MLF27 | UEP | 8.17 | O |  |  |  |
| MLF28 | Transmit switched diversity | 8.16.2 | O |  |  |  |