IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Channel Models for WBANs – Samsung Electronics |  |
| Date Submitted | [11 November, 2008] |  |
| Source | [Noh-Gyoung Kang, Chihong Cho, Seung-Hoon Park, Eun Tae Won] Samsung Electronics Co. Ltd. 416, Maetan-3dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea [Jeong-Wook Kim, Joon-seong Kang and Seong-Cheol Kim] Seoul National University Seoul, Korea | Voice : +82-31-279-7325 Fax : +82-31-279-5130 E-mail: [gyoung.kang@samsung.com] |
| Re: | [Body Area Network (BAN) Channel Model document] |  |
| Abstract | [This is a contribution to make a channel for wearable WBAN. It provides how channel model should be developed for wireless body area network. |  |
| Purpose | [The purpose of this document is to provide the work of the channel modeling subcommittee and recommendations on how the channel model for BAN can be used. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

| Date | Revision No. |
| --- | --- |
| 11/11/2008 | 15-08-0781-00-0006 |

Table of Contents

# 1. Introduction………………….………………………………………………………………4

# 2. Channel Model CM3 for UWB band …..…………………………………………………4

# 3. Channel Model CM4 for UWB band   ……………………………………………...…..16

# 4. References ……….……....………………………………………………………………..29

# 1. Introduction

This document illustrates channel models for the IEEE802.15.6. The scenario for the channel models are based on the channel model subcommittee report [1]. The models shown in this document include CM3 (body surface to body surface) and CM4 (body surface to external). For both CM3 and CM4, we measure the UWB frequency bands (3.1-5.1GHz, 7.25-8.5GHz).

# 2. Channel Model CM3 for UWB Band

In this section, channel models for the CM3 (body surface to body surface) at UWB bands are presented.

2.1. Effect of body posture

Transmitter antenna is placed in left waist and Receiver antennas are located in several positions.  Measurements are carried out in an anechoic chamber and office environments. All values are in dB scale.

A. 3.1-5.1GHz

|  | Action | left ear | right ear | left wrist | right wrist | right waist | left ankle | right ankle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anechoic Chamber | Standing | 62.2 | 61.6 | 64.9 | 67.5 | 64.2 | 72.8 | 69.1 |
|  | Sitting | 71.9 | 65.7 | 55.6 | 69.9 | 65.9 | 76.3 | 73.9 |
| Office | Standing | 73.8 | 70.4 | 61.4 | 70.9 | 74.3 | 76.4 | 68.3 |
|  | Sitting | 62.3 | 72.1 | 65.6 | 76.3 | 74.7 | 79.8 | 75.7 |

B. 7.25-8.5GHz

|  | Action | left ear | right ear | left wrist | right wrist | right waist | left ankle | right ankle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anechoic Chamber | Standing | 81.7 | 83.9 | 69.3 | 63.9 | 76.1 | 76.8 | 77.1 |
|  | Sitting | 84.4 | 85.2 | 73.6 | 75.5 | 82.38 | 79.8 | 95.2 |
| Office | Standing | 75.5 | 75.6 | 80.6 | 66.4 | 71.9 | 74.4 | 75.8 |
|  | Sitting | 67.1 | 84.5 | 67.0 | 67.1 | 67.5 | 70.6 | 83.4 |

2.2. Effect of body movement

Transmitter antenna is placed in left waist and Receiver antennas are located in several positions.  An arm and a leg are moved in forward or side direction during measurements.

A. 3.1-5.1GHz

|  |  | Anechoic Chamber |  | Office |  |
| --- | --- | --- | --- | --- | --- |
|  |  | mean | stdev | mean | stdev |
| Left wrist | Forward direction | 65.7 | 4.3 | 71.7 | 5.8 |
|  | Side direction | 70.9 | 6.0 | 76.6 | 7.7 |
| Right wrist | Forward direction | 72.9 | 4.4 | 73.6 | 2.3 |
|  | Side direction | 71.8 | 5.9 | 75.1 | 3.1 |
| Left ankle | Forward direction | 74.6 | 2.5 | 76.4 | 0.1 |
|  | Side direction | 67.9 | 6.8 | 70.4 | 8.4 |
| Right ankle | Forward direction | 70.8 | 2.5 | 69.9 | 2.3 |
|  | Side direction | 72.8 | 5.3 | 69.6 | 1.9 |

B. 7.25-8.5GHz

|  |  | Anechoic Chamber |  | Office |  |
| --- | --- | --- | --- | --- | --- |
|  |  | mean | stdev | mean | stdev |
| Left wrist | Forward direction | 76.9 | 6.6 | 74.1 | 3.4 |
|  | Side direction | 77.4 | 10.1 | 78.1 | 4.7 |
| Right wrist | Forward direction | 79.7 | 7.1 | 73.0 | 7.1 |
|  | Side direction | 79.5 | 8.1 | 75.9 | 6.0 |
| Left ankle | Forward direction | 82.4 | 7.9 | 75.9 | 2.2 |
|  | Side direction | 77.4 | 0.8 | 75.7 | 1.9 |
| Right ankle | Forward direction | 85.4 | 11.7 | 77.5 | 2.5 |
|  | Side direction | 82.0 | 6.9 | 78.1 | 3.3 |

# 3. Channel Model CM4 for UWB Band

In this section, channel models for the CM4 (body surface to external) at UWB bands are presented.

3.1. Effect of receiver antenna height

The transmitter antenna is placed on wooden stand at fixed height of 1 m from ground and receiver antenna is placed on wooden stand and located at different height, from 0.25 to 1.5 m with step of 0.25 m. The distance between transmitter and receiver are 1 m and 3 m. The tables below summarize the results.

A. 3.1-5.1GHz

| Rx Ant. Height | 0.25m | 0.5m | 0.75m | 1.0m | 1.25m | 1.5m |
| --- | --- | --- | --- | --- | --- | --- |
| Distance 1m | 45.5 | 45.4 | 44.2 | 43.8 | 44.5 | 44.2 |
| Distance 3m | 54.5 | 53.7 | 53.0 | 52.9 | 53.7 | 53.7 |

B. 7.25-8.5GHz

| Rx Ant. Height | 0.25m | 0.5m | 0.75m | 1.0m | 1.25m | 1.5m |
| --- | --- | --- | --- | --- | --- | --- |
| Distance 1m | 53 | 52 | 53.4 | 52.2 | 52.2 | 52.6 |
| Distance 3m | 54.7 | 54.9 | 61.7 | 62.2 | 61.7 | 55.3 |

3.2. Effect of body posture

Transmitter antenna is placed at wooden mount 1m high and receivers are located on body which is 3m apart from transmitter. Measurements are performed to compare the effects of body posture (standing or sitting on the chair).

A. 3.1-5.1GHz

|  | Anechoic Chamber |  |  |  | Office |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Action | Right ear | Right wrist | Right waist | Right ankle | Right ear | Right wrist | Right waist | Right ankle |
| Standing | 66.9 | 63.3 | 56.1 | 59.4 | 62.8 | 61.9 | 55.1 | 60.7 |
| Sitting | 60.4 | 61.1 | 60.6 | 55.7 | 60.4 | 61.6 | 54.5 | 57.1 |

B. 7.25-8.5GHz

|  | Anechoic Chamber |  |  |  | Office |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Action | Right ear | Right wrist | Right waist | Right ankle | Right ear | Right wrist | Right waist | Right ankle |
| Standing | 69.3 | 63.7 | 70.4 | 69.5 | 72.4 | 72.1 | 75.7 | 68.1 |
| Sitting | 71.5 | 64.2 | 71.9 | 63.8 | 74.4 | 73.9 | 76.3 | 73.7 |

3.3. Effect of body movement

Transmitter antenna is placed at wooden mount 1m high and receivers are located on body which is 3m apart from transmitter. An arm and a leg are moved in forward or side direction during measurements.

A. 3.1-5.1GHz

|  |  | Anechoic Chamber |  | Office |  |
| --- | --- | --- | --- | --- | --- |
|  |  | mean | stdev | mean | stdev |
| Wrist | Forward direction | 58.2 | 4.2 | 60.7 | 3.9 |
|  | Side direction | 63.6 | 3.5 | 64.5 | 2.2 |
| Ankle | Forward direction | 64.0 | 6.5 | 61.4 | 1.1 |
|  | Side direction | 57.2 | 3.1 | 61.0 | 0.5 |

B. 7.25-8.5GHz

|  |  | Anechoic Chamber |  | Office |  |
| --- | --- | --- | --- | --- | --- |
|  |  | mean | stdev | mean | stdev |
| Wrist | Forward direction | 64.6 | 2.3 | 73.1 | 2.0 |
|  | Side direction | 64.0 | 1.8 | 73.9 | 2.5 |
| Ankle | Forward direction | 66.1 | 4.8 | 65.4 | 3.8 |
|  | Side direction | 66.0 | 4.8 | 69.1 | 1.4 |

# 4. References

- K. Y. Yazdandoost, et al., “Channel Model for Body Area Network (BAN),” IEEE 802.15-08-0033-04.