IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | LB47 various resolutions |  |
| Date Submitted | [November 12, 2008] |  |
| Source | [James P. K. Gilb] [SiBEAM] [555 N. Mathilda, Suite 100 Sunnyvale, CA 94085] | Voice: [858-229-4822] Fax: [ ] E-mail: last name at ieee dot org |
| Re: |  |  |
| Abstract | Various LB47 resolutions |  |
| Purpose | Resolve comments for LB47 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Resolution for CID 128

Replaces Figure 158 with this one:

# Resolution for Number of PHYs

CIDs: 8, 11, 117, 132, 143, 145, 148, 219

All but one express common request, merge OFDM PHYs (one requests that any one of the three PHY).  However, the group has considered this issue and sees value in the three PHYs.

Suggest Reject. Each of the PHYs in the 802.15.3c draft address unique market segments that are cost sensitive and application constrained.  The SC PHY is aimed at battery powered devices that are extremely low cost.  The HSI PHY is optimized for devices with low-latency, bi-directional high-speed data.  The AV PHY provides a cost effective solution for consumer electronic devices that predominantly connect with high-speed data in only one direction and require low latency.

CID 8

CID 11

CID 117

CID 132

CID 143

CID 145

CID 148

CID 219

# Optional sync frames for DEVs

Capability_IE (Section 7.4.11)

Add a new bit assignment in DEV capabilities field,  Bit 36:  Sync frame capable

Add Section 7.4.36 “Sync Frame Frequency IE”

The Sync Frame Frequency IE shall be formatted as illustrated in Figure 1.

| Octets: 1 | 1 | 1 |
| --- | --- | --- |
| Sync control | Length | Element ID |

Figure 1: Sync frame frequency information element format

The Sync Control field shall be formatted as illustrated in Figure 2.

| Bits: 2 | 1 | 5 |
| --- | --- | --- |
| Reserved | Sync frame direction | Sync frame TX frequency |

Figure 2: Sync control field format

The Sync Frame TX Frequency field is set to the number of superframes between transmission of two Sync frames, as defined in 7.3.6, requested by the PNC.  If the Sync Frame TX Frequency field is set to zero, then the PNC is requesting that the DEV cease sending Sync frames.

Sync Frame Direction field shall be set to zero if the PNC is requesting omni-directional transmission of the Sync frame and shall be set to one if the PNC is requesting directional transmission of the Sync frame.  If the request is for directional transmission, then the Sync frame direction is a round robin of the DEV's available directions.

Add new subclause 8.6.7 “Sync. Frame Transmission and Virtually Dependent Piconets”

Sync frame transmission is an optional function that mitigates co-channel interference due to a hidden PNC node. It also provides a method to obtain synchronization among independent piconets.  A DEV indicates if it is capable of sending Sync frames by setting the Sync Frame Capable bit in the Capabilities IE.

The PNC controls the transmission of Sync frames by DEVs in the piconet by sending a Sync Frame Frequency IE, as defined in 7.4.??, in a Announce command to a DEV that is capable of sending Sync frames.  The PNC sets the frequency with which the frames will be sent by setting the Sync Frame TX Frequency field to the appropriate value.

When the a DEV that is capable of sending Sync frames receives the Sync Frame Frequency IE from the PNC, it sends a Sync frame in the first transmission opportunity in the next superframe.  The DEV continues sending the Sync frame every Sync Frame TX Frequency superframes. DEVs may use response frame in an Imp-ACK exchange to send the Sync frame.

A DEV that receives a Sync frame from another piconet may utilize the information in the frame to obtain network synchronization and mitigate interference.  A PNC may use Sync frames that it receives from DEVs in another piconet to synchronize to PNC timing and forming a virtually dependent piconet. The PNC may then adjust the CTAs in its piconet to avoid interference with the other piconet.