IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Matlab code for generating BAN fading power profile |  |
| Date Submitted | 19 December, 2008 |  |
| Source | [Guido Dolmans] [Holst Centre / IMEC-NL] HTC 31, Eindhoven, the Netherlands | Voice: +31402774094 Fax: +31402746400 E-mail: guido.dolmans@imec-nl.nl |
| Re: |  |  |
| Abstract | Matlab code for generating channel model CM3 (S4 and S5) in the 900 MHz and 2.4 GHz band. This is model B in the channel model document IEEEP802.15-08-0780. The models are based on an extensive measurement campaign carried out at IMEC. |  |
| Purpose | To aid the generation of realistic BAN channels. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

% Written by Andrew Fort and Guido Dolmans, Dec. 18, 2008)

% This model is meant for body to body surface CM3 (Scenario S4 & S5) for

% 900 MHz

%

%

clear all;

figure;hold;

d = [0.1:0.01:0.5];

lambda(1)=0.6512;

lambda(2)=48.7212;

lambda(3)=1.1519e-6;

plot(d,-10*log10(lambda(1).*exp(-lambda(2).*d) + lambda(3)),'r');

title('Exponential Pathloss with maximum value');

xlabel('Distance [m]');

ylabel('Pathloss [dB]');

figure;hold;

x = [-65:-20];

PK1=0.6061;

PK2=40.1395;

plot(-x,(PK2 + PK1*x),'r')

axis([20 65 0 30]);

title('K versus Pathloss');

xlabel('Pathloss [dB]');

ylabel('K [dB]');

% plots of rms delay spread

figure; hold;

x = [0:0.1e-9:30e-9];

% plot results for 15 cm separation

stde=1.3742e-10;

meane=4.5366e-9;

plot(x*1e9,cdf('norm',x*1e9,meane*1e9,stde*1e9),'k--');

% plot results for 45 cm separation

stde=3.5986e-9;

meane=1.0066e-8;

plot(x*1e9,cdf('norm',x*1e9,meane*1e9,stde*1e9),'k-');

title('15 & 45 cm distance');

xlabel('Delay Spread [ns]');

ylabel('CDF');

% Written by Andrew Fort and Guido Dolmans, Dec. 18, 2008)

% This model is meant for body to body surface CM3 (Scenario S4 & S5) for

% 2.4 GHz

%

%

clear all

figure;hold;

d = [0.1:0.01:0.5];

%plot(dall,-10*log10(PL),'bo');

lambda(1)=0.0026;

lambda(2)=46.0931;

lambda(3)=7.4683e-8;

plot(d,-10*log10(lambda(1).*exp(-lambda(2).*d) + lambda(3)),'r');

title('Exponential Pathloss with maximum value');

xlabel('Distance [m]');

ylabel('Pathloss [dB]');

figure;hold;

x = [-65:-20];

PK1=0.43;

PK2=30.6;

plot(-x,(PK2 + PK1*x),'r')

axis([20 65 0 30]);

title('K versus Pathloss');

xlabel('Pathloss [dB]');

ylabel('K [dB]');

% plots of rms delay spread

figure; hold;

x = [0:0.1e-9:30e-9];

% plot results for 15 cm separation

me=8.3577e-9;

ste=3.3232e-9;

plot(x*1e9,cdf('norm',x*1e9,me*1e9,ste*1e9),'k--');

title('15 cm & 45 cm separation')

% plot results for 45 cm separation

me=1.7158e-8;

ste=4.849e-9;

plot(x*1e9,cdf('norm',x*1e9,me*1e9,ste*1e9),'k-');

xlabel('Delay Spread [ns]');

ylabel('CDF');