IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Operating band (band/channel plan) |  |
| Date Submitted | 30 July 2026 |  |
| Source | [Benjamin A. Rolfe] [Blind Creek Associates] | Voice: [ +1.408.395.7732 ] Fax: [ ] E-mail: [ ben@blindcreek.com ] |
| Re: | Task Group 15.4g |  |
| Abstract | TG4g work in progress: technical guidance. (Work in Progress) |  |
| Purpose | Discussion within task group |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

802.15.4g PHY System Parameters

| PHY Parameter |  | Performance Critera, Constraints, Comments | Regional Regulatory |
| --- | --- | --- | --- |
| Operating band (band/channel plan) | - Channel change time min/max - Channel bandwidth - Channel spacing - Number of channels | License exempt class license Specified use (dedicated, application constrained) license exempt | Band availability varies greatly. Proposer should specify what regions they intend to target. |
| Modulation and Coding Scheme(s) | - Modulation method - Modulation accuracy | Suitable for low cost implementation Meet regulator domain |  |
| PHY frame structure | - Pre-amble - Sync Header, SFD len - codes and/or patterns - as appropriate to proposal |  |  |
| Synchronization and Timing | - might come from specific sync mechanisms or may be dependent on other PHY features |  |  |
| Data rate(s) |  | 40kbps to 1000 kbps in the PAR Specify intended supported rate(s) |  |
| Symbol / chip rate | - As appropriate to the proposed technique |  |  |
| Transmit Power | - MAX - MIN - Peak to Average - Management |  | Very definitely regionally dependent |
| PSD | - In band - Out of band |  |  |
| Chan availability (interference detection) | - Spectrum scanning - CCA |  |  |
| Link Quality | - Technique used - Frequency of assessment - Accuracy and resolution - Bi-directional (cooperative)? |  |  |
| Reliability enhancing features/methods | - Detection (check sequence) - Prevention (FEC) - Interference mitigation/avoidance - Collision avoidance |  |  |
| EVM |  |  |  |
| MAC dependencies |  |  |  |
| Co-existence features | - Adjacent channel rejection |  |  |

# PHY Parameter Dependency Matrix

| PHY Parameter |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Freq Band | Chan BW | # Chans | Mod | TX power | PSD | Data Rate | Range | D. Cycle | Coex-F |  |
| Freq Band |  | x | x | x | x | x |  | x | x | x |  |
| Chan. Bandwidth |  |  | x | x | x | x | x | x |  |  |  |
| Number Chan.s |  | x |  |  | x | x |  |  |  |  |  |
| Modulation |  | x |  | - |  | x | x | x |  |  |  |
| TX power |  |  |  |  | - |  |  |  | x | x |  |
| PSD |  |  |  |  | x | - |  |  | x | x |  |
| Data Rate |  |  |  |  |  |  | - |  | x | x |  |
| Range |  |  |  |  |  |  |  | - |  | x |  |
| Duty cycle |  |  |  |  |  |  |  |  | - | x |  |
| Co-ex footprint |  |  |  |  |  |  |  |  |  | - |  |
|  |  |  |  |  |  |  |  |  |  |  | - |

# Application/Performance Matrix

| Performance Parameter |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Latency | Data rate | Range | CexF | Err. Prob |  |  |  |  |  |  |
| Applications |  |  |  |  |  |  |  |  |  |  |  |
| Distribution Auto |  |  |  |  |  |  |  |  |  |  |  |
| Demand side Mgnt |  |  |  |  |  |  |  |  |  |  |  |
| - Bi-direct. |  |  |  |  |  |  |  |  |  |  |  |
| - Not |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |

Categories of applications

- Non-real time applications

- Real-time applications

- Sync/async

- Bi-directional or not

Definitions

Term: Definition