IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG4g PHY Technical Guidance Document (Work in Progress) |  |
| Date Submitted | 30 July 2026 |  |
| Source | [Benjamin A. Rolfe] [ Blind Creek] | Voice: [ +1.408.395.7732 ] Fax: [ ] E-mail: [ ben @ blindcreek.com ] |
| Re: | Task Group 15.4g |  |
| Abstract | TG4g work in progress: technical guidance for PHY proposals. (Work in Progress) |  |
| Purpose | Discussion within task group |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

802.15.4g PHY System Parameters

# Introduction

## Purpose

This document provides technical guidance for preparation of PHY proposals for task group 15.4g. The purpose of this document is to provide a guideline to develop technical proposals that satisfy the W-SUN application requirements, and provide a framework for evaluating proposals.  This document outlines the key characteristics that should be specified as a set of PHY layer parameters, and provides some performance criteria and constraints for parameter set.

The intent of the task group is to use a flexible and efficient process which provides sufficient descriptions of the technical requirements to enable relevant responses, with efficiency of effort while meeting the critical need for a timely standard.

## Methodology

The methodology provides a consensus approach to defining a minimal subset of features, characteristics, performance and constraints.  This document will provide

- A functional view of the PHY characteristics, in the form of specific parameters which define externally verifiable performance and interoperability characteristics;

- A dependency matrix, which considers the interdependency of tradeoffs which are made in the selection of PHY features;

- Application/performance matrix which characterizes by classes of SUN applications the priority of performance characteristics;

The parameters table provides guidance on developing complete technical proposals. This represents a subset of parameters, and the absence of a parameter should not be seen as a constraint. The parameter column consists of two sub-columns. The first identifies the parameter, which should be addressed in the proposal; the second provides some examples of how this may be addressed in a proposal; there may be alternatives appropriate to specifying the characteristic.  The performance criteria column includes constraints (derived form the PAR) or explanations.  The regulator column is intended to identify where regional differences in regulations (present and anticipated) may affect the PHY characteristics.

## High Level Requirements

The PAR establishes general requirements for the amendment in the “scope”, which can be summarized as follows:

- Alternate PHY amendment

  - MAC additions only where needed to support PHY

- License exempt frequency bands

  - No bands listed explicitly

- Low data rate: 40 kbps to 1000 kbps

- Optimal energy efficient link margin given the SUN environmental conditions

  - Principally outdoors, but also in basements, around corners…

- Supports a PHY payload (MPDU) of 1500 octets or more.

- Potentially dense deployments: at least 1000 direct neighbors

- Simultaneous operation for at least 3 co-located orthogonal networks

- Good coexistence mechanisms (both ways)

The PAR and 5C documents can be found on the 802.15 document page (https://mentor.ieee.org/802.15/documents):

15-08-0705-05-0nan-wnan-par.doc

15-08-0706-02-0nan-wnan-5c.doc

# PHY Parameters

| PHY Parameter |  | Performance Criteria, Constraints, Comments | Regional Regulatory |
| --- | --- | --- | --- |
| Parameter: | Example: |  |  |
| Operating band (band/channel plan) | - Channel change time min/max - Channel bandwidth - Channel spacing - Number of channels | License exempt class license Specified use (dedicated, application constrained) license exempt | Band availability varies greatly. Proposer should specify what regions they intend to target. |
| Modulation and Coding Scheme(s) | - Modulation method - Modulation accuracy (e.g. EVM) - Whitening - Chipping | Suitable for low cost implementation Meet regulator domain |  |
| PHY frame structure | - Pre-amble - Sync Header, SFD length - codes and/or patterns - as appropriate to proposal |  |  |
| Synchronization and Timing | - might come from specific sync mechanisms or may be dependent on other PHY features - clock accuracy / stability required |  |  |
| Data rate(s) |  | 40kbps to 1000 kbps in the PAR Specify intended supported rate(s) |  |
| Symbol / chip rate | - As appropriate to the proposed technique |  |  |
| Transmit Power | - MAX - MIN - Peak to Average - Management |  | Very definitely regionally dependent |
| PSD | - In band - Out of band |  |  |
| Receiver Sensitivity | - dBm | Specify conditions, for example: -85dBm PSDU length = 20 octets. PER < 1%. Power measured at antenna terminals. Interference not present. (see section 6.1.7 of 15.4-2006 for example) |  |
| Chan availability (interference detection) | - Spectrum scanning - CCA |  |  |
| Link Quality Indication | - Technique used - Frequency of assessment - Accuracy and resolution - Bi-directional (cooperative)? |  |  |
| Reliability enhancing features/methods | - Detection (check sequence) - Prevention (FEC) - Interference mitigation/avoidance - Collision avoidance |  |  |
| Co-existence features | - Adjacent channel rejection - Channel sensing, scanning, assessment - Coordination mechanisms for different radios in close proximity | (Key parameters which impact coexistence should be described) | Latest copy of doc # TBD is good guidance on coexistence scenarios. |
| MAC dependencies / support required | - Channel sequence control - Error control - Fragmentation support - Channel access - Scanning | (MAC features required to support the PHY) | Identify two categories: “Must have” MAC features “nice to have” MAC features |

| MAC Parameter (Additional MAC features within the scope of the PAR) |  | Performance Criteria, Constraints, Comments | Regional Regulatory |
| --- | --- | --- | --- |
| Duty cycle support |  |  |  |
| Channel dwell time |  |  |  |
| Channel access |  |  |  |
| Sequence control |  |  |  |
| … |  |  |  |

# PHY Parameter Dependency Matrix

| PHY Parameter |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Freq Band | Channel Bandwidth | Number of Channels | Modulation | TX power | PSD | Data Rate | Range | Duty Cycle | Co-ex footprint |  |
| Frequency Band | - | x | x | x | x | x |  | x | x | x |  |
| Channel Bandwidth |  | - | x | x | x | x | x | x |  | x |  |
| Number of Channels |  |  | - |  | x | x |  |  |  | x |  |
| Modulation |  |  |  | - |  | x | x | x |  |  |  |
| TX power |  |  |  |  | - | x |  |  | x | x |  |
| PSD |  |  |  |  |  | - |  |  | x | x |  |
| Data Rate |  |  |  |  |  |  | - |  | x | x |  |
| Range |  |  |  |  |  |  |  | - |  | x |  |
| Duty cycle |  |  |  |  |  |  |  |  | - | x |  |
| Co-ex footprint |  |  |  |  |  |  |  |  |  | - |  |
|  |  |  |  |  |  |  |  |  |  |  | - |

# Performance

This section can identify performance considerations, constraints and requirements.

## Channel Characteristics

The channel characteristics sub-group was presented measurement data from several real-world deployments which demonstrated a wide range of actual channel conditions.  The conclusions of that group are summarized in 802.15-09-0279-00-004g.

Regarding Multipath Mitigation, the subgroup recommends that the TG4g proposers state the following:

- For proposed PHY in widespread deployment, state how SUN multipath has been mitigated,

- Otherwise, state planned SUN multipath mitigation techniques.

## Application/Performance Matrix

| Applications | Performance Criteria Categories |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Latency/Reliability |  | Throughput (msg/day) |  | Density |  | Range |  | Cost |  | Co-existence |  |  |  |
|  | % success within 10s | % success within 2s | (msg/day) | Avg msg Size | Nodes per m2 | Adjacen-cies per node | Link Distance (range) |  | Cap Ex | Op Ex | Interference, like and non-like systems |  |  |  |
|  |  |  |  |  |  |  | Min | Max |  |  | V | A |  |  |
| Metering |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Device-to-Device (Mesh/Relay) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUN to HAN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dist. Automation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Demand side Mgnt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bi-direct. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uni-direct. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RTP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Mon/Control |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Env. Monitoring |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Monitoring |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lighting control |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Parking monitors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Waste Mgmt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Application types | Characteristic categories of applications | Performance Criteria categories |
| --- | --- | --- |
| Utility - Metering - House to house (mesh/relay) - SUN to HAN gateway - Distribution automation (throughout SUN) - Demand side management (bi-direction, uni-directional) - Real Time Pricing Other machine to machine - Environment monitoring - Other kinds of urban monitoring - Public lighting control - Car parking - Waste management | - Non-real time applications - Real-time applications - Sync/async - Bi-directional or not | - Latency (one way, two way) - Synchronization - Data rate - Radio range - Node (radio) density - Overlapping networks (SOP) - Payload packet size (fragment size) - Reliability (Error detection, error probability) - Low cost - Coexistence (victim, assailant) - Flexibility - Adaptability to RF environment |

# Frequency Bands to Be Considered

[include table based on the preliminary proposals and the regulatory information presented]

# Definitions

The following provides definition of specific terms in the context of discussion with respect to TG4g applications and PHY proposals.

| Term: | Definition |
| --- | --- |