IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG7 Technical Requirements Document (TRD) |  |
| Date Submitted | [4 May, 2009] |  |
| Source | [Sridhar Rajagopal, Samsung Electronics] (Editor) | E-mail: [sridhar.r@samsung.com] |
| Re: | [Visible Light Communications (VLC) Technical Requirements document] |  |
| Abstract | [VLC Technical Requirements] |  |
| Purpose | [This working document has been prepared to be a VLC Technical Requirements document] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

| List of contributors |  |
| --- | --- |
| Sridhar Rajagopal | Samsung Electronics |
| Dominic O’Brien | University of Oxford |
| Jae-seung Son | Samsung Electronics |
| Rick Roberts | Intel |
| Dae-Ho Kim | ETRI |
| Tae-Gyu Kang | ETRI |
| Sang-Kyu Lim | ETRI |
| Taehan Bae | Samsung Electronics |
| Larry Taylor | Discrete Time Communications |
| Soo-Young Chang |  |
| Eunhye Nam |  |
| Don Schultz |  |
| Harry Tan |  |

Table of Contents

1.	Definitions:	3

2.	Introduction	6

3.	VLC Technical Characteristics Summary	6

3.1.	High level description	6

3.2.	Overall requirements	6

4.	Channel model assumptions	7

5.	Topology	7

6.	Device characteristics of light sources and receivers	7

7.	Frequency usage	7

8.	Data Rates	7

9.	Transmission range	7

10.	Security	7

11.	Quality of Service (QoS)	7

12.	Power Consumption	8

13.	Coexistence with ambient light and other optical technologies	8

14.	Form Factor	8

15.	Complexity	8

16.	Mobility	8

17.	Regulatory requirements	8

18.	References	8

# Definitions:

| VLC | Visible Light Communication |
| --- | --- |
| TRD | Technical Requirement Document |
| LOS | Line of Sight |
| NLOS | Non Line of Sight |
| FOV | Field of View |
| LED | Light Emitting Diode |

General

This technical requirement document (TRD) describes the technical aspects that TG7 standard must fulfill, such as performance-related issues, reliability issues and availability issues. These types of requirements are often called quality of service (QoS) requirements; other requirements are usually maintenance-level requirements or external constraints, sometimes called compliance. Technical requirements are summarized as any other specifications; they have a name and a unique identifier. Technical requirements are documented in the same manner as any specifications, including a description, an example, a source or references to related technical requirements and a revision history.

TG7 needs to effectively define and manage requirements to ensure they are meeting needs of the VLC users, while proving compliance.

Ideally, requirements are:

• Correct (technically and legally possible)

• Complete (express a whole idea or statement)

• Clear (unambiguous and not confusing)

• Consistent (not in conflict with other requirements)

• Verifiable (it can be determined that the system meets the requirement)

• Traceable (uniquely identified and trackable)

• Feasible (can be accomplished within cost and schedule)

• Modular (can be changed without excessive impact)

• Design-independent (does not pose a specific solution on design)

Each requirement must first form a complete sentence, containing a subject and a predicate. These sentences must consistently use the verb “shall”, “will” or “must” to show the requirement's mandatory nature, and “should” or “may” to show that the requirement is optional. The whole requirement specifies a desired end goal or result and contains a success criterion or other measurable indication of the quality.

TRD needs to capture these levels of user requirements, maintaining intelligent traceability and change impact analysis between them.

Typical constraint requirements can specify:

• Performance

• Interfaces

• Security

• Safety

• Reliability

• Availability

• Maintainability

An efficient way of writing better requirements is to ensure they are clearly mapped to test cases. Making sure each requirement is clearly verifiable from the start, not only helps prepare later phases of the project, it also puts the developer in the correct state of mind. Requirements and their associated tests must also indicate what the system should not do, and what happens at the limits (degraded mode).

This rule also applies for compliance requirements: indicating how they shall be tested is a good way to write better requirements.

TRD need to implement a reliable and repeatable change control process that helps turn this challenge into an opportunity.

By providing examples and counter-examples of good requirements and documents, IEEE can enhance the quality, consistency, and completeness of the requirements. These can originally be templates, industry standards and rules inside a repository, such as the IEEE server.

Requirement typical sentence construction

Defects to avoid:

- Vagueness

- Weakness

- Over specification

- Subjectivity

- Multiplicity

- Unclear meaning

- Implicit meaning

Some words to be used with caution:

“adequate”, “applicable”, “appropriate”, “approximate”, “bad”, “best practice”, “between”, “clearly”, “compatible”, “completely”, “consider”, “could”, “down to”, “easy/easily”, “effective”, “efficient”, “equivalent”, “excellent”, “good”, “his/her”, “however”, “ideal”, “etc”, “in order to”, “include but shall not be limited to”, “least”, “like”, “low”, “maximise”, “may”, “most”, “minimum/mal”, “must”, “nearly”, “necessary”, “needed”, “normal”, “or”, “possible/bly”, “practicable”, “provide”, “quality”, “readily”, “relevant”, “safe/ly“, “same”, “should”, “significant”, “similar”, “so as”, “subject to”, “substantial”, “sufficient”, “suitable”, “support”, “target”, “typical”, “up to”, “user friendly”, “whether”, “will”, “with”, “worse”.

# Introduction

This document provides the technical contents of the project to develop PHY and MAC protocols for Visible Light Communications. This document will provide guidance on how to respond to a call for proposals. As for any communication protocol, the reference model used for this standard is the following:

PHY_SAP: PHY Service Access Point

PLCP: PHY Layer Convergence Protocol

MAC_SAP: MAC Service Access Point

PMD: Physical Medium Dependent (radio)

Figure 1, Reference partitioning

This document serves two purposes.  First, it summarizes the applications presented in response to VLC Study group and TG7 Call for Applications. Second, it describes and defines the fundamental requirements implied by applications but not necessarily stated explicitly.

# VLC Technical Characteristics Summary

This standard defines a PHY and MAC for short-range optical wireless communications using visible light in optically transparent media. The visible light spectrum extends from 380 to 780 nm in wavelength. The standard is capable of delivering data rates sufficient to support audio and video multimedia services and also considers mobility of the visible link, compatibility with visible-light infrastructures, impairments due to noise and interference from sources like ambient light; and a MAC layer that accommodates visible links. The standard adheres to any applicable eye safety regulations.

# High level description

Three device classes are considered: infrastructure, mobile and vehicle-mounted. All classes have both uni- and bi-directional data, with point-to-point and point-to-multipoint connectivity

|  | Infrastructure | Mobile | Vehicle-mounted |
| --- | --- | --- | --- |
| Internet Connectivity | Yes | No | No |
| Power supply | Ample | Limited | Ample |
| Form factor | Unconstrained | Constrained | Unconstrained |
| Light source | Intense | weak | Intense |
| Physical Mobility | No | Yes | Yes |
| Range | Short/long | Short | Long |
| Data rates | High/Low | High | Low |

Short range is defined as a device with expected operational range <= 3 m.

# Overall requirements

# Channel model assumptions

A typical RF channel allows allow link loss and bandwidth to be estimated for a generic system configuration. Variation in the specific geometry and the effect of fading is usually dealt with using a statistical distribution within the model. Extensive data collection has led to a series of models that allow verification that a system will operate under most reasonably foreseeable conditions. Table 1 shows the modeling parameters that might be required in the case of VLC.

| Parameter | Requirement | Comment |
| --- | --- | --- |
| Loss parameters |  |  |
| Loss | Required | Value for a particular configuration within a scenario |
| Max/Min | Required | Allows receiver dynamic range to be specified for a particular scenario |
| Statistics | Desirable | Useful for variable data rate schemes |
| Bandwidth parameters |  |  |
| Bandwidth | Required | Value for a particular configuration within a scenario |
| Max/Min | Required | Min is required to ensure sufficient bandwidth is available. Max useful for variable data rate schemes |
| Statistics | Desirable | Useful for variable data rate schemes |

Table 1. Channel modeling parameters

The levels of use of a model might be summarised as

- Verification. The model indicates that the maximum Loss, and dynamic range, and the minimum bandwidth are sufficient for the proposed application and scenario, so no further detailed modelling is required. This is likely to be appropriate when

  - Simple fixed configuration systems are used, and there is no reason to know the channel dynamics (as would be the case for adaptive transmitters, receivers and modulation.).

  - Other components constrain the system rather than maximum loss and minimum bandwidth, such as is the case for information broadcasting using VLC (where the LED provides the bandwidth constraint).

- Full Use. All parameters are required to estimate the performance of the system, and to check that any adaptation of the system operates properly.

# Types of channels

The type of VLC channel being used determines the level of detail of channel model that is required to adequately describe the channel, and create the parameters detailed in Table 1.

The paths to be considered are

- LOS-both single and multiple LOS paths [Main/primary focus for majority of applications]

- NLOS-both single and multiple NLOS paths

The strength of the dominant path relative to other paths is a key factor in determining the behaviour of the channel. If the dominant path is much stronger than the others then the channel is LOS-like, but if there are a number of paths of different lengths and approximately equal strength then there may be bandwidth limitations.

A Dominant Path Ratio (K) can be defined as

For the NLOS case this becomes

where  is the power in a particular NLOS path from transmitter to receiver.

For the LOS case .

For the mixed case, where multiple (or single) NLOS and/or LOS paths may exist then

Where is the power in a particular LOS path. (It should be noted that in the mixed case this is a slightly speculative definition, as it is uncertain as to whether this gives a sensible K value for all of the mixed cases).

The value of K at which only the dominant path can be considered is denoted as , which is estimated to be 13dB (when measured in the electrical domain) in [1].  If K is below this value then all paths need to be considered (or at least a subset of the strongest ones) and the interaction of the time delays of these paths will effect the bandwidth. In the table there is also a lower threshold, , where all the paths have almost the same strength and the channel is ‘tending’ towards a fully diffuse channel. Below this threshold it may be possible to treat the channel as fully diffuse, which may simplify the modelling process. Investigation is required to see whether this is the case.

A summary of initial observations concerning the different channel types is shown in Table 2.

| Channel type | NLOS |  | Mixed (LOS/NLOS) |  |  | Single LOS | Multiple LOS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dominant path ratio |  |  |  |  |  |  |  |  |
| Modelling considerations Simplifying assumptions | Consider all paths | Consider dominant NLOS path only | Consider NLOS paths only | Mixed-consider all paths | Consider dominant path only | Consider single path | Consider multiple LOS | Consider dominant path only |
| Modelling Techniques available (not exhaustive) | Ray-tracing, Integrating sphere[1] for NLOS | Ray-tracing, | Ray-tracing, Integrating sphere for NLOS | Geometric for LOS+Ray-tracing, Integrating sphere for NLOS | Geometric | Geometric | Geometric + time considerations | Geometric |
| Typical situation | Diffuse channel | Strong illumination of intermediate surface-'secondary source' | Shadowing of dominant LOS paths, but no strong NLOS. | No highly dominant path (either LOS or NLOS) but some stronger paths (LOS or NLOS) | Room with source(s) on ceiling provides strong LOS path(s) compared with any NLOS (diffuse) component from surfaces | Single source with receiver in coverage area. No reflections from other surfaces. | Multiple strong sources each with LOS to receiver | Receiver predominantly in coverage area of a single source |
| Exemplar VLC application |  |  | Information broadcasting using white LED illumination |  |  | Visible point to point link between PDAs | Outdoor transmission from traffic signal or signboard. |  |
| Comment | Diffuse channel when |  | LOS and NLOS channels likely to exist in indoor 'illumination' situations. Likely that path strength is LOS>>strong NLOS, and in the case of multiple LOS paths any LOS is stronger than the strongest NLOS. |  |  |  | Often the multiple LOS can be aggregated to a single LOS as the time difference between propagation does not create significant bandwidth delay |  |
| Bandwidth | Low | High | Low | Variable | High | Very High | High/Very High | Very High |
| Path Loss | High | High | High | Situation dependent | Situation dependent | Acceptable | Low | Low |

Table 2. Summary of channel type

# Topology

Both uni-direction and bi-directional communication must be supported. If bi-directional communication is possible, uni-directional case can be covered by the MAC layer. Many applications also show broadcasting requirements. Communication data flow could be symmetric or asymmetric. Both indoor and outdoor applications should be considered. Most applications have a primarily LOS connection. Most applications also show point-to-point or point-to-multipoint connectivity.

# Device characteristics of light sources and receivers

The TRD defines the physical characteristic requirements from light sources and receivers. Although variety of light sources and receivers may need to be supported for different applications, certain types of light sources or receivers may be excluded from being considered in the specification for performance or interference reasons.

| Specification | Minimum value | Maximum value |
| --- | --- | --- |
| Intensity of light source |  |  |
| FOV |  |  |
| Pulse duration |  |  |
| Detector responsivity |  |  |

# Frequency usage

The PAR has specified that the communication  for visible light use the frequency band between 380 – 780 nm. It may be expected that support for primary colors (Red, Green, Blue) and  White for infrastructure may be desired at a minimum. There needs to be allowance for manufacturer variations and device characteristics in defining the color spectrum range and spectral bandwidth for operation.

# Data Rates

The PAR states that the data rates should be sufficient to support audio and video multimedia services. The following table provides the maximum data rate available from different types of light sources.

| Light source | Maximum data rate for light source |
| --- | --- |
| Phosphor LED | 10 Mbps |
| Phosphorless LED | 100 Mbps |
| Resonant Cavity LED | 500 Mbps |
| Laser Diode | 1 Gbps |

However, there is a concern that the driver and corresponding circuitry may not be able to support these data rates. There is also a concern that the link margin in certain applications may not be sufficient in order to provide such high data rates. These concerns must be addressed in order to specify the data rate ranges.

# Transmission range

While the PAR specifies a short range communication standard, it is necessary to define the expected communication distances for the intensity allowed by the device type.  A reliability metric such as PER or BER in the range of 10-2/10-8 must also be specified in order to define the range metric accurately. The packet sizes needed must also be mentioned to support proposal development.

| Device Type | Min Range | Max Range |
| --- | --- | --- |
| Infrastructure |  |  |
| Mobile |  |  |
| Vehicle-mounted |  |  |

# Security

Need contributions

# Quality of Service (QoS)

Need contributions.

# Power Consumption

Power consumption may not an issue for devices belonging to the infrastructure class but may be important for mobile devices. Need contributions.

# Coexistence with ambient light and other optical technologies

VLC will need to co-exist with ambient lighting and other optical technologies. The tolerance levels to a few key impairments must be specified in order to ensure the technology will be robust in the presence of interference. The tolerance level should be specified for the highest data rate with the interference within the FOV of the receiver.

| Interference | Minimum Tolerance level (lux) |
| --- | --- |
| Outdoor (sunlight) |  |
| Indoor (incandescent, fluorescent lighting) |  |

# Form Factor

Need contributions

# Complexity

Need contributions

# Mobility and Link switching

There may be a need to support link switching due to mobility or interference. Limited MAC signaling and support may be required in order to support link switching. However, service continuity mechanisms could be considered out of scope of the standard. Issues such as link detection and switching time may need to be considered in this regard.

# Inherency

Brightness and color of illumination source must be maintained during communication for infrastructure devices.

# Dimming

The infrastructure link should support dimming, albeit possibly at reduced performance levels. It is possible that popular LED dimming options such as PWM could have an impact on communication.

# Regulatory requirements

Devices shall comply with the regulatory requirements specified for visible light communications. Requirements include eye safety regulations such as IEC 60825. Need to add reference to the regulatory document being developed.

# References

- VLC PAR

- TG6 Technical Requirements Document

- VLC Applications Summary Document

- VLC regulatory document

- List other contributions used that are not explicit in the above documents