IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG7 Technical Considerations Document (TCD) |  |
| Date Submitted | [25 June, 2009] |  |
| Source | [Sridhar Rajagopal, Samsung Electronics] (Editor) | E-mail: [sridhar.r@samsung.com] |
| Re: | [Visible Light Communications (VLC) Technical Considerations document] |  |
| Abstract | [VLC Technical Considerations] |  |
| Purpose | [This working document has been prepared to be a VLC Technical Considerations document] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

| List of contributors |  |
| --- | --- |
| Dae-Ho Kim | ETRI |
| Dominic O’Brien | University of Oxford |
| Don Schultz | Boeing |
| Douglas Castor | Interdigital |
| Eun Tae Won | Samsung Electronics |
| Eunhye Nam | ETRI |
| Farooq Khan | Samsung Electronics |
| Harry Tan | Aerospace Corporation |
| Jae-seung Son | Samsung Electronics |
| Larry Taylor | Discrete Time Communications |
| Praveen Gopalakrishnan | Intel |
| Rick Roberts | Intel |
| Sang-Kyu Lim | ETRI |
| Soo-Young Chang | CSUS |
| Sridhar Rajagopal | Samsung Electronics |
| Tae-Gyu Kang | ETRI |
| Taehan Bae | Samsung Electronics |
| Tom Matsumura | VLCC |
| Ying Li | Samsung Electronics |

Table of Contents

1.	Definitions:	4

2.	Introduction	7

3.	VLC Technical Characteristics Summary	7

3.1	High level description	8

4.	Channel model assumptions	8

4.1	Types of channels	9

5.	Topology	12

6.	Device characteristics of light sources and receivers	12

7.	Frequency usage	13

8.	Data Rates	13

9.	Transmission range	13

10.	Security	14

11.	Quality of Service (QoS)	14

12.	Power Consumption	14

13.	Coexistence with ambient light and other optical technologies	14

14.	Form Factor	15

15.	Complexity	15

16.	Mobility and Link switching	15

17.	Lighting Inherency	15

18.	Lighting Dimming	16

19.	Visibility support	16

20.	Regulatory requirements	16

21.	References	16

# Definitions:

| APD | Avalanche Photo Diode |
| --- | --- |
| FOV | Field of View |
| LD | Laser Diode |
| LED | Light Emitting Diode |
| LOS | Line of Sight |
| NLOS | Non Line of Sight |
| PIN | Diode with high resistance I layer between P and N layers |
| PWM | Pulse Width Modulation |
| TRD | Technical Requirement Document |
| VLC | Visible Light Communication |
| P2P | Point to Point |
| P2MP | Point to Multi Point |

General [2]

This technical considerations document (TCD) describes the technical aspects that TG7 standard must fulfill, such as performance-related issues, reliability issues and availability issues. These types of requirements are often called quality of service (QoS) requirements; other requirements are usually maintenance-level requirements or external constraints, sometimes called compliance. Technical requirements are summarized as any other specifications; they have a name and a unique identifier. Technical requirements are documented in the same manner as any specifications, including a description, an example, a source or references to related technical requirements and a revision history.

TG7 needs to effectively define and manage requirements to ensure they are meeting needs of the VLC users, while proving compliance.

Ideally, considerations are:

• Correct (technically and legally possible)

• Complete (express a whole idea or statement)

• Clear (unambiguous and not confusing)

• Consistent (not in conflict with other requirements)

• Verifiable (it can be determined that the system meets the requirement)

• Traceable (uniquely identified and trackable)

• Feasible (can be accomplished within cost and schedule)

• Modular (can be changed without excessive impact)

• Design-independent (does not pose a specific solution on design)

Each consideration must first form a complete sentence, containing a subject and a predicate. These sentences must consistently use the verb “shall”, “will” or “must” to show the requirement's mandatory nature, and “should” or “may” to show that the requirement is optional. The whole requirement specifies a desired end goal or result and contains a success criterion or other measurable indication of the quality.

TCD needs to capture these levels of user requirements, maintaining intelligent traceability and change impact analysis between them.

Typical constraint considerations can specify:

• Performance

• Interfaces

• Security

• Safety

• Reliability

• Availability

• Maintainability

An efficient way of writing better requirements is to ensure they are clearly mapped to test cases. Making sure each requirement is clearly verifiable from the start, not only helps prepare later phases of the project, it also puts the developer in the correct state of mind. Requirements and their associated tests must also indicate what the system should not do, and what happens at the limits (degraded mode).

This rule also applies for compliance requirements: indicating how they shall be tested is a good way to write better requirements.

TCD need to implement a reliable and repeatable change control process that helps turn this challenge into an opportunity.

By providing examples and counter-examples of good requirements and documents, IEEE can enhance the quality, consistency, and completeness of the requirements. These can originally be templates, industry standards and rules inside a repository, such as the IEEE server.

Requirement typical sentence construction

Defects to avoid:

- Vagueness

- Weakness

- Over specification

- Subjectivity

- Multiplicity

- Unclear meaning

- Implicit meaning

Some words to be used with caution:

“adequate”, “applicable”, “appropriate”, “approximate”, “bad”, “best practice”, “between”, “clearly”, “compatible”, “completely”, “consider”, “could”, “down to”, “easy/easily”, “effective”, “efficient”, “equivalent”, “excellent”, “good”, “his/her”, “however”, “ideal”, “etc”, “in order to”, “include but shall not be limited to”, “least”, “like”, “low”, “maximise”, “may”, “most”, “minimum/mal”, “must”, “nearly”, “necessary”, “needed”, “normal”, “or”, “possible/bly”, “practicable”, “provide”, “quality”, “readily”, “relevant”, “safe/ly“, “same”, “should”, “significant”, “similar”, “so as”, “subject to”, “substantial”, “sufficient”, “suitable”, “support”, “target”, “typical”, “up to”, “user friendly”, “whether”, “will”, “with”, “worse”.

Difference between Considerations and Requirements (TCD vs. TRD):

The group decided to use the term “considerations” instead of “requirements” in order to adopt a less rigid and formal process with the intention to be able to develop the standard quickly. This document serves to provide guidance to developing technical proposals for the IEEE 802.15.7 standard. The contents of the document are expected to be exactly the same as that expected from a technical requirements document.

# Introduction

This document provides the technical contents of the project to develop PHY and MAC protocols for Visible Light Communications. This document will provide guidance on how to respond to a call for proposals. As for any communication protocol, the reference model used for this standard is the following:

PHY_SAP: PHY Service Access Point

PLCP: PHY Layer Convergence Protocol

MAC_SAP: MAC Service Access Point

PMD: Physical Medium Dependent (radio)

Figure 1 Reference partitioning

This document serves two purposes:

- It summarizes the applications presented in response to VLC Study Group and TG7 Call for Applications.

- It describes and defines the fundamental requirements implied by applications but not necessarily stated explicitly.

# VLC Technical Characteristics Summary

This standard defines a PHY and MAC for short-range optical wireless communications using visible light in optically transparent media. The visible light spectrum extends from 380 to 780 nm in wavelength. The standard is capable of delivering data rates sufficient to support audio and video multimedia services and also considers mobility of the visible link, compatibility with visible-light infrastructures, impairments due to noise and interference from sources like ambient light; and a MAC layer that accommodates visible links. The standard adheres to any applicable eye safety regulations.

## High level description

Three device classes are considered: infrastructure, mobile and vehicle-mounted. All classes have both uni- and bi-directional data, with point-to-point and point-to-multipoint connectivity [3, 11]

Table 1 Device Classification

|  | Infrastructure | Mobile | Vehicle-mounted |
| --- | --- | --- | --- |
| Internet Connectivity | Yes | No | No |
| Power supply | Ample | Limited | Ample |
| Form factor | Unconstrained | Constrained | Unconstrained |
| Light source | Intense | Weak | Intense |
| Physical Mobility | No | Yes | Yes |
| Range | Short/long | Short | Long |
| Data rates | High/Low | High | Low |

Short range: device with expected operational range <= 3 m.

Long range: device with expected operational range > 3 m.

Low data rate: PHY data rate < 1 Mbps

High data rate: PHY data rate >= 1 Mbps

[Editor’s note] Items such as ample, limited, weak, intense and constrained are not precise -- no more details were available in [3, 11]

# Channel model assumptions

A typical RF channel allows link loss and bandwidth to be estimated for a generic system configuration. Variation in the specific geometry and the effect of fading is usually dealt with using a statistical distribution within the model. Extensive data collection has led to a series of models that allow verification that a system will operate under most reasonably foreseeable conditions. Table 2 shows the modeling parameters that might be required in the case of VLC. [15]

| Parameter | Requirement | Comment |
| --- | --- | --- |
| Loss parameters |  |  |
| Loss | Required | Value for a particular configuration within a scenario |
| Max/Min | Required | Allows receiver dynamic range to be specified for a particular scenario |
| Statistics | Desirable | Useful for variable data rate schemes |
| Bandwidth parameters |  |  |
| Bandwidth | Required | Value for a particular configuration within a scenario |
| Max/Min | Required | Min is required to ensure sufficient bandwidth is available. Max useful for variable data rate schemes |
| Statistics | Desirable | Useful for variable data rate schemes |

Table 2 : Channel modeling parameters

The levels of use of a model might be summarized as

- Verification. The model indicates that the maximum Loss, and dynamic range, and the minimum bandwidth are sufficient for the proposed application and scenario, so no further detailed modeling is required. This is likely to be appropriate when

  - Simple fixed configuration systems are used, and there is no reason to know the channel dynamics (as would be the case for adaptive transmitters, receivers and modulation.).

  - Other components constrain the system rather than maximum loss and minimum bandwidth, such as is the case for information broadcasting using VLC (where the LED provides the bandwidth constraint).

- Full Use. All parameters are required to estimate the performance of the system, and to check that any adaptation of the system operates properly.

## Types of channels

The type of VLC channel being used determines the level of detail of channel model that is required to adequately describe the channel, and create the parameters detailed in Table 2.

The paths to be considered are

- LOS-both single and multiple LOS paths [Main/primary focus for majority of applications]

- NLOS-both single and multiple NLOS paths

The strength of the dominant path relative to other paths is a key factor in determining the behavior of the channel. If the dominant path is much stronger than the others then the channel is LOS-like, but if there are a number of paths of different lengths and approximately equal strength then there may be bandwidth limitations.

A Dominant Path Ratio (K) can be defined as

For the NLOS case this becomes

where  is the power in a particular NLOS path from transmitter to receiver.

For the LOS case .

For the mixed case, where multiple (or single) NLOS and/or LOS paths may exist then

where is the power in a particular LOS path. (It should be noted that in the mixed case this is a slightly speculative definition, as it is uncertain as to whether this gives a sensible K value for all of the mixed cases).

The value of K at which only the dominant path can be considered is denoted as, which is estimated to be 13dB (when measured in the electrical domain) in [1].  If K is below this value then all paths need to be considered (or at least a subset of the strongest ones) and the interaction of the time delays of these paths will affect the bandwidth. In the table there is also a lower threshold, , where all the paths have almost the same strength and the channel is ‘tending’ towards a fully diffuse channel. Below this threshold it may be possible to treat the channel as fully diffuse, which may simplify the modeling process. Investigation is required to see whether this is the case.

A summary of initial observations concerning the different channel types is shown in Table 3.

| Channel type | NLOS |  | Mixed (LOS/NLOS) |  |  | Single LOS | Multiple LOS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dominant path ratio |  |  |  |  |  |  |  |  |
| Modeling considerations Simplifying assumptions | Consider all paths | Consider dominant NLOS path only | Consider NLOS paths only | Mixed-consider all paths | Consider dominant path only | Consider single path | Consider multiple LOS | Consider dominant path only |
| Modeling Techniques available (not exhaustive) | Ray-tracing, Integrating sphere for NLOS | Ray-tracing, | Ray-tracing, Integrating sphere for NLOS | Geometric for LOS+Ray-tracing, Integrating sphere for NLOS | Geometric | Geometric | Geometric + time considerations | Geometric |
| Typical situation | Diffuse channel | Strong illumination of intermediate surface-'secondary source' | Shadowing of dominant LOS paths, but no strong NLOS. | No highly dominant path (either LOS or NLOS) but some stronger paths (LOS or NLOS) | Room with source(s) on ceiling provides strong LOS path(s) compared with any NLOS (diffuse) component from surfaces | Single source with receiver in coverage area. No reflections from other surfaces. | Multiple strong sources each with LOS to receiver | Receiver predominantly in coverage area of a single source |
| Exemplar VLC application |  |  | Information broadcasting using white LED illumination |  |  | Visible point to point link between PDAs | Outdoor transmission from traffic signal or signboard. |  |
| Comment | Diffuse channel when |  | LOS and NLOS channels likely to exist in indoor 'illumination' situations. Likely that path strength is LOS>>strong NLOS, and in the case of multiple LOS paths any LOS is stronger than the strongest NLOS. |  |  |  | Often the multiple LOS can be aggregated to a single LOS as the time difference between propagation does not create significant bandwidth delay |  |
| Bandwidth | Low | High | Low | Variable | High | Very High | High/Very High | Very High |
| Path Loss | High | High | High | Situation dependent | Situation dependent | Acceptable | Low | Low |

Table 3: Summary of channel type

# Topology

Identified applications require point-to-point (P2P) communication for most usage scenarios. However, infrastructure applications also contain broadcast and/or multicast scenarios which require point-to-multipoint (P2MP) connectivity. Most applications primarily identify a LOS communication path. Unidirectional communications may be transmit only or receive only. Transmit only communications may be broadcast or directed. A receive only application may accept broadcast and/or directed data. Where data exchange is bi-directional, both symmetrical and asymmetrical data flows are defined. Bi-directional communications may be acknowledged or unacknowledged and may include broadcast and multi-cast data as well as directed data. Infrastructure devices must support P2P topologies and may support P2MP (star) topologies. Mobile and Vehicular devices may only support P2P topologies. The P2MP connectivity at the infrastructure should permit the mobile and vehicular devices to communicate with the infrastructure in a point-to-point mode.

- Device characteristics of light sources and receivers

The TCD defines the physical characteristic requirements from light sources and receivers. Although variety of light sources and receivers may need to be supported for VLC for different applications, certain types of light sources or receivers may be excluded from being considered in the specification for performance or interference reasons [8, 12]. The specifications may be device-class dependent.  Table 4 shows key device characteristics for light sources and receivers, assuming a single optical source and optical receiver. There is no artificial restriction placed on the choice of transmitter (LED, LD, ...) or receiver (PIN, APD, image sensor…) technology type as long as they satisfy considerations outlined in this proposal. The choice of the particular physical device type and associated circuitry such as drivers, lenses, filters is considered out of scope. However, it is possible that certain device types are more suitable than others to meet the technical considerations criteria. For example, optical transmitters such as LEDs might be more conducive for meeting considerations such as white light generation than LDs.

Table 4 : Key Device characteristics of light sources and receivers

| Tx/Rx Device Specification | Minimum value | Typical value | Maximum value |
| --- | --- | --- | --- |
| Intensity of light source |  | 100 – 500 mW/Sr (high data rate requires higher power; longer range requires higher power) |  |
| FOV (Receiver) | 10 degrees (need minimum to ensure mobility and robustness) |  | 30 degrees (high data rates), 45 degrees (low data rates) |
| Detector responsivity/sensitivity |  | 0.1 – 10 A/W (based on PIN/APD) [18] |  |
| Divergence angle (Transmitter) [Full angle] | 10 degrees (need minimum to ensure mobility and robustness) |  | 30 degrees (high data rates), 45 degrees (low data rates) |

# Frequency usage

The PAR has specified that the communication for visible light use the frequency band between 380 – 780 nm [1]. It may be expected that support for primary colors (Red, Green, Blue) and White for infrastructure may be desired at a minimum. [8]There needs to be allowance for manufacturer variations and device characteristics in defining the color spectrum range and spectral bandwidth for operation.

# Data Rates

The PAR states that the data rates should be sufficient to support audio and video multimedia services. [1]

However, there is a concern that the driver and corresponding circuitry may not be able to support high data rates. There is also a concern that the link margin in certain applications may not be sufficient in order to provide high data rates. These concerns must be addressed in order to specify the data rate ranges.

In order to satisfy wide variety of applications and categorize them, the application matrix document [3] has categorized devices into 2 classes of data rates:

Low data rate: PHY data rate < 1 Mbps

High data rate: PHY data rate >= 1 Mbps

VLC proposal contributions should use this classification as guidelines and could address one or both of these classes, based on the application(s) of interest. The division into low and high data rate classes could help define simpler PHY and MAC protocol designs for VLC [12,16,17], while satisfying wide range of applications provided in the application matrix document.

# Transmission range

While the PAR specifies a short range communication standard [1], it is necessary to define the expected communication distances for the intensity allowed by the device type.  A reliability metric such as PER or BER in the range of 10-2/10-8 must also be specified in order to define the range metric accurately. A PER of 8% is recommended for evaluation of TX range. While a range of packet sizes must be supported, the packet size needs to be fixed for evaluation of proposals. The packet size chosen for TX range evaluation is 256 bytes for low data rates and 1024 bytes for high data rate applications. Table 5 shows the data rates and ranges for the various application classes, adopted from the application matrix document [3]. The maximum range is defined as the distance that proposals should support for guaranteed operation. A middle range of 10 m has been additionally defined as the maximum range for classes based on B1 and D1 [10]. This has been done to allow more applications to meet the range requirements for these classes. The transmit power assumption needed for meeting the distance requirements should be specified via a link budget analysis for proposal evaluation,

Table 5: Data rates and ranges for various application classes

| Device Application class | Description | Data rate | Max Range (m) |
| --- | --- | --- | --- |
| A1 | Infrastructure to mobile | High | 3 |
| B1 | Infrastructure to mobile | Low | 10 |
| B2 | Infrastructure to vehicle-mounted | Low | 100 |
| C1 | Mobile to mobile | High | 3 |
| D1 | Mobile to mobile | Low | 10 |
| D3 | Vehicle-mounted to vehicle-mounted | Low | 100 |

# Security

Visible light communication provides security inherently by allowing the user to see the communication channel (visibility). However, additional mechanisms such as data confidentiality, integrity, authentication and encryption may be needed for certain applications such as kiosk downloads. Examples of security mechanisms that could be considered include AES-128 symmetric key cryptography and multi-level security approaches such as those used in IEEE 802.15.4a, where different levels of security are provided for different services. [9]

# Quality of Service (QoS)

QoS is important for applications such as multimedia and audio services that are mentioned in the PAR [1]. Different applications have varying requirements on data rate, frame loss and latency. Flexible QoS provisioning should be supported so that multiple applications could be supported. [9]

# Power Consumption

Power consumption may not an issue for devices belonging to the infrastructure class but may be important for mobile devices. Green and eco-friendly regulations are recommending power consumption be considered for the infrastructure as well by supporting mechanisms such as dimming. Power saving mechanisms such as sleep modes and duty cycling should be supported. Proposals addressing power consumption requirements could also consider issues such as battery life, process technology use, supply voltage considerations and average vs. peak power consumption. [9]

# Coexistence with ambient light and other optical technologies

VLC will need to co-exist with ambient lighting and other optical technologies. The tolerance levels to a few key impairments must be specified in order to ensure the technology will be robust in the presence of interference. The tolerance level should be specified for the highest data rate with the interference within the FOV of the receiver since interference would be the typical use case of operation. The acceptable PER degradation due to ambient lighting should be < 1 dB at 8% PER. The range for testing the ambient light co-existence is the range required for operation at 8% PER for the highest supported data rate.  Table 6 shows the suggested minimum tolerance level requirements for ambient light interference for indoor and outdoor applications. For the purposes of co-existence and ambient light testing, a flat frequency model can be assumed for these levels without a specific reference to an ambient light source. [9] It is possible that co-existence tolerance may be dependent on the sophistication of the receiver implementation. In such cases, more details about the receiver implementation should be provided for proposal evaluation.

Table 6 : Tolerance levels for ambient light

| Interference | Minimum Tolerance level (lux) |
| --- | --- |
| Outdoor | 10000 |
| Indoor | 500 |

# Form Factor

Form factor is an important consideration, especially for mobile devices. The form factor should support integration with existing drivers and dimmer circuits. [9]

# Complexity

The solution should have low complexity to support mass adoption. Ease of integration into existing LED-based products is an important consideration for complexity. The need to have lot of changes for external components and circuits such as drivers etc. should be minimized. [9]

# Mobility and Link switching

There may be a need to support link switching due to physical mobility or interference. Mobility can be of two types: physical and logical. As shown in Figure 2, physical mobility occurs when the VLC device M1 changes position due to movement within the coverage area of infrastructure I1 while logical mobility occurs when the device M1 changes the coverage area from infrastructure link I2 to I3 due to interference or channel selection [19, 20]. Both physical and logical mobility aspects should be considered. Limited MAC signaling and support may be required in order to support link switching. However, service continuity mechanisms involving multiple simultaneously active links (such as M1 connected to both I2 and I3 simultaneously) could be considered out of scope of the standard. Issues such as link detection for channel selection and switching time may also need to be considered in this regard [11].

Figure 2 : Physical and logical mobility

# Lighting Inherency

Brightness and color of illumination source must be maintained during communication for infrastructure devices. Proposals should ensure that the brightness of the light source is not affected due to VLC. [10]

# Lighting Dimming

The infrastructure link should support dimming, albeit possibly at reduced performance levels. It is possible that popular LED dimming options such as PWM could have an impact on communication [13].

# Visibility support

The PAR specifies that the MAC layer should support visible links [1]. In certain applications, the user may need to estimate and roughly aim the target device for correct alignment for communication. Having support for visibility in the standard can give the user several useful features as follows:

- Help the link alignment

- Help verify the link status

- Physical security (due to the beam visibility)

Visibility aspects should be considered in the MAC protocol design in order to support alignment and link status indication for communication [14].

# Regulatory requirements

Devices shall comply with the regulatory requirements specified for visible light communication [4]. Requirements include eye safety regulations such as IEC 60825. Other eye safety issues such as flicker [7] must be addressed.

# References

- VLC PAR document: IEEE 802.15-08-0656-03-0vlc

- TG6 Technical Requirements Document IEEE 802. 15-08-0644-08-0006

- VLC applications summary document : IEEE 802.15-09-0125-08-0007

- VLC regulatory document: IEEE 802.15-09-0202-01-0007

- VLC wavelength range: IEEE 802.15-08-0468-00-0vlc

- Light source consideration for VLC: IEEE 802.15-08-0681-00-0vlc

- VLC frame flicker: IEEE 802.15-09-0297-00-0007

- Comments for TRD input – Part 1 : IEEE 802.15-09-0373-00-0007

- Comments for TRD input – Part 2 : IEEE 802.15-09-0368-01-0007

- Technical Requirements for VLC applications: IEEE 802.15-09-0209-01-0007

- VLC device classification and mobility considerations: IEEE 802.15-09-0218-00-0007

- ISO 21214, link budget estimate and application subclasses: IEEE 802.15-09-0296-00-0007

- Dimming considerations for visible light communication: IEEE 802.15-09-0369-00-0007

- Visibility for VLC on TRD: IEEE 802.15-09-0372-01-0007

- VLC channel modeling: some preliminary observations: IEEE 802.15-09-0146-00-0007

- Some thoughts on VLC: IEEE 802.15-09-0111-00-0007

- Consideration for minimum PHY data rate requirement for TRD: IEEE 802.15-09-0221-01-0007

- Photodiode Technical Information, Hamamatsu Corporation

- VLC- Application Category Terms & Mobility: IEEE 802.15-09-0205-01-0007

- Considerations for getting a delicate VLC Application Summary: IEEE 802.15-09-0159-00-0007