IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | MAC Service Primitives for Time Slotted Channel Hopping |  |
| Date Submitted | [10 March 2009] |  |
| Source | [Chol Su Kang] [Kuor Hsin Chang] [Rick Enns] [Kris Pister] [Ludwig Winkel] [Clint Powell] [Jose Gutierrez] | Voice: [ ] Fax: [ ] E-mail: [ ckang@dustnetworks.com ] [ kuor-hsin.change@freescale.com ] [ enns@stanfordalumni.org ] [clinton.powell@freescale.com] [Jose.Gutierrez@emerson.com] [ludwig.winkel@siemens.com ] |
| Re: | [Task Group 15.4e: Enhancements to the 802.15.4 MAC] |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Content

1.1 New MAC Service Primitives	3

1.1.1	SET-SLOTFRAME	3

1.1.1.1	SET-SLOTFRAME.request	3

1.1.1.1.1	Semantics	3

1.1.1.1.2	When Generated	4

1.1.1.1.3	Effect on Receipt	4

1.1.1.2	SET-SLOTFRAME.confirm	4

1.1.1.2.1	Semantics	4

1.1.1.2.2	When Generated	4

1.1.1.2.3	Effect on Receipt	5

1.1.2	SET-LINK	5

1.1.2.1	SET-LINK.request	5

1.1.2.2	Semantics	5

1.1.2.2.1	When Generated	6

1.1.2.2.2	Effect on Receipt	6

1.1.2.3	SET-LINK.confirm	6

1.1.2.3.1	Semantics	6

1.1.2.3.2	When Generated	7

1.1.2.3.3	Effect on Receipt	7

1.1.3	TSCH-MODE.request	7

1.1.3.1.1	Semantics	7

1.1.3.1.2	When Generated	7

1.1.3.1.3	Effect on Receipt	7

1.1.3.2	TSCH-MODE.confirm	7

1.1.3.2.1	Semantics	8

1.1.3.2.2	When Generated	8

1.1.3.2.3	Effect on Receipt	8

1.1.3.3	LISTEN.request	8

1.1.3.3.1	Semantics	8

1.1.3.3.2	When Generated	9

1.1.3.3.3	Effect on Receipt	9

1.1.3.4	LISTEN.confirm	9

1.1.3.4.1	Semantics	9

1.1.3.4.2	When Generated	9

1.1.3.4.3	Effect on Receipt	9

1.1.3.5	LISTEN.indication	9

1.1.3.5.1	Semantics	9

1.1.3.5.2	When Generated	10

1.1.3.5.3	Effect on Receipt	10

1.2 Modified MAC Service Primitives	11

1.2.1	MCPS-DATA	11

1.2.1.1	MCPS-DATA.request	11

1.2.1.1.1	Semantics	11

1.2.1.1.2	When Generated	12

1.2.1.1.3	Effect on Receipt	12

1.2.1.2	MCPS-DATA.confirm	12

1.2.1.2.1	Semantics	12

1.2.1.2.2	When Generated	13

1.2.1.2.3	Effect on Receipt	13

1.2.1.3	MCPS-DATA.indication	13

## New MAC Service Primitives

| Primitive | Description | Request | Confirm | Response | Indication |
| --- | --- | --- | --- | --- | --- |
| SET-SLOTFRAME | Add, delete, or modify a SLOTFRAME | X | X |  |  |
| SET-LINK | Add, delete, or modify a link | X | X |  |  |
| TSCH-MODE | Time Slot Channel Hopping mode ON or OFF | X | X |  |  |
| LISTEN | Start listening for an advertisement | X | X |  | X |

### SET-SLOTFRAME

#### SET-SLOTFRAME.request

The SET-SLOTFRAME.request primitive is used to add, delete, or change a slotframe at the MAC layer.

##### Semantics

The semantics of the SET-SLOTFRAME.request primitive is as follows:

SET-SLOTFRAME.request	(

frameId,

operation,

size,

channelPage,

channelMap,

activeFlag

)

The following table specifies parameters for the SET-SLOTFRAME.request primitive.

- SET-SLOTFRAME.request Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| frameId | Integer |  | Unique identifier of the new frame |
| operation | Enumeration | ADD DELETE MODIFY | Operation to perform on the frame |
| size | Integer | 0-65535 | Number of slots in the new frame |
| channelPage | Integer | 0-32 | Channel page supported by PHY |
| channelMap | Bitmap |  | Indicating which frequency channels in the channel page are to be used for channel hopping |
| activeFlag | Enumeration | TRUE FALSE | Frame is active Frame is not active |

##### When Generated

An SET-SLOTFRAME.request is generated by the DNMA and issued to the MLME to create, delete or update a frame on the MAC layer.

##### Effect on Receipt

On receipt of an SET-SLOTFRAME.request, MLME shall verify the parameters passed with the primitive. If the requested operation is ADD, MLME shall attempt to add an entry into the macSlotFrameTable. If the operation is MODIFY, it shall attempt to update an existing slotframe record in the table. If operation is DELETE, all parameters except frameId and operation shall be ignored, and the slotframe record must be deleted from macSlotFrameTable. If there are links in the slotframe that’s being deleted, the links shall be deleted from MAC layer. If the device is in the middle of using a link in the slotframe that’s being updated or deleted, the update should be postponed until after the link operation completes.

#### SET-SLOTFRAME.confirm

The SET-SLOTFRAME.confirm primitive reports the results of the SET-SLOTFRAME.request command.

##### Semantics

The semantics of the SET-SLOTFRAME.confirm primitive is as follows:

SET-SLOTFRAME.confirm	(

frameId,

operation,

status

)

The following table specifies parameters for the SET-SLOTFRAME.confirm primitive.

- SET-SLOTFRAME.confirm Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| frameId | Integer |  | Unique identifier of the frame that was requested to be added |
| operation | Enumeration | ADD DELETE MODIFY | Operation to perform on the frame |
| status | Enumeration | SUCCESS INVALID_PARAMETER FRAME_NOT_FOUND MAX_FRAMES_EXCEEDED | Results of the SET-SLOTFRAME.request command |

##### When Generated

The SET-SLOTFRAME.confirm primitive is generated by the MLME when the SET-SLOTFRAME.request is completed.

If any of the arguments fail a range check, the status shall be INVALID_PARAMETER. If a new slotframe is being added and the macSlotFrameTable is already full, the status shall be MAX_SLOTFRAMES_EXCEEDED. If an update or deletion is being requested and the corresponding slotframe cannot be found, the status shall be SLOTFRAME_NOT_FOUND.

##### Effect on Receipt

On receipt of an SET-SLOTFRAME.confirm primitive, the DNMA application is notified of status of its corresponding SET-SLOTFRAME.request.

.

### SET-LINK

#### SET-LINK.request

SET-LINK.request primitive requests a new link to be added, or requests to delete an existing link at the MAC layer. operationType indicates whether the SET-LINK operation is to add or to delete a link.

#### Semantics

The semantics of the SET-LINK.request primitive is as follows:

SET-LINK.request to add a link	(

operationType (ADD_LINK or MODIFY_LINK),

linkHandle,

frameId,

timeslot,

chanOffset,

linkOptions,

linkType,

numberOfNodeAddr,

nodeAddrList

)

Or

SET-LINK.request to delete a link	(

operationType (DELETE_LINK),

linkHandle

)

The following table specifies parameters for the SET-LINK.request primitive with ADD_LINK or MODIFY_LINK operationType.

- SET-LINK.request Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| operationType | Enumeration | ADD_LINK, MODIFY_LINK, DELETE_LINK |  |
| linkHandle | Integer | 0x00–0xff | Unique identifier for the link |
| frameId | Integer |  | Frame ID of the link to be added |
| timeslot | Integer |  | Timeslot of the link to be added |
| chanOffset | Integer |  | Channel offset of the link |
| linkOptions | Bitmap |  | b001 = Transmit b010 = Receive b100 = Shared |
| linkType | Enumeration | NORMAL ADVERTISING DISCOVERY | Type of link |
| numberOfNodeAddr | Integer |  | Number of neighbor devices connected to the link |
| nodeAddrList | Integer |  | Address list of neighbor devices connected to the link |

##### When Generated

When operationType=ADD_LINK or MODIFY_LINK:

SET-LINK.request primitive is generated by the DNMA to add a link or to modify an existing link  in a slotframe.

When operationType=DELETE_LINK:

SET-LINK.request primitive is generated by the DNMA to delete an existing link at the MAC layer.

##### Effect on Receipt

When operationType=ADD_LINK or MODIFY_LINK:

On receipt of the SET-LINK.request, the MAC layer shall attempt to add the indicated link to its link table, and, if needed, the new neighbor to its neighbor table. The Connection table must also be updated to include the Connection to the indicated neighbor nodeAddr. Upon completion, the result of the operation must be reported through the corresponding SET-LINK.confirm primitive.

When operationType=DELETE_LINK:

On receipt of the SET-LINK request the device shall attempt to remove the link from macLinkTable. If the link is currently in use, the deletion shall be postponed until after the link operation completes. After deletion of the link, the MLME shall check if this was the last link to a neighbor device. If it was the last link, the MLME shall remove the neighbor record from the neighbor table.

#### SET-LINK.confirm

The MLME-ADD-LINK.confirm primitive indicates the result of add or delete link operation.

##### Semantics

The semantics of the SET-LINK.confirm primitive is as follows:

SET-LINK.confirm	(

status,

linkHandle

)

The following table specifies parameters for the SET-LINK.confirm primitive.

- SET-LINK.confirm Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS INVALID_PARAMETER UNKNOWN_SLOTFRAME MAX_LINKS_EXCEEDED MAX_NEIGHBORS_EXCEEDED SUCCESS LINK_NOT_FOUND | The result of the add link operation The result of the delete link operation |
| linkHandle | Integer |  | Unique identifier for the link |

##### When Generated

The SET-LINK.confirm is generated as a result of the SET-LINK.request operation.

##### Effect on Receipt

The layer that issued the SET-LINK.request to MAC may process the result of the operation. The status of the primitive shall indicate SUCCESS if the operation completed successfully. Otherwise, the status indicates the cause of the failure.

### TSCH-MODE.request

The TSCH-MODE.request puts the MAC into TSCH mode, or out of TSCH mode.

##### Semantics

The semantics of the TSCH-MODE.request primitive is as follows:

TSCH-MODE.request	(

modeSwtich

)

The following table specifies parameters for the TSCH-MODE.request primitive.

- TSCH-MODE.request Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| modeSwitch | Enumeration | ON, OFF | Target mode. This mode indicates whether TSCH mode should be started or stoped. |

##### When Generated

The TSCH-MODE.request may be generated by the higher layer after the device has received advertisements from the network and is synchronized.

##### Effect on Receipt

Upon receipt of the request, the MAC shall start operating its TSCH state machine using frames and links already contained in its database. To successfully complete this request the device must already be synchronized to a network.

#### TSCH-MODE.confirm

The TSCH-MODE.confirm primitive reports the result of the TSCH-MODE.request primitive.

##### Semantics

The semantics of the TSCH-MODE.confirm primitive is as follows:

TSCH-MODE.confirm	(

modeSwitch,

status

)

The following table specifies parameters for the TSCH-MODE.confirm primitive.

- TSCH-MODE.confirm Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| modeSwitch | Enumeration | ON, OFF | Target mode. This mode indicates whether this confirmation is due to TSCH mode ON request or OFF request. |
| status | Enumeration | SUCCESS NO_SYNC |  |

##### When Generated

The TSCH-MODE.confirm is generated by the MAC layer to indicate completion of corresponding request. If the corresponding request was to turn on the TSCH-MODE, but the MAC layer has not been synchronized to a network, the status shall be NO_SYNC. Otherwise, the status shall be SUCCESS.

If the corresponding request was to turn off the TSCH-MODE, the status shall be SUCCESS, and the MAC layer will stop the TSCH-MODE operation.

##### Effect on Receipt

The higher layer may use the indication to process the result of TSCH-MODE.request.

#### LISTEN.request

##### Semantics

The semantics of the LISTEN.request primitive is as follows:

LISTEN.request	(

channelPage,

rfChannel,

time

)

The following table specifies parameters for the LISTEN.request primitive.

- LISTEN.request Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| channelPage | Integer |  | Channel page supported by PHY |
| rfChannel | Integer |  | The first channel on which to listen |
| time | Integer |  | The amount of time to stay on each channel |

##### When Generated

The LISTEN.request shall be generated by the next higher layer to initiate search for a network.

##### Effect on Receipt

Upon receipt of the request the MAC layer shall activate the radio on the indicated channel and wait for an ADVERTISEMENT packet. Valid ADVERTISEMENT packets received in this state shall result in synchronization to the network and generation of LISTEN.indication. All other packets shall be dropped. The MAC shall stay in the listening state until the indicated time expires, TSCH-MODE.request is received.

#### LISTEN.confirm

##### Semantics

The semantics of the LISTEN.confirm primitive is as follows:

LISTEN.confirm	(

status

)

The following table specifies parameters for the LISTEN.confirm primitive.

- LISTEN.confirm Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS |  |

##### When Generated

The MAC layer shall generate LISTEN.confirm when it completes the listen operation started by LISTEN.request

##### Effect on Receipt

On receipt of the primitive, the higher layer may continue with its joining state machine.

#### LISTEN.indication

The LISTEN.indication indicates that a device received a packet while waiting for an ADVERTISEMENT packet.

##### Semantics

The semantics of the LISTEN.indication primitive is as follows:

LISTEN.indication	(

channelMap,

securityLevel,

joinPriority,

linkQuality,

links[]

)

The following table specifies parameters for the LISTEN.indication primitive.

- LISTEN.indication Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| channelMap | Bitmap |  | Map of channels |
| securityLevel | Enumeration |  | Security level in advertisement packet. See Table 95 in IEEE802.15.4-2006 |
| joinPriority | Integer |  | Join priority indicated in advertisement |
| linkQuality | Integer |  | Link quality indicated on the packet by the PHY layer |
| links |  |  | See Table 10 |

- LISTEN.indication Link Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| slotframeSize | Integer |  | Slotframe size |
| timeslot | Integer |  | Timeslot |
| chanOffset | Integer | 0-15 | Channel offset |
| linkType | Enumeration | ACTIVATE_LINK JOIN_LINK | Type of link |

##### When Generated

The LISTEN.indication shall be generated when an ADVERTISEMENT packet has been received by the device. Upon receiving a valid ADVERTISEMENT packet, the device that’s trying to join a network shall synchronize to the network using the absolute slot number in the packet, and generate indication to the higher layer.

##### Effect on Receipt

The higher layer may process the indication to collect the list of and information about neighbors that are available for joining.

## Modified MAC Service Primitives

| Primitive | Description | Request | Confirm | Response | Indication |
| --- | --- | --- | --- | --- | --- |
| MCPS-DATA | MCPS-DATA transports a data SPDU between peer SSCS entities. | X | X |  | X |

### MCPS-DATA

#### MCPS-DATA.request

The MCPS-DATA.request primitive requests the transfer of a data SPDU (i.e., MSDU) from a local SSCS

entity to a single peer SSCS entity from the specified list.

##### Semantics

The semantics of the MCPS-DATA.request primitive

MCPS-DATA.request	 (

SrcAddrMode,

DstAddrMode,

DstPANId,

DstAddr,

msduLength,

msdu,

msduHandle,

TxOptions,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

numberOfAdditionalDstAddr

additionalDstAddr,

numberOfLinkHandles,

linkHandleList

)

The following table specifies parameters for the MCPS-DATA.request primitive.

- MCPS-DATA.request Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SrcAddrMode |  |  | See Table 41 in IEEE802.15.4-2006 |
| DstAddrMode |  |  | See Table 41 in IEEE802.15.4-2006 |
| DstPANId |  |  | See Table 41 in IEEE802.15.4-2006 |
| DstAddr |  |  | See Table 41 in IEEE802.15.4-2006 |
| msduLength |  |  | See Table 41 in IEEE802.15.4-2006 |
| msdu |  |  | See Table 41 in IEEE802.15.4-2006 |
| msduHandle |  |  | See Table 41 in IEEE802.15.4-2006 |
| TxOptions |  |  | See Table 41 in IEEE802.15.4-2006 |
| SecurityLevel |  |  | See Table 41 in IEEE802.15.4-2006 |
| KeyIdMode |  |  | See Table 41 in IEEE802.15.4-2006 |
| KeySource |  |  | See Table 41 in IEEE802.15.4-2006 |
| KeyIndex |  |  | See Table 41 in IEEE802.15.4-2006 |
| numberOfAdditionalDstAddr |  |  | If number of Additional DstAddr is zero, no additionalDstAddr will follow. |
| additionalDstAddr |  |  | the data SPDU should be transferred to the destination specified by either DstAddr or any of DstAddr in additionalDstAddr. |
| numberOfLinkHandles |  |  | If number of Link Handles is zero, no Link Handle will follow. |
| linkHandleList |  |  | Use the link handles list to transfer the data SPDU. |

##### When Generated

The MCPS-DATA.request primitive is generated by a local SSCS entity when a data SPDU (i.e., MSDU) is

to be transferred to a peer SSCS entity.

##### Effect on Receipt

If numberOfLinkHandles is zero, the effect will be the same as it is described in Section 7.1.1.1.3 in IEEE802.15.4-2006.

Otherwise, linkHandleList contains the handles of the links. MAC should transfer the requested data SPDU on one of these links. MAC will select a link to transmit (or retransmit if ACK is not received) in earliest possible opportunity.

#### MCPS-DATA.confirm

The MCPS-DATA.confirm primitive reports the results of a request to transfer a data SPDU (MSDU) from

a local SSCS entity to a single peer SSCS entity.

##### Semantics

The semantics of the MCPS-DATA.confirm primitive are as follows:

MCPS-DATA.confirm	(

msduHandle,

status,

timestamp,

dstAddr,

linkHandle

)

The following table specifies parameters for the MCPS-DATA.confirm primitive.

- MCPS-DATA.confirm Parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| msduHandle |  |  | See Table 42 in IEEE802.15.4-2006 |
| status |  |  | See Table 42 in IEEE802.15.4-2006 |
| timestamp |  |  | See Table 42 in IEEE802.15.4-2006 |
| dstAddr |  |  | Destination Address that the data SPDU was transferred. |
| linkHandle |  |  | Handle of Link that the data SPDU was transferred. |

##### When Generated

The MCPS-DATA.confirm primitive is generated by the MAC sublayer entity in response to an MCPSDATA.

request primitive. The MCPS-DATA.confirm primitive returns a status of either SUCCESS,

indicating that the request to transmit was successful, or the appropriate error code. The status values are

fully described in 7.1.1.1.3 and subclauses referenced by 7.1.1.1.3.

##### Effect on Receipt

On receipt of the MCPS-DATA.confirm primitive, the SSCS of the initiating device is notified of the result

of its request to transmit. If the transmission attempt was successful, the status parameter will be set to

SUCCESS. Otherwise, the status parameter will indicate the error.

#### MCPS-DATA.indication

No change from IEEE802.15.4-2006.