IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG4g Proposal X-Ref (Work in Progress) |  |
| Date Submitted | 29 July 2026 |  |
| Source | [Benjamin A. Rolfe] [ Blind Creek] | Voice: [ +1.408.395.7732 ] Fax: [ ] E-mail: [ ben @ blindcreek.com ] |
| Re: | Task Group 15.4g technical proposals |  |
| Abstract | Summarizes the proposals given so far with respect to the PHY parameters table in doc # 09-0075 Rev 1 adds information posted since Vancouver meeting |  |
| Purpose | Discussion within task group and identification of collaboration opportunities |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

TG4g Proposals as of March 2009 Plenary closing

Preliminary Proposals Presented

|  | Short Title | Ref. Doc # | Author(s) | Company(s) |
| --- | --- | --- | --- | --- |
| 1 | Narrow band PHY | 15-09-0118-03 | Seibert, Rolfe, Flammer | SSN, BCA |
| 2 | Waveniss | 15-09-0120-02 | Dugas, Rouse, Schwoerer | Coronis, FT |
| 3 | Ultra-link Processing | 15-09-0128-00 | Myers, Howard, Lampe | On-Ramp |
| 4 | Multi-Rate PHY Proposal | 15-09-0124-00 | Schmidt, Eggert, Poegel, Bacher, Beyer | Atmel |
| 5 | Multi-Regional Sub-GHz PHY | 15-09-0135-01 | K. T. Le | TI |
| 6 | Flexible DSSS Proposal | 15-09-0116-01 | Enns, Change, Powell | Consultant, Freescale |
| 7 | Smart Grid Communications Proposal | 15-09-0127-02 | Mason, McCullough, Hart | Elster |

Additional PHY recommendations presented

| Short Title | Ref Doc # | Author(s) | Company(s) |
| --- | --- | --- | --- |
| Signal Processing options | 15-09-0126-01 | Steve Shearer | Consultant |
| Battery Operated Sensing Device Requirements for 802.15.4g | 15-09-0129-01 | Mark Thompson | Aclara |
| Trends in SUN capacity | 15-09-0114-00 | Monnerie | Landis+Gyr |
| Design Considerations for the SUN Network | 15-09-0131-01 | Kent |  |

I hope I didn’t’ mis-categorize your presentation.  This is strictly my interpretation of what I heard.

The following table maps from the presentations into the PHY parameters table from (15-09-0075).

Additions since March meeting

|  | Short Title | Ref. Doc # | Author(s) | Company(s) |
| --- | --- | --- | --- | --- |
| 8 | OFDM option for TG4g (doc#) | 15-09-0262-00 | Monnerie, Shearer, Shimada | Landis+Gyr, Consultant, Yokogawa Electric Corporation |

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PHY Parameter | NB-PHY | Wavenis | ULP | Multi-Rate | Multi-Region | Flex-DSS | SGCP | O-opt |
| Operating band | Multiple < 1GHz, 2.4GHz | 915MHz, 868MHz, 316-433MHz | 902MHz and 2.4GHz | 860MHz, 920MHz 2.4GHz | 902-928 MHz 863-870 MHz | same as 15.4 DSSS | 902-928 others possible | Multiple < 1GHz, 2.4GHz |
| Channel BW | < 250 kHz | Variable | 0.5MHz, 2MHz, 5MHz | Variable | Variable | < 5MHz and < 2MHz |  | 100kHz, 200kHz, 300kHz |
| Channel spacing | 300 kHz | 50kHz 300kHz |  | Variable | Variable | 5MHz (2.4) 2MHz (902) | 400 kHz | 100kHz MAC control for BW>100 |
| Modulation | M/FSK | GFSK | D-BPSK | GMSK | 2 & 4-GFSK | O-QPSK | FSK | OFDM-BPSK, OFDM-QPSK, OFDM-16QAM, OFDM-64QAM |
| FEC | None | BCH (31,21) rate 1/3 | Encode rate 1/2 | block code (4,n) | (128,120,4) SECDED/BCH | None |  | yes |
| Frequency Hopping | per PPDU (MAC) | 2-octet (PHY) | NA | MAC | MAC |  | per PPDU (MAC) | yes |
| PHY frame structure: |  |  |  |  |  |  |  |  |
| MAX payload | 2047 |  |  |  |  | 1500 | > 1500 | 2-47 |
| SHR | 56 bits pre + 16 bit SFD |  |  | Variable | Preamble + SFD |  | Variable pre +16 bit SFD | TBD |
| CRC | CRC-32 |  | CRC-32 |  | PCB |  | CRC-16 | CRC-32 |
| Data Whitening | 8-bit LFSR variable seed | Yes (TBD) |  | 8-bit | Yes |  | Upper Layer | no |
| Data rate(s) | 100 kbps | 20 to 100 kbps | Variable | Variable | 50/100/200/400 kbps | up to 1Mbps | 9.6kbps – 300kbps | 33kbps to 1Mbps |
| Symbol / chip rate | 100 sps |  | Variable | 2mcps; 1mcps; < 0.5 |  | Variable / 2Mcps |  | Variable |
| Transmit Power | up to 1 W | 10mW- 0.5W | up to 1 W | up to 1W | Up to 1W | [PSD] | up to 0.25W | up to 1 W |
| PSD | NA |  |  |  |  | 8dm/2khz |  | NA |
| TX Power Control | Yes | Dynamic | Dynamic |  | Dynamic | Optional | Yes | Yes |
| Chan availability (interference detection) | MAC and/or Higher layer | Higher layer |  |  |  | MAC |  | MAC and/or Higher layer |
| Link Quality | RSSI, NHL | RSSI | RSSI |  |  | RSSI, NHL |  | RSSI, MAC (ACK success rate) |
| Reliability enhancing features/methods | Hopping, CRC-32 | interleaving + FEC | Encode+interleave | Block coding | Hopping, FEC |  | Hopping | Frequency Hopping, CRC-32, FEC, channel equalization, bit redundancy, cyclic prefix, pilot tones |
| Co-existence features | Hopping, LDC | Hopping, TX power ctrl | Random user code delay, TX power control | Hopping | Hopping | 15.4 | hopping | Hopping, LDC |
| SoP support features | channel diversity | channel diversity | Gold code diversity | Channel diversity | Channel Plan | 15.4 | Split band into sub (2) | Channel diversity, CRC-32 seed, channel skipping/band splitting, hopping pattern seed |