IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG4g Proposal X-Ref (Work in Progress) |  |
| Date Submitted | 08 May 2009 |  |
| Source | [Benjamin A. Rolfe] [ Blind Creek] | Voice: [ +1.408.395.7732 ] Fax: [ ] E-mail: [ ben @ blindcreek.com ] |
| Re: | Task Group 15.4g technical proposals |  |
| Abstract | Summarizes the proposals given so far with respect to the PHY parameters table in doc # 09-0075 Rev 1 adds information posted since Vancouver meeting; Rev 2 adds more information posted since Vancouer. Rev 3 revices for proposal aggretation prior to May 2009 interim |  |
| Purpose | Discussion within task group and identification of collaboration opportunities |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Introduction

This document attempts to cross-reference the TG4g proposals as of 08 May 2009 (prior to the May interim meeting).  This revision includes the updates made from the preliminary proposals heard in March 2009 and adds a lot of new proposals submitted prior to the May 2009 meeting.

Section 2 tables list the proposals with the document numbers for reference. These serve as the index identifier for the cross reference tables.

Section 3 contains the cross reference tables. There are more proposals than can fit into a single cross-reference table, so I have split it into semi-arbitrary categories for working with:

- Narrow bandwidth frequency hopping FSK proposals;

- DSSS proposals;

- OFDM proposals;

- Other (everything else)

These divisions are meant only to provide for a means to fit all the information onto a page.  Some proposals may logically fit into more than one of these categories.

Three cross reference tables are provided in this version. The remaining proposals (the “other” category) did not so obviously fit into this cross reference format.

# List of Proposals

Preliminary proposals and recommendations presented in Vancouver, sorted by Document #.

| Ref. Doc # | Short Title | Author (company) |
| --- | --- | --- |
| 15-09-0114-00 | Trends in SUN capacity | Monnerie (Landis+Gyr) |
| 15-09-0116-01 | Flexible DSSS Proposal | Enns (Consultant), Chang, Powell (Freescale) |
| 15-09-0118-03 | Narrow band PHY | Seibert, Flammer (SSN), Rolfe (BCA) |
| 15-09-0120-02 | Waveniss | Dugas, Rouse (Corinis), Schwoerer (FT) |
| 15-09-0124-00 | Multi-Rate PHY Proposal | Schmidt, Eggert, Poegel, Bacher, Beyer (Atmel) |
| 15-09-0128-00 | Dynamic DSSS | Myers, Howard, (On-Ramp), Lampe (Consultant) |
| 15-09-0127-02 | Smart Grid Communications Proposal | Mason, McCullough, Hart (Elster) |
| 15-09-0126-01 | Signal Processing options | Steve Shearer (Consultant) |
| 15-09-0129-01 | Battery Operated Sensing Device Requirements for 802.15.4g | Mark Thompson (Aclara) |
| 15-09-0131-01 | Design Considerations for the SUN Network | Kent (ADI) |
| 15-09-0135-01 | Multi-Regional Sub-GHz PHY | K. T. Le (TI) |
|  | Added after March meeting, discussed in conference calls: |  |
| 15-09-0262-00 | OFDM option for TG4g (doc#) | Monnerie (Landis+Gyr), Shearer (Consultant), Shimada (Yokogawa) |

The following lists the proposals submitted for the May 2009 interim meeting, sorted by Document #.

| Document # | Title | Authors (Company) | Xref ID |
| --- | --- | --- | --- |
| 15-09-0124-03 | Multi-Rate PHY Proposal | Michael Schmidt, Dietmar Eggert, Frank Poegel, Torsten Bacher, Sascha Beyer (Atmel) | F1 D1 |
| 15-09-0274-00 | Low power features for SUN | Hirohito Nishiyama, Ryoji Ono, Seiichi Hiraoka (Mitsubishi) | M1 |
| 15-09-0275-01 | PHY Proposal for 802.15.4g based on OFDM technology using TV White Space | Sangsung Choi , Cheolho Shin, Byounghak Kim(ETRI) | O1 |
| 15-09-0278-00 | GFSK PHY proposal for Smart Utility Networks | Henk de Ruijter (Silicon Labs) | F2 |
| 15-09-0280-00 | NBFH Detailed PHY Proposal for 802.15.4g | Cristina Seibert, George Flammer (Silver Spring Networks); Dietmar Eggert, Michael Schmidt (Atmel); Jeritt Kent (ADI) | F3 |
| 15-09-0283-00 | Adopting Flexible DSSS Modulation for PHY Layer | Yaoxian Fu, Jie Shen, Tao Xing, Haitao Liu, Liang Li, Liang Zhang, Zhuang Zhao (SUMIT, Vinno, JiaXing Wireless Sensor Networks Engineering Center, Huawei) | D2 |
| 15-09-0285-00 | MAC and PHY proposal for 802.15.4g for Smart Utility Networks | Takaaki Hatauchi (Fuji) | M2 |
| 15-09-0288-00 | Battery Operated Sensing Device Operations in 802.15.4g | Mark Thompson (Aclara) | B1 |
| 15-09-0289-00 | Affordable OFDM for the SUN PHY | Steve Shearer (Consultant) | O2 |
| 15-09-0290-00 | Power and Spectrum Efficient PHY Proposal for 802.15.4g | Khanh Tuan Le, Per Torstein Roine (Texas Instruments) | F4 |
| 15-09-0291-00 | It's all about Future-Proofing | Emmanuel Monnerie (Landis+Gyr) | O3 |
| 15-09-0292-00 | Itron Proposal for 802.15 TG4g PHY Amendment | Daniel Popa, John Buffington (ITRON) | F5 |
| 15-09-0293-00 | OFDM PHY proposal for SUN | Rishi Mohindra (MAXIM) | O4 |
| 15-09-0294-00 | Affordable OFDM for SUN - detailed proposal | Steve Shearer (Consultant) | O2 |
| 15-09-0295-00 | 4g-flexible-dsss-final-proposal | Chang, Powell (Freescale), Enns (Consultant) | D3 |
| 15-09-0298-01 | Common Platform for narrow bandwidth frequency hopping PHY | Benjamin Rolfe (BCA), et al. | F6 |
| 15-09-0299-00 | Sensus'_Proposal_to_TG4g | Britton Sanderford (Sensus) | B2 |
| 15-09-0301-00 | Two Rate DSSS for SUN | Tae-Joon Park, Wun-Cheol Jeong, Hoyong Kang, Seong-Soon Joo (ETRI ) | D4 |
| 15-09-0302-00 | FHSS Neighborhood Area Network Communications Proposal | David Hart (Elster Electricity) | F7 |
| 15-09-0303-00 | DSSS Neighborhood Area Network Communications Proposal | Scott Weikel (Elster) | D5 |
| 15-09-0304-00 | Draft for 802.15 TG4g PHY amendment FHSS PHY w/ Interoperable Battery Powered Devices | David Hart (Elster Electricity) | F6 |
| 15-09-0305-00 | 2.4 GHz NBFH Proposal to TG4g | Soumitri Kolavennu (Honeywell) | F8 |
| 15-09-0306-00 | Robust multi-channel adaptation | Gahng-seop Ahn, Junsun Ryu, Myung Lee, Seong-soon Joo (CUNY, ETRI) | M3 |
| 15-09-0307-00 | Dynamic DSSS draft proposal for SUN | David Howard, Ted Meyers (On-Ramp) |  |
| 15-09-0308-00 | robust multi channel adaptation-description | Gahng-seop Ahn, Junsun Ryu, Myung Lee, ChangSub Shin, Seong-soon Joo (CUNY, ETRI) | M4 |
| 15-09-0310-00 | Coronis-FT-proposal-for-802-15-tg4-phy-amendment.doc | Jean Schwoerer (France Telecom R&D), John Rouse, Christophe Dugas (Coronis) | F9 |
| 15-09-0312-00 | PHY and MAC Proposals for low-power consumption SUN | Fumihide Kojima, Hiroshi Harada (NICT) | F10 |

# Cross Reference Tables

The following tables map from the presentations into the PHY parameters table from (15-09-0075).   Where entries are blank, nothing as specified in the proposal.

Table 1: Narrow band, FSK

|  | F1 | F2 | F3 | F4 | F5 | F7 | F8 | F9 | F10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PHY Parameter | MultiRate | GFSK-SI | NBFH-SAA | PSEP-TI | ITRON | FHSS-EL | 24GNBFH | Wavenis | LPC |
| Operating band (MHz) | 868-869.5 | 450-470, 868-870, 902-928, 950-956 | 902-928, 2400-2483.5 | 470-510, 902-928 863-870 | 902-928, 2400-2483.5 | 902-928 others possible | 2400-2483.5 | 915MHz, 868MHz, 316-433MHz |  |
| Channel BW (-20dB) | > 250 |  | < 250 |  | < 100 |  | < 1000 | 50 kHz; TBD wider channels | 30 to 150 |
| Channel spacing kHz | ≥ 250 | 300, 100, 400, 400 | 300 | 200, 250, 300 | 500, 1000 | 400 | 1000 | 50kHz 300kHz | ≤ 400 |
| Modulation | GMSK | GFSK | MSK | 2 & 4-GFSK | FSK | FSK |  | GFSK | FSK |
| FEC | R½ convol. | None Specified | None Specified | Block Parity SECDED | None Specified | None Specified | None Specified | BCH (31,21) rate 1/3 |  |
| Frequency Hopping |  | MAC | MAC | MAC | MAC | MAC |  | PHY |  |
| PHY frame structure: |  |  |  |  |  |  |  |  |  |
| MAX payload | > 1500 | 2047 | 2047 | 2047 | 1024 | > 1500 |  | > 1500 |  |
| SHR | 32-bit pre + SFD | 64-bit pre + 16-bit SFD | Variable pre + 16-bit SFD | Preamble + SFD | 32bpre+16b 32b SFD | Variable pre +16 bit SFD |  |  |  |
| CRC |  | CRC-32,16 | CRC-32 | PCB |  | CRC-16 |  |  |  |
| Data Whitening | 8-bit LFSR | 9-bit LFSR | 8-bit LFSR variable seed | Yes | None Specified | Upper Layer |  | Yes (TBD) |  |
| Data rate(s), kbps | 50, 100 | 40, 100 | 100 | 40,75, 50, 100, TBD | ≤ 114 | 9.6kbps – 300kbps |  | 19.2 | 20 to 160 |
| Symbol / chip rate | 100 kcps | 40, 100 ksps | 100 sps |  |  |  |  |  |  |
| Transmit Power or PSD | 50mW, 500mW | up to 1W | up to 1 W | Up to 1W | Up to 1W | up to 0.25W | As allowed | 10mW- 0.5W |  |
| TX Power Control |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
| Interference detection | LBT |  | MAC and/or Higher layer |  | MAC |  |  | Higher layer | MAC,HL |
| Link Quality |  |  | RSSI, NHL |  | RSSI, MAC |  |  | RSSI |  |
| Reliability enhancing features/methods | Whitening, FEC, AFA | Whitening, Freq Agility | Hopping, CRC-32 | Freq Agility, FEC | Freq Agility | Freq Agility | Freq Agility | interleaving + FEC | MAC, HL |
| Co-existence features | LBT, AFA | Freq Agility, LDC | Freq Agility, LDC | Freq Agility | Freq Agility | Freq Agility | Channel Plan | Freq Agility, TX power ctrl |  |
| SoP support features | Channel diversity | channel diversity | channel diversity | Channel Plan | channel diversity | Split band into sub (2) | Freq Agility | channel diversity |  |

Table 2: DSSS

|  | D1 | D2 | D3 | D4 | D5 | D6 |
| --- | --- | --- | --- | --- | --- | --- |
| PHY Parameter | MultiRate | AFDM | Flex-DSS | 2Rate | DSSSNANC | Dynamic DSSS |
| Operating band (MHz) | 860MHz, | 2400 | same as 15.4 DSSS | 2400 | 15.4-2006 | 902MHz and 2.4GHz |
| Channel BW (-20dB) |  | 15.4-2006 | 15.4-2006 | 15.4-2006 | 15.4-2006 | 0.5MHz, 2MHz, 5MHz |
| Channel spacing kHz | 600 | 15.4-2006 | 15.4-2006 | 15.4-2006 | 15.4-2006 |  |
| Modulation | GMSK |  | O-QPSK | O-QPSK | 15.4-2006 | D-BPSK |
| FEC | block code (4,n) |  | None | None | 15.4-2006 | Encode rate 1/2 |
| Frequency Hopping | MAC | None | None | None | None | MAC |
| PHY frame structure: |  |  |  |  |  |  |
| MAX payload | > 1500 |  | 1500 |  | 2047 | > 1500 |
| SHR | Variable pre (dynamically) + SFD |  |  |  | 15.4-2006 |  |
| CRC |  |  |  | 16 or 32-CRC | 15.4-2006 | CRC-32 |
| Data Whitening | 8-bit |  |  | None | None |  |
| Data rate(s), kbps | 400, 200, 100, 50, 25, 12.5, 6.25 |  | up to 1Mbps | 250, 250<DR<750 | 15.4-2006 | Variable |
| Symbol / chip rate | 400 kcps multiple- spreading factor |  | Variable / 2Mcps | Variable / 2Mcps | 15.4-2006 | Variable |
| Transmit Power or PSD | up to 1W |  |  |  | 15.4-2006 | up to 1 W |
| TX Power Control |  |  | 8dm/2khz |  | 15.4-2006 |  |
| Interference detection |  |  | Optional |  | 15.4-2006 | Yes |
| Link Quality |  |  | MAC |  | 15.4-2006 |  |
| Reliability enhancing features/methods |  |  | RSSI, NHL |  | 15.4-2006 | RSSI |
| Co-existence features |  |  |  |  | 15.4-2006 | Encode+interleave |
| SoP support features | Freq agility Channel diversity |  | 15.4 |  | 15.4-2006 | Random user code delay, TX power control, Gold code diversity |

Table 3: OFDM

|  | O1 | O2 | O3 | O4 |
| --- | --- | --- | --- | --- |
| PHY Parameter | MB-OFDM-WS | A-OFDM | OFDM-LG | OFDM-M |
| Operating band (MHz) | 470~698 | 700, 800, 900, 2400 | Multiple < 1GHz, 2.4GHz |  |
| Channel BW (-20dB) | 960 signal 7.5 per carrier | 243.75 signal 18.75 per carrier | 100, 200, 300 |  |
| Channel spacing kHz | 1000 | 300 | 100 MAC control for BW>100 |  |
| Modulation | QPSK | DPSK, pi/4 DQPSK | OFDM-BPSK, OFDM-QPSK, OFDM-16QAM, OFDM-64QAM | 64-QAM |
| FEC | Convolutional w/puncturing (rate 1/3, 1/4) | Convolutional w/puncturing (rate 1/8 to 2/3) | yes | Convolutional |
| Frequency Hopping | No | MAC | MAC | No |
| PHY frame structure: |  |  |  |  |
| MAX payload |  | 2047 | 2047 |  |
| SHR |  | variable len pre + 3 barker SFDs | TBD |  |
| CRC |  | CRC-32 | CRC-32 |  |
| Data Whitening |  | Yes | no |  |
| Data rate(s), kbps | 100,200,450,900 | 360,240,180,120,90,45,22.5 | 33kbps to 1Mbps | 288 to 1152 |
| Symbol / chip rate |  |  | Variable | variable |
| Transmit Power or PSD | up to 4W |  | up to 1 W |  |
| TX Power Control |  |  | NA |  |
| Interference detection |  |  | Yes |  |
| Link Quality | Database lookup |  | MAC and/or Higher layer |  |
| Reliability enhancing features/methods |  |  | RSSI, MAC (ACK success rate) |  |
| Co-existence features | Freq and time domain spreading | Freq agility, FEC, redundancy, cyclic prefix, pilot tomes | Frequency Hopping, CRC-32, FEC, channel equalization, bit redundancy, cyclic prefix, pilot tones |  |
| SoP support features | Coordinated usage | channel diversity, LDC | channel diversity, LDC |  |
| SoP support features |  |  | Channel diversity, CRC-32 seed, channel skipping/band splitting, hopping pattern seed |  |