IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | OFDM option for TG4g |  |
| Date Submitted | 31 March, 2009 |  |
| Source | Emmanuel Monnerie Landis+Gyr 30000 Mill Creek Avenue Alpharetta, GA 30024 USA Steve Shearer Wireless Consultant 3655 Bernal Avenue Pleasanton CA 94566 USA Shusaku Shimada Yokogawa Electric Corporation 2-9-32 Nakacho-town Musashino-city Tokyo, 180-8750 Japan | Voice: +1 678 258 1695 Fax: E-mail: Emmanuel . Monnerie @ landisgyr . com Voice: +1 905 997-0576 Fax: E-mail: shearer_inc @ yahoo . com Voice: Fax: E-mail: shusaku @ ieee . org |
| Re: | [802.15.4g] TG4g Call for Final Proposals, 31 March, 2009 |  |
| Abstract | This document provides specific OFDM PHY options for TG4g. |  |
| Purpose | This document supports the cross comparison between various proposals and may be use to support prospective options. Please note that this document does not necessarily represent the entirety of any new proposal, particularly if some merging occurs. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

OFDM PHY parameters

| PHY Parameter | OFDM proposal |
| --- | --- |
| Operating band | Multiple < 1GHz, 2.4GHz |
| Channel BW | 3 modes of operation: 100kHz, 200kHz and 300kHz |
| Channel spacing | Nominal spacing: 100kHz. Channel skipping controlled by MAC when BW>100kHz |
| Modulation | OFDM-BPSK, OFDM-QPSK, OFDM-16QAM, OFDM-64QAM |
| FEC | yes |
| Frequency Hopping | Pseudo Random Hopping Pattern defined by a seed |
| PHY frame structure: |  |
| MAX payload | 2047 |
| SHR | TBD |
| CRC | CRC-32 |
| Data Whitening | no |
| Data rate(s) | 33kbps to 1Mbps |
| Symbol / chip rate | Depends on number of tones N and bandwidth BW (BW=100kHz, 200kHz or 300kHz): 0.75*BW/N |
| Transmit Power | up to 1 W |
| PSD | NA |
| TX Power Control | MAC and/or Higher layer |
| Chan availability (interference detection) | MAC and/or Higher layer |
| Link Quality | RSSI, MAC (ACK success rate) |
| Reliability enhancing features/methods | Frequency Hopping, CRC-32, FEC, channel equalization, bit redundancy, cyclic prefix, pilot tones |
| Co-existence features | Hopping, LDC |
| SoP support features | Channel diversity, CRC-32 seed, channel skipping/band splitting, hopping pattern seed |

References

| Document | Title | Author |
| --- | --- | --- |
| 15-08-0109-02-004d-p802-15-4-d-proposal-ww-bpsk-with-afa-provisioning.pdf | p802-15-4-d-proposal-ww-bpsk-with-afa-provisioning | Shusaku Shimada (Yokogawa Co.) |
| 15-09-0114-01-004g-trends-in-sun-capacity.ppt | Trends in SUN capacity | Emmanuel Monnerie (Landis+Gyr) |
| 15-09-0126-01-004g-signal-processing-options.ppt | Signal Processing options | Steve Shearer (self) |
| 15-09-0258-00-004g-march-proposal-xref.doc | March Proposal Xref | Ben Rolfe (BCA) |