IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Overhead Reduction Subgroup Conference Call Minutes |  |
| Date | April 2, 2009 |  |
| Source | René Struik Certicom Corp. 5520 Explorer Drive, 4th Floor Mississauga, ON L4W 5L1 | E-mail: rstruik@certicom.com Phone: +1 (905) 501-6083 Fax: +1 (905) 507-4230 |
| Re: | Security and Efficiency Enhancements for IEEE 802.15.4e |  |
| Abstract | Minutes of conference calls related to security and efficiency enhancements proposed for 802.15.4e (proposals 08/828, 08/848, 08/849, 09/233) |  |
| Purpose | Security and efficiency enhancements of IEEE 802.15.4-2006 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

IEEE 802.15.4e Overhead Reduction Subgroup conference call, April 2, 2009

1. Attn

Liang Li, Kris Pister, René Struik

2. Schedule towards Montreal

There are five full weeks till the upcoming IEEE 802 meeting, Montreal, Quebec, May 9-13, 2009. Objectives are as follows:

- Provide sufficient editorial detail on how to implement overhead reduction proposal, pertaining to (a) formatting issues; (b) behavioral and processing details.

- Coordinate this effort with other proposals, so as to unify/harmonize efforts to extent possible.

3. Discussion Frame Control Field format proposal (draft 09/233r2)

- Discussion of short Frame Control Field (1-octet):

Changes to 1-octet short FCF as suggested in Michael Bahr's Factory Automation proposal (08/572r0), so as to make this short FCF (sFCF) a more application-independent optimization, while accommodating several elements not present in that proposal, such as indications as to optional security, versioning, and pending frames. Intention is that with sFCF, addressing fields are always muted.

Notes: Participants recognized there has been no need to enlarge the set of frame types between 802.15.4-2003 and 802.15.4-2006, nor is this anticipated with current 802.15.4e proposals or with anticipated cases for 802.15nan (802.15.4g). Nevertheless, should this need arise, one can use bit b6 with the proposed sFCF to accommodate enlargement of the set of frame types.

- Discussion of techniques for muting addressing fields:

Muting of destination and source addressing fields not sent over the air is indicated by using the currently (with 802.15.4-2006) reserved value of 0x01 with the destination and source addressing modes.

- Discussion of versioning:

Future versions of the standard (i.e., beyond 802.15.4-2006) are indicated by a "shifted" versioning field, thus allowing versioning info to be contained in the leftmost octet of the FCF and, thereby, also to be available if one uses the 1-octet sFCF. (With 802.15.4-2006, versioning info was contained in b12-b13 of the FCF, i.e., in the rightmost octet).

Participants expressed unanimous consent on FCF proposal and suggested posting the document to the 802.15.4e email reflector, to allow wider discussion.

4. Discussion of reduction of other MAC overhead

A short discussion ensued as to what minimum header information is necessary with the secured ACK mechanism, e.g., can one always purge the DSN field present in the 802.15.4-2006 ACK. More generally than with ACK, optimistically, intention is that if one uses a sFCF, one would go all the way and suppress other MAC header information as well. Nevertheless, this seemed to require a little bit more study, with various options and impact on behavior clearly spelled out (so as to ensure stable behavior in all cases). Moreover, various proposals make different assumptions on latency between received frame and corresponding ACK or cluster ACKs in a "group ACK". Idea here is to facilitate the underlying desired behavior, while making sure one can use streamlined parsing and processing (i.e., a single security processing section and not one per application domain).

5. Next meeting

Next conference call will be Thu April 16, 2009. {Note: the current conf call times coincide with the 802.15.4g (nan) group calls. New collision avoidance time to be investigated…}

6. Action items

(RS) Post FCF proposal (09/233r2). DONE.

(RS) Quantify impact of imposing interdependencies between overhead reduction techniques (e.g., should sFCF imply that one mutes DSN field, addressing fields, and auxiliary security field in its entirety? What is impact on robustness, failure recovery? flexibility?). Cf. also Item #4 above.

{Added after conf call}

Welcome to Michael Bahr to the Overhead Reduction Subgroup!