IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | MAC Enhancements for Time Slotted Channel Hopping |  |
| Date Submitted | [14 May 2009] |  |
| Source | [Chol Su Kang] [Kuor Hsin Chang] [Rick Enns] [Kris Pister] [Ludwig Winkel] [Clint Powell] [Jose Gutierrez] | Voice: [ ] Fax: [ ] E-mail: [ ckang@dustnetworks.com ] [ kuor-hsin.chang@freescale.com ] [ enns@stanfordalumni.org ] [ pister@eecs.berkeley.edu ] [ cpowell@ieee.org] [ Jose.Gutierrez@emerson.com] [ ludwig.winkel@siemens.com ] |
| Re: | [Task Group 15.4e: Enhancements to the 802.15.4 MAC] |  |
| Abstract | This document describes the Time Slotted Channel Hopping extension to the current IEEE802.15.4-2006 MAC. |  |
| Purpose | The purpose of this document is to specify the Time Slotted Channel Hopping extension to enhance the current MAC in IEEE802.15.4-2006 standard. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

New MAC service primitives

To be appended to Table 46

| Name | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| MLME-SET-SLOTFRAME | 7.1.17-A.1 |  |  | 7.1.17-A.2 |
| MLME-SET-LINK | 7.1.17-B.1 |  |  | 7.1.17-B.3 |
| MLME-TSCH-MODE | 7.1.17-C.1 |  |  | 7.1.17-C.2 |
| MLME-LISTEN | 7.1.17-D.1 | 7.1.17-E.2 |  | 7.1.17-D.2 |
| MLME-ADVERTISE | 7.1.17-E.1 | 7.1.17-E.2 |  | 7.1.17-E.3 |
| MLME-KEEP-ALIVE | 7.1.17-F.1 |  |  | 7.1.17-F.2 |
| MLME-JOIN | 7.1.17-G.1 | 7.1.17-G.2 |  | 7.1.17-G.3 |
| MLME-ACTIVATE | 7.1.17-H.1 | 7.1.17-H.2 |  | 7.1.17-H.3 |
| MLME-DISCONNECT | 7.1.17-I.1 | 7.1.17-I.2 |  | 7.1.17-I.3 |

Figure 40.A-Message sequence chart for TSCH procedure to find an advertising device

### MLME-SET-SLOTFRAME

#### MLME-SET-SLOTFRAME.request

The MLME-SET-SLOTFRAME.request primitive is used to add, delete, or change a slotframe at the MAC layer.

##### Semantics

The semantics of the MLME-SET-SLOTFRAME.request primitive is as follows:

MLME-SET-SLOTFRAME.request	(

slotframeId,

operation,

size,

channelPage,

channelMap,

activeFlag

)

The following table specifies parameters for the MLME-SET-SLOTFRAME.request primitive.

- MLME-SET-SLOTFRAME.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| slotframeId | Integer | 0x00-0xff | Unique identifier of the slotframe. |
| operation | Enumeration | ADD DELETE MODIFY | Operation to perform on the slotframe. |
| size | Integer | 0x0000-0xffff | Number of slots in the new frame. |
| channelPage | Integer | Selected from the available channel pages supported by the PHY (see 6.1.2) | Channel page supported by PHY. |
| channelMap | Bitmap | Array of bits | Indicating which frequency channels in the channel page are to be used for channel hopping. 27-bit bit field for Channel Page 0, 1, and 2 |
| activeFlag | Enumeration | TRUE FALSE | Frame is active. Frame is not active. |

##### When generated

An MLME-SET-SLOTFRAME.request is generated by the device management layer and issued to the MLME to create, delete, or update a frame on the MAC layer.

##### Effect on receipt

On receipt of an MLME-SET-SLOTFRAME.request, the MLME shall verify the parameters passed with the primitive. If the requested operation is ADD, the MLME shall attempt to add an entry into the macSlotframeTable. If the operation is MODIFY, it shall attempt to update an existing slotframe record in the table. If the operation is DELETE, all parameters except slotframeId and operation shall be ignored, and the slotframe record must be deleted from the macSlotFrameTable. If there are links in the slotframe that is being deleted, the links shall be deleted from the MAC layer. If the device is in the middle of using a link in the slotframe that is being updated or deleted, the update should be postponed until after the link operation completes.

#### MLME-SET-SLOTFRAME.confirm

The MLME-SET-SLOTFRAME.confirm primitive reports the results of the MLME-SET-SLOTFRAME.request command.

##### Semantics

The semantics of the MLME-SET-SLOTFRAME.confirm primitive is as follows:

MLME-SET-SLOTFRAME.confirm	(

slotframeId,

operation,

status

)

The following table specifies parameters for the MLME-SET-SLOTFRAME.confirm primitive.

- MLME-SET-SLOTFRAME.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| slotframeId | Integer | 0x00-0xff | Unique identifier of the slotframe to be added, modified, or deleted. |
| operation | Enumeration | ADD DELETE MODIFY | Operation to perform on the slotframe. |
| status | Enumeration | SUCCESS INVALID_PARAMETER SLOTFRAME_NOT_FOUND MAX_SLOTFRAMES_EXCEEDED | Results of the MLME-SET-SLOTFRAME.request command. |

##### When generated

The MLME-SET-SLOTFRAME.confirm primitive is generated by the MLME when the MLME-SET-SLOTFRAME.request is completed.

If any of the arguments fail a range check, the status shall be INVALID_PARAMETER. If a new slotframe is being added and the macSlotFrameTable is already full, the status shall be MAX_SLOTFRAMES_EXCEEDED. If an update or deletion is being requested and the corresponding slotframe cannot be found, the status shall be SLOTFRAME_NOT_FOUND.

##### Effect on receipt

On receipt of a MLME-SET-SLOTFRAME.confirm primitive, the device management application is notified of the status of its corresponding MLME-SET-SLOTFRAME.request.

### MLME-SET-LINK

#### MLME-SET-LINK.request

The MLME-SET-LINK.request primitive requests to add a new link, modify or delete an existing link at the MAC layer. The operationType parameter indicates whether the MLME-SET-LINK operation is to add or to delete a link.

#### Semantics

The semantics of the MLME-SET-LINK.request primitive is as follows:

MLME-SET-LINK.request to add a link	(

operationType (ADD_LINK or MODIFY_LINK),

linkHandle,

slotframeId,

timeslot,

chanOffset,

linkOptions,

linkType,

nodeAddr

)

Or

MLME-SET-LINK.request to delete a link	(

operationType (DELETE_LINK),

linkHandle

)

The following table specifies parameters for the MLME-SET-LINK.request primitive with the ADD_LINK or MODIFY_LINK operationType.

- MLME-SET-LINK.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| operationType | Enumeration | ADD_LINK, MODIFY_LINK, DELETE_LINK | Type of link management operation to be performed. |
| linkHandle | Integer | 0x00–0xFF | Unique identifier for the link. |
| slotframeId | Integer | 0x00-0xFF | Slotframe ID of the link to be added. |
| timeslot | Integer | 0x0000-0xFFFF | Timeslot of the link to be added. |
| chanOffset | Integer | 0x01-0xFF | Channel offset of the link. |
| linkOptions | Bitmap | b000 – b111 | b001 = Transmit. b010 = Receive. b100 = Shared. |
| linkType | Enumeration | NORMAL ADVERTISING | Type of link. |
| nodeAddr | Integer | 0x0000-0xffff | Address of neighbor connected to the link. 0xffff means the broadcasting to every node. |

##### When generated

When operationType=ADD_LINK or MODIFY_LINK:

MLME-SET-LINK.request primitive is generated by the device management layer to add a link or to modify an existing link in a slotframe.

When operationType=DELETE_LINK:

MLME-SET-LINK.request primitive is generated by the device management layer to delete an existing link at the MAC layer.

##### Effect on receipt

When operationType=ADD_LINK or MODIFY_LINK:

On receipt of the MLME-SET-LINK.request, the MAC layer shall attempt to add the indicated link to the macLinkTable and add the new neighbor to its neighbor table, if needed. Upon completion, the result of the operation must be reported through the corresponding MLME-SET-LINK.confirm primitive.

When operationType=DELETE_LINK:

On receipt of the MLME-SET-LINK request the device shall attempt to remove the link from the macLinkTable. If the link is currently in use, the deletion shall be postponed until after the link operation completes.

#### MLME-SET-LINK.confirm

The SET-LINK.confirm primitive indicates the result of add, modify or delete link operation.

##### Semantics

The semantics of the MLME-SET-LINK.confirm primitive is as follows:

MLME-SET-LINK.confirm	(

status,

linkHandle

)

The following table specifies parameters for the MLME-SET-LINK.confirm primitive.

- MLME-SET-LINK.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS INVALID_PARAMETER UNKNOWN_SLOTFRAME MAX_LINKS_EXCEEDED MAX_NEIGHBORS_EXCEEDED SUCCESS LINK_NOT_FOUND | Result of the add or modify link operation. Result of the delete link operation. |
| linkHandle | Integer | 0x00 – 0xFF | Unique identifier for the link. |

##### When generated

The MLME-SET-LINK.confirm is generated as a result of the MLME-SET-LINK.request operation.

##### Effect on receipt

The layer that issued the MLME-SET-LINK.request to the MAC may process the result of the operation. The status of the primitive shall indicate SUCCESS if the operation completed successfully. Otherwise, the status indicates the cause of the failure.

### MLME-TSCH-MODE

#### MLME-TSCH-MODE.request

The MLME-TSCH-MODE.request puts the MAC into TSCH mode, or out of TSCH mode.

##### Semantics

The semantics of the MLME-TSCH-MODE.request primitive is as follows:

MLME-TSCH-MODE.request	(

modeSwtich

)

The following table specifies parameters for the MLME-TSCH-MODE.request primitive.

- MLME-TSCH-MODE.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| modeSwitch | Enumeration | ON, OFF | Target mode. This mode indicates whether TSCH mode should be started or stopped. |

##### When generated

The MLME-TSCH-MODE.request may be generated by the higher layer after the device has received advertisements from the network and is synchronized to a network.

##### Effect on receipt

Upon receipt of the request, the MAC shall start operating its TSCH state machine using slotframes and links already contained in its database. To successfully complete this request the device must already be synchronized to a network.

#### MLME-TSCH-MODE.confirm

The MLME-TSCH-MODE.confirm primitive reports the result of the MLME-TSCH-MODE.request primitive.

##### Semantics

The semantics of the MLME-TSCH-MODE.confirm primitive is as follows:

MLME-TSCH-MODE.confirm	(

modeSwitch,

status

)

The following table specifies parameters for the MLME-TSCH-MODE.confirm primitive.

- MLME-TSCH-MODE.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| modeSwitch | Enumeration | ON, OFF | Target mode. This mode indicates whether this confirmation is due to TSCH mode ON request or OFF request. |
| status | Enumeration | SUCCESS NO_SYNC |  |

##### When generated

The MLME-TSCH-MODE.confirm is generated by the MAC layer to indicate completion of the corresponding request. If the corresponding request was to turn on the TSCH-MODE, but the MAC layer has not been synchronized to a network, the status shall be NO_SYNC. Otherwise, the status shall be SUCCESS.

If the corresponding request was to turn off the TSCH-MODE, the status shall be SUCCESS, and the MAC layer will stop the TSCH-MODE operation.

##### Effect on receipt

The higher layer may use the confirmation to process the result of MLME-TSCH-MODE.request.

### MLME-LISTEN

#### MLME-LISTEN.request

##### Semantics

The semantics of the MLME-LISTEN.request primitive is as follows:

MLME-LISTEN.request	(

time,

numPageChannel,

pageChannels[]

)

The following table specifies parameters for the MLME-LISTEN.request primitive.

- MLME-LISTEN.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| time | Integer | 0x0000 – 0xFFFF | The amount of time (one millisecond units) to stay on each channel. 0x0000 indicates that the device stays on the first channel. |
| numPageChannel | Integer | 0x01-0xFF | The number of page channel descriptors in the page channels array. |
| pageChannelsDes[] | Table 77-H | Table 77-H | Array of page channel descriptor. See Table 77-H for the format of page channel descriptor. |

- MLME-LISTEN.request pageChannelDesc parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| channelPageId | Integer | Selected from the available channel pages supported by the PHY (see 6.1.2) | Channel page ID. |
| numChannel | Integer | 0x01-0xFF | The number of channels in this channel page to be included in listening. |
| Channels[] | Array of Channel | Table 2. | The array of channels on which to listen. See Table 2 for the valid range of channels in each channel page. |

##### When generated

The MLME-LISTEN.request shall be generated by the next higher layer to initiate the search for a TSCH network.

##### Effect on receipt

Upon receipt of the request the MAC layer shall activate the radio on the indicated channel and wait for an Advertisement command. Valid Advertisement command frames received in this state shall result in the generation of MLME-ADVERTISE.indication. All other frames shall be dropped. The MAC shall stay in the listening state until the indicated time expires, or a MLME-TSCH-MODE.request is received. The higher layer selects the advertiser and the network before setting the slotframe, link(s), and TSCH mode. Advertisements continue to be received and passed on to the higher layer after leaving the listen state.

#### MLME-LISTEN.confirm

##### Semantics

The semantics of the MLME-LISTEN.confirm primitive is as follows:

MLME-LISTEN.confirm	(

status

)

The following table specifies parameters for the MLME-LISTEN.confirm primitive.

- MLME-LISTEN.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS INVALID_PARAMETER |  |

##### When generated

The MAC layer shall generate MLME-LISTEN.confirm when it completes the listen operation started by MLME-LISTEN.request.

##### Effect on receipt

On receipt of the primitive, the higher layer may continue with its joining state machine.

### MLME-ADVERTISE

#### MLME-ADVERTISE.request

##### Semantics

The semantics of the MLME-ADVERTISE.request primitive is as follows:

MLME-ADVERTISE.request	(

advertiseInterval,

channelPage,

channelMap,

hoppingSequenceId,

timeslotTemplateId,

securityLevel,

joinPriority,

numSlotframe,

slotframes[]

)

The following table specifies parameters for the MLME-ADVERTISE.request primitive.

- MLME-ADVERTISE.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| advertiseInterval | Integer | 0x0000 – 0xFFFF | Interval specifying the transmission of the Advertisement command. |
| channelPage | Integer | Selected from the available channel pages supported by the PHY (see 6.1.2) | Channel page supported by PHY. |
| channelMap | Bitmap | Array of bits | Map of channels to be included in the Advertisement command. |
| hoppingSequenceId | Integer | 0x0 – 0xF | ID of hopping sequence used. |
| timeslotTemplateId | Integer | 0x0 – 0xF | ID of timeslot template used. |
| securityLevel | Enumeration | Table 95 | Security level in the Advertisement command. See Table 95 in IEEE802.15.4-2006. |
| joinPriority | Integer | 0x00 – 0xFF | Join priority to be indicated in the Advertisement command. |
| numSlotframe | Integer | 0x0 – 0xF | Number of slotframes to be indicated in the Advertisement command. |
| Slotframes[] | Table 77-K | Table 77-K | See Table 77-K. |

- MLME-ADVERTISE.request Slotframe parameters (per slotframe)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| slotframeId | Integer | 0x00 – 0xFF | Slotframe ID. |
| slotframeSize | Integer | 0x00 – 0xFFFF | Slotframe size. |
| numLink | Integer | 0x0 – 0xF | Number of links for the specified slotframe to be indicated in the Advertisement command. |
| Links | Table 77-L | Table 77-L | See Table 77-L. |

- MLME-ADVERTISE.request Link parameters (per link)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| timeslot | Integer | 0x0000 – 0xFFFF | Timeslot. |
| chanOffset | Integer | 0x00 – 0xFF | Channel offset. |
| linkOption | Enumeration | TX RX SHARED_TX | Option of the link. |

##### When generated

The next higher layer requests the MAC layer to start sending Advertisement command frames using MLME-ADVERTISE.request so that new nodes can find the network and this device.

##### Effect on receipt

Upon receipt of the request the MAC layer shall send the Advertisement command frame on the first available TX link. Whenever the time specified in AdvertiseInterval lapses from the previous transmission of Advertisement command frame, the MAC layer shall repeat the Advertisement command frame on next TX link available. The remaining parameters specify the information to be included in the Advertisement command frame.

#### ADVERTISE.indication

The MLME-ADVERTISE.indication indicates that a device received an Advertisement command frame.

##### Semantics

The semantics of the MLME-ADVERTISE.indication primitive is as follows:

MLME-ADVERTISE.indication	(

PANId,

timingInformation,

channelPage,

channelMap,

hoppingSequenceId,

timeslotTemplateId,

securityLevel,

joinPriority,

linkQuality,

numSlotframes,

slotframes[]

)

The following table specifies parameters for the MLME-ADVERTISE.indication primitive.

- MLME-ADVERTISE.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PANId | Integer | 0x0000 – 0xFFFF | The PAN identifier indicated in the Advertisement command. |
| timingInformation |  |  | The time information (absolute slot number) of the timeslot in which the Advertisement command was received. |
| channelPage | Integer | Selected from the available channel pages supported by the PHY (see 6.1.2) | Channel page. |
| channelMap | Bitmap | Array of bits | Bit map of channels. |
| hoppingSequenceId | Integer | 0x0 – 0xF | ID of hopping sequence used. |
| timeslotTemplateId | Integer | 0x0 – 0xF | ID of timeslot template used. |
| securityLevel | Enumeration | Table 95 | Security level in advertisement packet See Table 95 in IEEE802.15.4-2006. |
| joinPriority | Integer | 0x00 – 0xFF | Join priority indicated in advertisement. |
| linkQuality | Integer | 0x00 – 0xFF | Link quality indicated in the frame by the PHY layer. |
| numSlotframes | Integer | 0x0 – 0xF | Number of slotframes indicated in the Advertisement command received. |
| slotframes[] | Table 77-K | Table 77-K | See Table 77-K. |

##### When generated

The MLME-ADVERTISE.indication shall be generated when an Advertisement command frame has been received by the device. Upon receiving a valid Advertisement command, the device shall be synchronized to the network and ready to enable the TSCH-MODE if requested by the higher layer.

##### Effect on receipt

The higher layer may wait and record more than one advertisement and then select the desired advertising device before configuring the superframe(s) and link(s) and before enabling TSCH-MODE. After joining a TSCH network, the high layer uses the indication to collect the list of neighbors and information about neighbors.

#### MLME-ADVERTISE.confirm

##### Semantics

The semantics of the MLME-ADVERTISE.confirm primitive is as follows:

MLME-ADVERTISE.confirm	(

status

)

The following table specifies parameters for the MLME-ADVERTISE.confirm primitive.

- MLME-ADVERTISE.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS INVALID_PARAMETER |  |

##### When generated

The MAC layer shall generate MLME-ADVERTISE.confirm when it starts sending the Advertisement command.

##### Effect on receipt

On receipt of the primitive, the higher layer may expect that it will receive the Join command on any of the links provided in the Advertisement command.

### MLME-KEEP-ALIVE

#### MLME-KEEP-ALIVE.request

##### Semantics

The semantics of the MLME-KEEP-ALIVE.request primitive is as follows:

MLME-KEEP-ALIVE.request	(

dstAddr,

linkHandle,

period

)

The following table specifies parameters for the MLME-KEEP-ALIVE.request primitive.

- MLME-KEEP-ALIVE.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| dstAddr | Integer | 0x0000 - 0xFFFF | Address of neighbor device to maintain the timing. |
| linkHandle | Integer | 0x00 – 0xFF | Unique identifier for the link to be used to send out the Keep-Alive command frame. |
| period | Integer | 0x0001 – 0xFFFF | Duration of quiet time in seconds that a Keep-Alive command frame should be sent if no traffic is present. |

##### When generated

##### Effect on receipt

Upon receipt of the request, the MAC layer shall monitor the frame sent to the destination node specified in the dstAddr parameter. If no frame is sent to the destination node for any duration defined by the period parameter, the MAC shall send Keep-Alive command frame to the node using linkHandle.

#### MLME-KEEP-ALIVE.confirm

##### Semantics

The semantics of the MLME-KEEP-ALIVE.confirm primitive is as follows:

MLME-KEEP-ALIVE.confirm	(

status

)

The following table specifies parameters for the MLME-KEEP-ALIVE.confirm primitive.

- MLME-KEEP-ALIVE.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS INVALID_PARAMETER |  |

##### When generated

The MAC layer shall generate MLME-KEEP-ALIVE.confirm to acknowledge that it received MLME-KEEP-ALIVE request.

##### Effect on receipt

### MLME-JOIN

#### MLME-JOIN.request

##### Semantics

The semantics of the MLME-JOIN.request primitive is as follows:

MLME-JOIN.request	(

linkHandle,

securityInformation,

numNeighbors,

neighbors[]

)

The following table specifies parameters for the MLME-JOIN.request primitive.

- MLME-JOIN.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| linkHandle | Integer | 0x00 – 0xFF | Unique identifier for the link to be used to send out the Join command frame. |
| securityInformation | Table 77-R | Table 77-R | See Table 77-R for the detail. |
| numNeighbors | Integer | 0x0 – 0xF | Number of neighbors found by the joining device. |
| neighbors | Table 77-S | ‎‎Table 77-S | Neighbor information for the number of neighbors specified in numNeighbors. See Table 77-S for the definition of a neighbor. |

- MLME-JOIN.request securityInformation parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TBD | TBD | TBD | The securityInformation definition will be defined with Security sub-group. |

- MLME-JOIN.request neighbors parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| neighborId | Integer | 0x0000 – 0xFFFF | 16 bit address of neighbor. |
| RSSI | Integer | -128 to 127 | Received signal strength (in dBm) of frames received from the neighbor. |

##### When generated

Device management of a new device (or device who lost connection with the TSCH network) will invoke this service primitive to join the TSCH network.

##### Effect on receipt

Upon receipt of the request, the MAC layer shall send either a Join command frame or data frame containing a higher layer management packet requesting to join the network, using the linkHandle parameter. The content of the Join command frame will be formatted using the other parameters and the format of Join command frame is specified in Section 7.3.12. If a data frame with the higher layer management packet is used instead of a Join command frame, the content of the higher layer payload of the data frame containing the request to join the network is constructed using the other parameters. The explicit format of the higher layer payload is out of scope of this document.

#### MLME-JOIN.indication

##### Semantics

The semantics of the MLME-JOIN.indication primitive is as follows:

MLME-JOIN.indication	(

linkHandle,

newNodeAddr,

securityInformation,

numNeighbors,

neighbors[]

)

The following table specifies parameters for the MLME-JOIN.indication primitive.

- MLME-JOIN.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| linkHandle | Integer | 0x00 – 0xFF | Unique identifier for the link that Join command frame is received. |
| newNodeAddr | array of octets | 64-bit binary string | 64-bit long address of new device sending the Join command. |
| securityInformation | Table 77-Q | Table 77-Q | See Table 77-Q. |
| numNeighbors | Integer | 0x0 – 0xF | Number of neighbors reported by the joining device. |
| Neighbors | Table 77-S | Table 77-S | Neighbor information for the number of neighbors specified in numNeighbors. See Table 77-S for the definition of a neighbor in neighbors. |

##### When generated

MLME-JOIN.indication indicates the Device Management layer that the MAC layer has received a Join command frame from a new device attempting to join the TSCH network.

##### Effect on receipt

Upon receipt of the MLME-JOIN.indication, the Device Management layer shall invoke the device management procedure to transfer the join attempt of the new device to the Device Manager.

#### MLME-JOIN.confirm

##### Semantics

The semantics of the MLME-JOIN.confirm primitive is as follows:

MLME-JOIN.confirm	(

status

)

The following table specifies parameters for the MLME-JOIN.confirm primitive.

- MLME-JOIN.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS INVALID_PARAMETER |  |

##### When generated

The MAC layer shall generate MLME-JOIN.confirm to acknowledge that it received the MLME-JOIN.request primitive.

##### Effect on receipt

### MLME-ACTIVATE

#### MLME-ACTIVATE.request

##### Semantics

The semantics of the MLME-ACTIVATE.request primitive is as follows:

MLME-ACTIVATE.request	(

linkHandle,

securityInformation,

numLinks,

links[]

)

The following table specifies parameters for the MLME-ACTIVATE.request primitive.

- MLME-ACTIVATE.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| linkHandle | Integer | 0x00 – 0xFF | Unique identifier for the link to be used to send out the ACTIVATE command frame. |
| securityInformation | Table 77-W | Table 77-W | See Table 77-W for details. |
| numLinks | Integer | 0x0 – 0xF | Number of links allocated to the joining device. |
| links | Table 77-L | ‎‎ Table 77-L | Link information for links allocated to the joining device. See Table 77-L for the definition of a link. |

- MLME-ACTIVATE.request securityInformation parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TBD | TBD | TBD | The securityInformation definition will be defined with the Security sub-group. |

##### When generated

##### Effect on receipt

Upon receipt of the request, the MAC layer shall send either the Activate command frame to activate the new joining device, or a data frame containing a higher layer management packet to activate the new joining device. The MAC shall send the Activate command frame to the node using the linkHandle parameter. The content of the Activate command is formatted using the other parameters. If a data frame with a higher layer management packet is used instead of Activate command frame, the content of the higher layer payload to activate the network is constructed using the other parameters. The explicit format of the higher layer payload is out of scope of this document.

#### MLME-ACTIVATE.indication

##### Semantics

The semantics of the MLME-ACTIVATE.indication primitive is as follows:

MLME-ACTIVATE.indication	(

linkHandle,

securityInformation,

numLinks,

links[]

)

The following table specifies parameters for the MLME-ACTIVATE.indication primitive.

- MLME-ACTIVATE.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| linkHandle | Integer | 0x00 – 0xFF | Unique identifier for the link that ACTIVATE command frame was received. |
| securityInformation | Table 77-V | Table 77-V | See Table 77-V. |
| numLinks | Integer | 0x0 – 0xF | Number of links allocated to the joining device. |
| links | Table 77-STable 77-L | Table 77-STable 77-L | Link information for the links allocated to the joining device. See Table 77-L for the definition of a link. |

##### When generated

MLME-ACTIVATE.indication indicates the device management layer that the MAC layer has received an Activate command frame from the link indentified with the linkHandle parameter.

##### Effect on receipt

Upon receipt of the MLME-ACTIVATE.indication, the device management layer shall process the securityInformation received to set up secure connections. It also should activate links granted.

#### MLME-ACTIVATE.confirm

##### Semantics

The semantics of the MLME-ACTIVATE.confirm primitive is as follows:

MLME-ACTIVATE.confirm	(

status

)

The following table specifies parameters for the MLME-ACTIVATE.confirm primitive.

- MLME-ACTIVATE.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS INVALID_PARAMETER |  |

##### When generated

The MAC layer shall generate MLME-ACTIVATE.confirm to acknowledge that it received MLME-ACTIVATE.request.

##### Effect on receipt

Figure 40.B-Message sequence chart for join and activate procedures

### MLME-DISCONNECT

#### MLME-DISCONNECT.request

##### Semantics

The semantics of the MLME-DISCONNECT.request primitive is as follows:

MLME-DISCONNECT.request	(

)

##### When generated

MLME-DISCONNECT.request primitive is used to initiate the graceful disconnection from TSCH network.

##### Effect on receipt

Upon receipt of the request, the MAC layer shall send either the Disconnect command frame or a data frame containing a higher layer management packet to indicate that it is about to leave the TSCH network on the advertising links. After the MAC sends the DISCONNECT command frames for macDisconnectTime, it shall release all link resource.

#### MLME-DISCONNECT.indication

##### Semantics

The semantics of the MLME-DISCONNECT.indication primitive is as follows:

MLME-DISCONNECT.indication	(

srcAddress,

)

The following table specifies parameters for the MLME-DISCONNECT.indication primitive.

- MLME-DISCONNECT.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| srcAddress | Integer | 0x0000 – 0xFFFF | 16-bit short address of the neighbor node from which the DISCONNECT command frame was received. |

##### When generated

MLME-DISCONNECT.indication indicates to the device management layer that the MAC layer has received a Disconnect command frame from a neighbor node, the address of which is indicated by srcAddress.

##### Effect on receipt

Upon receipt of the MLME-DISCONNECT.indication, the device management layer shall process the disconnection of the neighbor from which the Disconnect command frame is received.

#### MLME-DISCONNECT.confirm

##### Semantics

The semantics of the MLME-DISCONNECT.confirm primitive is as follows:

MLME-DISCONNECT.confirm	(

status

)

The following table specifies parameters for the MLME-DISCONNECT.confirm primitive.

- MLME-DISCONNECT.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS |  |

##### When generated

The MAC layer shall generate MLME-DISCONNECT.confirm to acknowledge that it received MLME-DISCONNECT request.

##### Effect on receipt

Modified MAC Service Primitives

### 7.1.1.1 MCPS-DATA.request

The MCPS-DATA.request primitive requests the transfer of a data SPDU (i.e., MSDU) from a local SSCS entity to a single peer SSCS entity specified in any of the destination addresses in DstAddr and additionalDstAddr.

7.1.1.1.1 Semantics of the service primitive

The semantics of the MCPS-DATA.request primitive are as follow:

MCPS-DATA.request	 (

SrcAddrMode,

DstAddrMode,

DstPANId,

DstAddr,

msduLength,

msdu,

msduHandle,

TxOptions,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

numberOfAdditionalDstAddr

additionalDstAddr,

numberOfLinkHandles,

linkHandleList

)

Table 41 specifies parameters for the MCPS-DATA.request primitive.

Table 41-MCPS-DATA.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| DstAddrMode | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| DstPANId | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| DstAddr | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| msduLength | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| msdu | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| msduHandle | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| TxOptions | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| SecurityLevel | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| KeyIdMode | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| KeySource | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| KeyIndex | Table 41 | Table 41 | See Table 41 in IEEE802.15.4-2006. |
| numberOfAdditionalDstAddr | Integer | 0x00-0x04 | If the number of additionalDstAddr is zero, no additionalDstAddr will follow. |
| additionalDstAddr | List of DstAddr | 0x0000-0xffff for each DstAddr | One or more alternate destination addresses. The data SPDU should be transferred to the destination specified by either DstAddr or any of destinations in additionalDstAddr. |
| numberOfLinkHandles | Integer | 0x00-0x04 | If the number of Link Handles is zero, no Link Handle will follow. |
| linkHandleList | List of Integer | 0x00-0xFF for each linkHandle | Use any link specified in linkHandleList to transfer the data SPDU. |

7.1.1.1.2 When generated

The MCPS-DATA.request primitive is generated by a local SSCS entity when a data SPDU (i.e., MSDU) is

to be transferred to a peer SSCS entity specified in any of destination addresses in DstAddr and additionalDstAddr.

7.1.1.1.3 Effect on receipt

On receipt of the MCPS-DATA.request primitive, the MAC sublayer entity begins the transmission of the

supplied MSDU.

The MAC sublayer builds an MPDU to transmit from the supplied arguments. The flags in the

SrcAddrMode and DstAddrMode parameters correspond to the addressing subfields in the Frame Control

field (see 7.2.1.1) and are used to construct both the Frame Control and addressing fields of the MHR. If

both the SrcAddrMode and the DstAddrMode parameters are set to 0x00 (i.e., addressing fields omitted), the

MAC sublayer will issue the MCPS-DATA.confirm primitive with a status of INVALID_ADDRESS.

If the msduLength parameter is greater than aMaxMACSafePayloadSize, the MAC sublayer will set the

Frame Version subfield of the Frame Control field to one.

The TxOptions parameter indicates how the MAC sublayer data service transmits the supplied MSDU. If the

TxOptions parameter specifies that an acknowledged transmission is required, the Acknowledgment

Request subfield of the Frame Control field will be set to one (see 7.5.6.4).

If the TxOptions parameter specifies that a GTS transmission is required, the MAC sublayer will determine

whether it has a valid GTS (for GTS usage rules, see 7.5.7.3). If a valid GTS could not be found, the MAC

sublayer will issue the MCPS-DATA.confirm primitive with a status of INVALID_GTS. If a valid GTS was

found, the MAC sublayer will defer, if necessary, until the GTS. If the TxOptions parameter specifies that a

GTS transmission is not required, the MAC sublayer will transmit the MSDU using either slotted CSMACA

in the CAP for a beacon-enabled PAN or unslotted CSMA-CA for a nonbeacon-enabled PAN.

Specifying a GTS transmission in the TxOptions parameter overrides an indirect transmission request.

If the TxOptions parameter specifies that an indirect transmission is required and this primitive is received

by the MAC sublayer of a coordinator, the data frame is sent using indirect transmission, i.e., the data frame

is added to the list of pending transactions stored on the coordinator and extracted at the discretion of the

device concerned using the method described in 7.5.6.3. Transactions with a broadcast destination address

will be transmitted using the mechanism described in 7.2.1.1.3. Transactions with a unicast destination

address can then be extracted at the discretion of each device concerned using the method described in

7.5.6.3. If there is no capacity to store the transaction, the MAC sublayer will discard the MSDU and issue

the MCPS-DATA.confirm primitive with a status of TRANSACTION_OVERFLOW. If there is capacity to

store the transaction, the coordinator will add the information to the list. If the transaction is not handled

within macTransactionPersistenceTime, the transaction information will be discarded and the MAC

sublayer will issue the MCPS-DATA.confirm primitive with a status of TRANSACTION_EXPIRED. The

transaction handling procedure is described in 7.5.5. If the TxOptions parameter specifies that an indirect

transmission is required and if the device receiving this primitive is not a coordinator, the destination

address is not present, or the TxOptions parameter also specifies a GTS transmission, the indirect

transmission option will be ignored.

If the TxOptions parameter specifies that an indirect transmission is not required, the MAC sublayer will

transmit the MSDU using CSMA-CA either in the CAP for a beacon-enabled PAN or immediately for a

nonbeacon-enabled PAN. If the TxOptions parameter specifies that a direct transmission is required and the

MAC sublayer does not receive an acknowledgment from the recipient after macMaxFrameRetries

retransmissions (see 7.5.6.4), it will discard the MSDU and issue the MCPS-DATA.confirm primitive with a

status of NO_ACK.

If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for

this frame, the MAC sublayer will set the Security Enabled subfield of the Frame Control field to one. The

MAC sublayer will perform outgoing processing on the frame based on the DstAddr, SecurityLevel,

KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.5.8.2.1. If any error occurs during

outgoing frame processing, the MAC sublayer will discard the frame and issue the MCPS-DATA.confirm

primitive with the error status returned by outgoing frame processing.

If the requested transaction is too large to fit in the CAP or GTS, as appropriate, the MAC sublayer shall

discard the frame and issue the MCPS-DATA.confirm primitive with a status of FRAME_TOO_LONG.

If the transmission uses CSMA-CA and the CSMA-CA algorithm failed due to adverse conditions on the

channel, and the TxOptions parameter specifies that a direct transmission is required, the MAC

sublayer will discard the MSDU and issue the MCPS-DATA.confirm primitive with a status of

CHANNEL_ACCESS_FAILURE.

If the MPDU was successfully transmitted and, if requested, an acknowledgment was received, the MAC

sublayer will issue the MCPS-DATA.confirm primitive with a status of SUCCESS.

If any parameter in the MCPS-DATA.request primitive is not supported or is out of range, the MAC

sublayer will issue the MCPS-DATA.confirm primitive with a status of INVALID_PARAMETER.

If numberOfAdditionalDstAddr is not zero and the transmission to the first transfer attempt to destAddr fails, then MAC should transfer the data SPDU to any of the alternate destinations specified in additionalDstAddr using the linkHandle provided in linkHandleList. MAC should transfer the requested data SPDU on one of these links. MAC will select a link to transmit (or retransmit if ACK is not received) in earliest possible opportunity.

7.1.1.2 MCPS-DATA.confirm

The MCPS-DATA.confirm primitive reports the results of a request to transfer a data SPDU (MSDU) from

a local SSCS entity to a single peer SSCS entity.

7.1.1.2.1 Semantics of the service primitive

The semantics of the MCPS-DATA.confirm primitive are as follows:

MCPS-DATA.confirm			 (

msduHandle,

status,

Timestamp,

dstAddr,

linkHandle

)

Table 42 specifies the parameters for the MCPS-DATA.confirm primitive.

7.1.1.2.2 When generated

The MCPS-DATA.confirm primitive is generated by the MAC sublayer entity in response to an MCPS-DATA.request primitive. The MCPS-DATA.confirm primitive returns a status of either SUCCESS, indicating that the request to transmit was successful, or the appropriate error code. The status values are fully described in 7.1.1.1.3 and subclauses referenced by 7.1.1.1.3.

Table 42-MCPS-DATA.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| moduleHandle | Table 42 | Table 42 | See Table 42 in IEEE802.15.4-2006. |
| status | Table 42 | Table 42 | See Table 42 in IEEE802.15.4-2006. |
| Timestamp | Table 42 | Table 42 | See Table 42 in IEEE802.15.4-2006. |
| DstAddr | Device address | As specified by the DstAddrMode parameter of MCPS-DATA.request | Destination address to which the data SPDU was transferred. |
| linkHandle | Integer | 0x00-0xFF | Handle of the link that the data SPDU was transferred. |

7.1.1.2.3 Appropriate usage

On receipt of the MCPS-DATA.confirm primitive, the SSCS of the initiating device is notified of the result

of its request to transmit. If the transmission attempt was successful, the status parameter will be set to

SUCCESS. Otherwise, the status parameter will indicate the error.

If the transmission attempt was successful, DstAddr is set to the address of the destination to which the data SPDU was transferred, and linkHandle is set to the handle of the link on which the data SPDU was transferred.

New MAC command frame

This table entry should be appended to Table 82.

| Command frame identifier | Command name | RFD |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | TX | RX |  |
| 0x0a | Advertisement |  | X | 7.3.10 |
| 0x0b | Keep-Alive | X | X | 7.3.11 |
| 0x0c | Join | X |  | 7.3.12 |
| 0x0d | Activate |  | X | 7.3.13 |
| 0x0e | Disconnect | X | X | 7.3.14 |

### Advertisement command

| Octets: Variable (see 7.2.2.4) | 1 | 6 | 1 | 1 | 1 | 1 | 1 | variable | 1 | Variable | 0/4/8/ 16 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MHR | Command Frame Identifier (see table 82) | Timing Informa-tion | Security Control Field | Join Control | Timeslot Template and Hopping Sequence ID | Channel Page/Map Length | Channel Page | Channel Map | Number of Slot-frames | Slotframe Info. and Links (for Each Slotframe) | MIC | MFR |

The Advertisement command is used by FFDs to invite new devices into the network. When a device wishes to join a network, it shall use the information in Advertisement command frames to synchronize to the network and request an association.

- Figure 64-A Advertisement Command Format

#### 7.3.10.1 MHR field

The Source Addressing Mode subfield of the Frame Control field shall be set to three (64-bit extended addressing). The Destination Addressing Mode subfield shall be set to the broadcast address, i.e. 0xFFFF.

The Frame Pending subfield of the Frame Control field shall be set to zero and ignored upon receipt, and the Acknowledgment Request subfield shall be set to zero. The Source PAN Identifier field shall contain the PAN identifier of the node. The Source Address field shall contain the value of aExtendedAddress.

The Sequence Number subfield shall be set to the least significant byte of the absolute slot number.

#### 7.3.10.2 Command Frame Identifier field

The Type field shall be set to Advertisement (0x0a).

#### Timing Information field

The Timing Information field shall be set to the time information (i.e. Absolute Slot Number) of the timeslot being used for transmission of this command frame.

#### Security Control field

| Bit 4–7 | Bit 0–3 |
| --- | --- |
| Reserved | Security Level |

Figure 64-B Security Control field

The Security Level subfield should be set to the security level supported. The definition of the Security Level subfield can be found in Table 95 in Section 7.6.2.2.1.

#### Join Control field

| Bit 4–7 | 0–3 |
| --- | --- |
| Reserved | Join Priority |

- Figure 64-C Join Control field

The Join Priority subfield can be used by a joining device to decide which Network Devices to include in its Association Request if it hears advertisements from more than one device.

A lower value of join priority indicates that the device is a preferred one to connect to.

#### Timeslot Template and Hopping Sequence ID field

The Timeslot Template and Hopping Sequence ID field shall be set to the ID of the timeslot template and the ID of the hopping sequence used by the MAC. The timeslot templates and hopping sequences are defined in MAC PIB.

| Bit 4–7 | 0–3 |
| --- | --- |
| Hopping Sequence ID | Timeslot Template ID |

- Figure 64-C Timeslot Template and Hopping Sequence ID field

#### Channel Page/Map Length field

The Channel Page/Map field shall be set to the combined length of following channel page and channel map fields.

#### Channel Page field

The Channel Page field shall be set to the channel page of channels that the joining device shall use for its hopping sequence.

#### Channel Map field

The Channel Map field shall be set to the channel map of channels that the joining device shall use for its hopping sequence.

#### Number of Slotframes field

The Number of Slotframes field is set to the total number of slotframes for which information is being advertised in this command frame.

#### Slotframe Information and Links (for each slotframe) field

Slotframe Information and Links field is included for each slotframe. The format of Slotframe Information and Links field is depicted as follow:

| Octets: 1 | 2 | 1 | Variable |
| --- | --- | --- | --- |
| Slotframe ID | Slotframe Size | Number of Links | Link Info. for each Link |

- Figure 64-D Slotframe and Links field

Slotframe ID subfield

Slotframe ID shall be set to the ID that uniquely identifies the slotframe.

Slotframe Size subfield

Slotframe Size shall be set to the size of the slotframe in number of timeslots.

Number of Links subfield

The Number of Links subfield shall be set to the number of links that belong to the specific slotframe indicated in preceding slotframe ID.

Link Information (for each link) subfield

The Link Information subfield describes the attributes of each link. The format of Link Information subfield is depicted as follow:

| Octets: 2 | 1 | 1 |
| --- | --- | --- |
| Timeslot | Channel Offset | Link Option |

- Figure 64-E Link Information field

Timeslot subfield

The Timeslot subfield shall be set to the timeslot of this link.

Channel Offset Information subfield

The Channel Offset Information subfield shall be set to the channel offset of this link.

Link Option subfield

The Link Option subfield indicates whether this link is a TX link, an RX link, or a SHARED TX link. SHARED TX links can be used for a joining device to send its Join command. RX links are used for a new device to receive Advertisement commands. RX links can also be used for a joining device to receive its Activate command from the network. It is possible for one link to be used as both SHARED_TX and RX link.

#### MIC

The message integrity check of the Advertisement command frame.

### Keep-Alive command

The Keep-Alive command frame is used to maintain time synchronization with a neighbor. If a device has not received any traffic within the Keep-Alive period, it may send a Keep-Alive command frame to its clock source in order to receive time synchronization information in the ACK to the Keep-Alive command frame.

| Octets: Variable (see 7.2.2.4) | 1 | 0/4/8/16 | 2 |
| --- | --- | --- | --- |
| MHR | Command frame Identifier (see table 82) | MIC | MFR |

- Figure 64-F Keep-Alive command format

#### MHR field

The Source Addressing Mode subfield of the Frame Control field shall be set to two (16-bit addressing). The Frame Pending subfield of the Frame Control field shall be set to zero and ignored upon receipt, and the Acknowledgment Request subfield shall be set to one.

The Destination Address field shall contain the 16-bit address of the node the Keep-Alive Command frame is destined to. The Source PAN Identifier field shall be set to the PAN ID of the node, macPANID. The Source Address field shall be set to the short 16-bit address contained in macShortAddress.

The Sequence Number subfield shall be set to the least significant byte of the absolute timeslot number.

#### MIC

The message integrity check of Keep-Alive command frame.

7.3.12 Join command

The Join command is used by a device to join the TSCH network through the advertiser. This command shall only be sent by a new device that wishes to join the TSCH network or a device that lost connection with the TSCH network.

All devices shall be capable of transmitting this command, although an RFD is not required to be capable of receiving it.

The Join command shall be formatted as illustrated in Figure 64.G.

| Octets: (see 7.2.2.4) | 1 | 1 | 1 | 1 | 0/3 | var | 0/3 | TBD | 0/4/8/16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MHR fields | Command frame Identifier (see Table 82) | Capability Information (see Figure 56) | Clock Accuracy Capability | Number of Neighbors | Neighbor 1 | … | Neighbor n | Join Security Information (TBD by Security sub-group) | MIC |

Figure 64.G-Join command format

7.3.12.1 MHR fields

The Source Addressing Mode subfield of the Frame Control field shall be set to three (64-bit extended addressing). The Destination Addressing Mode subfield shall be set to the same mode as indicated in Advertisement command frame to which the Join command refers.

The Frame Pending subfield of the Frame Control field shall be set to zero and ignored upon receipt, and the Acknowledgment Request subfield shall be set to one.

The Destination PAN Identifier field shall contain the identifier of the PAN to which to join. The

Destination Address field shall contain the address from the Advertisement frame that was transmitted by the coordinator to which the Join command is being sent. The PAN ID Compression subfield may be set to one and the Source PAN Identifier may be omitted. The Source Address field shall contain the value of aExtendedAddress.

The Sequence Number subfield shall be set to the least significant byte of the absolute timeslot number.

7.3.12.2 Capability Information field

The Capability Information field shall be formatted as illustrated in Figure 56.

7.3.12.3 Clock Accuracy Capability field

The Clock Accuracy Capability field shall be formatted as illustrated in Figure 65-H.

| Bit: 0 | 1 - 7 |
| --- | --- |
| 10ppm capable | reserved |

Figure 65.H-Clock Accuracy Capability

7.3.12.4 Join Security Information field

The definition of Join Security Information field shall contain the security information that the new device to mutually authenticate the new joining device and Security Manager. This will be defined by Security sub-group.

7.3.12.5 Number of Neighbor field

The Number of Neighbor field indicates the number of neighbors included in this command frame.

7.3.12.6 Neighbor field

The Neighbor field shall contain the information about the neighbors of the new device.

| Octets: 2 | 1 |
| --- | --- |
| 16 bit address of the neighbor | RSSI |

Figure 65.I-Neighbor

7.3.12.7 MIC

This field contains the message integrity check of the Join command frame.

7.3.13 Activate command

The Activate command allows the advertiser to communicate the results of a Join attempt back to the device requested joining. The Activate command can also include the description of slotframe and links for the joining device to communicate with the TSCH network.

This command shall only be sent by the advertiser to the device that is currently trying to join.

All devices shall be capable of receiving this command, although an RFD is not required to be capable of transmitting it.

| The Activate command shall be formatted as illustrated in Figure 65-J. Octets: (see 7.2.2.4) | 1 | 2 | 1 | 0/5 | Variable | 0/5 | TBD | 0/4/8/16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MHR fields | Command frame Identifier (see Table 82) | Short Address | Number of links | Link 1 | … | Link N | Activate Security Information (TBD by Security sub-group) | MIC |

Figure 65-J-Activate command format

7.3.13.1 MHR

The Destination Addressing Mode subfield of the Frame Control field shall be set to three (i.e., 64-bit extended addressing). The Source Addressing Mode subfield of the Frame Control field shall be set to two (i.e., 16-bit addressing.

The Frame Pending subfield of the Frame Control field shall be set to zero and ignored upon receipt, and the Acknowledgment Request subfield shall be set to one.

The Source PAN Identifier field shall contain the value of macPANId. The Source Address field shall contain the value of aCoordShortAddress.

The Destination PAN Identifier field should be set to 0xFFFF. Destination Address field shall contain the extended address of the device requesting to join the network.

The Sequence Number subfield shall be set to the least significant byte of the absolute timeslot number.

7.3.13.2 Short Address field

If the advertiser was not able to join this device to its PAN, the Short Address field shall be set to 0xffff, and the Join Status field shall contain the reason for the failure. If the was advertiser is able to Join the device to its PAN, this field shall contain the short address that the device shall use in its communications on the PAN until it is disconnected.

The device shall use the source PANID of the Activate command as its PANID.

7.3.13.3 Number of Links field

The Number of Links field shall be set to the total number of links assigned to new device being activated.

7.3.13.4 Link field

Link field shall have the description of link allocated to new device being activated. The format of Link field can be found in Section 7.3.10.10.

7.3.13.5 Activate Security Information field

The definition of Activate Security Information field shall contain the security information that the new device should use to securely communicate to the TSCH network. It may include keys for data link and session layers.

7.3.13.6 MIC

This field contains the message integrity check of the Activate command frame.

### Disconnect command

The Disconnect command frame can be used to notify neighbors that it is being disconnected from TSCH network. If a node receives a Disconnect command frame from its neighbor, it should remove the neighbor from its neighbor list.

| Octets: Variable (see 7.2.2.4) | 1 | 0/4/8/16 | 2 |
| --- | --- | --- | --- |
| MHR | Command frame Identifier (see table 82) | MIC | MFR |

- Figure 64-J Disconnect command format

#### MHR field

The Source Addressing Mode subfield of the Frame Control field shall be set to two (16-bit addressing). The Frame Pending subfield of the Frame Control field shall be set to zero and ignored upon receipt, and the Acknowledgment Request subfield shall be set to zero.

The Destination Addressing Mode subfield shall be set to the broadcast address, 0xFFFF.

The Source PAN Identifier field shall be set to the PAN ID of the node, macPANID. The Source Address field shall be set to the short 16-bit address contained in macShortAddress.

The Sequence Number subfield shall be set to the least significant byte of the absolute timeslot number.

#### MIC

The message integrity check of the Disconnect command frame.

7.5.1.5 Slotframe structure

A slotframe is a collection of timeslots repeating in time. The number of timeslots in a given slotframe (slotframe size) determines how often each timeslot repeats, thus setting a communication schedule for nodes that use the timeslots. When a slotframe is created, it is associated with a slotframe ID for identification. Every new slotframe instance in time is called a slotframe cycle. Figure 69.A shows how nodes may communicate in a sample three-timeslot slotframe. Nodes A and B communicate during timeslot 0, nodes B and C communicate during timeslot 1, and timeslot 2 is not being used. Every three timeslots, the schedule repeats.

- -Example of a three-timeslot slotframe

Several performance parameters are determined by slotframe size and how timeslots are assigned within a slotframe for communication. In general, shorter slotframes result in lower latency and increased bandwidth, but at the expense of increased power consumption. Long slotframes generally result in higher latency and lower bandwidth, but power consumption is reduced and the number of communication resources (links) is increased. This affects the scale of the network.

7.5.1.5.1 Multiple slotframes

A given network using timeslot-based access may contain several concurrent slotframes of different sizes. Slotframe size defines the bandwidth of a timeslot. A timeslot within a slotframe of a particular size repeats twice as fast as a timeslot within a slotframe that is twice as long, thus allowing for double throughput. Multiple slotframes may be used to define a different communication schedule for various groups of nodes or to run the entire network at different duty cycles.

A network device may participate in one or more slotframes simultaneously, and not all devices need to participate in all slotframes. By configuring a network device to participate in multiple overlapping slotframes of different sizes, it is possible to establish different communication schedules and connectivity matrices that all work at the same time.

Slotframes can be added, removed, and modified while the network is running. Even though this is the case, all slotframes logically start in the same place in time. Cycle 0, timeslot 0 of every slotframe occurs at the beginning of epoch, which is determined by the network device that starts the network. Because of this, timeslots in different slotframes are always aligned, even though beginnings and ends of slotframes may not be (see Figure 69.B). Because all slotframes begin at the same time, it is always possible to identify time of a given frame cycle and timeslot.

- - Multiple slotframes in the network

7.5.1.6 TSCH Channel Hopping

Devices in a TSCH network use channel hopping to combat the interference and fading. Device determines which channel to use for a given timeslot instance of a link. Transmit/receive channel shall be determined based on absolute slot number, the channel offset of the given link, and the channel hopping sequence table lookup. To communicate on a link assigned to it, a network device shall use the formula below to derive the index of the ChannelDescriptor in ChannelHoppingSequenceTable. ChannelDescriptor is consisted with channel page and channel number.

Index of ChannelDescriptor =

(Channel Offset + Absolute Slot Number) % number of channels in ChannelHoppingSequenceTable

Once the index of ChannelDescriptor of the ChannelHoppingSequenceTable is derived using the above formula, the device can find the channel page and channel number in the ChannelDescriptor. The format of ChannelHoppingSequenceTable can be found in Section 7.4.2.

7.5.1.6.1 TSCH Channel Hopping Sequence Table determination

Up to 16 channel hopping sequences are allowed in TSCH channel hopping. Each channel hopping sequence is uniquely identified with channel hopping sequence identifier. Each channel hopping sequence is stored in ChannelHoppingSequenceTable – see Section 7.4.2 for more detail.

During Advertisement and Discovery procedure, a new node finds which channel hopping sequence is being used in the TSCH network in Advertisement command frame.

7.5.1.6.1 Blacklisting channels

In some cases one or more channels in a ChannelHoppingSequenceTable may be unusable for use in the TSCH network.

This could be the case if there is another 802.15.4 application active nearby or if there is some other interference on one of the channels. The Network Manager has the option to blacklist these channels. This is achieved through the channelPage and channelMap parameter of MLME-SET-SLOTFRAME.request primitive or Advertisement command frame. The bit positions in the channelMap bitmask correspond channels in the channel page. If a bit is set in the channelMap bitmask, the corresponding channel number in the channel page is used; if a bit is not set, the corresponding channel in the channel page is not used.

Active ChannelHoppingSequenceTable is constructed by removing unused channels specified by the channelMap bitmask and the number of channels used in ChannelHoppingSequenceTable is adjusted to the total number of bits in channelMap that are set.

7.5.2.6 TSCH network formation

7.5.2.6.1 Overview

There are two components of network formation in the TSCH network: advertising and joining. As a part of advertising, network devices that are already part of the network may send command frames announcing the presence of the network. Advertisement command frames include time synchronization information and a unique PAN ID. A new device trying to join listens for the Advertisement command frames. If the device is pre-provisioned with a PAN ID, then it matches the advertised PAN ID with the provisioned one at the higher layer. If there is no provisioned PAN ID, the device does not look for a match. When at least one acceptable Advertisement command frame is received, the new device can attempt to join the network. A new device joins the network by sending a Join request command frame to an advertising node. In a centralized management system this join command is routed to the PAN coordinator. In a distributed management system it can be processed locally. When the device is accepted into the network, the advertiser activates the device by setting up slotframes and links between the new device and other existing devices. These slotframes and links can also be deleted and modified and new slotframes and links added any time after a device has joined the network.

A new network starts when the PAN coordinator starts to advertise (typically at the request of Network Manager residing in the PAN coordinator). Being the first node in the network, the PAN coordinator starts at least one slotframe, to which other network devices may later synchronize.

7.5.2.6.2 Advertising

In order for new devices to join a network they must first learn network information from some devices that are already part of the network. This is done through advertising. Network devices may send Advertisement command frames to invite new devices into the network.

Advertisement command frames contain the following information:

- PAN ID.

- Time information so new devices can synchronize to the network.

- Channel page and a list of RF channels in that channel page being used.

- Link and slotframe information so new devices know when they can transmit to the advertising device.

- Link and slotframe information so new devices know when to listen for transmits from the advertising network device.

7.5.2.6.3 Joining

After a new device hears at least one valid Advertisement command frame, it may synchronize to the network and start joining. Advertisement command frames contain information about the links through which the new device may communicate with the advertising neighbor, and through it forward frames to the Network Manager. The joining procedure may include a security handshake to mutually authenticate the joining device and the Network Manager and establish the secure session between the new device and the Network Manager in addition to allocating the communication resource to the joining device.

7.5.4 Synchronization

This subclause specifies the procedures for coordinators to generate beacon frames, and for devices to

synchronize with a coordinator, and for devices to synchronize to the TSCH network. For PANs supporting beacons, synchronization is performed by receiving and decoding the beacon frames. For PANs not supporting beacons, synchronization is performed by polling the coordinator for data or by time synchronized communication within a timeslot of the slotframe.

7.5.4.4 Synchronization in TSCH network

7.5.4.4.1 Timeslot communication

During a timeslot in a slotframe, one node typically sends a frame, and another sends back an acknowledgement if it successfully receives that frame. An acknowledgement can be positive (ACK) or negative (NACK). A positive acknowledge indicates that the receiver has successfully received the frame and has taken ownership of it for further routing. A negative acknowledgement indicates that the receiver cannot accept the frame at this time, but has heard it with no errors. Both ACKs and NACKs carry timing information used by nodes to maintain network synchronization. Frames sent to a unicast node address require that a link-layer acknowledgement be sent in response during the same timeslot as shown in Figure 69.C.

- -Timeslot diagram of acknowledged transmission

7.5.4.4.2 Node synchronization

Device-to-device synchronization is necessary to maintain connection with neighbors in a slotframe-based network. There are two methods for a device to synchronize to the network.

7.5.4.4.2.1 Acknowledgement-based synchronization

Unicast communication provides a basic method of time synchronization through the exchange of data and acknowledgement frames. The algorithm involves the receiver calculating the delta between the expected time of frame arrival and its actual arrival, and providing that information to the sender node.

The algorithm can be described as follows:

- Transmitter node sends a frame, timing the start symbol to be sent at TsTxOffset.

- Receiver records the timestamp TsRxActual of receiving the start symbol of the packet.

- Receiver calculates TimeAdj = TsTxOffset – TsRxActual.

- Receiver send back TimeAdj as part of acknowledgement packet.

- Transmitter receives the acknowledgement. If the receiver node is a clock source node, the transmitter adjusts its network clock by TimeAdj.

7.5.4.4.2.2 Frame-based synchronization

A node may synchronize its own network clock if it receives a frame from a clock source neighbor. The mechanism is similar to that of ACK-based synchronization. The receiver calculates the delta between expected time of frame arrival and its actual arrival time, and adjusts its own clock by the difference.

The algorithm can be described as follows:

- Receiver records the timestamp TsRxActual of receiving the start symbol of the packet.

- Receiver calculates TimeAdj = TsTxOffset – TsRxActual.

- Receiver adjusts its own network time by –TimeAdj.

Note that this procedure should only be executed if the node from which the frame is received is a clock source for the receiver.

Figure 69.D illustrates both time synchronization mechanisms. In both cases, the receiver calculates TimeAdj to either send back to the transmitter or to use locally.

- -Time synchronization

7.5.4.4.3 Network time synchronization

Precise time synchronization is critical to the operation of networks based on time division multiplexing. Since all communication happens in timeslots, the network devices must have the same notion of when each timeslot begins and ends, with minimal variation. The acknowledgement and frame-based synchronization are used for pair-wise synchronization, as outlined below. In a typical TSCH network, time propagates outwards from the PAN coordinator  A network device must periodically synchronize its network clock to at least one other network device. It may also provide its network time to one or more network devices. A network device determines whether to follow a neighbor’s clock based on the presence of a ClockSource flag in the corresponding neighbor’s record (configured by the Network Manager). The direction of time propagation is independent of data flow in the network.

A network device may have more than one neighbor as its clock source. In such cases, the device may synchronize its clock to any of the neighbors that are acting as its clock source.

7.5.4.4.4 Keep-Alive mechanism

In order to ensure that it remains synchronized with the TSCH network (and to detect when paths may be down) a network device shall ensure that it communicates with each of its clock sources at least once per Keep Alive period.

If a network device has not sent a packet to its clock parent within this interval, it shall send a Keep-Alive command frame and use the ACK to perform ACK-based synchronization as usual.

7.4.2 MAC PIB attributes

macDisconnectTime and macNumOfChannelHoppingSequence entry needs to be appended in Table 86 as follow.

| attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macDisconnectTime |  | Integer | 0x00-0xFFFF | Timer value of disconnect time |  |
| macNumChannelHoppingSequence |  | Integer | 0x0-0xF | Number of Channel Hopping Sequence Tables | 1 |

7.4.3 MAC PIB Tables

macSlotframeTable

| macSlotframeTable attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| slotframeId |  | Integer | 0x00-0xFF | Identifier of the slotframe |  |
| slotframeSize |  | Integer | 0x0000-0xFFFF | Number of timeslots in the slotframe |  |
| activeFlag |  | Boolean | 0x0-0x1 | Flag indicating if the slotframe is currently activated |  |
| channelPage |  | Integer | 0x00-0x1F | Channel Page of channels used in this slotframe |  |
| channelMap |  | Bitmap |  | Bitmap of active channels. |  |

macLinkTable

| macLinkTable attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| linkId |  | Integer | 0x00-0xFF | Identifier of Link |  |
| linkOption |  | Bitmap | 0x00-0x7 | Flags indicating whether the link is used for transmit, receive, or shared transmissions: |  |
| linkType |  | Integer | 0x00-0x2 | Enumeration indicating the type of link: Normal, or Advertising |  |
| slotframeId |  | Integer | 0x00-0xFF | Identifier of Slotframe to which this link belongs |  |
| nodeAddress |  | IEEE address | 16 bit address | Address of the node connected to this link |  |
| timeslot |  | Integer | 0x0000-0xFFFF | Timeslot for this link |  |
| channelOffset |  |  | 0x00-0xFF | Channel offset for this link |  |

macTimeslotTemplate

| Entry Name | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| Timeslot Template Id |  | Integer | 0x0-0xF | Identifier of Timeslot Template |  |
| TsCCAOffset |  | Integer | 0x0000-0xFFFF | The time between the beginning of timeslot and start of CCA operation |  |
| TsCCA |  | Integer | 0x0000-0xFFFF | Duration of CCA |  |
| TsTxOffset |  | Integer | 0x0000-0xFFFF | The time between the beginning of the timeslot and the start of packet transmission |  |
| TsRxOffset |  | Integer | 0x0000-0xFFFF | Beginning of the timeslot to when the receiver must be listening |  |
| TsRxAckDelay |  | Integer | 0x0000-0xFFFF | End of packet to when the transmitter must listen for Acknowledgment |  |
| TsTxAckDelay |  | Integer | 0x0000-0xFFFF | End of packet to start of Acknowledgment |  |
| TsRxWait |  | Integer | 0x0000-0xFFFF | The time to wait for start of packet |  |
| TsAckWait |  | Integer | 0x0000-0xFFFF | The minimum time to wait for start of an Acknowledgment |  |
| TsRxTx |  | Integer | 0x0000-0xFFFF | Transmit to Receive turnaround (12 symbols) |  |
| TsMaxAck |  | Integer | 0x0000-0xFFFF | Transmission time to send Acknowledgment |  |
| TsMaxTx |  | Integer | 0x0000-0xFFFF | Transmission time to send the maximum length packet (133 bytes) |  |

macHoppingSequenceTables

| Entry Name | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| ChannelHoppingSequenceTables |  | Array of Channel Hopping Sequence Table | 1 to 16 tables | Each ChannelHoppingSequenceTable describes a unique channel hopping sequence. See below for the structure of ChannelHoppingSequenceTable. |  |

ChannelHoppingSequenceTable

| Entry Name | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| ChannelHoppingSequenceId |  | Integer | 0x0-0xF | Identifier for a channel hopping sequene. |  |
| numberOfChannels |  | Integer | 1-1024 | Number of channel entries in a channel hopping sequence |  |
| channelDescriptorArray |  | Array of channelDescriptor |  | See below for the structure of channelDescriptor |  |

channelDescriptor

| Entry Name | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| ChannelPage |  | Integer | 0-32 | Identifier for a channel hopping sequene. |  |
| ChannelNumber |  | Integer | 0x00-0xFF | channel number of the channel |  |