IEEE P802.15.4g Draft

Wireless Neighborhood Area Networks

| Project | IEEE P802.15 TG4g Wireless Neighborhood Area Networks (WNANs) |  |
| --- | --- | --- |
| Title | DRAFT FOR 802.15 TG4g PHY AMENDMENT FHSS PHY w/ Interoperable Battery Powered DevicesFHSS PHY w/ Interoperable Battery Powered Devices |  |
| Date Submitted | 30 April 2009 |  |
| Source | [David G. Hart] [Elster Electricity] [208 South Rogers Lane, Raleigh, NC 27610] | Voice: [ ] Fax: [ ] E-mail: [david.g.hart@us.elster.com] |
| Re: | Proposed FHSS PHY Specification for 802.15.4g |  |
| Abstract | This document provides specifications for a Frequency Hopping Spread Spectrum (FHSS) physical layer for 802.15.4 submitted to the 802.15.4g Task Group for consideration in the PHY amendment. |  |
| Purpose | Support the preparation of proposals and initial drafting process |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

TABLE OF CONTENTS

1	Overview	3

1.1	General	3

1.2	Purpose	3

2	General Description of the Proposed PHY	3

2.1	Functionality to Support Neighborhood Area Networks	3

3	PHY Specification	4

3.1.1	Overview of Parameters	4

3.1.2	Channel Plan – 915 MHz Band	4

3.1.3	Channel Plan – 2.4 GHz Band	5

# Overview

## General

Neighborhood Area Networks (NANs) are large scale networks consisting of multiple types of devices. These devices may be powered and always on, or devices that have power constraints and have limited transmit and receive duty cycles. For a utility NAN, the most prominent devices types are electricity, gas, and water meters, but a utility network is not limited to these types of devices. Electricity meters can be considered a powered device, and these devices are typically the common denominator for most utility networks and therefore can act as the network infrastructure. Gas and water meters are typically battery powered

A smart utility network considers all types of utility devices and ensures that all device types can communicate with other network devices using the common PHY and MAC layers of the utility network protocol.

## Purpose

This document describes the Frequency Hopping Spread Spectrum PHY specifications to support the scope of the 802.15.4g Task Group as described in the IEEE PAR (Project Authorization Request). This proposal defines an alternative PHY and the MAC modifications required to support its implementation with 802.15.4.

# General Description of the Proposed PHY

## Functionality to Support Neighborhood Area Networks

The network must be compromised of robust, scalable network devices capable of today’s data requirements, but also capable of providing extended data capacity and response times for existing and future data requirements and device types. To meet these criteria, the PHY must support the following:

- Data rate for today’s requirements (40 kbps) but also for future needs (300 kbps)

- Ubiquitous network supporting various types of utility devices (electric, water, gas, etc)

- Dynamic scaling to very large networks

- Support for PHY frames of 1500 bytes or more

- Minimal infrastructure requirements (in the majority of cases, nothing required except the utility devices)

# PHY Specification

The following sections list the PHY details for all devices.

### Overview of Parameters

| Parameter | Value |
| --- | --- |
| Operating Band | 902-928 MHz |
| Channel BW | Per regulatory requirements |
| Data rates | 60/240 kbps |
| Channel Spacing | 200/400 kHz |
| Number of channels | 128/64 |
| Modulation | FSK |
| Frequency Hopping | MAC controlled |
| Data whitening | MAC layer |
| Transmit Power | Per regulatory requirements |
| Transmit Power Control | Available |
| PHY frame structure: |  |
| - Payload | Variable size per message (1 – 4095 octets) |
| - CRC | 2 octets |

### Channel Plan – 915 MHz Band

The channel assignment is calculated using a 400 kHz channel spacing in the 902-928 MHz band. The frequencies shown below are the center frequencies and a center frequency is computed as:

Freq = 902.4 + (Channel*0.4)

The following table lists the channel assignments, all frequencies are in MHz.

### Channel Plan – 2.4 GHz Band

The channel assignment is calculated using a 400 kHz channel spacing in the 2.4 GHz band. The frequencies shown below are the center frequencies and a center frequency is computed as:

Freq = 2400.4 + (Channel*0.4)

The following table lists the channel assignments, all frequencies are in MHz.