IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Required Changes for EGTS Extension |  |
| Date Submitted | Monday, May 11, 2009 (TBD) |  |
| Source | [Myung Lee, Gahng-Seop Ahn, Liang Li Seong-Soon Joo, Tae Rim Park, Betty Zhao, Ghulum Bhatti, Ning Gu, Jie Shen, Wei Hong, Qin Wang and Quan Wang, Wun-Cheol Jeong, Chang-Sub Shin, Anseok Lee] [140th St. and Convent Ave, New York, NY, USA] | Voice: [+1 212-650-7260] E-mail: [lee@ccny.cuny.edu] |
| Re: | [] |  |
| Abstract | [This document contains proposed changes to the IEEE P802.15.4e Draft to address required changes for EGTS.] |  |
| Purpose | [] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Content

1.	New or Modified MAC Primitives	3

1.1	MLME-GTS	3

1.1.1	MLME-GTS.request	3

1.1.2	MLME-GTS.confirm	6

1.1.3	MLME-GTS.indication	7

1.2	MLME-START	9

1.3	MCPS-DATA	12

1.4	MLME-EGTSinfo	12

1.5	MLME-BEACON-NOTIFY.indication	17

2.	Modified MAC command frames	17

2.1	Beacon fields	17

2.2	Association	20

2.2.1	Association request command	20

2.2.2	Association respond command	20

3.	New MAC command frames	21

3.1	EGTS handshake	21

3.2	EGTS information request	25

3.3	Beacon allocation notification	25

3.4	Beacon collision notification	26

4.	New MAC PIBs	26

5.	New Functional Description	28

5.1	EGTS-based Superframe Structure	28

5.2	EGTS Scheduling	33

5.3	Beacon Scheduling	43

5.4	Time Synchronization	44

5.5	Scanning through channels	44

5.5.1	Passive channel scan	44

5.6	Starting and realigning a PAN	45

5.6.1	Updating superframe configuration and channel PIB attributes	45

5.7	Beacon generation	45

5.8	Coexistence of beacon-enabled and nonbeacon-enabled mode	45

- New or Modified MAC Primitives

Table 1. Changes in MAC primitives

| Primitive | Change | Request | Confirm | Response | Indication |
| --- | --- | --- | --- | --- | --- |
| MLME-GTS | Modified | X | X |  | X |
| MLME-START | Modified | X |  |  |  |
| MCPS-DATA | Modified | X |  |  |  |
| MLME-BEACON-NOTIFY | Modified |  |  |  | X |
| MLME-EGTSinfo | New | X | X |  |  |

  - MLME-GTS

    - MLME-GTS.request

Insert in 7.1.7.1:

The MLME-GTS.request primitive allows a device to send a request to the PAN coordinator to allocate a new GTS slot or to the Destination device to allocate a new GTS slot. This primitive also used to deallocate GTS or EGTS.

If the value of the EGTS flag in the EGTSCharactersitics field set to ‘1’, the MLME-GTS.request primitive allows a Source device to send a request to a Destination device to allocate a new EGTS, or to deallocate / reallocate / change an existing EGTS. This primitive is also used by a Destination device to initiate an EGTS deallocation, reallocation, or change (suspend, reduce, or restart).

Insert in 7.1.7.1.1:

MLME-EGTS.request	(

GTSCharacteristics,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

EGTSCharacteristics,

EGTSFlag

)

Insert in Table 58:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EGTSFlag | Boolean | TRUE or FALSE | If this value is FALSE, the operation of this primitive is the same way defined in IEEE 802.15.4-2006 (definition of the GTSCharacteristics parameter see 7.3.9.2). If this value is TRUE, indicate that the request is for EGTS, the GTSCharacteristics parameter will use the definition of EGTSCharacteristics (see 7.3.10.2). |
| EGTSCharacteristics | EGTSCharacteristics | See 7.3.10.2 | The characteristics of the EGTS request, including whether the request is for the allocation of a new EGTS or the deallocation / reallocation / change of an existing EGTS. |

Insert in 7.1.7.1.2:

The MLME-GTS.request primitive can also be generated by the next higher layer of a Source device and issued to its MLME to request the allocation of a new EGTS or to request the deallocation / reallocation / change of an existing EGTS. It is also generated by the next higher layer of the Destination device and issued to its MLME to request the deallocation, reallocation, or change of an existing EGTS.

Insert in 7.1.7.1.3:

If the value of the EGTSFlag equals to ‘0’, the effect of MLME-GTS.request is the same as it is described in Section 7.1.7.1 in IEEE Std 802.15.4-2006. If the value of the EGTS flag in the EGTSCharacteristics field is set to ‘1’, the effect of MLME-GTS.request is as follows.

On receipt of the MLME-GTS.request primitive by a Source device, the MLME of the Source device attempts to generate an EGTS handshake command with subtype (see 1.7.3.9) with the information contained in this primitive and, if successful, sends it to a Destination device.

On receipt of the MLME-GTS.request primitive by a Destination device, the MLME of the Destination device generates an EGTS request command (see 1.7.3.9) with the information contained in this primitive and sends it to the Source device.

If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for this frame, the MLME will set the Security Enabled subfield of the Frame Control field to one. The MAC sublayer will perform outgoing processing on the frame based on the DstAddress, SecurityLevel, KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.5.8.2.1. If any error occurs during outgoing frame processing, the MLME will discard the frame and issue the MLME-GTS.confirm primitive with the error status returned by outgoing frame processing.

If the EGTS request command cannot be sent due to the channel condition, the MLME will issue the MLME-GTS.confirm primitive with a status of CHANNEL_ACCESS_FAILURE.

If the MLME successfully transmits an EGTS handshake command, the MLME will expect an acknowledgment in return. If an acknowledgment is not received, the MLME will issue the MLME-GTS.confirm primitive with a status of NO_ACK (see 7.5.6.4).

If an EGTS is being allocated (see 7.5.7.2) and the request has been acknowledged, the device will wait for a confirmation via an EGTS handshake allocation reply command frame from the destination device. If the MLME of the Destination device can allocate the requested EGTS, it will issue the MLME-GTS.indication primitive with the characteristics of the allocated EGTS and generate an EGTS handshake allocation reply command frame. If the MLME of the PAN coordinator cannot allocate the requested EGTS, it will generate an EGTS handshake allocation reply command frame with EGTS length field set to ‘0’.

If an EGTS handshake allocation reply command frame and the EGTS length matches the characteristics requested (indicating that the Destination device has approved the EGTS allocation request), the MLME of the Source device will issue the MLME-GTS.confirm primitive with a status of SUCCESS and a EGTSCharacteristics parameter with a characteristics type equal to one, indicating a GTS allocation. If the EGTS handshake allocation reply command frame is received with a EGTS length of zero (indicating that the Destination device has denied the EGTS allocation request), the device requesting the EGTS issues the MLME-GTS.confirm primitive with a status of DENIED, indicating that the GTSCharacteristics parameter is to be ignored.

If an EGTS is being deallocated (see 7.5.7.4) at the request of a device and the request has been acknowledged by the other device, the device will issue the MLME-GTS.confirm primitive with a status of SUCCESS and an EGTSCharacteristics parameter with a characteristics type equal to ‘1’, indicating a GTS deallocation. On receipt of an EGTS handshake deallocation request command, the device will acknowledge the frame and deallocates the EGTS. The MLME of the device will then issue the MLME-GTS.indication primitive with the appropriate EGTS characteristics. If the Destination device does not receive the deallocation request, counter measures can be applied by the PAN coordinator to ensure consistency is maintained (see 7.5.7.6).

If an EGTS of the Source device is being changed at the request of a Destination device and the request has been acknowledged by the Source device, the Destination device will issue the MLME-GTS.confirm primitive with a status of SUCCESS and an EGTSCharacteristics parameter with an EGTSCharacteristics type subfield set to 100 SUSPEND, 101 REDUCE or 110 RESTART indicating an EGTS change according to the MLME-GTS.request primitive. On receipt of an EGTS handshake command with a request type indicating an EGTS change (100 SUSPEND, 101 REDUCE or 110 RESTART), the Source device will acknowledge the frame and change the EGTS. The MLME of the Source device will then issue the MLME-GTS.indication primitive with the appropriate TS characteristics.

If any parameter in the MLME-GTS.request primitive is not supported or is out of range, the MLME will issue the MLME-GTS.confirm primitive with a status of INVALID_PARAMETER.

On the receipt of the MLME-GTS.request primitive, the MLME of a device attempts to generate a GTS request command or an EGTS request command (See 3.1) depending on the EGTSFlag parameter.

If EGTSFlag is TRUE, the MLME-GTS.request primitive allows a Source device to send a request to a Destination device to allocate or deallocate EGTS slots. This primitive is also used by a Destination device to initiate the EGTS deallocation.

If the ChannelDiversityMode subfield of EGTSCharacteristic parameter is ‘1’, the EGTS allocation and/or deallocation operate as Channel Hopping mode.

The main difference between Channel Adaptation mode and Channel Hopping mode is the EGTS ABT sub-field in EGTSCharacteristic parameter. In Channel Hopping mode, both the Source and the Desitination device exchange their Timeslot Allocation Bitmaps (TAB) in contrast to the ABT sub-block in Channel Adaptation mode.

    - MLME-GTS.confirm

Insert in 7.1.7.2:

The MLME-GTS.confirm primitive reports the results of a request to allocate a new GTS or EGTS. This primitive also reports the result of request to deallocate an existing EGTS.

If the value of the EGTSFlag equals to ‘1’, the MLME-GTS.confirm primitive reports the results of a request to allocate a new EGTS or to deallocate / reallocate / change an existing EGTS.

Insert in 7.1.7.2.1:

MLME-EGTS.confirm	(

GTSCharacteristics,

status,

EGTSFlag,

EGTSCharacteristics

)

Insert in Table 59:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EGTSFlag | Boolean | TRUE or FALSE | If this value is FALSE, the operation of this primitive is the same way defined in IEEE 802.15.4-2006 (definition of the GTSCharacteristics parameter see 7.3.9.2). If this value is TRUE, indicate that the request is for EGTS, the GTSCharacteristics parameter will use the definition of EGTSCharacteristics (see 7.3.10.2). |
| EGTSCharacteristics | EGTSCharacteristics | See 7.3.10.2 | The characteristics of the EGTS request, including whether the request is for the allocation of a new EGTS or the deallocation / reallocation / change of an existing EGTS. |

Insert in 7.1.7.2.2:

If the request to allocate, deallocate, reallocate, or change (suspend, reduce, or restart) an EGTS was successful, this primitive will return a status of SUCCESS and the EGTSCharacteristics type subfield of the EGTSCharacteristics parameter will be set accordingly. Otherwise, the status parameter will indicate the appropriate error code. The reasons for these status values are fully described in 7.1.7.1.3 and subclauses referenced by 7.1.7.1.3.

Insert in 7.1.7.2.3:

On receipt of the MLME-GTS.confirm primitive with the value of the EGTS flag in the EGTSCharactersitics field set to ‘1’, the next higher layer is notified of the result of its request to allocate, deallocate, or change an EGTS. If the request was successful, the status parameter will indicate a successful EGTS operation. Otherwise, the status parameter will indicate the error.

    - MLME-GTS.indication

Insert in 7.1.7.3:

If the value of the EGTSFlag equals to ‘1’, the MLME-GTS.indication primitive indicates that an EGTS has been allocated or that a previously allocated EGTS has been deallocated, reallocated, or changed.

Insert in 7.1.7.3.1:

MLME-EGTS.indication	(

DeviceAddress,

GTSCharacteristics,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

EGTSFlag,

EGTSCharacteristics

)

Insert in Table 60:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EGTSFlag | Boolean | TRUE or FALSE | If this value is FALSE, the operation of this primitive is the same way defined in IEEE 802.15.4-2006 (definition of the GTSCharacteristics parameter see 7.3.9.2). If this value is TRUE, indicate that the request is for EGTS, the GTSCharacteristics parameter will use the definition of EGTSCharacteristics (see 7.3.10.2). |
| EGTSCharacteristics | EGTSCharacteristics | See 7.3.10.2 | The characteristics of the EGTS request, including whether the request is for the allocation of a new EGTS or the deallocation / reallocation / change of an existing EGTS. |

Insert in 7.1.7.3.2:

This primitive is generated by the MLME of a Source or Destination device and issued to its next higher layer when an EGTS is allocated, deallocated, reallocated, or changed following the reception of an EGTS handshake command (see 1.7.3.9) by the MLME. The EGTS Characteristics type subfield of the EGTSCharacteristics parameter is set accordingly.

Insert in 7.1.7.3.3:

On receipt of the MLME-GTS.indication primitive with the value of the EGTS flag in the EGTSCharactersitics field set to ‘1’, the next higher layer is notified of the allocation, deallocation, reallocation, or change of a EGTS.

Insert in 7.1.7.4:

Figure 1 and Figure 2 illustrate the sequence of messages necessary for successful EGTS management. Figure 1 depicts the message flow for the case in which a Source device initiates the EGTS allocation, deallocation, reallocation, or change. Figure 2 depicts the message flow for the case in which a Destination device initiates the EGTS deallocation, reallocation, or change.

Figure 1—Message sequence chart for EGTS allocation initiated by a Source device

Figure 2—Message sequence chart for EGTS allocation initiated by a Destination device

  - MLME-START

Modify in 7.1.14.1.1:

MLME-START.request	(

PANId,

LogicalChannel,

ChannelPage,

SuperframeStartBank,

BeaconOrder,

SuperframeOrder

PANCoordinator

BatteryLifeExtension

CoorRealignment

CoorRealignSecurityLevel,

CoorRealignKeyIdMode,

CoorRealignKeySource

CoorRealignKeyIndex

BeaconSecurityLevel,

BeaconKeyIdMode,

BeaconKeySource,

BeaconKeyIndex,

EGTSFlag,

ChannelDivsrityMode,

DCHDescriptor

)

Insert in Table 72:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SuperframeStartBank | Integer | 0x00 – 0x1F | In superframe bank , the coordinator transmitis its own beacon frame. If this parameter is set to 0x00, beacon transmitting will begin immediately. Otherwise, the specified bank number is relative to the bank index of the received beacon with which the device synchronizes. This parameter is ignored if either the BeaconOrder parameter has a value of 15 or the PANCoordinator parameter is TRUE. |
| EGTSFlag | Boolean | TRUE or FALSE | If this value is FALSE, the operation of this primitive is the same way defined in IEEE 802.15.4-2006 (definition of the GTSCharacteristics parameter see 7.3.9.2). If this value is TRUE, indicate that the request is for EGTS, the GTSCharacteristics parameter will use the definition of EGTSCharacteristics (see 7.3.10.2). |
| ChannelDiversityMode | Integer | 0x00 – 0x01 | Indicates the type of channel diversity mode: 0x00 = Channel Adaptation (default) 0x01 = Channel Hopping (optional) This value is not valid for a non-beacon enabled PAN. |
| DCHDescriptor | DCHDescriptor Value |  | See Table 4. |

Table 4—Elements of DCHDescriptor

| Name | ID | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| CurrentChannelpage |  | Integer | 0-31 | The value of this attribute is set by the next higher layer. |  |
| CommonChannel |  | Integer | Selected from the available logical channels specified by the CurrentChannelPage parameter | The value of this attribute is set by the next higher layer. |  |
| ChannelHoppingSequenceLength |  | Integer |  | The value of this attribute is set by the next higher layer. |  |
| ChannelHoppingSequece |  | Set of Octets |  | The value of this attribute is set by the next higher layer. |  |
| ChannelOffset |  | Integer |  |  | 0 |
| ChannelOffsetBitmapLength |  | Integer |  |  | 0 |
| ChannelOffsetBitmap |  | Set of Octets |  |  | NULL |

Modify in 7.1.14.1.3:

If the SuperframeStartBank parameter is non-zero and the MLME is not currently tracking the beacon of the coordinator through which it is associated, the MLME will issue the MLME-START.confirm primitive with a status of TRACKING_OFF. (Liang: definition for 0x01—0Xff?)

  - MCPS-DATA

Modify in Table 41:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TxOptions | Bitmap | 5-bit field | The 5 bits (b0, b1, b2, b3, b4) indicate the transmission options for this MSDU. For b0, 1 = acknowledged transmission, 0 = unacknowledged transmission. For b1, 1 = GTS transmission, 0 = CAP transmission for a beacon-enabled PAN. For a nonbeacon-enabled PAN, bit b1 should always be set to 0. For b2, 1 = indirect transmission, 0 = direct transmission. For b3, 1 = CAP/EGTS transmission, 0 = CAP/GTS transmission (defined in IEEE 802.15.4-2006). For b4, 1 = High Priority transmission, 0 = Low Priority transmission. |

Insert in 7.1.1.1.3:

If the TxOptions parameter specifies that an EGTS transmission is required, the MAC sublayer will set the EGTS flag in the EGTS Characteristics field to one, indicating the EGTS transmission, and determine whether it has a valid EGTS (for EGTS usage rules, see 7.5.10). If a valid EGTS could not be found, the MAC sublayer will issue the MCPS-DATA.confirm primitive with a status of INVALID_GTS. If a valid EGTS was found, the MAC sublayer will defer, if necessary, until the EGTS.

  - MLME-EGTSinfo

Insert new subclause ‘7.1.18 Primitives for requesting EGTS information’:

MLME-EGTSinfo defines how a device can request EGTS information.

Insert new subclause ‘7.1.18.1 MEME-EGTSinfo.request’:

The MLME-EGTSinfo.request primitive allows a Source device to request the timestamp and the parameters of its EGTS from the Destination device.

Insert new subclause ‘7.1.18.1.1 Semantics of the service primitive’:

The semantics of the MLME-EGTSinfo.request primitive are as follows:

MLME-EGTSinfo.request     (

DstAddrMode,

DstAddress,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table XX specifies the parameters for the MLME-EGTSinfo.request primitive.

Table XX—MLME-EGTSinfo.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DstAddrMode | Integer | 0x02–0x03 | The addressing mode of the Destination device to which the request is intended. This parameter can take one of the following values: 2 = 16-bit short address, 3 = 64-bit extended address. |
| DstAddress | Device-Address | As specified by the DstAddrMode parameter | The address of the Destination device to which the request is intended. SecurityLevel Integer 0x00–0x07 The security level to be used (see Table 95 in 7.6.2.2.1). |
| SecurityLevel | Integer | 0x00-0x07 | The security level to be used (see Table 95 in 7.6.2.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (see Table 96 in 7.6.2.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key to be used (see 7.6.2.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.6.2.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |

Insert in ‘7.1.18.1.2  Appropriate usage’:

The MLME-EGTSinfo.request primitive is generated by the next higher layer of a Source device and issued to its MLME when the timestamp and the parameters of its EGTS are to be requested from the Destination device.

Insert in ‘7.1.18.1.3  Effect on receipt’:

On receipt of the MLME-EGTSinfo.request primitive by a device, the MLME of the device generates and sends an EGTS information request command (see XXX). The EGTS information request command is generated with the destination address information in the DstAddress parameter.

If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for this frame, the MLME will set the Security Enabled subfield of the Frame Control field to one. The MAC sublayer will perform outgoing processing on the frame based on the DstAddress, SecurityLevel, KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.5.8.2.1. If any error occurs during outgoing frame processing, the MLME will discard the frame and issue the MLME-EGTSinfo.confirm primitive with the error status returned by outgoing frame processing.

If the EGTS information request command cannot be sent due to a CSMA-CA algorithm failure, the MLME will issue the MLME-EGTSinfo.confirm primitive with a status of CHANNEL_ACCESS_FAILURE.

If the MLME successfully transmits an EGTS information request command, the MLME will expect an acknowledgment in return. If an acknowledgment is not received, the MLME will issue the MLME-EGTSinfo.confirm primitive with a status of NO_ACK (see 7.5.6.4?).

If an acknowledgment is received, the MLME will request that the PHY enable its receiver.

If a frame is received from the Destination device, the MLME will issue the MLME-EGTSinfo.confirm primitive with a status of SUCCESS.

If a frame is not received within macMaxFrameTotalWaitTime CAP symbols in a beacon-enabled PAN, or symbols in a nonbeacon-enabled PAN, the MLME will issue the MLME-EGTSinfo.confirm primitive with a status of NO_DATA.

If any parameter in the MLME-EGTSinfo.request primitive is not supported or is out of range, the MLME will issue the MLME-EGTSinfo.confirm primitive with a status of INVALID_PARAMETER.

Insert new subclause ‘7.1.18.2 MEME-EGTSinfo.confirm’:

The MLME-EGTSinfo.confirm primitive reports the results of a request for the timestamp and the EGTS parameters.

Insert new subclause ‘7.1.18.2.1 Semantics of the service primitive’:

The semantics of the MLME-GTSinfo.confirm primitive are as follows:

MLME-GTSinfo.confirm     (

EGTSCharacteristics,

Timestamp,

status

)

Table XX specifies the parameters for the MLME-GTSinfo.confirm primitive.

Table XX—MLME-EGTSinfo.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| EGTSCharacteristics | EGTS Characteristics | See 7.3.10.2 | The characteristics of the EGTS. |
| Timestamp | Integer | 0x000000–0xffffff | The time, in symbols, at which the data were transmitted/received??? (see 7.5.4.1). The value of this parameter will be considered valid only if the value of the status parameter is SUCCESS; if the status parameter is not equal to SUCCESS, the value of the Timestamp parameter shall not be used for any other purpose. The symbol boundary is described by macSyncSymbolOffset (see Table 86 in 7.4.1). This is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. |
| Status | Enumeration | SUCCESS, CHANNEL_ACCESS_FAILURE, NO_ACK, NO_DATA, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY or INVALID_PARAMETER. | The status of the EGTS information request. |

Insert new subclause ‘7.1.18.2.2 When generated’:

The MLME-GTSinfo.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME-GTSinfo.request primitive. If the request was successful, the status parameter will be equal to SUCCESS and the EGTS Characteristics Type field of the EGTSCharacteristics parameter will be set to Restart. Otherwise, the status parameter indicates the appropriate error code. The status values are fully described in ??? (MLME-GTSinfo.request Effect on receipt subclause).

Insert new subclause ‘7.1.18.2.3 Appropriate usage’:

On receipt of the MLME-GTSinfo.confirm primitive the next higher layer is notified of the result of the procedure to request the timestamp and the EGTS parameters from the Destination device.

Insert new subclause ‘7.1.18.2.4 EGTS information sequence chart’:

Figure 3 illustrates the sequence of messages necessary for successful EGTS information request. Figure 3 depicts the messages flow for the case in which the Source device requests the timestamp and the EGTS parameters from the Destination device.

Source device next higher layer

Source device MLME

Destination device MLME

MLME-GTSinfo.request

GTS information

MLME-GTSinfo.confirm

Acknowledgement

Acknowledgement

GTS information request

Figure 3 Message sequence chart for EGTS information request

  - MLME-BEACON-NOTIFY.indication

Insert new elements of PANDescriptor in ‘Table 55 – Elements of PANDescriptor‘

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| EGTS Superframe Spec | Bitmap |  | The EGTS superframe specification as specified in the received beacon frame. |
| BeaconBitmap | Bitmap |  |  |

- Modified MAC command frames

  - Beacon fields

Insert in ‘Figure 44’ :

| Octets: 2 | 1 | 4/10 | 0/5/6/10/14 | 2 | variable | variable | Variable | 4 | 4 | variable | variable | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Sequence Number | Addressing Fields | Auxiliary Security Header | Superframe Specificiation | GTS (Figure 45) | Pending address fields (Figure 46) | ChannelDiversitySpecification (Figure 44a) | EGTS Superframe Specification (Figure 52) | Time synchronization specification (Figure 54) | Beacon Bitmap (Figure 53) | Beacon Payload | FCS |
| MHR |  |  |  | MAC Payload |  |  |  |  |  |  |  | MFR |

Figure 44－Beacon frame format

Insert new Figure 44a:

| Bits: 0 | 1-7 | 8-15 | 16-23 | variable |
| --- | --- | --- | --- | --- |
| Channel Diverstiy Mode | Reserved | Channel Offset | ChannelOffset Bitmap Length | ChannelOffset Bitmap |

Figure 44a—Format of the ChannelDiversitySpecification fields

Insert ‘7.2.2.1.8 EGTS Superframe Specification field’ :

The EGTS Superframe Specification field is 4 octets in length and shall be formatted as illustrated in Figure 52.

| Bits: 0-3 | 4 | 5 | 6 | 7 | 8-23 | 24-31 |
| --- | --- | --- | --- | --- | --- | --- |
| Multi-superframe Order (MO) | EGTS Flag | CAP Reduction Flag | Embedded CAP/CFP flag | Reserved | CAP Index | Number of Sub-slots |

Figure 52－EGTS Superframe Specification Field

The Multi-superframe Order subfield is 4 bits in length and shall specify the length of time during which a group of superframes that is considered as one multi-superframe. See 7.5.1.1 for an explanation of the relationship between the Multi-superframe Order and the multi-superframe duration.

The EGTS Flag subfield is 1 bit in length. If this value is FALSE, the CFP of a superframe is operated the same way as defined in IEEE 802.15.4-2006. If this value is TRUE, the CFP of a superframe is operated as defined in 7.5.9 and 7.5.10.

The CAP Reduction Flag is 1 bit in length and shall specify whether the CAP Reduction is enabled or disabled (see 7.5.9). If this value is TRUE, the CAP Index field shall indicate the number of superframes before the next CAP begins.

The Embedded CAP Flag is 1 bit in length. If this value is ‘0’, the Embedded CAP is used (see 7.5.9). If this value is ‘1’, the Embedded CFP is used (see 7.5.9) and the Number of Sub-slots field shall indicate the number of sub-slots that are divided within a slot.

Insert ‘7.2.2.1.9 Beacon Bitmap field’:

The Beacon Bitmap Specification field is 8 bits in length and shall be formatted as illustrated in Figure 53.

| Octets: 2 | variable |
| --- | --- |
| SD Index | SD Bitmap |

Figure 53－Beacon Scheduling Specification Field

The SD Index subfield is 2 octets in length and specifies the SD bank number that is allocated to the Source device of the beacon.

The SD Bitmap subfield is 2(BO-SO) bits in length and indicates the beacon frame allocation information of neighbor node. This field is expressed by bitmap method which orderly represents the schedule of beacons. Corresponding bit shall be set to one if a beacon is allocated in that SD.

Insert ‘7.2.2.1.9 Time Synchronization Specification field’ :

Time Synchronization Specification field is 4 octets in length and shall be formatted as illustrated in Figure 54.

| Bits: 0 | 1-4 | 5-7 | 8-31 |
| --- | --- | --- | --- |
| Deferred Beacon flag | Deferred Beacon time | Reserved | Beacon Timestamp |

Figure 54－EGTS Superframe Specification Field

The Deferred beacon flag subfield is 1 bit in length and specifies whether CCA is required before transmitting beacon frame. If this subfield is set to one, device shall use CCA before transmitting beacon frame. If this subfield is set to zero, the device shall not use CCA before transmitting beacon.

The deferred beacon time subfield is 4 bits in length and specifies the number of backoff period for CCA. If deferred beacon flag is set to zero, this subfield shall be ignored.

The beacon timestamp subfiled is 3 octets in length and specifies the time of beacon transmission for time synchronization in symbol periods.

  - Association

    - Association request command

Modify the Figure 55:

The association request command shall be formatted as illustrated in Figure 55.

| octets: (see 7.2.2.4) | 1 | 1 | 1 |
| --- | --- | --- | --- |
| MHR fields | Command Frame Identifier (see Table 82) | Capability Information | Channel Offset |

Figure 55—Association request command format

Insert in 7.3.1.2:

The Channel Sequence Request subfield in the Capability Information field is 1 bit in length and shall be set to ‘1’ if the PAN is running on beacon-enabled mode and Channel Hopping mode.

Modify the Figure 56:

| bits: 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alternate PAN Coordinator | Device Type | Power Source | Receiver On When Idle | Channel Sequence Request | Reserved | Security Capability | Allocate Address |

Figure 56—Capability Information field format

Insert new subclause ‘7.3.1.3 Channel Offset field’:

The Channel Offset subfield is 8 bits in length and shall be set offset value specified by the next higher layer.

    - Association respond command

Modify the Figure 57:

The association response command shall be formatted as illustrated in Figure 57.

| octets: (see 7.2.2.4) | 1 | 2 | 1 | 1 | Variable |
| --- | --- | --- | --- | --- | --- |
| MHR fields | Command Frame Identifier (see Table 82) | Short Address | Association Status | Channel Hopping Sequence Length | Channel Hopping Sequence |

Figure 57—Association response command format

Insert new subclause ‘7.3.2.4 Channel Hopping Sequence Length’:

Channel Hopping Sequence Length subfield is 8 bits in length and shall specify the length of channel hopping sequence used in the PAN, if the PAN runs on beacon-enabled mode and Channel Hopping mode.

Insert new subclause ‘7.3.2.5 Channel Hopping Sequence’:

Channel Hopping Sequence subfield whos length is given by Channel Hopping Sequence Length subfield specifies the channel hopping sequence used in the PAN, if the PAN runs in beacon-enabled mode and Channel Hopping mode. (See Table 86)

- New MAC command frames

Insert in Table 82:

| Command Frame Identifier | Command Name | RFD |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | Tx | Rx |  |
| 0x0a | EGTS handshake | X | X |  |
| 0x0b | EGTS information | X | X |  |
| 0x0c | Beacon allocation notification |  |  |  |
| 0x0d | Beacon conflict notification | X |  |  |

  - EGTS handshake

Insert new subclause ‘7.3.10 EGTS handshake’:

The EGTS handshake command is used by an associated device that is requesting the allocation of a new EGTS or the deallocation, reallocation, or change of an existing EGTS from the Destination device. Only devices that have a 16-bit short address less than 0xfffe shall send this command.

This command is mandatory for 802.15.4e.

The EGTS request command shall be formatted as illustrated in Figure 3.

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier | EGTS Characteristics |

Figure 3—EGTS handshake command format

Insert new subclause ‘7.3.10.1 MHR fields’:

The Destination Addressing Mode subfield of the Frame Control field shall be set to two (e.g., 16-bit short addressing), and the Source Addressing Mode subfield shall also be set to two.

The Frame Pending subfield of the Frame Control field shall be set to zero and ignored upon reception, and the Acknowledge Request subfield shall be set to one.

The Source PAN Identifier field shall contain the value of macPANId, and the Source Address field shall contain the value of macShortAddress.

The Destination PAN Identifier field shall be set to 0xffff (e.g., broadcast PAN identifier), and the Destination Address field shall be set to 0xffff.

Insert new subclause ‘7.3.10.2 EGTS Characteristics fields’:

The EGTS Characteristics field shall be formatted as illustrated in Figure 4.

| Bit:0 | Bits: 1 | 2 | 3-10 | 11-13 | 14-15 | 16-31 | 32-47 | 48-51 | 52-67 | Variable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ChannelDiversityMode | EGTS Flag | EGTS Direction | EGTS Length | EGTS Characteristics Type | EGTS Handshake Type | Destination Address | EGTS slot identifier | EGTS ABT sub-block length | EGTS ABT sub-block index | EGTS ABT sub-block |

Figure 4—EGTS Characteristics field format

The ChannelDiversityMode subfield is 1 bit in length and shall be set to one of the nonreserved values listed in Table 1’ .

Table 1’—Values of the Channel Diversity Mode subfield

| Channel Diversity Mode value b0 | Description |
| --- | --- |
| 0 | Channel Adaptation mode |
| 1 | Channel Hopping mode |

The EGTS Flag subfield shall be set to ‘1’ to use EGTS or set to ‘0’ to use GTS.

The EGTS Length subfield shall contain the number of superframe slots being requested for the EGTS.

The EGTS Characteristics Type subfield is 3 bits in length and shall be set to one of the nonreserved values listed in Table 2.

Table 2—Values of the EGTS Characteristics Type subfield

| EGTS Characteristics Type value b2b1b0 | Description |
| --- | --- |
| 000 | Deallocation |
| 001 | Allocation |
| 010 | Reallocation |
| 011 | Duplicated Allocation Notification |
| 100 | Suspend |
| 101 | Reduce |
| 110 | Restart |
| 111 | Reserved |

The EGTS Handshake Type subfield is 2 bits in length and shall be set to one of the nonreserved values listed in Table 3.

Table 3—Values of the EGTS Handshake Type subfield

| EGTS Handshake Type value b1b0 | Description |
| --- | --- |
| 00 | Request |
| 01 | Reply |
| 10 | Notify |
| 11 | Reserved |

The Priotized Channel Access shall be set to one if EGTS should be reserved as high priority, or set to zero if EGTS should be reserved as low priority. When the EGTS request command is used in the EGTS change procedure, the Prioritized Channel Access shall be set according to the original EGTS.

The Destination Address shall contain the short address of the Destination device.

The EGTS slot identifier shall contain the channel number (1 octet) and beginning time slot number (1 octet) of the slots that are to be allocated or deallocated.

The EGTS ABT sub-block length shall contain the length of the EGTS ABT sub-block in units defined in Figure 5.

The EGTS ABT sub-block index shall indicate the beginning of the ABT Sub-block in the entire ABT as illustrated in Figure 5.

The EGTS ABT sub-block shall contain the sub-block of the Allocation Bitmap Table as illustrated in Figure 5.

Figure 5—ABT Sub-block

When Channel Hopping mode is used (i.e., ChannelDiversityMode is 1), TABs are exchanged between the Sender device and the Destination device as illustrated in Figure 5a.

Figure 5a—Timeslot Allocation Bitmap (TAB)

  - EGTS information request

Insert new subclause ‘7.3.11 EGTS information request’:

The EGTS information request command is used by a Source device that is requesting the timestamp and the EGTS parameters from the Source device.

The EGTS information request command shall be formatted as illustrated in Figure 6.

This command is mandatory for 802.15.4e.

| octets: 7 | 1 |
| --- | --- |
| MHR fields | Command Frame Identifier |

Figure 6 EGTS information request command format

The Acknowledgment Request subfield of the Frame Control field shall be set to one.

  - Beacon allocation notification

The Beacon allocation notification command is used by a device that selects vacant SD for using transmission of beacon frame.

The Beacon allocation notification command shall be formatted as illustrated in Figure 1.

| Octets: 7 | 1 | 2 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier | Allocation beacon SD index |

Figure 1 Beacon collision notification command format

The allocation beacon SD Index subfield shall contain the allocating SD index number for beacon frame.

  - Beacon collision notification

The Beacon collision notification command is used by a device that detects the collision of beacon frame.

The Beacon collision notification command shall be formatted as illustrated in Figure 2.

| Octets: 7 | 1 | 2 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier | collision SD Index |

Figure 2 Beacon collision notification command format

The Conflict SD Index subfield shall contain the SD index number of collision beacon frame.

- New MAC PIBs

Insert in Table 86:

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macEGTSFlag | 0x60 | Boolean | TRUE or FALSE | If this value is FALSE, the operation of this primitive is the same way defined in IEEE 802.15.4-2006 (definition of the GTSCharacteristics parameter see 7.3.9.2). If this value is TRUE, indicate that the request is for EGTS, the GTSCharacteristics parameter will use the definition of EGTSCharacteristics (see 7.3.10.2). |  |
| macChannelDiversityMode | 0x61 | Integer | 0-1 | Indicates the type of channel diversity mode: 0x00 = Channel Adaptation (default). 0x01 = Channel Hopping (optional). This value is not valid for a non-beacon enabled PAN. | 0x00 |
| macMultisuperframe Order | 0x62 | Integer | 0-15 | The length of a multisuperframe which is a cycle of the repeated EGTS schedule. | 15 |
| macConnecDev | 0x63 | Boolean | TRUE or FALSE | Indication of whether the device is a Connection Device. A value of TRUE indicates that the device is a Connection Device. | TRUE |
| macEGTSABT | 0x64 | Set of octets | See Figure 3 in subclause 7.3.10.2 | The allocation bitmap table of the EGTS schedule. | 0 |
| macEGTSACT | 0x65 | Set of octets |  | The allocation counter table of the EGTS allocated to the device. | 0 |
| macSDindex | 0x66 | Integer |  |  |  |
| macSDBitmap | 0x67 | Set of octets |  |  |  |
| macChannelHoppingSequence | 0x68 |  |  |  |  |
| macChannelOffset | 0x69 |  |  |  |  |
| macDeferredBeaconUsed | 0x6a | Boolean | TRUE or FALSE | Indication of whether the device uses CCA before transmitting beacon frame. A value of TRUE indicates that the device use CCA before transmitting beacon frame | FALSE |
| macSyncParent-ExtendedAddress | 0x6b | IEEE address | An extended 64-bit IEEE address | The 64-bit address of the coordinator through which the device is synchronized | - |
| macSyncParent ShortAddress | 0x6c | Integer | An extended 64-bit IEEE address | The 16-bit short address assigned to the coordinator through which the device is synchronized. A value of 0xfffe indicates that the coordinator is only using its 64-bit extended address. A value of 0xffff indicates that this value is unknown | 0xffff |
| macSyncParentSDIndex | 0x6d | Integer |  | Indication of SD |  |

- New Functional Description

  - EGTS-based Superframe Structure

Insert new subclause ‘7.5.9 EGTS-based Superframe Structure’:

The structure of EGTS-based superframe is described by the values of macBeaconOrder, macSuperframeOrder, and macMulti-superframeOrder. The MAC PIB attribute macBeaconOrder describes the interval at which the coordinator shall transmit its beacon frames. The value of macBeaconOrder, BO, and the beacon interval, BI, are related as follows: for 0 ≤ BO ≤ 14, BI = aBaseSuperframeDuration * 2BO symbols. If BO = 15, the coordinator shall not transmit beacon frames except when requested to do so, such as on receipt of a beacon request command. The value of macSuperframeOrder shall be ignored if BO = 15.

The MAC PIB attribute macSuperframeOrder describes the length of the superframe, which includes a beacon frame, a CAP period, and a CFP period. The value of macSuperframeOrder, SO, and the superframe duration, SD, are related as follows: for 0 ≤ SO ≤ BO ≤ 14, SD=aBaseSuperframeDuration * 2SO symbols.

The MAC PIB attribute macMulti-superframeOrder describes the length of a multi-superframe, which is a cycle of the repeated EGTS allocation schedule. The value of macSuperframeOrder, MO, and the superframe duration, MD, are related as follows: for 0 ≤ SO ≤ MO ≤ BO ≤ 14, MD=aBaseSuperframeDuration * 2MO symbols.

Each superframe shall be divided into aNumSuperframeSlots equally spaced slots of duration 2SO * aBaseSlotDuration and is composed of three parts: a beacon, a CAP and a CFP. The beacon shall be transmitted, without the use of CSMA, at the start of slot 0, and the CAP shall commence immediately following the beacon. The start of slot 0 is defined as the point at which the first symbol of the beacon PPDU is transmitted. The CFP, if present, follows immediately after the CAP and extends to the end of the active portion of the superframe. Any allocated EGTSs shall be located within the CFP.

An example of a superframe structure is shown in Figure 7. In this case, the beacon interval, BI, is eight times as long as the superframe duration, SD, and twice as long as the multi-superframe duration, MD.

Figure 7—EGTS-based Superframe Structure

In Channel Hopping mode (i.e., ChannelDiversityMode is ‘1’), channels and EGTS slots are determined by the channel hopping sequence and channel hopping offset values. Each device shall maintain series of timeslots whose channels are determined by channel hopping sequence and channel hopping offset values.

The usage of channels and EGTS slots in Channel Hopping mode is illustrated in Figure 7a.

Figure 7a—Illustration of channel and EGTS slot usage in EGTS-based Superframe Structure

In this example, channel hopping sequence is {1, 2, 3, 4, 5, 6} and the channel hopping offset values of two devices are 0 and 2 respectively. For the device with channel hopping offset value of 0, channels and EGTS slots can be represented in terms of EGTS slotidentifier (channel, time slot), i.e.,  (1, 1), (2, 2), (3, 3), (4, 4), (5, 5), (6, 6), (7, 1), (8, 2), (9, 3), and so on. Similarily, for the device with channel hopping offset value of 2, channels and EGTS slots are given as (1, 3), (2, 4), (3, 5), (4, 6), (5, 1), (6, 2), and so on.

Thus, channel used for the given EGTS slot shall be determined as:

C(time slot index) = CHSeq[ (time slot index + CHOffset) % CHSeqLength ]

Meanwhile, the number of EGTS slots in a multi-superframe is given by:

S = (7*2^(MO-SO)) slots

In Channel Hopping mode, each device exchanges the TABs in EGTS Scheduling procedure.

For instance, the Sender device shall transmit EGTS request command frame with TAB to the Destination device to request EGTS slot allocation. TAB shall represent the channel and EGTS slot usage of the Sender device by setting the bit values either to ‘1’ or ‘0’. Each bit shall be set to ‘1’ if the corresponding slot is already allocated to transmit or receive, otherwise shall be set to ‘0’ to denote that the slot is available.

In many application systems, it may be imperative to provide the sensor nodes with a retransmission opportunity, within the same superframe, for a data frame that failed in its GTS transmission. To satisfy that crucial requirement, the CFP of the MAC superframe shell be organized into several sub-periods as show in Figure 8. Specifically, these sub-periods are standard CFP (or SCFP) slots, a group acknowledgement (GACK 1) slot, extended CFP (ECFP) slots, and another group acknowledgement (GACK 2) slot. Beacon frame, transmitted by the coordinator at the beginning of the slot 0, shell specify the information about the CFP. That information shell be used to determine the start of the SCFP and its duration. For the failed GTS transmissions in SCFP, the coordinator shell dynamically allocate new time slots, in the ECFP, and inform the sensor nodes of these new allocations by transmitting the GACK 1 frame at the end of the SCFP. In addition to transmitting the information about dynamic resource assignment (i.e. allocation of time slots for use in ECFP), the GACK 1 frame shell also include a group acknowledgement (in the form of a bitmap) for the GTS frame successfully received by the coordinator during the SCFP. The use of group acknowledgement bitmap eliminates the need for the coordinator to switch over from Rx mode to Tx mode and then back to Rx mode while acknowledgeing individual GTS transmissions. Based on the resource allocation, as specified in the GACK 1 frame, the sending nodes shell retransmit their data frames in the allocated time slots in the ECFP. The coordinator shell transmit the GACK 2 frame after the completion of the ECFP. The GACK 2 frame shell contain the bitmap only indicating successful and failed reception of GTS frames during the ECFP. If the GACK 1 frame contains no resource assignments, the ECFP and GACK 2 shell be non-existant in that superframe.

Figure 8 – EGTS Superframe with ECFP and GACK

In addition to allow a retransmission of failed GTS transmissions in the SCFP, the ECFP shel also be used for allocating additional time slots on-demand to requesting nodes. A node shell request an additional GTS in the ECFP by setting a flag in the primitive while forwarding its data frame in the SCFP. The coordinator shell decide, based on the availability of time slots and/or priority mechanism, if to allocate the requested additional slots. The requesting node shell find the result of its request by checking the resource allocation in the following GACK 1 frame.

The structure of the GACK frame, as shown in Figure 9, shell contain the following fields.

PAN ID: This field shell identify the PAN of the transmitting coordinator.

Source ID: This field shell identify the transmitting coordinator in the PAN.

Group Ack Flags: It is a bitmap that indicates the state of transmission in each GTS in previous SCFP or ECFP. A bit having ‘1’ indicates the fact that the coordinator received the data frame successfully in the corresponding GTS. A ‘0’ means that the coordinator failed in receiving a data frame in the corresponding slot.

Channel Index: This field specifies the channel sequence to be followed in the ECFP or a tailing CAP, if allowed in a system, in a channel hopping system. This field shell be non-existant in the GACK 2 frame.

EGTS Device List: This list identifies the sensor nodes that are being allocated the time slots in ECFP portion of the superframe. This field shell be non-existant in the GACK 2 frame.

EGTS Index: It is a list that specifies the start of each GTS for the allocated nodes in the same order as in EGTS device list. This field is applicable only in those systems that allow a GTS to consist of multiple time slots. This field shell be non-existant in the GACK 2 frame.

EGTS Directions: This list specifies the direction of transmission (uplink or downlink) for each GTS. This is applicable only in the systems that allow the coordinator to transmit a frame to its sensor nodes by using a GTS. This field shell be non-existent in the GACK 2 frame.

CFP

GACK 1

GACK 2

ECFP

Beacon

Group

ACK

Flags

Channel

Index

PAN

ID

EGTS

Device

List

Source

ID

EGTS

Index

EGTS

Directions

Superframe

CAP

CFP

CAP

GACK 2

ECFP

GACK 1

Superframe

Beacon Interval

Figure 9 – GACK Frame Structure

The MAC sublayer shall ensure that the integrity of the superframe timing is maintained, e.g., compensating for clock drift error.

PANs that wish to use the superframe structure (referred to as a beacon-enabled PAN) shall set macBeaconOrder to a value between 0 and 14, both inclusive, and macSuperframeOrder to a value between 0 and the value of macBeaconOrder, both inclusive.

PANs that do not wish to use the superframe structure (referred to as a nonbeacon-enabled PAN) shall set both macBeaconOrder and macSuperframeOrder to 15. In this case, a coordinator shall not transmit beacons, except upon receipt of a beacon request command; all transmissions, with the exception of acknowledgment frames and any data frame that quickly follows the acknowledgment of a data request command (see 7.5.6.3), shall use an unslotted CSMA-CA mechanism to access the channel. In addition, EGTSs shall not be permitted.

The CAP reduction can be enabled by setting CAP reduction flag to ‘1’ in the EGTS superframe descriptor of a beacon frame. If the CAP reduction is enabled, every superframe except the first superframe in every multi-superframe does not have a CAP.  Figure 10 shows an example of a superframe structure when CAP reduction is enabled.

Figure 10—CAP Reduction in EGTS-based Superframe Structure

  - EGTS Scheduling

Insert new subclause ‘7.5.10 EGTS allocation and management’:

An EGTS (Enhanced Guaranteed Time Slot) allows a device to operate on the channel within a portion of the superframe that is dedicated (on the PAN) exclusively to that device. An EGTS shall be allocated by the destination device, and it shall be used only for communications between the source device and the destination device. A single EGTS may extend over one or more superframe slots.

An EGTS shall be allocated before use, with the destination device deciding whether to allocate an EGTS based on the requirements of the EGTS request and the current available capacity in the superframe. EGTSs shall be allocated on a first-come-first-served basis, and all EGTSs shall be placed contiguously at the end of the superframe and after the CAP (or after the beacon slot if CAP reduction is enabled). Each EGTS shall be deallocated when the EGTS is no longer required, and an EGTS can be deallocated at any time by the Destination device or the Source device that originally requested the EGTS. A device that has been allocated an EGTS may also operate in the CAP.

A data frame transmitted in an allocated EGTS shall use only short addressing.

The management of EGTSs shall be undertaken by the Destination device and the Source device. To facilitate EGTS management, the Destination device and the Source device shall be able to store all the information necessary to manage EGTSs. For each EGTS, the Destination device and the Source device shall be able to store its starting slot, length, and associated device address.

Insert new subclause ‘7.5.10.1 EGTS allocation’:

A device is instructed to request the allocation of a new EGTS through the MLME-GTS.request primitive with EGTS characteristics set according to the requirements of the intended application.

To request the allocation of a new EGTS, the MLME of the Source device of the requested EGTS shall send an EGTS handshake command (see 7.3.10) to the Destination device. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to one (allocation) and the Handshake Type subfield shall be set to zero (request). The length subfield shall be set according to the desired characteristics of the required EGTS. The slot identifier can be set to indicate the preferable beginning slot to be allocated. The EGTS ABT (Allocation Bitmap Table) sub-block shall be set according to the allocation status of the one-hop neighborhood of the Source device.

On receipt of an EGTS handshake command indicating an EGTS allocation request, the Destination device shall first check if there is available capacity in the current multi-superframe, based on the ABT of the Destination device and the ABT sub-block in the EGTS handshake command frame from the Source device. The Multi-superframe shall have available capacity if enough vacant slots exist in both ABTs (of the Destination device and the Source device) to satisfy the requested length. EGTSs shall be allocated on a first-come-first-served basis by the Destination device provided there is sufficient bandwidth available. The Destination can allocate the EGTS with the beginning slot specified in the slot identifier of the command if available. Then, the Destination device shall broadcast an EGTS handshake command to its one-hop neighbors. The destination address subfield of the EGTS of the EGTS Characteristics field in the EGTS handshake command shall be set to the Source device. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to one (allocation) and the Handshake Type subfield shall be set to one (reply). If the EGTS was allocated successfully, the length field shall be set to the requested length and the ABT sub-block field shall be set to represent the newly allocated slots. Otherwise, the length field shall be set to zero.

On receipt of an EGTS handshake command indicating an EGTS allocation reply, the device shall process the EGTS descriptor.

If the address in the destination address subfield of the EGTS Characteristics field in the EGTS handshake command (allocation reply) does not match the device’s short address, the device updates its ABT reflecting the neighbor’s newly allocated EGTS. If the newly allocated EGTS is conflicting with the device’s own EGTS, the device shall send an EGTS handshake command to the origin of the command. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to three (duplicate allocation notification) and the Handshake Type subfield shall be set to one (request).

If the address in the destination address subfield in the EGTS Characteristics field in the EGTS handshake command (allocation reply) matches the device’s short address, the MLME of the device shall then notify the next higher layer of whether the GTS allocation request was successful. This notification is achieved when the MLME issues the MLME-GTS.confirm primitive with a status of SUCCESS (if the length match the requested length) or DENIED (if the length did not match the requested length). Then, the Source device shall broadcast an EGTS handshake command to the Source device’s one hop neighbors. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to one (allocation) and the Handshake Type subfield shall be set to two (notify).

On receipt of an EGTS handshake command indicating an EGTS allocation notify, the device shall process the EGTS descriptor. The device updates its ABT reflecting the neighbor’s newly allocated EGTS. If the newly allocated EGTS is conflicting with the device’s own EGTS, the device shall send an EGTS handshake command to the origin of the command. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to three (duplicate allocation notification) and the Handshake Type subfield shall be set to one (request).

On receipt of an EGTS handshake command indicating an EGTS allocation duplicate allocation notification, the device shall reallocate the EGTS (see 7.5.10.3).

An example of EGTS allocation is shown in Figure 11.

Figure 11— Three-way Handshake for EGTS Allocation

Insert new subclause ‘7.5.10.2 EGTS deallocation’:

The Source device of an EGTS is instructed to request the deallocation of an existing EGTS through the MLME-GTS.request primitive (see 7.1.7.1) using the characteristics of the EGTS it wishes to deallocate. The Destination device can request the deallocation of an existing EGTS if the EGTS is expired. From this point onward, the EGTS to be deallocated shall not be used by the device, and its stored characteristics shall be reset.

To request the deallocation of an existing EGTS, the MLME shall send the EGTS handshake command (see 7.3.10) to the associated device (the Source or Destination of the EGTS to be deallocated). The Characteristics Type subfield of the EGTS Characteristics field of the handshake shall be set to zero (i.e., deallocation), and the Handshake Type subfield shall be set to one (i.e., request). The length subfields shall be set according to the characteristics of the EGTS to deallocate.

On receipt of an EGTS handshake command with the Characteristics Type subfield of the EGTS Characteristics field set to zero (deallocation), the device shall attempt to deallocate the EGTS. If the EGTS characteristics contained in the command do not match the characteristics of a known EGTS, the device shall ignore the request. If the EGTS characteristics contained in the EGTS request command match the characteristics of a known EGTS, the MLME of the device shall deallocate the specified EGTS and notify the next higher layer of the change. This notification is achieved when the MLME issues the MLME-GTS.indication primitive (see 7.1.7.3) with a EGTS Characteristics parameter containing the characteristics of the deallocated EGTS and a Characteristics Type subfield set to zero. Then, the device shall send an EGTS handshake command to its one-hop neighbors. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to zero (i.e., eeallocation), and the Handshake Type subfield shall be set to one (i.e., reply). The length subfields shall be set according to the characteristics of the EGTS to deallocate.

On receipt of an EGTS handshake command indicating an EGTS deallocation reply, the device shall process the EGTS descriptor.

If the address in the destination address subfield of the EGTS handshake command does not match the device’s short address, the device updates its ABT reflecting the neighbor’s deallocated EGTS.

If the address in the destination address subfield of the EGTS handshake command matches the device’s short address, the MLME of the device shall then notify the next higher layer of whether the GTS deallocation request was successful. This notification is achieved when the MLME issues the MLME-GTS.confirm primitive with a status of SUCCESS (if the length match the requested length) or DENIED (if the length did not match the requested length). Then, the device shall broadcast an EGTS handshake command to the device’s one hop neighbors. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to one (Deallocation) and the Handshake Type subfield shall be set to two (Notify).

On receipt of an EGTS handshake command indicating an EGTS deallocation reply, the device shall process the EGTS descriptor.  The device updates its ABT reflecting the neighbor’s deallocated EGTS.

Insert new subclause ‘7.5.10.3 EGTS reallocation’:

A device (the Source or Destination of the EGTS) is instructed to request the reallocation of an existing EGTS through the MLME-GTS.request primitive with EGTS characteristics set according to the requirements of the intended application. Also, a device shall request the reallocation of an existing EGTS on receipt of EGTS handshake command with the Characteristics Type subfield of the EGTS Characteristics field set to three (Duplicate Allocation Notification).

To request the reallocation of a new EGTS, the MLME shall send an EGTS handshake command (see 7.3.10) to the associated device. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to two (reallocation) and the Handshake Type subfield shall be set to zero (request). The length subfield shall be set according to the desired characteristics of the required EGTS. The EGTS ABT (Allocation Bitmap Table) sub-block shall be set according to the allocation status of the one-hop neighborhood of the device. The EGTS slot identifier subfield shall be set according to the characteristics of the EGTS to reallocate. The direction subfield shall be set to zero if the device is the Source of the EGTS or set to one if the device is the Destination of the EGTS.

On receipt of an EGTS handshake command indicating an EGTS reallocation request, the device shall attempt to reallocate the EGTS. If the EGTS characteristics contained in the command do not match the characteristics of a known EGTS, the device shall ignore the request. If the EGTS characteristics contained in the EGTS request command match the characteristics of a known EGTS, the device shall first check if there is available capacity in the current Multi-superframe, based on the ABT of the Destination device and the ABT sub-block in the EGTS handshake command frame from the Source device. The Multi-superframe shall have available capacity if enough vacant slots exist in both ABTs (of the Destination device and the Source device) to satisfy the requested length. EGTSs shall be allocated on a first-come-first-served basis by the Destination device provided there is sufficient bandwidth available. If a new EGTS is successfully allocated, the existing EGTS shall be removed from the device’s ABT. Then, the device shall broadcast an EGTS handshake command to its one-hop neighbors. The destination address subfield of the EGTS of the EGTS Characteristics field in the EGTS Handshake command shall be set to the origin. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to two (reallocation) and the Handshake Type subfield shall be set to one (reply). The EGTS slot identifier shall be set according to the characteristics of the EGTS to reallocate. If the EGTS was reallocated successfully, the length field shall be set to the requested length and the ABT sub-block field shall be set to represent the newly allocated slots. Otherwise, the length field shall be set to ‘0’.

On receipt of an EGTS handshake command indicating an EGTS allocation reply, the device shall process the EGTS descriptor.

If the address in the destination address subfield of the EGTS Characteristics field in the EGTS handshake command (reallocation reply) does not match the device’s short address, the device updates its ABT reflecting the neighbor’s reallocated EGTS. If the newly allocated EGTS is conflicting with the device’s own EGTS, the device shall send an EGTS handshake command to the origin of the command. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to three (Duplicate Allocation Notification) and the Handshake Type subfield shall be set to three (request).

If the address in the destination address subfield in the EGTS Characteristics field in the EGTS handshake command (reallocation reply) matches the device’s short address, the MLME of the device shall then notify the next higher layer of whether the EGTS reallocation request was successful. This notification is achieved when the MLME issues the MLME-GTS.confirm primitive with a status of SUCCESS (if the length match the requested length) or DENIED (if the length did not match the requested length). Then, the Source device shall broadcast an EGTS handshake command to the device’s one hop neighbors. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to two (reallocation) and the Handshake Type subfield shall be set to two (notify).

On receipt of an EGTS handshake command indicating an EGTS reallocation notify, the device shall process the EGTS descriptor. The device updates its ABT reflecting the neighbor’s reallocated EGTS. If the newly allocated EGTS is conflicting with the device’s own EGTS, the device shall send an EGTS handshake command to the origin of the command. The Characteristics Type subfield of the EGTS Characteristics field of the EGTS handshake command shall be set to three (Duplicate Allocation Notification) and the Handshake Type subfield shall be set to one (request).

On receipt of an EGTS handshake command indicating an EGTS allocation duplicate allocation notification, the device shall reallocate the EGTS.

Insert new subclause ‘7.5.10.4 EGTS expiration’:

The MLME of the Destination device shall attempt to detect when a device has stopped using an EGTS using the following rules:

— The MLME of the Destination device of EGTS shall assume that a device is no longer using its EGTS if a data frame is not received from the device in the GTS at least every 2*n multi-superframes, where n is defined below.

—The MLME of the Source device of EGTS shall assume that a device is no longer using its EGTS if a data frame is not requested from the higher layer of the device at least every 2*n multi-superframes, where n is defined below.

The value of n is defined as follows:

n = 2(8-macBeaconOrder) 		0 ≤ macBeaconOrder ≤ 8

n = 1 				9 ≤ macBeaconOrder ≤ 14

Insert new subclause ‘7.5.10.5 EGTS retrive’:

If a loss of synchronization occurs either in CAP or CFP before its EGTS slots of current superframe starting, the Source device shall be instructed to request the timestamp and the parameters of its EGTS through the MLME-EGTSinfo.request primitive.

To request the timestamp and the EGTS parameters, the MLME shall send the EGTS information request command (see XXX) to the Destination device. Because the EGTS information request command contains an acknowledgment request (see 7.3.3.1), the Destination device shall confirm its receipt by sending an acknowledgment frame.

On receipt of an EGTS information request command, the MLME of the Destination device shall send the timestamp and the EGTS parameters to the Source device.

On receipt of the acknowledgment to the EGTS information request command, the Source device shall enable its receiver for at most macMaxFrameTotalWaitTime CAP symbols in a beacon-enabled PAN, or symbols in a nonbeacon-enabled PAN, to receive the data frame containing the timestamp and the EGTS parameters from the Destination device.

On receipt of a data frame containing the timestamp and the EGTS parameters, the MLME of the Source device shall then notify the next higher layer of the EGTS information request was successful. This notification is achieved when the MLME issues the MLME-EGTSinfo.confirm (see XXX) primitive with a status of SUCCESS. The Source device shall synchronize to the Destination device by using the received timestamp and then continue to use its EGTS slots during current superframe.

If no corresponding data frame for the device is received within this time, the MLME of the Source device shall notify the next higher layer of the failure. This notification is achieved when the MLME issues the MLME-EGTSinfo.confirm primitive with a status of FAILURE. The Source device shall either end the procedure or try again by issuing the MLME-EGTSinfo.request primitive again.

Insert new subclause ‘7.5.10.6 EGTS change’:

The Destination device allocates the EGTS to the Source device according to the priority level indicated in the EGTS request command sent by the Source device. If the Destination device receives an EGTS request command of higher priority when its EGTS slots have already been used out, it shall either reduce or suspend part of the EGTS slots which are being used for lower priority data transmission to allocate new EGTS for the higher priority data transmission. And when the EGTS for the higher priority data transmission is finished, the Destination device can restart the EGTS slots which were either reduced or suspended previously. The procedure of EGTS change shall be initiated when a Destination device wants to reduce, suspend or restart the EGTS slots.

EGTS change (i.e., suspend, reduce or restart) is initiated by the Destination device through the MLME-GTS.request primitive (see 1.7.3.9).

When an EGTS change is initiated by the next higher layer of the Destination device, the MLME shall receive the MLME-GTS.request primitive with the EGTS Characteristics type subfield of the EGTS Characteristics parameter of the request set accordingly (i.e., 100 SUSPEND, 101 REDUCE or 110 RESTART), and all other subfields set according to the characteristics of the EGTS which the Destination device requests the Source device to change its original EGTS to.

To request the change of an existing EGTS, the MLME of the Destination device shall send the EGTS request command (see 1.7.3.9) to the Source device. The EGTS Characteristics type subfield of the EGTS Characteristics field of the request shall be set accordingly, and all other subfields set according to the characteristics of the EGTS which the Destination device requests the Source device to change its original EGTS to. Because the EGTS request command contains an acknowledgment request (see 7.3.3.1), the Source device shall confirm its receipt by sending an acknowledgment frame. On receipt of the acknowledgment to the EGTS request command, the MLME of the Destination device shall notify the next higher layer of the change. This notification is achieved when the MLME issues the MLME-GTS.confirm primitive (see 7.1.7.2) with a status of SUCCESS and a EGTSCharacteristics parameter with its EGTS Characteristics type subfield set accordingly, and all other subfields set according to the characteristics of the EGTS which the Destination device requests the Source device to change its original EGTS to.

On receipt of an EGTS request command with the EGTS Characteristics type subfield of the EGTS Characteristics field set accordingly, and all other subfields set according to the characteristics of the EGTS which the Destination device requests the Source device to change its original EGTS to, the Source device shall immediately change its EGTS. The MLME of the Source device shall notify the next higher layer of the change. This notification is achieved when the MLME issues the MLME-GTS.indication primitive (see 7.1.7.3) with an EGTSCharacteristics parameter containing the EGTS Characteristics type subfield set accordingly, and all other subfields set according to the characteristics of the EGTS which the Destination device requests the Source device to change its original EGTS to.

Insert new subclause ‘7.5.10.7 Channel Diversity EGTS allocation’:

If the data to transmit in the EGTS requires the higher transmission reliability, the device is instructed to request the allocation of a new Channel Diversity EGTS through the MLME-EGTS.request primitive with the EGTS Characteristics type subfield of the EGTS characteristics parameter set to 010 (Reallocation).

To request the allocation of a new Channel Diversity EGTS, the MLME shall send the EGTS request command (see 1.7.3.9) to the Destination device with the EGTS Characteristics type subfield of the EGTS characteristics parameter set to 010 (Reallocation), and all other subfields shall be set according to the desired characteristics of the required Channel Diversity EGTS.

On receipt of the EGTS request command with the EGTS Characteristics type subfield of the EGTS characteristics parameter set to 010 (Reallocation), the Destination device shall first decide whether allocating the Channel Diversity EGTS or the regular EGTS based on its own availability. If the Destination device decides to allocate the Channel Diversity EGTS, it shall contain the channel sequence in the ABT of the EGTS reply command. If not, the Destination device shall allocate a regular EGTS. (p.s. The remaining part of the Channel Hopping EGTS allocation and usage are same with the regular EGTS.)

Within the Channel Diversity EGTS, the devices switch to the next channel every slot according to the channel sequence in the ABT.

In Channel Hopping mode, devices don't exchange the ABT sub-block. Instead, Timeslots Allocation Bitmaps (TABs) are exchaged between the devices to allocate the channels and EGTS slots. The values of start position and length of TAB are followed by the EGTS ABT Sub-block Length and Index parameters.

To allocate channels and EGTS slots in Channel Hopping mode, the Sender shall send the EGTS request command including transmission schedule TAB to the Destination device.

On receipt of a EGTS request command, the Destination device shall check its own TAB and ABT to find available channels and EGTS slots to allocate new EGTS slots.

Once the Destination device selects channel and EGTS slots for the Sender device, it shall generate new TAB which represent the allocated EGTS slots and reply back on EGTS reply command to the Source device. The Sender device updates its own TAB and ABT with allocated channels and EGTS slots. After updating TAB and ABT, the Sender device broadcasts EGTS notify command with this new TAB to adjacent devices. (See Figure 12.)

As in Channel Adaptation mode, there may be packet collisions, since neighbor devices may use the same channel offset value as the Destination device. In order to avoid the collision, the neighbor devices using the same channel offset value shall update their ABT by overhearing EGTS notify command and EGTS reply command. Neighbor devices shall set the bitmap elements in their ABT to ‘1’ if the channel and EGTS slot are already allocated by the Source and the Destination Devices.

Figure 12—Illustration of DCH scheduling

  - Beacon Scheduling

When a new node wants to join a network, first it scans the channel. The new device uses the MLME-SCAN.request primitive in order to initiate a channel scan over a given list of channels. It searches for all coordinators transmitting beacon frames within the maximum BI period. Then these neighboring nodes would share their information of beacon bitmap with the new node. The beacon bitmap indicates the beacon frame allocation information for neighboring nodes. This field is expressed by bitmap method which orderly represents the schedule of beacons. Corresponding bit shall be set to 1 if a beacon is allocated in that SD. The new node will search the SD which is vacant (not set to 1) in all of the received beacon bitmap of beacon frame. Once new node finds vacant SD, It uses it as its own SD.

There can be beacon slot collision when two or more nodes are trying to compete for same SD slot number. As shown in Figure 13, node D and node E are new nodes that join the network.  These new nodes will receive the beacon bitmap from their neighbouring nodes. As it can be seen that node A is a common neighbouring node. Thus it can be the case when both new nodes E and D request same vacant SD number within same CAP. This happens due to hidden node problem, because node E and D are hidden to each other, and cannot listen each others transmission. When node A receives the SD number 2 request, within same CAP, by nodes E and D then it would deteremine which node has requested first. Node A will reply the beacon collision notification to the node which has requested later.

Figure 13: New node joining the network

  - Time Synchronization

The new node discovers its neighbouring node through the scanning process. Then it associate with one of the neighbouring node in order to be part of the network. The node with which the new node asscoiates is called its parent node.

Now the coordinator knows that the device would track its beacon. Thus the coordinator will determine the transmission time of its beacon. This beacon timestamp value is set just before transmitting the beacon frame. When the device gets this beacon timestamp value then it can synchronize with its paraent coordinator.

The effect of collision is inevitable in multiple nodes scenario, where more than one nodes try to use the same channel at the same time. In the case of collision, the node wait for some backoff duration and then it tries to re-send. Thus in case of collison, beacon transmission timestamp does not become valid. In order to avoid this problem, the coordinator sets macDefferedBeaconUsed value to be TRUE. When the device notice macDefferedBeaconUsed value to be TRUE then it knows that coordinator uses CCA for transmitting its beacon. Also the coodinator would add the number of tries it made for successful transmission. If the coordinator would be able to send its beacon after 3rd try then it would set DefferedBeaconFlag value to be 3. At the receipt of the beacon, the device would exactly knows when the beacon is sent (by adding beacon timestamp with (DefferedBeaconFlag value * 20 symbols) ). Thus perfect time syncronization becomes possible.

  - Scanning through channels

    - Passive channel scan

Insert in 7.5.2.1.3:

Channel Diversity Specification in the received beacon frame shall update the value of ChannelDiversitySpecification in PANDescriptor. This value is sent to the next higher layer via the MLME-SCAN.confirm primitive. The value of Channel Offset subfield in the received beacon shall update the value of macChannelOffsetBitmap in MAC PIB attributes. For instance, if Channel Offset is 0x01, the  value of macChannelOffsetBitmap corresonding channel shall set to ‘1’. Thus, the value of macChannelOffsetBitmap shall represent if the channel offset value is used among one hop neighbor devices.

  - Starting and realigning a PAN

    - Updating superframe configuration and channel PIB attributes

Insert in 7.5.2.3.4:

If a PAN uses EGTS and Channel Hopping mode is used (i.e., EGTSFlag is TRUE and ChannelDiversityMode is ‘1’), the MAC sublayer shall update the values of DCHDescriptor with the values of the DCHDescriptor parameter.

  - Beacon generation

Insert in 7.5.2.4:

If EGTSFlag is TRUE and Channel Hopping mode (i.e.,  ChannelDiversityMode is ‘1’) are used in the PAN, the MAC sublayer shall set the Channel Diversity Specification field of the beacon frame. The value of ChannelOffsetBitmap field, representing channel offset used among one hop neighbor devices, shall be set using  the value of macChannelOffsetBitmap in MAC PIB attributes.

  - Coexistence of beacon-enabled and nonbeacon-enabled mode

PANs that contain both the devices of beacon-enabled mode and the devices of nonbeacon-enabled mode shall also include the Connection Devices. The Connection Device works in the beacon-enabled mode and nonbeacon-enabled mode at the same time. The device of beacon-enabled mode shall either transmit periodic beacon or track the beacon for communication in the PAN. The device of nonbeacon-enabled mode shall neither transmit periodic beacon nor track the beacon for communication in the PAN. For example, a Connection Device can transmit periodic beacon to communicate with the devices of beacon-enabled mode in the PAN and request the data from the devices of nonbeacon-enabled mode in the PAN.