IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Required Changes for DCH over EGTS Structure |  |
| Date Submitted | Monday, May 11, 2009 (TBD) |  |
| Source | [Wun-Cheol Jeong, Chang-Sub Shin, Anseok Lee, Seong Soon Joo] [140th St. and Convent Ave, New York, NY, USA] | Voice: [+1 212-650-7260] E-mail: [lee@ccny.cuny.edu] |
| Re: | [] |  |
| Abstract | [This document contains proposed changes to the IEEE P802.15.4e Draft to address required changes for EGTS.] |  |
| Purpose | [] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Content

1.	New or Modified MAC Primitives	3

1.1	MLME-GTS	3

1.1.1	MLME-GTS.request	3

1.1.2	MLME-GTS.confirm	7

1.1.3	MLME-GTS.indication	8

1.2	MLME-START	10

1.3	MCPS-DATA	12

1.4	MLME-EGTSinfo	13

1.5	MLME-BEACON-NOTIFY.indication	18

2.	Modified MAC command frames	18

3.	Beacon fields	18

3.1	Association	21

3.1.1	Association request command	21

3.1.2	Association respond command	22

4.	New MAC command frames	22

4.1	EGTS handshake	23

4.2	EGTS information request	26

4.3	Beacon allocation notification	27

4.4	Beacon collision notification	27

5.	New MAC PIBs	27

6.	New Functional Description	30

6.1	EGTS-based Superframe Structure	30

6.2	EGTS Scheduling	32

6.3	Beacon Scheduling	36

6.4	Time Synchronization	37

6.5	Scanning through channels	38

6.5.1	Passive channel scan	38

6.6	Starting and realigning a PAN	38

6.6.1	Updating superframe configuration and channel PIB attributes	38

6.7	Beacon generation	38

6.8	Association	39

6.9	Coexistence of beacon-enabled and nonbeacon-enabled mode	39

- New or Modified MAC Primitives

Table 1. Changes in MAC primitives

| Primitive | Change | Request | Confirm | Response | Indication |
| --- | --- | --- | --- | --- | --- |
| MLME-GTS | Modified | X | X |  | X |
| MLME-START | Modified | X |  |  |  |
| MCPS-DATA | Modified | X |  |  |  |
| MLME-EGTSinfo | New | X | X |  |  |

  - MLME-GTS

    - MLME-GTS.request

Insert in 7.1.7.1:

If the value of the EGTS flag in the EGTSCharactersitics field set to ‘1’, the MLME-GTS.request primitive allows a Source device to send a request to a Destination device to allocate a new EGTS, or to deallocate / reallocate / change an existing EGTS. This primitive is also used by a Destination device to initiate an EGTS deallocation, reallocation, or change (suspend, reduce, or restart).

The MLME-GTS.request primitive allows a device to send a request to the PAN coordinator to allocate a new GTS slot or to the Destination device to allocate a new GTS slot. This primitive also used to deallocate GTS or EGTS.

Insert in 7.1.7.1.1:

MLME-EGTS.request	(

GTSCharacteristics,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

EGTSCharacteristics,

EGTSFlag

)

Insert in Table 58:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EGTSFlag | Boolean | TRUE or FALSE | If this value is FALSE, the operation of this primitive is the same way defined in IEEE 802.15.4-2006 (definition of the GTSCharacteristics parameter see 7.3.9.2). If this value is TRUE, indicate that the request is for EGTS, the GTSCharacteristics parameter will use the definition of EGTSCharacteristics (see 7.3.10.2). |
| EGTSCharacteristics | EGTSCharacteristics | See 7.3.10.2 | The characteristics of the EGTS request, including whether the request is for the allocation of a new EGTS or the deallocation / reallocation / change of an existing EGTS. |

Insert in 7.1.7.1.2:

The MLME-GTS.request primitive can also be generated by the next higher layer of a Source device and issued to its MLME to request the allocation of a new EGTS or to request the deallocation / reallocation / change of an existing EGTS. It is also generated by the next higher layer of the Destination device and issued to its MLME to request the deallocation, reallocation, or change of an existing EGTS.

Insert in 7.1.7.1.3:

If the value of the EGTSFlag equals to ‘0’, the effect of MLME-GTS.request is the same as it is described in Section 7.1.7.1 in IEEE Std 802.15.4-2006. If the value of the EGTS flag in the EGTSCharacteristics field is set to ‘1’, the effect of MLME-GTS.request is as follows.

On receipt of the MLME-GTS.request primitive by a Source device, the MLME of the Source device attempts to generate an EGTS handshake command with subtype (see 1.7.3.9) with the information contained in this primitive and, if successful, sends it to a Destination device.

On receipt of the MLME-GTS.request primitive by a Destination device, the MLME of the Destination device generates an EGTS request command (see 1.7.3.9) with the information contained in this primitive and sends it to the Source device.

If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for this frame, the MLME will set the Security Enabled subfield of the Frame Control field to one. The MAC sublayer will perform outgoing processing on the frame based on the DstAddress, SecurityLevel, KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.5.8.2.1. If any error occurs during outgoing frame processing, the MLME will discard the frame and issue the MLME-GTS.confirm primitive with the error status returned by outgoing frame processing.

If the EGTS request command cannot be sent due to the channel condition, the MLME will issue the MLME-GTS.confirm primitive with a status of CHANNEL_ACCESS_FAILURE.

If the MLME successfully transmits an EGTS handshake command, the MLME will expect an acknowledgment in return. If an acknowledgment is not received, the MLME will issue the MLME-GTS.confirm primitive with a status of NO_ACK (see 7.5.6.4).

If an EGTS is being allocated (see 7.5.7.2) and the request has been acknowledged, the device will wait for a confirmation via an EGTS handshake allocation reply command frame from the destination device. If the MLME of the Destination device can allocate the requested EGTS, it will issue the MLME-GTS.indication primitive with the characteristics of the allocated EGTS and generate an EGTS handshake allocation reply command frame. If the MLME of the PAN coordinator cannot allocate the requested EGTS, it will generate an EGTS handshake allocation reply command frame with EGTS length field set to ‘0’.

If an EGTS handshake allocation reply command frame and the EGTS length matches the characteristics requested (indicating that the Destination device has approved the EGTS allocation request), the MLME of the Source device will issue the MLME-GTS.confirm primitive with a status of SUCCESS and a EGTSCharacteristics parameter with a characteristics type equal to one, indicating a GTS allocation. If the EGTS handshake allocation reply command frame is received with a EGTS length of zero (indicating that the Destination device has denied the EGTS allocation request), the device requesting the EGTS issues the MLME-GTS.confirm primitive with a status of DENIED, indicating that the GTSCharacteristics parameter is to be ignored.

If an EGTS is being deallocated (see 7.5.7.4) at the request of a device and the request has been acknowledged by the other device, the device will issue the MLME-GTS.confirm primitive with a status of SUCCESS and an EGTSCharacteristics parameter with a characteristics type equal to ‘1’, indicating a GTS deallocation. On receipt of an EGTS handshake deallocation request command, the device will acknowledge the frame and deallocates the EGTS. The MLME of the device will then issue the MLME-GTS.indication primitive with the appropriate EGTS characteristics. If the Destination device does not receive the deallocation request, counter measures can be applied by the PAN coordinator to ensure consistency is maintained (see 7.5.7.6).

If an EGTS of the Source device is being changed at the request of a Destination device and the request has been acknowledged by the Source device, the Destination device will issue the MLME-GTS.confirm primitive with a status of SUCCESS and an EGTSCharacteristics parameter with an EGTSCharacteristics type subfield set to 100 SUSPEND, 101 REDUCE or 110 RESTART indicating an EGTS change according to the MLME-GTS.request primitive. On receipt of an EGTS handshake command with a request type indicating an EGTS change (100 SUSPEND, 101 REDUCE or 110 RESTART), the Source device will acknowledge the frame and change the EGTS. The MLME of the Source device will then issue the MLME-GTS.indication primitive with the appropriate EGTS characteristics.

If any parameter in the MLME-GTS.request primitive is not supported or is out of range, the MLME will issue the MLME-GTS.confirm primitive with a status of INVALID_PARAMETER.

On the receipt of the MLME-GTS.request primitive, the MLME of a device attempts to generate a GTS request command or an EGTS request command (See 1.7.3.9) depending on the EGTSMode parameter.

If EGTSFlag is TRUE, the MLME-GTS.request primitive allows a Source device to send a request to a Destination device to allocate or deallocate EGTS slots. This primitive is also used by a Destination device to initiate the EGTS deallocation.

If the ChannelDiversityMode subfield of EGTSCharacteristic parameter is ‘1’, the EGTS allocation and/or deallocation operate as Channel Hopping mode.

The main difference between Channel Adaptation mode and Channel Hopping mode is the EGTS ABT sub-field in EGTSCharacteristic parameter. In Channel Hopping mode, both the Source and the Desitination device exchange their Timeslot Allocation Bitmaps (TAB) in contrast to the ABT sub-block in Channel Adaptation mode.

    - MLME-GTS.confirm

Insert in 7.1.7.2:

If the value of the EGTSFlag equals to ‘1’, the MLME-GTS.confirm primitive reports the results of a request to allocate a new EGTS or to deallocate / reallocate / change an existing EGTS.

The MLME-GTS.confirm primitive reports the results of a request to allocate a new GTS or EGTS. This primitive also reports the result of request to deallocate an existing EGTS.

Insert in 7.1.7.2.1:

MLME-EGTS.confirm	(

GTSCharacteristics,

status,

EGTSFlag,

EGTSCharacteristics

)

Insert in Table 59:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EGTSFlag | Boolean | TRUE or FALSE | If this value is FALSE, the operation of this primitive is the same way defined in IEEE 802.15.4-2006 (definition of the GTSCharacteristics parameter see 7.3.9.2). If this value is TRUE, indicate that the request is for EGTS, the GTSCharacteristics parameter will use the definition of EGTSCharacteristics (see 7.3.10.2). |
| EGTSCharacteristics | EGTSCharacteristics | See 7.3.10.2 | The characteristics of the EGTS request, including whether the request is for the allocation of a new EGTS or the deallocation / reallocation / change of an existing EGTS. |

Insert in 7.1.7.2.2:

If the request to allocate, deallocate, reallocate, or change (suspend, reduce, or restart) an EGTS was successful, this primitive will return a status of SUCCESS and the EGTSCharacteristics type subfield of the EGTSCharacteristics parameter will be set accordingly. Otherwise, the status parameter will indicate the appropriate error code. The reasons for these status values are fully described in 7.1.7.1.3 and subclauses referenced by 7.1.7.1.3.

Insert in 7.1.7.2.3:

On receipt of the MLME-GTS.confirm primitive with the value of the EGTS flag in the EGTSCharactersitics field set to ‘1’, the next higher layer is notified of the result of its request to allocate, deallocate, or change an EGTS. If the request was successful, the status parameter will indicate a successful EGTS operation. Otherwise, the status parameter will indicate the error.

    - MLME-GTS.indication

Insert in 7.1.7.3:

If the value of the EGTSFlag equals to ‘1’, the MLME-GTS.indication primitive indicates that an EGTS has been allocated or that a previously allocated EGTS has been deallocated, reallocated, or changed.

Insert in 7.1.7.3.1:

MLME-EGTS.indication	(

DeviceAddress,

GTSCharacteristics,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

EGTSFlag,

EGTSCharacteristics

)

Insert in Table 60:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EGTSFlag | Boolean | TRUE or FALSE | If this value is FALSE, the operation of this primitive is the same way defined in IEEE 802.15.4-2006 (definition of the GTSCharacteristics parameter see 7.3.9.2). If this value is TRUE, indicate that the request is for EGTS, the GTSCharacteristics parameter will use the definition of EGTSCharacteristics (see 7.3.10.2). |
| EGTSCharacteristics | EGTSCharacteristics | See 7.3.10.2 | The characteristics of the EGTS request, including whether the request is for the allocation of a new EGTS or the deallocation / reallocation / change of an existing EGTS. |

Insert in 7.1.7.3.2:

This primitive is generated by the MLME of a Source or Destination device and issued to its next higher layer when an EGTS is allocated, deallocated, reallocated, or changed following the reception of an EGTS handshake command (see 1.7.3.9) by the MLME. The EGTS Characteristics type subfield of the EGTSCharacteristics parameter is set accordingly.

Insert in 7.1.7.3.3:

On receipt of the MLME-GTS.indication primitive with the value of the EGTS flag in the EGTSCharactersitics field set to ‘1’, the next higher layer is notified of the allocation, deallocation, reallocation, or change of a EGTS.

Insert in 7.1.7.4:

Figure 1 and Figure 2 illustrate the sequence of messages necessary for successful EGTS management. Figure 1 depicts the message flow for the case in which a Source device initiates the EGTS allocation, deallocation, reallocation, or change. Figure 2 depicts the message flow for the case in which a Destination device initiates the EGTS deallocation, reallocation, or change.

Figure 1—Message sequence chart for EGTS allocation initiated by a Source device

Figure 2—Message sequence chart for EGTS allocation initiated by a Destination device

  - MLME-START

Modify in 7.1.14.1.1:

MLME-START.request	(

PANId,

LogicalChannel,

ChannelPage,

SuperframeStartBank,

BeaconOrder,

SuperframeOrder

PANCoordinator

BatteryLifeExtension

CoorRealignment

CoorRealignSecurityLevel,

CoorRealignKeyIdMode,

CoorRealignKeySource

CoorRealignKeyIndex

BeaconSecurityLevel,

BeaconKeyIdMode,

BeaconKeySource

BeaconKeyIndex,

EGTSFlag,

ChannelDiversityMode,

DCHDescriptor

)

Insert in Table 72:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SuperframeStartBank | Integer | 0x00 – 0x1F | In superframe bank , the coordinator transmitis its own beacon frame. If this parameter is set to 0x00, beacon transmitting will begin immediately. Otherwise, the specified bank number is relative to the bank index of the received beacon with which the device synchronizes. This parameter is ignored if either the BeaconOrder parameter has a value of 15 or the PANCoordinator parameter is TRUE. |
| EGTSFlag | Boolean | TRUE or FALSE | If this value is FALSE, the operation of this primitive is the same way defined in IEEE 802.15.4-2006 (definition of the GTSCharacteristics parameter see 7.3.9.2). If this value is TRUE, indicate that the request is for EGTS, the GTSCharacteristics parameter will use the definition of EGTSCharacteristics (see 7.3.10.2). |
| ChannelDiversityMode | Integer | 0x00 – 0x01 | Indicates the type of channel diversity mode: 0x00 = Channel Adaptation (default) 0x01 = Channel Hopping (optional) This value is not valid for a non-beacon enabled PAN. |
| DCHDescriptor | DCHDescriptor Value |  | See Table 4. |

Table 4—Elements of DCHDescriptor

| Name | ID | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| CurrentChannelpage |  | Integer | 0-31 | The value of this attribute is set by the next higher layer. |  |
| CommonChannel |  | Integer |  | The value of this attribute is set by the next higher layer. |  |
| ChannelHoppingSequenceLength |  | Integer |  | The value of this attribute is set by the next higher layer. |  |
| ChannelHoppingSequece |  | Set of Octetets |  | The value of this attribute is set by the next higher layer. |  |
| ChannelOffset |  | Integer |  |  | 0 |
| ChannelOffsetBitmapLength |  | Integer |  |  | 0 |
| ChannelOffsetBitmap |  | Set of Octets |  |  | NULL |

Modify in 7.1.14.1.3:

If the SuperframeStartBank parameter is non-zero and the MLME is not currently tracking the beacon of the coordinator through which it is associated, the MLME will issue the MLME-START.confirm primitive with a status of TRACKING_OFF.

  - MCPS-DATA

Modify in Table 41:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TxOptions | Bitmap | 5-bit field | The 5 bits (b0, b1, b2, b3, b4) indicate the transmission options for this MSDU. For b0, 1 = acknowledged transmission, 0 = unacknowledged transmission. For b1, 1 = GTS transmission, 0 = CAP transmission for a beacon-enabled PAN. For a nonbeacon-enabled PAN, bit b1 should always be set to 0. For b2, 1 = indirect transmission, 0 = direct transmission. For b3, 1 = CAP/EGTS transmission, 0 = CAP/GTS transmission (defined in IEEE 802.15.4-2006). For b4, 1 = High Priority transmission, 0 = Low Priority transmission. |

Insert in 7.1.1.1.3:

If the TxOptions parameter specifies that an EGTS transmission is required, the MAC sublayer will set the EGTS flag in the EGTS Characteristics field to one, indicating the EGTS transmission, and determine whether it has a valid EGTS (for EGTS usage rules, see 7.5.10). If a valid EGTS could not be found, the MAC sublayer will issue the MCPS-DATA.confirm primitive with a status of INVALID_GTS. If a valid EGTS was found, the MAC sublayer will defer, if necessary, until the EGTS.

  - MLME-EGTSinfo

Insert new subclause ‘7.1.18 Primitives for requesting EGTS information’:

MLME-EGTSinfo defines how a device can request EGTS information.

Insert new subclause ‘7.1.18.1 MEME-EGTSinfo.request’:

The MLME-EGTSinfo.request primitive allows a Source device to request the timestamp and the parameters of its EGTS from the Destination device.

Insert new subclause ‘7.1.18.1.1 Semantics of the service primitive’:

The semantics of the MLME-EGTSinfo.request primitive are as follows:

MLME-EGTSinfo.request     (

DstAddrMode,

DstAddress,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table XX specifies the parameters for the MLME-EGTSinfo.request primitive.

Table XX—MLME-EGTSinfo.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DstAddrMode | Integer | 0x02–0x03 | The addressing mode of the Destination device to which the request is intended. This parameter can take one of the following values: 2 = 16-bit short address, 3 = 64-bit extended address. |
| DstAddress | Device-Address | As specified by the DstAddrMode parameter | The address of the Destination device to which the request is intended. SecurityLevel Integer 0x00–0x07 The security level to be used (see Table 95 in 7.6.2.2.1). |
| SecurityLevel | Integer | 0x00-0x07 | The security level to be used (see Table 95 in 7.6.2.2.1). |
| KeyIdMode | Integer | 0x00–0x03 | The mode used to identify the key to be used (see Table 96 in 7.6.2.2.2). This parameter is ignored if the SecurityLevel parameter is set to 0x00. |
| KeySource | Set of 0, 4, or 8 octets | As specified by the KeyIdMode parameter | The originator of the key to be used (see 7.6.2.4.1). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |
| KeyIndex | Integer | 0x01–0xff | The index of the key to be used (see 7.6.2.4.2). This parameter is ignored if the KeyIdMode parameter is ignored or set to 0x00. |

Insert in ‘7.1.18.1.2  Appropriate usage’:

The MLME-EGTSinfo.request primitive is generated by the next higher layer of a Source device and issued to its MLME when the timestamp and the parameters of its EGTS are to be requested from the Destination device.

Insert in ‘7.1.18.1.3  Effect on receipt’:

On receipt of the MLME-EGTSinfo.request primitive by a device, the MLME of the device generates and sends an EGTS information request command (see XXX). The EGTS information request command is generated with the destination address information in the DstAddress parameter.

If the SecurityLevel parameter is set to a valid value other than 0x00, indicating that security is required for this frame, the MLME will set the Security Enabled subfield of the Frame Control field to one. The MAC sublayer will perform outgoing processing on the frame based on the DstAddress, SecurityLevel, KeyIdMode, KeySource, and KeyIndex parameters, as described in 7.5.8.2.1. If any error occurs during outgoing frame processing, the MLME will discard the frame and issue the MLME-EGTSinfo.confirm primitive with the error status returned by outgoing frame processing.

If the EGTS information request command cannot be sent due to a CSMA-CA algorithm failure, the MLME will issue the MLME-EGTSinfo.confirm primitive with a status of CHANNEL_ACCESS_FAILURE.

If the MLME successfully transmits an EGTS information request command, the MLME will expect an acknowledgment in return. If an acknowledgment is not received, the MLME will issue the MLME-EGTSinfo.confirm primitive with a status of NO_ACK (see 7.5.6.4?).

If an acknowledgment is received, the MLME will request that the PHY enable its receiver.

If a frame is received from the Destination device, the MLME will issue the MLME-EGTSinfo.confirm primitive with a status of SUCCESS.

If a frame is not received within macMaxFrameTotalWaitTime CAP symbols in a beacon-enabled PAN, or symbols in a nonbeacon-enabled PAN, the MLME will issue the MLME-EGTSinfo.confirm primitive with a status of NO_DATA.

If any parameter in the MLME-EGTSinfo.request primitive is not supported or is out of range, the MLME will issue the MLME-EGTSinfo.confirm primitive with a status of INVALID_PARAMETER.

Insert new subclause ‘7.1.18.2 MEME-EGTSinfo.confirm’:

The MLME-EGTSinfo.confirm primitive reports the results of a request for the timestamp and the EGTS parameters.

Insert new subclause ‘7.1.18.2.1 Semantics of the service primitive’:

The semantics of the MLME-GTSinfo.confirm primitive are as follows:

MLME-GTSinfo.confirm     (

EGTSCharacteristics,

Timestamp,

status

)

Table XX specifies the parameters for the MLME-GTSinfo.confirm primitive.

Table XX—MLME-EGTSinfo.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| EGTSCharacteristics | EGTS Characteristics | See 7.3.10.2 | The characteristics of the EGTS. |
| Timestamp | Integer | 0x000000–0xffffff | The time, in symbols, at which the data were transmitted/received??? (see 7.5.4.1). The value of this parameter will be considered valid only if the value of the status parameter is SUCCESS; if the status parameter is not equal to SUCCESS, the value of the Timestamp parameter shall not be used for any other purpose. The symbol boundary is described by macSyncSymbolOffset (see Table 86 in 7.4.1). This is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. |
| Status | Enumeration | SUCCESS, CHANNEL_ACCESS_FAILURE, NO_ACK, NO_DATA, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY or INVALID_PARAMETER. | The status of the EGTS information request. |

Insert new subclause ‘7.1.18.2.2 When generated’:

The MLME-GTSinfo.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME-GTSinfo.request primitive. If the request was successful, the status parameter will be equal to SUCCESS and the EGTS Characteristics Type field of the EGTSCharacteristics parameter will be set to Restart. Otherwise, the status parameter indicates the appropriate error code. The status values are fully described in ??? (MLME-GTSinfo.request Effect on receipt subclause).

Insert new subclause ‘7.1.18.2.3 Appropriate usage’:

On receipt of the MLME-GTSinfo.confirm primitive the next higher layer is notified of the result of the procedure to request the timestamp and the EGTS parameters from the Destination device.

Insert new subclause ‘7.1.18.2.4 EGTS information sequence chart’:

Figure 3 illustrates the sequence of messages necessary for successful EGTS information request. Figure 3 depicts the messages flow for the case in which the Source device requests the timestamp and the EGTS parameters from the Destination device.

Source device next higher layer

Source device MLME

Destination device MLME

MLME-GTSinfo.request

GTS information

MLME-GTSinfo.confirm

Acknowledgement

Acknowledgement

GTS information request

Figure 3 Message sequence chart for EGTS information request

  - MLME-BEACON-NOTIFY.indication

Insert new elements of PANDescriptor in ‘Table 55 – Elements of PANDescriptor‘

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| EGTS Superframe Spec | Bitmap |  | The EGTS superframe specification as specified in the received beacon frame. |
| BeaconBitmap | Bitmap |  |  |

- Modified MAC command frames

- Beacon fields

Insert in ‘Figure 44’ :

| Octets: 2 | 1 | 4/10 | 0/5/6/10/14 | 2 | variable |  | variable |  | Variable | 4 | 4 | variable | variable | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Sequence Number | Addressing Fields | Auxiliary Security Header | Superframe Specificiation | GTS (Figure 45) |  | Pending address fields (Figure 46) |  | Channel Diversity Specification | EGTS Superframe Specification (Figure 52) | Time synchronization specification (Figure 53) | Beacon Bitmap (Figure 54) | Beacon Payload | FCS |
| MHR |  |  |  |  |  |  |  | MAC Payload |  |  |  |  |  | MFR |

Figure 44－Beacon frame format

Insert new Figure 46a:

| Bits: 0 | 1-7 | 8-15 | 16-23 | variable |
| --- | --- | --- | --- | --- |
| Channel Diverstiy Mode | Reserved | Channel Offset | ChannelOffset Bitmap Length | ChannelOffset Bitmap |

Figure 46a—Format of the ChannelDiversitySpecification fields

Insert ‘7.2.2.1.8 EGTS Superframe Specification field’ :

The EGTS Superframe Specification field is 4 octets in length and shall be formatted as illustrated in Figure 52.

| Bits: 0-3 | 4 | 5 | 6 | 7 | 8-23 | 24-31 |
| --- | --- | --- | --- | --- | --- | --- |
| Multi-superframe Order (MO) | EGTS Flag | CAP Reduction Flag | Embedded CAP/CFP flag | Reserved | CAP Index | Number of Sub-slots |

Figure 52－EGTS Superframe Specification Field

The Multi-superframe Order subfield is 4 bits in length and shall specify the length of time during which a group of superframes that is considered as one multi-superframe. See 7.5.1.1 for an explanation of the relationship between the Multi-superframe Order and the multi-superframe duration.

The EGTS Flag subfield is 1 bit in length. If this value is FALSE, the CFP of a superframe is operated the same way as defined in IEEE 802.15.4-2006. If this value is TRUE, the CFP of a superframe is operated as defined in 7.5.9 and 7.5.10.

The CAP Reduction Flag is 1 bit in length and shall specify whether the CAP Reduction is enabled or disabled (see 7.5.9). If this value is TRUE, the CAP Index field shall indicate the number of superframes before the next CAP begins.

The Embedded CAP Flag is 1 bit in length. If this value is ‘0’, the Embedded CAP is used (see 7.5.9). If this value is ‘1’, the Embedded CFP is used (see 7.5.9) and the Number of Sub-slots field shall indicate the number of sub-slots that are divided within a slot.

Insert ‘7.2.2.1.9 Time Synchronization Specification field’ :

Time Synchronization Specification field is 4 octets in length and shall be formatted as illustrated in Figure 53.

| Bits: 0 | 1-4 | 5-7 | 8-31 |
| --- | --- | --- | --- |
| Deferred Beacon flag | Deferred Beacon time | Reserved | Beacon Timestamp |

Figure 53－EGTS Superframe Specification Field

The Deferred beacon flag subfield is 1 bit in length and specifies whether CCA is required before transmitting beacon frame. If this subfield is set to one, device shall use CCA before transmitting beacon frame. If this subfield is set to zero, the device shall not use CCA before transmitting beacon.

The deferred beacon time subfield is 4 bits in length and specifies the number of backoff period for CCA. If deferred beacon flag is set to zero, this subfield shall be ignored.

The beacon timestamp subfiled is 3 octets in length and specifies the time of beacon transmission for time synchronization in symbol periods.

Insert ‘7.2.2.1.10 Beacon Bitmap field’:

The Beacon Bitmap Specification field is 8 bits in length and shall be formatted as illustrated in Figure 53.

| Octets: 2 | variable |
| --- | --- |
| SD Index | SD Bitmap |

Figure 52－Beacon Scheduling Specification Field

The SD Index subfield is 2 octets in length and specifies the SD bank number that is allocated to the Source device of the beacon.

The SD Bitmap subfield is 2(BO-SO) bits in length and indicates the beacon frame allocation information of neighbor node. This field is expressed by bitmap method which orderly represents the schedule of beacons. Corresponding bit shall be set to one if a beacon is allocated in that SD.

Insert ‘7.2.2.1.9 Time Synchronization Specification field’ :

Time Synchronization Specification field is 4 octets in length and shall be formatted as illustrated in Figure 53.

| Bits: 0 | 1-4 | 5-7 | 8-31 |
| --- | --- | --- | --- |
| Deferred Beacon flag | Deferred Beacon time | Reserved | Beacon Timestamp |

Figure 53－EGTS Superframe Specification Field

The Deferred beacon flag subfield is 1 bit in length and specifies whether CCA is required before transmitting beacon frame. If this subfield is set to one, device shall use CCA before transmitting beacon frame. If this subfield is set to zero, the device shall not use CCA before transmitting beacon.

The deferred beacon time subfield is 4 bits in length and specifies the number of backoff period for CCA. If deferred beacon flag is set to zero, this subfield shall be ignored.

The beacon timestamp subfiled is 3 octets in length and specifies the time of beacon transmission for time synchronization in symbol periods.

  - Association

    - Association request command

Modify the Figure 55:

The association request command shall be formatted as illustrated in Figure 55.

| octets: (see 7.2.2.4) | 1 | 1 | 1 |
| --- | --- | --- | --- |
| MHR fields | Command Frame Identifier (see Table 82) | Capability Information | Channel Offset |

Figure 55—Association request command format

Insert in 7.3.1.2:

The Channel Sequence Request subfield in the Capability Information field is 1 bit in length and shall be set to ‘1’ if the PAN is running on beacon-enabled mode and channel hopping mode.

Modify the Figure 56:

| bits: 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alternate PAN Coordinator | Device Type | Power Source | Receiver On When Idle | Channel Sequence Request | Reserved | Security Capability | Allocate Address |

Figure 56—Capability Information field format

Insert new subclause ‘7.3.1.3 Channel Offset field’:

The Channel Offset subfield is 8 bits in length and shall be set offset value specified by the next higher layer.

    - Association respond command

Modify the Figure 57:

The association response command shall be formatted as illustrated in Figure 57.

| octets: (see 7.2.2.4) | 1 | 2 | 1 | 1 | Variable |
| --- | --- | --- | --- | --- | --- |
| MHR fields | Command Frame Identifier (see Table 82) | Short Address | Association Status | Channel Hopping Sequence Length | Channel Hopping Sequence |

Figure 57—Association response command format

Insert new subclause ‘7.3.2.4 Channel Hopping Sequence Length’:

Channel Hopping Sequence Length subfield is 8 bits in length and shall specify the length of channel hopping sequence used in the PAN, if the PAN runs on beacon-enabled mode and Channel Hopping mode.

Insert new subclause ‘7.3.2.5 Channel Hopping Sequence’:

Channel Hopping Sequence subfield whos length is given by Channel Hopping Sequence Length subfield specifies the channel hopping sequence used in the PAN, if the PAN runs in beacon-enabled mode and channel hopping mode. (See Table 86)

- New MAC command frames

Insert in Table 82:

| Command Frame Identifier | Command Name | RFD |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | Tx | Rx |  |
| 0x0a | EGTS handshake | X | X |  |
| 0x0b | EGTS information | X | X |  |
| 0x0c | Beacon allocation notification |  |  |  |
| 0x0d | Beacon conflict notification | X |  |  |

  - EGTS handshake

Insert new subclause ‘7.3.10 EGTS handshake’:

The EGTS handshake command is used by an associated device that is requesting the allocation of a new EGTS or the deallocation, reallocation, or change of an existing EGTS from the Destination device. Only devices that have a 16-bit short address less than 0xfffe shall send this command.

This command is mandatory for 802.15.4e.

The EGTS request command shall be formatted as illustrated in Figure 3.

| Octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier | EGTS Characteristics |

Figure 3—EGTS handshake command format

Insert new subclause ‘7.3.10.1 MHR fields’:

The Destination Addressing Mode subfield of the Frame Control field shall be set to two (e.g., 16-bit short addressing), and the Source Addressing Mode subfield shall also be set to two.

The Frame Pending subfield of the Frame Control field shall be set to zero and ignored upon reception, and the Acknowledge Request subfield shall be set to one.

The Source PAN Identifier field shall contain the value of macPANId, and the Source Address field shall contain the value of macShortAddress.

The Destination PAN Identifier field shall be set to 0xffff (e.g., broadcast PAN identifier), and the Destination Address field shall be set to 0xffff.

Insert new subclause ‘7.3.10.2 EGTS Characteristics fields’:

The EGTS Characteristics field shall be formatted as illustrated in Figure 4.

| Bit: 0 | 1 | 2-9 | 9-11 | 12-13 | 14 | 15-30 | 31-46 | 47-50 | 51-66 | Variable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel Diversity Mode | EGTS Flag | EGTS Length | EGTS Characteristics Type | EGTS Handshake Type | Priotized Channel Access | Destination Address | EGTS slot identifier | EGTS ABT sub-block length | EGTS ABT sub-block index | EGTS ABT sub-block |

Figure 4—EGTS Characteristics field format

The ChannelDiversityMode subfield is 1 bit in length and shall be set to one of the nonreserved values listed in Table 1’ .

Table 1’—Values of the Channel Diversity Mode subfield

| Channel Diversity Mode value b0 | Description |
| --- | --- |
| 0 | Channel Adaptation |
| 1 | Channel Hopping |

The EGTS Flag subfield shall be set to ‘1’ to use EGTS or set to ‘0’ to use GTS.

The EGTS Length subfield shall contain the number of superframe slots being requested for the EGTS.

The EGTS Characteristics Type subfield is 3 bits in length and shall be set to one of the nonreserved values listed in Table 2.

Table 2—Values of the EGTS Characteristics Type subfield

| EGTS Characteristics Type value b2b1b0 | Description |
| --- | --- |
| 000 | Allocation |
| 001 | Deallocation |
| 010 | Reallocation |
| 011 | Duplicated Allocation Notification |
| 100 | Suspend |
| 101 | Reduce |
| 110 | Restart |
| 111 | Reserved |

The EGTS Handshake Type subfield is 2 bits in length and shall be set to one of the nonreserved values listed in Table 3.

Table 3—Values of the EGTS Handshake Type subfield

| EGTS Handshake Type value b1b0 | Description |
| --- | --- |
| 00 | Request |
| 01 | Reply |
| 10 | Notify |
| 11 | Reserved |

The Priotized Channel Access shall be set to one if EGTS should be reserved as high priority, or set to zero if EGTS should be reserved as low priority. When the EGTS request command is used in the EGTS change procedure, the Prioritized Channel Access shall be set according to the original EGTS.

The Destination Address shall contain the short address of the Destination device.

The EGTS slot identifier shall contain the channel number (1 octet) and beginning time slot number (1 octet) of the slots that are to be allocated or deallocated.

The EGTS ABT sub-block length shall contain the length of the EGTS ABT sub-block in units defined in Figure 5.

The EGTS ABT sub-block index shall indicate the beginning of the ABT Sub-block in the entire ABT as illustrated in Figure 5.

The EGTS ABT sub-block shall contain the sub-block of the Allocation Bitmap Table as illustrated in Figure 5.

Figure 5—ABT Sub-block

When channel hopping is used (i.e., ChannelDiversityMode is 1), TABs are exchanged between the Sender device and the Destination device as illustrated in Figure 5-1.

Figure 5-1—Timeslot Allocation Bitmap (TAB)

  - EGTS information request

The EGTS information request command is used by a Source device that is requesting the timestamp and the EGTS parameters from the Source device.

The EGTS information request command shall be formatted as illustrated in Figure XX.

This command is mandatory for 802.15.4e.

| octets: 7 | 1 |
| --- | --- |
| MHR fields | Command Frame Identifier |

Figure XX EGTS information request command format

The Acknowledgment Request subfield of the Frame Control field shall be set to one.

  - Beacon allocation notification

The Beacon allocation notification command is used by a device that selects vacant SD for using transmission of beacon frame.

The Beacon allocation notification command shall be formatted as illustrated in Figure XX.

| Octets: 7 | 1 | 2 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier | Allocation beacon SD index |

Figure XX Beacon collision notification command format

The allocation beacon SD Index subfield shall contain the allocating SD index number for beacon frame.

  - Beacon collision notification

The Beacon collision notification command is used by a device that detects the collision of beacon frame.

The Beacon collision notification command shall be formatted as illustrated in Figure XX.

| Octets: 7 | 1 | 2 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier | collision SD Index |

Figure XX Beacon collision notification command format

The Conflict SD Index subfield shall contain the SD index number of collision beacon frame.

- New MAC PIBs

Insert in Table 86:

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macEGTSFlag | 0x60 | Boolean | TRUE or FALSE | If this value is FALSE, the operation of this primitive is the same way defined in IEEE 802.15.4-2006 (definition of the GTSCharacteristics parameter see 7.3.9.2). If this value is TRUE, indicate that the request is for EGTS, the GTSCharacteristics parameter will use the definition of EGTSCharacteristics (see 7.3.10.2). |  |
| macChannelDiversityMode | 0x61 | Integer | 0-1 | Indicates the type of channel diversity mode: 0x00 = Channel Adaptation (default). 0x01 = Channel Hopping (optional). This value is not valid for a non-beacon enabled PAN. | 0x00 |
| macMultisuperframe Order | 0x62 | Integer | 0-15 | The length of a multisuperframe which is a cycle of the repeated EGTS schedule. | 15 |
| macConnecDev | 0x63 | Boolean | TRUE or FALSE | Indication of whether the device is a Connection Device. A value of TRUE indicates that the device is a Connection Device. | TRUE |
| macEGTSABT | 0x64 | Set of octets | See Figure 3 in subclause 7.3.10.2 | The allocation bitmap table of the EGTS schedule. | 0 |
| macEGTSACT | 0x65 | Set of octets |  | The allocation counter table of the EGTS allocated to the device. | 0 |
| macSDindex | 0x66 | Integer |  |  |  |
| macSDBitmap | 0x67 | Set of octets |  |  |  |
| macChannelHoppingSequence | 0x66 |  |  |  |  |
| macChannelOffset | 0x67 |  |  |  |  |
| macDeferredBeaconUsed | 0x68 | Boolean | TRUE or FALSE | Indication of whether the device use CCA before transmitting beacon frame. A value of TRUE indicates that the device use CCA before transmitting beacon frame | FALSE |
| macSyncParent-ExtendedAddress | 0x69 | IEEE address | An extended 64-bit IEEE address | The 64-bit address of the coordinator through which the device is synchronized | - |
| macSyncParent ShortAddress | 0x70 | Integer | An extended 64-bit IEEE address | The 16-bit short address assigned to the coordinator through which the device is synchronized. A value of 0xfffe indicates that the coordinator is only using its 64-bit extended address. A value of 0xffff indicates that this value is unknown | 0xffff |
| macSyncParentSDIndex | 0x71 | Integer |  | Indication of SD |  |
| macTAB | 0x72 | Bitmap |  | The Timeslot Allocation Bitmap (TAB) for Channel Hopping mode. Each bit corresponding to EGTS slot set to 1 if the time slot is allocated to transmit or receive, set to 0 otherwise. |  |

- New Functional Description

  - EGTS-based Superframe Structure

Insert new subclause ‘7.5.9 EGTS-based Superframe Structure’:

More descriptions will be added here!

Figure 6—EGTS-based Superframe Structure

More descriptions will be added here!

Figure 7—CAP Reduction in EGTS-based Superframe Structure

More descriptions will be added here!

In channel hopping mode (i.e., ChannelDiversityMode is ‘1’), channels and EGTS slots are determined by the channel hopping sequence and channel hopping offset values. Each device shall maintain series of timeslots whose channels are determined by channel hopping sequence and channel hopping offset values.

The usage of channels and EGTS slots in channel hopping mode is illustrated in Figure 7-1.

Figure 7-1—Illustration of channel and EGTS slot usage in EGTS-based Superframe Structure

In this example, channel hopping sequence is {1, 2, 3, 4, 5, 6} and the channel hopping offset values of two devices are 0 and 2 respectively. For the device with channel hopping offset value of 0, channels and EGTS slots can be represented in terms of EGTS slotidentifier (channel, time slot), i.e.,  (1, 1), (2, 2), (3, 3), (4, 4), (5, 5), (6, 6), (7, 1), (8, 2), (9, 3), and so on. Similarily, for the device with channel hopping offset value of 2, channels and EGTS slots are given as (1, 3), (2, 4), (3, 5), (4, 6), (5, 1), (6, 2), and so on.

Thus, channel used for the given EGTS slot shall be determined as:

C(time slot index) = CHSeq[ (time slot index + CHOffset) % CHSeqLength ]

Meanwhile, the number of EGTS slots in a multi-superframe is given by:

S = (7*2^(MO-SO)) slots

In channel hopping mode, each device exchanges the TABs in EGTS Scheduling procedure.

For instance, the Sender device shall transmit EGTS request command frame with TAB to the Destination device to request EGTS slot allocation. TAB shall represent the channel and EGTS slot usage of the Sender device by setting the bit values either to ‘1’ or ‘0’. Each bit shall be set to ‘1’ if the corresponding slot is already allocated to transmit or receive, otherwise shall be set to ‘0’ to denote that the slot is available.

  - EGTS Scheduling

Insert new subclause ‘7.5.10 EGTS allocation and management’:

More descriptions will be added here!

In channel hopping mode, devices don't exchange the ABT sub-block. Instead, Timeslots Allocation Bitmaps (TABs) are exchaged between the devices to allocate the channels and EGTS slots. The values of start position and length of TAB are followed by the EGTS ABT Sub-block Length and Index parameters.

To allocate channels and EGTS slots in channel hopping mode, the Sender shall send the EGTS request command including transmission schedule TAB to the Destination device.

On receipt of a EGTS request command, the Destination device shall check its own TAB and ABT to find available channels and EGTS slots to allocate new EGTS slots.

Once the Destination device selects channel and EGTS slot for the Sender device, it shall generate new TAB and reply back on EGTS reply command to the Source device. The Sender device updates its own TAB and ABT with allocated channels and EGTS slots. After updating TAB and ABT, the Sender device broadcasts EGTS notify command with this new TAB to adjacent devices. (See Figure 7-2.)

As in channel adaptation mode, there may be packet collisions, since neighbor devices may use the same channel offset value as the Destination device. In order to avoid the collision, the neighbor devices using the same channel offset value shall update their ABT by overhearing EGTS notify command and EGTS reply command. Neighbor devices shall set the bitmap elements in their ABT to ‘1’ if the channel and EGTS slot are already allocated by the Source and the Destination Devices.

Figure 7-2—Illustration of TAB scheduling

Insert new subclause ‘7.5.10.1 EGTS allocation’:

More descriptions will be added here!

Insert new subclause ‘7.5.10.4 EGTS deallocation’:

More descriptions will be added here!

Insert new subclause ‘7.5.10.4 EGTS reallocation’:

More descriptions will be added here!

Insert new subclause ‘7.5.10.4 EGTS retrive’:

If a loss of synchronization occurs either in CAP or CFP before its EGTS slots of current superframe starting, the Source device shall be instructed to request the timestamp and the parameters of its EGTS through the MLME-EGTSinfo.request primitive.

To request the timestamp and the EGTS parameters, the MLME shall send the EGTS information request command (see XXX) to the Destination device. Because the EGTS information request command contains an acknowledgment request (see 7.3.3.1), the Destination device shall confirm its receipt by sending an acknowledgment frame.

On receipt of an EGTS information request command, the MLME of the Destination device shall send the timestamp and the EGTS parameters to the Source device.

On receipt of the acknowledgment to the EGTS information request command, the Source device shall enable its receiver for at most macMaxFrameTotalWaitTime CAP symbols in a beacon-enabled PAN, or symbols in a nonbeacon-enabled PAN, to receive the data frame containing the timestamp and the EGTS parameters from the Destination device.

On receipt of a data frame containing the timestamp and the EGTS parameters, the MLME of the Source device shall then notify the next higher layer of the EGTS information request was successful. This notification is achieved when the MLME issues the MLME-EGTSinfo.confirm (see XXX) primitive with a status of SUCCESS. The Source device shall synchronize to the Destination device by using the received timestamp and then continue to use its EGTS slots during current superframe.

If no corresponding data frame for the device is received within this time, the MLME of the Source device shall notify the next higher layer of the failure. This notification is achieved when the MLME issues the MLME-EGTSinfo.confirm primitive with a status of FAILURE. The Source device shall either end the procedure or try again by issuing the MLME-EGTSinfo.request primitive again.

Insert new subclause ‘7.5.10.5 EGTS change’:

The Destination device allocates the EGTS to the Source device according to the priority level indicated in the EGTS request command sent by the Source device. If the Destination device receives an EGTS request command of higher priority when its EGTS slots have already been used out, it shall either reduce or suspend part of the EGTS slots which are being used for lower priority data transmission to allocate new EGTS for the higher priority data transmission. And when the EGTS for the higher priority data transmission is finished, the Destination device can restart the EGTS slots which were either reduced or suspended previously. The procedure of EGTS change shall be initiated when a Destination device wants to reduce, suspend or restart the EGTS slots.

EGTS change (i.e., suspend, reduce or restart) is initiated by the Destination device through the MLME-GTS.request primitive (see 1.7.3.9).

When an EGTS change is initiated by the next higher layer of the Destination device, the MLME shall receive the MLME-GTS.request primitive with the EGTS Characteristics type subfield of the EGTS Characteristics parameter of the request set accordingly (i.e., 100 SUSPEND, 101 REDUCE or 110 RESTART), and all other subfields set according to the characteristics of the EGTS which the Destination device requests the Source device to change its original EGTS to.

To request the change of an existing EGTS, the MLME of the Destination device shall send the EGTS request command (see 1.7.3.9) to the Source device. The EGTS Characteristics type subfield of the EGTS Characteristics field of the request shall be set accordingly, and all other subfields set according to the characteristics of the EGTS which the Destination device requests the Source device to change its original EGTS to. Because the EGTS request command contains an acknowledgment request (see 7.3.3.1), the Source device shall confirm its receipt by sending an acknowledgment frame. On receipt of the acknowledgment to the EGTS request command, the MLME of the Destination device shall notify the next higher layer of the change. This notification is achieved when the MLME issues the MLME-GTS.confirm primitive (see 7.1.7.2) with a status of SUCCESS and a EGTSCharacteristics parameter with its EGTS Characteristics type subfield set accordingly, and all other subfields set according to the characteristics of the EGTS which the Destination device requests the Source device to change its original EGTS to.

On receipt of an EGTS request command with the EGTS Characteristics type subfield of the EGTS Characteristics field set accordingly, and all other subfields set according to the characteristics of the EGTS which the Destination device requests the Source device to change its original EGTS to, the Source device shall immediately change its EGTS. The MLME of the Source device shall notify the next higher layer of the change. This notification is achieved when the MLME issues the MLME-GTS.indication primitive (see 7.1.7.3) with an EGTSCharacteristics parameter containing the EGTS Characteristics type subfield set accordingly, and all other subfields set according to the characteristics of the EGTS which the Destination device requests the Source device to change its original EGTS to.

Insert new subclause ‘7.5.10.4 Channel Diversity EGTS allocation’:

If the data to transmit in the EGTS requires the higher transmission reliability, the device is instructed to request the allocation of a new Channel Diversity EGTS through the MLME-EGTS.request primitive with the EGTS Characteristics type subfield of the EGTS characteristics parameter set to 010 (Reallocation).

To request the allocation of a new Channel Diversity EGTS, the MLME shall send the EGTS request command (see 1.7.3.9) to the Destination device with the EGTS Characteristics type subfield of the EGTS characteristics parameter set to 010 (Reallocation), and all other subfields shall be set according to the desired characteristics of the required Channel Diversity EGTS.

On receipt of the EGTS request command with the EGTS Characteristics type subfield of the EGTS characteristics parameter set to 010 (Reallocation), the Destination device shall first decide whether allocating the Channel Diversity EGTS or the regular EGTS based on its own availability. If the Destination device decides to allocate the Channel Diversity EGTS, it shall contain the channel sequence in the ABT of the EGTS reply command. If not, the Destination device shall allocate a regular EGTS. (p.s. The remaining part of the Channel Hopping EGTS allocation and usage are same with the regular EGTS.)

Within the Channel Diversity EGTS, the devices switch to the next channel every slot according to the channel sequence in the ABT.

  - Beacon Scheduling

When a new node wants to join a network, first it scans the channel. The new device uses the MLME-SCAN.request primitive in order to initiate a channel scan over a given list of channels. It searches for all coordinators transmitting beacon frames within the maximum BI period. The neighboring nodes which receive MLME-SCAN.request would reply with the MLME-SCAN.confirm primitive. Then these neighboring nodes would share their bitmap table with the new node. The Bitmap table indicates the beacon frame allocation information for neighboring nodes. This field is expressed by bitmap method which orderly represents the schedule of beacons. Corresponding bit shall be set to 1 if a beacon is allocated in that SD. The new node will search the SD which is vacant (not set to 1) in all of the received bitmap tables. Once it finds that vacant SD number then it can request the allocation of that SD to the node with which it is associated (i.e. parent node)

There can be beacon slot collision when two or more nodes are trying to compete for same SD slot number. It can be best described by the Figure 3. As shown in Figure C, node D and node E are new nodes that join the network.  These new nodes will receive the bitmap tables from their neighbouring nodes. As it can be seen that node A is not only a common neighbouring node, but also the parent node for both nodes E and D. Thus it can be the case when both new nodes E and D request same vacant SD number within same CAP. This happens due to hidden node problem, because node E and D are hidden to each other, and cannot listen each others transmission. When node A receives the SD number 2 request, within same CAP, by nodes E and D then it would deteremine which node has requested first. Node A will reply the MLME.CONFIRM notification to the node which has requested first, and would send MLME.COLLISION message to the other node.

Figure 3: New node joining the network

Figure 4: Bitmap tables for neighboring nodes

The scenario described in 1.1.2 becomes worse when beacon collision happens several times. This is mainly caused when two or more new nodes request for the SD allocation using same slot number of the same CAP. In this case the parent node will reply the MLME.COLLISION notification to the SD allocation requesting nodes. When these nodes receives the collision notification then they would select other vacant SD.

  - Time Synchronization

The new node, i.e. device, discovers its neighbouring node through the scanning process. Then it associate with one of the neighbouring node in order to be part of the network. The node with which the new node asscoiates is called its parent node, i.e. coordinator.

Now the coordinator knows that the device would track its beacon. Thus the coordinator will determine the transmission time of its beacon. This timestamp is known as macSyncSymbolOffset value, which is sent just before the beacon fram. When the device gets this macSyncSymbolOffset value then it selects its own beacon offset accordingly. The the device keeps syncronise its timer with the coordinator through tracking the coordinator’s beacon every time.

The effect of collision is inevitable in multiple nodes scenario, where more than one nodes try to use the same channel at the same time. In the case of collision, the node backs off for some time and then it tries to re-send. Thus in case of collison, beacon transmission timestamp does not become valid. In order to avoid this problem, the coordinator sets macDefferedBeaconUsed value to be TRUE. When the device notice macDefferedBeaconUsed value to be TRUE then it knows that coordinator uses CCA for transmitting its beacon. Also the coodinator would add the number of tries it made for successful transmission. If the coordinator would be able to send its beacon after 3rd try then it would set DefferedBeaconFlag value to be 3. At the receipt of the beacon, the device would exactly knows when the beacon is sent (by adding beacon timestamp with (DefferedBeaconFlag value * 20 symbols) ). Thus perfect time syncronization becomes possible

  - Scanning through channels

    - Passive channel scan

Insert in 7.5.2.1.3:

If EGTSFlag is TRUE and ChannelDiversityMode is ‘1’, a passive scan is performed on the logical channel, spcified in the macCommonChannel parameter, for longer than one beacon interval. Channel Diversity Specification in the received beacon frame shall update the value of ChannelDiversitySpecification in PANDescriptor. This value is sent to the next higher layer via the MLME-SCAN.confirm primitive. The value of Channel Offset subfield in the received beacon shall update the value of macChannelOffsetBitmap in MAC PIB attributes. For instance, if Channel Offset is 0x01, the  value of macChannelOffsetBitmap corresonding channel shall set to ‘1’. Thus, the value of macChannelOffsetBitmap shall represent if the channel offset value is used among one hop neighbor devices.

  - Starting and realigning a PAN

    - Updating superframe configuration and channel PIB attributes

Insert in 7.5.2.3.4:

If a PAN uses EGTS and channel hopping is used (i.e., EGTSFlag is TRUE and ChannelDiversityMode is ‘1’), the MAC sublayer shall update the values of DCHDescriptor with the values of the DCHDescriptor parameter.

  - Beacon generation

Insert in 7.5.2.4:

If EGTSFlag is TRUE and channel hopping (i.e., EGTSMode is TRUE and ChannelDiversityMode is ‘1’) are used in the PAN, the MAC sublayer shall set the Channel Diversity Specification field of the beacon frame. The value of ChannelOffsetBitmap field, representing channel offset used among one hop neighbor devices, shall be set using  the value of macChannelOffsetBitmap in MAC PIB attributes.

  - Association

Insert in 7.5.3.1:

Following the selection of a PAN with which to associate, the next higher layers shall request through the MLME-ASSOCIATE.request primitive that the MLME configures the following PHY and MAC PIB attributes to the values necessary for association :

— macChannelOffset shall be set equal to the value of ChannelOffset element in DCHDescriptor parameter.

On receipt of the association response command, the MAC sublayer shall assign the value of Channel Hopping Sequence field to the macChannelHoppingSequence in the MAC PIB attribute.

  - Coexistence of beacon-enabled and nonbeacon-enabled mode

PANs that contain both the devices of beacon-enabled mode and the devices of nonbeacon-enabled mode shall also include the Connection Devices. The Connection Device works in the beacon-enabled mode and nonbeacon-enabled mode at the same time. The device of beacon-enabled mode shall either transmit periodic beacon or track the beacon for communication in the PAN. The device of nonbeacon-enabled mode shall neither transmit periodic beacon nor track the beacon for communication in the PAN. For example, a Connection Device can transmit periodic beacon to communicate with the devices of beacon-enabled mode in the PAN and request the data from the devices of nonbeacon-enabled mode in the PAN.