IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Required Changes for Low Energy Extension |  |
| Date Submitted | Sunday, August 30, 2009 |  |
| Source | [Wei Hong, Li Liang, Betty Zhao, Hirohito Nishiyama, Fumihide Kojima, Hiroshi Harada, Takaaki Hatauchi, Kazuyuki Yasukawa] [510 2nd St., Suite 410 San Francisco, CA USA] | Voice: [+1 415-692-0828] E-mail: [whong@archrock.com] |
| Re: | [] |  |
| Abstract | [This document contains proposed changes to the IEEE P802.15.4e Draft to address required changes for Low Energy.] |  |
| Purpose | [] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Content

1.	Summary of Changes	5

2.	Definitions	5

3.	New MAC frames	5

3.1	Existing Frames	5

3.2	Wakeup Frame	6

3.3	Secure Acknowledgement Frame (to be combined with other proposals)	7

3.4	New optional MHR field	8

4.	New MAC PIBs	8

5.	New Functional Description	8

7.5.1.1.1 Contention access period (CAP)	9

7.5.6 Transmission, reception and acknowledgement	9

7.5.6.7 <New Section>Coordinated Sampled Listening (CSL)	9

7.5.6.7.1 CSL idle listening	9

7.5.6.7.2 CSL transmission	9

7.5.6.7.2.1 Unicast transmission	9

7.5.6.7.2.2 Multicast transmission	10

7.5.6.7.2.3 Utilizing the optional CSL sync field	10

7.5.6.7.3 CSL reception	10

7.5.6.7.4 CSL over multiple channels	11

- Summary of Changes

- 3. Definitions

  - add Coordinated Sampled Listening (CSL) concepts

  - add Receiver Initiated Transmission (RIT) concepts

- 5. General description

  - 5.5.5 Power consumption considerations

    - add general description of CSL and RIT

- 7. MAC sublayer specification

  - 7.1. MAC sublayer service specification

    - add one MLME primitive for RIT support

  - 7.2. MAC frame formats – add two new frame types and a new optional MHR field

  - 7.3. MAC command frames

    - add one MAC command frame for RIT support

  - 7.4. MAC constants and PIB attributes

    - add five PIB attributes for CSL

    - add three PIB attributes for RIT

  - 7.5. MAC functional specification

    - add Coordinated Sampled Listening (CSL)

    - add Receiver Initiated Transmission (RIT)

  - 7.6. Security suite specifications – no change

  - 7.7. Message sequence charts illustrating MAC-PHY iteraction – no change

- Definitions

- Coordinated Sampled Listening (CSL): A low-energy mode to the MAC which allows receiving devices to periodically sample the channel(s) for incoming transmissions at low duty cycles.  The receiving device and the transmitting device are coordinated to reduce transmit overhead.

- CSL Period: The period in which receiving devices sample the channel(s) for incoming transmissions.

- CSL Phase: The length of time between now and the next channel sample.

- CSL Payload Frame: a beacon, data or command frame.

- CSL Wakeup Frame: a special short frame transmitted back-to-back before the payload frame to ensure its reception by CSL receiving device.

- CSL Wakeup Frame Sequence: a sequence of back-to-back wakeup frames up to the duration of the CSL Period.

- CSL Rendezvous Time (RZTime): 2-octet timestamp in wakeup frame payload indicating the expected length of time in milliseconds between the end of the wakeup frame transmission and the beginning of the payload frame transmission.

- CSL Channel Sample: The operation to perform ED on a channel and attempt to receive wakeup frame when energy is detected.

- Receiver Initiated Transmission (RIT): An alternative low-energy mode to CSL in which receiving devices periodically broadcast data request frames and transimitting devices only transmit to a receiving device upon receiving a data request frame.  RIT is suitable for the following application scenarios:

  - Low data traffic rate and loose latency requirement (tens of seconds per transmission)

  - Local regulations restricting the duration of continuous radio transmissions (e.g., 950MHZ band in Japan).

- General description

- 5.5.5 Power consumption considerations

(add the following text to the end of the subsection)

Two low-energy mechanisms are provided to further reduce energy consumption by allowing devices to communicate while maintaining low duty cycles.  They are Coordinated Sampled Listening (CSL) and Receiver Initiated Transmission (RIT).

Coordinated Sampled Listening (CSL) allows receiving devices to periodically sample the channel(s) for incoming transmissions at low duty cycles.  The receiving device and the transmitting device are coordinated to reduce transmit overhead.

Receiver Initiated Transmission (RIT) allows receiving devices to periodically broadcast data request frames and transimitting devices only transmit to a receiving device upon receiving a data request frame.  RIT is suitable for the following application scenarios:

- Low data traffic rate and loose latency requirement (tens of seconds per transmission)

- Local regulations restricting the duration of continuous radio transmissions (e.g., 950MHZ band in Japan).

- New MAC sublayer service specifications

- 7.1.16d <New section> primitive for notifying erroneous frame reception (RIT mode only)

MLME-SAP FCS error primitive defines how a device may be notified when data frame is received with error in the RIT mode.

This primitive is optional for a device supporting RIT mode.

- 7.1.16d.1 MLME-FRAME-ERROR.indication (RIT mode only)

The MLME-FRAME-ERROR.indication primitive is used to notify the reception of an erroneous data frame by the MAC sublayer to the next higher layer.

- 7.1.16d.1.1 Semantics of the service primitive

The semantics of the MLME-FRAME-ERROR.indication primitive are as follows:

MLME-FRAME-ERRORindication(
status,
)

Table X specifies the parameters for the MLME-FRAME-ERROR.indication primitive.

Table X – MLME-FRAME-ERROR.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | FCS_ERROR, SECURITY_ERROR | The status of the erroneous reception of data frame in RIT mode |

- 7.1.16d.1.2 When generated

The MLME-FRAME-ERROR.indication primitive is generated by MLME and issued to its next higher layer upon reception of an erroneous data frame in RIT mode.

- 7.1.16d.1.2 Appropriate usage

On recept of the MLME-FRAME-ERROR.indication primitive, the next higher layer is notified of the reception of an erroneous data frame at the MAC sublayer.

- 7.1.17 MAC enumeration description

{add the following row at the bottom of Table 78- MAC enumerations description}

| Enumeration | Value | Description |
| --- | --- | --- |
| FCS_ERROR | TBD | The received data frame contains incorrect value in the FCS field in the MFR. |

- New MAC frames

  - Existing Frames

Figure 41 shows the general MAC frame format of IEEE802.15.4-2006.

Figure 41 -- General MAC Frame Format

Figure 42 shows the 2-octet frame control field (FCF) in IEEE802.15.4-2006.

Figure 42 -- Format of the Frame Control Field

Table 79 contains the frame type values of the existing frames with two new frame types added.

Table 79―Values of the frame type subfield

| b2 b1 b0 | Description |
| --- | --- |
| 000 | Beacon |
| 001 | Data |
| 010 | Acknowledgment |
| 011 | MAC command |
| 100 | CSL Wakeup |
| 101 | Secure Acknowledgment |
| 110-111 | Reserved |

  - Wakeup Frame

The following figure illustrates the format of wakeup frame.

| Octets: 2 FCF | 1 DSN | 2 Dest PAN | 2 Dest Addr | 2 RZTime | 2 FCS |
| --- | --- | --- | --- | --- | --- |

The FCF is set as follows :

- Frame type = 100

- Security enabled = 0

- Frame pending = 0

- Ack request = 0

- PAN ID compression = 0

- Dest addressing mode = 0x2

- Frame version = 0x1

- Source addressing mode = 0x0

The payload of this frame contains a 2-octet Rendezvous Time (RZTime) which is the expected length of time in milliseconds between the end of the transmission of the wakeup frame and the beginning of the transmission of the payload frame.  This number is automatically filled in by the MAC layer when wakeup frames are constructed.  The last wakeup frame in a wakeup sequence must have RZTime = 0.

  - New optional MHR field

The reserved FCF reserved bit 7 is renamed to the CSL sync bit.  When the bit is set to 1, a 4-octet optional CSL sync field is added to the end of the current MHR.  The following figure illustrates the format of the CSL sync field.

| Octets: 2 CSL Phase | 2 CSL Period |
| --- | --- |

They represent the CSL phase and period of the transmitting device of the frame.  This informmation helps eliminate or reduce the wakeup sequence for subsequent transmissions from the receiving device.

  - Secure Acknowledgement Frame

The following figure illustrates the format of an secure acknowledgement frame.

| Octets: 2 FCF | 1 DSN | 2 Dest PAN | 2 Dest Addr | 0/5/6/10/14 Auxiliary security header | 2 CSL Phase | 2 CSL Period | 2 FCS |
| --- | --- | --- | --- | --- | --- | --- | --- |

The FCF is set as follows :

- Frame type = 101

- Security enabled = 0 or 1

- Frame pending = 0

- Ack request = 0

- PAN ID compression = 0

- CSL sync = 1

- Dest addressing mode = 0x2

- Frame version = 0x1

- Source addressing mode = 0x0

The payload of this frame contains 2-octet CSL Phase and 2-octet CSL Period.  They represent the CSL phase and period of the receiving device.  This information helps the transmitting device eliminate or reduce the wakeup sequence for subsequent transmissions to the same destination.

- 7.3 New Mac Command

{add the following row at the bottom of Table 82- MAC enumerations description}

Table 82 – MAC command frames

| Command frame identifier | Command name | RFD |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | Tx | Rx |  |
| 0xXX | RIT data request |  |  | <New section> |

- <New Section> RIT data request command (RIT mode only)

The RIT data request command allows a device to request data from its neighboring devices in the RIT mode. This command shall only be sent and received in the RIT mode (macRitPeriod : non zero value). This command is optional and applicable for FFD only. The RIT data request command shall be formatted as illustrated in Figure XX.

| Octets : (see 7.2.2.4) | 1 |
| --- | --- |
| MHR fields | Command Frame Identifier （see Table 82） |

- <New section> MHR fields

The Frame Pending subfield of the Frame Control field shall be set to zero and ignored upon reception, and the Acknowledgement Request subfield shall also be set to zero. All other subfields shall be set appropriately according to the intended use of the command frame.

- New MAC PIBs

Table 86—MAC PIB attributes

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macCSLPeriod |  | Integer | 0 … 65535 | CSL sampled listening period in milliseconds. 0 means always listening, i.e., CSL off. | 0 |
| macCSLMaxPeriod |  | Integer | 0 … 65535 | Maximum CSL sampled listening period in milliseconds in the entire PAN. This determines the length of the wakeup sequence when communicating to a device whose CSL listen period is unknown. NHL may set this attribute to 0 to stop sending wakeup sequences with proper coordination with neighboring devices. | macCSLPeriod |
| macCSLChannelMask |  | Integer |  | 32-bit bitmap relative to phyCurrentPage of channels. It represents the list of channels CSL operates on. 0 means CSL operates on phyCurrentChannel of phyCurrentPage. | 0 |
| macCSLFramePendingWaitT |  | Integer |  | Number of milliseconds to keep the receiver on after receiving a payload frame with FCF frame pending bit set to 1. |  |
| macSecAckWaitDuration |  | Integer |  | The maximum number of symbols to wait for a secure acknowledgement frame to arrive following a transmitted data frame. |  |
| macRitPeriod |  | Integer | 0x000000 -0xffffff | The interval (in unit periods) for periodical transmission of RIT data request command in RIT mode. The unit period is aBaseSuperframeDuration. 0 means RIT is off | 0 |
| macRitDataWaitPeriod |  | Integer | 0x00 – 0xff | The maximum time (in unit period) to wait for Data frame after transmission of RIT data request command frame in RIT mode. The unit period is aBaseSuperframeDuration. | 0 |
| macRitTxWaitTime |  | Integer | macRitPeriod - 0xffffff | The maximum time (in unit periods) that a transaction is stored by a device in RIT mode. The unit period is aBaseSurperframeDuration. | 0 |

- New Functional Description

- 7.5.1.1.1 Contention access period (CAP)

- When macCSLPeriod is set to non-zero, CSL is deployed in CAP.

- macRitPeriod shall not be set to non-zero in a beacon-enabled PAN.

- 7.5.2.1 Scanning through channels

- When macCSLPeriod is set to non-zero, CSL is deployed in channel scans.  When macCSLMaxPeriod is set to non-zero, each coordinator broadcasts beacon frames with wakeup sequence.  This allows devices to perform channel scans with low duty cycles.

- 7.5.6 Transmission, reception and acknowledgement

- 7.5.6.7 <New Section>Coordinated Sampled Listening (CSL)

- The coordinated sampled listening (CSL) mode is turned on when the PIB attribute macCSLPeriod is set to non-zero and turned off when macCSLPeriod is set to zero.  In CSL mode, transmission, reception and acknowledgement work as follows.  Figure X illustrates the basic CSL operations.

- Figure X. Basic CSL operations

- 7.5.6.7.1 CSL idle listening

- During idle listening, CSL performs a channel sample every macCSLPeriod milliseconds.  If the channel sample does not detect energy on the channel, CSL disables receiver for macCSLPeriod milliseconds and then perform the next channel sample.  If the channel sample receives a wakeup frame, CSL checks the destination address in the wakeup frame.  If it matches macShortAddress, CSL disables receiver until the Rendezvous Time (RZTime) in the wakeup frame from now and then enables receiver to receive the payload frame.  Otherwise, CSL disables receiver until RZTime from now plus the transmission time of the payload frame and the secure acknowledgment frame and then resume channel sampling.

- 7.5.6.7.2 CSL transmission

- Each CSL transmission of a payload frame is preceded with a sequence of back-to-back wakeup frames (wakeup sequence).

- 7.5.6.7.2.1 Unicast transmission

- In unicast transmissions, the wakeup sequence length can be long or short based on the following two cases:

- Unsynchronized transmission: This is the case when the MAC layer does not know the CSL phase and period of the destination device.  In this case, the wakeup sequence length is macCSLMaxPeriod.

- Synchronized transmission: This is the case when the MAC layer knows the CSL phase and period of the destination device.  In this case, the wakeup sequence length is only the guard time against clock drift based on the last time when CSL phase and period updated about the destination device.

- If the next higher layer has multiple frames to transmit to the same destination, it can set the FCF frame pending bit to 1 in all but the last frame to maximize the throughput.

- CSL unicast transmission is performed in the following steps by the MAC layer:

- Perform CSMA-CA to acquire the channel

- If the previous acknowledged payload frame to the destination has the frame pending bit set and is within macCSLFramePendingWaitT milliseconds, go to step 5.

- If it is a synchronized transmission, wait until the destination device’s next channel sample.

- For the duration of wakeup sequence length (short or long)

  - Construct wakeup frame with the destination short address and remaing time to payload frame transmission (at the end of wakeup sequence)

  - Transmit wakeup frame

- Transmit payload frame

- Wait for up to macSecAckWaitDuration symbol time for the secure acknowledgement frame if the ack request subfield in the payload frame is set to 1.

- If the secure acknowledgment frame is received, update CSL phase and period information about the destination device from the acknowledgment CSL sync field.

- If the secure acknowledgement frame is not received, start retransmission process.

- 7.5.6.7.2.2 Multicast transmission

- Multicast transmission is the same as as unicast transmission except the following:

- It is always unsynchronized transmission.

- The destination address in wakeup frames is set to 0xffff.

- 7.5.6.7.2.3 Utilizing the optional CSL sync field

- Selectively the next higher layer may set the CSL sync bit in FCF in a frame to propogate CSL phase and period information among the neighboring devices. When the bit is set, the MAC layer automatically appends the CSL sync fields to the end of MHR.

- 7.5.6.7.3 CSL reception

- When a payload frame is received, the MAC layer performs the following steps:

- Immediately send back a secure acknowledgment frame with the destination address set as the transmitting device and its own CSL phase and period filled in the CSL sync field.  The acknowledgment frame can be optionally authenticated and/or encrypted depending on the current security mode.

- If CSL sync bit in the received payload frame is set to 1, the CSL phase and period information about the transmitting device is updated with the information in the CSL sync field.

- If FCF frame pending bit in the received payload frame is set to 1, keep receiver on for macCSLFramePendingWaitT milliseconds before going back to CSL idle listening.  Otherwise, start CSL idle listening.

- 7.5.6.7.4 CSL over multiple channels

- When macCSLChannelMask is set to non-zero, the CSL operations are extended to all the channels selected in the bitmap.  CSL idle listening performs channel sample on each channel from the lowest number to the highest in a round-robin fashion.  In the unsynchronized case, CSL transmission transmits a wakeup sequence of the length number_of_channels*macCSLMaxPeriod before each payload frame.  In the synchronized case, CSL transmission calculates the next channel sample time and channel number and transmits at the next channel sample time on the right channel with a short wakeup sequence.  In this case, CSL phase is the duration from now to the next channel sample on the first channel selected in macCSLChannelMask.

- 7.5.6.7.5 Turning off CSL mode to reduce latency

- The next higher layer has the option to turn off sampled listening and stop sending wakeup sequences to reduce latency for urgent messages.  This assumes that the higher layer manages the coordination between the sender and receiver in turning on and off sampled listening.  To turn off sampled listening, the higher layer simply sets macCSLPeriod to zero.  To turn on sampled listening, the high layer restores macCSLPeriod to their previous non-zero values.  Similarly, to stop sending wakeup sequences, the higher layer sets macCSLMaxPeriod to zero and restores it to its previous value to return to normal CSL mode.  To request a neighboring device to turn off sampled listening, the higher layer must send a frame to the device with frame pending bit set to 1.  This prevents CSL from turning off the radio before the request is processed.

- 7.5.6.8 <New Section> Receiver Initiated Trasmission (RIT)

- The Receiver Initiated Transmission (RIT) is an alternative low energy MAC for non beacon-enabled PAN (BO=15). RIT mode is turned on when PIB attribute macRitPeriod is set to non-zero value and is turned off when macRitPeriod is set to zero. In RIT mode, transmission, reception and acknowledgement work as follows.

- RIT mode is applicable to low duty cycle, low traffic load type of applications and especially suitable in the case that consecutive radio emission time is limited by regional or national regulation (e.g., 950MHz band in Japan).  macCSLPeriod (in coordinated sample listening) and macRitPeriod cannot be set to non-zero value at the same time. Figure X illustrates the basic RIT operations.

Figure X. Basic RIT operations

- 7.5.6.8.1 Periodical RIT data request transmission and reception

- In RIT mode, a device transmits RIT data request command every macRitPeriod with a use of unslotted CSMA-CA. The destination address of the command may be broadcast address (0xffff) or the address of intended transmitter of data (associated coordinator). After the transmission of RIT data request command frame, the device waits macRitDataWaitPeriod for incoming frame (except RIT data request frame) and goes back to idle state till the next periodical transmission of RIT data request command. When a device is in the receiving state after transmission of RIT data request command, RIT data request command frame from another device shall be discarded.

- Upon reception of a data frame after the transmission of RIT data request command, it notifies its arrival to the next higher layer by instigating MCPS-DATA.indication.  Upon reception of a data frame with error (FCS or security), it notifies its erroneous reception to the next higher layer by instigating MLME-FRAME-ERROR.indication.

- At this point (instigation of MCPS-DATA.indication or MLME-FRAME-ERROR.indication), the device may set macRitPeriod primitive to zero (RIT off) at the discretion of the next higher layer. If this is the case, it will stop periodical transmission of RIT data request command and become always active until macRitPeriod primitive is set to non zero value by the next higher layer again. During this period (macRitPeriod equals to zero), all transactions will be handled as those of normal non beacon-enabled PAN (RxOnWhenIdle: False).

Figure X. message sequence chart for starting RIT mode

- 7.5.6.8.2 Data transmission in RIT mode

- In order to transmit data frame in RIT mode, MCPS-DATA.request primitive (with TxOption indirect) shall be instigated by the next higher layer at first. When the primitive is instigated, the device shall stop its periodical transmisstion of RIT data request, enable its receiver and wait reception of RIT data request command frame from neighboring devices for at most macRitTxWaitTime. During period, all other frames except RIT data request command shall be discarded.

- Upon reception of RIT data request command frame, the MAC sublayer sends the pending data with a use of unslotted CSMA-CA. In case that the Destination PAN identifier field and the Destination address field of the received RIT data request command are broadcast (0xffff) and the DstPANId and DstAddr parameters of instigated MCPS-DATA.request are also broadcast, the Destination PAN indentifier field and the Destination Address field of the outgoing data frame shall be set as the Source PAN identifier field and the Source Address field of the received RIT data request command, respectively.

- At the completion of data transmission, MCPS-DATA.confirm shall be instigated by the MAC sublayer to the next higher layer. At this point, the device shall restart its transmission of periodical RIT data request transmission. Also at this point, the device may set macRitPeriod primitive to zero (RIT off) at the discretion of the next higher layer. If this is the case, it will continue to stop periodical transmission of RIT data request command and become always active until macRitPeriod primitive is set to non zero value by the next higher layer again. During this period (macRitPeriod equals to zero), all transactions will be handled as those of normal non beacon-enabled PAN (RxOnWhenIdle: False).

- 7.5.6.8.3 Multicast transmission

- Multicast transmission shall not be supported in RIT mode.

Figure X. Message sequence chart for data transmission in RIT mode