IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Common Platform Summary Table |  |
| Date Submitted | 26 June 2009 |  |
| Source | [Benjamin A. Rolfe] [BCA] [address] | Voice: [ ] Fax: [ ] E-mail: [ ] |
| Re: | TG4g: Narrow-band FSK PHY proposals |  |
| Abstract | Table summarizing the features included in NB-FSK common platform; Rev 1 includes comments from email discussion and 17-June-09 Teleconference. |  |
| Purpose | Promote technical discussion and assist in developing a merged FSK PHY |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

FHSS-FSK Common Platform

Common Platform Working Summary

# Introduction

## Purpose and Scope

The following summarizes the Common Platform Proposal. The intent of this table is to capture the current “state” of the CP during the process of merging proposals, and to facilitate discussions necessary to reach convergence.  More details are captured in 15-09-0298-02 (and further revisions).

## References

The content of this table is drawn form the following proposals submitted to TG4g.

|  | Document # | Document Title |
| --- | --- | --- |
|  | 15-09-0124-03 | multi rate phy proposal.ppt |
|  | 15-09-0274-00 | low power features for sun.ppt |
|  | 15-09-0278-00 | gfsk phy proposal for smart utility networks.ppt |
|  | 15-09-0280-00 | nbfh detailed phy proposal for 802 15 4g.pdf |
|  | 15-09-0285-00 | mac and phy proposal for 802 15 4g for smart utility networks.ppt |
|  | 15-09-0288-01 | battery operated sensing device operations in 802 15 4g.ppt |
|  | 15-09-0289-01 | affordable ofdm for the sun phy.pptx |
|  | 15-09-0290-01 | power and spectrum efficient phy proposal for 802 15 4g.ppt |
|  | 15-09-0292-01 | itron proposal for 802 15 tg4g phy amendment.doc |
|  | 15-09-0299-00 | sensus proposal to tg4g.ppt |
|  | 15-09-0300-00 | nbfh detailed phy proposal presentation.ppt |
|  | 15-09-0302-01 | fhss neighborhood area network communications proposal.ppt |
|  | 15-09-0304-00 | draft for 802 15 tg4g phy amendment fhss phy w interoperable battery powered devices.doc |
|  | 15-09-0305-00 | 2 4 ghz nbfh proposal to tg4g.ppt |
|  | 15-09-0310-00 | coronis ft proposal for 802 15 tg4 phy amendment doc.doc |
|  | 15-09-0312-00 | phy and mac proposals for low power consumption sun.pdf |
|  | 15-09-0396-00 | summary of itron proposal for 802 15 tg4 ammendment.ppt |

# FHSS-FSK Common Platform Summary Table

Highlighted remarks are things to discuss as a group (sub-group) via email reflector, conference calls, and face to face.

| FHSS-FSK Common Platform |  |  |
| --- | --- | --- |
| Specification | Proposed specification or approach | Remarks |
| Operating Frequency | Support multiple bands to include: 314; 316; Possibly in China. 316; 433. 400; TBD; Japan (TBD). 470; 510; China (to be verified). 780; TBD; Possibly in China. 863; 870; US and Europe. 902; 928; US. 950; 956; Japan. 2400; 2483; Most places. | Channel spacing, BW, available data rates available vary by band. Discussed including an informative annex on “band agnostic implementation guidance” to enable adaptation of the standard other bands as may be (or become) available; General agreement. 314-316MHz and 780MHz under consideration in China, more information is pending; Additional band(s) in Japan may be under consideration, more information pending. |
| Transmit Power | Maximum TX power is implementation dependent; | The standard allows for implementations that conform to appropriate regulations; |
| PHY Service Specification | Per 15.4-2009, extended as necessary (TBD). | 09-0280-00 section 6.2 is a good place to start. |
| PPDU Format | Multiple PHR formats are supported via unique SFD values; | 4 PHR formats identified (see PHR Variations) Can consolidate, or need more options? |
|  | Support for variable length preamble, with length and bit pattern provided by MAC via PHY PIB |  |
|  | 11 and 12 bit length field |  |
|  | Frame control to support signaling of FEC/data rate, forward compatibility via header extension bit |  |
|  | 32-bit CRC and 16-bit CRC (MAC selects which) |  |
| Channel Spacing, BW, and Symbol/data Rates | Multiple data rates supported, band-specific choices: Chan Spacing kHz; Chan BW kHz; Modula-tion; Nominal kbps; Note. 50; 50; 2-GFSK; 19.2; PHY hopping with interleaving and BCH. 300; 250; 2-MSK, 2-GFSK; 100; 902-928MHz; 2.4GHz. 300; 250; 4-GFSK; 200; 902-928MHz; 2.4GHz. 250; TBD; 2-GFSK; 50, 100; 863-870MHz band. 250; TBD; 4-GFSK; 200; 863-870MHz band. 200; TBD; 2-FSK; 60. 400; TBD; 2-FSK; 240. 400; TBD; 2-GFSK; 100; 950MHz band (Japan). 600; 500; 2-MSK, 2-GFSK; 200; 2.4GHz. 600; 500; 4-MSK, 4-GFSK; 400; 2.4GHz. 500; 500; 2-GFSK; 200; 868-870MHz band. 500; 500; 4-GFSK; 400; 868-870MHz band. | Need more information on 400MHz band in Japan; Need to specify which combinations work in which bands more completely (470, 400M not included yet) Need all these options? |
| FEC | None Convolutional coding (15-09-289, 15-09-0299) rate ½, ¾. Block Parity SECDED (15-09-0290) BCH(21,31) (15-09-0310) Reed-Solomon (15-09-0292) BCC | Can we converge on one or two? |
| Scrambler | 8-bit (variable seed) and 9-bit (15.4d compatible). Controlled by MAC (can be disabled). | 8-bit fits into the PHR; 9-bit for compatibility with 15.4d |
| FSK/GFSK | Support for ‘switch-able’ Gaussian filter in all bands; FSK Modulation parameters: h=0.5 (MSK) GFSK Modulation Parameters: - h = 0.75, BT = 0.5 - h=0.5 BT=0.3 - h=0.3 BT=0.5 - h=1 BT=0.5 | Can we converge on fewer GFSK options? |
| Transmit Power Control | Variable, controlled by MAC via PHY PIB; Adopt PHY-PIB definitions in 09-0280-00 which supports power levels > 30dBm (such as up to 5W per 15-09-0299) | 09-0280-00 provides a method that supports implementation dependent granularity and range, solves the problem identified in 09-298 about limited range of the current 15.4 definition. |
| Data Interleaving | W/PHY hopping mode | Used with PHY hopping (FT/Coronis); may be useful with slow hopping, need further input; |
| Hopping | PHY hopping (a PSDU is split across multiple channels); MAC hopping (1 or more PSDU sent before changing channels) | Control of hopping is MAC function |
| CCA Function | 15.4a-2007 adds CCA Mode 4 ALOHA (CCA shall always report an idle medium). Specify that this mode may apply to the 15.4g PHY. Apply LBT CCS modes in 15.4-2006; |  |
| Channel Quality Measurement | Provide LQI measurement via PHY signal strength measurement (RSSI); Work with TG4g for MAC layer features for channel quality; | Needs some MAC support too: - Packet error counts - Ignore CRC option |
| Min. Receiver Sensitivity | TBD | May be band specific? May be implementation specific? |
| Min. Adjacent Channel Rejection | TBD | May be band specific? May be implementation specific? |
| Low Energy Features | Channel sampling function (slide 16 on doc#15-09-0274-00-0004g) Sampling enabled(SMP_ON) control (slide 15 on doc#15-09-0274-00-0004g) Other PHY layer sensing/adaptation mechanisms for energy saving; PHY Hopping, lower data rate (Wavenis) mode | Coordinating MAC functionality with TG4e “low energy” sub-group; |

# PHR Variations

| Octets: variable | 2 | 1 | 2 |  |  | variable |
| --- | --- | --- | --- | --- | --- | --- |
| Bits: variable | 16 | 8 | 1 | 4 | 11 |  |
| Preamble | SFD | Scrambler Seed | E X T | FCTRL (DR/ FEC) | Frame Length | PSDU Includes FCS |
| SHR |  | PHR |  |  |  | PHY Payload |

Figure 1: Structure of PPDU based on 15-09-0300-00

| Octets: variable | 2 | 2 |  |  | variable |
| --- | --- | --- | --- | --- | --- |
| Bits: variable | 16 | 1 | 3 | 12 |  |
| Preamble | SFD | E X T | FCTRL | Frame Length | PSDU Includes FCS |
| SHR |  | PHR |  |  | PHY Payload |

Figure 2: Structure of PPDU with short header, 12 bit Length Field, based on 15-09-302-01:

| Octets: variable | 2 | 1 or 2 |  |  | variable |
| --- | --- | --- | --- | --- | --- |
| Bits: variable | 16 | 1 | 4 | 7 or 11 |  |
| Preamble | SFD | PHR Len | R F U | Frame Length | PSDU Includes FCS |
| SHR |  | PHR |  |  | PHY Payload |

Figure 3: Compatible with legacy 802.15.4d devices, based on 15-09-0278-00

Regarding Figure 3, the PHR Len bit = 0 signals a 7 bit length field, and there are no RFU bits; When PHR Len = 1 the Frame Length is 11 bits and the remaining 4 bits are reserved.

| Octets: | 2 or 4 | 2 |  |  | 2 | variable |
| --- | --- | --- | --- | --- | --- | --- |
| Bits: variable | 16 or 32 | 4 | 1 | 11 | 16 |  |
| Preamble | SFD | NID | E X T | Frame Length | Frame Header Protection | PSDU Includes FCS |
| SHR |  | PHR |  |  |  | PHY Payload |

Figure 4: PHR format based on 15-09-0396-00