IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | DSSS Common Platform Summary Table |  |
| Date Submitted | 15 June 2009 |  |
| Source | [Clint Powell, Kuor-Hsin Chang, Roberto Aiello] [Self, self, self] [address] | Voice: [480-586-8457, 408-623-5093, 650-714-1184] Fax: [ ] E-mail: [cpowell@ieee.org, kchang60@yahoo.com, aiello@ieee.org] |
| Re: | [If this is a proposed revision, cite the original document.] [If this is a response to a Call for Contributions, cite the name and date of the Call for Contributions to which this document responds, as well as the relevant item number in the Call for Contributions.] [Note: Contributions that are not responsive to this section of the template, and contributions which do not address the topic under which they are submitted, may be refused or consigned to the “General Contributions” area.] |  |
| Abstract | Table summarizing all features proposed for DSSS common platform |  |
| Purpose | Promote technical discussion and assist in developing a merged DSSS PHY |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

DSSS Common Platform Working Summary

# Introduction

## Purpose and Scope

The following summarizes a potential DSSS Common Platform Proposal. The intent of this table is to capture the current “state” of the DSSS CP during the process of merging proposals, and to facilitate discussions necessary to reach convergence.  More details are captured in 15-09-0xxx-00 (and further revisions).

DSSS is a good fit for SUN’s requirements, as explained in the PAR, because it provides some advantages compared to other technologies.  Applications require “good communication range, robustness, and coexistence characteristics”, need “to keep infrastructure to a minimum”.

In addition, “Applications for Wireless Smart Metering Utility Network further intensify the need for maximum range as many devices are located sub-optimally. An example is Wireless Smart Metering Utility Network devices located in rural areas as at the end of electricity feeders where doubling range reduces cost by a factor of four as the area covered increases by the same factor”.

Even though data rate requirement is 40kbps, the system must offer as close to 100% coverage probability as possible.

## References

The content of this table is drawn form the following proposals submitted to TG4g.

|  | Document # | Document Title |
| --- | --- | --- |
|  | 15-09-0124-04 | multi-rate-phy-proposal |
|  | 15-09-0283-00 | adopting-flexible-dsss-modulation-for-phy-layer |
|  | 15-09-0295-01 | flexible-dsss-final-proposal |
|  | 15-09-0301-01 | two-rate-dsss-for-sun |
|  | 15-09-0303-01 | dsss-neighborhood-area-network-communications-proposal |
|  | 15-09-0356-00 | dynamic-dsss-draft-proposal-for-sun-presentation |

# DSSS Common Platform Summary Table

Remarks are things to discuss as a group (sub-group) via email reflector, conference calls, and face to face.

| DSSS Common Platform |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification |  | Proposed specification or approach | Remarks |  |
| Operating Frequency |  | Support multiple bands to include: 779; 787; China. 863; 870; EU. 902; 928; US. 2400; 2483; World Wide. | Any other bands for DSSS? |  |
| Transmit Power |  | Maximum TX power is regionally regulatory dependent |  |  |
| PHY Service Specification |  | Per 15.4-2006, extended as necessary (TBD) | Is 09-0280-00 section 6.2 a good place to start? |  |
| PPDU Format |  | Support for variable length preamble (as a function of PSDU Length), with length and bit pattern provided by MAC via PHY PIB | Other options/variants needed? Change needed in aMaxPHYPacketSize to 4095 octets. 4 PHR formats identified (see PPDU Variations). |  |
|  |  | 11 and 12 bit length field |  |  |
|  |  | Frame control to support signaling of FEC/data rate, forward compatibility via header extension bit |  |  |
|  |  | Packet Length Parity Check - to Identify Packet Length Errors and Save on Power |  |  |
|  |  | Extended CRC |  |  |
|  |  | Increase PHY Header to 2 or 3 Octets |  |  |
|  |  | Modified Uplink and Downlink Frame Structure to Support D-DSSS with RPMA |  |  |
|  |  | The spreading mode of SHR PHR and PHY payload can be adjusted separately by MAC via PHY PIB |  |  |
|  |  | The spreading mode of PHY payload can be adjusted real time, according to the pattern indicator at PHR field |  |  |
| Channel Spacing, Modulation, Data Rates, Band | Multiple data rates supported, band-specific choices: Chan Spacing MHz; Modulation; Chip Rate kcps; Bit Rate kbps; Band MHz; Note. 2; O-QPSK; 1000; 250 125 62.5; 779-787 902-928; Has not been explicitly proposed, but inferred in 15-09-0295-00. 1; O_QPSK; 500 / 1000(option); 500 125 63 31 16 8; 779-787. 0.6, 0.9; 2-GMSK; 400; 400 200 100 50 25 25 12.5 6.25; 863-870; LBT w/AFA. 2; O-QPSK; 1000; 562.5 750; 902-928; Using CA-CDM (Constant Amplitude Code Division Multiplex).. 1 2 4; D-BPSK; 500 1000 2000; 0.03-62.5 0.06-125 0.12-250; non band specific; Using D-DSSS w/RPMA. 5; O-QPSK; 2000; 500 250 125 62.5; 2400. | Any other spacing, mods, rates or bands for DSSS? |
| FEC | None Rate ½ |  |  | What type of FEC? |
| O-QPSK (G)MSK D-BPSK | Ala 802.15.4-2006 GMSK Modulation Parameters: h = 0.5, BT = 0.3 ... 0.5 |  |  |  |
| Spreading Code | 16-ary PN codes, 32-ary PN codes defined in IEEE 802.15.4-2006. 4-ary Orthogonal , CA-CDM. Block codes C(64,4) C(32,4) C(16,4) C(8,4). Parameterized gold code for 8-8192. |  |
| CRC Length | CRC-16 for short packets (Packet Length ≤ 127 bytes, legacy 802.15.4 packets) CRC-32 for long packets (Packet Length >127 bytes) |  |  |  |
| Transmit Power Control | Variable, controlled by MAC via PHY PIB Adopt PHY-PIB definitions in 09-0280-00 which supports power levels > 30dBm (such as up to 5W per 15-09-0299) |  |  | 09-0280-00 provides a method that supports implementation dependent granularity and range, solves the problem identified in 09-298 about limited range of the current 15.4 definition. |
| CCA Function | Apply LBT CCA modes in 15.4-2006. |  |  |  |
| Channel Quality Measurement | Provide LQI measurement (TBD). Work with TG4e for MAC layer features for channel quality. |  |  |  |
| Min. Rx Sens. | TBD |  |  |  |
| Min. Adj. Ch. Rej. | TBD |  |  |  |
| Beacon enabled network | Apply Beacon Enable Network in 15.4-2006 |  |  |  |

# PPDU Variations

| Octets: variable | 2 | 1 |  | 1 |  | variable |
| --- | --- | --- | --- | --- | --- | --- |
| Bits: variable | 16 | 7 | 1 | 5 | 3 |  |
| Preamble | SFD | Frame Length | Extended Length | Upper Extended Address | Future | PSDU Includes FCS |
| SHR |  | PHR |  |  |  | PHY Payload |

Figure 1: Structure of PPDU based on 15-09-0303-01 - Option 1:

| Octets: variable | 2 | 1 |  | 2 |  | variable |
| --- | --- | --- | --- | --- | --- | --- |
| Bits: variable | 16 | 7 | 1 | 12 | 4 |  |
| Preamble | SFD | Frame Length (value = 4) | Reserved | Extended Frame Length | Future | PSDU Includes FCS |
| SHR |  | PHR |  |  |  | PHY Payload |

Figure 2: Structure of PPDU based on 15-09-0303-01 - Option 2:

| Octets: variable | 2 | 1 |  | 1 |  |  | variable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: variable | 16 | 7 | 1 | 4 | 1 | 3 |  |
| Preamble | SFD | Frame Length | Extended Length | Upper Extended Address | Parity Check | Future | PSDU Includes FCS |
| SHR |  | PHR |  |  |  |  | PHY Payload |

Figure 3: Structure of PPDU based on 15-09-0295-00:

| Octets: variable | 2 | 1 |  | 1 |  |  | variable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: variable | 16 | 7 | 1 | 4 | 1 | 3 |  |
| Preamble | SFD | Frame Length | Extended Length | Upper Extended Address | Parity Check | Future | PSDU Includes FCS |
| SHR |  | PHR |  |  |  |  | PHY Payload |

| Figure 4: Structure of PPDU based on 15-09-0283-00: Octets: variable | 2 | 1 |  | 1 |  |  | variable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: variable | 16 | 7 | 1 | 4 | 3 | 1 |  |
| Preamble | SFD | Frame Length | Extended Length | Upper Extended Address | pattern indicator | Parity Check | PSDU Includes FCS |
| SHR |  | PHR |  |  |  |  | PHY Payload |