IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | A Modification of Required Changes for Low Energy Extension Document |  |
| Date Submitted | July 15, 2009 |  |
| Source | NICT, Fuji electronics, Mitsubishi, Tokyo Gas | Voice: [] E-mail: [f-kojima@nict.go.jp] |
| Re: | [] |  |
| Abstract | [This document contains proposed changes to the IEEE P802.15.4e Draft to address required changes for Low Energy.] |  |
| Purpose | [] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Content

1.	Summary of Changes	5

2.	Definitions	5

3.	New MAC frames	5

3.1	Existing Frames	5

3.2	Wakeup Frame	6

3.3	Secure Acknowledgement Frame (to be combined with other proposals)	7

3.4	New optional MHR field	8

4.	New MAC PIBs	8

5.	New Functional Description	8

7.5.1.1.1 Contention access period (CAP)	9

7.5.6 Transmission, reception and acknowledgement	9

7.5.6.7 <New Section>Coordinated Sampled Listening (CSL)	9

7.5.6.7.1 CSL idle listening	9

7.5.6.7.2 CSL transmission	9

7.5.6.7.2.1 Unicast transmission	9

7.5.6.7.2.2 Multicast transmission	10

7.5.6.7.2.3 Utilizing the optional CSL sync field	10

7.5.6.7.3 CSL reception	10

7.5.6.7.4 CSL over multiple channels	11

- Summary of Changes

- 3. Definitions – add Coordinated Sampled Listening concepts

- add The Receiver Initiated Transmission (RIT) concepts

- 7. MAC sublayer specification

  - 7.1. MAC sublayer service specification – no change

  - 7.2. MAC frame formats – add two new frame types and a new optional MHR field

  - 7.3. MAC command frames – no change

    - Modify Data Request command to allow broadcast and acknowledgement field set to zero

  - 7.4. MAC constants and PIB attributes – add five PIB attributes

    - Add one PIB attribute for low energy superframe utilization

    - Add two more additional PIB attributes for RIT

    - Add three additional PIB attributes for sample listening control

  - 7.5. MAC functional specification – add Coordinated Sampled Listening

    - Add and modify some sentences in 7.5.1 channel access section

    - Add RIT as an alternative for low duty cycle, low traffic load application with consecutive transmission restriction by regional regulations

  - 7.6. Security suite specifications – no change

  - 7.7. Message sequence charts illustrating MAC-PHY iteraction – no change

- Definitions

- Coordinated Sampled Listening (CSL): A low-energy mode to the MAC which allows receiving devices to periodically sample the channel(s) for incoming transmissions at low duty cycles.  The receiving device and the transmitting device are coordinated to reduce transmit overhead.

- CSL Period: The period in which receiving devices sample the channel(s) for incoming transmissions.

- CSL Phase: The length of time between now and the next channel sample.

- CSL Payload Frame: a beacon, data or command frame.

- CSL Wakeup Frame: a special short frame transmitted back-to-back before the payload frame to ensure its reception by CSL receiving device.

- CSL Wakeup Frame Sequence: a sequence of back-to-back wakeup frames up to the duration of the CSL Period.

- CSL Rendezvous Time (RZTime): 2-octet timestamp in wakeup frame payload indicating the expected length of time in milliseconds between the end of the wakeup frame transmission and the beginning of the payload frame transmission.

- CSL Channel Sample: The operation to perform ED on a channel and attempt to receive wakeup frame when energy is detected.

- New MAC frames

  - Existing Frames

Figure 41 shows the general MAC frame format of IEEE802.15.4-2006.

Figure 41 -- General MAC Frame Format

Figure 42 shows the 2-octet frame control field (FCF) in IEEE802.15.4-2006.

Figure 42 -- Format of the Frame Control Field

Table 79 contains the frame type values of the existing frames with two new frame types added.

Table 79―Values of the frame type subfield

| b2 b1 b0 | Description |
| --- | --- |
| 000 | Beacon |
| 001 | Data |
| 010 | Acknowledgment |
| 011 | MAC command |
| XXX | CSL Wakeup |
| YYY | Secure Acknowledgment |
| ZZZ-111 | Reserved |

  - Wakeup Frame

The following figure illustrates the format of wakeup frame.

| Octets: 2 FCF | 1 DSN | 2 Dest PAN | 2 Dest Addr | 2 RZTime | 2 FCS |
| --- | --- | --- | --- | --- | --- |

The FCF is set as follows :

- Frame type = XXX

- Security enabled = 0

- Frame pending = 0

- Ack request = 0

- PAN ID compression = 0

- Dest addressing mode = 0x2

- Frame version = 0x1

- Source addressing mode = 0x0

The payload of this frame contains a 2-octet Rendezvous Time (RZTime) which is the expected length of time in milliseconds between the end of the transmission of the wakeup frame and the beginning of the transmission of the payload frame.  This number is automatically filled in by the MAC layer when wakeup frames are constructed.  The last wakeup frame in a wakeup sequence must have RZTime = 0.

  - New optional MHR field

The reserved FCF reserved bit 7 is renamed to the CSL sync bit.  When the bit is set to 1, a 4-octet optional CSL sync field is added to the end of the current MHR.  The following figure illustrates the format of the CSL sync field.

| Octets: 2 CSL Phase | 2 CSL Period |
| --- | --- |

They represent the CSL phase and period of the transmitting device of the frame.  This informmation helps eliminate or reduce the wakeup sequence for subsequent transmissions from the receiving device.

  - Secure Acknowledgement Frame (to be combined with other proposals)

The following figure illustrates the format of an secure acknowledgement frame.

| Octets: 2 FCF | 1 DSN | 2 Dest PAN | 2 Dest Addr | 0/5/6/10/14 Auxiliary security header | 2 CSL Phase | 2 CSL Period | 2 FCS |
| --- | --- | --- | --- | --- | --- | --- | --- |

The FCF is set as follows :

- Frame type = YYY

- Security enabled = 0 or 1

- Frame pending = 0

- Ack request = 0

- PAN ID compression = 0

- CSL sync = 1

- Dest addressing mode = 0x2

- Frame version = 0x1

- Source addressing mode = 0x0

The payload of this frame contains 2-octet CSL Phase and 2-octet CSL Period.  They represent the CSL phase and period of the receiving device.  This information helps the transmitting device eliminate or reduce the wakeup sequence for subsequent transmissions to the same destination.

- New MAC PIBs

Table 86—MAC PIB attributes

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| cslPeriod |  | Integer | 0 … 1023 | CSL sampled listening period in milliseconds. 0 means always listening, i.e., CSL off. | 0 |
| cslMaxPeriod |  | Integer | 0 … 1023 | Maximum CSL sampled listening period in the entire PAN. 0 means cslMaxPeriod is the same as cslPeriod. | 0 |
| cslChannelMask |  | Integer |  | 32-bit bitmap relative to phyCurrentPage of channels. It represents the list of channels CSL operates on. 0 means CSL operates on phyCurrentChannel of phyCurrentPage. | 0 |
| cslFramePendingWaitT |  | Integer |  | Number of milliseconds to keep the receiver on after receiving a payload frame with FCF frame pending bit set to 1. |  |
| macSecAckWaitDuration |  | Integer |  | The maximum number of symbols to wait for a secure acknowledgement frame to arrive following a transmitted data frame. |  |
| macLowEnergySuperframeSupported |  | Boolean | TRUE or FALSE | Indication of whether the low energy superframe is operational or not. If this attribute is TRUE, the coordinator shall not transmit beacon frames regardless of BO value. | Imple-mentation specific |
| ritPeriod |  | Integer | 0 … 1023 | Receiver Initiated Transmission interval in 10 milliseconds. 0 means RIT off | 0 |
| ritDataWaitPeriod |  | Integer | 0 … 255 | In RIT, waiting period for Data frame after Data Request command in milliseconds |  |
| sslUseWakeUpSequence |  | Boolean | 0,1 | 1:Transmit frame using WakeUp sequence. 0:Transmit frame not using WakeUp sequence | 0 |
| sslSampleTiming |  | Integer | 0 … 1023 | Timing for sampling from receiving becon frame in milliseconds. | 0 |
| sslRemainRxEnablePeriod |  | Integer | 0 … 1023 | Remain RX-ENABLE period after receiving a wake up sequence and following data. 0 means return to sampling mode immediately. | 0 |

- New Functional Description

- 7.5.1.1. Superframe structure

- (Second paragraph) If BO = 15 and macLowEnergySuperframeSupported is FALSE, the coordinator shall not transmit beacon frames except when requested to do so, such as on receipt of a beacon request command. The value of macSuperframeOrder shall be ignored if BO = 15. Moreover, if macLowEnergySuperframeSupported is TRUE the coordinator shall not transmit beacon frames except when requested to do so, regardless of BO value.

- (Third paragraph) If BO = 15 and macLowEnergySuperframeSupported is FALSE, the superframe shall not exist (the value of macSuperframeOrder shall be ignored),

- 7.5.1.1.1. Contention access period (CAP)

- When cslPeriod is set to non-zero, CSL will be deployed in CAP.

- ritPeriod cannot be set to non-zero in case of beacon enabled PAN

- (Second paragraph) All frames, except acknowledgment frames and any data frame that quickly follows the acknowledgment of a data request command (see 7.5.6.3), transmitted in the CAP shall use a slotted CSMA-CA mechanism to access the channel. A device transmitting within the CAP shall ensure that its transaction is complete (i.e., including the reception of any acknowledgment) one IFS period (see 7.5.1.3) before the end of the CAP when macLowEnergySuperframeSupported is FALSE. If this is not possible, the device shall defer its transmission until the CAP of the following superframe. When macLowEnergySuperframeSupported is TRUE, on the other hand, transaction shall beensured to be completed one IFS period before the end of the inactive period. Finally, if a device senses frame in CAP that does not end within CAP when macLowEnergySuperframeSupported is TRUE, the device may continue receiving the frame until it ends before the end of the inactive period. When macLowEnergySuperframeSupported is TRUE, the coordinator shall not locate GTSs in order to avoid the interference from the frames in CAP. When macLowEnergySuperframeSupported is TRUE, the coordinator shall notify the devices that already associated or intend to associate the condition of macLowEnergySuperframeSupported in the beacon frames.

- .

- 7.5.1.2 Incoming and outgoing superframe timing

- (Second paragraph) The beacon order and superframe order shall may be equal for all superframes on a PAN. All devices shall may interact with the PAN only during the active portion of a superframe.

- 7.5.6 Transmission, reception and acknowledgement

- 7.5.6.7 <New Section>Coordinated Sampled Listening (CSL)

- The coordinated sampled listening (CSL) mode is turned on when the PIB attribute cslPeriod is set to non-zero and turned off when cslPeriod is set to zero.  In CSL mode, transmission, reception and acknowledgement work as follows.  Figure X illustrates the basic CSL operations.

- Figure X. Basic CSL operations

- 7.5.6.7.1 CSL idle listening

- During idle listening, CSL performs a channel sample every cslPeriod milliseconds.  If the channel sample does not detect energy on the channel, CSL disables receiver for cslPeriod milliseconds and then perform the next channel sample.  If the channel sample receives a wakeup frame, CSL checks the destination address in the wakeup frame.  If it matches macShortAddress, CSL disables receiver until the Rendezvous Time (RZTime) in the wakeup frame from now and then enables receiver to receive the payload frame.  Otherwise, CSL disables receiver until RZTime from now plus the transmission time of the payload frame and the secure acknowledgment frame and then resume channel sampling.

- 7.5.6.7.2 CSL transmission

- Each CSL transmission of a payload frame is preceded with a sequence of back-to-back wakeup frames (wakeup sequence).

- 7.5.6.7.2.1 Unicast transmission

- In unicast transmissions, the wakeup sequence length can be long or short based on the following two cases:

- Unsynchronized transmission: This is the case when the MAC layer does not know the CSL phase and period of the destination device.  In this case, the wakeup sequence length is cslMaxPeriod or cslPeriod if cslMaxPeriod is zero.

- Synchronized transmission: This is the case when the MAC layer knows the CSL phase and period of the destination device.  In this case, the wakeup sequence length is only the guard time against clock drift based on the last time when CSL phase and period updated about the destination device.

- If the next higher layer has multiple frames to transmit to the same destination, it can set the FCF frame pending bit to 1 in all but the last frame to maximize the throughput.

- CSL unicast transmission is performed in the following steps by the MAC layer:

- Perform CSMA-CA to acquire the channel

- If the previous acknowledged payload frame to the destination has the frame pending bit set and is within cslFramePendingWaitT milliseconds, go to step 5.

- If it is a synchronized transmission, wait until the destination device’s next channel sample.

- For the duration of wakeup sequence length (short or long)

  - Construct wakeup frame with the destination short address and remaing time to payload frame transmission (at the end of wakeup sequence)

  - Transmit wakeup frame

- Transmit payload frame

- Wait for up to macSecAckWaitDuration symbol time for the secure acknowledgement frame if the ack request subfield in the payload frame is set to 1.

- If the secure acknowledgment frame is received, update CSL phase and period information about the destination device from the acknowledgment CSL sync field.

- If the secure acknowledgement frame is not received, start retransmission process.

- 7.5.6.7.2.2 Multicast transmission

- Multicast transmission is the same as as unicast transmission except the following:

- It is always unsynchronized transmission.

- The destination address in wakeup frames is set to 0xffff.

- 7.5.6.7.2.3 Utilizing the optional CSL sync field

- Selectively the next higher layer may set the CSL sync bit in FCF in a frame to propogate CSL phase and period information among the neighboring devices. When the bit is set, the MAC layer automatically appends the CSL sync fields to the end of MHR.

- 7.5.6.7.3 CSL reception

- When a payload frame is received, the MAC layer performs the following steps:

- Immediately send back an secure acknowledgment frame with the destination address set as the transmitting device and its own CSL phase and period filled in the CSL sync field.  The acknowledgment frame can be optionally authenticated and/or encrypted depending on the current security mode.

- If CSL sync bit in the received payload frame is set to 1, the CSL phase and period information about the transmitting device is updated with the information in the CSL sync field.

- If FCF frame pending bit in the received payload frame is set to 1, keep receiver on for cslFramePendingWaitT milliseconds before going back to CSL idle listening.  Otherwise, start CSL idle listening.

- 7.5.6.7.4 CSL over multiple channels

- When cslChannelMask is set to non-zero, the CSL operations are extended to all the channels selected in the bitmap.  CSL idle listening performs channel sample on each channel from the lowest number to the highest in a round-robin fashion.  In the unsynchronized case, CSL transmission transmits a wakeup sequence of the length number_of_channels*cslMaxPeriod before each payload frame.  In the synchronized case, CSL transmission calculates the next channel sample time and channel number and transmits at the next channel sample time on the right channel with a short wakeup sequence.  In this case, CSL phase is the duration from now to the next channel sample on the first channel selected in cslChannelMask.

- 7.5.6.8 <New Section> Receiver Initiated Trasmission (RIT)

- The Receiver Initiated Transmission (RIT) mode is turned on when the PIB attribute ritPeriod is set to non-zero and turned off when ritPeriod is set to zero.  In RIT mode, transmission, reception and acknowledgement work as follows.

- RIT mode is applicable to low duty cycle, low traffic load type of applications and in the case that consecutive radio transmission is limited by regional regulation (e.g., 950MHz band in Japan).  cslPeriod and ritPeriod cannot be set to non-zero value at the same time. Figure X illustrates the basic RIT operations.

- Figure X. Basic RIT operations

- 7.5.6.8.1 RIT periodical Data request transmission

- In RIT, a node transmits Data Request command with broardcast address as destination and ack request field set to one. After the transmission of Data Request command, it waits ritDataWaitPeriod for Data frame and goes back to sleep till the next periodical Data Request transmission.

- 7.5.6.8.2 RIT Data transmission

- In RIT mode, MLME-DATA.request comes from the next higher layer, it enables the receiver to wait for Data Request command for the maximum of ritPeriod. In case that the sender node receives the Data Request command, it transmits Data frame.

- 7.5.6.8.2.1 Multicast transmission

- Multicast transmission is the same as as unicast transmission described above except the following:

- The sender node waits Data frame for ritPeriod even after the Data transmission to another device finishes.