IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Enhanced MAC Metrics |  |
| Date Submitted | [2 November 2009] |  |
| Source | [Tim Godfrey] [Craig Rodine] | Voice: [ ] Fax: [ ] E-mail: |
| Re: | [Task Group 15.4e: Enhancements to the 802.15.4 MAC] |  |
| Abstract | Metrics to enable higher layers to assess network performance and behavior and aide in MSDU segmentation decisions |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

- New MAC PIB Attributes

To be appended to Table 127

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macCounterBits † | 0x70 | integer | 8,16, or 32 | Defines the counter size in bits for attributes 0x71 through 0x78. | 8 |
| macRetryCount † | 0x71 | integer | 0 – 2^macCounterBits-1 | The number of transmitted frames that required exactly one retry before acknowledgement | 0 |
| macMultipleRetryCount † | 0x72 | integer | 0 – 2^macCounterBits-1 | The number of transmitted frames that required more than one retry before acknowledgement | 0 |
| macTXFailCount † | 0x73 | integer | 0 – 2^macCounterBits-1 | The number of transmitted frames that did not result in an acknowledgement after macMaxFrameRetries | 0 |
| macTXSuccessCount † | 0x74 | integer | 0 – 2^macCounterBits-1 | The number of transmitted frames that were acknowledged within macAckWaitDuration after the initial data frame transmission | 0 |
| macFCSErrorCount † | 0x75 | integer | 0 – 2^macCounterBits-1 | The number of received frames that discarded due to an incorrect FCS | 0 |
| macSecurityFailure † | 0x76 | integer | 0 – 2^macCounterBits-1 | The number of received data frames that were returned from the procedure described in 7.5.9.2.3 (Incoming frame security procedure) with any status other than “SUCCESS” | 0 |
| macDuplicateFrameCount † | 0x77 | integer | 0 – 2^macCounterBits-1 | The number of received data frames that contained the same sequence number as a frame previously received (accounting for wrap-around of macDSN). | 0 |
| macRXSuccessCount † | 0x78 | integer | 0 – 2^macCounterBits-1 | The number of received data frames that were received correctly | 0 |

Insert the following section following Table 127:

7.4.3 MAC Performance Metrics

MAC PIB attributes 0x71 through 0x78 are provided to enable the assessment of network performance and connection quality by higher layers. MAC PIB attribute 0x70 (macCounterBits) defines the size of the counters providing attributes 0x71 through 0x78. The value shall be 8, 16, or 32 bits (at the implementer’s choice depending on the PHY characteristics and other considerations). The counters implementing MAC PIB attributes 0x71 through 0x78 shall wrap to 0 when incremented beyond their maximum value (2^macCounterBits-1).

Attributes 0x71 through 0x78 are read only, but may be reset by higher layers. Writes to these attributes through MLME-SET.request shall discard the value parameter and reset the attribute to 0.

The attributes macRetryCount, macMultipleRetryCount, macTXFailCount, macTXSuccessCount relate to data frame transmission. Each MSDU transferred into the MAC layer through the MCPS-DATA.request primitive shall increment exactly one of these four attribute counters depending on the final disposition of the frame as described in Table 127.

The attributes macFCSErrorCount, macSecurityFailure, macDuplicateFrameCount, macRXSuccessCount relate to data frame reception. Each MSDU transferred out of the MAC layer through the MCPS-DATA.indication primitive shall increment at least one of these four attribute counters based on the status of the frame as described in Table 127.

To create a profile of PHY-related metrics, it is recommended that higher layers store relevant parameters of MCPS-DATA.indication such as mpduLinkQuality and DataRate for each unique Source Address (SrcAdr). The higher layers may also query PLME-ED.request to establish an idle channel noise measurement.