IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Revision to MAC Enhancements for Time Slotted Channel Hopping |  |
| Date Submitted | 4 December 2009 |  |
| Source | [Jonathan Simon] [Kuor Hsin Chang] [Chol Su Kang] [Rick Enns] [Kris Pister] [Ludwig Winkel] [Clint Powell] [Jose Gutierrez] | Voice: [ ] Fax: [ ] E-mail: [ jsimon@dustnetworks.com] [ kuor-hsin.chang@us.elster.com ] [ cholsukang@gmail.com] [ enns@stanfordalumni.org ] [ pister@eecs.berkeley.edu ] [ ludwig.winkel@siemens.com ] [ cpowell@ieee.org] [ Jose.Gutierrez@emerson.com] |
| Re: | Task Group 15.4e: Enhancements to the 802.15.4 MAC |  |
| Abstract | This document describes the revision to the Time Slotted Channel Hopping draft for 4e with document number of 15-09-0281-05-004e. |  |
| Purpose | The purpose of this document is to enhance the Time Slotted Channel Hopping draft for 4e. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Changes to document 604 rev 3

7.1.1.1.3 Line 20-21 – Remove text “using the linkHandle provided in linkHandleList. MAC should transfer the requested data SPDU on one of these links.”

7.1.21.1.1.2 Table 120.a – replace row “size” as follows:

| size | Integer | 0x0000-0xffff | Number of timeslots in the new slotframe |
| --- | --- | --- | --- |

7.1.21.1.2.3 line 33 – change “add-on” to “add”

7.1.21.2.1.2 Table 120.c – replace row “chanOffset” as follows:

| chanOffset | Integer | 0x00-0xnn | nn = number of active channels in the channel map for the slotframe used |
| --- | --- | --- | --- |

7.1.21.4.2.2 Line 7 – Add text “If the any of the fields of the MLME-LISTEN.request are not valid, the status of the primitive shall indicate INVALID PARAMETER.”

7.1.21.5.2.1 Table 120.l – replace “slotframes[]” row as follows

| Slotframes[] | Table 120.m | Table 120.m | See table 120.m. Slotframes and links are from received Advertisement command frame. |
| --- | --- | --- | --- |

Add tables 120.m and 120.n as follows:

Table 120.m—MLME-ADVERTISE.indicate slotframe parameters (per slotframe)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| slotframeId | Integer | 0x00 – 0xFF | Slotframe ID. |
| slotframeSize | Integer | 0x0000 – 0xFFFF | Slotframe size. |
| numLink | Integer | 0x0 – 0xF | Number of links for the specified slotframe |
| links | Table 120.n | Table 120.n | Table 120.n for parameters (per link) |

Table 120.n—MLME-ADVERTISE.indicate Link parameters (per link)

| timeslot | Integer | 0x0000 – 0xFFFF | Timeslot. |
| --- | --- | --- | --- |
| chanOffset | Integer | 0x00 – 0xFF | Channel offset. |
| linkOption | Enumeration | TX RX SHARED_TX | Option of the link. |

7.1.21.5.3.2 Line 5 – Add text “If the any of the fields of the MLME-ADVERTISE.request are not valid, the status of the primitive shall indicate INVALID PARAMETER.”

7.1.21.6.1.1 Line 15 – remove linkHandle from MLME-KEEP-ALIVE.request.

7.1.21.6.2.2 Line 12.  Add text “If the dstAddr of the MLME-KEEP-ALIVE.request is not 0xFFFF and the dstAddr does not exist in the devices neighbor table, the status of the primitive shall indicate INVALID PARAMETER.”

7.1.21.7.2.1 Line 22 – remove linkHandle from MLME-JOIN.indication.

7.1.21.7.2.1 Table 120.s – remove linkHandle row from table

7.1.21.7.3.2 Line 19 - Add text “If the any of the fields of the MLME-JOIN.request are not valid, the status of the primitive shall indicate INVALID PARAMETER.”

7.1.21.8.1.1 Table 120.x – replace row “chanOffset” as follows”

| chanOffset | Integer | 0x00-0xnn | nn = number of active channels in the channel map for the slotframe used |
| --- | --- | --- | --- |

7.1.21.8.1.3 Line 1 – remove text “The MAC shall send the Activate command frame to the node using the linkHandle parameter.”

7.1.21.8.2.1  Line 13 – add “slotframes[]” parameter to the MLME-ACTIVATE.indication primitive

7.1.21.8.2.1 Table 120.y – add row to end of table

| Slotframes[] | Table 120.z | Table 120.z | See table 120.z. Slotframes and links are from received Activate command frame. |
| --- | --- | --- | --- |

Add tables 120.z and 120.aa as follows:

Table 120.z—MLME-ACTIVATE.indicate slotframe parameters (per slotframe)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| slotframeId | Integer | 0x00 – 0xFF | Slotframe ID. |
| slotframeSize | Integer | 0x0000 – 0xFFFF | Slotframe size. |
| numLink | Integer | 0x0 – 0xF | Number of links for the specified slotframe |
| links | Table 120.aa | Table 120.aa | Table 120.aa for parameters (per link) |

Table 120.aa—MLME-ACTIVATE.indicate Link parameters (per link)

| timeslot | Integer | 0x0000 – 0xFFFF | Timeslot. |
| --- | --- | --- | --- |
| chanOffset | Integer | 0x00 – 0xFF | Channel offset. |
| linkOption | Enumeration | TX RX SHARED_TX | Option of the link. |

7.1.21.8.2.3 Line 22 - Add text “The device management layer should use the information in the MLME-ACTIVATE.indication to add slotframes and links using the MLME-SET-SLOTFRAME.request and MLME-SET-LINK.request primitives.  The device management layer should delete slotframes and links obtained from previous advertisements if not explicitly added in the activate command frame” after “…set up secure connections.” and before “Resolution between …”.

7.1.21.8.3.2 Line 5 – Add text “If the any of the fields of the MLME-ACTIVATE.request are not valid, the status of the primitive shall indicate INVALID PARAMETER.”

7.3.10.1.1 Figure 103.a – Insert column after Timeslot Template and Hopping Sequence ID:

| Octets: Variable (see 7.2.2.4) | 1 | 6 | 1 | 1 | 1 | 22 |
| --- | --- | --- | --- | --- | --- | --- |
| MHR | Command Frame Identifier | Timing Informa-tion | Security Control Field | Join Control | Timeslot Template and Hopping Sequence ID | Timeslot Template (without Timeslot Template ID) |

7.3.10.1.2 Line 6 – change text “an ExtendedAddress” to “aExtendedAddress” (note to editor, this is a MAC constant from table 127)

7.3.10.1.7 – After this section, insert a section titled “Timeslot Template” with text “The Timeslot Template field shall be set to the macTimeslotTemplate from the MAC PIB (see table 127.d) corresponding to the template ID in the previous field, minus the timeslot template ID.”

7.3.10.1.10 – Remove (duplicate of 7.3.10.1.9)

7.3.10.2.2 Line 11 – change text “an ExtendedAddress” to “aExtendedAddress” (note to editor, this is a MAC constant from table 127)

7.3.10.3.2 Line 18 – insert “)” after “addressing.”

7.3.10.3.2 Line 21 – change text “macPANId” to “macPANId,” (note to editor, there are other instances of PIB values not being italicized in the PA section)

7.3.10.3.2 Line 22 – change text “a CoordShortAddress” to “macCoordShortAddress,” (note to editor, this is a MAC constant from table 127)

7.4.2.2.1 Table 127.a – change row “macDisconnectTime” as follows:

| macDisconnectTime | 0x64 | Integer | 0x0000-0xFFFF | Time to send out Disassociate frames before disconnecting |  |
| --- | --- | --- | --- | --- | --- |

7.4.2.2.3 Table 127.c  - remove “Join” from description of linkType

7.5.1.5.1 Line 12-13 – Replace text “Mapping of logical channel to physicalchannel is to be jointly defined with Channel Hopping/Channel Diversity subgroup” with text “A hopping sequence is a sorted list of physical channels that correspond to the logical channels selected. There must be a one-to-one mapping of logical channel to physical channel for two devices operating in a PA-network to agree on which channel is in use, and as such hopping sequence lengths are limited in a PA-network to the number of channels in use. Hopping sequence ID 0 is reserved to indicate that the logical channel and physical channel match. If other hopping sequences are to be used, a higher layer must define them and these must be configured prior to joining a PA-network.”