IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Revision to Low Energy text in 15-09-0604-04-004e draft |  |
| Date Submitted | Monday January 18, 2010 |  |
| Source | [Wei Hong] [510 2nd St., Suite 410 San Francisco, CA USA] | Voice: [+1 415-692-0828] E-mail: [whong@archrock.com] |
| Re: | [] |  |
| Abstract | [This document contains revisions to low energy text in 15-09-0604-04-004e draft.] |  |
| Purpose | [] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

- Revisions to Low Energy text in 15-09-0604-04-004e

- Section M.5 (page 152): replace the entire section with the following:

- The Low Energy (LE) mechanisms are suitable for applications that are willing to trade low latency for low energy consumption.  They allow radios to operate down to a fraction of 1% duty cycles while presenting an always-on illusion.  Devices can always talk to each other without any pre-arranged synchronization schedules.  The Low Energy mechanisms are applicable in the non-beacon mode as well as in the CAP periods of the beacon mode.  It is also possible for the upper layer to temporarily turn off the Low Energy mechanisms by operating the radio at 100% duty cycle for emergency messages.

- There are two Low Energy mechanisms CSL and RIT.  CSL is suitable for applications with relatively low latency requirements, e.g., < 1 second.  RIT is suitable for applications with a high latency tolerance, e.g., tens of seconds.  RIT is also required in cases where the local regulation limits the duration of continuous transmissions to too small a period for CSL to be effective.

- 7.3.14.2 (page 100): Replace title to CSL Wakeup Frame.  Move entire section to 7.2.7.2.  It does not belong in command frames.

- 7.3.14.4 (page 101): move entire section to 7.3.14.1.3.