IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | FSK Modulation Diagram |  |
| Date Submitted | [21 January 2010] |  |
| Source | [Ben Rolfe] [Blind Creek Associates] | Voice: [ ] Fax: [ ] E-mail: [ ben@blindcreek.com ] |
| Re: | [FSK pre-ballot comment resolution] |  |
| Abstract | Diagram showing modulation as described by comment # 24 (and related comments). Rev 2 adds text. |  |
| Purpose | Resolves comments from doc #0031, CIDs: 214, 106, 182, 184, 215, 24, 279, 248, 28, 45, 328. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Resolves comment IDs: 214, 106,182,184,215,24,279,248,28,45,328

The following diagram is submitted to accompany the text in the comment resolution XLS.

Bit to symbol mapping for 4 level FSK modulation:  Resolution was to accept the mapping as proposed in CID#24 from 10-0031.

The nominal frequency deviation, Δf, shall be the symbol rate * modulation index / 2. For 2-level FSK/GFSK modulation, symbol '0' shall be modulated on - Δf and symbol '1' shall be modulated on +Δf. For 4-level FSK/GFSK modulation, two bits shall be mapped to each symbol. The symbol encoding for both 2-level and 4-level FSK/GFSK modulation is shown in Table 75a.

| 2FSK/GFSK |  |
| --- | --- |
| Symbol (Binary) | Deviation * Δf |
| 0 | -1 |
| 1 | +1 |
| 4FSK/GFSK |  |
| Symbol (Binary) | Deviation * Δf |
| 10 | +3 |
| 11 | +1 |
| 01 | -1 |
| 00 | -3 |

Table m1: FSK symbol encoding