IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title |  |  |
| Date Submitted | [21 January, 2010] |  |
| Source | [Matt Boytim, Kory Fifield] [Sensus] [114 Northpark Blvd. Suite 10 Covington, Louisiana 70433] | Voice: [(765)432-2463] Fax: [ ] E-mail: [Matt.Boytim@sensus.com] [kory.fifield@sensus.com] |
| Re: | Clause 6_fsk_122409.pdf |  |
| Abstract | This document contains tables for reference in comments to the document “Clause 6_fsk_122409.pdf” regarding the SUN amendment to 802.15.4. Specifically, it contains information to replace the “Dedicated use bands” TBD’s in Table 1 and Table 1a. |  |
| Purpose | The frequency bands and modulation modes contained are recommended for inclusion in the SUN amendment 802.15.4. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

| PHY | Frequency | Spreading Parameters |  | Data parameters |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (MHz) | (MHz) | Chip rate | Modulation | Bit rate | Symbol rate | Symbols |
|  |  | kchip/sec |  | (kb/s) | (ksymbols/s) |  |
| 220 | 220-222 | - | GFSK | 5 | 5 | Binary |
|  |  | - | GFSK | 10 | 10 | Binary |
|  |  | - | GFSK | 20 | 20 | Binary |
|  |  | - | GFSK | 40 | 40 | Binary |
| 450 | 450-470 | - | GFSK | 5 | 5 | Binary |
|  |  | - | GFSK | 10 | 10 | Binary |
|  |  | - | GFSK | 20 | 20 | Binary |
|  |  | - | GFSK | 40 | 40 | Binary |
| 863 | 863-870 | - | GFSK | 5 | 5 | Binary |
|  |  | - | GFSK | 10 | 10 | Binary |
|  |  | - | GFSK | 10 | 10 | Binary |
|  |  | - | GFSK | 20 | 20 | Binary |
|  |  | - | GFSK | 40 | 40 | Binary |
|  |  | - | 4-GFSK | 20 | 10 | 4-ary |
|  |  | - | 4-GFSK | 40 | 20 | 4-ary |
|  |  | - | 4-GFSK | 80 | 40 | 4-ary |
| 896 | 896-902 | - | GFSK | 5 | 5 | Binary |
|  | 928-960 | - | GFSK | 10 | 10 | Binary |
|  |  | - | GFSK | 10 | 10 | Binary |
|  |  | - | GFSK | 20 | 20 | Binary |
|  |  | - | GFSK | 40 | 40 | Binary |
|  |  | - | 4-GFSK | 20 | 10 | 4-ary |
|  |  | - | 4-GFSK | 40 | 20 | 4-ary |
|  |  | - | 4-GFSK | 80 | 40 | 4-ary |
| 1427 | 1427-1452 | - | GFSK | 5 | 5 | Binary |
|  | 1492-1518 | - | GFSK | 10 | 10 | Binary |
|  | 1605-1625 | - | GFSK | 20 | 20 | Binary |
|  | 1800-1830 | - | GFSK | 40 | 40 | Binary |
|  |  | - | 4-GFSK | 20 | 10 | 4-ary |
|  |  | - | 4-GFSK | 40 | 20 | 4-ary |
|  |  | - | 4-GFSK | 80 | 40 | 4-ary |

Table 1

| Frequency band (MHz) | Parameter | Mandatory data rate | Optional data rate #1 | Optional data rate #2 |
| --- | --- | --- | --- | --- |
| 220 | Data rate (kb/s) | 5/10 | 20 | 40 |
|  | Modulation | GFSK (BT=0.5) | GFSK (BT=0.5) | GFSK (BT=0.5) |
|  | Modulation index | 0.5 | 0.5 | 0.5 |
|  | Channel spacing (kHz) | 12.5/25 | 50 | 100 |
| 450-470 | Data rate (kb/s) | 5/10 | 20 | 40 |
|  | Modulation | GFSK (BT=0.5) | GFSK (BT=0.5) | GFSK (BT=0.5) |
|  | Modulation index | 0.5 | 0.5 | 0.5 |
|  | Channel spacing (kHz) | 12.5/25 | 50 | 100 |
| 863-870 | Data rate (kb/s) | 5/10 | 20/40 | 80 |
|  | Modulation | GFSK (BT=0.5) | 4-GFSK (BT=0.5) | 4-GFSK (BT=0.5) |
|  | Modulation index | 0.5 | 0.33 | 0.33 |
|  | Channel spacing (kHz) | 12.5/25 | 25/50 | 100 |
| 896-902 | Data rate (kb/s) | 5/10 | 20/40 | 80 |
| 928-960 | Modulation | GFSK (BT=0.5) | 4-GFSK (BT=0.5) | 4-GFSK (BT=0.5) |
|  | Modulation index | 0.5 | 0.33 | 0.33 |
|  | Channel spacing (kHz) | 12.5/25 | 25/50 | 100 |
| 1427-1452 | Data rate (kb/s) | 5/10 | 20/40 | 80 |
| 1492-1518 | Modulation | GFSK (BT=0.5) | 4-GFSK (BT=0.5) | 4-GFSK (BT=0.5) |
| 1605-1625 | Modulation index | 0.5 | 0.33 | 0.33 |
| 1800-1830 | Channel spacing (kHz) | 12.5/25 | 25/50 | 100 |

Table 1A