IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15.7 Task Group Visible-Light Communication (TG-VLC) |  |
| --- | --- | --- |
| Title | Color stabilization for CSK |  |
| Date Submitted | 9th February 2010 |  |
| Source | Joachim W. Walewski and Michael Bahr | Voice: +49-89-636-45850 Fax: E-mail: joachim.walewski@siemens.com |
| Re: |  |  |
| Abstract | We describe an extension of CSK for duplex links, so that the point of gravity of the CSK constellation diagram can be kept stable in case of changes in the output characteristics of a CSK transmitter. This contribution is intended as a seamless extension of the Rx stabilization scheme already described in IEEE 802.15.7 D0. Both PHY and MAC mechanisms are discussed in detail. |  |
| Purpose | Contribution of standard-text material for the preparation of IEEE 802.15.7 D0 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Document # | 15-10-0089-02-0007 |  |

- Nomenclature

| Symbol | Declaration |
| --- | --- |
| □(b) | Binary representation of □ |
| {.}A/D | A/D converted value of {.} |
| {.}D/A | D/A converted value of {.} |
| ak | D/A scaling factor for color k |
| bk | A/D scaling factor for color k |
| a | One-dimensional quantity |
| a | Column vector |
| A | Matrix |
| ekl | Sensitivity of photo detector for colour l when receiving colour k [A/W] |
| i | ac. current [A] |
| I | Current [A] |
| IDC | Bias current |
| p | a.c. component of optical power [W] |
| P | Optical power [W] |
| PD | Photodetector |
| qk | ~ quantum efficiency for LED of color k [W/A] |
| r, g, b | Signals for red, green, and blue |
| Rx | Receiver |
| s | Signal |
| t | Time [s] |
| tkl | Transmittance from LED k to photodetector l |
| T | Transmittance matrix [unit 1] |
| Tcomp | Time between two color compensations [s] |
| Tx | Transmitter |

- Description of use case

The quantum efficiency of LEDs are dependent on their operation time and age and also on various environmental conditions, temperature being the most important factor. When using CSK modulation this leads to a change of the center of gravity and a distortion of the CSK constellation diagram of the emitted signal. Clause 6.9.2.2.2 describes how the receiver adapts to such changes. In this Clause we describe how an extension of the same mechanism can be used for counteracting the above change on the transmitter side and hence to stabilize the color of the light emitted. Notice that this compensation scheme only is applicable to duplex links.

- Control loop

The control-loop model for the color-stabilization scheme is shown in Figure X 1. Calibration sequences as discussed in Clause 4 are sent from the tri-band Tx to the tri-band Rx, where they are used to calculate compensation factors for the driving currents fed back to the LEDs in the tri-band Tx. A back link is used to relay these factors back to the tri-band Tx, where they are used to correct the LED driving currents in order to compensate for the distortion of the CSK constellation diagram and its misplaced center of gravity.

Figure X 1 – Control loop for a color-stabilized CSK link. The calibration sequences are discussed in detail in Clause 4. The back link used for routing the compensation factors back to the tri-band Tx does not need to be a CSK link.

- PHY-layer mechanism

  - Equal ac and dc quantum efficiency

First, we discuss the case for which the characteristic functions of the employed LEDs are (close to) linear, and the quantum efficiency for dc driving currents is the same as for ac driving currents. Figure X 2 displays the alteration of the characteristic function due to a change in quantum efficiency. For an ac driving current i the initial radiant power drops from p0 to p’0. In order to regain the radiant power p0 the driving current has to be changed to i’. The compensation factor for the ac current is defined as

c:=i'i.

In the following we explain how the compensation factors are inferred from the channel matrix H for a CSK link.

Figure X 2 – Characteristic curve of an LED (radiant power as a function of driving current) for two quantum efficiencies and ac driving currents.

As shown in Figure X 3 the signals to be transmitted, viz.sTx(b), are translated into the ac current vector iTx by aid of D/A convertors. The ac component of the tri-band optical output of the transmitter is, according to , given by

pTx=QiTx,

where the elements of the diagonal matrix Q are proportional to quantum efficiencies of the LEDs in each band. Only a portion t of the emitted ac radiant power in each band is detected by the detectors on the receiver side.

Figure X 3 – Definition of system vectors and matrices for a CSK link.

According to Figure X 3 and the received ac radiant power is

pRx=TpTx

and the pertinent ac current output of the photodetectors

iRx=EpRx,

where E represents the spectral sensitivity of the photodetectors. In a next step the ac currents are translated into digital representations, viz.sRx(b), by aid of D/A convertors.

Figure X 4 – Definition of further transfer matrices for a CSK link.

So, in essence, the relation between the transmitted and received signal is given by

sRx(b)=BETQAsTx(b)D/AA/D,

and the channel matrix H is defined as (see Figure X 3 and Figure X 4)

H:=BETQA,

the elements of which are estimated by aid of the scheme outlined in Clause 6.9.2.2.2.

A change in the quantum efficiency of an LED in any of the three bands, i.e. q  q’, results in the change of matrix Q to Q’ and thus to a change of the channel matrix H to H’. Instead of adjusting the received signals according to the compensation scheme in Clause 6.9.2.2.2, compensation factors for the driving currents of the CSK-modulated Tx are inferred and relayed back to Tx. A diagonal matrix of compensation values is chosen so that

sTx(b)→C(b)sTx(b),

and thus

sRx(b)≈H'(b)C(b)sTx(b)

for a sufficient bit resolution of the A/D and D/A convertors. With the above definitions one finds that

C(b)=H'(b)−1H(b).

The correction values sent back to the Tx are

c(b)=ci(b)cj(b)ck(b)=diagC(b).

Figure X 5 – Implementation of a color-stabilized CSK link. In the CSK receiver a channel-estimation module in the receiver estimates compensation factors and sends them back to the CSK transmitter via an additional channel.

  - DC quantum efficiency different from ac quantum efficiency

For practical LEDs the dc characteristic function is different from the corresponding ac function due to, e.g., saturation and a thermal inertia of the LED module.

For the estimation of the dc correction factors one sends each of the four Walsh slots displayed in Figure 3y so many times over that the LEDs have reached their thermal equilibrium. However, the length of the total training sequence has to be shorter than the inverse of the cuff-off frequency. The last receive values at end of each Walsh slot are then used to estimate a channel Matrix HDC. The DC compensation values are inferred with the same scheme as in Clause 4.1. In terms of a nonlinear characteristic dc curve, more than one iteration is needed in order to arrive at the initial center of gravity for the CSK constellation diagram. This iteration only converges in case the characteristic dc curve is strictly monotonic.

- MAC-layer mechanisms

  - General guidelines for estimation of the compensation factors

In order for the proposed compensation scheme to function a reference setting of the channel matrix H is needed. In the following this reference is referred to as H0. Recurring estimations of the channel matrix at time tc result in the estimates Hc. If the elements of Hc differ significantly from those of the diagonal elements of H0, the ac driving currents are corrected with the diagonal elements of Hc(b)−1H(b). A decision about whether an update can be made by the value of the diagonal elements of C(b) itself. If these values differ significantly from 1, an update of the current compensation factors is made. Whether this difference is significant can either be decided by preset value (e.g., 5% difference) or the decision threshold can be estimated from the data itself. One example is to acquire a histogram of the diagonal values of C(b). A significant value can then be defined as a value of the diagonal elements lying outside a pre-defined confidence interval, e.g. the 95% confidence interval.

In order to avoid damage or saturation of the LEDs or a bit-level saturation of the A/D and D/A convertors, one can introduce a-priori maximum levels for the three values in sTx(b).

Change that affects all channels similarly, e.g. a change in the link length, can be identified by comparing all diagonal values of C(b). If all elements experience the same relative change the compensation factors for the Tx driving currents shall not be updated.

All of the above applies in spirit to the scheme outlined in Clause 4.2.

If the center of gravity is to be moved, e.g. for a CSK-modulated lighting device, the output color of which is to be changed upon a user request, the updated of the compensation factors has to be suppressed while this change is conducted. After the move, H0 has to be updated in order to represent the new color setting.

If the off-diagonal elements of C(b) significantly differ from zero this indicates other impairments of the CSK-modulated link than those hither discussed. An example for such impairment is the blocking of only one of the photodiodes on the receiver side against cross talk. E.g., photodiode i still receives light emitted from LED i, but not longer from the other bands. In such a case the Tx driving current shall not be compensated for. Instead one can use this information for, e.g., error messages created by the CSK-modulated Tx, the Rx, or both. Another option is to update H0 and to use the update for further compensation.

  - MAC-layer signaling

    - DC and ac quantum efficiency equal

In order to perform the color stabilization process according to Clause 4.1, the respective CSK-modulated transmitter (host device) and CSK receiver (client device) perform the message sequence as shown in Figure X 6.

If the host device wants to start the color stabilization process, it sends the ‘Request ac color compensation’ command to the client device. The ac training frame that is embedded in the PHY Header serves the client device for the estimation of the current channel properties. The corresponding values of the channel matrix are saved as the reference matrix H0 Also, the client device sends an acknowledgment message ‘Ack ac’ (viz. acknowledge color compensation) back to the host device, confirming the capability of performing the ac color stabilization process and confirming the reception of the ac color-compensation request.

If the host device does not receive the ‘Ack ac’ meassage within macAckColorCompWaitTime time, it resends the “Request ac color compensation” message. After macReqColorCompRetries unsuccessful retries, the host device assumes that the client device is not capable of performing the color-stabilization process.

After receiving an “Ack ac’, a color-compensation training message, initiated by the host device, is sent to the client. The ‘Training ac’ message contains the training information necessary for ac training.

By aid of this message the current channel property is inferred on the client side and saved in the matrix H’. In contrast to the Rx compensation scheme outlined in Clause 6.9.2.2.2  not the current channel matrix, viz. H’, but rather H0 is used for equalization of the received signals (i, j, k  x, y). Based on the current channel matrix H’ and the reference matrix H0  the client calculates the compensation vector c and sends it back to the host device in a ‘AC compensation coefficients’ message.

Upon the reception of the ‘AC compensation coefficients’ message containing the ac compensation vector, the host device decides whether a current correction is mandated, and if yes multiplies the vector elements in the compensation module (see Figure X 5). Examples for pertinent decision strategies are outlined in Clause ‏5.1).

After a pre-defined time Tcomp  the host sends again a training sequence in a ‘Training ac’ message as described above and the client device updates the current channel matrix H’, infers the compensation vector c, and sends it back to the host device in a ‘AC compensation coefficients’ message.

This cycle is repeated with the frequency (Tcomp-1), until the host device sends a stop message (‘End color compensation’) to the client device.

Figure X 6 – Message-sequence chart describing the MAC exchange for stabilizing the center of gravity of the CSK constellation diagram for the case that ac and dc characteristic curves exhibit the same slopes.

    - DC quantum efficiency different from ac quantum efficiency

In order to perform the color stabilization process according to Clause 4.1, the respective CSK-modulated transmitter (host device) and CSK receiver (client device) perform the message sequence as shown in Figure X 7.

If the host device wants to start the color stabilization process, it sends the ‘Request ac & dc color compensation’ command to the client device. The ac & dc training frames that are embedded in the PHY header, or, alternatively in the MAC header, serve the client device for the estimation of the current channel properties. The corresponding values of the channel matrixes are saved as the reference matrix H0 and HDC,0, respectively . Also, the client device sends an acknowledgment message ‘Ack ac & dc’ (viz. acknowledge color compensation) back to the host device, confirming the capability of performing the ac color stabilization process and confirming the reception of the ac color-compensation request. This message may contain additional status and capability information, which are not further defined here.

If the host device does not receive the ‘Ack ac & dc’ message within macAckColorCompWaitTime time, it resends the “Request ac & dc color compensation” message. After macReqColorCompRetries unsuccessful retries, the host device assumes that the client device is not capable of performing the color-stabilization process.

After receiving an “Ack ac & dc’, a color-compensation training message, initiated by the host device, is sent to the client. The pertinent ‘Training ac & dc’ message contains the training information necessary for ac & dc training.

By aid of this message the current channel property is inferred on the client side and saved in the matrix In contrast to the Rx compensation scheme outlined in Clause 6.9.2.2.2  not the current channel matrix, viz. H’, but rather H0 is used for equalization of the received signals (i, j, k  x, y). Based on the current channel matrixes H’ and H’DC and the reference matrixes H0  and HDC,0, the client calculates the compensation vectors c and cDC and sends them back to the host device in a ‘AC & dc compensation coefficients’ message.

Upon the reception of the ‘AC & dc compensation coefficients’ message containing the ac and dc compensation vectors, the host device decides whether a current correction is mandated, and if yes multiplies the vector elements in the compensation module (see Figure X 5). Examples for pertinent decision strategies are outlined in Clause ‏5.1).

After a pre-defined time Tcomp  the host sends again a training sequence in a ‘Training ac & dc’ message as described above and the client device updates the current channel matrixes H’ and H’DC, infers the compensation vectors c and cDC, and sends them back to the host device in a ‘AC & dc compensation coefficients’ message.

This cycle is repeated with the frequency (Tcomp-1), until the host device sends a stop message (‘End color compensation’) to the client device.

Figure X 7 - Message-sequence chart describing the MAC exchange for stabilizing the center of gravity of the CSK constellation diagram for the case that ac and dc characteristic curves exhibit different slopes.

    - Command-frame identifier

| Command-frame identifier | Command name | RFD |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | Tx | Rx |  |
| 0x0f | AC color stabilization |  |  | ??? |
| 0x10 | AC & dc color stabilization |  |  | ??? |

Table X 1 - Command-frame identifiers for color-stabilization purposes

    - Color-stabilization command

The color-stabilization command defines a set of command frames used during the process of color stabilization. The general structure of the Color Stabilization command is shown in .

| Octets: 7 | 1 | 1 | Variable |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier | Color-stabilization command identifier | Command-specific information |

Table X 2 – Structure of Color-stabilization command

The MHR fields are the MAC header fields for a command frame according to the MAC specification.

The Command-frame identifier field contains the value ac compensation or for ac & dc compensation as defined by Table X 1, which indicates an ac color-stabilization command frame or an ac& dc color-stabilization command frame.

The color-stabilization-command identifier field contains an integer value that identifies the specific color stabilization command. Possible color stabilization commands and their corresponding values are given in .

| Command Value | Command |
| --- | --- |
| 0x00 | Request color compensation |
| 0x01 | Acknowledge color compensation |
| 0x02 | Color-compensation training |
| 0x03 | Color-compensation coefficients |
| 0x04 | End color compensation |

Table X 3 – Color-stabilization command identifiers valid for ac and ac & dc compensation.

      - ‘Request ac color compensation’ command

The ‘Request ac color compensation’ command is used to initiate the subsequent transmission of ac color-compensation vectors by the client device.

The ‘Request ac color compensation’ command is sent in a color-stabilization command frame with the value of the color-compensation command-identifier field set to the value in , indicating the request of color compensation.

The structure of the ‘Request ac color compensation’ command follows the general structure of the Color Stabilization command as shown in . The structure of the Specific Information field is shown in .

| Octets: 7 | 1 | 1 | 0 |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier for ‘Ac color stabilization’ (Table X 1) | Color-stabilization command identifier ‘Request color compensation’ (Table X 3) | N/A |

Table X 4 – Structure of specific information for “Request ac color compensation” command.

      - Request ‘AC & dc color compensation’ command

The ‘Request ac color compensation’ command is used to initiate the subsequent transmission of ac & dc color-compensation vectors by the client device.

The ‘Request ac & dc color compensation’ command is sent in a color-stabilization command frame with the value of the color-compensation command-identifier field set to the value in , indicating the request of color compensation.

The structure of the ‘Request ac & dc color compensation’ command follows the general structure of the Color Stabilization command as shown in . The structure of the Specific Information field is shown in .

| Octets: 7 | 1 | 1 | 0 |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier: for ‘AC & dc color stabilization’ (Table X 1) | Color-stabilization command identifier ‘Request color compensation’ (Table X 3) | N/A |

Table X 5 – Structure of specific information for “Request ac & dc color compensation” command.

      - ‘Acknowledge ac color compensation’ command

The ‘Acknowledge ac color compensation’ command (‘Ack ac’) is used to acknowledge the receipt of a “Request ac color compensation’ command and to simultaneously indicate the capability to perform the requested color stabilization procedure. It may contain additional status and capability information.

The Acknowledge Color Compensation command is sent in a color-stabilization command frame with the value of the color-compensation command-identifier field set to the value of , indicating Acknowledge color compensation.

The structure of the ‘Acknowledge ac color compensation’ command follows the general structure of the color-stabilization command as shown in . The structure of the Specific Information field is shown in .

| Octets: 7 | 1 | 1 | User-defined |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier for ‘Ac color stabilization’ (Table X 1) | Color-stabilization command identifier for ‘Acknowledge color compensation’ (Table X 3) | User-defined (additional status and capability information) |

Table X 6 – Structure of Specific Information for Acknowledge Color Compensation command

      - ‘Acknowledge ac & dc color compensation’ command

The ‘Acknowledge ac color compensation’ command (‘Ack ac & dc’) is used to acknowledge the receipt of a “Request ac & dc color compensation’ command and to simultaneously indicate the capability to perform the requested color stabilization procedure. It may contain additional status and capability information.

The Acknowledge Color Compensation command is sent in a color-stabilization command frame with the value of the color-compensation command-identifier field set to the value of , indicating ‘Acknowledge color compensation’.

The structure of the ‘Acknowledge ac & dc color compensation’ command follows the general structure of the color-stabilization command as shown in . The structure of the Specific Information field is shown in .

| Octets: 7 | 1 | 1 | User-defined |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier: for ‘AC & dc color stabilization’ (Table X 1) | Color-stabilization command identifier for ‘Acknowledge color compensation’ (Table X 3) | User-defined (additional status and capability information) |

Table X 7 – Structure of Specific Information for ‘Acknowledge ac & dc’ color compensation command

      - ‘Training ac’ command

The ‘Training command ac’ is used to provide input for the channel-matrix estimator in the client device (see Figure X 5).

The ‘Training ac’ command is sent in a color-stabilization command frame with the value of the color-compensation command-identifier field set to the value of , indicating color-compensation training.

The structure of the ‘Training ac’ command follows the general structure of the Color Stabilization command as shown in . The structure of the Specific Information field is shown in Figure xxx

| Octets: 7 | 1 | 1 | See clause ??? |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier for ‘Ac color stabilization’ (Table X 1) | Color-stabilization command identifier for ‘Color-compensation coefficients’ (Table X 3) |  |

Table X 8 –Structure of specific information for ‘Training ac’ command

      - ‘Training ac & dc’ command

The ‘Training command ac & dc’ is used to provide input for the channel-matrix estimator in the client device (see Figure X 5).

The ‘Training ac & dc’ command is sent in a color-stabilization command frame with the value of the color-compensation command-identifier field set to the value of , indicating color-compensation training.

The structure of the ‘Training ac’ command follows the general structure of the color-stabilization command-identifier as shown in. The structure of the Specific Information field is shown in .

| Octets: 7 | 1 | 1 | See clause ??? |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier: for ‘AC & dc color stabilization’ (Table X 1) | olor-stabilization command identifier for ‘Color-compensation coefficients’ (Table X 3) |  |

Table X 9 –Structure of specific information for ‘Training ac’ command

      - ‘AC compensation coefficients’ command

The ‘AC compensation coefficients’ command is used to transfer the compensation vector c from the client to the host device.

The ‘AC compensation coefficients’ command is sent in a color-stabilization command frame with the value of the color-compensation command-identifier field set to the value of Table X 3 indicating color-compensation coefficients.

The structure of the “AC compensation coefficients” command follows the general structure of the Color Stabilization command as shown in Table X 2. The structure of the Specific Information field is shown in Table X 10

| Octets: 7 | 1 | 1 | 6 |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier: for ‘AC color stabilization’ (Table X 1) | Color-stabilization command identifier for ‘Color-compensation coefficients’ (Table X 3) | AC compensation-factor vector |

Table X 10 – Structure of specific information for ‘AC compensation coefficients’ command.

      - ‘AC & dc compensation coefficients’ command

The ‘AC & dc compensation coefficients’ command is used to transfer the compensation vectors c and cDC from the client to the host device.

The ‘AC & dc compensation coefficients’ command is sent in a color-stabilization command frame with the value of the color-compensation command-identifier field set to the value of Table X 3 indicating color-compensation coefficients.

The structure of the “AC & dc compensation coefficients’ command follows the general structure of the Color Stabilization command as shown in Table X 2. The structure of the Specific Information field is shown in Table X 10

| Octets: 7 | 1 | 1 | 6 | 6 |
| --- | --- | --- | --- | --- |
| MHR fields | Command-frame identifier: for ‘AC & dc color stabilization’ (Table X 1) | Color-stabilization command identifier for ‘Color-compensation coefficients’ (Table X 3) | AC compensation-factor vector | DCcompensation-factor vector |

Table X 11 – Structure of specific information for ‘AC compensation coefficients’ command.

5.2.4.5 ‘End color compensation’ command

The ‘End color compensation’ command is used to stop the color stabilization process.

The ‘End color compensation’ command is sent in a color-stabilization command frame with the value of the color-compensation command-identifier field set to the value in Table X 3 indicating ‘End color compensation’.

The structure of the ‘End color compensation’ command follows the general structure of the color-stabilization command as shown in Table X 2. The structure of the Specific Information field is shown in Table X 12.

| Octets: 7 | 1 | 1 | 0 |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier: for ‘AC color stabilization’ or ‘AC & dc color stabilization’ (Table X 1), depending on what stabilization process to be terminated | Color-stabilization command identifier for ‘End color compensation (Table X 3) | N/A |

Table X 12 – Structure of Specific Information for ‘End color compensation’ command