- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Modified text clause 6.9.1.1 |  |
| Date Submitted | [10 February, 2010] |  |
| Source | [Dae Ho Kim, Sang-Kyu Lim, Ill Soon Jang, Tae-Gyu Kang] [ETRI] [Daejeon, Korea] | Voice:[+82-42-860-5648] Fax: [+82-42-860-5218] E-mail:[dhkim7256@etri.re.kr] |
| Re: | [Response to committee remedy of TG 7 comment resolution] |  |
| Abstract | [Clause 6.9.1.1 (Line Code 4B6B) has been modified according to the committee remedy of TG 7 comment resolution.] |  |
| Purpose | [Description of what the authors wants P802.15 to do with the information in the document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

      - Line Code 4B6B

Line code 4B6B expands 4-bit codes to 6-bit symbols in order that the ratios of 1 and 0 are the same (3:3) and the duty cycle in 6-bit one symbol becomes always 50 %. It can help to acquire non-flickering, DC-balance, disparity, and clock recovery by enough state changes.

Encoding table of line code 4B6B is presented in Error: Reference source not foundx.

- Table x– Encoding table of line code 4B6B

| Name | 4b | 6b | Description |
| --- | --- | --- | --- |
| 0 | 0000 | 001110 | Hex data 0 |
| 1 | 0001 | 001101 | Hex data 1 |
| 2 | 0010 | 010011 | Hex data 2 |
| 3 | 0011 | 010110 | Hex data 3 |
| 4 | 0100 | 010101 | Hex data 4 |
| 5 | 0101 | 100011 | Hex data 5 |
| 6 | 0110 | 100110 | Hex data 6 |
| 7 | 0111 | 100101 | Hex data 7 |
| 8 | 1000 | 011001 | Hex data 8 |
| 9 | 1001 | 011010 | Hex data 9 |
| A | 1010 | 011100 | Hex data A |
| B | 1011 | 110001 | Hex data B |
| C | 1100 | 110010 | Hex data C |
| D | 1101 | 101001 | Hex data D |
| E | 1110 | 101010 | Hex data E |
| F | 1111 | 101100 | Hex data F |

The main features are as follows.

- Always 50% duty cycle during one symbol

- DC balanced line code

- Error detection capability

- Run length is limited 4

- Allow reasonable clock recovery

- Figure 32-An example of 4B6B at NRZ OOK