IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15.7 Task Group Visible-Light Communication (TG-VLC) |  |
| --- | --- | --- |
| Title | Revised text for Clause 5.5.3.1.2 |  |
| Date Submitted | March 2010 |  |
| Source | Joachim W. Walewski and Michael M. Bahr, Siemens AG, Corporate Technology, Communication Technologies | Voice: +49-89-636-45850 Fax: E-mail: joachim.walewski@siemens.com |
| Re: |  |  |
| Abstract | Revised text for Clause 5.5.3.1.2. |  |
| Purpose | Contribution of standard-text material for the preparation of IEEE 802.15.7 D2 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Document # | 15-10-0152-03-0007 |  |

5.5.3.1.2 Inter-frame Flicker Compensation

The average brightness discrepancy between the data transmission time and the idle time might result in

accidental inter-frame flicker because the VLC light source might always be in the "ON" or "OFF" state during idle time while

no data is transmitted. Therefore, to compensate the inter-frame flicker, after each data frame one sends an idle pattern, whose average

brightness is equal to that of the data frame; that is, the adjustment pattern is chosen so, that the emitted optical energy of the data and the adjacent idle frame together are independent of the content of the data frame and equal from one transmission cycle to the next.  The idle pattern has the same duty cycle as

the modulated frame, but the pulse repetition rate may be set lower so as not to cause "in-band" modulation

interference with any VLC modulation. For example, one can send a constant signal.  Any types of idle pattern can be used for compensation.

It is preferable that the idle patterns do not conflict with the code words generated by the line code for the data.