IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Draft Text for Narrowband Physical Layer |
| Date Submitted | 18 March, 2010 |
| Source | Anuj Batra, Texas Instruments Peter Bradley, Zarlink Semiconductor Clint Chaplin, Samsung Mark Dawkins, Toumaz Technology Shinsuke Hara, NICT Srinath Hosur, Texas Instruments Dong-Sun Kim, KETI Jaehwan Kim, ETRI Young-Hwan Kim, KETI Ryuji Kohno, NICT Cheolhyo Lee, ETRI Seung-Hoon Park, Samsung JuneChul Roh, Texas Instruments Didier Sagan, Zarlink Semiconductor Jae-Gi Son, KETI Kenichi Takizawa, NICT Alan Wong, Toumaz Technology |
| Re: |  |
| Abstract | Draft text for narrowband physical layer |
| Purpose | Discussion |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Table of Contents

1	Acronyms and abbreviations	4

2	Narrowband PHY Specification	5

2.1	Data Rate-dependent Parameters	6

2.1.1	402 – 405 MHz	6

2.1.2	420 – 450 MHz	6

2.1.3	863 – 870 MHz	7

2.1.4	902 – 928 MHz	7

2.1.5	950 – 956 MHz	7

2.1.6	2360 – 2400 MHz	8

2.1.7	2400 – 2483.5 MHz	8

2.2	PLCP Preamble	9

2.3	PLCP Header	12

2.3.1	PHY Header	12

2.3.2	Header Check Sequence	14

2.3.3	BCH Encoder for PLCP Header	14

2.4	PSDU	15

2.4.1	Data Scrambler	15

2.4.2	BCH Encoder for PSDU	16

2.4.3	Pad Bits	18

2.4.4	Spreading	18

2.4.5	Bit Interleaver	18

2.4.6	Constellation Mapping	19

2.5	General Requirements	21

2.5.1	Operating Frequency Bands	21

2.5.2	Channel Numbering	21

2.5.3	Preamble Sequence Assignment	22

2.6	PHY Layer Timing	23

2.6.1	Inter-frame Spacing	23

2.6.2	Receive-to-Transmit Turnaround Time	23

2.6.3	Transmit-to-Receive Turnaround Time	24

2.6.4	Time between Successive Transmissions	24

2.6.5	Center Frequency Switch Time	24

2.7	Transmitter Specifications	24

2.7.1	Transmit PSD Mask	24

2.7.2	Transmit Power	25

2.7.3	Transmit power-on and power-down ramp	25

2.7.4	Transmit Center Frequency Tolerance	26

2.7.5	Symbol Clock Frequency Tolerance	26

2.7.6	Clock Synchronization	26

2.7.7	Transmitter Constellation Error	26

2.7.8	Adjacent Channel Power Ratio	27

2.8	Receiver Specifications	27

2.8.1	Receiver Sensitivity	27

2.8.2	Adjacent Channel Rejection	28

2.8.3	Receiver Energy Detection	28

2.8.4	ED Threshold	28

2.8.5	ED Measurement Time	29

2.8.6	Receiver Clear Channel Assessment	29

# Acronyms and abbreviations

BCH Code	Bose, Ray-Chaudhuri, Hocquenghem Code

DBPSK	Differential Binary Phase-Shift Keying

DQPSK	Differential Quadrature Phase-Shift Keying

D8PSK	Differential 8-Phase-Shift Keying

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

GFSK	Gaussian Frequency-Shift Keying

FCS	Frame Check Sequence

HCS	Header Check Sequence

MAC	Media Access Control

PHY	Physical Layer

PLCP	Physical Layer Convergence Protocol

PPDU	Physical Layer Protocol Data Unit

PSDU	Physical Service Data Unit

SIFS	Short Inter-frame Spacing

SRRC 	Square-Root Raised Cosine

# Narrowband PHY Specification

This clause specifies a narrowband PHY option for IEEE 802.15.6. The PHY is responsible for the following tasks:

- Activation and deactivation of the radio transceiver

- CCA within the current channel

- Data transmission and reception

This clause also provides a method for transforming a physical-layer service data unit (PSDU) into a physical-layer protocol data unit (PPDU). During the transmission, the PSDU shall be pre-appended with a physical-layer preamble and a physical-layer header in order to create the PPDU. At the receiver, the physical-layer preamble and physical-layer header serve as aids in the demodulation, decoding and delivery of the PSDU.

The PLCP header is the second main component of the PPDU (see Section ). The purpose of this component is to convey the necessary information about the PHY and MAC parameters to aid in decoding of the PSDU at the receiver.  The PLCP header can be further decomposed into a RATE field, a LENGTH field, a SCRAMBLER SEED, a BURST MODE field, reserved bits, a header check sequence (HCS), and BCH parity bits. The BCH parity bits are added in order to improve the robustness of the PLCP header. The PLCP header shall be transmitted using the given header data rate in the operating frequency band.

The PSDU is the last component of the PPDU (see Section 2.4). This component is formed by concatenating the MAC header with the MAC frame body and frame check sequence (FCS). The PSDU is then scrambled and optionally encoded by a BCH code. The PSDU shall be transmitted using any of the available data rates available in the operating frequency band.

Figure 1 — Standard PPDU structure

When transmitting the packet, the PLCP preamble is sent first, followed by the PLCP header and finally the PSDU.  All multiple byte fields shall be transmitted with least significant byte first and each byte shall be transmitted with least significant bit (LSB) first.

A compliant device shall be able to support transmission and reception in one of the following frequency bands: 402 – 405 MHz, 420 – 450 MHz, 863 – 870 MHz, 902 – 928 MHz, 950 – 956 MHz, 2360 – 2400 MHz and 2400 – 2483.5 MHz.

## Data Rate-dependent Parameters

The data-rate dependent parameters for each of the possible frequency bands of operation are provided in subsequent sections. This standard is intended to conform to established regulations in United States, Europe, Japan and Korea.

### 402 – 405 MHz

A compliant device shall be able to support transmission and reception at a data rate of 75.9, 151.8 and 303.6 kbps.

Table 1 — Modulation parameters for PLCP Header and PSDU

| Packet Component | Modulation | Symbol Rate (ksps) | Code Rate (k/n) | Spreading Factor (S) | Pulse Shape | Information Data Rate (kbps) | Support |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PLCP Header | /2-DBPSK | 187.5 | 19/31* | 2 | SRRC | 57.5 | Mandatory |
| PSDU | /2-DBPSK | 187.5 | 51/63 | 2 | SRRC | 75.9 | Mandatory |
| PSDU | /2-DBPSK | 187.5 | 51/63 | 1 | SRRC | 151.8 | Mandatory |
| PSDU | /4-DQPSK | 187.5 | 51/63 | 1 | SRRC | 303.6 | Mandatory |
| PSDU | /8-D8PSK | 187.5 | 51/63 | 1 | SRRC | 455.4 | Optional |
| * BCH (31, 19) code is a shortened code derived from a BCH (63, 51) code |  |  |  |  |  |  |  |

### 420 – 450 MHz

A compliant device shall be able to support transmission and reception at a data rate of 75.9 and 151.8 kbps.

Table 2 — Modulation parameters for PLCP Header and PSDU

| Packet Component | Modulation | Symbol Rate (ksps) | Code Rate (k/n) | Spreading Factor (S) | BT | Information Data Rate (kbps) | Support |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PLCP Header | GMSK | 187.5 | 19/31* | 2 | 0.5 | 57.5 | Mandatory |
| PSDU | GMSK | 187.5 | 51/63 | 2 | 0.5 | 75.9 | Mandatory |
| PSDU | GMSK | 187.5 | 51/63 | 1 | 0.5 | 151.8 | Mandatory |
| PSDU | GMSK | 187.5 | 1/1 | 1 | 0.5 | 187.5 | Optional |
| * BCH (31, 19) code is a shortened code derived from a BCH (63, 51) code |  |  |  |  |  |  |  |

### 863 – 870 MHz

A compliant device shall be able to support transmission and reception at a data rate of 101.2, 202.4 and 404.8 kbps.

Table 3 — Modulation parameters for PLCP Header and PSDU

| Packet Component | Modulation | Symbol Rate (ksps) | Code Rate (k/n) | Spreading Factor (S) | Pulse Shape | Information Data Rate (kbps) | Support |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PLCP Header | /2-DBPSK | 250 | 19/31* | 2 | SRRC | 76.6 | Mandatory |
| PSDU | /2-DBPSK | 250 | 51/63 | 2 | SRRC | 101.2 | Mandatory |
| PSDU | /2-DBPSK | 250 | 51/63 | 1 | SRRC | 202.4 | Mandatory |
| PSDU | /4-DQPSK | 250 | 51/63 | 1 | SRRC | 404.8 | Mandatory |
| PSDU | /8-D8PSK | 250 | 51/63 | 1 | SRRC | 607.1 | Optional |
| * BCH (31, 19) code is a shortened code derived from a BCH (63, 51) code |  |  |  |  |  |  |  |

### 902 – 928 MHz

A compliant device shall be able to support transmission and reception at a data rate of 121.4, 242.9 and 485.7 kbps.

Table 4 — Modulation parameters for PLCP Header and PSDU

| Packet Component | Modulation | Symbol Rate (ksps) | Code Rate (k/n) | Spreading Factor (S) | Pulse Shape | Information Data Rate (kbps) | Support |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PLCP Header | /2-DBPSK | 300 | 19/31* | 2 | SRRC | 91.9 | Mandatory |
| PSDU | /2-DBPSK | 300 | 51/63 | 2 | SRRC | 121.4 | Mandatory |
| PSDU | /2-DBPSK | 300 | 51/63 | 1 | SRRC | 242.9 | Mandatory |
| PSDU | /4-DQPSK | 300 | 51/63 | 1 | SRRC | 485.7 | Mandatory |
| PSDU | /8-D8PSK | 300 | 51/63 | 1 | SRRC | 728.6 | Optional |
| * BCH (31, 19) code is a shortened code derived from a BCH (63, 51) code |  |  |  |  |  |  |  |

### 950 – 956 MHz

A compliant device shall be able to support transmission and reception at a data rate of 101.2, 202.4 and 404.8 kbps.

Table 5 — Modulation parameters for PLCP Header and PSDU

| Packet Component | Modulation | Symbol Rate (ksps) | Code Rate (k/n) | Spreading Factor (S) | Pulse Shape | Information Data Rate (kbps) | Support |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PLCP Header | /2-DBPSK | 250 | 19/31* | 2 | SRRC | 76.6 | Mandatory |
| PSDU | /2-DBPSK | 250 | 51/63 | 2 | SRRC | 101.2 | Mandatory |
| PSDU | /2-DBPSK | 250 | 51/63 | 1 | SRRC | 202.4 | Mandatory |
| PSDU | /4-DQPSK | 250 | 51/63 | 1 | SRRC | 404.8 | Mandatory |
| PSDU | /8-D8PSK | 250 | 51/63 | 1 | SRRC | 607.1 | Optional |
| * BCH (31, 19) code is a shortened code derived from a BCH (63, 51) code |  |  |  |  |  |  |  |

### 2360 – 2400 MHz

A compliant device shall be able to support transmission and reception at a data rate of 121.4, 242.9, 485.7 and 971.4 kbps.

Table 6 — Modulation parameters for PLCP Header and PSDU

| Packet Component | Modulation | Symbol Rate (ksps) | Code Rate (k/n) | Spreading Factor (S) | Pulse Shape | Information Data Rate (kbps) | Support |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PLCP Header | /2-DBPSK | 600 | 19/31* | 4 | SRRC | 91.9 | Mandatory |
| PSDU | /2-DBPSK | 600 | 51/63 | 4 | SRRC | 121.4 | Mandatory |
| PSDU | /2-DBPSK | 600 | 51/63 | 2 | SRRC | 242.9 | Mandatory |
| PSDU | /2-DBPSK | 600 | 51/63 | 1 | SRRC | 485.7 | Mandatory |
| PSDU | /4-DQPSK | 600 | 51/63 | 1 | SRRC | 971.4 | Mandatory |
| * BCH (31, 19) code is a shortened code derived from a BCH (63, 51) code |  |  |  |  |  |  |  |

### 2400 – 2483.5 MHz

A compliant device shall be able to support transmission and reception at a data rate of 121.4, 242.9, 485.7 and 971.4 kbps.

Table 7 — Modulation parameters for PLCP Header and PSDU

| Packet Component | Modulation | Symbol Rate (ksps) | Code Rate (k/n) | Spreading Factor (S) | Pulse Shape | Information Data Rate (kbps) | Support |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PLCP Header | /2-DBPSK | 600 | 19/31* | 4 | SRRC | 91.9 | Mandatory |
| PSDU | /2-DBPSK | 600 | 51/63 | 4 | SRRC | 121.4 | Mandatory |
| PSDU | /2-DBPSK | 600 | 51/63 | 2 | SRRC | 242.9 | Mandatory |
| PSDU | /2-DBPSK | 600 | 51/63 | 1 | SRRC | 485.7 | Mandatory |
| PSDU | /4-DQPSK | 600 | 51/63 | 1 | SRRC | 971.4 | Mandatory |
| * BCH (31, 19) code is a shortened code derived from a BCH (63, 51) code |  |  |  |  |  |  |  |

## PLCP Preamble

A preamble shall be added prior to the PLCP header in order to aid the receiver in packet detection, timing synchronization and carrier-offset recovery. Two unique preambles are defined in order to mitigate false alarms due to networks operating on adjacent channels. The mapping between channel number and preamble is defined in Section 2.5.3. Each preamble is constructed by concatenating a length-63 m-sequence with a 010101010101101101101101101 extension sequence. The former sequence can be used to implement packet detection, coarse-timing synchronization and carrier-offset recovery, while the latter sequence can be used to implement fine-timing synchronization.

The two preamble sequences are defined in Table 8 and Table 9. The preambles shall be transmitted at the symbol rate for the desired band of operation and will be encoded using the same modulation parameters as defined for the PLCP Header in the preceding tables

Table 8 — Preamble Sequence #1

| Bit | Bit Value | Bit | Bit Value | Bit | Bit Value | Bit | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b0 | 0 | b23 | 0 | b46 | 1 | b69 | 0 |
| b1 | 1 | b24 | 0 | b47 | 0 | b70 | 1 |
| b2 | 0 | b25 | 1 | b48 | 0 | b71 | 0 |
| b3 | 1 | b26 | 1 | b49 | 0 | b72 | 1 |
| b4 | 0 | b27 | 1 | b50 | 0 | b73 | 0 |
| b5 | 1 | b28 | 0 | b51 | 1 | b74 | 1 |
| b6 | 1 | b29 | 0 | b52 | 0 | b75 | 1 |
| b7 | 0 | b30 | 0 | b53 | 0 | b76 | 0 |
| b8 | 0 | b31 | 1 | b54 | 0 | b77 | 1 |
| b9 | 1 | b32 | 0 | b55 | 0 | b78 | 1 |
| b10 | 1 | b33 | 1 | b56 | 0 | b79 | 0 |
| b11 | 0 | b34 | 1 | b57 | 1 | b80 | 1 |
| b12 | 1 | b35 | 1 | b58 | 1 | b81 | 1 |
| b13 | 1 | b36 | 1 | b59 | 1 | b82 | 0 |
| b14 | 1 | b37 | 0 | b60 | 1 | b83 | 1 |
| b15 | 0 | b38 | 0 | b61 | 1 | b84 | 1 |
| b16 | 1 | b39 | 1 | b62 | 1 | b85 | 0 |
| b17 | 1 | b40 | 0 | b63 | 0 | b86 | 1 |
| b18 | 0 | b41 | 1 | b64 | 1 | b87 | 1 |
| b19 | 1 | b42 | 0 | b65 | 0 | b88 | 0 |
| b20 | 0 | b43 | 0 | b66 | 1 | b89 | 1 |
| b21 | 0 | b44 | 0 | b67 | 0 |  |  |
| b22 | 1 | b45 | 1 | b68 | 1 |  |  |

Table 9 — Preamble Sequence #2

| Bit | Bit Value | Bit | Bit Value | Bit | Bit Value | Bit | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b0 | 0 | b23 | 1 | b46 | 1 | b69 | 0 |
| b1 | 1 | b24 | 0 | b47 | 1 | b70 | 1 |
| b2 | 1 | b25 | 0 | b48 | 0 | b71 | 0 |
| b3 | 0 | b26 | 1 | b49 | 0 | b72 | 1 |
| b4 | 1 | b27 | 0 | b50 | 0 | b73 | 0 |
| b5 | 0 | b28 | 0 | b51 | 1 | b74 | 1 |
| b6 | 0 | b29 | 1 | b52 | 1 | b75 | 1 |
| b7 | 0 | b30 | 1 | b53 | 1 | b76 | 0 |
| b8 | 1 | b31 | 1 | b54 | 0 | b77 | 1 |
| b9 | 0 | b32 | 1 | b55 | 1 | b78 | 1 |
| b10 | 0 | b33 | 0 | b56 | 0 | b79 | 0 |
| b11 | 0 | b34 | 0 | b57 | 1 | b80 | 1 |
| b12 | 0 | b35 | 0 | b58 | 1 | b81 | 1 |
| b13 | 1 | b36 | 0 | b59 | 1 | b82 | 0 |
| b14 | 0 | b37 | 0 | b60 | 1 | b83 | 1 |
| b15 | 1 | b38 | 1 | b61 | 1 | b84 | 1 |
| b16 | 1 | b39 | 1 | b62 | 1 | b85 | 0 |
| b17 | 0 | b40 | 0 | b63 | 0 | b86 | 1 |
| b18 | 0 | b41 | 1 | b64 | 1 | b87 | 1 |
| b19 | 1 | b42 | 1 | b65 | 0 | b88 | 0 |
| b20 | 0 | b43 | 1 | b66 | 1 | b89 | 1 |
| b21 | 1 | b44 | 0 | b67 | 0 |  |  |
| b22 | 0 | b45 | 0 | b68 | 1 |  |  |

## PLCP Header

A PLCP header shall be added after the PLCP preamble to convey information about the PHY and MAC parameters that is needed at receiver in order to decode the PSDU.  The BCH encoded PLCP header shall be formed as shown in Figure 2:

- Format the PHY header as shown in Figure 2 based on information  provided by the MAC

- Calculate the 4-bit HCS value over the PHY header using the CRC-4 ITU polynomial: 1+x+x4

- Apply a BCH (31, 19) code, which is shortened code derived from a BCH (63, 51) code, to the concatenation of the PHY header (15 bits) and HCS (4 bits)

The resulting encoded bits are modulated using the appropriate parameters specified in Section 2.1 for the desired frequency band of operation.

Figure 2 — Block diagram of PLCP header construction

### PHY Header

The PHY header contains information about the data rate of the MAC frame body, the length of the MAC frame body (which does not include the MAC header or the FCS) and information about the next packet – whether it is being sent in a burst mode.

The PHY header field shall be composed of 15 bits, numbered from 0 to 14 as illustrated in Figure 3. Bits 0-2 shall encode the RATE field, which conveys the information about the type of modulation, the symbol rate, frequency deviation or pulse shape, the coding rate, and the spreading factor used to transmit the PSDU. Bits 4-11 shall encode the LENGTH field, with the least-significant bit being transmitted first. Bit 13 shall encode whether or not the packet is being transmitted in the burst (streaming) mode. Bit 14 shall encode the scrambler seed. All other bits which are not defined in this Clause shall be understood to be reserved for future use and shall be set to zero.

Figure 3 — PHY Header bit assignment

#### Data Rate (RATE)

Depending on the data rate (RATE), bits R0–R2 shall be set according to the values in Table 10.

Table 10 — Rate-dependent parameters

|  | 402 – 405 MHz | 420 – 450 MHz | 863 – 870 MHz, 950 – 956 MHz | 902 – 928 MHz | 2360-2400 MHz 2400 – 2483.5 MHz |
| --- | --- | --- | --- | --- | --- |
| R0 – R2 | Data Rate (kbps) | Data Rate (kbps) | Data Rate (kbps) | Data Rate (kbps) | Data Rate (kbps) |
| 000 | 75.9 | 75.9 | 101.2 | 121.4 | 121.4 |
| 100 | 151.8 | 151.8 | 202.4 | 242.9 | 242.9 |
| 010 | 303.6 | 187.5 | 404.8 | 485.7 | 485.7 |
| 110 | 455.4 | Reserved | 607.1 | 728.6 | 971.4 |
| 001 | Reserved | Reserved | Reserved | Reserved | Reserved |
| 101 | Reserved | Reserved | Reserved | Reserved | Reserved |
| 011 | Reserved | Reserved | Reserved | Reserved | Reserved |
| 111 | Reserved | Reserved | Reserved | Reserved | Reserved |

#### PLCP Length Field (LENGTH)

The PLCP length field shall be an unsigned 8-bit integer that indicates the number of un-coded information bytes in the MAC frame body (which does not include the MAC header or the FCS).

#### Burst Mode Field (BM)

The MAC shall set the burst mode (BM) bit, as defined in Table 11, to indicate whether the next packet is part of a packet “burst”, i.e. burst mode transmission. In burst mode, the inter-frame spacing shall be equal to a pMIFS (see Section 2.6.4).

Table 11 — Burst Mode field

| Burst Mode (BM) bit | Next Packet Status |
| --- | --- |
| 0 | Next packet is not part of burst |
| 1 | Next packet is part of burst |

#### Scrambler Seed Mode Field (SS)

The MAC shall set the scrambler seed bit (SS) according to the scrambler seed identifier value. This bit value corresponds to the seed value chosen for the data scrambler.

### Header Check Sequence

The PHY header shall be protected with a 4-bit (CRC-4 ITU) header check sequence (HCS). The HCS shall be the ones complement of the remainder generated by the modulo-2 division of the PHY header by the polynomial: 1+x+x4. The HCS bits shall be processed in the transmit order. A schematic of the processing order is shown in Figure 4. The registers shall be initialized to all ones.

Figure 4 — Block diagram of a CRC-4 implementation

### BCH Encoder for PLCP Header

The PLCP header shall use a systematic BCH (31, 19, t = 2) code is a shortened code derived from a BCH (63, 51, t = 2) code, to improve the robustness of the PLCP header. A description of the BCH (63, 51, t = 2) code can be found in Section 2.4.2.2. The shortened bits are removed prior to transmission.

## PSDU

The PSDU is the last major component of the PPDU and shall be constructed as shown in Figure 5.

- Form the non-scrambled PSDU by pre-pending the 7-byte MAC header to the MAC frame body and appending a 2-byte FCS to the result.

- The resulting PSDU is scrambled according to Section 2.4.1.

- If the code rate (k/n) < 1, the scrambled output is:

  - Divided into blocks of messages, where each message may contain shortened bits, according to Section 2.4.2.1;

  - The resulting messages are then encoded into codewords using a BCH (63, 51) encoder to achieve the desired code rate, according to Section 2.4.2;

  - Finally, the shortened bits are removed from each of the codewords.

- Pad bits are then added in order to ensure alignment on a symbol boundary according to Section 2.4.3.

- If the spreading factor is 2 or 4, the resulting un-coded or coded bits are spread using a repetition code, according to Section 2.4.4,  and then interleaved using a bit interleaver defined in Section 2.4.5.

- Finally, the resulting un-coded or coded and potentially spread and interleaved bit steam is then mapped onto the appropriate constellation (see Section 2.4.6), which is determined by the data rate and frequency band of operation (see Section 2.1).

Figure 5 — Block diagram of PSDU construction

### Data Scrambler

A side-stream scrambler with polynomialG(x)=1+x2+x12+x13+x14 shall be used to whiten the PSDU. Figure 6 shows a typical implementation of the side-stream scrambler. The output of the scrambler is generated as:

x[n]=x[n−2]⊕x[n−12]⊕x[n−13]⊕x[n−14]

where “” denotes modulo-2 addition. Table 12 defines the initialization vector, xinit, for the side-stream scrambler as a function of the scrambler seed (SS) value.

Figure 6 — Block diagram of a side-stream scrambler

Table 12 — Scrambler Seed Selection

| Scrambler Seed (SS) | Initialization Vector xinit = x[-1] x[-2] … x[-14] |
| --- | --- |
| 0 | 0 0 1 0 1 1 1 1 0 0 1 1 0 1 |
| 1 | 0 0 0 0 0 0 0 1 0 0 1 1 1 1 |

The MAC shall set the scrambler seed to 0 when the PHY is initialized and the scrambler seed shall be incremented, using a 1-bit rollover counter, for each frame sent by the PHY.

At the receiver, the side-stream de-scrambler shall be initialized with the same initialization vector, xinit, used by the transmitter. The initialization vector is determined from the scrambler seed value in the PHY header of the received frame.

### BCH Encoder for PSDU

A code rate of 51/63 shall be supported by a systematic BCH encoder. The information bits shall be encoded using BCH encoding process defined in Section 2.4.2.1. The definition for the systematic BCH encoder shall be defined in Section 2.4.2.2.

#### BCH Encoding Process

The scrambled PSDU shall be encoded using the following procedure:

- Compute the number of bits in the PSDU NPSDU:

NPSDU=NMACheader+NMACFrameBody+NFCS×8

where  NMACheader is the number of bytes in the MAC header, NMACFrameBody is the number of bytes in the MAC frame body and NFCS is the number of bytes in the FCS.

- Calculate the number of BCH codeword NCW:

NCW=NPSDUk

where  k is the number of message bits for the selected BCH code.

- Compute the number of shortening bits, Nshorten, to be padded to the NPSDU data bits before encoding as follows:

Nshorten=NCW×k−NPSDU

- The shortening bits shall be equally distributed over all NCW codewords with the first remNshorten,NCW codewords being shortened one bit more than the remaining codewords. Let

Nspcw=NshortenNCW.

Thus, the first remNshorten,NCW codewords will have Nspcw+1 shortened bits (message bits that are set to 0), while the remaining codewords will have Nspcw shortened bits. After encoding, the shortened bits shall be discard prior to transmission, i.e., the shortened bits are never transmitted on-air.

The BCH encoding process is illustrated in Figure 7 for a single codeword.

Figure 7 — BCH encoding process for a single codeword

#### BCH (63, 51) Encoder

The generator polynomial for a systematic BCH (63, 51, t = 2) code is given by:

g(x)=1+x3+x4+x5+x8+x10+x12.

The parity bits are determined by computing the remainder polynomial r(x):

r(x)=i=011rixi=x12m(x)modg(x),

where m(x) is the message polynomial:

m(x)=i=050mixi,

and ri,i=0,…,11 and mi,i=0,…,50 are elements of . The message polynomial m(x) is created as follows: m50 is the first bit of the message and m0 is the last bit of the message, which maybe a shortened bit. The order of the parity bits is as follows: r11 is the first parity bit transmitted, r10 is the second parity bit transmitted, and r0 is the last parity bit transmitted.

### Pad Bits

Pad bits shall be appended after the BCH encoder to ensure that the bit stream aligns on a symbol boundary. The number of pad bits, Npad, that shall be inserted is a function of the number of PSDU bits NPSDU, the number of codewords NCW, the number of parity bits (n−k), and the modulation constellation size M:

Npad=log2(M)×NPSDU+NCW×(n−k)log2(M)−NPSDU+NCW×(n−k).

The pad bits shall be appended to scrambled and encoded PSDU and all of the appended pad bits shall be set to 0.  In the case of un-coded transmission, NCW is set to zero.

### Spreading

For a spreading factor of 2, each input bit is repeated two times (see illustration in Figure 8a). For a spreading factor of 4, each input bit is repeated four times (see illustration in Figure 8b).

Figure 8 — Spreading scheme

### Bit Interleaver

The output of the spreader shall be interleaved prior to modulation to provide robustness against error propagation. The exact structure of the bit interleaver depends on the number of un-coded or coded bits that will be transmitted on-air, which is given by:

Ntotal=NPSDU+NCW×(n−k)+Npad,

where NCW is set to zero in the case of un-coded transmission.

If rem(Ntotal,2)=0, the bit interleaving operation is performed by first grouping the spread bits into blocks of 2S bits, where S is the spreading factor, and then using a block interleaver of size S2 to permute the bits. Let the sequences a(i) and b(i), where i=0,1,…,2S−1, represent the input and output bits of the S3 bit interleaver, respectively. The output of the S2 bit interleaver is given by the following relationship:

b(i)=aS×rem(i,2)+i2i=0,1,…,2S−1.

If rem(Ntotal,2)=1, the bit interleaving operation is performed by grouping the first 3S spread bits into a single block and then using a block interleaver of size S3 to permute the bits within that single block. Let the sequences  and , where i=0,1,…,3S−1, represent the input and output bits of the S3 bit interleaver, respectively. The output of the S3 bit interleaver is given by the following relationship:

b(i)=aS×rem(i,3)+i3i=0,1,…,3S−1.

The remaining spread bits are then grouped into blocks of 2S bits and interleaved using the block interleaver of size S2 shown in (19).

### Constellation Mapping

#### Gaussian Minimum Shift Keying (GMSK)

For the GMSK constellation, the un-coded or coded, potentially spread and interleaved binary bit stream b(n), n = 0,1,…, N-1 shall be mapped onto a corresponding frequency deviation f, which shall be the product of the symbol rate and a modulation index of 0.5. The relationship between the bit stream b(n) and the frequency deviation is given in Table 13.

Table 13 — GMSK symbol mapping

| b(n) | frequency deviation |
| --- | --- |
| 0 | f |
| 1 | +f |

#### Differential Phase-Shift Keying (D-PSK)

For the D-PSK constellations, the coded potentially spread and interleaved bit stream shall be mapped onto one of three rotated and differentially-encoded constellations: /2-DBPSK, /4-DQPSK, or /8-D8PSK. The encoded information is carried in the phase transitions between symbols. For the PLCP preamble to PLCP header transition, the phase change is relative to the last symbol for the PLCP preamble. For the PLCP header to PSDU transition, the phase change is relative to the last symbol for the PLCP header.

The binary bit stream b(n),n=0,1,…,N−1 shall be mapped onto a corresponding complex-values sequence S(k),k=0,1,…,N/log2(M)−1 as follows:

S(k)=S(k−1)exp(jϕk)k=1,2,…,N/log2(M)−1.

whereS(0)=exp(jπ/M)and the relationship between the bit stream b(n) and the phase change ϕk is given in  through Table 16 for /2-DBPSK, /4-DQPSK, or /8-D8PSK, respectively.

Table 14 — /2-DBPSK Mapping

| b(n) | k |
| --- | --- |
| 0 | /2 |
| 1 | 3/2 |

Table 15 — /4-DQPSK Mapping

| b(2n) | b(2n+1) | k |
| --- | --- | --- |
| 0 | 0 | /4 |
| 0 | 1 | 3/4 |
| 1 | 0 | 7/4 |
| 1 | 1 | 5/4 |

Table 16 — /8-D8PSK Mapping

| b(3n) | b(3n+1) | b(3n+2) | k |
| --- | --- | --- | --- |
| 0 | 0 | 0 | /8 |
| 0 | 0 | 1 | 3/8 |
| 0 | 1 | 0 | 7/8 |
| 0 | 1 | 1 | 5/8 |
| 1 | 0 | 0 | 15/8 |
| 1 | 0 | 1 | 13/8 |
| 1 | 1 | 0 | 9/8 |
| 1 | 1 | 1 | 11/8 |

## General Requirements

### Operating Frequency Bands

A compliant device shall be able to support transmissions and reception in one of the following frequency bands:

- 402 – 405 MHz

- 420 – 450 MHz

- 863 – 870 MHz

- 902 – 928 MHz

- 950 – 956 MHz

- 2360 – 2400 MHz

- 2400 – 2483.5 MHz

### Channel Numbering

The relationship between center frequency, fc, and channel number, nc, is shown in Table 17.

Table 17 — Relationship between Center Frequency and Channel Number

| Frequency Band (MHz) | Relationship between fc and nc |
| --- | --- |
| 402 – 405 | fc = 402.15 + 0.30  nc (MHz), nc = 0, …, 9 |
| 420 – 450 | fc = 420.30 + 0.50 g1(nc) (MHz), nc = 0, …, 13 |
| 863 – 870 | fc = 863.20 + 0.40 g2(nc) (MHz), nc = 0, …, 11 |
| 902 – 928 | fc = 903.50 + 0.50  nc (MHz), nc = 0, …, 47 |
| 950 – 956 | fc = 951.10 + 0.40  nc (MHz), nc = 0, …, 11 |
| 2360 – 2400 | fc = 2362.00 + 1.00  nc (MHz), nc = 0, …, 37 |
| 2400 – 2483.5 | fc = 2402.00 + 1.00  nc (MHz), nc = 0, …, 78 |

The mapping functions g1(nc)and g2(nc)used in the 420 – 450 MHz and 863 – 870 MHz frequency bands, respectively are defined as follows:

g1(nc)=nc0≤nc≤1nc+6.8752≤nc≤4nc+13.4nc=5nc+35.0256≤nc≤7nc+40.9258≤nc≤9nc+47.2510≤nc≤11, and

g2(nc)=nc0≤nc≤7nc+0.5nc=8nc+19≤nc≤12nc+1.5nc=13.

### Preamble Sequence Assignment

The relationship between channel number, nc, and preamble sequence used in the PLCP preamble is shown in Table 18.

Table 18 — Relationship between Channel Number and Preamble

| Preamble Sequence | Relationship between preamble sequence and nc |
| --- | --- |
| 1 | rem(nc,2) = 0 |
| 2 | rem(nc,2) = 1 |

## PHY Layer Timing

The values for the PHY layer timing parameters are defined in Table 19.

Table 19 — Physical layer timing parameters

| PHY Parameter | Value |
| --- | --- |
| pSIFS | 50 s |
| pMIFS | 20 s |
| pEDTime | 8 preamble symbols |
| pCCATime | 63 preamble symbols |
| pChannelSwitchTime | 100 s |

The values for pEDTime and pCCATime shall be those specified in Table 19 or the values specified by the local regulatory requirements, which is lower.

### Inter-frame Spacing

The inter-frame spacing parameters are given in Table 20.

Table 20 — Inter-frame spacing parameters

| MAC Parameter | Value |
| --- | --- |
| SIFS | pSIFS |
| MIFS | pMIFS |

### Receive-to-Transmit Turnaround Time

The RX-to-TX turnaround time shall not be greater than pSIFS. The turnaround is defined as time elapsed from when the last sample of the last received symbol is present on the air interface, to the time when first sample of the first transmitted symbol of the PLCP preamble for the next frame is present on the air interface.

### Transmit-to-Receive Turnaround Time

The TX-to-RX turnaround time shall not be greater than pSIFS. The turnaround is defined as the time elapsed from when the last sample of the last transmitted symbol is present on the air interface until the time when the receiver is ready to begin the reception of first sample for the next PHY frame.

### Time between Successive Transmissions

For burst mode transmissions, the inter-frame spacing between uninterrupted successive transmissions by a device shall be fixed to exactly pMIFS. The inter-frame spacing is defined as the time elapsed from when the last sample of the last transmitted symbol is present on the air interface, to the time when the first sample of the first transmitted symbol of the PLCP preamble for the following packet is present on the air interface.

### Center Frequency Switch Time

The center frequency switch time is defined as the interval from when the PHY transmits or receives the last valid symbol on one center frequency until it is ready to transmit or receive the next symbol on a different center frequency. The center frequency switch time shall not exceed pChannelSwitchTime.

## Transmitter Specifications

### Transmit PSD Mask

The transmitted spectral mask shall be less than –X dBr (dB relative to the maximum spectral density of the signal, see Table 21) for f−fc≥fBW/2, where fc is channel center frequency and fBW is the channel bandwidth and is a function of the frequency band of operation and is defined in Table 21.

Table 21 — Channel bandwidth as a function of the frequency band of operation

| Frequency (MHz) | –X dBr | fBW |
| --- | --- | --- |
| 402 – 405 | –20 | 300 kHz |
| 420 – 450 | –40 | 320 kHz |
| 863 – 870 | –20 | 400 kHz |
| 902 – 928 | –20 | 500 kHz |
| 950 – 956 | –20 | 400 kHz |
| 2360 – 2400 | –20 | 1 MHz |
| 2400 – 2483.5 | –20 | 1 MHz |

The transmitted spectral density also shall comply with all regulations defined by local regulatory bodies.

### Transmit Power

A transmitter shall be capable of transmitting at least –10 dBm EIRP in all frequency bands, except for 402 – 405 MHz, where a transmitter shall be capable of transmitting at most –16 dBm EIRP. Devices should transmit lower power when possible in order to reduce interference to other devices and systems.

The maximum transmit power is limited by local regulatory bodies.

### Transmit power-on and power-down ramp

The transmit power-on ramp for 10% to 90% of maximum power shall be no greater than 5 symbols. The transmit power-on ramp is shown in Figure 9. The transmit power-down ramp for 90% to 10% maximum power shall be no greater than 5 symbols. The transmit power-down ramp is shown in Figure 10.

Figure 9 — Transmit power-on ramp

Figure 10 — Transmit power-down ramp

The transmit power ramps shall be constructed such that the emissions conform to the local spurious frequency regulations.

### Transmit Center Frequency Tolerance

The transmitted center frequency tolerance shall be 20 ppm maximum.

### Symbol Clock Frequency Tolerance

The symbol clock frequency tolerance shall be 20 ppm maximum.

### Clock Synchronization

The transmit center frequencies and the symbol clock frequency shall be derived from the same reference oscillator.

### Transmitter Constellation Error

The modulation accuracy of the transmitter is determined via an error-vector magnitude (EVM) measurement, which is calculated over N baud-spaced received complex values Ik,Qk. A decision is made for each received complex value. The ideal position of the chosen symbol is represented by the vector Ik,Qk. The error vector δIk,δQk is defined as the distance from the ideal position to the actual position of the received complex values, i.e., (Ik,Qk)=(Ik,Qk)+(δIk,δQk).

The EVM is defined as shown in the equation below:

EVM=1Nk=1NδIk2+δQk2S2×100%,

where S is the magnitude of the vector to the ideal constellation point. A transmitter shall have EVM values less than or equal to those listed in Table 22 when measure for N = TBD symbols. The EVM shall be measured on baseband I and Q samples after the received signal is passed through a reference receiver, which shall perform the following operations: matched SRRC filtering, carrier-frequency offset estimation and symbol timing recovery while making the measurements.

Table 22 — Permissible EVM numbers as a function of constellation size

| Constellation | EVM Error |
| --- | --- |
| /2-DBPSK | 20.0 % |
|  /4-DQPSK | 12.5 % |
|  /8-D8PSK | 7.0 % |

### Adjacent Channel Power Ratio

## Receiver Specifications

### Receiver Sensitivity

For a packet error rate (PER) of less than 10% with a PSDU of 255 bytes, the minimum receiver sensitivity numbers in AWGN for the highest data rate in each operating frequency band shall be the numbers listed in Table 23. The minimum input levels are measured at the antenna connector, where a noise figure of 10 dB (referenced at the antenna), an implementation loss of 6 dB, and antenna gain of 0 dBi for both the transmitter and receiver have been assumed.

Table 23 — Receiver Sensitivity Numbers

| Frequency Band (MHz) | Information Data Rate (kbps) | Minimum Sensitivity (dBm) |
| --- | --- | --- |
| 402 – 405 | 75.9 | –98 |
|  | 151.8 | –95 |
|  | 303.6 | –92 |
|  | 455.4 | –86 |
| 420 – 450 | 75.9 | –93 |
|  | 151.8 | –90 |
|  | 187.5 | –87 |
| 863 – 870 950 – 956 | 101.2 | –97 |
|  | 202.4 | –94 |
|  | 404.8 | –90 |
|  | 607.1 | –85 |
| 902 – 928 | 121.4 | –96 |
|  | 242.9 | –93 |
|  | 485.7 | –89 |
|  | 728.6 | –84 |
| 2360 – 2400 2400 – 2483.5 | 121.4 | –95 |
|  | 242.9 | –93 |
|  | 485.7 | –90 |
|  | 971.4 | –86 |

### Adjacent Channel Rejection

### Receiver Energy Detection

The receiver energy detection (ED) measurement is an estimate of the received signal power within the bandwidth of the channel. It is intended for use by a network/MAC layer as part of a channel selection algorithm. No attempt is made to identify or decode signals on the channel.

### ED Threshold

The minimum ED value (zero) shall indicate received power less than either

- 10 dB above the specified receiver sensitivity (see Section 2.8.1) OR

- that which is prescribed by local regulatory requirements,

which ever is lower.

The range of received power spanned by the ED values shall be at least 40 dB. Within this range, the mapping from the received power in decibels to ED value shall be linear with an accuracy of ± 6 dB.

### ED Measurement Time

The ED measurement time, to average over, shall be either

- pEDTime  (8 preamble symbol periods) OR

- that which is prescribed by local regulatory requirements,

which ever is longer in duration.

### Receiver Clear Channel Assessment

The PHY shall provide the capability to perform CCA according to at least one of the following three methods:

CCA Mode 1: Energy above threshold. CCA shall report a busy medium upon detecting any energy above the ED threshold.

CCA Mode 2: Carrier sense only. CCA shall report a busy medium only upon the detection of a signal compliant with this standard with the same modulation and characteristics of the PHY that is currently in use by the device. This signal may be above or below the ED threshold. The CCA detection time shall be equal to pCCATime.

CCA Mode 3: Carrier sense with energy above threshold. CCA shall report a busy medium using a logical combination of

  - Detection of a signal with the modulation and characteristics of this standard and

  - Energy above the ED threshold, where the logical operator may be AND or OR

The CCA parameters are subject to the following criteria:

a) The ED threshold shall correspond to a received signal power as prescribed in section 2.8.4

b) The CCA detection time shall be equal to pCCATime (see Section 2.6)

Any CCA procedures required by local regulatory requirements shall also be supported.