IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | MAC and Security Baseline Proposal – Normative Text |
| Date Submitted | March 16, 2010 |
| Source | Saied Abedi, Gahng S. Ahn, M. A. Ameen, Thenmozhi Arunan, Anuj Batra, Ashutosh Bhatia, Kiran Bynam, Hind Munzer-Chebbo, Chihong Cho, David Davenport, Guido Dolmans, Giriraj Goyal, Jin-Meng Ho, Srinath Hosur, Ichirou Ida, Jeongsik In, Noh-Gyoung Kang, Farooq Khan, Eui-Jik Kim, Ryuji Kohno, Kyung-Sup Kwak, Youngmi Kwon, Cheolhyo Lee, HyungSoo Lee, Myung J Lee, Huan-Bang Li, Arun Naniyat, Dries Neirynck, Okundu Omeni, Maulin Patel, Seung-Hoon Park, Yongsuk Park, Ranjeet K. Patro, Rick Powell, Sridhar Rajagopal, JuneChul Roh, Olivier Rousseaux, Jerome Rousselot, Didier Sagan, Tim Schmidl, Neal Seidl, Georgios Selimis, Chang-sub Shin, Feng Shu, Hui-Hsia Sung, Sana Ullah, Sunil Vadgama, Dong Wang, Euntae Won, Alan Wong, Kaoru Yokoo, J.S Yoon, Yan Zhang, Bin Zhen |
| Re: | Response to IEEE 802.15.6 call for proposals |
| Abstract | This submission provides the normative text for a joint MAC and security proposal based on the corresponding individual proposals submitted earlier by the individuals listed in the Source row. |
| Purpose | To prepare a baseline draft on MAC and security for the IEEE 802.15.6 task group |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Table of Contents

1	Scope	1

2	Normative references	2

3	Definitions	3

4	Acronyms and abbreviations	7

5	General framework elements	9

5.1	Network topology	9

5.2	Reference model	9

5.3	Time base	10

5.4	State diagram	11

5.4.1	Secured communication	11

5.4.2	Unsecured communication	12

5.5	Security paradigm	12

6	MAC frame formats	14

6.1	Conventions	14

6.2	General format	14

6.2.1	MAC header	15

6.2.2	MAC frame body	21

6.2.3	FCS	22

6.3	Management type frames	23

6.3.1	Beacon	23

6.3.2	Security Association	26

6.3.3	Security Disassociation	30

6.3.4	Pairwise Temporal Key (PTK)	31

6.3.5	Group Temporal Key (GTK)	32

6.3.6	Connection Request	33

6.3.7	Connection Assignment	35

6.3.8	Multinode Connection Assignment	38

6.3.9	Disconnection	39

6.3.10	Command	40

6.4	Control type frames	41

6.4.1	Immediate Acknowledgement (I-Ack)	42

6.4.2	Block Acknowledgement (B-ACK)	42

6.4.3	Immediate Acknowledgement + Poll (I-Ack+Poll)	42

6.4.4	Block Acknowledgement +Poll (B-ACK+Poll)	43

6.4.5	Poll	43

6.4.6	Timed-Poll (T-Poll)	43

6.4.7	Wakeup	43

6.4.8	B2	44

6.5	Data type frames	44

6.6	MAC / PHY Capability fields	44

6.6.1	MAC Capability	44

6.6.2	PHY Capability	46

6.7	Information elements (Ies)	47

6.7.1	Slotted Superframe IE	49

6.7.2	Explicit Uplink Request IE	49

6.7.3	Implicit Uplink Request IE	50

6.7.4	Explicit Downlink Request IE	52

6.7.5	Implicit Downlink Request IE	52

6.7.6	Explicit Bilink Request IE	52

6.7.7	Implicit Bilink Request IE	52

6.7.8	Explicit Delayed Bilink Request IE	52

6.7.9	Implicit Delayed Bilink Request IE	53

6.7.10	Group Connection IE	54

6.7.11	Uplink Assignment IE	55

6.7.12	Downlink Assignment IE	56

6.7.13	Bilink Assignment IE	56

6.7.14	Delayed Uplink Assignment IE	56

6.7.15	Node Connection Assignment IE	57

6.7.16	Nibble Encoded Channel Order IE	58

6.7.17	Octet Encoded Channel Order IE	58

6.7.18	Channel Hopping IE	58

6.7.19	Application Specific IE	59

7	MAC functions	60

7.1	Frame processing	60

7.1.1	Abbreviated addressing	60

7.1.2	Full addressing	61

7.1.3	Priority mapping	61

7.1.4	Frame reception	62

7.1.5	Frame transfer	62

7.1.6	Frame retry	63

7.1.7	Frame acknowledgement	63

7.1.8	Duplicate detection	67

7.1.9	Fragmentation and reassembly	67

7.2	Access classification and division	67

7.2.1	Beacon mode with beacon period boundaries	68

7.2.2	Non-beacon mode with superframe boundaries	69

7.2.3	Non-beacon mode without superframe boundaries	69

7.3	BAN creation and connection setup	70

7.4	Random access (connectionless contention-based access)	71

7.4.1	CSMA/CA	72

7.4.2	Slotted Aloha access	74

7.5	Improvised and unscheduled access (connectionless contention-free access)	76

7.5.1	Unscheduled access	76

7.5.2	Improvised access	76

7.6	Scheduled access and variants (connection-oriented contention-free access)	80

7.6.1	Starting scheduled allocations	80

7.6.2	Using scheduled allocations	81

7.6.3	Modifying scheduled allocations	83

7.6.4	Aborting scheduled allocations	83

7.6.5	Ending scheduled allocations	83

7.7	MICS band communication	83

7.7.1	Unconnected mutual discovery	83

7.7.2	Connected mutual discovery	84

7.7.3	Medical implant event report	88

7.8	Two-hop star topology extension	89

7.8.1	Relay discovery procedure for connected node	89

7.8.2	Relay discovery procedure for unconnected node	90

7.8.3	RTT Ending	91

7.9	Clock synchronization and guardtime provisioning	93

7.10	Power management	95

7.10.1	Hibernation—macroscopic power management	96

7.10.2	Sleep—microscopic power management	96

7.11	Coexistence and interference mitigation	97

7.11.1	Beacon shifting	97

7.11.2	Channel hopping	98

7.11.3	Time sharing and load reduction	100

7.12	MAC / PHY capability handling and Application Specific IE usage	102

7.13	MAC sublayer parameters	102

8	Security services	105

8.1	Master keys	105

8.1.1	Master key (MK) pre-shared association	106

8.1.2	Unauthenticated association	106

8.1.3	Public key hidden association	108

8.1.4	Password authenticated association	109

8.1.5	Display authenticated association	112

8.1.6	Disassociation	114

8.2	Temporal keys	115

8.2.1	PTK creation	115

8.2.2	GTK distribution	117

8.3	Message security	117

8.3.1	Frame authentication, encryption, and decryption	118

8.3.2	Replay protection	122

# Scope

This standard specifies a medium access control (MAC) sublayer in support of a physical layer for wireless body area networks (BANs).  It also specifies message security services provided at the MAC sublayer and security key generations performed inside or/and outside the MAC sublayer.

# Normative references

The following standards contain provisions which, through references in this text, constitute provisions of this standard. At the time of publication, the editions indicated were valid. All standards are subject to revision, and parties to agreements based on this standard are encouraged to investigate the possibility of applying the most recent editions of the standards listed below.

IEEE Std 802-2001, IEEE Standard for Local and Metropolitan Area Networks: Overview and Architecture.

IEEE Std 1363-2000, IEEE Standard Specifications for Public-Key Cryptography.

FIPS Pub 197, Advanced Encryption Standard (AES), November 2001.

FIPS Pub 186-3, Digital Signature Standard (DSS), June 2009.

NIST Special Publication 800-38B, Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication, May 2005.

NIST Special Publication 800-90, Recommendation for Random Number Generation Using Deterministic Random Bit Generators, March 2007.

ISO/IEC Standard　18033-3, Information technology – Security techniques – Encryption algorithms – Part 3: Block ciphers, 2005.

ISO/IEC Standard 10116, Information technology – Security techniques – Modes of operation for an n-bit block cipher, 2006.

ISO/IEC Standard 19772, Information technology – Security techniques – Authenticated encryption, 2009.

ISO/IEC 10646, Universal Multiple-Octet Coded Character Set (UCS), December 2003.  Amendment 1, November 2005.  Amendment 2, July 2006.  Amendment 3, February 2008.

IETF RFC 4086, Randomness Requirements for Security, June 2005.

# Definitions

For the purposes of this standard, the following terms and definitions apply. The Authoritative Dictionary of IEEE Standards Terms, Seventh Edition, should be referenced for terms not defined in this clause.

abbreviated address: A one-octet identifier (ID) selected as an address for a node, hub, or body area network (BAN) in frame exchanges.

active state: An internal power management state that is ready for frame reception and transmission.

active superframe: A superframe in which frame transmission typically occurs within the body area network (BAN) of the hub announcing such a superframe.

AES-128 CCM: A message security protocol that employs the Advanced Encryption Standard (AES) forward cipher function for 128-bit keys for message encryption/decryption based on counter mode and message authentication based on cipher block chaining (CBC) mode.

allocation: One or more time intervals that a node or a hub obtains using an access method for initiating one or more frame transactions.  An allocation comprises one or more allocation intervals.  Reference to allocation of a node means that the node is the sender or recipient in the allocation.

allocation interval: A continuous time interval in an allocation, comprising one or more consecutive allocation slots.  Reference to allocation interval of a node means that the node is the sender or recipient in the allocation interval.

allocation slot: A time unit used to designate the lengths of medium access related time intervals, such as beacon period and allocation interval.

association: security association.

awake state: A term synonymous to active state in this standard.

beacon: A frame transmitted by a hub to facilitate network management, such as the coordination of medium access and power management of the nodes in the body area network (BAN) of the hub, and to facilitate clock synchronization therein.

beacon period: A repetitive time interval to which medium access is referenced and in which a beacon is transmitted when appropriate, comprising the same number of time units (allocation slots) of equal duration.

bilink: A communications link for transfer of management and data traffic from a hub to a node or/and vice versa.

bilink allocation: An allocation with allocation interval(s) in which a hub or a node initiates one or more frame transactions to transmit management and data traffic to a node or a hub, respectively, and the recipient returns acknowledgment if required, with the provision that the node initiates frame transaction(s) only after receiving a poll from the hub.

bilink allocation / delay: A bilink allocation in which the scheduled start of a bilink allocation interval is subject to delay.

body area network (BAN): A logical network partition comprising a hub and zero or more nodes whose medium access and power management are coordinated by the hub.

contended allocation: A non-reoccurring time interval, within a random access phase (RAP) or a contention access phase (CAP), that a node obtains using random access for initiating a frame transaction.  A contended allocation is an uplink allocation, suitable for “unpredictable” uplink traffic (for example, due to data rate variations and/or channel impairments).

contention access: An access method, based on carrier sense multiple access with collision avoidance (CSMA/CA) or slotted Aloha access but not both, by which a node obtains a time interval in a contention access phase (CAP) for initiating one or more frame transactions.  As an access method, contention access is synonymous with random access.

contention access phase (CAP): A time span set aside by a hub and announced via a preceding non-beacon frame for contention access to the medium by the nodes in the body area network (BAN) of the hub.

connected node: A node that has a connection with a hub.

connection: A relationship between a node and a hub in a body area network (BAN), substantiated by a Connected_NID assigned to the node by the hub and by a wakeup arrangement between them, and optionally by one or more scheduled allocations between them.

delayed-polling access: A variant of scheduled-polling access, by which the scheduled start of frame transactions is subject to delay.

downlink: A communications link for transfer of management and data traffic from a hub to a node.

downlink allocation: An allocation with allocation interval(s) in which a hub initiates one or more frame transactions to transmit management and data traffic to a node and the node returns acknowledgment if required.

entity authentication: Corroboration of the identity of the node or the hub in a security association procedure.

exclusive access phase (EAP): A time span set aside by a hub in a beacon period for transfer of the traffic of the highest user priority (for emergency or medical event report).

frame: An uninterrupted sequence of octets delivered by the medium access control (MAC) sublayer to the physical (PHY) layer, or vice versa, within a node or a hub.

frame transaction: Exchange of one or more functionally related frames between a node and a hub, without interruption by other frames to facilitate frame transmission and reception as well as frame acknowledgment if required.  An example frame transaction consists of a frame transmission followed by an acknowledgment return.

hibernation state: A term synonymous to inactive state in this standard.

hub: An entity that possesses a node’s functionality and coordinates the medium access and power management of the nodes in its body area network (BAN).

hub identifier (HID): An abbreviated address of a hub.

improvised access: An access method, based on impromptu polling or posting by a hub, by which a hub grants to a node or itself a polled or posted allocation, typically outside scheduled downlink and uplink allocations, for initiating one or more frame transactions by the node or hub, respectively.  Improvised access is based on dynamic scheduling as opposed to fixed scheduling with scheduled access.

inactive state: An internal power management state that is not ready for frame reception and transmission.

inactive superframe: A superframe in which no frame transmission occurs within the body area network (BAN) of the hub announcing such a superframe.

master key (MK): A secret bit string activated or established between a node and a hub in a security association procedure and used to create a pairwise temporal key (PTK) shared between them.

medical implant event: A term referenced in regulatory documents governing the use of the medical device radiocommunication (MedRadio) service band, which includes the medical implant communications service (MICS) band.

multi-periodic (m-periodic) allocation: A scheduled allocation that has allocation intervals reoccurring in every mth beacon period (superframe) with m being an integer larger than one.  An m-periodic allocation is an uplink allocation, a downlink allocation, or a bilink allocation, suitable for low duty cycle periodic or quasi-periodic traffic.

message authentication: corroboration of the origin of a message in a message transfer.

node: An entity that contains a medium access control (MAC) sublayer, a physical (PHY) layer, and a node management entity (NME) and that provides message security services in accordance with this standard.  Unless otherwise noted, reference to nodes refers to nodes that are not acting as hubs.

node identifier (NID): An abbreviated address of a single node or of a logical group of nodes.

non-secure frame: A term that is interchangeable with unsecured frame.

one-periodic (1-periodic) allocation: A scheduled allocation that has allocation intervals reoccurring in every beacon period (superframe).  A 1-periodic allocation is an uplink allocation, a downlink allocation, or a bilink allocation, suitable for high duty cycle periodic or quasi-periodic traffic.

pairwise temporal key (PTK): A secret bit string shared between a node and a hub and used to secure frames transferred between them.

pairwise temporal key (PTK) creation: A procedure used to create a PTK between a node and a hub based on a master key shared between them, and to confirm possession of a shared MK between the node and the hub.

poll: A control frame or its variant sent by the hub to grant an immediate polled allocation to the addressed node or to inform the node of an intended future poll or post.

polled allocation: A non-reoccurring time interval, typically outside scheduled downlink and uplink allocations, that a hub grants to a node using polling access for initiating one or more frame transactions by the node.  A polled allocation is an uplink allocation, suitable for “ordinary”, “unexpected”, or “extra” uplink traffic (for example, due to data rate variations and/or channel impairments).

polling access: An access method, based on impromptu or scheduled polling by a hub, by which a hub grants to a node a polled allocation, typically outside scheduled downlink and uplink allocations, for initiating one or more frame transactions by the node.

post: A contention-free frame transaction initiated by a hub with a node within its body area network (BAN).  A post starts a posted allocation.

posted allocation: A non-reoccurring time interval that a hub grants to itself using posting access for initiating a frame transaction.  A posted allocation is a downlink allocation, suitable for “unexpected” or “extra” downlink traffic (for example, due to network management needs, data rate variations, and/or channel impairments).

posting access: An access method, based on impromptu or scheduled posting by a hub, by which a hub grants to itself a posted allocation, typically outside scheduled uplink allocations, for initiating one or more frame transactions by the hub.

random access: An access method, based on carrier sense multiple access with collision avoidance (CSMA/CA) or slotted Aloha access but not both, by which a node obtains a time interval in a random access phase (RAP) for initiating one or more frame transactions.

random access phase (RAP): A time span set aside by a hub and announced via a beacon frame for random access to the medium by the nodes in the body area network (BAN) of the hub.

scheduled access: An access method, based on advance reservation and scheduling, by which a node and a hub obtain one or more scheduled reoccurring time intervals for initiating frame transactions.

scheduled allocation: One or more scheduled reoccurring time intervals that a node or a hub obtains using scheduled access for initiating frame transactions.  A scheduled allocation is an uplink allocation, a downlink allocation, or a bilink allocation, suitable for high or low duty cycle periodic or quasi-periodic traffic.

scheduled-polling access: A combination of scheduled access and polling access, by which a node and a hub obtain one or more scheduled reoccurring time intervals and impromptu non-reoccurring time intervals within the scheduled time intervals, for initiating frame transactions on uplink and/or downlink.

secured communication: Exchange of secured frames.

secure frame: A term that is interchangeable with secured frame.

secured frame: A frame that is secured with authenticity, integrity, confidentiality if required, and replay protection.

security association: A procedure used to identify a node and a hub to each other and establish a new master key shared between them or activate an existing master key pre-shared between them.

sleep state: A term synonymous to inactive state in this standard.

superframe: A term that is interchangeable with beacon period, used especially when no beacons are transmitted.

type-I polled allocation: A polled allocation the length of which is specified in terms of the duration of time granted for transmission.

type-II polled allocation: A polled allocation the length of which is specified in terms of the number of frames granted for transmission.

uplink: A communications link for transfer of management and data traffic from a node to a hub.

uplink allocation: An allocation with allocation interval(s) in which a node initiates one or more frame transactions to transmit management and data traffic to a hub and the hub returns acknowledgment if required.

unconnected node: A node that does not have a connection with a hub in a body area network (BAN).

unsecured communication: Exchange of unsecured frames.

unsecured frame: A frame that is not secured with authenticity, integrity, confidentiality, or replay protection.

# Acronyms and abbreviations

AES	Advanced Encryption Standard

B-Ack	block acknowledgment

BAN	body area network

CAP	contention access phase

CBC	cipher block chaining

CCA	clear channel assessment

CCM	counter mode for message encryption and cipher block chaining (CBC) mode for message authentication

CMAC	(block) cipher-based message authentication code algorithm

CP	contention probability (for Aloha access)

CRC	cyclic redundancy check

CSMA/CA	carrier sense multiple access with collision avoidance

CW	contention window (for CSMA/CA)

EAP	exclusive access phase

G-Ack	group acknowledgment

FCS	frame check sequence

GT	guardtime

GTK	group temporal key

HID	hub identifier

HME	hub management entity

I-Ack	immediate acknowledgment

IE	information element

KCK	key confirmation key

KMAC	key message authentication code

L-Ack	late acknowledgment (acknowledgment later)

LFSR	linear feedback shift register

MAC	medium access control

MIC	message integrity code

MICS	medical implant communications service

MK	master key

MSDU	MAC service data unit

N-Ack	no acknowledgment

NID	node identifier

NME	node management entity

PHY	physical layer

PN	pseudo-random

PTK	pairwise temporal key

RAP	random access phase

RX	receive or reception

SAP	service access point

SIFS	short inter-frame space

TK	temporal key

TX	transmit or transmission

UP	user priority

UWB	ultra wide band

# General framework elements

This clause provides the basic framework required of all nodes and hubs.  The framework serves as a prerequisite to supporting the functions of nodes and hubs and their interactions specified later in detail in this standard.  It covers the following fundamental aspects— the network topology used for medium access, the reference model used for functional partitioning, the time base used for access scheduling, the state diagram used for frame exchange, and the security paradigm used for message protection.

## Network topology

All nodes and hubs shall be organized into logical sets, referred to as body area networks (BANs) in this specification, and coordinated by their respective hubs for medium access and power management as illustrated in Figure 1.  There shall be one and only one hub in a BAN, whereas the number of nodes in a BAN may range from zero to mMaxBANSizes.  In a one-hop star BAN, frame exchanges may occur directly only between nodes and the hub of the BAN.  In a two-hop extended star BAN, the hub and a node may optionally exchange frames via a relay capable node.

Figure 1 — Network topology

Access coordination at the MAC sublayer between BANs is not specified in this standard.  Optional mechanisms for coexistence and interference mitigating between adjacent or overlapping BANs are provided.  Nodes referenced in this standard are in the context of a given BAN, unless noted otherwise.

## Reference model

All nodes and hubs shall be internally partitioned into a physical (PHY) layer and a medium access control (MAC) sublayer, in accordance with the ISO/OSI-IEEE 802 reference model, as shown in Figure 2.  Direct communications between a node and a hub shall transpire at the PHY layer and MAC sublayer as specified in this standard; the PHY and MAC sublayer of a node or a hub may operate in one channel at any given time.  Message security services shall occur at the MAC sublayer, and security key generations may take place inside and/or outside the MAC sublayer.

Figure 2 — Reference model

Within a node or a hub, the MAC provides its service to the MAC client through the MAC service access point (SAP) located immediately above the MAC sublayer, while the PHY provides its service to the MAC through the PHY SAP located between them.  On transmission, the MAC client passes MAC service data units (MSDUs) to the MAC sublayer via the MAC SAP, and the MAC sublayer passes MAC frames (also known as MAC protocol data units or MPDUs) to the PHY layer via the PHY SAP.  On reception, the PHY layer passes MAC frames to the MAC sublayer via the PHY SAP, and the MAC sublayer passes MSDUs to the MAC client via the MAC SAP.

There may be a logical node management entity (NME) or hub management entity (HME) that exchanges network management information with the PHY and MAC as well as with other layers.  The HME is a superset of the NME in terms of the management functionality they each support.  However, the presence of the NME or HME and the partitioning between the NME or HME and the MAC or the PHY is not mandated, nor is the behavior of the NME or HME specified, in this standard.

## Time base

All nodes and hubs shall establish a time reference base as shown in Figure 3, if their medium access is to be scheduled in time, where the time axis is divided into beacon periods (superframes) of equal length and each beacon period is composed of allocation slots of equal length and numbered from 0, 1, ....  An allocation interval may be referenced in terms of the numbered allocation slots comprising it, and a point of time may be referenced in terms of the numbered allocation slot preceding or following it as appropriate.

Figure 3 — Time reference base

If time reference is needed for access scheduling in its BAN, the hub shall choose the boundaries of beacon periods (superframes) and hence of the allocation slots therein.  In beacon mode operation for which beacons are transmitted, the hub shall communicate such boundaries by transmitting beacons at the start or other specified locations of beacon periods (superframes), and optionally timed frames (T-Poll frames) containing their transmit time relative to the start time of current beacon period (superframe).  In non-beacon mode operation for which beacons are not transmitted but time reference is needed, the hub shall communicate such boundaries by transmitting timed frames (T-Poll frames) also containing their transmit time relative to the start time of current superframe.

A node requiring a time reference in the BAN shall derive and recalibrate the boundaries of beacon periods (superframes) and allocation slots from reception of beacons or/and timed frames (T-Poll frames).

A frame transmission may span more than one allocation slot, starting or ending not necessarily on an allocation slot boundary.

## State diagram

All nodes and hubs shall go through certain stages, i.e., states, at the MAC level before they exchange user (MAC client) data, as shown in Figure 4, where frames permitted or required to exchange between a node and a hub at each state are also indicated.    State classification and transition is defined with respect to a pair of a node and a hub, but is often referenced in the name of the node only.  Broadcast or multicast frames, such as beacons, are frame exchanges between a hub and a group of nodes but not between a hub and a node; their transmissions are not dictated by the pairwise state diagram specified in this subclause.

(a) Secured communication

(b) Unsecured communication

Figure 4 — Access state diagram

### Secured communication

A node and a hub shall follow the MAC state diagram Figure 4(a) for secured communication if either of them requires secured frame exchanges with the other.

#### Orphan state

At this state, the node does not have any relationship with the hub for secured communication.  It is the state that the node initially enters in relation with the hub.  The node and the hub shall not transmit any frames to each other —other than Security Association and control unsecured frames.  The node may exchange Security Association frames with the hub to establish a security association, i.e., to activate a pre-shared MK or generate a new master key (MK), and to authenticate with each other if so required, thereby transitioning to next state, Associated state.  However, if the node and the hub fail to activate or establish a shared MK, they shall not advance to Associated state.

#### Associated state

At this state, the node is associated, i.e., holds a shared master key, with the hub for PTK creation.  The node and the hub shall not transmit any frames to each other —other than Disassociation, Pairwise Temporal Key (PTK), and control unsecured frames.  The node may exchange PTK frames with the hub to establish a secured relationship, i.e., to confirm possession of a shared MK and to create a PTK, thereby transitioning to next state, Secured state.  However, if the node and the hub fail to create a PTK, they shall not advance to Secured state.  To repeal a security association and hence the current MK, either the node or the hub may send a Disassociation frame to the other, thus moving back to Orphan state.

#### Secured state

At this state, the node is secured, i.e., holds a PTK, with the hub for message security, i.e., for secured frame exchanges.  The node and the hub shall not transmit any frames to each other—other than Disassociation, Connection Request, and Connection Assignment secured frames and control unsecured frames.  The node may exchange Connection Request and Connection Assignment frames with the hub to establish a connection, thereby transitioning to next and final state, Connected state.  However, if the node and the hub fail to establish a connection, they shall not advance to Connected state.

#### Connected state

At this state, the node is connected, i.e., holds an assigned Connected_NID, a wakeup arrangement, and optionally one or more scheduled allocations with the hub for abbreviated node addressing, desired wakeup, and optionally scheduled access.  The node and the hub may transmit any secured frames to each other—other than Security Association secured frames, but shall not transmit any unsecured frames to each other—other than control unsecured frames as appropriate.  To repeal the Connected_NID, the wakeup arrangement, and the scheduled allocations of the node, either the node or the hub may send a Disconnection frame to the other, thereby moving back to Secured state.

### Unsecured communication

A node and a hub shall follow the MAC state diagram Figure 4(b) for unsecured communication if neither of them requires secured frame exchanges with the other.

#### Orphan state

At this state, the node does not have any relationship with the hub for unsecured communication.  It is the state that the node initially enters in relation with the hub.  The node and the hub shall not transmit any frames to each other—other than Connection Request, Connection Assignment, and control unsecured frames.  The node may exchange Connection Request and Connection Assignment frames with the hub to establish a connection, thereby transitioning to next and final state, Connected state.  However, if the node and the hub fail to establish a connection, they shall not advance to Connected state.

#### Connected state

At this state, the node is connected, i.e., holds an assigned Connected_NID, a wakeup arrangement, and optionally one or more scheduled allocations with the hub for abbreviated node addressing, desired wakeup, and optionally scheduled access.  The node and the hub may transmit any unsecured frames to each other—other than Security Association, Disassociation, and PTK frames, but shall not transmit secured frames to each other.  To repeal the Connected_NID, the wakeup arrangement, and the scheduled allocations of the node, either the node or the hub may send a Disconnection frame to the other, thereby moving back to Orphan state.

To change to secured communications between them, the node and the hub shall disconnect from each other, thus moving back to Orphan state and then following the state diagram Figure 4(a) for secured communication.

## Security paradigm

All nodes and hubs shall be offered three security levels in this standard:

  - Level 0 – unsecured communication.  At this level, messages are transmitted in unsecured frames, which provide no measures for message authenticity and integrity validation, confidentiality and privacy protection, and replay defense.

  - Level 1 – authentication but not encryption.  At this level, messages are transmitted in secured authenticated but not encrypted frames, which provide measures for message authenticity and integrity validation and replay defense but not confidentiality and privacy protection.

  - Level 2 – authentication and encryption.  At this level, messages are transmitted in secured authenticated and encrypted frames, which provide measures for message authenticity and integrity validation, confidentiality and privacy protection, and replay defense.

During association, a node and a hub shall jointly select a security level suitable for their subsequent secured frame exchanges, and whether to require authentication for control frames, based on their respective security requirements and certain information specific to each other.

For unicast secured communication, the node and the hub shall activate a pre-shared master key (MK) or establish a new MK via an unauthenticated or authenticated association, and shall create a pairwise temporal key (PTK) via a PTK creation procedure.  For multicast secured communication, the hub shall distribute a group temporal key (GTK) to the corresponding multicast group.

The node and the hub shall follow the security structures shown in Figure 5 to perform security key generations and provide message security services.  A “session” indicated in this figure refers to a time span in which a temporal key remains valid.  The length of the “session” is determined by the security policy governing data transfers between the two communicating parties.  It is further limited by the technical restrictions on the reuse of the same temporal key over successive messages (i.e., MAC frames in this specification).

Figure 5 — Security hierarchy

# MAC frame formats

This clause defines the formats of MAC frames.  It starts in 6.1 with the general conventions for specifying the transmit order of MAC frames and their constituent fields, followed in 6.2 by the format definition for the common fields that are present in all MAC frames.  The next three subclauses 6.3-6.4.7 define each frame type and subtype in detail.  The final two subclauses 6.6 and 6.7 define MAC and PHY Capability fields and a number of information elements that appear in some MAC frames.

## Conventions

A MAC frame is an ordered sequence of fields delivered to or from the physical layer service access point (PHY SAP).  Each figure in Clause 6 depicts the fields contained in a MAC frame, or a part thereof, from left to right in the transmit order, with fields that are optional or selectively absent drawn in dash where possible.  The figure also indicates the number of octets contained in each field and the corresponding octet transmit order, on top of the field, as illustrated in Figure 6.

Figure 6 — Example figure depicting fields aligned with octet boundaries

Unless otherwise noted, an atomic field, i.e., a field that is not in turn comprised of other fields, denotes a numerical value encoded in unsigned binary.  If such a field contains F octets (F > 1), octet 0 is the octet containing the least-significant bits of that field and is the first octet transmitted of the field, whereas octet F-1 is the octet containing the most-significant bits of that field and is the last octet transmitted of the field.  The octet order is indicated as “L-R”, i.e., from left to right, above a multi-octet non-atomic field.

In a figure that depicts certain fields not aligned with octet boundaries, the number of bits and the corresponding bit order of encoding are shown instead for each field in the figure, as illustrated in Figure 7.  Bits are ordered continually across the fields that are not aligned with octet boundaries, from left to right, starting from bit 0, i.e., the least-significant bit of the bits comprising these fields.  Bit numbering restarts from 0 in fields located on octet boundaries.

Figure 7 — Example figure depicting fields not aligned with octet boundaries

Each field is defined, or set, based on the perspective of the node or the hub that is sending the frame containing that field, referred to as the sender.  It is parsed based on its definition by the hub or the node intended to receive the frame containing it, referred to as the recipient or the intended recipient.

Reserved fields are set to 0 on transmission and ignored on reception.  If some values in a field are reserved, the field is not set to any of those reserved values on transmission.  Unless otherwise noted, fields that are set to reserved values or defined based on other fields that are set to reserved values are ignored on reception.

## General format

A MAC frame consists of a fixed-length MAC header, a variable-length MAC frame body, and a fixed-length FCS field as shown in Figure 8.  The MAC frame body has an octet length L_FB such that 0 ≤ L_FB ≤ pMaxFrameBodyLength, and is present only if it has a nonzero length.

Figure 8 — MAC frame format

### MAC header

The MAC header is formatted as shown in Figure 9.

Figure 9 — MAC header format

#### Frame Control

The Frame Control is formatted as shown in Figure 10.  [JMH’s note:  (a) I swapped the Retry / B2 / Poll Type field with the First Frame / On Time field, in particular, moved the former to after Frame Subtype and Frame Type fields, given its dependence on them and its close relationship to them.  (b) I reused the Retry bit in beacons for indication of B2’s presence, which is a cleaner way than using a bit in the MAC Capability for this purpose as I had suggested at the F2F.  (c) I further reused the Fragment Number field in beacons for indicating the coexistence mechanisms being enabled.]

Figure 10 — Frame Control format

##### Protocol Version

The Protocol Version field is set to 0 for this revision of the standard.  All other values are reserved.  This field is invariant in size and place across all revisions of this standard.

##### Acknowledgment (Ack) Policy

The Ack Policy field is set according to Table 1 to indicate the acknowledgement requirement of the current frame.

Table 1 — Acknowledgment (Ack) Policy field encoding

| Field value b1 b2 | Acknowledgment requirement |
| --- | --- |
| 00 | No acknowledgment (N-Ack) or group acknowledgment (G-Ack) |
| 10 | Immediate acknowledgment (I-Ack) |
| 11 | Block acknowledgment later (L-Ack) |
| 01 | Block acknowledgment (B-Ack) |

The group acknowledgment (G-Ack) value is applicable to frames sent to a hub and of frame type set to data and frame subtype set to mG-AckDataSubtype.

##### Security Level

The Security Level field is set according to Table 2 such that it indicates the security level of the current frame.

Table 2 — Security Level field encoding

| Field value b3 b4 | Security level of current frame |
| --- | --- |
| 00 | Level 0 – frame not secured |
| 10 | Level 1 – frame authenticated but not encrypted |
| 01 | Level 2 – frame authenticated and encrypted |
| 11 | Reserved |

##### TK Index

The TK Index field is set as follows to indicate the pairwise temporal key (PTK) or group temporal key (GTK) being used to secure the current frame:

  - In frames secured by a PTK, it is set to the value of the PTK Index field in the PTK frames that were exchanged in creating the PTK.

  - In frames secured by a GTK, it is set to the value of the GTK Index field in the GTK frame that was exchanged in distributing the GTK.

The TK Index field is reserved in unsecured frames.

##### Relay

The Relay field is set to 1 in frames sent to or from a relaying node in a two-hop extended star network.  It is reserved in all other frames.

##### First Frame / On Time

The First Frame / On Time field is set to 1 if this is the first frame sent by a hub at the start of a scheduled allocation interval.  It is set to 0 in all other frames.

##### Frame Subtype

The Frame Subtype field is set to indicate the subtype of the current frame of a given type according to Table 3.

##### Frame Type

The Frame Type field is set to indicate the type of the current frame according to Table 3.

Table 3 — Frame Type and Frame Subtype field encoding

| Frame Type value b4 b5 | Frame Type name | Frame Subtype value b0 b1 b2 b3 | Frame Subtype name |
| --- | --- | --- | --- |
| 00 | Management | 0000 | Beacon |
| 00 | Management | 1000 | Reserved |
| 00 | Management | 0100 | Security Association |
| 00 | Management | 1100 | Security Disassociation |
| 00 | Management | 0010 | PTK |
| 00 | Management | 1010 | GTK |
| 00 | Management | 0110-1110 | Reserved |
| 00 | Management | 0001 | Connection Request |
| 00 | Management | 1001 | Connection Assignment |
| 00 | Management | 0101 | Multinode Connection Assignment |
| 00 | Management | 1101 | Disconnection |
| 00 | Management | 0011-0111 | Reserved |
| 00 | Management | 1111 | Command |
| 10 | Control | 0000 | I-Ack |
| 10 | Control | 1000 | B-Ack |
| 10 | Control | 0100-1100 | Reserved |
| 10 | Control | 0010 | I-Ack+Poll |
| 10 | Control | 1010 | B-Ack+Poll |
| 10 | Control | 0110 | Poll |
| 10 | Control | 1110 | T-Poll |
| 10 | Control | 0001-1011 | Reserved |
| 10 | Control | 0111 | Wakeup |
| 10 | Control | 1111 | B2 |
| 01 | Data | 1110 | Emergency |
| 01 | Data | 0000-0110 0001-1111 | User defined data subtype |
| 11 | Reserved | 0000-1111 | Reserved |

##### More Data

The More Data field is set as follows:

  - In frames sent by a node to the hub,

    - it is set 0 if the node has no pending frame transactions to initiate with the hub, or

    - it is set to 1 if the node has one or more pending frame transactions to initiate with the hub.

  - In unicast management and data type frames sent by a hub to a node,

    - it is set to 0 if the hub is not to send a post to the node after the current frame transaction, or

    - it is set to 1 if the hub is to send a post to the node pSIFS after the end of the current frame transaction.

  - In I-Ack and B-Ack frames sent by a hub to a node,

    - it is set to 0 if the hub is not to send a post to the node after the end of the current allocation interval, or

    - it is set to 1 if the hub is to send a post to the node at or after the end of the current allocation interval.  In the latter case, when the hub is to send a post to the node is indicated in the Sequence Number / Poll-Post Window and Fragment Number / Coexistence / Next fields of the current frame.

  - In I-Ack+Poll, B-Ack+Poll, Poll, and T-Poll frames sent by a hub to a node,

    - it is set to 0 if the hub grants to the node through this frame a polled allocation starting pSIFS after the end of the current frame, with the end or size of the polled allocation indicated in the Sequence Number / Poll-Post Window field of the current frame, or

    - it is set to 1 if the hub grants to the node through this frame no polled allocation but is to send another poll  or a post to the node at a future time as indicated in the Sequence Number / Poll-Post Window and Fragment Number / Coexistence / Next fields of the current frame.

  - In all other frames, it is reserved.

##### Retry / B2 / Poll Type

The Retry / B2 / Poll Type field is set as follows:

  - In beacon frames, it is used as a B2 field, which is set to 1 if a B2 frame is to be transmitted in the current beacon period, or is set to 0 otherwise.

  - In non-beacon management or data frames that were transmitted previously and are being transmitted again, it is used as a Retry field, which is set to 1.

  - In I-Ack+Poll, B-Ack+Poll, Poll, and T-Poll frames sent by a hub to a node, if the More Data field is set to 0 indicating a polled allocation, it is used as a Poll Type field, which is set to 0 for a type-I polled allocation or 1 for a type-II polled allocation, in conjunction with the Poll-Post Window Field encoding for these frames as defined in next subclause.

  - In all other frames, it is reserved.

##### Sequence Number / Poll-Post Window

The Sequence Number / Poll-Post Window field is set as follows:

  - In beacon frames, it is used as a Sequence Number field, which is set as follows:

    - It is incremented by one from its value applicable to the last beacon period (superframe), active or inactive.

    - It is also referred to as the sequence number of the current beacon period (superframe).

  - In non-beacon management type frames, it is used as a Sequence Number field, which is set to the sequence number of the current beacon period (superframe) as follows:

    - It is set to the sequence number of the beacon that is or was transmitted in the current beacon period.

    - It is set to the sequence number of the beacon that would otherwise have been or be transmitted in the current superframe if no beacon was or is to be transmitted in this superframe.

  - In I-Ack and B-Ack frames sent by a hub to a node, it is used as a Poll-Post Window field, which is set as follows:

    - If the More Data field of the current frame is set to 0, it is reserved.

    - If the More Data field of the current frame is set to 1, it is set to B such that the hub is to send a poll or post to the node at the start of the allocation slot numbered B, in a beacon period indicated in the Fragment Number  / Coexistence / Next field of the current frame.

  - In I-Ack+Poll, B-Ack+Poll, Poll, and T-Poll frames sent by a hub to a node, it is used as a Poll-Post Window field, which is set as follows:

    - If the More Data field of the current frame is set to 0, indicating that the node is granted via this frame a polled allocation starting pSIFS after the end of this frame, it is set to A such that the polled allocation ends at the end of the allocation slot numbered A in the current beacon period (superframe) for a Type-I polled allocation, or is set to the maximum number of frames the polled node is granted to transmit in the polled allocation for a Type-II polled allocation.  Whether the polled allocation is of type-I or type-II is indicated in the preceding Poll Type field.

    - If the More Data field of the current frame is set to 1, indicating that the node is granted via this frame no polled allocation but is to be sent another poll or a post, it is set to B such that the hub is to send next poll or post at the start of the allocation slot numbered B, in a beacon period indicated in the Fragment Number / Coexistence / Next field of the current frame.

  - In Wakeup frames locally broadcast by a hub in its body area network (BAN), it is used as a Poll-Post Window field, which is set as follows:

    - If the current frame is to keep only one node at active state for a duration of D, it is set to D in units of 20 milliseconds.

    - If the current frame is to bring multiple nodes to active state at the end of a duration of D, it is set to D in units of 20 milliseconds.

  - In B2 frames locally broadcast by a hub in its body area network (BAN), it is used as a Poll-Post Window field, which is set to C such that the current frame starts a contention access phase (CAP) ending at the end of the allocation slot numbered C in the current beacon period (superframe).

  - In data type frames, it is used as a Sequence Number field, which is set as follows:

    - It is incremented by one from its value in the frame that was of the same subtype and addressed to the same recipient(s) and that contained the previous MSDU or part thereof.

    - It has the same value in frames containing fragments of the same MSDU.

  - In all other frames, it is reserved.

##### Fragment Number / Coexistence / Next

The Fragment Number / Coexistence / Next field is set as follows:

  - In beacon frames, it is used as a Coexistence field, which is formatted as shown in Figure 11.  The Beacon Shifting field is set to 1 if beacon shifting is currently enabled, or is set to 0 otherwise.  The Channel Hopping field is set to 1 if channel hopping is currently enabled, or is set to 0 otherwise.  The Inactive Superframe Starting field is set to 1 if one or more inactive superframes are enabled (starting) at the end of the current beacon period (superframe), or is set to 0 otherwise.  The Time Sharing field is set to 1 if time sharing with one or more body area networks (BANs) is currently enabled, or is set to 0 otherwise.  [JMH’s note:  I came up with this bitmap after the F2F to facilitate the definition/determination of which optional coexistence related fields is present in the beacon frame payload.  See 6.3.1 Beacon.]

Figure 11 — Coexistence format

  - In non-beacon management type frames containing a frame payload not fragmented or the first fragment of a fragmented frame payload, it is used as a Fragment Number field, which is set to 0.

  - In non-beacon management type frames containing a non-first fragment of a fragmented frame payload, it is used as a Fragment Number field, which is incremented by one from its value in the frame containing the previous fragment of the frame payload.

  - In data type frames containing an MSDU not fragmented or the first fragment of a fragmented MSDU, it is used as a Fragment Number field, which is set to 0.

  - In data type frames containing a non-first fragment of a fragmented MSDU, it is used as a Fragment Number field, which is incremented by one from its value in the frame containing the previous fragment of the MSDU.

  - In I-Ack and B-Ack frames sent by a hub to a node,

    - if the More Data field of the current frame is set to 0, it is reserved;

    - if the More Data field of the current frame is set to 1, indicating that the hub is to send a post to the node at the start of the allocation slot indicated in the Poll-Post Window field of the current frame, it is used as a Next field, which is set to N such that the allocation slot is the one located in the current beacon period if N = 0 or in the next Nth beacon period not counting the current beacon period if N > 0.

  - In I-Ack+Poll, B-Ack+Poll, Poll, and T-Poll frames sent by a hub to a node,

    - if the More Data field of the current frame is set to 0, indicating that the node is granted via this frame a polled allocation starting pSIFS after the end of this frame and ending at the end of the allocation slot indicated in the Sequence Number field of the current frame, it is reserved;

    - if the More Data field of the current frame is set to 1, indicating that the node is granted via this frame no polled allocation but is to be sent another poll or post at the start of the allocation slot indicated in the Poll-Post Window field of the current frame, it is used as a Next field, which is set to N such that the allocation slot is the one located in the current beacon period if N = 0 or in the next Nth beacon period not counting the current beacon period if N > 0.

  - In all other frames, it is reserved.

#### Recipient ID

The Recipient ID field is set to the abbreviated address (i.e., NID or HID) of the recipient of the current frame.

#### Sender ID

The Sender ID field is set to the abbreviated address (i.e., NID or HID) of the sender of the current frame.

#### BAN ID

The BAN ID field is set to the abbreviated address of the body area network (BAN) in which the current frame is transferred.

### MAC frame body

The MAC frame body, when it has a nonzero length, is formatted as shown in Figure 12.  The length of the MAC frame body L_FB must not exceed pMaxFrameBodyLength.

Figure 12 — MAC frame body format

The Security Sequence Number and Message Integrity Code (MIC) fields are not present in unsecured frames, as indicated by the Security Level field of the MAC header of the current frame.

#### Security Sequence Number

The Security Sequence Number field is set as follows for nonce construction and replay detection:

  - In frames secured with a new pairwise temporal key (PTK) or group temporal key (GTK), it is set to 1.

  - In frames secured with a used PTK, whether they are transmitted for the first time or they are retries, it is incremented by one from its value in the last transmitted frame secured with the same PTK.

  - In frames secured with a used GTK, whether they are transmitted for the first time or they are retries, it is incremented by one from its value in the last transmitted frame secured with the same GTK.

The value of the Security Sequence Number field increments in frames secured with the same PTK or GTK, rather than in frames of the same frame type or frame subtype.  It increments even if the current frame transmission is a retransmission of an earlier transmission.

#### Frame Payload

The Frame Payload field is set as follows:

  - In management type frames, prior to encryption (if any) it is set to a sequence of fields to be communicated to the recipient(s), with the fields defined in 6.3.

  - In control type frames, it is set to a sequence of fields to be communicated to the recipient(s), with the fields defined in 6.4.

  - In data type frames, prior to encryption (if any) it is set to a sequence of octets passed as an MSDU through the MAC SAP to the MAC, without altering the order of the sequence.

  - In data type frames with the Relay field in their MAC header set to 1, prior to encryption (if any) it is set to the relayed MAC frame that would otherwise be sent directly between a node and a hub without relay.

If a frame payload is fragmented and carried in multiple frames, the Frame Payload field is set to a fragment of the otherwise unfragmented frame payload.

The length of the Frame Payload field, denoted as L_FP in Figure 12, must be such that the length of the MAC frame body does not exceed pMaxFrameBodyLength.  If the Frame Payload has a zero length, i.e., if a frame does not have a frame payload, then the frame does not have a MAC frame body either.

#### MIC

The MIC field is set to a message authentication code for preserving the authenticity and integrity of the MAC header and the MAC frame body of the current secured frame, as further specified in 8.3.1.5.

### FCS

The FCS field is formatted as shown in Figure 13, where a15 is the least-significant bit of the field, and a0 is the most-significant bit.  The bits a15, a14, …, a0 are the binary coefficients of  a CRC polynomial of degree 15 denoted as

R(x) = a15x15 + a14x14 + … + a1x + a0

Figure 13 — FCS format

The CRC polynomial is calculated over a calculation field using the following CRC-16-CCITT standard generator polynomial of degree 16:

G(x) = x16 + x12 + x5 + 1

The calculation field is the MAC frame except the FCS field for this specification. It is mapped to a message polynomial M(x) of degree k-1, where k is the number of bits in the calculation field. The least-significant bit of the first octet presented to the PHY SAP is the coefficient of the xk-1 term, the next least-significant bit is the coefficient of the xk-2 term, …, and the most-significant bit of the last octet transmitted is the coefficient of the x0 term.

The CRC polynomial is the remainder resulting from the (modulo 2) division of [x16 × M(x)] by G(x):

R(x) = x16 × M(x)  mod G(x)

## Management type frames

A management type frame contains certain mandatory fixed-length fields and some optional variable length components referred to as information elements.

### Beacon

A beacon frame contains a Frame Payload that is formatted as shown in Figure 14.  It is locally broadcast by a hub in every beacon period.

Figure 14 — Frame Payload format for beacon frames

#### Sender Address

The Sender Address field is set to the IEEE MAC address of the hub sending the current beacon.

#### Beacon Period Length

The Beacon Period Length field is set to the length of the current beacon period (superframe) in units of allocation slots.  It is set to 0 to encode a value of 256 allocation slots.

If beacon shifting is enabled, a beacon period must have 4N allocation slots in length, where N is an integer.  The allocation slots in a beacon period are numbered 0, 1, …, 4N-1, starting from the allocation slot that starts at the beacon transmission time of the beacon period to the allocation slot that ends at the end of the beacon period and, if the beacon transmission time is not at the start of the beacon period, wrapping back to the allocation slot that starts at the start of the beacon period and finally to the allocation slot that ends at the beacon transmission time, as shown in Figure 15.  A beacon period has four quarters, which are comprised of allocation slots 0 to N-1, N to 2N-1, 2N to 3N-1, and 3N to 4N-1, respectively.  An allocation interval that does not cross a quarter boundary will not be fragmented regardless of which beacon shifting sequence is being used.

Figure 15 — Allocation slot ordering and numbering in beacon periods

#### Allocation Slot Length

The Allocation Slot Length field is set to L such that the length of an allocation slot is equal to pAllocationSlotMin  + L×pAllocationSlotResolution.

#### RAP1 Length

The RAP1 Length field is set to the length of random access phase 1 (RAP1), in units of allocation slots, which starts at the end of exclusive access phase (EAP1) in the current beacon period in absence of beacon shifting or in the next beacon period in presence of beacon shifting.  The value of this field must not be smaller than the value of the Minimum RAP1 Length field in any Connection Assignment frame sent by the hub transmitting this beacon.  EAP1 is specified through Connection Assignment frames as defined in 6.3.7.

#### RAP2 Length

The RAP2 Length field is set to the length of random access phase 2 (RAP2), in units of allocation slots, which starts at the end of EAP2 in the current beacon period in absence of beacon shift or in the next beacon period in presence of beacon shifting.  EAP2 is also specified through Connection Assignment frames as defined in 6.3.7.

#### MAC Capability

The MAC Capability is defined as in 6.6.1.

#### PHY Capability

The PHY Capability is defined as in 6.6.2.

#### Beacon Shifting Sequence

The Beacon Shifting Sequence is present only if beacon shifting is currently enabled as encoded according to Figure 11.  When present, it is formatted as shown in Figure 16 to indicate the beacon transmission time in the current beacon period.

Figure 16 — Beacon Shifting Sequence format

##### Beacon Shifting Sequence Index

The Beacon Shifting Sequence Index field is set according to Table 4 to the index m of the PN sequence Pm(n) governing the beacon transmission time pattern.

Table 4 — Beacon Shifting sequences

| Beacon Shifting Sequence Index decimal value | Beacon Shifting Sequence values | Beacon Shifting Sequence pattern (“…” denotes pattern repeat) |
| --- | --- | --- |
| 0 | PN0(n) = n mod 2, n = 0, 1, … | PN0(n) = 0, 1, 0, 1, … |
| 1 | PN1(n) = 2×PN0(n), n = 0, 1, … | PN1(n) = 0, 2, 0, 2, … |
| 2 | PN2(n) = n mod 4, n = 0, 1, … | PN2(n) = 0, 1, 2, 3, … |
| 3 | PN3(n) = [PN0(n) + PN2 (n)]/2 mod 2 + [PN0(n) + PN1(n) + PN2 (n)] mod 4, n = 0, 1, … | PN3(n) = 0, 1, 3, 2, … |
| 4 | PN4(n) = [PN0(n) + PN1(n) + PN2(n)]/2, n = 0, 1, … | PN4(n) = 0, 2, 1, 3, … |
| 5 | PN5(n) = {PN2(n) + [PN0(n) + PN2(n)]/2} mod 4, n = 0, 1, … | PN5(n) = 0, 2, 3, 1, … |
| 6 | PN6(n) = PN1(n)+{[ PN0(n)+ PN2(n)]/2 mod 2}, n = 0, 1, … | PN6(n) = 0, 3, 1, 2, … |
| 7 | PN7(n) = [PN1(n) + PN2(n)] mod 4, n = 0, 1, … | PN7(n) = 0, 3, 2, 1, … |
| 8-15 | Reserved | Reserved |

##### Beacon Shifting Sequence Phase

The Beacon Shifting Sequence Phase field is set according to Table 4 to the phase n of the chosen PN sequence Pm(n) in the current beacon period.

#### Channel Hopping State

The Channel Hopping State field is present only if channel hopping is currently enabled as encoded according to Figure 11.  When present, it is set to the current state of a 16-bit maximum-length linear feedback shift register (LFSR) used to generate the channel hopping sequence by the hub sending this beacon.

#### Next Channel Hop

The Next Channel Hop field is present only if channel hopping is currently enabled as encoded according to Figure 11.  When present, it is set to the sequence number of the beacon period (superframe) in which the hub sending the current beacon is to hop to another channel according to its channel hopping sequence.  Channel hopping is independent of inactive duration as defined in Error: Reference source not found.

#### Inactive Duration

The Inactive Duration field is present only if inactive superframes are enabled (starting) at the end of the current beacon period (superframe).  When present, it is set to the number of inactive superframes before the occurrence of next active superframe; it is set to 0 to encode a value of 256 inactive superframes.

### Security Association

A Security Association frame contains a Frame Payload that is formatted as shown in Figure 17.  It is exchanged between a node and a hub to activate a pre-shared MK or generate a new shared MK.

Figure 17 — Frame Payload format for Security Association frames

#### Recipient Address

The Recipient Address field is set to the IEEE MAC address of the recipient of the current frame.

#### Sender Address

The Sending Address field is set to the IEEE MAC address of the sender of the current frame.

#### Security Suite Selector

The Security Suite Selector is formatted as shown in Figure 18.

Figure 18 — Security Suite Selector format

##### Security Association Protocol

The Security Association Protocol field is set according to Table 5 to indicate a security association protocol being run for the security association.

Table 5 — Security Association Protocol field encoding

| Field value decimal | Security Association protocol |
| --- | --- |
| 0 | Master key pre-shared association |
| 1 | Unauthenticated association |
| 2 | Public key hidden association |
| 3 | Password authenticated association |
| 4 | Display authenticated association |
| 5-7 | Reserved |

##### Security Level Required

The Security Level Required field is set to the security level required by this sender according to Table 6.

Table 6 —Security Level Required field encoding

| Field value decimal | Security level required |
| --- | --- |
| 0 | Level 0 – unsecured communication |
| 1 | Level 1 – authentication but not encryption |
| 2 | Level 2 – authentication and encryption |
| 3 | Reserved |

##### Control Frame Authentication

The Control Frame Authentication field is set to 1 if control type frames sent from or to this sender must be authenticated but not encrypted when they are required to have security level 1 or 2.  It is set to 0 if control type frames sent from or to this sender must be neither authenticated nor encrypted even when they are otherwise required to have security level 1 or 2.

##### Message Security Protocol

The Message Security Protocol field is set according to Error: Reference source not foundTable 7 to indicate a message authentication and encryption protocol selected by this sender for subsequent secured communication.

Table 7 — Message Security Protocol field encoding

| Field value decimal | Message security protocol |
| --- | --- |
| 0 | AES-128 CCM |
| 1-255 | Reserved |

[MAC group’s note:  Camellia-CCM and SEED are other ciphers that are being discussed in this group.]

#### Association Sequence Number

The Association Sequence Number field is set to the number (i.e., position) of the current Security Association frame in the run of the chosen security association protocol.  In particular, it is set to 1 in the first Security Association frame of the protocol, 2 in the second Security Association frame, 3 in the third, etc.  The first Security Association frame is the Security Association frame transmitted or retransmitted by the node initializing the security association, the second Security Association frame is the Security Association frame transmitted or retransmitted by the responding hub, and the like.

#### Security Association Data

The Security Association Data field is specific to the security association protocol being run.

For master key pre-shared association, the Security Association Data field is not present.

For unauthenticated association, public key hidden association, password authenticated association, and display authenticated association, the Security Association Data field is formatted as shown in Figure 19.

Figure 19 — Security Association Data format for security association protocols 1-4

##### Sender Nonce

The Sender Nonce field is set to a statistically unique number per sender and per security association procedure, except otherwise indicated:

  - For unauthenticated association, public key hidden association, and password authenticated association,

    - in the first and second Security Association frames of the current security association procedure, the field is set afresh and independently to an integer randomly drawn with a uniform distribution over the interval (0, 2^128);

    - in the third Security Association frame, the field is set to its value contained in the first Security Association frame of the procedure.

  - For display authenticated association,

    - in the first Security Association frame of the current security association procedure, the field is set to 0;

    - in the second and third Security Association frames, the field is set afresh and independently to an integer randomly drawn with a uniform distribution over the interval (0, 2^128).

##### Sender PKx

The Sender PKx field is set to the X-coordinate of the sender’s elliptic curve public key, except otherwise indicated:

  - For unauthenticated association,

    - in the first and second Security Association frames of the current security association procedure, the field is set to the X-coordinate of the sender’s elliptic curve public key PK = (PKx, PKY);

    - in the third Security Association frame, the field is set to its value contained in the first Security Association frame of the procedure.

  - For public key hidden association,

    - in the first and third Security Association frames of the current security association procedure, the field is set to 0;

    - in the second Security Association frame, the field is set to the X-coordinate of the sender’s elliptic curve public key PK = (PKx, PKY).

  - For password authenticated association,

    - in the first and third Security Association frames of the current security association procedure, the field is set to the X-coordinate of the sender’s password-scrambled elliptic curve public key PK' = (PK'x, PK'Y);

    - in the second Security Association frame, the field is set to the X-coordinate of the sender’s elliptic curve public key PK = (PKx, PKY).

  - For display authenticated association,

    - in the first Security Association frame of the current security association procedure, the field is set to 0;

    - in the second and third Security Association frames, the field is set to the X-coordinate of the sender’s elliptic curve public key PK = (PKx, PKY).

##### Sender PKY

The Sender PKY field is set to the Y-coordinate of the sender’s elliptic curve public key, except otherwise indicated:

  - For unauthenticated association,

    - in the first and second Security Association frames of the current security association procedure, the field is set to the Y-coordinate of the sender’s elliptic curve public key PK = (PKx, PKY);

    - in the third Security Association frame, the field is set to its value contained in the first Security Association frame of the procedure.

  - For public key hidden association,

    - in the first and third Security Association frames of the current security association procedure, the field is set to 0;

    - in the second Security Association frame, the field is set to the Y-coordinate of the sender’s elliptic curve public key PK = (PKx, PKY).

  - For password authenticated association,

    - in the first and third Security Association frames of the current security association procedure, the field is set to the Y-coordinate of the sender’s password-scrambled elliptic curve public key PK' = (PK'x, PK'Y);

    - in the second Security Association frame, the field is set to the Y-coordinate of the sender’s elliptic curve public key PK = (PKx, PKY).

  - For display authenticated association,

    - in the first Security Association frame of the current security association procedure, the field is set to 0;

    - in the second and third Security Association frames, the field is set to the Y-coordinate of the sender’s elliptic curve public key PK = (PKx, PKY).

##### MK_KMAC

The MK_KMAC field is set to a key message authentication code (KMAC) for certain fields of the frame payloads of the Security Association frames of the current security association procedure, except otherwise indicated:

  - For unauthenticated association,

    - in the first Security Association frame of the current security association procedure, the field is set to 0;

    - in the second Security Association frame, the field is set to a KMAC for certain fields of the frame payloads of the first and second Security Association frames of the procedure;

    - in the third Security Association frame, the field is set to a KMAC for certain fields of the frame payloads of the second and third Security Association frames.

  - For public key hidden association,

    - in the first Security Association frame of the current security association procedure, the field is set to 0;

    - in the second Security Association frame, the field is set to a KMAC for certain fields of the frame payloads of the first and second Security Association frames of the procedure if the sender of this frame has the public key of the sender of the first Security Association frame, or it is set to 0 otherwise;

    - in the third Security Association frame, the field is set to a KMAC for certain fields of the frame payloads of the second and third Security Association frames.

  - For password authenticated association,

    - in the first Security Association frame of the current security association procedure, the field is set to 0;

    - in the second Security Association frame, the field is set to a KMAC for certain fields of the frame payloads of the first and second Security Association frames of the procedure if the sender of this frame has a shared password with the sender of the first Security Association frame, or it is set to 0 otherwise;

    - in the third Security Association frame, the field is set to a KMAC for certain fields of the frame payloads of the second and third Security Association frames.

  - For display authenticated association,

    - in the first Security Association frame of the current security association procedure, the field is set to an initial witness equal to a KMAC for certain fields of the frame payload of the third Security Association frame of the procedure;

    - in the second and third Security Association frames, the field is set to 0.

### Security Disassociation

A Security Disassociation frame contains a Frame Payload that is formatted as shown in Figure 20.  It is transmitted by either an associated node or a hub to repeal an existing security association, i.e., the shared master key (MK).

Figure 20 — Frame Payload format for Security Disassociation frames

#### Recipient Address

The Recipient Address field is set to the IEEE MAC address of the recipient of the current frame.

#### Sender Address

The Sending Address field is set to the IEEE MAC address of the sender of the current frame.

##### Sender Nonce

The Sender Nonce field is set to a statistically unique number per sender and per security disassociation procedure, i.e., it is set afresh and independently to an integer randomly drawn with a uniform distribution over the interval (0, 2^128).

##### DA_KMAC

The DA_KMAC field is set to a key message authentication code (KMAC) for certain fields of the frame payload of this Security Disassociation frame.

### Pairwise Temporal Key (PTK)

A PTK frame contains a Frame Payload that is formatted as shown in Figure 21.  It is exchanged between a node and the hub with which the node is associated to create a PTK based on a shared master key (MK).

Figure 21 — Frame Payload format for PTK frames

#### Recipient Address

The Recipient Address field is set to the IEEE MAC address of the recipient of the current frame.

#### Sender Address

The Sending Address field is set to the IEEE MAC address of the sender of the current frame.

#### Message Number

The Message Number field is set to the number (i.e., position) of the current PTK frame in the current PTK creation procedure.  In particular, it is set to 1 in the first PTK frame of the procedure, 2 in the second PTK frame, and 3 in the third.  The first PTK frame is the PTK frame transmitted or retransmitted by the node or hub initializing the procedure, the second PTK frame is the PTK frame transmitted or retransmitted by the responding hub or node, and the third PTK frame is the PTK frame transmitted or retransmitted by the initiating node or hub again.  The other values of the field are reserved.

#### PTK Index

The PTK Index field in the first PTK frame transmitted or retransmitted by the node initiating the PTK creation procedure is set as follows to identify the PTK being created:

  - If no PTK was previously created with the responding node, it is set to 0.

  - Otherwise, it is set to 1 minus its value used in successfully creating the last PTK between the sender and the recipient.

The PTK Index field in the second and third PTK frames of the PTK creation procedure is set to its value contained in the first PTK frame of the procedure.

The PTK Index field takes on a value of either 0 or 1.  The other values of the field are reserved.

#### Sender Nonce

The Sender Nonce field is set to a statistically unique number per sender and per PTK creation procedure:

  - In the first and second PTK frames of the current PTK creation procedure, it is set afresh and independently to an integer randomly drawn with a uniform distribution over the interval (0, 2^128).

  - In the third PTK frame of the procedure, it is set to its value contained in the first PTK frame.

#### PTK_KMAC

The PTK_KMAC field is set to a key message authentication code (KMAC) certain fields of the frame payloads of the PTK frames of the current PTK creation procedure, except otherwise indicated:

  - In the first PTK frame of the current PTK creation procedure, it is set to 0.

  - In the second PTK frame, it is set to a KMAC for certain fields of the frame payloads of the first and second PTK frames of the procedure if the sender of this frame has a shared master key (MK) with the sender of the first PTK frame, or it is set to 0 otherwise.

  - In the third PTK frame, it is set to a KMAC for certain fields of the frame payloads of the second and third PTK frames.

### Group Temporal Key (GTK)

A GTK frame contains a Frame Payload that is formatted as shown in Figure 22.  It is transmitted by a hub to distribute a GTK to a secured node for securing multicast traffic.

Figure 22 — Frame Payload format for GTK frames

#### Recipient Address

The Recipient Address field is set to the IEEE MAC address of the recipient of the current frame.

#### Sender Address

The Sending Address field is set to the IEEE MAC address of the sender of the current frame.

#### GTK NID

The GTK NID field is set according to Table 12 to the broadcast or multicast NID that is to appear in the NID field of the MAC header of the frames secured by the GTK distributed in the current frame.

#### GTK Index

The GTK Index field is set as follows to identify the GTK being distributed:

  - If no GTK was previously distributed by this hub for the GTK NID indicated in the current frame, it is set to 0.

  - Otherwise, it is set to 1 minus its value used in successfully distributing the last GTK by this hub for the indicated GTK NID.

The GTK Index field takes on a value of either 0 or 1.  The other values of the field are reserved.

#### GTK SSN

The GTK SSN field is set to the security sequence number of the last frame secured with the GTK distributed in the current frame and addressed to the GTK NID indicated in the current frame.

#### GTK

The GTK field is set to the bit string representing the GTK being distributed in the current frame.

### Connection Request

A Connection Request frame contains a Frame Payload that is formatted as shown in Figure 23.  It is transmitted by a node to request creation or modification of a connection with a hub, in particular, a Connected_NID, a wakeup arrangement, and optionally a scheduled allocation.

Figure 23 — Frame Payload format for Connection Request frames

#### Recipient Address

The Recipient Address field is set to the IEEE MAC address of the recipient of the current frame.

#### Sender Address

The Sending Address field is set to the IEEE MAC address of the sender of the current frame.

#### Former Hub Address

The Former Hub Address field is set as follows:

  - If the sending node was not connected to another hub previously, it is set to 0.

  - Otherwise, it is set to the IEEE MAC address of the hub with which the node was last connected.

#### MAC Capability

The MAC Capability is defined as in 6.6.1.

#### PHY Capability

The PHY Capability is defined as in 6.6.2.

#### Connection Change Indicator

The Connection Change Indicator is formatted as shown in Figure 24.  It indicates certain fields that follow in the current frame have been changed in value since their last exchange between the sender and the recipient.

Figure 24 — Connection Change Indicator format

##### Wakeup Phase

The Wakeup Phase field is set to 1 if its value has been changed, or is set to 0 otherwise.

##### Wakeup Period

The Wakeup Period field is set to 1 if its value has been changed, or is set to 0 otherwise.

##### Uplink Request/Assignment IE

The Uplink Request/Assignment IE field is set to 1 if the value of the Explicit Uplink Request IE, Implicit Uplink Request IE, or Uplink Assignment IE has been changed, or is set to 0 otherwise.

##### Downlink Request/Assignment IE

The Downlink Request/Assignment IE field is set to 1 if the value of the Explicit Downlink Request IE, Implicit Downlink Request IE, or Downlink Assignment IE has been changed, or is set to 0 otherwise.

##### Bilink Request/Assignment IE

The Bilink Request/Assignment IE field is set to 1 if the value of the Explicit Bilink Request IE, Implicit Bilink Request IE, or Bilink Assignment IE has been changed, or is set to 0 otherwise.

##### Delayed Bilink Request/Assignment IE

The Delayed Bilink Request/Assignment IE field is set to 1 if the value of the Explicit Delayed Bilink Request IE, Implicit Delayed Bilink Request IE, or Delayed Bilink Assignment IE has been changed, or is set to 0 otherwise.

##### Channel Order IE

The Channel Order IE field is set to 1 if the value of the Nibble Encoded Channel Order IE or Octet Encoded Channel Order IE has been changed, or is set to 0 otherwise.

#### Wakeup Phase

The Wakeup Phase field is set to the sequence number of the next beacon period (superframe) in which the node is willing to wake up for frame reception and transmission.

#### Wakeup Period

The Wakeup Period field is set to the length, in units of beacon periods (superframes), between the start of successive wakeups of this node, and is effective from the next wakeup indicated in the preceding Wakeup Phase field.  It is set to 0 to encode a value of 256 beacon periods.  The beacon periods (superframes) in which the node will wakeup for frame reception and transmission as defined by this field and the preceding one are referred to as the node’s wakeup beacon periods (superframes)

The value of this field determines whether the IEs in this frame and the responding Connection Assignment frame denote 1-periodic or m-periodic allocations:

  - If Wakeup Period = 1, these IEs denote 1-periodic allocations.

  - If Wakeup Period ≠ 1, these IEs denote m-periodic allocations.

#### Explicit/Implicit Uplink Request IE

The Explicit/Implicit Uplink Request IE is either an Explicit Uplink Request IE or an Implicit Uplink Request IE defined as in 6.7.2 and 6.7.3, respectively.

#### Explicit/Implicit Downlink Request IE

The Explicit/Implicit Uplink Request IE is either an Explicit Downlink Request IE or an Implicit Downlink Request IE defined as in 6.7.4 and 6.7.5, respectively.

#### Explicit/Implicit Bilink Request IE

The Explicit/Implicit Bilink Request IE is either an Explicit Bilink Request IE or an Implicit Bilink Request IE defined as in 6.7.6 and 6.7.7, respectively.

#### Explicit/Implicit Delayed Bilink Request IE

The Explicit/Implicit Delayed Bilink Request IE is either an Explicit Delayed Bilink Request IE or an Implicit Delayed Bilink Request IE defined as in 6.7.8 and 6.7.9, respectively.

#### Group Connection IE

The Group Connection IE is defined as in 6.7.10.

### Connection Assignment

A Connection Assignment frame contains a Frame Payload that is formatted as shown in Figure 25.  It is transmitted by a hub to respond to a connection request or to change an earlier connection assignment.

Figure 25 — Frame Payload format for Connection Assignment frames

#### Recipient Address

The Recipient Address field is set to the IEEE MAC address of the recipient of the current frame.

#### Sender Address

The Sending Address field is set to the IEEE MAC address of the sender of the current frame.

#### Status Code

The Status Code field is set to the status of the connection assignment encoded according to Table 8.

Table 8 — Status Code field encoding

| Field value decimal | Status |
| --- | --- |
| 0 | Connection request accepted |
| 1 | Connection request rejected – invalid or unsupported frame format |
| 2 | Connection request rejected – no reason |
| 3 | Connection request rejected – no more channel bandwidth for a new connection |
| 4 | Connection request rejected – no more Connected_NID for a new connection |
| 5 | Connection request rejected – no more internal resources for a new connection |
| 6-10 | Reserved |
| 11 | Connection assignment modified |
| 12-255 | Reserved |

#### B+EAP1 Length

The B+EAP1 Length field is set to the combined length of beacon and exclusive access phase 1 (EAP1), in units of allocation slots, with EAP1 starting pSIFS after the beacon.

#### Minimum RAP1 Length

The Minimum RAP1 Length field is set to the least length guaranteed for random access phase 1 (RAP1), in units of allocation slots, with RAP1 starting immediately after EAP1.

#### EAP2 Start

The EAP2 Start field is set to the number of the allocation slot that starts exclusive access phase 2 (EAP2).

#### EAP2 Length

The EAP2 Length field is set to the length of exclusive access phase 2 (EAP2), in units of allocation slots.

#### MAC Capability

The MAC Capability is defined as in 6.6.1.

#### PHY Capability

The PHY Capability is defined as in 6.6.2.

#### NID

The NID field is set to an NID that is uniquely assigned or reassigned to the addressed node within the body area network (BAN) according to Table 12.

#### Connection Change Indicator

The Connection Change Indicator is defined as in 6.3.6.6.

#### Wakeup Phase

The Wakeup Phase field is defined as in 6.3.6.6.1.  The field in this frame supersedes the field previously exchanged between the sender and the recipient.

#### Wakeup Period

The Wakeup Period field is defined as in 6.3.6.6.2.  The field in this frame supersedes the field previously exchanged between the sender and the recipient.

#### Slotted Superframe IE

The Slotted Superframe IE is defined as in 6.7.1.

#### Uplink Assignment IE

The Uplink Assignment IE is defined as in 6.7.11.

#### Downlink Assignment IE

The Downlink Assignment IE is defined as in 6.7.12.

#### Bilink Assignment IE

The Bilink Assignment IE is defined as in 6.7.13.

#### Delayed Bilink Assignment IE

The Delayed Bilink Assignment IE is defined as in 6.7.14.

#### Group Connection IE

The Group Connection IE is defined as in 6.7.10.

#### Channel Order IE

The Channel Order IE is either a Nibble Encoded Channel Order IE or an Octet Encoded Channel Order IE as defined as in 6.7.15 and 6.7.17, respectively.

#### Channel Hopping IE

The Channel Hopping IE is defined as in 6.7.18.

### Multinode Connection Assignment

A Multinode Connection Assignment frame contains a Frame Payload that is formatted as shown in Figure 26.  It is optionally locally broadcast by a hub to respond to connection requests from multiple nodes or to change earlier connection assignments to multiple nodes.

Figure 26 — Frame Payload format for Multinode Connection Assignment frames

All the fields are defined as in 6.3.7 for Connection Assignment frames, except the Node Connection Assignment IE which is defined as in 6.7.15.

### Disconnection

A Disconnection frame contains a Frame Payload that is formatted as shown in Figure 27.  It is transmitted by a hub to repeal the connection with a node or by a node to repeal the connection with a hub.

(a) Sent by a node

(b) Sent by a hub

Figure 27 — Frame Payload format for Disconnection frames

#### Recipient Address

The Recipient Address field is set to the IEEE MAC address of the recipient of the current frame.

#### Sender Address

The Sending Address field is set to the IEEE MAC address of the sender of the current frame.

#### New Hub Address

The New Hub Address field in Disconnection frames sent by a node is set as follows:

  - If the node is not newly connected with another hub, it is null, i.e., not present.

  - Otherwise, it is set to the MAC address of the hub with which the node is newly connected.

The ith New hub Address field in Disconnection frames sent by a hub is set as follows:

  - If the hub does not have a suggested ith preferred new hub for the addressed node, it is null, i.e., not present.

  - Otherwise, it is set to the MAC address of the suggested ith preferred new hub for the addressed node.

### Command

A Command frame contains a Frame Payload that is formatted as shown in Figure 28.  It is optionally transmitted by a hub or node.

Figure 28 — Frame Payload format for Command frames

#### Recipient Address

The Recipient Address field is set to the IEEE MAC address of the recipient of the current frame.

#### Sender Address

The Sending Address field is set to the IEEE MAC address of the sender of the current frame.

#### Command ID

The Command ID field is set according to Table 9 such that it identifies the specific command of the current frame.

Table 9 — Command ID field encoding

| Field value decimal | Command name |
| --- | --- |
| 0 | Command – Coexistence Request |
| 1 | Command – Coexistence Response |
| 2 | Command – Battery Level Indication |
| 3 | Command – Battery Level Inquiry |
| 4-255 | Reserved |

#### Command Data

The Command Data field is specific to the command conveyed in the current frame.

#### (a)	Command – Coexistence Request

The Command Data field is formatted as shown in Figure 29 in a Command – Coexistence Request frame, which is optionally transmitted by a hub to another hub to request coexistence on the same physical channel.

Figure 29 — Command Data format for Command – Coexistence Request frames

#### (b)	Command – Coexistence Response

The Command Data field is formatted as shown in Figure 30 in a Command – Coexistence Response frame, which is optionally transmitted by a hub in response to another hub to provide coexistence information on the same physical channel.

Figure 30 — Command Data format for Command – Coexistence Response frames

#### (c)	Command – Battery Level Indication

The Command Data field is formatted as shown in Figure 31 in a Command – Battery Level Indication frame, which is optionally transmitted by a node unsolicited or in response to a command – battery level inquiry.

Figure 31 — Command Data format for Command – Battery Level Indication frames

##### Battery Power Remaining

The Battery Power Remaining field is set to p, 0 ≤  p ≤ 100,  if the battery power remaining at this sender is p%.  It is set to 255 if the battery power remaining is unknown.

##### Battery Time Remaining

The Battery Time Remaining field is set to t, 0 ≤  t ≤  253, if the battery time remaining at this sender assuming current medium access and data processing activities is t minutes .  It is set to 254 if the battery time remaining is equal to or more than 254 minutes.  It is set to 255 if the battery time remaining is unknown.

#### (d)	Command – Battery Level Inquiry

The Command Data field is not present in a Command – Battery Level Inquiry frame, which is optionally transmitted by a hub to make an inquiry about the battery level of the addressed node.

## Control type frames

A control type frame contains no frame payload or a frame payload of variable length.

### Immediate Acknowledgement (I-Ack)

An I-Ack frame contains no Frame Payload.  It is transmitted by a node or a hub to acknowledge receipt of the preceding frame.

### Block Acknowledgement (B-ACK)

A B-Ack frame selectively contains a Frame Payload that is formatted as shown in Figure 32.  It is transmitted by a node or a hub to acknowledge the reception status of certain preceding data type frames each containing a whole MSDU.

Figure 32 — Frame Payload format for B-Ack frames

#### Oldest Frame Expected

The Oldest Frame Expected field is set to the sequence number of the oldest frame that is of the same frame subtype as the frame preceding the current B-Ack frame and that is still expected to be, but not yet, received.

#### Next Block Size

The Next Block Size field is set to the maximum number of data type frames permitted in the next block transmission from the acknowledged node or hub to the acknowledging hub or node, where the next block transmission is a transmission of data type frames whose reception status will be provided in next B-Ack frame and whose frame subtype is the same as that of the data type frame preceding the current B-Ack frame.  The length of the frame preceding the current B-Ack frame was used to estimate the value of this field.

#### Frame Status Bitmap

The Frame Status Bitmap field is set as follows to indicate the reception status of the frames that are of the same frame subtype as the frame preceding the current B-Ack frame and that are subsequent in sequence to the frame indicated in the Oldest Frame Expected field:

  - The field comprises N/8 octets, where N is the number of these frames and f is the least integer that is not smaller than f.

  - The least-significant bit of these octets denotes the oldest of these frames, i.e., the frame that is immediately subsequent in sequence to the frame indicated in the Oldest Frame Expected field.

  - Each successive bit denotes a successive frame, i.e., a frame with a successive sequence number, of the same frame subtype.

  - A bit is set to 1 if the corresponding frame is received, or is set to 0 otherwise.

### Immediate Acknowledgement + Poll (I-Ack+Poll)

An I-Ack+Poll frame contains no Frame Payload.  It is transmitted by a hub to acknowledge receipt of the preceding frame and to send a poll to the addressed node.  The I-Ack+Poll frame is equivalent in function to an I-Ack frame followed by a Poll frame.

### Block Acknowledgement +Poll (B-ACK+Poll)

A B-Ack+Poll frame selectively contains a Frame Payload that is formatted as defined in 6.4.2.  It is transmitted by a hub to acknowledge the reception status of certain preceding data type frames and to send a poll to the addressed node.  The B-Ack+Poll frame is equivalent in function to a B-Ack frame followed by a Poll frame.

### Poll

A Poll frame contains no Frame Payload.  It is transmitted by a hub to grant to the addressed node an immediate polled allocation that starts pSIFS after the end of the frame or to inform the node of a future poll or post.

### Timed-Poll (T-Poll)

A T-Poll frame contains a Frame Payload that is formatted as shown in Figure 33.  It is transmitted by a hub to grant to the addressed node an immediate polled allocation that starts pSIFS after the end of the frame or to inform the node of a future poll or post, while providing the hub’s current time for the node’s clock synchronization.

Figure 33 — Frame Payload format for T-Poll frames

#### Current Superframe Number

The Current Superframe Number field is set to the sequence number of the current superframe (beacon period) as follows:

  - It is set to the sequence number of the beacon that is or was transmitted in the current beacon period.

  - It is set to the sequence number of the beacon that would otherwise have been or be transmitted in the current superframe if no beacon was or is to be transmitted in this superframe.

#### Current Allocation Slot Number

The Current Allocation Slot Number field is set to S such that the hub starts sending this frame during the allocation slot numbered S.

#### Current Slot Offset

The Current Slot Offset field is set to O in units of four microseconds such that the hub starts sending this frame at O after the start of the allocation slot indicated in the preceding field.

### Wakeup

A Wakeup frame selectively contains a Frame Payload that is formatted as shown in Figure 34.  It is unicast or multicast by a hub to inform some node(s) of the time (duration) for being at active state.  [JMH’s note:  I’ve defined this frame based on my understanding.  Also see the definition of Poll-Post Window for this frame.]

Figure 34 — Frame Payload format for Wakeup frames

Zero or multiple NID fields are present.  When present, each NID field is set as follows:

  - If the current frame is to keep only one node at active state for a specified duration as indicated in the Poll-Post Window of the current MAC header, it is unicast to the node, and no NID field is present.

  - If the current frame is to bring multiple nodes to active state at the end of a specified duration as indicated in the Poll-Post Window of the current MAC header, it is locally multicast, and multiple NID fields are present, with each set to the NID of one of those nodes.

### B2

A B2 frame contains a Frame Payload that is formatted as shown in Figure 35.  It is locally broadcast by a hub to announce a contention access phase (CAP) and provide group acknowledgment.   [JMH’s note:  using NID is more accurate than using slot number, since a frame sent by a node in but close to the start of slot m could be received by the hub as in slot m-1 due to mutual clock drifts.  Also see the definition of Poll-Post Window for this frame]

[

Figure 35 — Frame Payload format for B2 frames

One or more NID fields are present.  Each NID field is set to the NID of a node from which a frame requiring group acknowledgment was received since the last transmitted B2 frame.

## Data type frames

A data type frame contains a full, a fragmented, or no MSDU.

A data type frame of Emergency subtype is transmitted to indicate an emergency or medical event report.

A data type frame of a user-defined data subtype is transmitted such that all data type frames of the same data subtype contain MSDUs of the same application, or a group of MSDUs of the same attributes.

## MAC / PHY Capability fields

A MAC Capability field and a PHY Capability field are included in beacons and some other management type frames.  A hub and a node typically have different functional requirements for the capability of supporting a given function as listed in these fields, especially in the MAC Capability field.  Indication of being capable of supporting a function implies being willing to enable and perform the function as well.

### MAC Capability

The MAC Capability is formatted as shown in Figure 36.  [JMH’s note:  I’ve added more fields here.]

Figure 36 — MAC Capability format

##### CSMA/CA

The CSMA/CA field is set to 1 if the sender supports CSMA/CA in random access phase 1 (RAP1), random access phase 2 (RAP2), and contention access phase (CAP), or is set to 0 otherwise.

##### Slotted Aloha Access

The Slotted Aloha Access field is set to 1 if the sender supports slotted Aloha access in random access phase 1 (RAP1), random access phase 2 (RAP2), and contention access phase (CAP), or is set to 0 otherwise.

##### Type-I Polling Access

The Type-I Polling Access field is set to 1 if the sender supports Type-I polled allocations, or is set to 0 otherwise.

##### Type-II Polling Access

The Type-II Polling Access field is set to 1 if the sender supports Type-II polled allocations, or is set to 0 otherwise.

##### Scheduled Access

The Scheduled Access field is set to 1 if the sender supports scheduled allocations, or is set to 0 otherwise.  The sender supports scheduled-polling access if and only if it supports both scheduled allocations and Type-I or Type-II polled allocations.

##### Delayed-Polling Access

The Delayed-Polling Access field is set to 1 if the sender supports scheduled allocations that are subject to delay and Type-I or Type-II polled allocations, or is set to 0 otherwise.

##### G-Ack

The G-Ack field is set to 1 if the sender supports group acknowledgment, or is set to 0 otherwise.

##### L-Ack / B-Ack

The L-Ack / B-Ack field is set to 1 if the sender supports both L-Ack and B-Ack acknowledgment, or is set to 0 otherwise.

##### Group Connection

The Group Connection field is set to 1 if the sender supports group connection, or is set to 0 otherwise.

##### Multinode Connection Assignment

The Multinode Connection Assignment field is set to 1 if the sender supports multimode connection assignment, or is set to 0 otherwise.

##### Relaying Node

The Relaying Node field is set to 1 if the sender supports the functionality required of a relaying node in a two-hop extended star body area network (BAN), or is set to 0 otherwise.

##### Relayed Hub/Node

The Relayed Node field is set to 1 if the sender supports the functionality required of a relayed hub or node in a two-hop extended star body area network (BAN), or is set to 0 otherwise.

##### Always Active

The Always Active field is set to 1 if the sender will be always in active state ready to receive and transmit frames, or is set to 0 otherwise.

##### Battery Level Indication

The Battery Level Indication field is set to 1 if the sender supports battery level indication, or is set to 0 otherwise.

### PHY Capability

The PHY Capability is formatted as shown in Figure 37.

Figure 37 — PHY Capability format

##### Low Power Wakeup Radio

The Low Power Wakeup Radio field is set to 1 if the sender supports a low power wakeup radio, or is set to 0 otherwise.

##### Data Rate

One or more Data Rate fields are defined.  A Data Rate field is set to 1 if the sender supports the corresponding data rate for both transmission and reception in the operating frequency band according to Table 10, or is set to 0 otherwise.  [JMH’s note:  These fields need to be reconciled with the data rates specified in the PHY draft.]

Table 10 — Data Rate field representation

| Frequency band (MHz) | Data Rate 1 (kbps) | Data Rate 2 (kbps) | Data Rate 3 (kbps) | Data Rate 4 (kbps) | Data Rate 5 (kbps) | Data Rate 6 (kbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 402-405 |  |  |  |  |  |  |
| 902-928 |  |  |  |  |  |  |
| 863-870 950-956 |  |  |  |  |  |  |
| 2360-24xx |  |  |  |  |  |  |
| UWB |  |  |  |  |  |  |

## Information elements (Ies)

An IE is formatted as shown in Figure 38.  It is optionally contained in certain management type frames.

Figure 38 — IE format – general

The Element ID field is set to the value that identifies the information element according to Table 11.

The Length field is set to the length, in octets, of the IE-specific Information field that follows.

The Information field is set based on the Element ID as defined below.

Table 11 — Information elements defined in this standard

| Element ID decimal value | IE name | Description |
| --- | --- | --- |
| 0 | Slotted Superframe IE | Specifies superframe (beacon period) and allocation slot lengths |
| 1 | Explicit Uplink Request IE | Specifies time-based requirements by a node for scheduled uplink allocation(s) |
| 2 | Implicit Uplink Request IE | Specifies rate-based requirements by a node for scheduled uplink allocation(s) |
| 3 | Explicit Downlink Request IE | Specifies time-based requirements by a node for scheduled downlink allocation(s) |
| 4 | Implicit Downlink Request IE | Specifies rate-based requirements by a node for scheduled downlink allocation(s) |
| 5 | Explicit Bilink Request IE | Specifies time-based requirements by a node for scheduled bilink allocation(s) |
| 6 | Implicit Bilink Request IE | Specifies rate-based requirements by a node for scheduled bilink allocation(s) |
| 7 | Explicit Delayed Bilink Request IE | Specifies time-based requirements by a node for scheduled bilink allocation(s) that are subject to delay |
| 8 | Implicit Delayed Bilink Request IE | Specifies rate-based requirements by a node for scheduled delayed bilink allocation(s) that are subject to delay |
| 9 | Group Connection IE | Indicates a group connection request and the corresponding number of nodes included in the request |
| 10 | Reserved | Reserved |
| 11 | Uplink Assignment IE | Specifies scheduled uplink allocation(s) assigned to a node |
| 12 | Downlink Assignment IE | Specifies scheduled downlink allocation(s) assigned to a node |
| 13 | Bilink Assignment IE | Specifies scheduled bilink allocation(s) assigned to a node |
| 14 | Delayed Bilink Assignment IE | Specifies scheduled uplink allocation(s) assigned to a node that are subject to delay |
| 15 | Node Connection Assignment IE | Specifies connection assignment to a node |
| 16 | Nibble Encoded Channel Order IE | Specifies a list of 4-bit encoded channels in an operating band containing no more than 15 channels in the order of their selection by a hub |
| 17 | Octet Encoded Channel Order IE | Specifies a list of 8-bit encoded channels in an operating band containing more than 15 channels in the order of their selection by a hub |
| 18 | Channel Hopping IE | Specifies current channel dwell length and phase if channel hopping is enabled |
| 19-244 | Reserved | Reserved |
| 255 | Application Specific IE | Provides user-defined application-specific information |

### Slotted Superframe IE

A Slotted Superframe IE is formatted as shown in Figure 39.  It is optionally contained in Connection Assignment frames to indicate the superframe (beacon period) and allocation slot lengths.

Figure 39 — Slotted Superframe IE format

#### Superframe Length

The Superframe Length field is set to the length of the current superframe (beacon period) in units of allocation slots.  It is set to 0 to encode a value of 256 allocation slots.

#### Allocation Slot Length

The Allocation Slot Length field is set to L such that the length of an allocation slot is equal to pAllocationSlotMin  + L×pAllocationSlotResolution.

### Explicit Uplink Request IE

An Explicit Uplink Request IE is formatted as shown in Figure 40.  It is optionally contained in Connection Request frames to request, using time-based requirements, for creation or modification of one or more scheduled uplink allocations.

Figure 40 — Explicit Uplink Request, Explicit Downlink Request, or Explicit Bilink Request IE format

One or more Explicit Allocation Request fields are present.  Each Explicit Allocation Request is formatted as shown in Figure 41 to describe the time-based requirements of an allocation for servicing the data belonging to a given user priority.

Figure 41 — Explicit Allocation Request format

#### Allocation ID

The Allocation ID identifies an allocation requested by the node.  It is comprised of the Frame Subtype and User Priority fields as defined below.

#### Frame Subtype

The Frame Subtype field is set to the frame subtype of the data type frames to be transferred in this requested allocation.

#### User Priority

The User Priority field is set to the user priority of the frame payloads to be transferred in this requested allocation.

#### Maximum Gap

The Maximum Gap field is set to the largest length, in units of allocation slots, of the gap between any two adjacent allocation intervals of this requested allocation in the same beacon period (superframe) or across beacon periods (superframes), if the Wakeup Period field in the current frame has a value of 1.  It is reserved otherwise.

#### Minimum Gap

The Minimum Gap field is set to the smallest length, in units of allocation slots, of the gap between any two adjacent allocation intervals of this requested allocation in the same beacon period (superframe) or across beacon periods (superframes).

#### Minimum Length

The Minimum Length field is set to the smallest length, in units of allocation slots, of any of the allocation intervals of this requested allocation.

#### Allocation Length

The Allocation Length field is set to the overall length, in units of allocation slots, of the allocation intervals of this requested allocation in each wakeup beacon period (superframe) of this node.

### Implicit Uplink Request IE

An Implicit Uplink Request IE is formatted as shown in Figure 42.  It is optionally contained in Connection Request frames to request, using rate-based requirements, for creation or modification of one or more scheduled uplink allocations.

Figure 42 — Implicit Uplink Request, Implicit Downlink Request, or Implicit Bilink Request IE format

One or more Implicit Allocation Request fields are present.  Each Implicit Allocation Request is formatted as shown in Figure 43 to describe the rate-based requirements of an allocation for servicing the data belonging to a given user priority.   [JMH’s note:  I’ve refined the field names and populated the definitions.]

Figure 43 — Implicit Allocation Request format

#### Allocation ID

The Allocation ID identifies an allocation requested by the node.  It is comprised of the Frame Subtype and User Priority fields as defined below.

#### Frame Subtype

The Frame Subtype field is set to the frame subtype of the data type frames to be transferred in this requested allocation.

#### User Priority

The User Priority field is set to the user priority of the MAC service data units (MSDUs) to be transferred in this requested allocation.

#### Minimum MSDU Size

The Minimum MSDU Size field is set to the minimum size, in units of 16 octets, of the MSDUs to be transferred in this requested allocation.

#### MSDU Arrival Rate

The MSDU Arrival Rate field is set to the expected arrival rate of MSDUs, in units of MSDUs per second, to be transferred in this requested allocation.  It is set to 0 for an unspecified value.

#### MSDU Service Period

The Service Period field is set to the desired time interval, in units of 20 milliseconds, between successive transfers of the MSDUs in this requested allocation.  It is set to 0 for an unspecified value.

#### MSDU Error Rate

The MSDU Error Rate field is set to R such that the target reception error rate of MSDUs is equal to (100-R)%.

### Explicit Downlink Request IE

An Explicit Downlink Request IE is formatted as shown in Figure 40 in conjunction with Figure 41.  It is optionally contained in Connection Request frames to request, using time-based requirements, for creation or modification of one or more scheduled downlink allocations.

### Implicit Downlink Request IE

An Implicit Downlink Request IE is formatted as shown in Figure 42 in conjunction with Figure 43.  It is optionally contained in Connection Request frames to request, using rate-based requirements, for creation or modification of one or more scheduled downlink allocations.

### Explicit Bilink Request IE

An Explicit Bilink Request IE is formatted as shown in Figure 40 in conjunction with Figure 41.  It is optionally contained in Connection Request frames to request, using time-based requirements, for creation or modification of one or more scheduled bilink allocations.

### Implicit Bilink Request IE

An Implicit Bilink Request IE is formatted as shown in Figure 42 in conjunction with Figure 43.  It is optionally contained in Connection Request frames to request, using rate-based requirements, for creation or modification of one or more scheduled bilink allocations.

### Explicit Delayed Bilink Request IE

An Explicit Delayed Bilink Request IE is formatted as shown in Figure 44.  It is optionally contained in Connection Request frames to request, using time-based requirements, for creation or modification of one or more scheduled bilink allocations that are subject to delay.

Figure 44 — Explicit Delayed Bilink Request IE format

One or more Explicit Delayed Allocation Request fields are present.  Each Explicit Delayed Allocation Request is formatted as shown in Figure 45 to describe the time-based requirements of an allocation for servicing the data belonging to a given user priority.

Figure 45 — Explicit Delayed Allocation Request format

#### Allocation ID

The Allocation ID identifies an allocation requested by the node.  It is comprised of the Frame Subtype and User Priority fields as defined below.

#### Frame Subtype

The Frame Subtype field is set to the frame subtype of the data type frames to be transferred in this requested allocation.

#### User Priority

The User Priority field is set to the user priority of the frame payloads to be transferred in this requested allocation.

#### Maximum Gap

The Maximum Gap field is set to the largest length, in units of allocation slots, of the gap between any two adjacent allocation intervals of this requested allocation in the same beacon period (superframe) or across beacon periods (superframes), if the Wakeup Period field in the current frame has a value of 1.  It is reserved otherwise.

#### Minimum Gap

The Minimum Gap field is set to the smallest length, in units of allocation slots, of the gap between any two adjacent allocation intervals of this requested allocation in the same beacon period (superframe) or across beacon periods (superframes).

#### Minimum Length

The Minimum Length field is set to the smallest length, in units of allocation slots, of any of the allocation intervals of this requested allocation.

#### Allocation Length

The Allocation Length field is set to the overall length, in units of allocation slots, of the allocation intervals of this requested allocation in each wakeup beacon period (superframe) of this node.

#### Maximum Delay

The Maximum Delay field is set to D, in units of allocation slots, such that this node is willing to wait for a transmission addressed to it till D from the scheduled start of the allocation interval specified in the preceding two fields.

### Implicit Delayed Bilink Request IE

An Implicit Delayed Bilink Request IE is formatted as shown in Figure 46.  It is optionally contained in Connection Request frames to request, using rate-based requirements, for creation or modification of one or more scheduled bilink allocations that are subject to delay.

Figure 46 — Implicit Delayed Bilink Request IE format

One or more Implicit Delayed Allocation Request fields are present.  Each Implicit Delayed Allocation Request is formatted as shown in Figure 47 to describe the rate-based requirements of an allocation for servicing the data belonging to a given user priority and a corresponding maximum delay.

Figure 47 — Implicit Delayed Allocation Request format

#### Allocation ID

The Allocation ID identifies an allocation requested by the node.  It is comprised of the Frame Subtype and User Priority fields as defined below.

#### Frame Subtype

The Frame Subtype field is set to the frame subtype of the data type frames to be transferred in this requested allocation.

#### User Priority

The User Priority field is set to the user priority of the MAC service data units (MSDUs) to be transferred in this requested allocation.

#### Minimum MSDU Size

The Minimum MSDU Size field is set to the minimum size, in units of 16 octets, of the MSDUs to be transferred in this requested allocation.

#### MSDU Arrival Rate

The MSDU Arrival Rate field is set to the expected arrival rate of MSDUs, in units of MSDUs per second, to be transferred in this requested allocation.

#### MSDU Service Period

The Service Period field is set to the desired time interval, in units of 20 milliseconds, between successive transfers of the MSDUs in this requested allocation.

#### MSDU Error Rate

The MSDU Error Rate field is set to R such that the target reception error rate of MSDUs is equal to (100-R)%.

#### Maximum Delay

The Maximum Delay field is set to the maximum delay, in units of allocation slots, from the scheduled start of each allocation interval of this requested allocation within which this node is willing to wait for a transmission addressed to it.

### Group Connection IE

A Group Connection IE is formatted as shown in Figure 48.  It is optionally contained in Connection Request or Connection Assignment frames to indicate that the current connection request or assignment is made for a specified number of nodes.

Figure 48 — Group Connection IE format

The Group Size field is set to the number of nodes represented by the current Connection Request or Connection Assignment frame.

### Uplink Assignment IE

An Uplink Assignment IE is formatted as shown in Figure 49.  It is optionally contained in Connection Assignment frames to assign or reassign one or more scheduled uplink allocations to the addressed node.

Figure 49 — Uplink Assignment, Downlink Assignment, or Bilink Assignment IE format

One or more Allocation Assignment fields are present.  Each Allocation Assignment is formatted as shown in Figure 50 to specify an allocation interval of an assigned allocation for the data belonging to a given user priority.  One or more Allocation Assignment fields are included in this IE to specify an allocation.  Still more Allocation Assignment fields are included in the IE to specify additional allocations.

Figure 50 — Allocation Assignment format

#### Allocation ID

The Allocation ID identifies the allocation being assigned or reassigned to the node.  It is set to the Allocation ID used to identify the allocation requested earlier by the node.

#### Frame Subtype

The Frame Subtype field is set to the frame subtype of the data type frames to be transferred in this assigned allocation.

#### User Priority

The User Priority field is set to the user priority of the frame payloads to be transferred in this assigned allocation.

#### Interval Start

The Interval Start field is set to the number of the allocation slot that starts an allocation interval of this assigned allocation.

#### Interval End

The Interval End field is set to the number of the allocation slot that ends the allocation interval whose start time is specified in the preceding field.

### Downlink Assignment IE

An Uplink Assignment IE is formatted as shown in Figure 49 in conjunction with Figure 50.  It is optionally contained in Connection Assignment frames to assign one or more scheduled downlink allocations to the addressed node.

### Bilink Assignment IE

A Bilink Assignment IE is formatted as shown in Figure 49 in conjunction with Figure 50.  It is optionally contained in Connection Assignment frames to assign one or more scheduled Bilink allocations to the addressed node.

### Delayed Uplink Assignment IE

A Delayed Bilink Assignment IE is formatted as shown in Figure 51.  It is optionally contained in Connection Assignment frames to assign or reassign one or more scheduled bilink allocations that are subject to delay to the addressed node.

Figure 51 — Delayed Bilink Assignment IE format

One or more Delayed Allocation Assignment fields are present.  Each Delayed Allocation Assignment is formatted as shown in Figure 52 to specify an allocation interval of an assigned allocation for the data belonging to a given user priority and a corresponding maximum delay.  One or more Delayed Allocation Assignment fields are included in this IE to specify an allocation.  Still more Allocation Assignment fields are included in the IE to specify additional allocations.

Figure 52 — Delayed Allocation Assignment format

#### Allocation ID

The Allocation ID identifies the allocation being assigned or reassigned to the node.  It is set to the Allocation ID used to identify the allocation requested earlier by the node.

#### Frame Subtype

The Frame Subtype field is set to the frame subtype of the data type frames to be transferred in this assigned allocation.

#### User Priority

The User Priority field is set to the user priority of the frame payloads to be transferred in this assigned allocation.

#### Interval Start

The Interval Start field is set to the number of the allocation slot that starts an allocation interval of this assigned allocation.

#### Interval End

The Interval End field is set to the number of the allocation slot that ends the allocation interval whose start time is specified in the preceding field.

#### Maximum Delay

The Maximum Delay field is set to D, in units of allocation slots, such that this node is required to wait for a transmission addressed to it till D from the scheduled start of the allocation interval specified in the preceding two fields.

### Node Connection Assignment IE

A Node Connection Assignment IE is formatted as shown in Figure 53.  It is optionally contained in Multinode Connection Assignment frames to assign or reassign one or more scheduled allocations to the identified node.

Figure 53 — Node Connection Assignment IE format

#### Node Address

The Node Address field is set to the IEEE MAC address of the node to which the current IE applies.

#### NID

The NID field is set to an NID that is uniquely assigned or reassigned to the node within the body area network (BAN) according to Table 12.

#### Wakeup Phase

The Wakeup Phase field is defined as in 6.3.6.6.1.  The field in this frame supersedes the field previously exchanged between the sender and the node.

#### Wakeup Period

The Wakeup Period field is defined as in 6.3.6.6.2.  The field in this frame supersedes the field previously exchanged between the sender and the node.

#### Allocation Assignment

One or more Allocation Assignment fields are present.  Each Allocation Assignment is formatted as shown in Figure 50 to specify an allocation interval of an assigned allocation for the data belonging to a given user priority.  One or more Allocation Assignment fields are included in this IE to specify an allocation.  Still more Allocation Assignment fields are included in the IE to specify additional allocations.

### Nibble Encoded Channel Order IE

A Nibble Encoded Channel Order IE is formatted as shown in Figure 54.  It is optionally contained in Connection Assignment frames to indicate some or all channels in the operating band that has no more than 15 operating channels and the order in which those channels are to be selected as operating channels.

Figure 54 — Nibbled Encoded Channel Order IE and Octet Encoded Channel Order IE format

The Channel Ordered List field is set as follows:

  - The four least-significant bits are set to the channel number of the channel which will be the first candidate to each instance of channel selection.

  - Each successive four bits are set to the channel number of the channel which will be the next candidate to the instance of channel selection.

  - If the list conveys an odd number of channels, four bits with a binary value of 1111 are padded as the most-significant bits to the field.

### Octet Encoded Channel Order IE

An Octet Encoded Channel Order IE is formatted as shown in Figure 54.  It is optionally contained in Connection Assignment frames to indicate some or all channels in the operating band that has more than 15 operating channels and the order in which those channels are to be selected as operating channels.

The Channel Ordered List field is set as follows:

  - The eight least-significant bits are set to the channel number of the channel which will be the first candidate to each instance of channel selection.

  - Each successive eight bits are set to the channel number of the channel which will be the next candidate to the instance of channel selection.

### Channel Hopping IE

A Channel Hopping IE is formatted as shown in Figure 55.  It is optionally contained in Connection Assignment frames to enable channel hopping.  [JMH’s note:  Since all coexistence mechanisms are optional, I had to place the channel dwell length/phase parameters into an IE.]

Figure 55 — Channel Hopping IE format

#### Channel Dwell Length

The Channel Dwell Length field is set to the length of the time, in units of beacon periods (superframes), over which the hub sending the current frame is to dwell at any chosen operating channel before hopping to another one.  It is set to 0 to encode a value of 256 beacon periods (superframes).

#### Channel Dwell Phase

The Channel Dwell Phase field is set to H if the hub sending the current frame is to hop to another channel according to its channel hopping sequence at the end of the next Hth beacon period (superframe) not counting the current beacon period (superframe) if H > 0 or at the end of the current beacon period (superframe) if H = 0.

### Application Specific IE

An Application Specific IE is formatted as shown in Figure 56.  It is optionally contained in some management type frames to convey application-specific information.

Figure 56 — Application Specific IE format

#### OUI-24

The OUI-24l field is set to the 24-bit Organizationally Unique Identifier (OUI) assigned by the IEEE Registration Authority to the vendor or manufacturer that defines this IE.

#### Application Specific Data

The Application Specific Data field is set by the owner (“assignee” in IEEE terms) of the OUI.

# MAC functions

This clause specifies MAC sublayer functionality. It starts with the ground rules for preparing frame transmission and performing frame reception in 7.1.  The next subclause 7.2 is an access umbrella encompassing the access modes and structures provided in this standard for medium access.  With these overall preparations, medium access starts in earnest in 7.3 with the creation of a body area network (BAN) by a hub and the unconnected exchange (“connection setup”) between unconnected nodes and the hub.  The following three subclauses 7.4-7.6 specify the operation of a variety of selectable access schemes for obtaining allocation intervals of preferred attributes. Additional access steps are given in 7.7 for communication in the tightly regulated medical implant communications service (MICS) band.  Two-hop access extension of star BAN topology is optionally made available in .

Supplemental MAC functions follow in the remaining subclauses. Clock synchronization and guardtime provisioning are addressed in 7.9.  Power management is expounded in 7.10.  Coexistence support and interference mitigation are optionally offered in 7.11.  MAC / PHY optional capability handling and Application Specific IE usage are elucidated in 7.12.  MAC sublayer parameters are listed in 7.13.

## Frame processing

This subclause provides fundamental rules on preparing MAC frames for transmission and processing them on reception.

### Abbreviated addressing

A hub shall select a one-octet body area network identifier (BAN ID) from an integer between x00 and xFF, inclusive, as its BAN’s abbreviated address contained in the MAC header of all frames sent from or to the hub.  The hub should select a BAN ID that is currently not being used by neighbor BANs.

A husb shall select a one-octet hub identifier (HID) from an integer in the Connected_NID subset as specified in Table 12 as its abbreviated address contained in the MAC header of all frames sent from or to the hub.  The hub shall not select an HID equal to a Connect_NID that is currently being used for a node connected with it.  The hub should select an HID that is currently not being used by neighbor hubs.

A one-octet node identifier (NID) selected in accordance with Table 12 shall be used as a node’s abbreviated address contained in the MAC header of all frames sent (unicast) from or to the node, or shall be used as a group of nodes’ common abbreviated address contained in the MAC header of all frames sent (multicast or broadcast) to the nodes by a hub.

Table 12 — NID selection

| NID value in hex | NID subtotal | NID notation | NID usage |
| --- | --- | --- | --- |
| x00 | 1 | Unconnected_Broadcast_NID | For broadcast to unconnected nodes |
| x01 | 1 | Unconnected_NID | For unicast from/to unconnected nodes in a BAN |
| x02-F5 | 244 | Connected_NID | For unicast from/to connected nodes in a BAN |
| xF6 | 1 | Reserved | Reserved |
| xF7-FD | 7 | Multicast_NID | For multicast to connected nodes in a BAN |
| xFE | 1 | Local_Broadcast_NID | For broadcast to all nodes in a BAN |
| xFF | 1 | Broadcast_NID | For broadcast to all nodes in all BANs |

An unconnected node without an assigned Connected_NID shall choose the Unconnected_NID as its NID in its first (management type) frame, and retries thereof, sent to a hub before being connected with the hub.

Upon receiving the first frame from an unconnected node, a hub shall assign as the node’s NID a Connected_NID that is not being used for itself or another node in its BAN, and shall use that Connected_NID as the Recipient ID of the MAC header in its I-Ack and all subsequent frames sent to the node.

The unconnected node shall be ready to receive frames from the hub, without regard to the Recipient ID field of the MAC header, before finding its assigned Connected_NID.  The node shall use as its NID in all subsequent exchanges with the hub the Connected_NID of the Recipient ID field in the MAC header of the I-Ack frame or next (management type) frame with the frame payload containing a Recipient Address field set to its IEEE MAC address.  The node may retry its first frame after failing to receive the expected first management type frame from the hub.

If the node receives a frame from the hub with the Recipient ID in the MAC header set to its current Connected_NID but with the Recipient Address in the frame payload not set to its IEEE MAC address, the node shall consider itself to have disconnected with the hub and lost its Connected_NID accordingly.  It may employ the above procedure again to obtain its Connected_NID.

### Full addressing

A separate IEEE MAC address shall be used to uniquely identify a sender or a recipient when desired.  In particular, the IEEE MAC address of a hub sending a beacon is included in the MAC frame payload of the beacon.  The IEEE MAC addresses of both the sender and the recipient of other management type frames are included in the MAC frame payload of those frames.

Exceptionally, an unconnected node may set to 0 the Recipient Address field of the first management type frame it sends to a hub, if the node has not yet known the IEEE MAC address of the hub.

A recipient shall check the Recipient Address and Sender Address fields of the MAC frame payload of a received management type frame to determine if the frame was indeed addressed to it from an expected sender, taking the aforementioned exception into account.

### Priority mapping

User priority values, when referenced in prioritizing medium access of data and management type frames, shall be determined based on the designation of frame payloads (traffic) contained in the frames according to Table 13.  [MAC group’s note:  The mapping in the table is subject to revision.]

Table 13 — User priority mapping

| Priority | User Priority | Traffic designation | Frame type |
| --- | --- | --- | --- |
| Lowest Highest | 0 | Background (BK) | Data |
|  | 1 | Best effort (BE) | Data |
|  | 2 | Excellent effort (EE) | Data |
|  | 3 | Controlled load (CL) | Data |
|  | 4 | Video (VI) | Data |
|  | 5 | Voice (VO) | Data |
|  | 6 | Medical data or network control | Management |
|  | 7 | Emergency or medical event report | Data |

### Frame reception

A node shall receive a frame if the following conditions are met:

  - The Recipient ID field of the MAC header of the frame is set to its own NID, any applicable multicast_NID, the Local_Broadcast_NID, or the Broadcast_NID.

  - The Sender ID field of the MAC header of the frame is set to the HID of the desired hub with which to exchange frames.

  - The BAN ID field of the MAC header of the frame is set to an expected value.

  - The Protocol Version of the MAC header of the frame is set to a value it supports.

  - The FCS of the frame is valid, i.e., equal to the FCS value it calculates over the applicable fields received.

A hub shall receive a frame if the following conditions are met:

  - The Recipient ID field of the MAC header of the frame is set to its own HID.

  - The Sender ID field of the MAC header of the frame is set to the NID of an expected sender or the Unconnected_NID.

  - The BAN ID field of the MAC header of the frame is set to an expected value.

  - The Protocol Version of the MAC header of the frame is set to a value it supports.

  - The FCS of the frame is valid.

The node or the hub shall ignore a received frame, aside from performing applicable acknowledgment, whose MAC frame payload has a Sender Address field that is not set to the IEEE MAC address of the expected sender or whose MAC frame payload has a Recipient Address field that is not set to its own IEEE MAC address.

The node or the hub shall ignore a received frame, aside from performing applicable acknowledgment, that is detected to be a duplicate as described in 7.1.8.

### Frame transfer

A sender shall transmit MAC service data units (MSDUs) contained in data type frames of the same frame subtype and addressed to the same recipient(s) in the octet order in which they arrived at the local MAC service access point (SAP).

When fragmenting an MSDU, the sender shall extract the first fragment, the second fragment, and so on, in sequential octet order.  The sender shall transmit the first fragment, then the second fragment, and so on, accordingly.

A sender may transmit an MSDU earlier than another MSDU, even if the former arrived at the local MAC SAP later than the latter, so long as the two MSDUs are contained in data type frames not of the same frame subtype or  not addressed to the same recipient(s).

A recipient shall release to the MAC client MSDUs that were transmitted by the same sender and contained in data type frames of the same frame subtype in the octet order in which they were received.

### Frame retry

A node or a hub may retry a frame, i.e., may transmit a frame that was previously transmitted but not necessarily received, to the same recipient(s), as appropriate, taking into consideration such factors as delay requirements, fairness policies, channel conditions, and medium availability.

### Frame acknowledgement

A sender shall set the Ack Policy field of the MAC header of a frame to be transmitted according to Table 1 and Table 14.  A recipient shall acknowledge a received frame if the criteria in 7.1.3 for qualifying a frame as received are met and if required by the acknowledgment policy set in the frame as further described below.

Table 14 — Acknowledgment (Ack) Policy field setting

| Frame Type name | Frame Subtype name | Ack Policy field |
| --- | --- | --- |
| Management | Beacon | N-Ack |
| Management | Security Association | I-Ack |
| Management | Security Disassociation | I-Ack |
| Management | PTK | I-Ack |
| Management | GTK | I-Ack |
| Management | Connection Request | I-Ack |
| Management | Connection Assignment | I-Ack |
| Management | Multinode Connection Assignment | N-Ack |
| Management | Disconnection | I-Ack |
| Management | Command | I-Ack |
| Control | I-Ack | N-Ack |
| Control | B-Ack | N-Ack |
| Control | I-Ack+Poll | N-Ack |
| Control | B-Ack+Poll | N-Ack |
| Control | Poll | I-Ack or N-Ack |
| Control | T-Poll | I-Ack or N-Ack |
| Control | Wakeup | I-Ack for unicast N-Ack for multicast |
| Control | B2 | N-Ack |
| Data | Data subtype set to mG-AckDataSubtype | G-Ack |
| Data | User defined data subtype other than mG-AckDataSubtype | N-Ack, I-Ack, L-Ack, or B-Ack |

#### No Acknowledgment (N-Ack)

A sender may send a frame with the Ack Policy field set to N-Ack to require no acknowledgment at all, as permitted by Table 14.

A recipient shall not acknowledge a received frame containing an Ack Policy field set to N-Ack, either immediately or later.

The sender may retry a data type frame with the Ack Policy field set to N-Ack, as appropriate.

#### Group Acknowledgment (G-Ack)

A node, but not a hub, may send to a hub a frame with the Ack Policy field set to G-Ack if the following two conditions are satisfied:

  - The frame is a data type frame of mG-AckDataSubtype.

  - The hub supports G-Ack as indicated in the latter’s MAC Capability field.

A hub shall acknowledge data type frames with the Ack Policy field set to G-Ack and the Frame Subtype field set to mG-AckDataSubtype that were received since its last B2 transmission by including the NIDs of the nodes from which those frames were received in the frame payload of the next B2 frame.  The hub should send such a B2 frame as soon as permitted.

The node may retry the frame requiring G-Ack upon failing to receive the expected B2 frame or the expected acknowledgment in the next received B2 frame.

An example for group acknowledgment (G-Ack) is shown in Figure 57.

Figure 57 — Group acknowledgment (G-Ack) illustration

#### Immediate Acknowledgment (I-Ack)

A sender may send a frame with the Ack Policy field set to I-Ack to require an immediate acknowledgment.

A recipient shall acknowledge a received frame with the Ack Policy field set to I-Ack by sending an I-Ack frame back pSIFS after the end of the received frame.

Examples for immediate acknowledgment are shown in Figure 58.

Figure 58 — Immediate acknowledgment (I-Ack) illustration

#### Block Acknowledgment Later (L-Ack) and Block Acknowledgment (B-Ack)

A source—a node or a hub—may send a frame with the Ack Policy field set to B-Ack if the following two conditions are satisfied:

  - The frame contains a whole MSDU.

  - The recipient supports L-Ack / B-Ack as indicated in the latter’s MAC Capability field.

The recipient shall acknowledge a received frame with the Ack Policy field set to B-Ack by sending a B-Ack frame back pSIFS after the end of the received frame.  The B-Ack frame shall contain a frame payload as defined in 6.4.2 unless the following two conditions are both true:

  - No older frames of the same frame subtype as the last received frame are still expected to be received.

  - Only one frame in the next block transmission is allowed.

A block transmission starts from the first frame sent after the last B-Ack frame received and ends with the last frame sent before receiving next B-Ack frame.

The source node may send a frame with the Ack Policy field set to L-Ack if the following conditions are all satisfied:

  - The frame contains a whole MSDU.

  - It has sent a frame of the same frame subtype with the Ack Policy field set to B-Ack and received a B-Ack frame acknowledging that frame and containing a frame payload.

  - That B-Ack frame was the last B-Ack frame received from the recipient.

The source shall not transmit more frames in a block transmission than allowed as specified in the last B-Ack frame received.  These frames should not be longer than the frame sent in the last block transmission.  The source shall end a block transmission with a frame with the Ack Policy field set to B-Ack.

The source shall send frames in a block transmission in the order of increasing sequence number values, which are not necessarily consecutive if the block transmission contains retransmitted frames, considering that sequence number wraparound is also increasing the sequence number value.  The source shall not retransmit frames that are older than the frame indicated in the Oldest Frame Expected field of the last B-Ack frame received.  It should retransmit frames, starting with the oldest frame expected, that were not received as indicated in the Frame Status Bitmap field of that B-Ack frame.

The source, once starting a block transmission, shall not transmit frames of another frame type or subtype until it has finished the block transmission.

The recipient shall not acknowledge immediately a received frame with the Ack Policy field set to L-Ack.  Rather, it shall indicate the reception status of the frames newer than the oldest frame still expected through the B-Ack frame it returns at the end of the current block transmission.

The source may retransmit in an appropriate time the last frame which had the Ack Policy field set to B-Ack after failing to receive the expected B-Ack frame.

The recipient may implement a timeout that enables it to stop waiting for missing old frames if appropriate, hence allowing new MSDUs to be released to the MAC client and some B-Ack buffer resources to be freed without receiving those missing frames.

Examples for late acknowledgment (L-Ack) and block acknowledgment (B-Ack) are shown in Figure 59.

Figure 59 — Late acknowledgment (L-Ack) and block acknowledgment (B-Ack) illustration

### Duplicate detection

A received frame shall be treated by the recipient as a duplicate of the last management or data type frame received if the Retry bit in its MAC header is set to 1 and the two frames have identical values in the following fields of their MAC header: Recipient ID, Sender ID, BAN ID, Protocol Version, Security Level, Frame Type, Frame Subtype, Sequence Number, and Fragment Number.

### Fragmentation and reassembly

A sender may fragment only MAC service data units (MSDUs) to be transferred in data type frames with the Ack Policy field set to N-Ack, G-Ack, or I-Ack.

The sender shall not fragment any MSDU to more than mMaxFragmentCount fragments. All fragments, except the last one, of the same MSDU shall have the same length.  The sender shall not alter the length of the fragments of an MSDU, by refragmentation or recombination, in retransmitting them.

The sender shall not transmit any other fragments of an MSDU after discarding one fragment of the MSDU.

The sender shall set to 0 the Fragment Number field in the data type frame containing the first fragment of an MSDU.  It shall set the field in the data type frame containing each subsequent fragment of the same MSDU to one plus the Fragment Number value in the data type frame containing the previous fragment.  The sender shall not alter the Fragment Number field in a data type frame in retransmitting the frame.

The sender shall set the Sequence Number field in all data type frames containing the fragments of the same MSDU to the same value.

A recipient shall completely reassemble an MSDU in the correct order before delivery to the MAC client.  The recipient shall discard any MSDU with missing fragments.

## Access classification and division

To provide or support time referenced allocations in its body area network (BAN), a hub shall establish a time base as specified in 5.3 which divides the time axis into beacon periods (superframes) regardless of whether it is to transmit beacons.  In such cases, the hub shall transmit a beacon in each beacon period, except in inactive superframes, unless prohibited by regulations such as imposed in MICS band.  The hub may shift (rotate) its beacon transmission time, as specified in 7.11.1, from one offset from the start of current beacon period to another offset from the start of next beacon period, thereby shifting the time reference for all scheduled allocations, to prevent large-scale repeated transmission collisions between its BAN and neighbor BANs.

In cases where a hub is not to provide or support time referenced allocations in its BAN, it may operate without establishing a time base nor superframe boundaries and hence without transmitting beacons at all.

Equivalently, a hub shall operate in beacon mode transmitting a beacon in every beacon period other than in inactive superframes to enable time referenced allocations, unless regulations as applicable in the MICS band disallow beacon transmission.  In the latter case, a hub shall operate in non-beacon mode transmitting no beacons, with superframe and allocation slot boundaries established if access to the medium in its BAN involves time referencing, or without superframe or allocation slot boundaries if access to the medium in its BAN involves no time referencing.

In summary, a hub shall operate in one of the following three access modes:

  - beacon mode with beacon period superframe boundaries;

  - non-beacon mode with superframe boundaries;

  - non-beacon mode without superframe boundaries.

### Beacon mode with beacon period boundaries

In this mode, a hub shall divide each active beacon period into applicable access phases as illustrated in Figure 60.  The hub may announce some superframes (beacon periods) as inactive superframes where it transmits no beacons and provides no access phases, if there are no allocation intervals scheduled in those superframes.

Figure 60 — Layout of access phases in a beacon period (superframe) for beacon mode

The hub shall place the access phases – exclusive access phase 1 (EAP1), random access phase 1 (RAP1), type-I/II access phase, exclusive access phase 2 (EAP2), random access phase 2 (RAP2), type-I/II access phase, and contention access phase – in the order stated and shown above. The hub may set to zero the length of any of these access phases, but shall not have RAP1 shorter than the guaranteed minimum length communicated in Connection Assignment frames sent to nodes that are still connected with it.  To provide a non-zero length CAP, the hub shall transmit a preceding B2 frame.

A type-I/II access phase is either a type-I or type-II access phase as described below but not both.  The two type-I/II access phases may be both of type-I, both of type-II, or one of type-I and the other of type-II.  If one is of type-I and the other is of type-II, either one may appear before the other.

#### Type-I access phase

The hub may schedule uplink allocation intervals, downlink allocation intervals, and bilink allocation intervals anywhere in a type-I access phase.  It may improvise type-I and type-II polled allocation intervals as well as posted allocation intervals anywhere outside the scheduled allocation intervals in the type-I access phase.  It may also provide type-I and type-II polled allocation intervals within scheduled bilink allocation intervals in a type-I access phase.

As defined in 6.2.1.1.10 and 6.2.1.1.10,

  - a type-I polled allocation is conveyed in terms of its time duration (the maximum time the polled node may use for its frame transactions in the allocation);

  - a type-II polled allocation is conveyed in terms of a frame count (the maximum number of frames the polled node may transmit in the allocation).

These allocation intervals along with the corresponding access methods by which they are obtained are illustrated in Figure 61.

Figure 61 — Allocation intervals and access methods permitted in a type-I access phase

#### Type-II access phase

The hub may schedule bilink allocation intervals and delayed bilink allocation intervals anywhere in a type-II access phase, except that it shall not schedule any bilink allocation intervals after delayed bilink allocation intervals in the same type-II access phase.  It may improvise type-II, but not type-I, polled allocation intervals as well as posted allocation intervals anywhere outside the scheduled bilink allocation intervals and delayed bilink allocation intervals in the type-II access phase.  It may also provide type-II, but not type-I, polled allocation intervals within scheduled bilink allocation intervals and delayed bilink allocation intervals.  These allocation intervals along with the corresponding access methods by which they are obtained are illustrated in Figure 62.

Figure 62 — Allocation intervals and access methods permitted in a type-II access phase

### Non-beacon mode with superframe boundaries

In this mode, a hub may have only a type-I or type-II access phase as described in 7.2.1 in any superframe (beacon period), but not both or any other access phases, as illustrated in Figure 63.

Figure 63 — Layout of access phases in a superframe (beacon period) for non-beacon mode

### Non-beacon mode without superframe boundaries

In this mode, a hub may provide only unscheduled type-II polled allocations, as illustrated in Figure 64.

Figure 64 — Allocation intervals and access methods permitted for non-beacon mode          without superframe (beacon period) boundaries

## BAN creation and connection setup

A hub shall choose an operating channel to start a body area network (BAN), taking into account policy regulations, channel conditions, application requirements, coexistence considerations, etc.  The hub shall choose a new channel when required by regulations as applicable in the medical implant communications service (MICS) band.  The hub may hop to new channels periodically, as specified in 7.11.2, to effect frequency diversity and interference mitigation.

The hub shall choose and enable an applicable access mode as described in 7.2 to support desired access methods.

If the hub selected non-beacon mode without superframe boundaries, it shall transmit Poll frames each addressed to Unconnected_Broadcast_NID and providing an immediate type-I polled allocation to enable unconnected nodes’ connection or reconnection with it.

If the hub selected non-beacon mode with superframe boundaries, it shall transmit T-Poll frames each addressed to Unconnected_Broadcast_NID and providing an immediate type-I or type-II polled allocation to enable unconnected nodes’ connection or reconnection with it.

If the hub selected beacon mode with superframe boundaries, it may transmit T-Poll frames each addressed to Unconnected_Broadcast_NID and providing an immediate type-I or type-II polled allocation to facilitate unconnected nodes’ connection or reconnection with it.

Within the scope of this subclause, an unconnected polled allocation is a type-I or type-II polled allocation granted  by a hub via a Poll or T-Poll frame addressed to Unconnected_Broadcast_NID, and an unconnected node is a node without a scheduled uplink allocation, a scheduled bilink allocation, or a delayed bilink allocation, in the BAN identified in the MAC header of those Poll or T-Poll frames.

An unconnected node may send a management type frame to the hub in an unconnected polled allocation with contention probability P = 1/2.  If the node does not receive an expected acknowledgment, it may retry it in another unconnected polled allocation with contention probability P = max(1/8, (1/2) / (R+1)/2), where R counts the retries of the frame, i.e., R equals 1 for the first retry of the frame, 2 for the second retry, and so on.  The function x is defined to be the least integer not smaller than x.  With contention probability P, the node shall transmit if z ≤ P, where z is a value the node has newly drawn at random from the interval [0, 1].

The hub shall not provide unconnected polled allocations with a time duration shorter than pUnconnectedPolledAllocationMin, which is the time required for sending the longest management type frame containing no information elements (IEs) and returning an immediate acknowledgment at a mandatory data rate, plus pSIFS and the appropriate guard time.

The unconnected node may disregard the indicated time duration of a type-I polled allocation in sending a management type frame containing no IEs, before receiving a Connection Assignment frame with a Slotted Superframe IE providing the necessary time information for determining such a time duration.  The unconnected node may later send another management type frame of the same frame subtype, such as Connection Request, with updated information (e.g., on wakeup phase, wakeup period, and Explicit Uplink Request IEs), after finding the beacon period (superframe) and allocation slot lengths.

After the hub receives and acknowledges the first management type frame sent from an unconnected node, it should provide polled allocations specifically addressed (unicast) to this node to facilitate the transmission of the unconnected node.  The unconnected node should stay in active state until it does not need polled or posted allocations that are not announced in advance.

Figure 65 illustrates how to instantiate or obtain unconnected and unicast polled allocations for or by an unconnected node.

Figure 65 — Unconnected and unicast polled allocations for unconnected nodes

The node may also use CSMA/CA or slotted Aloha access as described in 7.4, if capable, to transmit its frames before and after its connection with the hub.

The hub may locally broadcast a Multinode Assignment frame granting scheduled allocations to multiple nodes that are capable of handling multimode connection assignment as indicated in their last exchanged MAC Capability field.

## Random access (connectionless contention-based access)

In exclusive access phase 1(EAP1), random access phase 1 (RAP1), exclusive access phase 2(EAP2), random access phase 2 (RAP2), and contention access phase (CAP), as depicted in 7.2, allocations may only be contended allocations, which are non-reoccurring time intervals valid per instance of access.  The access method for obtaining the contended allocations shall be

  - CSMA/CA as specified in 7.4.1 if pRandonAccess is set to CSMA/CA, or

  - slotted Aloha access as specified in 7.4.2 if pRandonAccess is set to Slotted Aloha.

[JMH’s note:  pRandomAccess for narrowband PHY is set to CSMA/CA, and for UWB is set to Slotted Aloha.]

A hub or nodes may obtain contended allocations in EAP1 and EAP1, if it needs to send data type frames of the highest user priority (i.e., containing an emergency or medical event report) as defined in Table 13.  The hub may obtain such a contended allocation pSIFS after the start of EAP1 or EAP2 without actually performing the CSMA/CA or slotted Aloha access procedure.  Only nodes may obtain contended allocations in RAP1, RAP2, and CAP, to send management or data type frames.

To send data type frames of the highest user priority based on CSMA/CA, a hub or node may treat the combined EAP1 and RAP1 as a single EAP1, and the combined EAP2 and RAP2 as a single EAP2, so as to allow continual invocation of CSMA/CA and improve channel utilization.  To send data type frames of the highest user priority based on slotted Aloha access, a hub or node may treat RAP1 as another EAP1 but not a continuation of EAP1, and RAP2 as another EAP2 but not a continuation of EAP2, due to the time slotted attribute of slotted Aloha access.

Prioritized access for traffic of differing user priorities (UPs) shall be attained through the predefined relationships in Table 13 and Table 15 between contention window (CW) bounds CWmax and CWmin and UP for CSMA/CA and between contention probability (CP) thresholds CPmax and CPmin and UP for slotted Aloha access.

Table 15 — Contention window bounds for CSMA/CA                                                                                        and contention probability thresholds for slotted Aloha access

| User Priority | CSMA/CA |  | Slotted Aloha access |  |
| --- | --- | --- | --- | --- |
|  | CWmin | CWmax | CPmax | CPmin |
| 0 | 16 | 64 | 1/8 | 1/16 |
| 1 | 16 | 32 | 1/8 | 3/32 |
| 2 | 8 | 32 | 1/4 | 3/32 |
| 3 | 8 | 16 | 1/4 | 1/8 |
| 4 | 4 | 16 | 3/8 | 1/8 |
| 5 | 4 | 8 | 3/8 | 3/16 |
| 6 | 2 | 8 | 1/2 | 3/16 |
| 7 | 1 | 4 | 1 | 1/4 |

### CSMA/CA

To employ CSMA/CA, a node shall maintain a backoff counter and contention window to determine when it obtains a new contended allocation as described below, and shall initialize the backoff counter to 0.

#### Starting a contended allocation

To obtain a new contended allocation, a node shall set its backoff counter to a sample of an integer random variable uniformly distributed over the interval [1, CW], when its backoff counter has a value of 0 and the node has at least one frame of user priority UP to transmit or retransmit, where CW is a contention window chosen as follows.

  - If the node did not obtain any contended allocation previously, it shall set the CW to CWmin[UP].

  - If the node succeeded, i.e., if the node received an expected acknowledgment, I-Ack or B-Ack frame, to its last frame transmission, in the last contended allocation it had obtained, it shall set the CW to CWmin[UP] as well.

  - If the node transmitted a frame requiring no acknowledgment or group acknowledgment at the end of its last contended allocation, it shall keep the CW unchanged.

  - If the node failed, i.e., if the node did not receive an expected acknowledgment to its last frame transmission, in the last contended allocation it had obtained,

    - it shall keep the CW unchanged if this was the mth time the node had failed consecutively, where m is an odd number;

    - it shall double the CW if this was the nth time the node had failed consecutively, where n is an even number.

  - If doubling the CW would have the new CW exceed CWmax[UP], the node shall set the CW to CWmax[UP].

The node shall lock the backoff counter when any of the following events occurs:

  - The backoff counter is reset upon decrementing to 0.

  - The channel is busy.  If the channel is busy because the node detected a frame transmission, the channel remains busy until at least the end of the frame transmission without the node having to re-sense the channel.

  - The current time is outside any RAP or CAP if UP does not have the highest value (i.e., not for an emergency or medical event report) according to Table 13, or is outside any EAP, RAP, or CAP if UP has the highest value (i.e., for an emergency or medical event report).

  - The current time is at the start of a CSMA slot within an EAP, RAP, or CAP, but the time between the end of the slot and the end of the EAP, RAP, or CAP is not long enough for completing a frame transaction and setting aside a nominal guardtime mGT_Nominal.

The node shall unlock the backoff counter when both of the following conditions are met:

  - The channel has been idle for pSIFS within an RAP or CAP if UP does not have the highest value (i.e., not for an emergency or medical event report) according to Table 13, or within an EAP, RAP, or CAP if UP has the highest value (i.e., for an emergency or medical event report).

  - The time duration between the current time plus a CSMA slot and the end of the EAP, RAP, or CAP is long enough for completing a frame transaction plus a nominal guardtime mGT_Nominal.

Upon unlocking its backoff counter, the node shall decrement its backoff counter by one for each idle CSMA slot that follows, as shown in Figure 66.  In particular, the node shall treat a CSMA slot to be idle if it determines that the channel has been idle between the start of the CSMA slot and pCCATime later, decrementing the backoff counter effectively pCCATime after the start of the CSMA slot, so that the node will transmit a frame to the transport medium (i.e., air) at the end of the CSMA slot in case its backoff counter reaches 0, as further described below.  Each CSMA slot shall have a fixed duration of pCSMASlotLength.

Figure 66 — CSMA slot structure

If the backoff counter reaches 0 in the current CSMA slot, the node shall have obtained a contended allocation that starts at the end of the current CSMA slot with a maximum length of mCSMADurationLimit.  Figure 67 illustrates how to start and use contended allocations based on CSMA/CA.

Figure 67 — CSMA/CA illustration

#### Using a contended allocation

A node may transmit a frame for which it obtained the contended allocation at the start of the allocation.  The recipient shall transmit an acknowledgment frame, I-Ack or B-Ack, when required, pSIFS after the end of the previous frame.

To resume its transmission following a received acknowledgment frame, the node may transmit a new frame or retransmit an old frame, with an UP not lower than the UP used to obtain the current contended allocation, pSIFS after the end of the acknowledgment frame.  The node shall end its transmission in the allocation such that the last transmission in the allocation completes at least the nominal guardtime GTn = mGT_Nominal earlier than the end of the allocation or of the current EAP, RAP, or CAP, whichever is earlier.

#### Modifying a contended allocation

The node shall not modify a contended allocation to a different location without performing another instance of CSMA/CA.

#### Aborting a contended allocation

A node shall treat a contended allocation to have been aborted after failing to receive the expected acknowledgment frame in the allocation.

A node may start a new contended allocation procedure as specified in 7.4.1 to obtain another contended allocation.

#### Ending a contended allocation

A node may at any time end a contended allocation by not following with another frame transmission in the allocation.

### Slotted Aloha access

To employ slotted Aloha access, a node shall maintain a contention probability to determine if it obtains a new contended allocation in an Aloha slot as described below.

#### Starting a contended allocation

To obtain a new contended allocation for the transmission or retransmission of a frame of user priority UP, a node shall set its contention probability (CP) as follows.

  - If the node did not obtain any contended allocation previously, it shall set the CP to CPmax[UP].

  - If the node succeeded, i.e., if the node received an expected acknowledgment, I-Ack or B-Ack frame, to its last frame transmission, in the last contended allocation it had obtained, it shall set the CP to CPmax[UP] as well.

  - If the node transmitted a frame requiring no acknowledgment or group acknowledgment at the end of its last contended allocation, it shall keep the CP unchanged.

  - If the node failed, i.e., if the node did not receive an expected acknowledgment to its last frame transmission, in the last contended allocation it had obtained,

    - it shall keep the CP unchanged if this was the mth time the node had failed consecutively, where m is an odd number;

    - it shall halve the CP if this was the nth time the node had failed consecutively, where n is an even number.

  - If halving the CP would make the new CP smaller than CPmin[UP], the node shall set the CP to CPmin[UP].

With the CP set to CP as stated above, the node shall have obtained a contended allocation in the current Aloha slot if z ≤ CP, where z is a value the node has newly drawn at random from the interval [0, 1].  Each Aloha slot shall be of equal length predetermined to be pAlohaSlotLength as shown in Figure 68.

Figure 68 — Aloha slot structure

An Aloha slot available to the node for contention shall start at the beginning of an RAP or CAP if UP does not have the highest value (i.e., not for an emergency or medical event report) according to Table 13, or after the start of an EAP, RAP, or CAP if UP has the highest value (i.e., for an emergency or medical event report).  Successive Aloha slots may be available to the node for contention if they are fully located within the EAP, RAP, or CAP.  Figure 69 illustrates how to start and use contended allocations based on slotted Aloha access.

Figure 69 — Slotted Aloha access illustration

#### Using a contended allocation

A node may transmit a frame for which it obtained the contended allocation in the allocation.  The recipient shall transmit an acknowledgment frame, I-Ack or B-Ack, when required, pSIFS after the end of the previous frame.

To resume its transmission following a received acknowledgment frame, the node may transmit a new frame or retransmit an old frame, with an UP not lower than the UP used to obtain the current contended allocation, pSIFS after the end of the acknowledgment frame.  The node shall end its transmission in the allocation such that the last transmission in the allocation completes at least the nominal guardtime GTn = mGT_Nominal earlier than the end of the current EAP, RAP, or CAP.

#### Modifying a contended allocation

The node shall not modify a contended allocation obtained in an Aloha slot to a different Aloha slot without performing another instance of slotted Aloha access.

#### Aborting a contended allocation

A node may continue its transmission within a contended allocation, without aborting the allocation, after failing to receive the expected acknowledgment frame in the allocation.

A node may start a new contended allocation procedure as specified in 7.4.1 to obtain another contended allocation in a different Aloha slot.

#### Ending a contended allocation

A node may at any time end a contended allocation by not following with another frame transmission in the allocation.

## Improvised and unscheduled access (connectionless contention-free access)

### Unscheduled access

A hub may employ unscheduled polling and posting access to send polls or posts at any time to grant polled or posted allocations either in beacon mode or non-beacon mode as characterized in 7.2, so long as the addressed nodes indicated that they will always be in active state through their last transmitted MAC Capability field (i.e., with Always Active bit set to 1).  A node that has so indicated shall constantly be in active state ready to receive unscheduled polls or posts.

### Improvised access

A hub may employ improvised polling and posting access to send polls or posts at previously announced times based on Table 16 and as described below to grant polled or posted allocations, either in beacon mode or non-beacon mode as also noted in 7.2, for on-demand contention-free frame exchanges outside the scheduled allocations within their body area network (BAN).  A polled or posted allocation contains an explicit or implicit time interval that does not reoccur subsequently without the hub invoking another instance of improvised access.

Table 16 — Frames and fields (6.2.1.1.7 - 6.2.1.1.12) sent by a hub enabling polls and posts

|  | Immediate (short distance) | Future (long distance) |
| --- | --- | --- |
| Poll | I-Ack+Poll, B-Ack+Poll, Poll, or T-Poll frame, with More Data = 0, Poll-Post Window = A, Fragment Number / Coexistence / Next = Reserved: Type-I polled allocation (Poll Type = 0) A polled allocation is intended to start pSIFS after the end of the current frame, and to end at the end of allocation slot A located in the current beacon period (superframe). Type-II polled allocation (Poll Type = 1) A polled allocation is intended to start pSIFS after the end of the current frame, and to end after the polled node has sent A frames or requested an I-Ack or B-Ack frame, whichever is earlier. | I-Ack+Poll, B-Ack+Poll, Poll, or T-Poll frame, with More Data = 1, Poll-Post Window = B, Next = N: No polled allocation follows this frame, but another poll or post is intended to be sent at the start of allocation slot B located in the current beacon period (superframe) if N = 0 or in the next Nth beacon period (superframe) if N > 0. |
| Post | Non-beacon management or data type frame, with More Data = 1: A post is intended to start pSIFS after the end of the current frame transaction. | I-Ack or B-Ack frame, with More Data = 1, Poll-Post Window = B, Next = N: A post is intended to be sent at the start of allocation slot B located in the current beacon period (superframe) if N = 0 or in the next Nth beacon period (superframe) if N > 0. |
|  | A poll providing another poll but not a polled allocation may be considered a post. |  |

#### Polling access

A hub may send polls and grant type-I or type-II polled allocations to a node only if both of them support polling access of the corresponding type as indicated in their last exchanged MAC Capability field (i.e., with Type-I or Type-II Polling Access bit set to 1).  A hub may send polls addressed to Unconnected_Broadcast_NID and granting type-I or type-II polled allocations any time outside beacons and scheduled allocations, as described in 7.3 for unconnected exchange, without regard to the polling access settings in the MAC Capability indicated by the nodes in its body area network (BAN).

##### Starting a polled allocation

To obtain a polled allocation for initiating another frame transaction, a node shall set the More Data field to 1 in the frame it is transmitting.  The node should also set the Ack Policy field to I-Ack or B-Ack in some management or data type frames being transmitted.  This enables the hub to send the node an immediate or future poll at an announced time through an I-Ack+Poll or B-Ack+Poll frame as described in Table 16.

To grant an immediate polled allocation to a node, a hub shall send to the node a Poll or T-Poll frame when appropriate or an I-Ack+Poll or B-Ack+Poll frame when required to return an acknowledgment according to Table 16, with the Ack Policy field set to 0.  To grant no immediate polled allocation but to inform of an intended future poll or post, a hub shall send one of these frames also according to Table 16.  The hub should send a Poll or T-Poll frame at the time indicated in a frame previously sent to the node.  The hub may send to a node a Poll or T-Poll frame at a time not indicated earlier to the node if the node has also indicated to be always in active state in its last transmitted MAC Capability field (i.e., with Always Active bit set to 1).

Figure 70 illustrates instances of polls and polled allocations.

Figure 70 — Example polls and polled allocations

##### Using a polled allocation

A node that has indicated support for polling access shall receive Poll and T-Poll frames at times that have been announced in frames previously sent to it by the hub, as described in Table 16.  A node that has indicated to be always in active state shall be ready to receive Poll and T-Poll frames at any time.  The node shall also receive I-Ack+Poll and B-Ack+Poll frames at times expected for acknowledgment arrivals.  The node shall initiate a frame transaction at the start of a polled allocation granted to it through a preceding Poll, T-Poll, I-Ack+Poll, or B-Ack+Poll frame, as also illustrated in Figure 70.  The node shall send an I-Ack frame pSIFS after a Poll or T-Poll frame providing no immediate polled allocation and requiring an immediate acknowledgment.

A hub shall transmit an acknowledgment frame I-Ack or B-Ack when required, or I-Ack+Poll or B-Ack+Poll for another polled allocation, pSIFS after the end of the previous frame in a polled allocation.

In a type-I polled allocation, the node may start another frame transaction, for retransmitting old frames or /and transmitting new frames, pSIFS after the end of the expected acknowledgment frame regardless of whether it received the acknowledgment frame.  The node shall ensure that the frame transactions, including acknowledgment frames if required, in a polled allocation stay within the polled allocation interval, taking the appropriate guardtime into account.

In a type-II polled allocation, the node shall not transmit more frames than granted for transmission via the preceding Poll frame or its variant, nor the node shall transmit any more frames after transmitting a frame requiring return an I-Ack or B-Ack frame.  The hub shall ensure in granting a type-II polled allocation that the frame transaction, including  the acknowledgment frame if required, stays within the intended polled allocation interval, taking the appropriate guardtime into account.

##### Modifying a polled allocation

A hub may modify a polled allocation of a node by sending the node another poll through an I-Ack+Poll or B-Ack+Poll frame within the polled allocation that expands the allocation interval, effectively granting a new polled allocation in place of the existing one.

##### Aborting a polled allocation

A node may relinquish the remaining allocation interval of a polled allocation by indicating that it has no more data to transmit in the frame being transmitted and then by transmitting no more frames in the interval, or by requesting an immediate or block acknowledgment in case of a type-II polled allocation.

A hub shall not reclaim a polled allocation on the basis that it has not received any frame in the allocation, unless the node has explicitly relinquished the allocation as described above.

##### Ending a polled allocation

A node may at any time decline future type-I or type-II polled allocations to it by sending a modified Connection Request frame that contains a MAC Capability indicating no capability for the polling access of the corresponding type.

A hub may at any time decline future polled allocations to all the nodes connected with it by indicating no poll support in the MAC Capability of its subsequent beacons.

#### Posting access

A hub may send posts to a node while granting posted allocations outside the scheduled allocation of the node, so long as it has informed the node of the pending posts through previously transmitted frames or if the node has indicated to be always in active state in its last transmitted MAC Capability field (i.e., with Always Active bit set to 1).

##### Starting a posted allocation

To obtain a posted allocation for receiving a post, a node that does not have a scheduled downlink allocation shall send at least one management or data type frame with the Ack Policy field set to I-Ack or B-Ack in each of its wakeup beacon periods (superframes).  This enables the hub to inform the node, through an I-Ack or B-Ack frame as described in Table 16, of a specific time for sending the node a future post containing essential network management information or a critical user message at.

To send an immediate post to a node, a hub shall set the More Data field to 1 in the non-beacon management or data type frame it is transmitting, as also described in Table 16.  To send a future post or poll to a node, a hub shall send to the node an I-Ack or B-Ack frame when required to return an acknowledgment, as noted in the above.

Figure 71 illustrates instances of posts and posted allocations.

Figure 71 — Example posts and posted allocations

##### Using a posted allocation

A hub shall initiate frame transactions at the start of its posted allocations that have been announced in earlier frames as described in Table 16.

A node shall receive posts and participate in the frame transactions in its posted allocations that have been indicated to it earlier through other frames.  A node that has indicated to be always in active state shall be ready to receive posts at any time.

The node shall transmit an acknowledgment frame I-Ack or B-Ack when required, pSIFS after the end of the previous frame in a posted allocation.  If the hub has announced in the current frame that it has another immediate post, it may initiate another frame transaction, for retransmitting old frames or/and transmitting new frames, pSIFS after the end of the expected acknowledgment frame regardless of whether it received the acknowledgment frame.  The hub shall ensure that the frame transaction, including the acknowledgment frame if required, in a posted allocation stays within the intended posted allocation interval, taking the appropriate guardtime into account.

##### Modifying a posted allocation

A hub shall not modify a posted allocation once it initializes a frame transaction in the allocation, as a posted allocation is just long enough for a frame transaction.  A hub may grant itself another posted allocation for initializing another frame transaction after informing the target node of the time of the new allocation through the current frame.

##### Aborting a posted allocation

A hub shall not abort a posted allocation once it initializes a frame transaction in the allocation.

##### Ending a posted allocation

A node shall not end a pending posted allocation nor potential future posted allocations, as it needs to receive all the posts—which might carry essential network management information or critical user messages—from the hub with which it is connected.

A hub should not end a pending posted allocation once it has announced it through a frame.

## Scheduled access and variants (connection-oriented contention-free access)

As outlined in 7.2, a node and a hub may employ scheduled access to obtain scheduled uplink allocations and scheduled downlink allocations, scheduled-polling access to obtain scheduled bilink allocations and polled allocations therein, and delayed polling access to obtain delayed bilink allocations and polled allocations therein.  Scheduled uplink allocations, scheduled downlink allocations, scheduled bilink allocations, and delayed bilink allocation, collectively referred to as scheduled allocations, may be 1-periodic or m-periodic allocations, except that a node shall not have both 1-periodic and m-periodic allocations in the same body area network (BAN).

To have a 1-periodic uplink or downlink allocation, which has one or more allocation intervals spanning the same allocation slots in every beacon period (superframe), the node shall wake up in every beacon period, i.e., the node shall treat all beacon periods as its wakeup beacon periods, as illustrated in Figure 72(a).

To have an m-periodic allocation, which has one or more allocation intervals spanning the same allocation slots in every m > 1 beacon periods (superframes), the node shall wakeup in the beacon periods containing those allocation intervals, i.e., the node shall treat those beacon periods as its wakeup beacon periods, as illustrated in Figure 72(b).

(a) Example 1-periodic allocation

(b) Example m-periodic allocation

Figure 72 — Scheduled uplink, downlink, and bilink allocations, and delayed bilink allocations

### Starting scheduled allocations

To obtain one or more new scheduled allocations, a node shall send a Connection Request frame to the hub when permitted to do so, setting the Wakeup Period field in the frame to 1 for 1-periodic allocations and to m > 1 for m-periodic allocations.  The node shall include in the frame

  - an Explicit Uplink Request or Implicit Uplink Request IE if scheduled uplink allocations are needed;

  - an Explicit Downlink Request or Implicit Downlink Request IE if scheduled downlink allocations are needed;

  - an Explicit Bilink Request or Implicit Bilink Request IE if scheduled bilink allocations are needed;

  - an Explicit Delayed Bilink Request or Implicit Delayed Bilink Request IE if delayed bilink allocations (i.e., scheduled bilink allocations that are subject to delay)  are needed;

  - a Group Connection IE if the current connection request is made for a group of nodes.

In these IEs, the Minimum Interval Length and Minimum Allocation Length fields, when present, shall have nonzero values, and one or both of the MSDU Arrival Rate and MSDU Service Period fields, when present, shall have nonzero values.

To grant scheduled allocations, the hub shall respond with a Connection Assignment frame to the node, including

  - an Uplink Assignment IE if scheduled uplink allocations are granted;

  - a Downlink Assignment IE if scheduled downlink allocations are granted;

  - a Bilink Assignment IE if scheduled bilink allocations are granted;

  - a Delayed Bilink Assignment IE if delayed bilink allocations are granted;

  - a Group Connection IE if the current connection assignment is made for a group of nodes.

In these IEs, the Interval Start and Interval End fields shall have nonzero values.

The hub may grant scheduled bilink allocations in lieu of delayed bilink allocations requested by the node, but not vice versa.

### Using scheduled allocations

Upon receiving the Connection Assignment frame granting it scheduled uplink allocation intervals, the node may start initiating frame transactions with the hub therein, as illustrated in Figure 73(a).  The hub shall be ready to receive the frames transmitted by the node and acknowledge them when appropriate during these allocation intervals.

Upon successfully sending the Connection Assignment frame granting the node scheduled downlink allocation intervals, the hub may start initiating frame transactions with the node therein, as also illustrated in Figure 73(a).  The node shall be ready to receive the frames transmitted by the hub and acknowledge them when appropriate during these allocation intervals.

Upon successfully sending the Connection Assignment frame granting the node scheduled bilink allocation intervals, the hub may start initiating frame transactions with the node, or the hub may start sending to the node a poll granting an immediate polled allocation for the node to initiate one or more frame transactions therein, in these allocation intervals, as illustrated in Figure 73(b).  The poll should be a Poll frame if it is sent at the start of the scheduled bilink allocation or if the hub does not need to provide a time reference update to the node, or should be a T-Poll frame otherwise.  The node shall not initiate a frame transaction, until it receives a Poll or T-Poll frame, in a bilink allocation interval.  The recipient, the node or the hub, shall be ready to receive the frames transmitted by the sender and return appropriate frames during the allocation intervals.

Upon successfully sending the Connection Assignment frame granting the node delayed bilink allocation intervals, the hub may start initiating frame transactions with the node, or the hub may start sending to the node a poll granting an immediate polled allocation for the node to initiate one or more frame transactions therein, in these allocation intervals, as illustrated in Figure 73(c).  The poll should be a Poll frame if it is sent at the start of the scheduled bilink allocation or if the hub does not need to provide a time reference update to the node, or should be a T-Poll frame otherwise.  The hub may delay its first frame transmission to the node up to the maximum delay from the start of the delayed bilink allocation interval as specified in the connection assignment.  If the hub fails to send a frame to the node within the maximum delay, the hub shall not send a frame to the node in the interval.  The node shall not initiate a frame transaction, until it receives a Poll or T-Poll frame, in a delayed bilink allocation interval.  The recipient, the node or the hub, shall be ready to receive the frames transmitted by the sender and return appropriate frames during the allocation intervals.  If the node fails to receive a frame addressed to it within the maximum delay of the start of the interval from the hub, it may be in sleep state in the remaining interval.

The node or the hub shall transmit next frame pSIFS after the end of the current frame.  The node or the hub may initiate another frame transaction in the remaining scheduled uplink or downlink allocation interval, respectively, pSIFS after the end of the expected acknowledgment frame regardless of whether it received the acknowledgment frame.  In the remaining scheduled bilink allocation interval, after the end of the preceding frame transactions initiated by the hub or after the end of the preceding polled allocation interval, the hub may initiate more frame transactions with the node, or the hub may send to the node another poll granted another immediate polled allocation for the node to initiate one or more frame transactions therein.

The node and the hub shall ensure that the frame transactions, including acknowledgment frames if required, stay within their scheduled uplink or downlink allocation intervals, respectively, taking the appropriate guardtime into account.  The hub shall ensure that the frame transactions, including acknowledgment frames if required, stay within the scheduled bilink allocation intervals, also accounting for the appropriate guardtime.  The hub shall ensure that the frame transactions, including acknowledgment frames if required, stay within the delayed bilink allocation intervals, or till the maximum delay applicable to the next delayed bilink allocation interval that follows immediately, also accounting for the appropriate guardtime.

In a type-I polled allocation interval, the node shall ensure that the frame transactions, including acknowledgment frames if required, stay within the polled allocation intervals.  In a type-II polled allocation interval, the node shall not transmit more frames than granted for transmission therein or after it has requested an I-Ack or B-Ack frame for acknowledgment.

In a scheduled uplink allocation interval, the node may through its current data type frame indicate no more data for transmission in the remaining interval.  In a scheduled downlink allocation interval, a scheduled bilink allocation interval, or a delayed bilink allocation interval, the hub may send a data type frame indicating no more data with or without frame payload to indicate no more transmissions in the remaining interval; the node may enter sleep state accordingly in the remaining interval.

(a) Frame transactions in scheduled uplink allocation interval and scheduled downlink allocation interval

(b) Frame transactions in scheduled bilink allocation intervals

(c) Frame transactions in delayed bilink allocation intervals

Figure 73 — Example scheduled allocation intervals

### Modifying scheduled allocations

A node may modify existing scheduled allocations by sending another Connection Request frame specifying the new requirements.  The hub shall treat this request as a new one, except that it shall set the Connection Change Indicator field in its responding Connection Assignment frame with reference to the last Connection Assignment frame it sent to the node.  In particular, if the hub rejects the modifications but maintains the existing allocations, it shall respond with a Connection Assignment frame with the Connection Change Indicator field set to 0 and the other fields kept to the respective values contained in the last Connection Assignment frame sent to the node.

A hub may, but should not where possible, modify scheduled allocations of a node on its own by sending the node an unsolicited Connection Assignment frame specifying the new assignments to those allocations, setting the Connection Change Indicator field in the frame with reference to the last Connection Assignment frame it sent to the same node.

### Aborting scheduled allocations

A node or a hub shall treat an existing scheduled allocation to have been aborted after failing to receive any frame in the last mScheduledAllocationAborted allocation intervals of the allocation.  Subsequently, the hub may reclaim the aborted scheduled allocations.

A node or a hub shall transmit at least one frame requiring an immediate return of a frame, such as an I-Ack or B-Ack frame, or a poll if applicable, in every allocation interval of a scheduled allocation so as to reduce the chance of experiencing an abortion of the scheduled allocation.

A node and a hub may start a new scheduled allocation procedure as specified in 7.6.1 to reinstate their lost scheduled allocations or obtain their replacements.

A node or a hub shall treat their connection to have been aborted after all the scheduled allocations of the node with the hub have been aborted.

### Ending scheduled allocations

A node may at any time end scheduled allocations by sending a modified Connection Request frame that contains Allocation Request fields with the Allocation ID fields identifying those allocations and with the corresponding Minimum Interval Length and Minimum Allocation Length fields or MSDU Arrival Rate and MSDU Service Period fields both set to 0.

A hub may, but should not where possible, at any time end any scheduled allocations of a node by sending the node a modified Connection Assignment frame that contains Allocation Assignment fields with the Allocation ID fields identifying those allocations and the Interval Start and Interval End fields set to 0.

A node or a hub may send a Disconnection frame to end their connection, i.e., to void the node’s Connected_NID, wakeup arrangement, and scheduled allocations with the hub.

## MICS band communication

In the medical implant communications service (MICS) band, a hub shall operate in non-beacon mode with or without superframe boundaries as specified in 7.2.  The hub may choose a new channel only when required by applicable regulations and considerations.  An implant shall communicate as a node with a hub.  The hub and the node may perform a mutual discovery procedure described below immediately before their frame exchanges.

### Unconnected mutual discovery

A hub may facilitate mutual discovery with unconnected nodes and connection with them as follows.

As illustrated in Figure 74, every pMICSUnconnectedPollPeriod or shorter the hub shall transmit a group of up to pMICSUnconnectedPolls unconnected T-Poll frames separated by pMICSUnconnectedPollSeparation.  The hub shall listen for a frame arrival after each transmission and before the next transmission of such an unconnected T-Poll frame.  An unconnected T-Poll frame is a T-Poll frame addressed to Unconnected_Broadcast _NID as defined in Table 12 and providing an unconnected polled allocation as specified in 7.3.

An unconnected node in need of a connection may discover a hub as follows.

The unconnected node should cyclically tune to each MICS band channel for pMICSUnconnectedPollRxTime, until it receives an unconnected T-Poll frame and hence discovers a hub.  The unconnected node should not further change the channel unless recommended otherwise.

Figure 74 — Unconnected mutual discovery and frame exchange in the MICS band

The pMICSUnconnectedPollPeriod parameter value is obtained as a result of an overall consideration of connection latency, power consumption, channel utilization efficiency, interference, and other factors.  The parameter pMICSUnconnectedPolls has a value designed to provide enough unconnected T-Poll frames so that a node can receive one of them within a cycle of tuning to each of the MICS band channels.  The parameter pMICSUnconnectedPollSeparation has a value such that the hub can detect a frame sent by a node following its last unconnected T-Poll frame before it would otherwise send the next unconnected T-Poll frame.  The parameter  pMICSUnconnectedPollRxTime has a value to ensure with a certain margin that a node can receive one of the two adjacent unconnected T-Poll frames once it tunes to the transmit channel.

Now that the hub and the unconnected node are on the same channel, they are in a position to follow the connection setup procedure specified in 7.3 to exchange management type frames necessary for a connection, including a wakeup arrangement and scheduled allocations as appropriate.  The hub should provide the node with an ordered list of channels it intends to choose in decreasing likelihood when required to choose a new channel.  The hub should subsequently select a new channel, when required by the regulations, in the order indicated in the list.  The hub may update nodes with a new channel order list in view of changes in the channel conditions such as interference levels.

### Connected mutual discovery

A hub may facilitate mutual discovery with connected nodes immediately prior to frame exchanges with them, based on any or a combination of the following discovery procedures.

#### Unicast poll aided discovery

As illustrated in Figure 75, prior to more frame exchanges with a connected node, the hub shall transmit a group of up to pMICSPolls Poll frames separated by pMICSPollSeparation, each addressed to the node and providing an immediate polled allocation.  The hub shall listen for a frame arrival after each transmission and before the next transmission of such a Poll.

The connected node shall be in active mode in anticipation of pending frame exchanges with the hub by scheduling or other means, accounting for the appropriate guardtime.  It should tune to the channel in which it last received a frame with a valid FCS from the hub.  After dwelling on the current channel for pMICSPollRxTime, it should switch to another channel in accordance with the channel order last provided by the hub, until it receives a Poll frame addressed to it.  The node should not further change the channel unless recommended otherwise.

Figure 75 — Unicast poll aided connected discovery and frame exchange in the MICS band

Now that the hub and the unconnected node are on the same channel, they are in a position to exchange more frames with each other using appropriate access methods as specified in 7.5 and 7.6 in view of 7.2.

While cycling through the MICS band channels for mutual discovery with the hub, upon receiving a Poll frame sent by the hub but not addressed to them on a channel, nodes should skip that channel from their subsequent channel cycling for the duration of the polled allocation granted via the Poll frame.  [JMH’s note:  The mechanism described above this paragraph was extracted from doc 0327-01.  While incorporating Ranjeet’s text on “unicast wakeup mechanism”, I found there is substantial overlap in the functional procedure between the description above and Ranjeet’s.  I therefore added this last paragraph to pick up the channel skipping idea unique to Ranjeet’s text. However, Ranjeet subsequently indicated “WiBAN would stick to using wakeup frame based unciast and multicast wakeup”, so here are his text and figures for the unicast wakeup, which is also intended to be an optional mechanism despite the prescriptive “shalls” that appear in the text.]

Wakeup frame format:

Figure 1: Wakeup frame payload format

Unicast wakeup mechanism:

Hub shall employ the in-band wakeup mechanism to wakeup an implant device as described below.

Unicast wakeup mechanism shall be employed to wakeup an unconnected device with known IEEE address or a connected device. Hub must have selected a MICS channel following the MICS regulations and shall send the wakeup messages in the selected channel to wakeup a device.

To wakeup an unconnected device with known IEEE address, hub shall send a wakeup frame to the node with ACK policy set to immediate-ACK, with Unconnected NID as recepient ID, IEEE address as recepient address in the wakeup frame payload, session duration and time to start the session in the frame payload, and then wait for an ACK frame from the node.

To wakeup a connected device, hub shall send a wakeup frame to the node with ACK policy set to immediate-ACK, with connected NID as recepient ID, and session duration and time to start the session in the frame payload, and then wait for an ACK frame from the node.

If it failed to detect ACK frame arrival in pMICSWakeupSeparation separation, hub shall send another wakeup frame to the node. The hub shall send wakeup frames until it receives an ACK frame from the node, with maximum up to pMICSWakeup consecutive wakeup frames. The pMICSWakeup number of wakeup frame ensures that device receives at least one wakeup frame.

A node, when not in session expected to sniff on available channels, one by one in a periodic manner. Upon successfully receiving a wakeup frame, node shall send an ACK frame to the hub. Node and hub shall have communciation session after the session start time offset as indicated in the wakeup frame payload.

After receiving a wakeup frame in a channel, unintended node shall exclude the channel for sniffing for session duration specified in the wakeup frame. The device shall resume sniffing in the channel, after the session expires. Figure 2 illustrates the unicast devcie wakeup.

Figure 2: Example of single conencted or unconnected device wakeup

#### Multicast lockup aided discovery

As illustrated in Figure 76, prior to more frame exchanges with a group of connected node, the hub shall transmit a group of up to pMICSMcastPolls Poll frames separated by pMICSMcastPollSeparation, each addressed to the Multicast_NID of the group and providing no immediate polled allocation but announcing a future poll starting at the intended beginning of the first individual communication session with a node of the group.  In particular, the time of the future poll is encoded according to Table 16.

The hub shall transmit a poll addressed to a node of the group at the indicated future poll time and providing an immediate polled allocation.

The connected nodes of the group shall be in active mode in anticipation of pending frame exchanges with the hub by scheduling or other means, accounting for the appropriate guardtime.  Each of them should tune to the channel in which it last received a frame with a valid FCS from the hub.  After dwelling on the current channel for pMICSMCastPollRxTime, each should switch to another channel in accordance with the channel order last provided by the hub, until it receives a Poll frame addressed to the Multicast_NID of its group.  It should not further change the channel unless recommended otherwise.  It may enter sleep state until the start of the first individual session as announced in the received Poll frame, when it shall be in active state to receive a unicast Poll frame.

Figure 76 — Multicast poll aided connected discovery and frame exchange in the MICS band

Now that the hub and the connected nodes of the group are on the same channel, they are in a position to exchange more frames with each other using appropriate access methods as specified in 7.5 and 7.6 in view of 7.2.  [JMH’s note:  Ranjeet, I took an extra step to illustrate and expounded your lockup idea.  Given that you are now using “GroupID” for lockup, I found multicast polls to serve your intent very well – and more efficiently as they do not have a frame payload.  I’ll keep the Wakeup frame in Clause 6 for now, but it should be removed later as it is no longer needed.]

[JMH’s further note: I wrote this subclause and drew the figure above while incorporating Ranjeet’s multicast lockup and unicast wakeup mechanism.  As mentioned in the previous subclause, Ranjeet asked to stick to his multicast wakeup mechanism, so here are his text and figure on this mechanism.  Again it is intended to be an optional mechanism.]

Multicast wakeup mechanism:

Hub shall employ the in-band wakeup mechanism to wakeup multiple implant nodes as described below. This method is applicable for wakeup of multiple connected nodes with NIDs.

Hub must have selected a MICS channel following the MICS regulations and shall send the wakeup messages in the selected channel to wakeup connected nodes. Multicast wakeup has two phases: Lockup and Confirm phase.

When the Lockup phase starts, hub shall send a wakeup frame to multiple connected nodes with Ack policy set to No-ACK, Multicast group NID as recepient ID, session duration and time to start the session in the frame payload, and list of individual NIDs in the wakeup frame payload.

The hub shall continuously send pMICSWakeup number of wakeup frames with frame separation of pMICSMcastWakeupSeparation, without expecting acknowledgment from any connected device. The pMICSWakeup number of wakeup frames shall ensure that every connected device shall receive at least one wakeup frame.

Upon successfully receiving a wakeup frame, intended connected device shall lock itself to the channel in which it received the wakeup frame and wait for another wakeup frame in the confirm phase in the same channel. Intended connected nodes shall interpret the time to start session field information as time to start it’s confirm phase and may become active only at that time, after lock up.

In the confirm phase, hub shall send a wakeup frame to the individual node, with ACK policy set to immediate-ACK,  connected NID as recepient ID, and session duration and time to start the session in the frame payload, and then wait for an ACK frame from the node. Upon successfully receiving the wakeup frame, node shall send an ACK frame to the hub. Hub shall repeat above step for other locked up nodes.

After completion of one round of wakeup, hub shall send wakeup frame to the nodes whose acknowledgment not received by the hub.

### Medical implant event report

When not transmitting, a hub should stay on receive mode in the channel selected according to its channel order list communicated to the nodes connected with it.

A node connected with a hub may transmit frames reporting a medical implant event in its next scheduled bilink allocation interval, if available, following a Poll or T-Poll frame granting an immediate polled allocation to it by the hub, using the a connected mutual discovery procedure as specified in 7.7.2.

The node may also transmit such frames anytime as illustrated in Figure 77.  In particular, the node should transmit an Emergency data frame with no frame payload and with the Ack Policy field set to I-Ack, in the first channel of the channel order list last communicated to it by the hub.  It should retry the frame for up to pMICSNodeEmergencyRetries times on this channel upon failing to receiving an expected acknowledgment.  If it still receives no acknowledgment, it should retry the frame in the next MICS channel on the channel order list, and again in another channel at pMICSFrameSpace after the end of its last frame transmission, until it receives an expected I-Ack frame or pauses its transmission.

After receiving an I-Ack frame, it should proceed to transmit the Emergency data frames (with incremental sequence numbers) containing frame payloads generated from the medical implant event.  The node shall set to 1 the More Data field in the MAC header of these Emergency data frames except the last one, and shall set to 0 the More Data field in the MAC header of that last frame to indicate the end of the medical event report transfer.

On receiving an Emergency data frame with More Data field set to 1 from the node, the hub should not initiate its own frame transactions with this node or another one until it has received all Emergency data frames as indicated by the More Data field value.

Figure 77 — Medical implant event report outside scheduled allocation intervals in the MICS band

After retrying a frame for up to pMICSHubMaxRetries without receiving an expected response, the hub should enter the receive mode to receive possible emergency data frames.

## Two-hop star topology extension

A node or a hub may employ the relay mechanism for a two-hop access extension (EST), also known as Restricted Tree Topology (RTT), as described below.  [JMH’s note:  I have not had a chance to read or edit this subclause sent to me by Hind today (March 09, 2010).  It is intended to provide an optional relay mechanism.]

The node shall discover a secure relay in its range when required. Two approaches may be considered depending on whether the node have been already connected to hub and have scheduled allocation with the hub or  the node is not connected and there is no assumption on the nature of allocation.

### Relay discovery procedure for connected node

Figure 78 shows the signaling of RTT for the establishment of RTT and the subsequent transfer of data

When the connected node detects that it has a poor communication link with the hub, it stays awake in order to discover a relay capable node (RN) and establish a new link to it so that the node can continue its transmission to the hub.

A node discovers a RN as follows: when the SN overhears an acknowledgement (ACK) to data or management frames originated from RN and destined to the hub of the same BAN, this implies two things that: 1) RN is within the range of both hub and the node and that 2) the link between the RN and the hub is a fairly reliable link assuming the link is symmetric. At this point, we assume that SN has discovered a RN.

Therefore, the Node will initiate the establishment of a new link with the discovered RN by sending a connection request to the RN. Then, if RN accepts the connection request from the node, it relays the connection request to the hub.

This signaling is needed to make the hub aware of the connection request from thenode, the hub allocates more radio resource to the existing connection link between RN and hub in order to accommodate the additional relayed transmissions from the node.

The hub also allocates radio resources for the new link between the RN and the node and informs the RN of the update of radio resource allocation through Connection Assignment 1, as shown in Figure 78, the relay bit on this occasion is set to be R = 0. The hub also informs the RN of the additional radio resource allocation for the new link between the RN and the node through the Connection Assignment 2 signal shown in Figure 78, which is then relayed to the node. Finally, the node transfers data to the hub through the RN following the new radio resource allocation.

Figure 78 — RTT signaling initiated by a connected node having a scheduled allocation experiencing with a bad connection

### Relay discovery procedure for unconnected node

Figure 79 illustrates the signaling protocol for the establishment of RTT and the subsequent transfer of data. The assumption here is that it is the responsibility of the SNs to discover the relay capable node RN and request connection to. A RN may optionally provide synchronization to nodes (unconnected, orphans or connected).  This is achieved when the relay sends a Broadcast Message (BM) which includes a time stamp and resource allocation specification in which nodes can communicate with the relay node. The purpose of this broadcast message is two fold:1) to provide synchronization to orphan nodes,  new node, or a SN having a  bad connection and 2) to provide an opportunity for establishing a connection to RN in order to relay their transmission to hub. How often the relay will send the Broadcast Message is a policy issue and should not violate the power consumption constraint of the RN and the availability of resource allocation in the network. The BM will assist the SN to discover a relay node and to wake up at the resource allocation advertised by the relay’s BM , in order to establish connection with the relay. A RN may optionally exercise admission control over connection requests from nodes to restrict the number of connections or nodes that the relay can support taking into account its power consumption and other policies such as security policy to prevent third party attack etc.

A node should keep track of this broadcast message during their resource allocation to ensure connectivity and reliability of transmission especially in the event of bad connection or if they become orphan. Therefore, if a SN experience a bad connection or become orphan,, Knowing the advertised allocation in BM will enable that SN to wake up at the same time of the RN. Then that SN will authenticate the relay. if the authentication is successful, SN will issue an Association Request message to connect to the relay node. Once accepted the Association Request message is relayed to the hub to request for additional resource allocation for communication between that relay and that SN. Then the hub updates the connection to the relay, and also allocates additional  resources for the second hop, the link between that RN and that SN. Finally, That SN transfers its data to the hub through that RN according to the new resource allocation.

Figure 79 — RTT signaling initiated by an unconnected / connected node

### RTT Ending

Two approaches are considered for ending of the RTT session depending on who initiates the ending: node or  hub.

#### Initiated by Node

Figure 80 shows the signaling related to ending the RTT session as initiated by the SN. As illustrated, when the connection of the SN improves, it should revert back to direct communication with the hub, rather than through the Relay. It is assumed that the reliability of the connection can be detected when SN receives a number of broadcast messages from the hub over a predefined period of time. The SN then sends a connection request with radio resource allocation cancellation to the RN to signal the ending of the RTT session as shown in Figure 80.

This message is then relayed to the hub. Consequently, the hub updates the radio resource allocation accordingly, removing the additional radio resource allocation related to the link between the SN and the RN. The hub informs the RN and the SN of this update through Connection Assignment 1 and Connection Assignment 2, respectively, for the link between hub and the RN, and the link between the RN and the SN. The RN then forwards the Connection Assignment 2 to the SN, following which the SN transmits its data directly to hub.

Figure 80 — RTT session ending initiated by the node

#### Initiated by Hub

Figure 81 shows the signalling related to ending the RTT session when initiated by the hub. When the hub overhears communication from the SN for a period of time, it sends a Connection Assignment 1 with resource allocation modification to the RN. The hub also sends a Connection Assignment 2 with radio resource cancellation to RN, for forward transmission to the SN. On reception of the latter message , the SN sends an ACK back to the hub through the RN, which in turn sends an ACK to hub as illustrated in Figure 4. Consequently, the hub updates the radio resource allocation accordingly, removing the additional radio resource allocation related to the link between the SN and the RN.

Figure 81 — RTT session ending by the hub

## Clock synchronization and guardtime provisioning

A node or a hub shall maintain a clock with a minimum resolution of mClockResolution and with a minimum accuracy of mClockAccuracy to time its frame transmission and reception.

A node or a hub shall time its transmission and reception in any of its allocation intervals according to its local clock, setting aside appropriate GTs as specified below and illustrated in Figure 82:

  - The hub shall commence its beacon transmission at the nominal start of the beacon transmission.

  - The hub shall commence its transmission in a downlink allocation interval at the nominal start of the interval, and the hub shall end its transmission in the interval early enough such that the last transmission in the interval completes at least a guardtime of GTn prior to the nominal end of the interval.

  - The hub shall commence its reception in an uplink allocation interval at least GTn prior to the nominal start of the interval.

  - If a node’s last synchronization to the hub was less than SIn ago at the nominal start of its next uplink allocation interval, the node shall commence its transmission in the interval at that nominal start time, and the node shall end its transmission in the interval early enough such that the last transmission in the interval completes at least a guardtime of GTn prior to the nominal end of the interval.

  - If a node’s last synchronization to the hub was less than SIn ago at the nominal start of the next beacon transmission, the node shall commence its reception of the beacon at least GTn prior to that nominal start time.

  - If a node’s last synchronization to the hub was less than SIn ago at the nominal start of its next scheduled downlink or bilink allocation interval, the node shall commence its reception in the interval at least GTn prior to that nominal start time.

  - If a node’s last synchronization to the hub was SIn + SIa ago at the nominal start of its next scheduled uplink allocation interval, the node shall commence its transmission in the interval a guardtime of GTa later than that nominal start time, and the node shall end its transmission in the interval early enough such that the last transmission in the interval completes at least a guardtime of GTn + GTa prior to the nominal end of the interval.

  - If a node’s last synchronization to the hub was SIn + SIa ago at the nominal start of its next scheduled downlink or bilink allocation interval, the node shall commence its reception in the interval at least GTn + GTa earlier than that nominal start time.

A node shall synchronize to its hub through the beacons, T-Poll frames, or the first frames (on-time frames) in scheduled allocation intervals received from the hub.  In particular, the node shall advance or delay its clock by a total amount of

D = TS – TL, if TS > TL

or

D = TL – TS, if TS < TL

respectively, where TS is the time when a beacon, a T-Poll frame, or a frame with the “First Frame / On Time” bit in its MAC header set to 1 was scheduled to start on the transport medium (i.e., air), and TL is the time when the frame was received according to the local clock.

The various guardtime components shall be determined as follows:

mGT_Nominal = GTn = GT0 + 2×Dn,    Dn = SIn×mClockAccuracy

GT0 = pSIFS + pSynchResolution + mTxResolution,    mTxResolution = mClockResolution

GTa = 2×Da,    Da = SIa×mClockAccuracy

The parameter GTn = mGT_Nominal has a predefined value as listed in Table 17, and designates the guardtime nominally observed by a node or a hub.  The parameter GT0 comprises the RX-TX or TX-RX turnaround time pSIFS, the synchronization error pSynchResolution, and the transmission timing error mTxResolution, which are all of fixed values that are independent of clock drifts and listed in Table 17 as well.  Thus the value of GT0 is also predefined.  Given the values of GTn and GT0, the parameter Dn represents the nominally allowed clock drift of a node or hub relative to an ideal (nominal) clock.  With the minimum clock accuracy of mClockAccuracy also given in Table 17, Dn is predetermined too; it delimits a nominal synchronization interval SIn over which the clock drift of any node is accounted for in the nominal guardtime GTn.

The parameter SIa denotes the synchronization interval that has accrued in addition to SIn since last synchronization.  The corresponding additional clock drift Da is a function of SIn and accounts for the required additional guardtime GTa.  The values of these two parameters are specific to the node and time of concern.

An illustration of clock drift and guardtime provisioning is given in Figure 82.

Figure 82 — Clock drifts and guartimes

## Power management

A node not indicating to be always in active state, referred to as a node in short in this subclause, may hibernate across a number of beacon periods (superframes), and may sleep over some time intervals even in its wakeup beacon periods (superframes), as clarified below.

### Hibernation—macroscopic power management

To hibernate—without receiving or transmitting any traffic—in some beacon periods, the node shall set the Wakeup Period field in its last Connection Request frame to an integer larger than 1, while setting the Wakeup Phase field in the frame to a value specifying its intended next wakeup beacon period.

To wakeup—for receiving or/and transmitting frames—in every beacon period, the node shall set the Wakeup Period field in its last Connection Request frame to 1, while setting the Wakeup Phase field in the frame to a value identifying the next beacon period.

The intended recipient hub of the Connection Request frame should honor the values of the received Wakeup Period and, to a less extend, Wakeup Period fields whenever possible, but may set them to different values if need be, in its responding Connection Assignment frame.  The hub may later modify these values by sending to the node another Connection Assignment frame if warranted by new conditions.

If the hub sets the Wakeup Period field in its responding frame to an integer larger than 1, it may grant only m-periodic allocations to the node, with the allocation intervals being in the node’s wakeup beacon periods, in accordance with the node’s last connection request whenever possible, but shall not grant to the node any 1-periodic allocations.

If the hub sets the Wakeup Period field in its responding frame to 1, it may grant only 1-periodic allocations to the node, with the allocation intervals being in every beacon period, in accordance with the node’s last connection request whenever possible, but shall not grant to the node any m-periodic allocations.

If the Wakeup Period value in the Connection Assignment frame last received from the hub is larger than 1, the node shall wake up in each of its wakeup beacon periods based on the latest Wakeup Period and Wakeup Phase values provided in that frame by the hub, to transmit or/and receive frames in the granted m-periodic allocation intervals, and to receive the beacon if needed.

If the Wakeup Period value in the Connection Assignment frame last received from the hub is 1, the node shall wake up in every beacon period, to transmit or/and receive frames in the granted 1-periodic allocation intervals, and to receive the beacon if appriproate.

Figure 83  illustrates macroscopic power management across beacon periods.

Figure 83 — Macroscopic power management

### Sleep—microscopic power management

A node shall wake up to receive a beacon from a hub when it needs a beacon reception to synchronize with the hub or to obtain certain information contained in a beacon.

A node shall wake up to receive and transmit frames in its scheduled allocations in its wakeup beacon periods, as illustrated in Figure 84.

In addition, the node shall stay active participating in frame transactions in its expected posted allocations, as illustrated in Figure 84(a).  The hub should arrange to have the posted allocations of a node to occur in the node’s wakeup beacon periods, if possible.  If the node did not receive a frame at the announced time for a pending post, it should stay in receive mode until the hub could have finished a frame transaction for the post and retransmitted a frame pSIFS later, as also illustrated in Figure 84(a), unless it needs to make a turnaround to transmit mode.

If the node has indicated its support for polls through its MAC Capability field of its last Connection Request frame, it shall also stay active in such times as to receive announced polls and initiate frame transactions in its polled allocations, as illustrated in Figure 84(b).  The hub should arrange to have the polled allocations of a node to occur in the node’s wakeup beacon periods, if possible.

Outside the time intervals noted in the above, the node may sleep—without receiving or transmitting any traffic.

Figure 84 illustrates microscopic power management in a wakeup beacon period .

Figure 84 — Microscopic power management

## Coexistence and interference mitigation

A hub may employ one or more of the mechanisms described in 7.11.1-7.11.3 for coexistence and/or interference mitigation between its body area network (BANs) and neighbor BANs.

### Beacon shifting

A hub may transmit its beacons at different time offsets relative to the start of the beacon periods by including a Beacon Shifting Sequence field in its beacons as defined in 6.3.1.8.  A hub should choose a beacon shifting sequence that is not being used by its neighbor hubs to mitigate potential repeated beacon collisions and scheduled allocation conflicts between overlapping or adjacent BANs operating on the same channel.

As shown in Figure 85, the hub shall transmit a beacon out of its physical layer (PHY) at a time t = PNm(n)×BP/4 relative to the start of beacon period n.  Here, m is the beacon shifting sequence index that the hub has chosen for its BAN, BP is the length of its beacon period, and n is the phase of the chosen sequence (n = 0, 1, … ) for this beacon period.

Figure 85 — Beacon shifting illustration

As defined in 6.3.1.2, the allocation slots in a beacon period shift around with the beacon transmit time.  The access phases – exclusive access phase 1 (EAP1), random access phase 1 (RAP 1), exclusive access phase 2 (EAP2), random access phase 2 (RAP2), and contention access phase (CAP) – shown in Figure 60 are referenced to numbered allocation slots and shift around with the beacon in the beacon period accordingly.  The RAP1 Length and RAP2 Length fields contained in the beacon of the current beacon period now refer to RAP1 and RAP2 in the next beacon period.

The hub shall ensure in choosing these access phases and the beacon shifting sequence that beacon shift does not result in a split of any of the aforementioned access phases into two parts.

Scheduled allocation intervals are also referenced to numbered allocation slots and shift around with the beacon transmit time accordingly in the beacon period as shown in Figure 86.  A scheduled allocation interval in a beacon period may be split into two portions as a result of shifting around the beacon period, but the aggregate length remains the same.

(a) 1-priodic allocation

(b) m-periodic allocation

Figure 86 — Beacon and scheduled allocation interval shifting

### Channel hopping

A hub may change its operating channel periodically by including the Channel Hopping State and Next Channel Hop fields in its beacons as defined in 6.3.1.8.  A hub should choose a channel hopping sequence that is not being used by its neighbor hubs.

The hub shall hop to another channel after dwelling on the current channel for a fixed number of beacon periods as communicated to the nodes connected with the hub through Connection Assignment frames.  The hub shall not hop to a new channel in the middle of a beacon period.

A hub shall generate a channel hopping sequence based on the maximum-length Galois linear feedback shift register (LFSR) defined in Figure 87 and by the following generator polynomial

x16 + x14 + x13 + x11 + 1

The state of the LFSR at stage k is given by

Yk = 20×rk,0 + 21×rk,1 + … + 215×rk,15

Yk represents the binary value read from the bits rk,0, rk,1, …, rk,15 of the individual registers at stage k, with rk,0 being the least-significant bit and rk,15 being the most-significant bit.

Figure 87 — 16-bit Galois LFSR for channel hopping sequence generation

Given the current state Yk of the LFSR, the hub shall generate the next state Yk+1 of the LFSR, i.e., the state of the LFSR at the next stage, stage k+1.  Accordingly, the hub shall generate the channel number Ck+1 of the next channel from the channel number Ck of the current channel and the next state Yk+1 of the LFSR as follows:

Zk+1 = Yk+1 mod Nreduced

Ck+1 = (Ck + Zk+1 + Nsep) mod Nch

In the above, the notation mod denotes modular operation.  Nch = pChannelsTotal is the number of total channels in the operating frequency band as listed in Table 18.  Nsep = pChannelSeparation is the minimum number of channels separated between two consecutive hops as illustrated in Figure 88.  Nreduced  is the number of channels available for each hop on account of the channel separation constraint Nsep, and is given by

Nreduced = Nch – 2Nsep + 1

The channels are numbered from 0 through Nch – 1.  Given the current channel number Ck, the next channel number Ck+1 will be such that |Ck+1 – Ck| ≥ Nsep.  All channels are selected with equal probability, as also shown in Figure 88.

Figure 88 — Channel separation in consecutive hops

The hub shall set the initial state Y0 of its LFSR to the 16 least-significant bits of its IEEE MAC address, with r0,0 corresponding to the least-significant bit.  The hub shall select the channel number C0 of its initial channel as follows:

C0 = Y0 mod Nch

To remain connected with a hub, a node shall hop to the same channel as the hub.

If required for regulatory compliance, a hub shall select a channel based on the applicable regulatory requirements, and shall dwell on the channel for an indefinite period of time.  To communicate this selection to nodes, the hub shall set the channel hopping state to 0.  To communicate with the hub, a node shall also dwell on the current channel for an indefinite period of time as well.  This is equivalent to setting Nsep = pChannelSeparation to 0 in calculating the next channel number from the current channel number based on the equations given in the above.

### Time sharing and load reduction

[JMH’s note:  The following “coexistence” text was provided by Ranjeet.  All of the mechanisms specified therein are intended to be optional, just as the other coexistence mechanisms described earlier.  The subclause heading I coined in the above does not seem to cover the scope of that text.]

Co-existence

A prospective BAN coordinator shall select a channel to operate prior to starting a new BAN. Coordinator may initiate a channel scan over a given list of channels in a given frequency band to select a channel for network operation. Coordinator may perform energy detection based channel scan or active channel scan or passive channel scan or combination thereof to select a channel for network operation.

In energy detection (ED) based scanning method; coordinator shall repeatedly perform energy detection based measurement in a channel for mEDScanDuration duration. The maximum energy measurement obtained during this duration shall be noted by the coordinator before moving on to the next channel in the channel list. A coordinator shall be able to store between one and an implementation-specified maximum number of channel energy measurements. The ED scan shall terminate when either the number of channel energy measurements stored equals the implementation-specified maximum or peak energy detected in a channel is lesser than the energy detection threshold. Coordinator may select an unoccupied channel for network operation, at the termination of ED scan.

In active scan based method, the coordinator shall select a channel, and send a BAN enquiry frame. Upon successful transmission of the BAN enquiry frame, the coordinator shall enable its receiver for at least mActiveScanDuration duration and wait for BAN enquiry response frame. BAN enquiry response should be transmitted by an existing coordinator in response to the BAN enquiry frame from a BAN coordinator. If no BAN enquiry response frame is received in mActiveScanDuration duration a channel, coordinator shall monitor the network activity for mNetworkActivityDuration duration in the same channel.

During mActiveScanDuration duration, coordinator shall reject all frames and record the information contained in BAN enquiry response frame, when received, before moving on to the next channel in the channel list. The BAN enquiry response frame shall contain a BAN descriptor pertained to network co-existence. A coordinator shall be able to store between one and an implementation-specified maximum number of BAN descriptor received from BAN enquiry response frames.

Active scan shall terminate either when the number of active channel scan equals the implementation-specified maximum or when no enquiry response frame is received in mActiveScanDuration duration and followed by no network activity is detected for mNetworkActivityDuration duration in a channel. Coordinator may select an unoccupied channel for network operation, at the termination of active scan.

In passive channel scan, coordinator shall try to locate beacon or B2 message in a channel for mPassiveScanDuration duration.  The inactive duration field indicated in the beacon and the BAN descriptor field in the B2 message may be used for BANs’ co-existence when applicable. The coordinator shall record BAN descriptor before moving on to the next channel in the channel list. A coordinator shall be able to store between one and an implementation-specified maximum number of BAN descriptor. The passive scan shall terminate when the number of BAN descriptor stored equals the implementation-specified maximum. Coordinator may select an unoccupied channel for network operation, at the termination of passive scan.

Coordinator shall select an occupied channel when no unoccupied channel is selected at the termination of ED scan or Active scan or Passive scan or combination thereof. Coordinator shall utilize the information gathered during the ED scan and active scan and passive scan to select an occupied channel and operate in Time Sharing or Offset Synchronization or Interference Mitigation mode.

Time sharing mode:

When all channels are occupied, coordinator shall select an occupied channel for network operation to operate in time sharing mode.

To operate in time sharing mode, coordinator shall perform passive scan if not performed and record BAN descriptor from all channels, and select an occupied channel for network operation. The BAN descriptor field in B2 message contains coordinator address, length of contention access period in current superframe, beacon period, offset to next superframe start time and minimum reservation period and priority of the piconet.

Upon selecting an occupied channel and finding an existing BAN coordinator, coordinator shall send connection request frame to the existing BAN coordinator, using CAP/RAP1/RAP2 access mechanism, with recipient address as existing coordinator address, source address as its own address and with its allocation requirement. The coordinator shall use the BAN descriptor in B2 message or fields in Beacon message to arrive at the allocation requirement.

Upon receiving coexistence request frame, existing coordinator shall respond with coexistence assignment frame to the prospective coordinator, by providing exclusive time duration or rejecting the coexistence assignment request, to the prospective coordinator. Existing coordinator may adjust assignment of its connected device for allocation of exclusive time duration to the new coordinator.  Existing coordinator shall use the priority information of prospective coordinator while responding with the coexistence assignment frame. For prioritization, coordinator with only medical device should have priority over coordinator with medical and non-medical devices; coordinator with medical and non-medical devices should have priority over coordinator with only non-medical devices.

Offset BAN synchronization mode:

When existing coordinator refuses to allow releasing its time resource to other coordinator, due to limited time resources being available on existing BAN or due to existing coordinator having an equal or higher priority than prospective, the prospective coordinator shall choose to operate in offset BAN synchronization mode.

Upon selecting an occupied channel and finding an existing BAN coordinator, coordinator shall send connection request frame to the existing BAN coordinator, using CAP/RAP1/RAP2 access mechanism, with recipient address as existing coordinator address, source address as its own address and with its duty cycle and PHY data rate requirement. The coordinator shall use the BAN descriptor in B2 message or fields in Beacon message to arrive at the data rate/duty cycle requirements. Upon receiving coexistence request frame, existing coordinator shall respond with coexistence response frame, allowing or rejecting the coexistence.

This mode of operation is possible when existing coordinator is willing to adjust its data rates or limit its duty cycle based on the request from the prospective coordinator.

Interference mitigation mode for unsynchronized BANs:

The prospective coordinator can use this mode, when coexistence request is rejected for time sharing mode and offset synchronization mode. In this mode, Piconets cannot or do not wish to interact, hence no message exchange is required in this mode. A coordinator shall start in an unsynchronized manner and may receive transmissions from other Piconets and may have overlapping transmissions with other Piconets. When possible, the coordinator shall select a logical channel for network operation while minimizing the impact on the existing Piconets. When possible, piconet shall perform passive scan on all logical channels in all frequency bands. Once a particular piconet is found to already exist, the device will estimate piconet information such as the number of devices on that piconet to obtain a traffic estimate, the received signal strength indication to figure out how far away the devices on that piconet, existing data rate used by devices in the piconet and priority information related to the existing piconet.  The current device may obtain this information either by listening to beacon message or B2 message in logical channels. At the end of the passive scan, the device trying to establish a new piconet will look at the collected information from the scan and make a decision about selecting a new logical channel and frequency band. Note that logical channel is different than the channel term used in the specification and it is specific to UWB physical layer.

Short-term coexistence:

For dynamic or semi-dynamic network scenario, coordinator may not select at short term any of the coexistence mechanism as discussed in the previous section and may use load control based protocol for short-term co-existence with existing coexistence mechanisms or without any existing ones.

An existing coordinator shall trigger load control, when inter-piconet interference is severe, and where severe inter-piconet interference is recognized by increase in packet error rate, sever drop in the signal strength for a sufficient long duration or by tracking the overflow of total used capacity only when inter-piconet message exchange is possible. Coordinator may perform load control by reducing traffic load from connected nodes or by disconnecting connected nodes or combination of reducing traffic from connected nodes and disconnecting connected nodes.

To reduce traffic from one or multiple connected nodes, coordinator shall send unsolicited connection assignment frame to nodes with reduced slot assignment and change indication field set to 1. The priority should be given to nodes with medical traffic over nodes with non-medical traffic for reducing traffic load. Coordinator may perform prioritization among the nodes with medical traffic for reducing traffic load. Coordinator may perform prioritization among the nodes with non-medical traffic for reducing traffic load.

To disconnect one or multiple nodes, coordinator shall send disconnection frame to nodes. The priority should be given to nodes with medical traffic over nodes with non-medical traffic for disconnecting nodes. Coordinator may perform prioritization among the nodes with medical traffic for disconnecting nodes. Coordinator may perform prioritization among the nodes with non-medical traffic for disconnecting nodes.

Coordinator shall perform Additive Increase and Multiplicative Decrease slot assignment/node connection to change and restore the traffic load with different rate according to traffic class. The rule should be designed for piconets to be operated fairly.

## MAC / PHY capability handling and Application Specific IE usage

A node or a hub shall be ready to carry out a function that it has indicated to be capable of supporting in its last transmitted MAC Capability or PHY Capability field.  A node or a hub shall not initiate or execute a function that its communication partner has indicated to be incapable of supporting through the last MAC Capability or PHY Capability field sent by the partner.

To transmit beacons, a hub shall use the data rate equal to pBeaconDataRate.  To transmit Poll and T-Poll frames addressed to Unconnected_Broadcast_NID, a hub shall use the lowest data rate in the operating frequency band mandated for the underlying PHY.

A hub may include one or more Application Specific IEs at the end of its beacon.

A node may include one or more Application Specific IEs at the end of its Connection Request frame.  A hub may also include one or more Application Specific IEs at the end of its Connection Assignment frame.

A recipient shall ignore unrecognized Application Specific IEs.

## MAC sublayer parameters

Table 17 provides the values for the MAC sublayer parameters.

Table 17 — MAC sublayer parameters

| mCSMADurationLimit | 2 × Allocation Slot Length |
| --- | --- |
| mClockAccuracy | 20 ppm |
| mClockResolution | 10 s |
| mG-AckDataSubtype | 15 |
| mGT_Nominal | Allocation Slot Length / 10 |
| mMaxChannelChangeTime | 256 beacon periods |
| mMaxFragmentCount | 16 |
| mMaxBANSize | 64 |
| mScheduledAllocationAborted | 32 |

Table 18 - Table 19 provide the values of the PHY dependent parameters used by the MAC sublayer.

Table 18 — PHY-dependent MAC sublayer parameters pertaining to narrowband PHY

| pAllocationSlotMin |  |
| --- | --- |
| pAllocationSlotResolution |  |
| pBeaconDataRate |  |
| pCCATime |  |
| pChannelSeparation |  |
| pChannelsTotal |  |
| pClockAccuracy |  |
| pCSMASlotLength |  |
| pMaxFrameBodyLength |  |
| pMICSAckTxTime |  |
| pMICSChannelsTotal |  |
| pMICSChannelSwitchTime |  |
| pMICSFrameSpace |  |
| pMICSHubMaxRetries |  |
| pMICSMcastPollRxTime |  |
| pMICSMcastPolls |  |
| pMICSMcastPollSeparation |  |
| pMICSNodeEmergencyRetries |  |
| pMICSPollRxTime |  |
| pMICSPolls |  |
| pMICSPollSeparation |  |
| pMICSPollTxTime |  |
| pMICSPreambleTxTime |  |
| pMICSPLCPHeaderTxTime |  |
| pMICSUnconnectedPollPeriod |  |
| pMICSUnconnectedPollRxTime |  |
| pMICSUnconnectedPolls |  |
| pMICSUnconnectedPollSeparation |  |
| pMICSUnconnectedPollTxTime |  |
| pRandomAccess | CSMA/CA |
| pSynchResolution |  |
| pSIFS |  |
| pUnconnectedPolledAllocationMin |  |

Table 19 — PHY-dependent MAC sublayer parameters pertaining to UWB PHY

| pAllocationSlotMin |  |
| --- | --- |
| pAllocationSlotResolution |  |
| pAlohaSlotLength |  |
| pBeaconDataRate |  |
| pCCATime |  |
| pClockAccuracy |  |
| pCSMASlotLength |  |
| pMaxFrameBodyLength |  |
| pRandomAccess | Slotted Aloha |
| pSynchResolution |  |
| pSIFS |  |
| pUnconnectedPolledAllocationMin |  |

# Security services

This clause expounds on the elements of the security hierarchy introduced in Figure 5.  Security in this standard starts with a negotiation of the desired security suite between the two communicating parties, a node and a hub.  The security selection in turn sets off a security association between the two parties for activating a pre-shared or generating a new shared master key (MK).  Several security association protocols suitable for a variety of use cases are provided in 8.1, which finishes off with security disassociation for legitimately repealing a shared master key between the two parties.  Pairwise temporal key (PTK) creation and group temporal key (GTK) distribution are then described in 8.2.

Treated in 8.3 is message security at the MAC level, i.e., message authentication and encryption, based on the Advanced Encryption Standard (AES) forward cipher function for 128-bit keys operating on counter mode and cipher block chaining (CBC) mode, respectively.  As part of message security, replay protection is also considered in this subclause.

## Master keys

The security association protocols specified in 8.1.2-8.1.5 shall be based on the Diffie-Hellman key exchange employing the elliptic curve public key cryptography.  The elliptic curve, characterized by the following equation

y2 = x3 + ax + b (mod p),  a, b   GF(p),  4a3 + 27b2 ≠ 0

where GF(p) is a prime finite field, shall have the following values for its coefficients and domain parameters, as specified for Curve P-192 in FIPS Pub186-2, with p (an odd prime), r (order of base point G), and a (a coefficient) given in decimal form, and coefficient b and base point G = (Gx, Gy) given in hex:

p = 6277101735386680763835789423207666416083908700390324961279

r = 6277101735386680763835789423176059013767194773182842284081

a = -3 mod p

b = 64210519 e59c80e7 0fa7e9ab 72243049 feb8deec c146b9b1

Gx = 188da80e b03090f6 7cbf20eb 43a18800 f4ff0afd 82ff1012

Gy = 07192b95 ffc8da78 631011ed 6b24cdd5 73f977a1 1e794811

The private keys (also called secret keys) SKA and SKB of the elliptic curve public key cryptography for the two communicating parties, a node and a hub, respectively, shall be chosen as unique 192-bit integers in the range [1, r-1].  The corresponding 192-bit public keys PKA and PKB shall be computed as follows:

PKA = SKA ×G, PKB = SKB ×G

where × denotes scalar multiplication of the base point G = (Gx, Gy)by an integer as described in A.9.2 of IEEE Std P1363-2000.  A received public key, denoted by an X-coordinate value or a pair of X-coordinate and Y-coordinate values, shall be treated valid only if it is a non-infinity point on the elliptic curve defined in the above, i.e., that its X and Y coordinates shall satisfy the elliptic curve equation given above.

On starting a security association procedure base on one of these security association protocols, the node and the hub shall independently generate a fresh 128-bit random number as their Sender Nonce used in the procedure.  Prior to initiating a security disassociation procedure as specified in 8.1.6, the initiator, the node or the hub, shall generate a fresh 128-bit random number as its Sender Nonce used in the procedure.

In these association protocols, the cipher-based message authentication code (CMAC) algorithm as specified in the NIST Special Publication 800-38B, with the AES forward cipher function under a 128-bit key as specified in FIPS Pub 197, is used to compute key message authentication codes (KMAC) and the desired shared master key.  Specifically, the functional notation CMAC(K, M) represents the 128-bit output of the CMAC applied under key K to message M based on the AES forward cipher function.

Moreover, the bit string truncation functions LMB_n(S) and RMB_n(S) designate the n leftmost and the n rightmost bits of the bit string S, respectively.  The sign || denotes concatenation of bit strings that are converted according to FIPS Pub 180-2 from certain fields of the frames of concern.

### Master key (MK) pre-shared association

A node and a hub shall each have a secret pre-shared MK prior to running the MK pre-shared association protocol to activate their pre-shared MK as their shared MK for their PTK creation, with the benefit of keeping third parties not possessing the secret MK from launching impersonation attacks via the PTK creation procedure.

To initiate a procedure for the MK pre-shared association protocol, the node shall transmit the first Security Association frame of the procedure, indicating its selected security suite in the frame.

To continue the association procedure, the recipient hub shall transmit the second Security Association frame of the procedure.  The hub may accept the security suite selected by the node.  Alternatively, the hub may propose a different one by setting the Security Suite Selector field to a desired value accordingly.

If the node receives the second Security Association frame containing a security suite selector different from that included in the first Security Association frame, it shall abort the current association procedure and may initiate a new association procedure taking into account the security suite selector it last received from the hub.

Upon successfully sending the second Security Association frame without changing the security suite selector, the hub shall activate the pre-shared MK as their shared MK, treating the node’s true identity as unauthenticated but the association procedure as completed.  Upon receiving the second Security Association frame with the same security suite selector as contained in the first one, the node shall also activate the pre-shared MK as their shared MK, treating the hub’s true identity unauthenticated but the association procedure completed as well.  The node shall proceed to the PTK creation procedure to create a PTK with the hub, meanwhile performing mutual authentication of each other based on the claimed pre-shared MK.

The MK pre-shared association procedure is illustrated in Figure 89.

Figure 89 — MK pre-shared association procedure

### Unauthenticated association

A node and a hub shall each require no authentication credentials such as a shared secret or human intervention prior to running the unauthenticated association protocol to generate their shared MK for their PTK creation, without the benefit of keeping third parties from launching impersonation attacks.

To initiate a procedure for the unauthenticated association protocol, the node shall transmit the first Security Association frame of the procedure, indicating its selected security suite in the frame.

To continue the association procedure, the recipient hub shall transmit the second Security Association frame of the procedure.  The hub may accept the security suite selected by the node, setting the MK_KMAC field of the Security Association Data as depicted in Figure 19 to MK_KMAC_2 as calculated below.  Alternatively, the hub may propose a different one by setting the Security Suite Selector field to a desired value accordingly, setting the MK_KMAC field to 0.

If the node receives the second Security Association frame containing a security suite selector different from that included in the first Security Association frame, it shall abort the current association procedure and may initiate a new association procedure taking into account the security suite selector it last received from the hub.

To complete the association procedure that was not aborted, the node shall send the third Security Association frame of the procedure, setting the MK_KMAC field of the Security Association Data as depicted in Figure 19 to MK_KMAC_3 as also calculated below.  The node shall send this Security Association frame only after it has received the second Security Association frame with the MK_KMAC field set to MK_KMAC_2.

Upon successfully sending the third Security Association frame, the node shall compute the shared MK as given below, treating the hub’s true identity as unauthenticated but the association procedure as completed.  Upon receiving the third Security Association frame with the MK_KMAC field set to MK_KMAC_3, the hub shall also compute the shared MK as given below, treating the node’s true identity unauthenticated but the association procedure completed as well.

The node and the hub shall each compute a DHKey as follows:

DHKey = X(SKA×PKB) = X(SKB×PKA) = X(SKA×SKB×G)

The node and the hub shall each derive MK_KMAC_2 and MK_KMAC_3 as follows:

P_2 = CMAC(DHKey, Address_A || Address_B || Nonce_A || Nonce_B ||Security_Suite_Selector)

P_3 = CMAC(DHKey, Address_B || Address_A || Nonce_B || Nonce_A||Security_Suite_Selector)

MK_KMAC_2 = LMB_64(P2), MK_KMAC_3 = LMB_64(P3)

The node and the hub shall each derive their shared MK as follows:

MK = CMAC(DHKey, Nonce_A || Nonce_B)

In the above, X(P) = X(PX, PY) = PX = X-coordinate of P, which is computed from SKA×PKB at the node and from SKB×PKA at the hub, respectively.

  - SKA is the node’s 192-bit private key (an integer) kept secret by the node.

  - SKB is the hub’s 192-bit private key kept secret by the hub.

  - PKA is the node’s 192-bit public key (a pair of X and Y coordinates) transmitted by the node.

  - PKB is the hub’s 192-bit public key (a pair of X and Y coordinates) transmitted by the hub.

  - Address_A is the Sender Address field of the frame payload of the first Security Association frame.

  - Address_B is the Recipient Address field of the frame payload of the first Security Association frame.

  - Nonce_A is the Sender Nonce field of the frame payload of the first Security Association frame.

  - Nonce_B is the Sender Nonce field of the frame payload of the second Security Association frame.

  - Security_Suite_Selector is the Security Suite Selector field of the frame payload of the first Security Association frame.

The unauthenticated association procedure is illustrated in Figure 90.

Figure 90 — Unauthenticated association procedure

### Public key hidden association

A node and a hub shall have a secret transfer of the node’s public key to the hub, typically through an out-of-band channel, prior to running the public key hidden association protocol to generate their shared MK for their PTK creation, with the benefit of keeping third parties from launching impersonation attacks.

To initiate a procedure for the public key hidden association protocol, the node shall transmit the first Security Association frame of the procedure, indicating its selected security suite in the frame.

To continue the association procedure, the recipient hub shall transmit the second Security Association frame of the procedure.  The hub may accept the security suite selected by the node, setting the MK_KMAC field of the Security Association Data as depicted in Figure 19 to MK_KMAC_2 as calculated below.  Alternatively, the hub may propose a different one by setting the Security Suite Selector field to a desired value accordingly, setting the MK_KMAC field to 0.

If the node receives the second Security Association frame containing a security suite selector different from that included in the first Security Association frame, it shall abort the current association procedure and may initiate a new association procedure taking into account the security suite selector it last received from the hub.

To complete the association procedure that was not aborted, the node shall send the third Security Association frame of the procedure, setting the MK_KMAC field of the Security Association Data as depicted in Figure 19 to MK_KMAC_3 as also calculated below.  The node shall send this Security Association frame only after it has received the second Security Association frame with the MK_KMAC field set to MK_KMAC_2.

Upon successfully sending the third Security Association frame, the node shall compute the shared MK as given below, treating the hub’s true identity as authenticated and the association procedure as completed.  Upon receiving the third Security Association frame with the MK_KMAC field set to MK_KMAC_3, the hub shall also compute the shared MK as given below, treating the node’s true identity authenticated and the association procedure completed as well.

The node and the hub shall each compute a DHKey as follows:

DHKey = X(SKA×PKB) = X(SKB×PKA) = X(SKA×SKB×G)

The node and the hub shall each derive MK_KMAC_2 and MK_KMAC_3 as follows:

P_2 = CMAC(DHKey, Address_A || Address_B || Nonce_A || Nonce_B ||Security_Suite_Selector)

P_3 = CMAC(DHKey, Address_B || Address_A || Nonce_B || Nonce_A||Security_Suite_Selector)

MK_KMAC_2 = LMB_64(P2), MK_KMAC_3 = LMB_64(P3)

The node and the hub shall each derive their shared MK as follows:

MK = CMAC(DHKey, Nonce_A || Nonce_B)

In the above, X(P) = X(PX,PY) = PX = X-coordinate of P, which is computed from SKA×PKB at the node and from SKB×PKA at the hub, respectively.

  - SKA is the node’s 192-bit private key (an integer) kept secret by the node.

  - SKB is the hub’s 192-bit private key kept secret by the hub.

  - PKA is the node’s 192-bit public key (a pair of X and Y coordinates) transferred only to the hub by a secure out-of-band channel.

  - PKB is the hub’s 192-bit public key (a pair of X and Y coordinates) transmitted by the hub.

  - Address_A is the Sender Address field of the frame payload of the first Security Association frame.

  - Address_B is the Recipient Address field of the frame payload of the first Security Association frame.

  - Nonce_A is the Sender Nonce field of the frame payload of the first Security Association frame.

  - Nonce_B is the Sender Nonce field of the frame payload of the second Security Association frame.

  - Security_Suite_Selector is the Security Suite Selector field of the frame payload of the first Security Association frame.

The public key hidden association procedure is also illustrated in Figure 90 as well.

### Password authenticated association

A node and a hub shall each have a secret shared password prior to running the password authenticated association protocol to generate their shared MK for their PTK creation, with the benefit of keeping third parties not possessing the secret password from launching impersonation attacks.

To initiate a procedure for the password authenticated association protocol, the node shall transmit the first Security Association frame of the procedure, indicating its selected security suite in the frame , and setting the Sender PKX and Sender PKY fields of the Security Association Data as depicted in Figure 19 to the X-coordinate PK'AX and Y-coordinate PK'AY, respectively, of the node’s password-scrambled public key PK'A as calculated below.

To continue the association procedure, the recipient hub shall transmit the second Security Association frame of the procedure.  The hub may accept the security suite selected by the node, setting the MK_KMAC field of the Security Association Data to MK_KMAC_2 as calculated below.  Alternatively, the hub may propose a different one by setting the Security Suite Selector field to a desired value accordingly, setting the MK_KMAC field to 0.

If the node receives the second Security Association frame containing a security suite selector different from that included in the first Security Association frame, it shall abort the current association procedure and may initiate a new association procedure taking into account the security suite selector it last received from the hub.

To complete the association procedure that was not aborted, the node shall send the third Security Association frame of the procedure, setting the Sender PKX and Sender PKY fields of the Security Association Data as depicted in Figure 19 to the values of the corresponding fields of the Security Association Data in the first Security Association frame of the procedure, and setting the MK_KMAC field of the Security Association Data to MK_KMAC_3 as also calculated below.  The node shall send this Security Association frame only after it has received the second Security Association frame with the MK_KMAC field set to MK_KMAC_2.

Upon successfully sending the third Security Association frame, the node shall compute the shared MK as given below, treating the hub’s true identity as authenticated and the association procedure as completed.  Upon receiving the third Security Association frame with the MK_KMAC field set to MK_KMAC_3, the hub shall also compute the shared MK as given below, treating the hub’s true identity authenticated and the association procedure completed as well.

The node shall compute its password-scrambled public key PK'A = (PK'AX, PK'AY) from its public or private key and the password shared with the hub as follows:

PK'A  = PKA – (MX + 1)×Q(PW) = SKA×G – (MX + 1)×Q(PW)

Q(PW) =  (QX = PW+MX, QY = even positive integer)

The hub shall recover the node’s public key from the received password-scrambled public key PK'A = (PK'AX, PK'AY) for the subsequent DHKey computation as follows:

PKA = PK'A + (MX + 1)×Q(PW),   Q(PW) =  (QX = PW+MX, QY = even positive integer)

The parameters involved in these equations are defined below:

  - PW is a positive integer converted according to IEEE Std P1363-2000 from the UTE-16BE representation of the shared password by treating the leftmost octet as the octet containing the most-significant bits.

  - MX is the smallest nonnegative integer such that QX = PW+MX is the X-coordinate of a point on the elliptic curve defined earlier.

  - Q(PW) is the point on the elliptic curve with X-coordinate = QX and Y-coordinate = QY of an even positive integer.

The node shall choose a private key SKA such that the X-coordinate of PKA is not equal to the X-coordinate of (MX + 1)×Q(PW).

The node and the hub shall each compute a DHKey as follows:

DHKey = X(SKA×PKB) = X(SKB×PKA) = X(SKA×SKB×G)

The node and the hub shall each derive MK_KMAC_2 and MK_KMAC_3 as follows:

P_2 = CMAC(DHKey, Address_A || Address_B || Nonce_A || Nonce_B ||Security_Suite_Selector)

P_3 = CMAC(DHKey, Address_B || Address_A || Nonce_B || Nonce_A ||Security_Suite_Selector)

MK_KMAC_2 = LMB_64(P2), MK_KMAC_3 = LMB_64(P3)

The node and the hub shall each derive their shared MK as follows:

MK = CMAC(DHKey, Nonce_A || Nonce_B)

In the above, X(P) = X(PX,PY) = PX = X-coordinate of P, which is computed from SKB×PKA at the hub and from SKA×PKB at the node, respectively.

  - SKA is the node’s 192-bit private key (an integer) kept secret by the node.

  - SKB is the hub’s 192-bit private key kept secret by the hub.

  - PKA is the node’s 192-bit public key (a pair of X and Y coordinates) kept secret by the node.

  - PKB is the hub’s 192-bit public key (a pair of X and Y coordinates) transmitted by the hub.

  - Address_A is the Sender Address field of the frame payload of the first Security Association frame.

  - Address_B is the Recipient Address field of the frame payload of the first Security Association frame.

  - Nonce_A is the Sender Nonce field of the frame payload of the first Security Association frame.

  - Nonce_B is the Sender Nonce field of the frame payload of the second Security Association frame.

  - Security_Suite_Selector is the Security Suite Selector field of the frame payload of the first Security Association frame.

The password authenticated association procedure is illustrated in Figure 91.

Figure 91 — Password authenticated association procedure

### Display authenticated association

A node and a hub shall each have a display of a 5-digit decimal number prior to running the display authenticated association protocol to generate their shared MK for their PTK creation, with the benefit of keeping third parties not displaying the same generated secret from launching impersonation attacks.

To initiate a procedure for the display authenticated association protocol, the node shall transmit the first Security Association frame of the procedure, indicating its selected security suite in the frame, setting the MK_KMAC field of the Security Association Data as depicted in Figure 19 to Witness as calculated below.

To continue the association procedure, the recipient hub shall transmit the second Security Association frame of the procedure.  The hub may accept the security suite selected by the node or propose a different one by setting the Security Suite Selector field to a desired value accordingly.

If the node receives the second Security Association frame containing a security suite selector different from that included in the first Security Association frame, it shall abort the current association procedure and may initiate a new association procedure taking into account the security suite selector it last received from the hub.

To complete the association procedure that was not aborted, the node shall send the third Security Association frame of the procedure.  The hub shall not treat this frame as valid and shall display a number of 0 if the MK_KMAC field contained in the received first Security Association frame of the procedure is not equal to Witness as given below.

The node shall display a 5-digit decimal number Display_A immediately after successfully sending the third Security Association frame, and the hub shall also display a 5-digit decimal number Display_B immediately after determining the received third Security Association frame to be valid.  If the node and the hub display the same 5-digit number, they shall each be informed through their respective user interfaces that their mutual authentication has succeeded.  Otherwise, they shall each be informed that their mutual authentication has failed.

Upon determining that their mutual authentication has succeeded, the node and the hub shall each compute the shared MK as given below, treating their authenticated association procedure as completed.

The node and the hub shall each compute a DHKey as follows:

DHKey = X(SKA×PKB) = X(SKB×PKA) = X(SKA×SKB×G)

The node and the hub shall each derive Witness as follows:

Witness = CMAC(Nonce_A, Address_A || Address_B || PKAX || PKAY)

The node and the hub shall also compute Display_A and Display_B, respectively, as follows:

H = CMAC(DHKey, Address_A || Address_B || Nonce_A || Nonce_B ||Security_Suite_Selector)

D = RMB_16(H)

Display_A = BS2DI(D),   Display_B = BS2DI(D)

The node and the hub shall each derive their shared MK as follows:

MK = CMAC(DHKey, Nonce_A || Nonce_B)

In the above, X(P) = X(PX,PY) = PX = X-coordinate of P, which is computed from SKA×PKB at the node and from SKB×PKA at the hub, respectively.  BS2DI(BS) converts the bit string BS, based on IEEE Std P1363-2000, to a positive decimal integer by treating the leftmost bit of the string as the most-significant bit of the integer.

  - SKA is the node’s 192-bit private key (an integer) kept secret by the node.

  - SKB is the hub’s 192-bit private key kept secret by the hub.

  - PKA is the node’s 192-bit public key (a pair of X and Y coordinates) transmitted by the node in the third Security Association frame.

  - PKB is the hub’s 192-bit public key (a pair of X and Y coordinates) transmitted by the hub in the second Security Association frame.

  - PKAX and PKAY are the X-coordinate and Y-coordinate, respectively, of the node’s 192-bit public key (a pair of X and Y coordinates) contained in the third Security Association frame.

  - Address_A is the Sender Address field of the frame payload of the third Security Association frame.

  - Address_B is the Recipient Address field of the frame payload of the third Security Association frame.

  - Nonce_A is the Sender Nonce field of the frame payload of the third Security Association frame.

  - Nonce_B is the Sender Nonce field of the frame payload of the second Security Association frame.

  - Security_Suite_Selector is the Security Suite Selector field of the frame payload of the first Security Association frame.

The display authenticated association procedure is illustrated in Figure 92.

Figure 92 — Display authenticated association procedure

### Disassociation

To initiate a security disassociation procedure for nullifying an existing security association and hence the shared MK and PTK with a hub or a node, the node or the hub shall send a Security Disassociation frame, setting the DA_KMAC field of the frame payload as depicted in Figure 20 to DA_KMAC.  Upon successfully sending the Security Disassociation frame, the sender shall erase the MK and the corresponding PTK materials from its internal storage.

Upon receiving a Security Disassociation frame with the DA_KMAC field set to DA_KMAC, the recipient shall also erase the MK and the corresponding PTK materials from its internal storage.

To send a Security Disassociation, the sender shall independently generate a new 128-bit cryptographic random number as its Sender Nonce in the Security Disassociation frame.

The node and the hub shall compute DA_KMAC as follows:

DA_KMAC =  CMAC(MK, Address_A || Address_B || Nonce_A)

The input fields to the computation above are defined as follows:

  - MK is the shared MK to be repealed.

  - Address_A is the Sender Address field of the frame payload of the Security Disassociation frame.

  - Address_B is the Recipient Address field of the frame payload of the Security Disassociation frame.

  - Nonce_A is the Sender Nonce field of the frame payload of the Security Disassociation frame.

The security disassociation procedure is illustrated in Figure 93.

Figure 93 — Security Disassociation procedure

## Temporal keys

On starting a pairwise temporal key (PTK) creation procedure as described in , the two communicating parties, a node and a hub, shall independently generate a fresh 128-bit random number as their Sender Nonce used in the procedure.  On starting a group temporal key (GTK) distribution procedure as described in 8.2.2, the hub shall generate a fresh 128-bit cryptographic grade random number as the GTK if the GTK is to be distributed the first time.

In the PTK creation procedure, the cipher-based message authentication code (CMAC) algorithm as specified in the NIST Special Publication 800-38B, with the AES forward cipher function under a 128-bit key as specified in FIPS Pub 197, is used to compute key message authentication codes (KMAC).  Specifically, the functional notation CMAC(K, M) represents the 128-bit output of the CMAC applied under key K to message M based on the AES forward cipher function.

Moreover, the bit string truncation functions LMB_n(S) and RMB_n(S) designate the n leftmost and the n rightmost bits of the bit string S, respectively.  The sign || denotes concatenation of bit strings that are converted according to FIPS Pub 180-2 from certain fields of the frames of concern.

### PTK creation

A node and a hub shall have a 128-bit secret shared master key (MK) resulting from a successful run of a security association protocol such as provided in 8.1 prior to running a pairwise temporal key (PTK) creation procedure to generate a PTK for exchanging secured frames with each other.

To initiate a PTK creation procedure, the node or the hub—referred to as the initiator—shall transmit the first PTK frame of the procedure to the intended recipient—referred to as the responder.

To continue the PTK creation procedure, the responder shall transmit the second PTK frame of the procedure, setting the PTK_KMAC field of the frame payload as depicted in Figure 21 to PTK_KMAC_2 as calculated below.  The responder shall set the PTK_KMAC field to 0 if it does not have a shared MK with the initiator.

To complete the PTK creation procedure, the initiator shall send the third PTK frame of the procedure, setting the PTK_KMAC field of the frame payload as depicted in Figure 21 to PTK_KMAC_3 as also calculated below.  The initiator shall send this PTK frame only after it has received the second PTK frame with the PTK_KMAC field set to PTK_KMAC_2.

Upon successfully sending the third PTK frame, the initiator shall compute a new PTK as given below, treating the responder’s true identity as authenticated and the PTK creation procedure as completed.  Upon receiving the third PTK frame with the PTK_KMAC field set to PTK_KMAC_3, the responder shall also compute the new PTK, treating the initiator’s true identity authenticated and the PTK creation procedure completed as well.

The initiator and the responder shall independently generate a new 128-bit cryptographic random number as their Sender Nonce in a PTK creation procedure.

The initiator and the responder shall each derive the PTK, KCK, PTK_KMAC_2, and PTK_KMAC_3 as follows:

PTK = CMAC(MK, Address_I || Address_R || Nonce_I || Nonce_R ||  PTK_Index)

KCK = CMAC(MK, Address_R || Address_I || Nonce_R || Nonce_I || PTK_Index)

P = CMAC(KCK, Address_I || Address_R || Nonce_R || Nonce_I || PTK_Index)

PTK_KMAC_2 = LMB_64(P), PTK_KMAC_3 = RMB_64(P)

The fields that form the message of CMAC correspond to the fields in the PTK frames of the current PTK creation procedure and are converted to bit strings according to FIPS Pub 180-2:

  - Address_I is the Sender Address field of the frame payload of the first PTK frame.

  - Address_R is the Recipient Address field of the frame payload of the first PTK frame.

  - Nonce_I is the Sender Nonce field of the frame payload of the first PTK frame.

  - Nonce_R is the Sender Nonce field of the frame payload of the second PTK frame.

  - PTK_Index is the PTK Index field of the frame payload of the first PTK frame.

The PTK creation procedure is illustrated in Figure 94.

Figure 94 — PTK creation procedure

### GTK distribution

A node and a hub shall have a pairwise temporal key (PTK) prior to running a group temporal key (GTK) distribution procedure to transport a GTK to the node for multicasting secured frames to the node and others.

To send the GTK to the node, the hub shall send to the node a GTK frame containing the GTK and secured with the PTK being used between the hub and the node.

## Message security

A hub shall transmit a beacon, if appropriate, as an unsecured frame or as a secured frame authenticated, but not encrypted, by a group temporal key (GTK) distributed to the nodes that are secured with it.  Nodes that do not have a secured relationship with the hub may receive and process the beacon without validating the message integrity code (MIC) included therein.

At their Secured or Connected State, a node and a hub shall exchange and process only secured frames at a security level negotiated during their last association, with the following additional considerations:

  - GTK frames shall always be secured, both authenticated and encrypted.

  - Poll frames shall never be authenticated or encrypted.

  - Control type frames other than Poll frames

    - shall be neither authenticated nor encrypted if the hub and the node have agreed to applying no control type frame authentication;

    - shall be authenticated but not encrypted if the hub and the node have agreed to applying control type frame authentication.

A recipient shall ignore a received frame with an unexpected security level, other than performing acknowledgment if needed.  A recipient shall also ignore a received secured frame with an invalid MIC, i.e., the MIC value calculated from the received frame as described in 8.3.1.5 is not the same as the MIC field contained in the received frame, except again for returning an acknowledgment.

### Frame authentication, encryption, and decryption

Secured frames shall be authenticated, and encrypted/decrypted when required, based on AES-128 CCM, i.e., the CCM mode as specified in the NIST Special Publication 800-38C, with the AES forward cipher function for 128-bit keys as specified in FIPS Pub 197 applied as the underlying block cipher algorithm.

Prior to exchanging secured unicast frames, the two communicating parties, a node and a hub, shall have a pairwise temporal key (PTK) for use as the AES key applied to the unicat frames.  They may have an additional PTK before the current PTK is retired.  Prior to multicast secured frames to a group, the hub shall have distributed a group temporal key (GTK) to the nodes of the group for use as the AES key applied to the multicast frames.

A temporal key, PTK or GTK, shall be retired no later than when the Security Sequence Number field of the last frame secured by the key has reached the maximum value supported by the field.  It may be retired earlier as needed.

The length of what is referred to as the Message Authentication Code (MAC) for message (frame) authentication in NIST Special Publication 800-38C but as the Message Integrity Code (MIC) in this standard—to be distinguished from another accustomed standing of the term MAC for medium access control—shall be four octets.  That is, in the NIST Special Publication 800-38C, t = 4.  Also, q = 2 shall be chosen as the octet length of the binary representation of the octet length of the frame payload.

The bit order of each input block to the CCM invocation and AES encryption shall be formatted as illustrated in Figure 95.  It is the concatenation of the bits orders of the ordered octets of the constituent fields of the block, where the octet order of each constituent field is defined below, and the bits of each octet are ordered such that the most-significant bit (msb) is the first bit of the octet while the least-significant bit (lsb) is the last bit of the octet. The first octet or the first bit of a given component is shown on the left, and the last octet or the last bit is shown on the right, in the context of the component.  The bit notations input0, …, input127 correspond to those used for AES input block formation specified in FIPS Pub 197.

Figure 95 — Bit order for AES input blocks

#### Nonce formation

The Nonce as a required input field to each instance of CCM frame authentication and encryption/decryption is a 13-octet field that is formatted as shown in Figure 96.  Here, the octets of the MAC header and Security Number fields are each ordered from left to right in accordance with their transmit order as defined in 6.1 and 6.2.

Figure 96 — Nonce format

#### Initial block B0 construction

The block B0 as the first input block to the cipher block chaining (CBC) for frame authentication, i.e., MIC computation, is a 16-octet field that is formatted as shown in Figure 97.  Here, Q = L_FP is the octet length of the frame payload as defined in Figure 12 and is encoded with the octet containing the most-significant bits on the left and the octet containing the least-significant bits on the right.

Only this block is present if the current frame does not have a frame payload.

(a) Frame payload encrypted

(b) Frame payload not encrypted

Figure 97 — Initial block B0 format

#### Payload blocks B1, …, Bm construction

The blocks B1, …, Bm as the subsequent input blocks to the CBC frame authentication, and also as the input blocks to the counter mode encryption/decryption, i.e., ciphertext computation and plain text recovery, if the frame payload is to be encrypted/decrypted, are each a 16-octet field that is formatted as shown in Figure 98.  Here, a =L_FP is the octet length of the frame payload as defined in Figure 12 and is encoded with the octet containing the most-significant bits on the left and the octet containing the least-significant bits on the right, and the Frame Payload field is ordered from left to right in consistency with it transmit order as defined in 6.1 and 6.2.

These blocks are constructed from the unencrypted or decrypted frame payload.  The last block contains one or more padded zero octets on the right end if the frame payload is not an integral multiple of 16 octets.

None of these blocks is present if the current frame does not have a frame payload.

(a) Frame payload encrypted

(b) Frame payload not encrypted

Figure 98 — Payload blocks B1, …, Bm format

#### Counter blocks Ctr0, …, Ctrm formation

The block Ctr0 as the input block to the counter mode encryption of the CBC output for MIC computation, and each of the blocks Ctr1…, Ctrm as the input blocks to the counter mode encryption/decryption if the frame payload is to be encrypted/decrypted, is a 16-octet field that is formatted as shown in Figure 99.  Here, i = 0, …, m, respectively, and is encoded with the octet containing the most-significant bits on the left and the octet containing the least-significant bits on the right.

Figure 99 — Counter blocks Ctr0, …, Ctrm format

#### MIC computation

The MIC field in an authenticated frame is calculated as shown in Figure 100, where

MIC = LMB_n(M),   M = AES(Ctr0)  Xm

X0 = AES(B0),  Xi = AES(Bi  Xi-1), i = 1, …, m

Here, LMB_n(M) designates the n leftmost bits of the bit string M, the symbol  denotes bitwise exclusive-OR, and AES(B) represents the output of the forward cipher function of the AES block cipher algorithm  applied to block B under the AES key PTK or GTK used to secure the frame.  The MIC is ordered for transmission from its first octet on the left to its last octet on the right, as also illustrated in Figure 100.  The octet notations out0 , …, out15 correspond to those used for AES output block formation specified in FIPS Pub 197.

The blocks required for the MIC computation are constructed from the unencrypted version of the frame to be transmitted at the sender side, and from the decrypted version of the received frame at the recipient side if the frame is encrypted.

Figure 100 — MIC calculation and transmit order

#### Frame payload encryption

The encrypted frame payload in an encrypted frame is formatted as shown in Figure 101, where

B'i = Bi  AES(Ctri), i = 1, …, m-1

B'm = L_n(Bm)  L_n(AES(Ctrm))

Here, the symbol  denotes bitwise exclusive-OR, and L_n(B) designates the n leftmost octets of B.  Moreover, AES(Ctri) represents the output of the forward cipher function of the AES block cipher algorithm applied to the counter block Ctri under the AES key PTK or GTK used to secure the frame.  The encrypted frame payload has the same length as the unencrypted frame payload, so that n ≤ 16 is the number of octets in Bm excluding the zero padding octets if any.

Each encrypted block is ordered for transmission from its first octet on the left to its last octet on the right, as also illustrated in Figure 101.  The octet notations out0, …, out15 correspond to those used for AES output block formation specified in FIPS Pub 197.

Figure 101 — Encrypted Frame Payload format for encrypted frames

#### Frame payload decryption

The frame payload in an encrypted frame is decrypted as shown in Figure 102, where

Bi = B'i  AES(Ctri), i = 1, …, m-1

B–m = B'm  L_n(AES(Ctrm))

The decrypted frame payload has the same length as the encrypted frame payload, so that n ≤ 16 is the number of octets in the last block B'm of the encrypted frame payload received.  The last decrypted block B–m is padded with 16 – n zero octets at the right end to form the last block Bm as shown in Figure 98(a) for MIC calculation over the received frame as described in 8.3.1.5.

Each decrypted block is ordered for MIC calculation, and delivery to the MAC client if the MIC is valid, from its first octet on the left to its last octet on the right, as also illustrated in Figure 102.  Again the octet notations out0, …, out15 correspond to those used for AES output block formation specified in FIPS Pub 197.

Figure 102 — Decrypted Frame Payload format for encrypted frames

### Replay protection

A recipient shall treat a received frame with a valid MIC value, i.e., with the MIC field contained in the received frame equal to the MIC value calculated from the received frame, as a replay of a previously received frame if the Security Sequence Number field of the current frame has a value equal to or smaller than

  - GTK_SSN if the current frame is secured by a group temporal key (GTK) that has not been used for any previously received frames with a valid MIC field, where GTK_SSN is the value of the GTK SSN field contained in the Group Temporal Key (GTK) frame through which the node received this GTK from the sender of the current frame;

  - SSN if the current frame is secured by a PTK or GTK that has been used for one or more previously received frames with a valid MIC field, where SSN is the value of the Security Sequence Number field found in the last received frame containing a valid MIC and secured by this PTK or GTK.

The recipient shall discard all detected replayed frames.

IEEE publications are available from the Institute of Electrical and Electronics Engineers, 445 Hoes Lane, P.O. Box 1331, Piscataway, NJ 08855-1331, USA (http://standards.ieee.org).

FIPS publications and NIST special publications are available from the National Institute for Standards and Technology, 100 Bureau Drive, Stop 8900, Gaithersburg, MD 20899-8900 (http://www.nist.gov).

ISO/IEC publications are available from the ISO Central Secretariat, Case Postale 56, 1 rue de Varembé, CH-1211, Genève 20, Switzerland/Suisse. ISO/IEC publications are also available in the United States from the Sales Department, American National Standards Institute, 11 West 42nd Street, 13th Floor, New York, NY 10036, USA.

IETF RFCs are available from the Internet Engineering Task Force at http:///www.ietf.org.