IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Draft Text for Narrowband Physical Layer |
| Date Submitted | 18th of March 2010 |
| Source | Marco Hernandez, Kiran Bynam, Igor Dotlic, Huan-Bang Li, Ruyji Kohno, Mikyoung Oh, Tetsushi Ikegami, Haruka Suzuki , Ouvry Laurent, Jean Schwoerer, Seung Hoon Park, HYongsoo Lee, John Farserotu, John Gerris, Jerome Rousselot, Dries Neirynck, Kathleen Philips, Guido Dolmans, Olivier Rousseaux, Noh-Gyoung Kang and Chihong Cho. |
| Re: |  |
| Abstract | The present document describes the draft text of the UWB PHY for 802.15.6 body area networks. |
| Purpose | Normative text for the UWB PHY of 802.15.6 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Table of Contents

1	Acronyms and Abbreviations	4

2	General Description	5

2.1	Modes of operation	5

2.1.1	High QoS mode	5

2.2	Rules for use of modes and options	6

3	Definitions of FFDs (coordinators) and RFDs (devices)	6

3.1	Priority of resources	6

4	UWB Frame Format	7

4.1	UWB symbol structure	7

5	UWB Modulations	8

5.1	Non-coherent Modulation	8

5.1.1	Symbol mapper at half rate	8

5.1.2	Interleaver	10

5.1.3	Pulse shaping	10

5.1.4	Detection	10

5.2	Differentially coherent modulation	11

5.2.1	Interleaver	11

5.2.2	DBPSK/DQPSK	11

5.2.3	Pulse shaping	12

5.2.4	Optional differential PSK modulation with spreading	13

6	Operating Frequency Bands	13

7	Transmission Bandwidth	13

8	Data Rates	13

9	PSDU timing parameters	13

9.1	PRF parameter	14

9.2	Pulse waveform position parameter	14

9.3	Hop parameter	14

9.4	Pulse waveform duration parameter	14

9.5	Symbol duration parameter	14

9.6	Modulation mode 2 parameter	14

9.7	Symbol rate parameter	15

9.8	FEC rate parameter	15

9.9	Bit rate parameter	15

9.10	Modulation mode 1 parameter	15

9.11	Symbol rate parameter	15

9.12	FEC rate parameter	15

9.13	Bit rate parameter	15

9.14	Number of pulses parameter	15

9.15	Peak PRF parameter	15

10	SHR Preamble	15

11	Start frame delimiter	16

12	FEC	16

13	Pulse Shape Waveform	16

13.1	Criteria for pulse shape waveforms	16

13.2	Short pulse waveforms	16

13.3	Chirp pulse waveform	17

13.4	Chaotic pulse waveform	17

14	Hybrid Type II ARQ Mechanism	18

14.1	Error detection codes	20

14.2	Invertible codes 	20

15	FM-UWB PHY	20

15.1	System characteristics	20

15.2	Transmitter	21

15.3	Receiver architecture	23

16	Normative References	24

Appendix A	25

Soft detection	25

# Acronyms and Abbreviations

FM-UWB	Frequency Modulation Ultra-Wide Band

IR-UWB	Impulse Radio Ultra-Wide Band

# General Description

The wideband (UWB) PHY specification is designed to provide robust performance for BAN. Indeed, UWB transceivers allow low implementation complexity (critical for low power consumption). Moreover, the signal power levels are in the order of those used in the MICS band, i.e., safety power levels for the human body, besides of low interference to other devices.

The use of this spectrum combined with novel low complexity transceiver architectures, allow the implementation of low cost and low power devices for BAN. Moreover, such devices provide robust operation in multipath fading and interference.

The band of operation is 7.25 GHz to 8.5 GHz, which is available globally without regulatory restrictions like DAA systems. However, the use of one channel in the low band of UWB (channel 3 IEEE 802.15.4a band plan) and one channel in the high band of UWB are considered mandatory.

## Modes of operation

In order to ensure interoperability, a mandatory mode is required. Therefore, a compliant UWB PHY shall support the following:

- One mandatory modulation (see 5.1).

- One mandatory center frequency in the low band and high band of UWB (see 6).

- One mandatory bandwidth (see 7).

- One mandatory data rate (see 8).

- One mandatory preamble (see 10).

- One mandatory FEC (see ).

- One mandatory HARQ in the high QoS mode (see 14).

### High QoS mode

## Rules for use of modes and options

The UWB specification allows operation in the UWB band. Such UWB band is divided in sub-bands or channels (see ). The implementer is free to select any sub-band for implementation.   However, a channel in the low band of UWB and a channel in the high band of UWB are mandatory and the rest are optional.

There are two types of pulse shape waveforms supported. Namely, a concatenation or burst of short pulses (each short pulse of duration 2 nsec) and long pulse shape waveforms are supported. There is not mandatory pulse shape. However, implementers can choose a pulse shape from a pool of pulse shapes (see 13). Notice that two bits in the PHY header (PHR) indicate the type of pulse shape employed by the transmitter.

On the other hand, all beacon preambles are transmitted at the mandatory data rate during association. For synchronization, the preamble consists of repetitions of Kasami sequences of length 63. Such preamble can be used for non-coherent and differentially coherent detectors. Therefore, a preamble sequence is mandatory.

A combination of on-off signaling with 64-ary waveform coding and differentially encoded binary/quaternary phase shift keying modulation schemes are used to support non-coherent and differently coherent transceivers.  Different data rates are obtained by changing pulse waveform duration and modulation scheme (while maintaining the same data rate).  On-off signaling (non-coherent modulation) is mandatory and DBPSK/DQPSK (differentially coherent modulation) is optional for the default mode. In the high QoS mode, DBPSK/DQPSK (differentially coherent modulation) is mandatory and on-off signaling (non-coherent modulation) is optional.

FEC is mandatory at the transmitter, but optional during association (transmission of beacons). Also,   FEC is optional to decode at the receiver as systematic encoding is employed. That is, a receiver simply would ignore parity bits.

# Definitions of FFDs (coordinators) and RFDs (devices)

Full function devices (FFDs) shall have an IR-UWB radio or IR-UWB plus FM-UWB radios.

Reduced function devices (RFDs) shall have an IR-UWB radio or FM-UWB radio.

Only FFDs shall send beacon frame and coordinate beacon-enabled networks.

Only FFDs shall form non-beacon enabled networks.

## Priority of resources

Priority of resources is bundle around medical and non-medical applications.

Medical applications are given higher priority than non-medical applications in the form of two parameters:

1) If bandwidth of BAN is scarce, medical applications shall give higher priority than non-medical applications.

2) If medical receiver suffers degradation on sensitivity above 1 dB, the coordinator shall enforce non-medical devices to reduce EIRP at transmitters.

# UWB Frame Format

The UWB frame format or physical layer protocol data unit (PPDU) is formed by appending the preamble synchronization header (SHR), physical layer header (PHR) and physical layer service data unit (PSDU), respectively.

Figure 1 – UWB PPDU structure.

The preamble SHR is formed of repetitions of Kasami sequences for timing synchronization. This is divided into two parts. The first part is intended for timing synchronization while the second part is the start frame delimiter (SFD) for frame synchronization.

The PHR contains information about the data rate of the PSDU, duration of the preamble SHR and length of the payload. The PHR is protected with parity check bits appended at the end against channel errors.

The modulation during SHR, PHR and PSDU is on-off signaling for the non-coherent PHY and DBPSK for the differentially coherent PHY.

## UWB symbol structure

The UWB symbol structure supports on-off signaling and DBPSK/DQPSK. Each symbol timeTsym, consists of an integer number of basis waveform positions,Nw, each of duration Tw. The symbol duration is divided into two intervals of duration Tsym2 in order to enable on-off signaling.  The duty cycle factor during transmission is given by the ratio when a pulse waveform is on (Tw) over the symbol time (when a pulse waveform is on and off during a symbol time), i.e. η=TwTsym. The additional Nw−1 or (Nw2)−1 waveform positions are used for time hopping in order to support multi-BANs for coexistence.

Figure 2 – UWB symbol structure.

# UWB Modulations

The modulations are categorized into two modes of operation. Namely, on-off signaling is employed for the non-coherent PHY (mode 1) and DBPSK/DQPSK are employed for the differentially coherent PHY (mode 2).

## Non-coherent Modulation

The non-coherent PHY uses on-off signaling (on-off modulation in combination with 64-ary waveform coding). Such modulation strategy assigns binformation bits from an alphabet of size M=2b with a coded-pulse sequence of length 2b (half rate) from a code set alphabet of the same size. This represents the concept of the proposal Group PPM (GPPM).

The non-coherent PHY flow follows the schematic diagram of figure 3.

Figure 3 – Non-coherent PHY schematic diagram.

### Symbol mapper at half rate

A data symbol contains 6 bits (b0,b1,...,b5). The transmitting symbol is given by the mapping  (b0,b1,...,b5)→(d0,d1,...,d11) as indicated in Table 5.1. PSDU bits (after FEC) and PHR shall be encoded using such symbol mapper.

Table 5.1 – Symbol mapper for 64-ary waveform-coding.

| Data symbol decimal | Data symbol binary (b0,b1,...,b5) | Codeword (d0,d1,...,d11) | Data symbol decimal | Data symbol binary (b0,b1,...,b5) | Codeword (d0,d1,...,d11) |
| --- | --- | --- | --- | --- | --- |
| 0 | 000000 | 000000111111 | 32 | 100000 | 100110001011 |
| 1 | 000001 | 000011011011 | 33 | 100001 | 100101011010 |
| 2 | 000010 | 000011101101 | 34 | 100010 | 100100101110 |
| 3 | 000011 | 000111000111 | 35 | 100011 | 100011101010 |
| 4 | 000100 | 001001010111 | 36 | 100100 | 100001001111 |
| 5 | 000101 | 011001110100 | 37 | 100101 | 100001111100 |
| 6 | 000110 | 000111111000 | 38 | 100110 | 100100010111 |
| 7 | 000111 | 000011110110 | 39 | 100111 | 100010011101 |
| 8 | 001000 | 000110011110 | 40 | 101000 | 011011000101 |
| 9 | 001001 | 000110110011 | 41 | 101001 | 011011101000 |
| 10 | 001010 | 001100011011 | 42 | 101010 | 011101000110 |
| 11 | 001011 | 101010010011 | 43 | 101011 | 011101011000 |
| 12 | 001100 | 011001001011 | 44 | 101100 | 100001110011 |
| 13 | 001101 | 000101101011 | 45 | 101101 | 011110110000 |
| 14 | 001110 | 110100011001 | 46 | 101110 | 011101100001 |
| 15 | 001111 | 000101011101 | 47 | 101111 | 011110000011 |
| 16 | 010000 | 010001011110 | 48 | 110000 | 010001100111 |
| 17 | 010001 | 010101101100 | 49 | 110001 | 110010100110 |
| 18 | 010010 | 001111010001 | 50 | 110010 | 010010101011 |
| 19 | 010011 | 001111100100 | 51 | 110011 | 010010010111 |
| 20 | 010100 | 001011011100 | 52 | 110100 | 010100001111 |
| 21 | 010101 | 001111001010 | 53 | 110101 | 010100110101 |
| 22 | 010110 | 001110101001 | 54 | 110110 | 010010111100 |
| 23 | 010111 | 001011100011 | 55 | 110111 | 010011110001 |
| 24 | 011000 | 011110001100 | 56 | 111000 | 011000011101 |
| 25 | 011001 | 001010001111 | 57 | 111001 | 011000110011 |
| 26 | 011010 | 001001111001 | 58 | 111010 | 010111100010 |
| 27 | 011011 | 001001101110 | 59 | 111011 | 011011010010 |
| 28 | 011100 | 001010111010 | 60 | 111100 | 010100111010 |
| 29 | 011101 | 001100110110 | 61 | 111101 | 010101010011 |
| 30 | 011110 | 001010110101 | 62 | 111110 | 010111010100 |
| 31 | 011111 | 101100100101 | 63 | 111111 | 010111001001 |

### Interleaver

### Pulse shaping

After symbol mapper, interleaving and SHR insertion, the pulse shaping shall place a pulse waveform when an input bit is 1, according to the UWB symbol structure and absence of a pulse waveform when an input bits is 0. Thus, the transmitting signal is given by

x(t)=mn=011dnmw(t−nTsym2−6mTsym−hmk)

Where m is the m-th transmitting symbol, Tsymis the symbol time, hmkis a time hopping sequence for the k-th BAN and w(t)is the pulse waveform given by

w(t)=i=1Ncpbp(t−(i−1)Tp)

The pulse waveform has durationTw=NcpbTp, where Ncpb=1in case of a long pulse shape waveform and Tpis the pulse shapep(t) duration. When Ncpb>1a burst of short pulses (typically 2 nsec each short pulse) is transmitted.

### Detection

On-ff signaling does not require defining a threshold for detection as in on-off demodulation. The implementer shall employ soft-detection. Soft detection can be either the correlation of the received signal with every sequence in the codebook at time lag zero or minimum Euclidian distance detection (see Appendix A).

## Differentially coherent modulation

The differentially coherent PHY uses differentially encoded BPSK/QPSK. The differentially coherent PHY flow follows the schematic diagram of figure 4.

Figure 4 – Differentially coherent PHY schematic diagram.

### Interleaver

### DBPSK/DQPSK

The bits of SHR, PHR (after encoding) and PSDU (after encoding and interleaving) shall be differentially encoded such that the PSK transmitting symbols are given by

cm=cm−1exp(jϕm)

where cmrepresents the m-th differentially encoded BPSK or QPSK symbol,m=0,1,...,N−1 and c−1=1. Such symbol carries either one bit of information (differentially encoded BPSK) or two bits of information (differentially encoded QPSK). The mapping between information bits onto ϕm is given in tables 5.2 and 5.3

Table 5.2 - Mapping of information bits onto ϕmfor DBPSK.

| bm | ϕm |
| --- | --- |
| 0 | 0 |
| 1 | π |

Table 5.3 – Mapping of information bits onto ϕmfor DQPSK.

| am | bm | ϕm |
| --- | --- | --- |
| 1 | 1 | π |
| 0 | 1 | π2 |
| 0 | 0 | 0 |
| 1 | 0 | −π2 |

### Pulse shaping

After the generation of DBPSK/DQPSK symbols, the pulse shaping shall place a pulse waveform according to the UWB symbol structure. Thus, the transmitting signal is given by

xt=m=0N−1cmw(t−mTsym−hmk)

wherem is the m-th transmitting symbol, Tsymis the symbol time, hmkis a time hopping sequence for the k-th BAN and w(t)is the pulse waveform given by

w(t)=i=1Ncpbp(t−(i−1)Tp)

The pulse waveform has durationTw=NcpbTp, where Ncpb=1in case of a long pulse shape waveform and Tpis the pulse shapep(t) duration. When Ncpb>1a burst of short pulses (typically 2 nsec each short pulse) is transmitted.

### Optional differential PSK modulation with spreading

In order to enhance interference rejection, two optional data rates use differentially encoded BPSK/QPSK with spreading.  The differentially encoded symbols are given by

e(k,l)=b(l)I(k)conj(e(k−1,l))

Where kis the symbol number, lis the number of chip in a symbol, e(k,l) is information phase of the l-th chip in the k-th symbol, b(l)is a Barker sequence of length 7.

# Operating Frequency Bands

The low band of UWB is divided into sub-bands according to the IEEE 802.15.4a frequency band plan [1]. Channel 3 of such band plan is mandatory.  The high of UWB (7.25 GHz to 8.5 GHz) is divided into sub-bands . Channel 1 of such band plan is mandatory.

# Transmission Bandwidth

The mandatory bandwidth satisfies the IEEE 802.15.4a UWB spectral mask of 650 MHz at -10 dB [1].

# Data Rates

Data rates range from 0.5 Mbps up to 10 Mbps. The mandatory data rate is 0.5 Mbps.

# PSDU timing parameters

The PSDU timing and data rate parameters are given in table 9.1. Notice that in case of long pulse shape waveform option, the pulse repetition frequency (number of pulses transmitted in one second) or PRF coincides with the mean PRF (number of pulses transmitted in a symbol period divided by the symbol period) because a single pulse is transmitted per symbol duration.  In case of short pulse shape option, the peak PRF is defined as the maximum rate at which a transmitter emits pulses.  This definition is not applicable for the long pulse shape option. The description of the different parameters in table 9.1 is given as follows:

Table 9.1 – Timing Parameters.

## PRF parameter

In case of long pulse shape option, PRF indicates the number of pulses transmitted in one second. It is equal to the mean PRF (number of pulses transmitted in a symbol period divided by the symbol period) as one pulse is transmitted per symbol duration.

## Pulse waveform position parameter

It indicates the number of possible pulse waveform positions within a symbol time:Nw=TsymTw.

## Hop parameter

It gives the number of pulse waveform positions that can contain an active waveform for time hopping (Nhop).

## Pulse waveform duration parameter

It is the pulse waveform duration and computed asTw=NcpbTp. In case of the long pulse shape optionNcpbis 1. In case of short pulse option Ncpb is larger than 1. Notice Tpis the pulse shape duration.

## Symbol duration parameter

It is the PSDU symbol duration (Tsym).

## Modulation mode 2 parameter

It indicates either differentially encoded BPSK or differentially encoded QPSK.

## Symbol rate parameter

It is the number of information symbols transmitted per second and computed asRs=1Tsym.

## FEC rate parameter

It is the channel coding rate, i.e., number of information bits divided by the number of coded bits.

## Bit rate parameter

It gives the bit rate on the air for the modulation mode 2 and computed as Rb=Rs⋅FEC rate in case of DBSPK modulation and Rb=2⋅Rs⋅FEC rate  in case of DQPSK.

## Modulation mode 1 parameter

It indicates on-off signaling (on-off modulation in combination with 64-ary waveform coding).

## Symbol rate parameter

It is the number of information symbols transmitted per second and computed asRs=1Tsym.

## FEC rate parameter

It is the channel coding rate, i.e., number of information bits divided by the number of coded bits.

## Bit rate parameter

It gives the bit rate on the air for the modulation mode 1 and computed asRb=Rs⋅FEC rate.

## Number of pulses parameter

It indicates the number of pulses shapes. In case of the long pulse shape optionNcpbis 1. In case of short pulse option Ncpb is larger than 1. Notice Tpis 2 nsec in case of short pulse shape option.

## Peak PRF parameter

It is defined as the maximum rate at which a transmitter emits short pulse waveforms. This definition is applicable for the short pulse shape option only and it is not applicable for the long pulse shape option.

# SHR Preamble

In order to perform coarse synchronization, data aided in the form of a preamble is employed. Such preamble is detectable for non-coherent and differential coherent receivers. Kasami sequences of length 63 shall be used as preamble for both the non-coherent and differentially coherent PHYs. The SHR preamble shall be modulated with on-off signaling for the non-coherent PHY and DBPSK modulated for the differentially coherent PHY.

There are eight Kasami sequences defined in table 10.1.  Those can be used according to the band plan and the co-existence scheme used. Notice eight logical  channels can be used per channel.

The preamble set  shall be divided into two pools. Each pool has a set of 4 preambles. The first pool shall be used for odd number of channels . The second pool shall be used for even number of channels . The coordinator might scan all the logical channels and use the preamble with minimum received power level.

Coexistence of BANs and the interference mitigation can be achieved by virtue of the inherent duty cycling in the modulation and the preamble sequences with good correlation properties.

Table 10.1 – Unipolar Kasami sequences.

# Start frame delimiter

After the preamble, a start frame delimiter (SFD) is transmitted for frame synchronization. The SFD is balance, i.e., when its correlation window is running through the preamble, the output is zero.  Thus, the transition of correlation from preamble to SFD does not degrade the detection of SFD.

# FEC

# Pulse Shape Waveform

There is not mandatory pulse shape. However, the pulse waveform duration and PRF are mandatory according to the timing parameters of Table 9.1 in case of long pulse shape option. In case of short pulse shape option, the pulse waveform duration and peak PRF are mandatory according to the timing parameters of Table 9.1. Notice pulse shape waveforms have to fulfill the spectral mask of IEEE 802.15.4a [1] and regulatory spectral mask where applicable.

## Criteria for pulse shape waveforms

## Short pulse waveforms

Conventional short pulses of 2 nsec defined by IEEE 802.15.4a for UWB transmission are supported [1].

## Chirp pulse waveform

A chirp pulse waveform is supported and given by

p(t)=cos2πf0t+kc2t2=Reexpj2πf0t+kc2t2       −tch2≤t≤tch2

where f0  is the central frequency, Tchis the chirp duration, kis the chirp slope defined as kc=ΔfTchand Δfis the chirp frequency sweep. Figure 5 shows the spectral characteristics of chirp pulses of duration 60 and 100 nsec and frequencies sweep of 550 and 580 MHz compared with the IEEE802.15.4a spectral mask.

Figure 5 – Spectral characteristics of chirp pulses of duration 60 and 100 nsec compared with the IEEE802.15.4a spectral mask.

## Chaotic pulse waveform

Chaotic pulses are near constant envelope signals that are produced by the concatenation of four different triangular or sawtooth waveforms. The sum of these triangular or sawtooth waveforms feds a voltage controlled oscillator (VCO). This is similar to frequency modulating a carrier signal with the concatenation of the triangular signals. The center frequency of the oscillator is directly proportional to the DC offset of the VCO.

Figure 6 - Chaos waveform generator using triangular waveform

The duration of the triangular pulses and the peak to peak amplitudes are chosen to achieve the 500 MHz bandwidth. The period of the four triangular pulses are chosen as 3, 19, 53 and 59 nsec respectively. Amplitudes are chosen in the ratio of 0.5, 0.2, 0.8, 1 respectively. This summation results in a spectrum shape with 10 dB bandwidth of 500 MHz. The spectrum of the chaotic pulse waveform is shown in figure 7.

Figure 7 – Power spectral density of chaotic pulses with 10 dB bandwidth of 500 MHz.

# Hybrid Type II ARQ Mechanism

This configuration is based on invertible codes. A code is said invertible if acknowledge of only parity bits, the information bits can be recovered by an inversion process of such parity bits. Packets detected in error by a cyclic redundancy code (CRC) are not discarded, but rather stored for posterior use. This HARQ scheme requires storing a packet at the transmitter and receiver. Codewords are form by information bits and parity bits of the same length. Packets are created by information bits only or parity bits only. The mechanism of retransmission is alternate repetitions of packets with the information bits or packets with parity bits at the time. The retransmissions continue until the information bits are finally recovered either by inversion or decoding process or a maximum number of retransmission is reached.

Figure 8 shows the flow diagram of HARQ Type II with invertible codes. As HARQ requires the interaction of PHY and MAC, the part of the MAC mechanism for HARQ is described in Section x.x of the MAC document P802.15-09-0196-02-0006. The part of the PHY mechanism is through FEC and CRC coding, decoding and FEC inversion.

Figure 8 – Flow diagram of HARQ Type II with invertible codes.

## Error detection codes

The cyclic redundancy check code is defined in the MAC document P802.15-09-0196-02-0006. In figure 8, the CRC encoding for parity bits is represented asC0(⋅). The CRC decoding is given byC0−1(⋅).

## Invertible codes

In figure 8, the FEC encoding for parity bits is represented asC1(⋅). The FEC decoding is given byC1−1(⋅) and FEC inversion of parity bits to retrieve information bits D is represented asPD−1.

# FM-UWB PHY

FM-UWB is an optional PHY targeting low data rate medical BAN.  FM-UWB exploits high modulation index analog FM to obtain an ultra-wide signal. Frequency modulation has the unique property that the RF bandwidth BRF is not only related to the bandwidth fm of the modulating signal, but also to the modulation index  that can be chosen freely. This yields either a bandwidth efficient narrow-band FM signal ( < 1) or a (ultra) wideband signal ( >> 1) that can occupy any required bandwidth.

FM-UWB constitutes an analog implementation of a spread-spectrum system. This constant-envelope approach, where peak power equals average power, yields a flat spectrum with steep spectral roll-off. Instantaneous de-spreading in the receiver makes that the FM-UWB radio behaves like a narrowband FSK radio from a synchronization and detection point-of-view. FM-UWB technology combines low complexity with robustness against interference and multipath. The following sub-sections present the FM-UWB system and its properties.

## System characteristics

FM-UWB radios shall satisfy the system characteristics defined in table 15.1. Operation shall be in the 7.25 – 8.5 GHz band. Other UWB bands are appropriate for supporting international operation and mobility and in accordance with the relevant regulatory domains.

Table 15.1 - FM-UWB system characteristics.

| Parameter | Value |
| --- | --- |
| RF center frequency | 6.4 – 8.7 GHz |
| RF bandwidth | 500 MHz |
| RF output power | -15 dBm |
| Subcarrier frequency | 1.0, 1.25, 1.50, 1.75 MHz |
| Subcarrier modulation | FSK,  = 1 |
| Raw bit rates | 31.25, 62.5, 125 and 250 kbps |
| Receiver sensitivity | <-85 dBm1 |
| TX, RX switching time | < 100 s |
| RX synchronization time | < 500 s |

- At BER ≤ 10-6

## Transmitter

Figure 12.2 provides a block diagram of the FM-UWB transmitter. The low modulation index digital Frequency Shift Key (FSK) is followed by high modulation index analog FM, creating a constant-envelope UWB signal [GER1]. The transmitter shall consist of a 1-2 MHz subcarrier oscillator generating a triangular signal that is FSK modulated by the transmit data. This subcarrier signal m(t) modulates the RF VCO, yielding a constant-envelope UWB signal with a flat power spectral density and steep spectral roll-off.

Figure 8 - FM-UWB transmitter block diagram.

The FM-UWB signal V(t) with amplitude A and carrier frequency fc(ωc=2πfc)is obtained by modulating the RF oscillator with the subcarrier m(t) of frequency fm(ωm=2πfm) such that

m(t)=VmM(ωmt)

Where Mrepresents the triangular waveform of the subcarrier signal. An RF oscillator sensitivity of K0 [rad/V-s] yields a deviation Δω=2πΔfequal to Δω=KOVm,resulting in a FM signal V(t) that can be expressed mathematically as

Vt=Asinωct+ϕt

=Asinωct+KO−∞tmτdτ

=Asinωct+Δω−∞tMτdτ

Figure 9 illustrates the power spectral density (PSD) of an ideal FM-UWB signal. The signal shall be compliant with the relevant regulatory limits. The signal power is –15 dBm (112 mVpp in a 50  load). The subcarrier frequency is 1 MHz and the deviation Δfis 250 MHz, yielding a modulation index  of 250. The –10 dB bandwidth is 500 MHz.

The subcarrier signal m(t)shall modulate the RF oscillator that implements FM with high modulation index and yields a constant-envelope UWB signal with a flat PSD and steep spectral roll-off at its output.

The PSD of a wideband FM signal is determined by and has the shape of the probability density function (PDF) of the modulating signalm(t). The use of a triangular subcarrier waveform, which is characterized by a uniform PDF, results in a flat RF spectrum. From an implementation point of view the triangular waveform is straightforward to generate using either analog or digital circuit techniques.

Figure 9 - Frequency spectrum of FM-UWB signal at 7.5 GHz center frequency.

Transmitter RF center frequencies with 576 MHz channel spacing

Transmitter RF center frequencies with 512 MHz channel spacing (MBOA compatible)

## Receiver architecture

The receiver shall demodulate the FM-UWB signal without frequency translation (i.e., no mixing). Figure 10 shows the receiver block diagram.  The low complexity receiver comprises a LNA, a wideband FM demodulator, and low-frequency subcarrier filtering, amplification and demodulation circuitry.

Figure 10 - FM-UWB receiver block diagram.

The absence of carrier synchronization allows for fast synchronization. Due to the instantaneous de-spreading in the wideband FM demodulator, the system behaves like a narrowband FSK system where synchronization is limited by the bit synchronization time.

# Normative References

[1]	IEEE Std 802.15.4a™ 2007.

Appendix A

Soft detection

Assuming symbol and frame synchronization, let Z0,...,Z2b−1 be the quantized values of E0,...,E2b−1after analog to digital conversion (ADC), where En represents sufficient statistics after the RF front-end. Let d'm,n be defined as d'm,n=2dnm−1 such thatd'm,n∈±1.

Decision rule 1:  Correlation of the received signal with every sequence at time lag zero

d=argmaxmZm

where Zm=d'm,0Z0+,...,+d'm,2b−1Z2b−1.