IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Draft Text for Wideband Physical Layer |
| Date Submitted | 20th of May 2010 |
| Source | Marco Hernandez, Kiran Bynam, Igor Dotlic, Huan-Bang Li, Ruyji Kohno, Mikyoung Oh, Tetsushi Ikegami, Haruka Suzuki , Ouvry Laurent, Jean Schwoerer, Seung Hoon Park, June Chul Roh, HYongsoo Lee, John Farserotu, John Gerris, Jerome Rousselot, Dries Neirynck, Kathleen Philips, Guido Dolmans, Olivier Rousseaux, Noh-Gyoung Kang and Chihong Cho. |
| Re: |  |
| Abstract | The present document describes the draft text of the UWB PHY for 802.15.6 body area networks. |
| Purpose | Normative text for the UWB PHY of 802.15.6 |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Table of Contents

1	Acronyms and Abbreviations	5

2	General Description	7

2.1	Modes of operation	7

2.1.1	High QoS mode	7

2.2	Rules for use of modes and options	8

3	Definitions of FFDs (coordinators) and RFDs (devices)	8

3.1	Priority of resources	8

4	UWB Frame Format	9

5	PSDU construction	9

5.1	MPDU	10

5.2	Scrambler	10

5.3	Systematic BCH(63,51) encoder	11

5.4	Pad bits	11

5.5	Bit interleaving	12

6	PHR construction	12

6.1	PHR information	13

6.1.1	Data rate	13

6.1.2	Frame length	13

6.1.3	HARQ enable	13

6.1.4	Scrambler word (sync-word)	13

6.2	PHR protection	13

6.2.1	CRC-4-ITU	13

6.2.2	Shortened BCH(31,19) encoder	13

7	SHR Preamble	14

7.1	Start frame delimiter	15

8	UWB symbol structure	15

9	Data Rates	16

10	PSDU timing parameters	16

10.1	PRF parameter	17

10.2	Pulse waveform position parameter	17

10.3	Hop parameter	17

10.4	Pulse waveform duration parameter	17

10.5	Spreading parameter (Table 5)	17

10.6	Symbol duration parameter	17

10.7	Modulation mode 2 parameter (Table 5)	17

10.8	Symbol rate parameter	17

10.9	FEC rate parameter	17

10.10	Bit rate parameter	17

10.11	Number of pulses parameter	18

10.12	Nguard parameter	18

10.13	Peak PRF parameter	18

11	UWB Modulations	18

11.1	Scrambling for a burst of pulse shapes	18

11.1.1	Optional dynamic scrambling	19

11.2	Time hopping sequence	20

11.3	Non-coherent Modulation	21

11.3.1	Symbol mapper at half rate	21

11.3.2	Pulse shaping	23

11.3.3	Detection	23

11.4	Differentially coherent modulation	24

11.4.1	DBPSK/DQPSK	24

11.4.2	Pulse shaping	25

11.4.3	Differentially encoded PSK modulation with spreading	25

12	Operating frequency bands	26

13	Clock accuracy and central frequency alignment	26

14	Transmit spectrum mask	26

15	Pulse Shape Waveform	27

15.1	Short pulse waveforms	27

15.2	Chirp pulse waveform	28

15.3	Chaotic pulse waveform	31

16	Hybrid Type II ARQ Mechanism	32

16.1	Error detection codes	32

16.2	Invertible codes	32

16.3	PHR iteration	32

17	FM-UWB PHY	34

17.1	Data rates	34

17.2	System characteristics	34

17.3	Preamble	34

17.4	Transmitter	35

17.5	Receiver architecture	37

18	Clear Channel Assessment	38

19	Normative References	38

Appendix A	39

Soft detection	39

Appendix B	40

Shortened BCH(31,19)	40

# Acronyms and Abbreviations

BAN	Body Area Network.

BCH	Bose, Ray-Chaudhuri, Hocquenghem Code

FM-UWB	Frequency Modulation Ultra-Wide Band.

IR-UWB	Impulse Radio Ultra-Wide Band.

PLCP	Physical Layer Convergence Procedure.

DAA	Detect And Avoid.

PPDU	PHY Protocol Data Unit.

SHR	Synchronization Header

PHR	PHY Header

PSDU	PHY Service Data Unit.

CCA	Clear Channel Assessment

FFD	Full Function Device

RFD	Reduced Function Device

EIRP	Equivalent Isotropically Radiated Power

SAP	Service Access Point

HARQ	Hybrid Automatic Repeat Request

MPDU	MAC Protocol Data Unit

LFSR	Linear Feedback Shift Register

LSB	Least Significant Bit

MSB	Most Significant Bit

MICS	Medical Implant Communication Service

VCO	Voltage Control Oscillator

# General Description

The ultra wideband (UWB) PHY specification is designed to provide robust performance for BANs. UWB transceivers allow low implementation complexity (critical for low power consumption). Moreover, the signal power levels are in the order of those used in the MICS band, for example, safety power levels for the human body and low interference to other devices.

The UWB PHY provides three levels of functionality:

- UWB PHY gives a frame exchange between the MAC and PHY under the control of the physical layer convergence procedure (PLCP) sub-layer. Such PLCP constructs the PHY layer protocol data unit (PPDU) by concatenating the preamble synchronization header (SHR), physical layer header (PHR) and physical layer service data unit (PSDU), respectively.

- The PLCP sub-layer converts the bits of information of PPDU into RF signals for the wireless media.

- UWB PHY provides clear channel assessment (CCA) indication to the MAC in order to verify activity on the wireless media.

## Modes of operation

In order to ensure interoperability, a mandatory mode is required. Therefore, a compliant UWB PHY shall support the following:

- One mandatory FEC (see 5.3).

- One mandatory preamble (see 7).

- One mandatory data rate (see 9).

- One mandatory modulation (see 11.3).

- One mandatory center frequency in the low band and high band of UWB (see 12.1).

- One mandatory bandwidth (see 14).

- One mandatory HARQ in the high QoS mode (see 16).

### High QoS mode

Applications with high QoS mode shall employ the differentially coherent PHY and HARQ.

## Rules for use of modes and options

The UWB band is divided into two band groups: low band and high band (see 12). A compliant UWB device shall transmit in at least one of the specified band groups. The low band and high band are allocated into operating frequency channels as shown in table 10. The implementer is free to select any frequency channel for implementation. However, a UWB device that implements the low band shall support channel 3. The rest low-band channels are optional. A UWB device that implements the high band shall support channel 7. The rest of high-band channels are optional.

There are two types of pulse shape waveforms supported:

- A concatenation or burst of short pulses.

- Long pulse waveforms.

There is not a mandatory pulse shape. However, implementers can choose a pulse shape from a pool of pulse shape waveforms (see 15).

On the other hand, all beacon preambles are transmitted at the mandatory data rate during association. For synchronization, the preamble consists of repetitions of Kasami sequences of length 63. Such preamble can be used for non-coherent and differentially coherent detectors.

A combination of on-off signaling with 64-ary waveform coding and differentially encoded binary/quaternary phase shift keying modulation schemes are used to support non-coherent and differently coherent transceivers.  Different data rates are obtained by changing pulse waveform duration and modulation scheme (while maintaining the same data rate).  On-off signaling (non-coherent modulation) is mandatory and DBPSK/DQPSK (differentially coherent modulation) is optional for the default mode. In the high QoS mode, DBPSK/DQPSK (differentially coherent modulation) is mandatory and on-off signaling (non-coherent modulation) is optional.

FEC is mandatory for the transmission of PHR and PSDU, but optional during association (transmission of beacons). Moreover, FEC decoding is optional at the receiver for the PSDU, as systematic encoding is employed. That is, a receiver simply would ignore parity bits.

# Definitions of FFDs (coordinators) and RFDs (devices)

Full function devices (FFDs) shall have an IR-UWB radio or IR-UWB plus FM-UWB radios.

Reduced function devices (RFDs) shall have an IR-UWB radio or FM-UWB radio.

Only FFDs shall send beacon frame and coordinate beacon-enabled networks.

Only FFDs shall form non-beacon enabled networks.

## Priority of resources

Priority of resources is given around medical and non-medical applications.

Medical applications are given higher priority than non-medical applications in the form of two parameters:

1) If bandwidth of BAN is scarce, medical applications shall give higher priority than non-medical applications.

2) If medical receiver suffers degradation on sensitivity above 1 dB, the coordinator shall enforce non-medical devices to reduce EIRP at transmitters.

# UWB Frame Format

The UWB frame format or physical layer protocol data unit (PPDU) is formed by concatenating the preamble synchronization header (SHR), physical layer header (PHR) and physical layer service data unit (PSDU), respectively as illustrated in Figure 1.

Figure 1 - UWB PPDU structure.

The PSDU contains the payload from the MAC via SAP, which is formatted for transmission (see 5).

The PHR contains information about the data rate of the PSDU, length of the payload, HARQ and scrambler seed. The PHR is protected with error detection parity bits CRC-4-ITU and parity bits of a FEC (shortened BCH code) appended at the end against channel errors (see 6.1 and 6.2).

The preamble SHR is formed of repetitions of Kasami sequences. The SHR is divided into two parts. The first part is intended for coarse timing synchronization while the second part is the start frame delimiter (SFD) for frame synchronization (see 7).

# PSDU construction

The information bits of the PSDU are formatted for transmission. The PSDU construction process is illustrated in Figure 2.

Figure 2 - PSDU construction.

## MPDU

The MAC provides to the PHY with the MAC protocol data unit (MPDU) that consists of MAC header (7 bytes), MAC payload (0-256 bytes) and CRC-16-ITU (2 bytes) parity bits as illustrated in Figure 3.

Figure 3 - MPDU structure.

## Scrambler

Data whitening is applied through scrambling in order to minimize the DC bias on data if long strings of 1s or 0s are contained in the MPDU. An additive or synchronous scrambler with polynomialG(x)=1+x2+x12+x13+x14shall be employed. Figure 4 shows a typical implementation of the side-stream scrambler. The output of the scrambler is generated as:

x[n]=x[n−2]⊕x[n−12]⊕x[n−13]⊕x[n−14]

where “” denotes modulo-2 addition. Table 1 defines the initialization vector (sync-word), xinit, for the additive scrambler as a function of the scrambler seed (SS) value. The sync-words are placed in the data stream through equal intervals, i.e., each frame.

Figure 4 — Block diagram of a side-stream scrambler

Table 1— Scrambler Seed Selection

| Scrambler Word (S) | Initialization Vector xinit = x[-1] x[-2] … x[-14] |
| --- | --- |
| 0 | 0 0 1 0 1 1 1 1 0 0 1 1 0 1 |
| 1 | 0 0 0 0 0 0 0 1 0 0 1 1 1 1 |

The MAC shall set the scrambler word to 0 when the PHY is initialized and the scrambler word shall be incremented, using a 1-bit rollover counter, for each frame sent by the PHY.

At the receiver, the additive de-scrambler shall be initialized with the same initialization vector, xinit, used by the transmitter. The initialization vector is determined from the scrambler seed value in the PHY header of the received frame.

## Systematic BCH(63,51) encoder

The generator polynomial for a systematic BCH (63, 51) code is given by:

g(x)=1+x3+x4+x5+x8+x10+x12.

The parity bits are determined by computing the remainder polynomial r(x):

r(x)=i=011rixi=x12m(x)modg(x),

where m(x) is the message polynomial:

m(x)=i=050mixi,

and ri,i=0,…,11 and mi,i=0,…,50 are elements of GF(2). The message polynomial m(x) is created as follows: m50 is the first bit of the message and m0 is the last bit of the message. The order of the parity bits is as follows: r11 is the first parity bit transmitted and r0 is the last parity bit transmitted.

Figure 5 - BCH (63,51) codeword.

## Pad bits

Pad bits sub-system shall append pad bits in order to ensure that its input bit stream aligns on a symbol boundary.  The number of pad bits is given by

Npad=log2(M)×NPSDU+NCW×(n−k)log2(M)−NPSDU+NCW×(n−k)

where Mis the cardinality of the constellation of a given modulation scheme,  NPSDUis the number of PSDU bits, NCW=NPSDUk is the number of BCH codewords and (n−k)is the number of BCH parity bits.

All appended pad bits shall be set to 0.  In the case of un-coded transmission, NCW is set to zero. Notice that the total number of bits to be transmitted on the air is given by

Ntotal=NPSDU+NCW×(n−k)+Npad

## Bit interleaving

In order to protect against channel fading (quasi-static or dynamic) bit interleaving is applied prior to the mapping of modulation symbols. A binary labeling function that maps blocks of bits to signal constellation symbols is already defined by the non-coherent modulation. For DQPSK modulation, it is Gray encoding. Thus, this is a form of bit-interleaved coded modulation.

A simple algebraic interleaver that can be generated on-the-fly is defined as

Π(n)=nbModNI

where NIis the interleaver’s length, Π(n)denotes the new position to which index n is interleaved, ModNIis modulo NIarithmetic. The seeding parameter bsatisfies 0≤b<NIand bis a relative prime to NIto ensure one-to-one mapping. The interleaver’s length shall be set to NI=64and seeding parameter shall be set tob=39.

Notice that in the last codeword, the interleaver is active only in the last remaining bitsNrem=rem(Ntotal,NI).

# PHR construction

The PHY header (PHR) contains information about the data rate, frame length, HARQ enable, and scrambler seed. The information in the PHR is used by the receiver in order to decode the PSDU. The PHR is protected by a combination of CRC-4-ITU error detection (4 parity bits) and shortened systematic BCH (31,19) FEC coding (12 parity bits). The concatenation of PHR, CRC-4-ITU parity bits and shortened BCH(31,19) parity bits forms the PHY header.

Figure 6 – PHR configuration.

Figure 7 – PHR construction.

## PHR information

The PHR information is formed of 15 bits as illustrated in Table 2.

Table 2 - PHR information structure.

| Bit 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R0 | R1 | R2 | L0 | L1 | L2 | L3 | L4 | L5 | L6 | L7 | r | H1 | H2 | S |
| Data rate |  |  | Frame length |  |  |  |  |  |  |  | a | HARQ |  | b |

- Reserved.

- Reserved.

- Scrambler word.

The description of the different fields of the PHR is as follows:

### Data rate

Indication of data rates (R0,R1,R2) for the non-coherent, differentially coherent and FM-UWB modulations are denoted  in tables 4, 5 and 12, respectively.

### Frame length

A variable frame length is indicated with eight bits (L0,...,L7), i.e., from 0 to 255 octets or bytes.

### HARQ enable

The 13th bit H0=1 indicates HARQ enabled (see 16.3 for H1).

### Scrambler word (sync-word)

The 15th bit indicates the sync-word for the bit scrambler as indicated in Table 1. This field shall be set up by the MAC.

## PHR protection

### CRC-4-ITU

The PLCP sub-layer shall append 4-bits from CRC-4 ITU error detection coding to the PHR information. The CRC-4-ITU shall be the 1’s complement of the remainder generated by the modulo-2 division of the PHR information by the polynomial: 1+x+x4.

### Shortened BCH(31,19) encoder

The PLCP sub-layer shall append 12 parity bits from a shortened BCH(31,19) encoding (obtained from the BCH(63,51) encoder) to the PHR information and CRC-4-ITU parity bits. Thus, the same BCH encoder and decoder are used for the PSDU and PHR (see Appendix B).

# SHR Preamble

In order to perform coarse synchronization, data aided in the form of a preamble is employed. Such preamble is detectable for non-coherent, differential coherent receivers. Kasami sequences of length 63 shall be used as preamble for both the non-coherent and differentially coherent PHYs.

There are eight Kasami sequences defined in table 3.  Those can be used according to the band plan and the co-existence scheme used. Notice eight logical  channels can be used per channel.

The preamble set  shall be divided into two pools. Each pool has a set of 4 preambles. The first pool shall be used for odd number of channels . The second pool shall be used for even number of channels . The coordinator might scan all the logical channels and use the preamble with minimum received power level.

Coexistence of BANs and the interference mitigation can be achieved by virtue of the inherent duty cycling in the modulation and the preamble sequences with good correlation properties.

Table 3 – Eight Kasami sequences of length 63.

| C1 | 1 1 1 1 1 1 0 1 0 1 0 1 1 0 0 1 1 0 1 1 1 0 1 1 0 1 0 0 1 0 0 1 1 1 0 0 0 1 0 1 1 1 1 0 0 1 0 1 0 0 0 1 1 0 0 0 0 1 0 0 0 0 0 |
| --- | --- |
| C2 | 0 0 0 1 1 0 0 0 1 0 0 1 0 0 1 0 0 0 1 0 1 1 0 0 0 1 1 0 0 1 1 1 1 0 0 1 1 0 0 1 0 1 0 1 1 1 0 0 0 1 1 0 1 0 1 0 1 0 1 0 0 1 0 |
| C3 | 1 0 0 0 1 1 1 1 1 0 1 1 1 1 0 0 0 1 1 1 0 0 0 0 1 1 0 1 1 1 1 0 1 1 1 0 1 0 1 1 1 0 1 1 1 0 0 1 1 0 1 0 0 0 0 1 0 0 1 1 0 0 1 |
| C4 | 0 1 0 0 0 1 0 0 0 0 1 0 1 0 1 1 0 1 0 1 1 1 1 0 1 0 0 0 0 0 1 0 0 1 0 1 0 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 0 1 0 0 1 1 1 1 1 0 0 |
| C5 | 1 0 1 0 0 0 0 1 1 1 1 0 0 0 0 0 1 1 0 0 1 0 0 1 1 0 1 0 1 1 0 0 0 0 0 0 1 1 1 0 0 1 1 1 0 0 1 0 0 0 1 1 0 1 1 0 0 0 0 1 1 1 0 |
| C6 | 1 1 0 1 0 0 1 1 0 0 0 0 0 1 0 1 0 0 0 0 0 0 1 0 0 0 1 1 1 0 1 1 0 0 1 0 0 0 0 0 0 0 1 0 1 1 1 0 1 0 0 0 1 1 1 1 0 1 1 0 1 1 1 |
| C7 | 0 1 1 0 1 0 1 0 0 1 1 1 0 1 1 1 1 1 1 0 0 1 1 1 1 1 1 1 0 0 0 0 1 0 1 1 0 1 1 1 0 0 0 0 0 0 0 0 1 1 0 1 0 0 1 1 1 1 0 1 0 1 1 |
| C8 | 0 0 1 1 0 1 1 0 1 1 0 0 1 1 1 0 1 0 0 1 0 1 0 1 0 0 0 1 0 1 0 1 0 1 1 1 1 1 0 0 1 0 0 1 0 1 1 1 1 1 1 1 1 1 0 1 1 0 0 0 1 0 1 |

Notice that Kasami sequences are binary and in table 3, -1 was replaced by 0.  Every Kasami sequence is indexed by Cifor i=1,...,8as illustrated in table 3.

The preamble SHR consists  of Nsync=4 repetions of the symbol Si. Such symbol is obtanined by a Kasami sequence of table 3 zero-padded by Lchips. The symbol Sishall be computed as

Si=Ci⊗δL

where δL=(1,0,...,0)1xL and the operator ⊗indicates Kronecker product. Figure x illustrates the construction of symbol Si.

Figure x – Construction of symbol Sifrom Kasami sequence Ci.

For transmission, every element of the Kasami sequence shall have duration ofTw=8 nsec. The chip time shall have duration of Tc=128 nsec corresponding to L=16 for zero-padding.

The preamble symbol Sishall use OOK modulation  for the non-coherent PHY and shall use differentially encoded BPSK modulation for the differentially coherent PHY.

## Start frame delimiter

After the preamble, a start frame delimiter (SFD) is transmitted for frame synchronization. The SFD is balance, for example, when its correlation window is running through the preamble, the output is zero.  Thus, the transition of correlation from preamble to SFD does not degrade the detection of SFD.

# UWB symbol structure

The UWB symbol structure supports on-off signaling and DBPSK/DQPSK. Each symbol timeTsym, consists of an integer number of basis waveform positions,Nw, each of duration Tw. The symbol duration is divided into two intervals of duration Tsym2 in order to enable on-off signaling.  The duty cycle factor during transmission is given by the ratio when a pulse waveform is on (Tw) over the symbol time (when a pulse waveform is on and off during a symbol time), i.e. η=TwTsym. The additional Nw−1 or (Nw2)−1 waveform positions are used for time hopping in order to support multi-BANs for coexistence.

Figure 8 – UWB symbol structure.

# Data Rates

Data rates range from 0.5 Mbps up to 10 Mbps. The mandatory data rate (uncoded) is 0.5 Mbps.

# PSDU timing parameters

The PSDU timing and data rate parameters are given in table 4 and 5. Notice that in case of long pulse shape waveform option, the pulse repetition frequency (number of pulses transmitted in one second) or PRF coincides with the mean PRF (number of pulses transmitted in a symbol period divided by the symbol period), because a single pulse is transmitted per symbol duration.  In case of short pulse shape option, the peak PRF is defined as the maximum rate at which a transmitter emits pulses.  This definition is not applicable for the long pulse shape option.

Table 4 - Data rates for non-coherent modulation.

| R0,R1,R2 | PRF | Nw | Nhop | Tw | Tsym | Sym. rate | FEC | Bit rate | Ncpb | Nguard | P.PRF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (MHZ) |  |  | nsec | nsec | (Msps) | rate | (Mbps) |  |  | (MHz) |
| 0 0 0 | 0.4882 | 32 | 16 | 64 | 2048 | 0.4882 | 0.81 | 0.3954 | 32 | 2 | 499.2 |
| 0 0 1 | 0.9765 | 32 | 16 | 32 | 1024 | 0.9765 | 0.81 | 0.791 | 16 | 4 | 499.2 |
| 0 1 0 | 1.9531 | 32 | 16 | 16 | 512 | 1.9531 | 0.81 | 1.582 | 8 | 8 | 499.2 |
| 0 1 1 | 3.9062 | 32 | 16 | 8 | 256 | 3.9062 | 0.81 | 3.164 | 4 | 10 | 499.2 |
| 1 0 0 | 7.8125 | 32 | 16 | 4 | 128 | 7.8125 | 0.81 | 6.3281 | 2 | 12 | 499.2 |
| 1 0 1 | 15.625 | 32 | 16 | 2 | 64 | 15.625 | 0.81 | 12.656 | 1 | 12 | 499.2 |
| 1 1 0 |  |  |  |  |  |  |  |  |  |  |  |
| 1 1 1 |  |  |  |  |  |  |  |  |  |  |  |

Data rates corresponding to 1 1 0 and 1 1 1 are reserved.

Table 5 – Data rates for the differentially coherent modulation.

| R0,R1,R2 | PRF | Nw | Nhop | Tw | Spr | Tsym | Modul. | Sym. rate | FEC | Bit rate | Ncpb | Nguard | P.PRF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (MHZ) |  |  | nsec |  | nsec | Mode 2 | (Msps) | rate | (Mbps) |  |  | (MHz) |
| 0 0 0 | 0.4882 | 32 | 32 | 64 | 1 | 2048 | DBPSK | 0.4882 | 0.81 | 0.3954 | 32 | 2 | 499.2 |
| 0 0 1 | 0.9765 | 32 | 32 | 32 | 1 | 1024 | DBPSK | 0.9765 | 0.81 | 0.791 | 16 | 4 | 499.2 |
| 0 1 0 | 1.9531 | 32 | 32 | 16 | 1 | 512 | DBPSK | 1.9531 | 0.81 | 1.582 | 8 | 8 | 499.2 |
| 0 1 1 | 3.9062 | 32 | 32 | 8 | 1 | 256 | DBPSK | 3.9062 | 0.81 | 3.164 | 4 | 16 | 499.2 |
| 1 0 0 | 7.8125 | 32 | 32 | 4 | 1 | 128 | DBPSK | 7.8125 | 0.81 | 6.3281 | 2 | 20 | 499.2 |
| 1 0 1 | 15.625 | 32 | 32 | 4 | 1 | 128 | DQPSK | 7.8125 | 0.81 | 12.656 | 2 | 20 | 499.2 |
| 1 1 0 | 3.9062 | 32 | 32 | 8 | 7 | 1792 | DBPSK | 0.558 | 0.81 | 0.452 | 4 | 16 | 499.2 |
| 1 1 1 | 3.9062 | 32 | 32 | 8 | 7 | 1792 | DQPSK | 0.558 | 0.81 | 0.904 | 4 | 16 | 499.2 |

The description of the different parameters is as follows:

## PRF parameter

In case of long pulse shape option, PRF indicates the number of pulses transmitted in one second. It is equal to the mean PRF (number of pulses transmitted in a symbol period divided by the symbol period) as one pulse is transmitted per symbol duration.

## Pulse waveform position parameter

It indicates the number of possible pulse waveform positions within a symbol time:Nw=TsymTw.

## Hop parameter

It gives the number of pulse waveform positions that can contain an active waveform for time hopping (Nhop).

## Pulse waveform duration parameter

It is the pulse waveform duration and computed asTw=NcpbTp. In case of the long pulse shape optionNcpbis 1. In case of short pulse option Ncpb is larger than 1. Notice Tpis the pulse shape duration.

## Spreading parameter (Table 5)

It indicates the number of chips per symbol.

## Symbol duration parameter

It is the PSDU symbol duration (Tsym).

## Modulation mode 2 parameter (Table 5)

It indicates either differentially encoded BPSK or differentially encoded QPSK.

## Symbol rate parameter

It is the number of information symbols transmitted per second and computed asRs=1Tsym.

## FEC rate parameter

It is the channel coding rate, i.e., number of information bits divided by the number of coded bits.

## Bit rate parameter

It gives the bit rate on the air and computed as Rb=Rs⋅FEC rate in case of non-coherent and DBSPK modulation and Rb=2⋅Rs⋅FEC rate  in case of DQPSK.

## Number of pulses parameter

It indicates the number of pulses shapes. In case of the long pulse shape optionNcpbis 1. In case of short pulse option Ncpb is larger than 1. Notice Tp=1/499.2 MHz in case of short pulse shape option.

## Nguard parameter

It is employed for the time hopping generator and computed asτmaxTw.

## Peak PRF parameter

It is defined as the maximum rate at which a transmitter emits short pulse waveforms. This definition is applicable for the short pulse shape option only and it is not applicable for the long pulse shape option.

# UWB Modulations

Once the PLCP sub-layer has constructed the PPDU, i.e., PHR, SHR and PSDU, the PPDU information bits are transformed into RF signals for transmission. For that purpose, there are three modulation schemes. PPDU information bits shall be modulated by either: non-coherent modulation (see 11.3), differentially coherent modulation (see 11.4) or FM-UWB modulation (see 17).

## Scrambling for a burst of pulse shapes

In order to avoid DC bias due to long strings of short pulses with the same polarity (especially in low data rates), spectral shaping through scrambling shall be used (chip scrambling).

A burst of short pulses shall employ the sequences indicated in table 6.

Table 6 - Static scrambling sequences.

| Ncpb | sn |
| --- | --- |
| 1 | 1 |
| 2 | 1 0 |
| 4 | 1 0 1 1 |
| 8 | 1 1 0 1 0 1 0 0 |
| 16 | 1 0 0 0 0 1 0 1 0 1 0 0 1 1 0 1 |
| 32 | 1 0 0 0 1 1 1 1 1 0 0 0 1 1 0 1 0 0 1 0 0 0 0 1 0 1 0 1 1 1 0 1 |

### Optional dynamic scrambling

Optionally, the implementer may use dynamic or time-varying scrambling. In dynamic scrambling, the LFSR shall be clocked at the peak PRF of 499.2 MHz, Ncpb times during each burst period.

The time-varying scrambling sequence smNcpb + n shall be generated from a common LFSR. The polynomial of the LFSR shall beg(x)=1+x2+x12+x13+x14.  Figure 9 shows a typical implementation of the LFSR. By the given polynomial, the corresponding scrambling sequence is generated as

sn=sn − 2⊕sn − 12⊕sn − 13⊕sn − 14, n=0, 1, 2,…

where ⊕ denotes modulo-2 addition.

Figure 9 — Block diagram of scrambling sequence and time-hopping sequence generation.

The LFSR shall be initialized upon the transmission of the first bit of the PHR. The LFSR shall not be reset after transmission of the PHR. The initial state of the LFSR shall be determined from the preamble code. The first 14 bits of the preamble code shall be loaded into the LFSR. Table 7 shows the initial state for the LFSR for each preamble code.

The LFSR shall be clocked at the peak PRF of 499.2 MHz, Ncpb times during each symbol period. The scrambling sequence for the mth symbol shall be smNcpb + n wheren=0, 1, …, Ncpb−1.

Table 7 — Initial state of LFSR for scrambling sequence and time-hopping sequence generation.

| Preamble code number (see 7) | Initial state of LFSR [s−14 s−13 s−12 … s−1] |
| --- | --- |
| 0 | 1 1 1 1 1 1 0 1 0 1 0 1 1 0 |
| 1 | 0 0 0 1 1 0 0 0 1 0 01 0 0 |
| 2 | 1 0 0 0 1 1 1 1 1 0 1 1 1 1 |
| 3 | 0 1 0 0 0 1 0 0 0 0 1 0 1 0 |
| 4 | 1 0 1 0 0 0 0 1 1 1 1 0 0 0 |
| 5 | 1 1 0 1 0 0 1 1 0 0 0 0 0 1 |
| 6 | 0 1 1 0 1 0 1 0 0 1 1 1 0 1 |
| 7 | 0 0 1 1 0 1 1 0 1 1 0 0 1 1 |

## Time hopping sequence

The time-hopping sequence value for the m-th symbol shall be calculated as follows:

- Generate an integer number z(k) using the left-most k shift registers (see in Figure ) as follows:

z(m)=20smNcpb+21smNcpb + 1+ … +2k − 1smNcpb + k − 1

where k=log2(Nhop). As shown Tables 4 and 5, the number of hop burst Nhop is always a power of two, and consequently k is always an integer.

- Calculate the following relevant parameters (or use a look-up table):

α=h(k−1)−γ,

Nreduced=Nhop−α

where γ=Nhop−Nguard−1 and γ can be pre-computed for each data rate.

The parameter Nguard shall be computed as

Nguard=τmaxTw

where τmax is the maximum expected delay spread of channel.

Values of Nguard  are provided in table 3 and 4 for τmax=128 nsec.

- Finally generate the time-hopping sequence value for the m-th symbol as follows:

h(m)= z(m),if h(m−1)≤γ  z(m)+c(m) mod Nreduced + α,if h(m−1)>γ

where c(m)=[m mod 28].

## Non-coherent Modulation

The non-coherent PHY uses on-off signaling (on-off modulation in combination with 64-ary waveform coding). Such modulation strategy assigns binformation bits from an alphabet of size M=2b with a coded-pulse sequence of length 2b (half rate) from a code set alphabet of the same size. This represents the concept of the proposal Group PPM (GPPM).

Figure 10 – Non-coherent modulation/demodulation flow diagram.

### Symbol mapper at half rate

A data symbol contains 6 bits (b0,b1,...,b5). The transmitting symbol is given by the mapping  (b0,b1,...,b5)→(d0,d1,...,d11) as indicated in Table 8.

Table 8 – Symbol mapper for 64-ary waveform-coding.

| Data symbol decimal | Data symbol binary (b0,b1,...,b5) | Codeword (d0,d1,...,d11) | Data symbol decimal | Data symbol binary (b0,b1,...,b5) | Codeword (d0,d1,...,d11) |
| --- | --- | --- | --- | --- | --- |
| 0 | 000000 | 000000111111 | 32 | 100000 | 100110001011 |
| 1 | 000001 | 000011011011 | 33 | 100001 | 100101011010 |
| 2 | 000010 | 000011101101 | 34 | 100010 | 100100101110 |
| 3 | 000011 | 000111000111 | 35 | 100011 | 100011101010 |
| 4 | 000100 | 001001010111 | 36 | 100100 | 100001001111 |
| 5 | 000101 | 011001110100 | 37 | 100101 | 100001111100 |
| 6 | 000110 | 000111111000 | 38 | 100110 | 100100010111 |
| 7 | 000111 | 000011110110 | 39 | 100111 | 100010011101 |
| 8 | 001000 | 000110011110 | 40 | 101000 | 011011000101 |
| 9 | 001001 | 000110110011 | 41 | 101001 | 011011101000 |
| 10 | 001010 | 001100011011 | 42 | 101010 | 011101000110 |
| 11 | 001011 | 101010010011 | 43 | 101011 | 011101011000 |
| 12 | 001100 | 011001001011 | 44 | 101100 | 100001110011 |
| 13 | 001101 | 000101101011 | 45 | 101101 | 011110110000 |
| 14 | 001110 | 110100011001 | 46 | 101110 | 011101100001 |
| 15 | 001111 | 000101011101 | 47 | 101111 | 011110000011 |
| 16 | 010000 | 010001011110 | 48 | 110000 | 010001100111 |
| 17 | 010001 | 010101101100 | 49 | 110001 | 110010100110 |
| 18 | 010010 | 001111010001 | 50 | 110010 | 010010101011 |
| 19 | 010011 | 001111100100 | 51 | 110011 | 010010010111 |
| 20 | 010100 | 001011011100 | 52 | 110100 | 010100001111 |
| 21 | 010101 | 001111001010 | 53 | 110101 | 010100110101 |
| 22 | 010110 | 001110101001 | 54 | 110110 | 010010111100 |
| 23 | 010111 | 001011100011 | 55 | 110111 | 010011110001 |
| 24 | 011000 | 011110001100 | 56 | 111000 | 011000011101 |
| 25 | 011001 | 001010001111 | 57 | 111001 | 011000110011 |
| 26 | 011010 | 001001111001 | 58 | 111010 | 010111100010 |
| 27 | 011011 | 001001101110 | 59 | 111011 | 011011010010 |
| 28 | 011100 | 001010111010 | 60 | 111100 | 010100111010 |
| 29 | 011101 | 001100110110 | 61 | 111101 | 010101010011 |
| 30 | 011110 | 001010110101 | 62 | 111110 | 010111010100 |
| 31 | 011111 | 101100100101 | 63 | 111111 | 010111001001 |

### Pulse shaping

After the symbol mapper and SHR insertion, the pulse shaping shall place a pulse waveform when an input bit is 1, according to the UWB symbol structure and absence of a pulse waveform when an input bit is 0. Thus, the transmitting signal is given by

x(t)=mn=011dnmw(t−nTsym2−6mTsym−hmk)

where m is the m-th transmitting symbol, Tsymis the symbol time, hmkis a time hopping sequence for the k-th BAN and w(t)is the pulse waveform given by

w(t)=i=1Ncpb(1−2smNcpb+(i−1))p(t−(i−1)Tp)

The pulse waveform has durationTw=NcpbTp, where Ncpb=1in case of a long pulse shape waveform and Tpis the pulse shape,p(t), duration. WhenNcpb>1, a burst of short pulses is transmitted. Notice that if Ncpb=1 then smshall be set to 0.

### Detection

On-off signaling does not require defining a threshold for detection as in on-off demodulation. The implementer is recommended to employ soft-detection. Soft detection can be either the correlation of the received signal with every sequence in the codebook at time lag zero or minimum Euclidian distance detection (see Appendix A).

## Differentially coherent modulation

The differentially coherent modulation uses differentially encoded BPSK/QPSK.

Figure 11 – Differentially coherent modulation/demodulation flow diagram.

### DBPSK/DQPSK

The PPDU information bits shall be differentially encoded such that the PSK transmitting symbols are given by

cm=cm−1exp(jϕm)

where cmrepresents the m-th differentially encoded BPSK or QPSK symbol,m=0,1,...,N−1 and c−1=1. Such symbol carries either one bit of information (differentially encoded BPSK) or two bits of information (differentially encoded QPSK). The mapping between information bits onto ϕm is given in tables 9 and 10.

Table 9 - Mapping of information bits onto ϕmfor DBPSK.

| bm | ϕm |
| --- | --- |
| 0 | 0 |
| 1 | π |

Table 10 – Mapping of information bits onto ϕmfor DQPSK.

| am | bm | ϕm |
| --- | --- | --- |
| 1 | 1 | π2 |
| 0 | 1 | π |
| 0 | 0 | −π2 |
| 1 | 0 | 0 |

### Pulse shaping

After the generation of DBPSK/DQPSK symbols, the pulse shaping shall place a pulse waveform according to the UWB symbol structure. Thus, the transmitting signal is given by

xt=m=0N−1cmw(t−mTsym−hmk)

wherem is the m-th transmitting symbol, Tsymis the symbol time, hmkis a time hopping sequence for the k-th BAN and w(t)is the pulse waveform given by

w(t)=i=1Ncpb(1−2smNcpb+(i−1))p(t−(i−1)Tp)

The pulse waveform has durationTw=NcpbTp, where Ncpb=1in case of a long pulse shape waveform and Tpis the pulse shape,p(t), duration. When Ncpb>1a burst of short pulses with Tp=1/499.2 MHz is transmitted. Notice that if Ncpb=1 then smshall be set to 0.

### Differentially encoded PSK modulation with spreading

In order to enhance interference rejection, two data rates use differentially encoded BPSK/QPSK with spreading.  The differentially encoded symbols are given by

xt=m=0N−1cmi=0NB−1seqm(i)w(t−iTsym−mNBTsym−hmk)

seqm(i)=seqm−1(i)b(i)

where b(i)is a Barker code of length NB and seq−1(i)=1  ∀i.

# Operating frequency bands

The UWB band is divided into two band groups: low band and high band as shown in table 11. A compliant UWB device shall transmit in at least one of the specified band groups.

The low band and high band are divided into operating frequency channels as shown in table 11. A UWB device that implements the low band shall support channel 2. The remaining low-band channels are optional. A UWB device that implements the high band shall support channel 7. The remaining high-band channels are optional.

Table 11 – UWB PHY band allocation.

| Band group | Channel number | Central frequency (MHz) | Bandwidth (MHz) | Channel attribute |
| --- | --- | --- | --- | --- |
| Low band | 1 | 3494.4 | 499.2 | Optional |
|  | 2 | 3993.6 | 499.2 | Mandatory |
|  | 3 | 4492.8 | 499.2 | Optional |
| High band | 4 | 6489.6 | 499.2 | Optional |
|  | 5 | 6988.8 | 499.2 | Optional |
|  | 6 | 7488.0 | 499.2 | Optional |
|  | 7 | 7987.2 | 499.2 | Mandatory |
|  | 8 | 8486.4 | 499.2 | Optional |
|  | 9 | 8985.6 | 499.2 | Optional |
|  | 10 | 9484.8 | 499.2 | Optional |
|  | 11 | 9984.0 | 499.2 | Optional |

# Clock accuracy and central frequency alignment

A UWB transmitter shall have a clock accuracy of ±20 ppm. That is a PRF accuracy of ±20 ppm for the long pulse shape waveforms and peak PRF accuracy of ±20 ppm for short pulse shape waveforms. Furthermore, the central frequencies of the band plan shall have an accuracy of ±20 ppm.

# Transmit spectrum mask

The transmit spectrum shall be less than -10 dBr (dB relative to the maximum spectral density of the signal) for  0.651Tp<|f−fc|<0.81Tp ,  -18 dBr for 0.81Tp<|f−fc|<1Tp and -20 dBr for |f−fc|>1Tp.

Notice that Tp=2 nsec. For example, the transmit spectrum mask for channel 7 (fc=7987.2 MHz) is shown in figure 12. The transmit spectrum mask shall be fulfilled by the non-coherent, differentially coherent and FM-UWB PHYs.

Figure 12 - Transmit spectrum mask for band 9.

# Pulse Shape Waveform

There is not a mandatory pulse shape. However, the pulse waveform duration and PRF are mandatory according to the timing parameters of tables 4 and 5 in case of long pulse shape option. In case of short pulse shape option, the pulse waveform duration and peak PRF are mandatory according to the timing parameters of tables 4 and 5. Notice pulse shape waveforms have to fulfill the transmit spectrum mask (see 14) and regulatory spectral mask where applicable.

## Short pulse waveforms

A complaint short pulse shape waveform,p(t), shall be constrained by the absolute value of its cross-correlation with a reference pulse r(t)of at least 0.8 in the main lobe. Such cross-correlation is defined as

ϕ(τ)=1ErEpRer(t)p(t+τ)dt

where Erand Epare the energies of r(t)and p(t), respectively. The reference pulse (having the square root raised cosine spectrum) shall be given by

r(t)=1−β+4βπ                                                       t=0β21+2πsinπ4β+1−2πcosπ4β     t=±Tp4β

The roll-off factor shall be set toβ=0.5, Tp=2 nsec. The implementer is free to choose the truncation ofr(t).

Figure 13 – PSD of r(t)centered at 4492.8 MHz (channel 3) forβ=0.5, Tp=2 nsec truncated to 20Tpsatisfying the transmit spectrum mask.

## Chirp pulse waveform

A compliant chirp pulse waveform in baseband complex representation shall satisfy the following equation:

p(t)=ψ(t)expj2π−Tw2tfi(t')dt'+θ0

whereθ0 is an arbitrary constant phase, ψ(t) is a window function given by

ψ(t)=ψu(t),−Tw2≤t≤−Tw2+Tu,1,−Tw2+Tu<t<Tw2−Td,ψd(t),−Tw2−Td≤t≤Tw2,0,elsewhere,

Tw is the pulse duration defined in tables 4 and 5. Tu and Td are transition times bounded by 0<Tu≤2ns and 0<Td≤2ns respectively. ψu(t) is an arbitrary continuous monotonic non-negative function that satisfies ψu(−Tw2)=0 and ψu(−Tw2+Tu)=1, while ψd(t) is an arbitrary continuous monotonic non-negative function that satisfies ψd(Tw2−Td)=1 and ψd(Tw2)=0. An example of a compliant window function for Tw=32ns is shown in Figure 14.

The chirp’s instantaneous frequency,fi(t) is defined as

fi(t)=Kct+ferr(t),−Tw2≤t≤Tw2.

where Kc is the constant chirping slope that corresponds with the ideal linear chirp and given by

Kc=ΔfTw

The chirp’s frequency sweep shall be set toΔf=f1−f0=520 MHz.

Notice that ferr(t) is an arbitrary instantaneous frequency error function that shall be bounded by

−Tw2Tw2ferr(t)2dtTw0.05Δf

An example of a compliant instantaneous frequency function for Tw=32ns is shown in Figure 15. Examples of the ideal linear chirp pulses’ spectra for different values ofTw, using a Tukey window with Tu=Td=2ns as ψ(t) are shown in Figure 16.

Figure 14 - Example of a window function for Tw=32 nsec and Tukey window with Tu=Td=2 nsec.

Figure 15 – Example of an instantaneous frequency function forTw=32 nsec.

Figure 16 –Power spectral density of chrip pulses fullfiling the transmit spectral mask.

Notice that in case offerr(t)=f0, p(t) is the ideal linear up chirp given by

p(t)=ψ(t)expj2πf0t+Kc2t2+θ0

## Chaotic pulse waveform

Chaotic pulses are near constant envelope signals that are produced by the concatenation of four different triangular or sawtooth waveforms. The sum of these triangular or sawtooth waveforms feeds a voltage controlled oscillator (VCO). This is similar to frequency modulating a carrier signal with the concatenation of the triangular signals. The center frequency of the oscillator is directly proportional to the DC offset of the VCO.

Figure 17 - Chaos waveform generator using triangular waveform

The duration of the triangular pulses and the peak to peak amplitudes are chosen to achieve the 500 MHz bandwidth. The period of the four triangular pulses are chosen as 3, 19, 53 and 59 nsec respectively. Amplitudes are chosen in the ratio of 0.5, 0.2, 0.8, 1 respectively. This summation results in a spectrum shape with 10 dB bandwidth of 500 MHz. The spectrum of the chaotic pulse waveform is shown in figure 18.

Figure 18 – Power spectral density of chaotic pulses with 10 dB bandwidth of 500 MHz.

# Hybrid Type II ARQ Mechanism

This configuration is based on invertible codes. Assuming systematic codewords (codewords consist of information bits plus parity bits), a code is said invertible when a decoder can recover the information bits by an inversion process of parity bits.

Packets detected in error by a cyclic redundancy code (CRC) are not discarded, but rather stored for posterior use. This HARQ scheme requires storing a packet at the transmitter and receiver. Codewords are form by information bits and parity bits of the same length. Packets are created by information bits only or parity bits only. The mechanism of retransmission is alternate repetitions of packets with the information bits or packets with parity bits at the time. The retransmissions continue until the information bits are finally recovered either by inversion or decoding process or a maximum number of retransmission is reached.

Figure 19 shows the flow diagram of HARQ Type II with invertible codes. As HARQ requires the interaction of PHY and MAC, the part of the MAC mechanism for HARQ is described in Section 7.12.2 of the TG6 Draft Document P802.15-10-0245-04-0006. The part of the PHY mechanism is through FEC coding, decoding, FEC inversion and storage of a packet.

## Error detection codes

The cyclic redundancy check code is defined in the MAC document P802.15-09-0196-02-0006. In figure 18, the CRC encoding for parity bits is represented asC0(⋅). The CRC decoding is given byC0−1(⋅).

## Invertible codes

In figure 8, the FEC encoding for parity bits is represented asC1(⋅). The FEC decoding is given byC1−1(⋅) and FEC inversion of parity bits to retrieve information bits D is represented asPD−1.

## PHR iteration

When HARQ is enabled, the fields (H0,H1) in the PHR are set to (1,0). Information bits (D) and its FEC parity bits (PD) form two packets, which are stored at the transmitter. The transmitter starts in state 0 (table 11) and only sendsD. In case of successful reception, the MAC will send an ACK.  Thus the transmitter flushes the storedD, PD and it starts a new transmission. In case of unsuccessful reception (NACK in figure 19), table 12 illustrates the algorithmic flow of HARQ.

Table 12 – Flow of H0,H1 and Rx action in case of packet failure (NACK).

| State | H0 | H1 | Tx | Rx Action |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | Send | If CRC fails go to 1 |
| 1 | 1 | 1 | Send | If CRC and FEC decoding fails go to 2 |
| 2 | 1 | 0 | Send | If CRC and FEC decoding fails go to 1 |

(H0=0,H1=0)  HARQ is disable. (H0=0,H1=1) is reserved.

Figure 19 – Flow diagram of HARQ Type II with invertible codes.

# FM-UWB PHY

FM-UWB is an optional PHY targeting low data rate medical BAN.  FM-UWB exploits high modulation index analog FM to obtain an ultra-wide signal. Frequency modulation has the unique property that the RF bandwidth BRF is not only related to the bandwidth fm of the modulating signal, but also to the modulation index β that can be chosen freely. This yields either a bandwidth efficient narrow-band FM signal (β<1) or a ultra wideband signal (β >> 1) that can occupy any required bandwidth.

FM-UWB constitutes an analog implementation of a spread-spectrum system. This constant-envelope approach, where peak power equals average power, yields a flat spectrum with steep spectral roll-off. After wideband FM demodulation (analog instantaneous de-spreading) in the receiver, the FM-UWB radio behaves like a narrowband FSK radio from a synchronization and detection point-of-view. FM-UWB technology combines low complexity with robustness against interference and multipath. The following sub-sections present the FM-UWB system and its properties.

## Data rates

The data rates indicated in the PHR are defined in table 13. The mandatory data rate (uncoded) is 250 kbps.

Table 13 – FM-UWB data rates.

| R0,R1,R2 | Sym. rate | FEC | Bit rate |
| --- | --- | --- | --- |
|  | (ksps) | rate | (kbps) |
| 0 0 0 | 31.25 | 0.81 | 25.31 |
| 0 0 1 | 62.5 | 0.81 | 50.62 |
| 0 1 0 | 125 | 0.81 | 101.25 |
| 0 1 1 | 250 | 0.81 | 202.5 |

Data rates corresponding to 100, 101, 101, 111 are reserved.

## System characteristics

FM-UWB radios shall satisfy the system characteristics defined in table 14. Operation shall be according to the frequency band plan of table 10 (see 12).  Moreover, FM-UWB shall satisfy the transmit spectrum mask of Section 14.

## Preamble

The preamble SHR shall be formed by the 2nd Kasami sequence defined in table 3.

Table 14 - FM-UWB system characteristics.

| Parameter | Value |
| --- | --- |
| RF center frequency | 6.4 – 8.7 GHz |
| RF bandwidth | 500 MHz |
| RF output power | -15 dBm |
| Subcarrier frequency | 1.0, 1.25, 1.50, 1.75 MHz |
| Subcarrier modulation | FSK, β = 1 |
| Raw bit rates | 31.25, 62.5, 125 and 250 kbps |
| Receiver sensitivity | <-85 dBm1 |
| TX, RX switching time | < 100 μs |
| RX synchronization time | < 500 μs |

1 at BER ≤ 10-6

## Transmitter

Figure 20 provides a block diagram of the FM-UWB transmitter. The low modulation index of frequency shift keying (FSK) is followed by high modulation index of frequency modulation (FM), creating a constant-envelope UWB signal. The transmitter shall consist of a 1-2 MHz subcarrier oscillator generating a triangular signal that is FSK modulated by the transmit data. This subcarrier signal m(t) modulates the RF VCO (wideband frequency modulator), yielding a constant-envelope UWB signal with a flat power spectral density and steep spectral roll-off.

Figure 20 - FM-UWB transmitter block diagram.

The FM-UWB signal V(t) with amplitude A and carrier frequency fc(ωc=2πfc)is obtained by modulating the RF oscillator with the subcarrier m(t) of frequency fm(ωm=2πfm) such that

m(t)=VmM(ωmt)

where Mrepresents the triangular waveform of the subcarrier signal. An RF oscillator sensitivity of K0 [rad/V-s] yields a deviation Δω=2πΔfequal toΔω=KOVm, resulting in a FM signal V(t) that can be expressed mathematically as

Vt=Asinωct+ϕt

=Asinωct+KO−∞tmτdτ

=Asinωct+Δω−∞tMτdτ

Figure 21 illustrates the power spectral density (PSD) of an ideal FM-UWB signal. The signal shall be compliant with the relevant regulatory limits. The signal power is –15 dBm (112 mVpp in a 50 Ω load). The subcarrier frequency is 1 MHz and the deviation Δfis 250 MHz, yielding a modulation index β of 250. The –10 dB bandwidth is 500 MHz.

The subcarrier signal m(t)shall modulate the RF oscillator that implements FM with high modulation index and yields a constant-envelope UWB signal with a flat PSD and steep spectral roll-off at its output.

The PSD of a wideband FM signal is determined by and has the shape of the probability density function (PDF) of the modulating signalm(t). The use of a triangular subcarrier waveform, which is characterized by a uniform PDF, results in a flat RF spectrum. From an implementation point of view the triangular waveform is straightforward to generate using either analog or digital circuit techniques.

Figure 21 - Frequency spectrum of FM-UWB signal at 7.5 GHz center frequency.

## Receiver architecture

It is recommended to demodulate the FM-UWB signal without frequency translation (no mixing). Figure 22 shows an example of a receiver block diagram.  The low complexity receiver comprises a LNA, a wideband FM demodulator, and low-frequency subcarrier filtering, amplification and demodulation circuitry.

Figure 22 - FM-UWB receiver block diagram.

The absence of carrier synchronization allows for fast synchronization. Due to the FM demodulation (instantaneous de-spreading) in the wideband FM demodulator, the system behaves like a narrowband FSK system where synchronization is limited by the bit synchronization time.

# Clear Channel Assessment

The receiver may perform clear channel assessment (CCA) by using either the energy detection or preamble detection.  However, slotted aloha shall be employed.

# Normative References

[1]	IEEE Std 802.15.4a™ 2007.

Appendix A

Soft detection

Assuming symbol and frame synchronization, let Z0,...,Z2b−1 be the quantized values of E0,...,E2b−1after analog to digital conversion (ADC), where En represents sufficient statistics after the RF front-end. Let d'm,n be defined as d'm,n=2dnm−1 such thatd'm,n∈±1.

Decision rule 1:  Correlation of the received signal with every sequence at time lag zero

d=argmaxmZm

where Zm=d'm,0Z0+,...,+d'm,2b−1Z2b−1.

Appendix B

Shortened BCH(31,19)

A shortened BCH(31,19) encoder is implemented from the BCH(63,51) encoder by the following process:

Take 19 information bits, then append 32 zero bits to have 51-bit message block. Encode with BCH(63,51) and remove 32 zero bits in the systematic portion of the codeword such that 31 encoded bits are transmitted. A reverse operation is applied in the decoder.