IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Samsung-ETRI's EFC Proposal for HBC PHY |  |
| Date Submitted | March 2010 |  |
| Source | Jahng Sun Park, HyunKuk Choi, SangYun Hwang, Seong-Jun Song, SeokYong Lee, Eun Tae Won Jung-hwan Hwang, Hyung-il Park, Tae-young Kang, Sung-weon Kang | Samsung Electronics E-mail: jahng.park@samsung.com ETRI E-mail: jhhwang@etri.re.kr |
| Re: | In response to TG6 Call for Proposal: 15-08-0811-03-0006-tg6-call-proposals.doc |  |
| Abstract | Draft text for Samsung Electronics and ETRI’s EFC proposal for HBC PHY. |  |
| Purpose | Discussion |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

1.	Acronyms and Abbreviations	5

2.	HBC PHY Specification	6

2.1.	EFC Packet Structure	6

2.2.	EFC Transmitter	6

2.3.	PHY Preamble	7

2.4.	Start Frame Delimiter (SFD)	9

2.5.	PHY Header	10

2.5.1.	Sync Field	11

2.5.2.	D Field	12

2.6.	S2P and FS-Spreader	12

2.7.	Rate Indicator and Pilot Signal	13

2.7.1.	Rate Indicator using SFD	13

2.7.2.	Pilot Signal	15

2.8.	Transmitter Specifications	15

2.8.1.	Transmit Mask	15

2.8.2.	Transmit Power	16

2.8.3.	Clock Frequency Tolerance	16

2.8.4.	Transmit Timing Requirements	17

2.9.	Receiver Specifications	18

2.9.1.	Receiver Sensitivity	18

List of Tables

Table 1 – Gold Code Generation Polynomials for Preamble	8

Table 2 – Code Set for Preamble	8

Table 3 – FSC Bit Mapping for Preamble (@ 16MHz)	8

Table 4 – Gold Code Generation Polynomials for SFD	10

Table 5 – Code Set for SFD	10

Table 6 – FSC Bit Mapping for SFD (@ 16MHz)	10

Table 7 – PHY Header Field Description	11

Table 8 – Symbol-to-Chip Mapping	13

Table 9 – S2P and FS-Spreader Spreading Results	13

Table 10 – SFD Time Offset and Data Rate Mapping for RI	14

Table 11 – Pilot Insertion Periods	15

Table 12 – TX Mask Specification (for fc = 16MHz)	16

Table 13 – Clock Frequency Tolerance Example	17

Table 14 – Transmit Timing Requirements	17

Table 15 – Minimum Receiver Sensitivity Level	18

List of Figures

Figure 1 – EFC Packet Structure	6

Figure 2 – EFC Transmitter Block Diagram	7

Figure 3 – Preamble Field	7

Figure 4 – Preamble Generation Block Diagram	7

Figure 5 – Preamble Detection Performance	9

Figure 6 – SFD/RI Signal Generation Block Diagram	9

Figure 7 – Header Generation Block Diagram	11

Figure 8 – S2P and FS-Spreader Block Diagram	12

Figure 9 – Rate Indicator using SFD	14

Figure 10 – Pilot Insertion in PSDU	15

Figure 11 – TX Mask Spectrum (for fc = 16MHz)	16

Figure 12 – Transmit Timing Information	17

# Acronyms and Abbreviations

EFC	Electric-Field Communication

FCS	Frame Check Sequence

FSC	Frequency Shift Code

FSDT	Frequency Selective Digital Transmission

HBC	Human Body Communication

HCS	Header Check Sequence

MAC	Media Access Control

PHY	Physical Layer

PLCP	Physical Layer Convergence Protocol

PPDU	Physical Layer Protocol Data Unit

PSDU	Physical Service Data Unit

# HBC PHY Specification

This section presents the proposed EFC specification for HBC PHY.  It covers the entire Physical Layer (PHY) protocol for Body Area Networks, such as packet structure, modulation, preamble/SFD, etc.  This standard is intended to conform to established regulations in the United States, Europe, Japan and Korea.

The EFC specification is designed to provide robust performance for HBC.  EFC transmitter is implemented with only digital circuits and needs one electrode (instead of antenna), and EFC receiver can be implemented without any blocks related to RF carrier signals (mixer, VCO, ADC/DAC, etc.).  These allows EFC device to have very low implementation complexity and low power consumption.

There are two bands of operation centered at 16MHz and 27MHz with the bandwidth of 4MHz.  Both operating bands are for the United States, Japan, and Korea, and the operating band centered at 27MHz is for Europe.  More information regarding the regulation conformation is given in “IEEE 802.15.6 Regulation Subcommittee Report” (doc. IEEE P802.15-08-0034-12-0006).

## EFC Packet Structure

The EFC packet is composed of Preamble, SFD (Start Frame Delimiter), Header, and PHY Payload (PSDU). And PHY Payload is composed of MAC Header, MAC Payload, and FCS (Frame Check Sequence).  The packet structure with PHY Header fields is shown in Figure 1.  The description of each header field is given in Table 7 of Section 2.5.

Figure 1 – EFC Packet Structure

## EFC Transmitter

EFC transmitter uses FSDT (Frequency Selective Digital Transmission) scheme; data is spread in frequency domain using frequency selective spread codes and transmitted in digital form. The dominant frequency where most of transmitting signal is distributed can be selected by using specific frequency selective spread code. The EFC transmitter is composed of following blocks as shown in Figure 2:

- Preamble Generator

- SFD/RI Generator

- Header Generator

- Serial-to-Parallel (S2P)

- FS-Spreader  [FS = Frequency Selective]

- Pilot Generator

- MUX

The generated Preamble, SFD/RI, Header, PSDU, and Pilot signals are sent to an electrode via a MUX.  Since preamble and SFD are fixed data patterns, they are pre-generated and sent ahead of the packet header and payload.  These different signals are transmitted in sequence via a MUX and the electrode. The frequency band for EFC is from 10 MHz to 50 MHz.

Figure 2 – EFC Transmitter Block Diagram

## PHY Preamble

A preamble sequence is transmitted four times (PR1 to PR4) to ensure packet synchronization by the receiver (see Figure 3).

Figure 3 – Preamble Field

Each preamble sequence is created by spreading a 128-bit gold code sequence via Frequency Shift Code (FSC).  FSC uses a repeated [0, 1] code, and the spreading factor (SF) is decided by the number of time FSC is repeated.  If the FSC used is [0, 1], the SF is 2, and if the FSC used is [0, 1, 0, 1], the SF is 4.  For EFC packet preamble, a SF of 4 is used.  In other words, [0, 1, 0, 1] is used for FSC operation.  The operating clock frequency is fCK.  Figure 4 show the preamble generation block diagram.

Figure 4 – Preamble Generation Block Diagram

Table 1 shows the Gold Code Generation Polynomials, and Table 2 shows the code set used for generating preamble sequence.  Table 3 shows the FSC bit mapping used for preamble sequence generation.

Table 1 – Gold Code Generation Polynomials for Preamble

|  | Polynomial 1 | Polynomial 2 |
| --- | --- | --- |
| Polynomial | x10 + x3 + 1 | x10 + x8 + x3 + x2 +1 |
| Initial value | [1:10] (0010010001) | [1:10] (0011111010) |

Table 2 – Code Set for Preamble

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| Bit | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| Code | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| Bit | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| Code | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| Bit | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| Code | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| Bit | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| Code | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| Bit | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| Code | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| Bit | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| Code | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| Bit | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| Code | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |

Table 3 – FSC Bit Mapping for Preamble (@ 16MHz)

| 16MHz channel | 0 | [1, 0, 1, 0] |
| --- | --- | --- |
|  | 1 | [0, 1, 0, 1] |

The specified preamble sequence length was chosen since it provides the desired preamble detection performance even without any RF carrier signal related blocks (mixer, VCO, ADC/DAC, etc.).  The preamble miss detection performance versus SNR is shown in Figure 5.

Figure 5 – Preamble Detection Performance

(Data rate = 125kbps; 128 bit / 2 oversampling)

## Start Frame Delimiter (SFD)

During packet reception, the receiver finds the start of the packet by detecting preamble sequence, and then it finds the starting point of the frame by detecting Start Frame Delimiter (SFD).  Unlike preamble sequence, SFD sequence is sent only once.  The SFD sequence is generated by applying FSC with SF of 4 to a 128-bit gold code sequence.  Figure 6 shows the SFD signal generation block.

By using “time offset,” the SFD field can also indicate the transmitted packets data rate.  With this “Rate Indicator” (RI), the receiver does not need to refer to the PHY header to detect the incoming packet’s data rate.  This allows the header along with the payload be transmitted at the same high data rate increasing transmission efficiency.  More about Rate Indicator is given in Section 2.7.

Figure 6 – SFD/RI Signal Generation Block Diagram

Table 4 shows the Gold Code Generation Polynomials, and Table 5 shows the code set used for generating SFD sequence.  Table 6 shows the FSC bit mapping used for SFD sequence generation.

Table 4 – Gold Code Generation Polynomials for SFD

|  | Polynomial 1 | Polynomial 2 |
| --- | --- | --- |
| Polynomial | x10 + x3 + 1 | x10 + x8 + x3 + x2 +1 |
| Initial Values | [1:10] (0101100000) | [1:10] (0000100010) |

Table 5 – Code Set for SFD

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| Bit | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| Code | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| Bit | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| Code | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 |
| Bit | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| Code | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| Bit | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| Code | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| Bit | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| Code | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| Bit | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| Code | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| Bit | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| Code | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |

Table 6 – FSC Bit Mapping for SFD (@ 16MHz)

| 16MHz channel | 0 | [1, 0, 1, 0] |
| --- | --- | --- |
|  | 1 | [0, 1, 0, 1] |

## PHY Header

When DRF mode (instead of RI) is used, the Header signal is generated as shown in Figure 7.  If RI method is used, the header signal is generated by the block shown in Figure 8.  Table 7 shows the description of each PHY header field.

Figure 7 – Header Generation Block Diagram

Table 7 – PHY Header Field Description

| Bit Position | Field | Length (bit) | Values | Description |
| --- | --- | --- | --- | --- |
| 0 ~ 2 | Data Rate | 3 | 000: 125 Kbps 001: 250 Kbps 010: 500 Kbps 011: 1 Mbps 100: 2 Mbps 101: - 110: - 111: - | PSDU data rate (RI can also be used. See Section 2.7.1) |
| 3 ~ 4 | Pilot Info | 3 | 000: 0.125 ms (16 bytes) 001: 0.25 ms (32 bytes) 010: 0.5 ms (64 bytes) 011: 1 ms (128 bytes) 100: 2 ms (256 bytes) 101: 4 ms (512 bytes) 110: no insertion | Pilot Insertion Interval (Section 2.7.2) |
| 6 ~ 7 | Sync | 2 | 00, 01: no sync 10: frame sync 11: Superframe sync | Frame Sync Information (Section 2.5.1) |
| 8 | D | 1 | 0: normal mode 1: dedicated mode | Dedicated mode (Section 2.5.2) |
| 9 ~ 11 | BAN ID | 3 | 000 ~ 111 | Current BAN ID |
| 12 ~ 15 | Reserved | 4 | - | - |
| 16 ~ 23 | PSDU Length | 8 | 0 ~ 255 | PSDU Length in Bytes |
| 24 ~ 31 | CRC8 | 8 | - | CRC value of PHY Header |

### Sync Field

This field is for synchronization between the master device and slave nodes.  When this field is set to 1x, the internal timing information is updated with the value of registers as below.

- 10 (frame sync):	updated with the pre-set values of the slot and chip count registers. (used for every downlink frame that is sent in sync with the frame period)

- 11 (superframe sync):	updated with the pre-set values of the frame, slot and chip count registers (used for Broadcast frames transmitted at a specified time in a superframe)

### D Field

Dedicated mode allows 1:1 full bandwidth transmission between the master device and a particular slave device.  D field indicates whether the current frame is transmitted in dedicated mode or not.  Other slave devices, if any, stop any communication when this field is set to 1.  Any new device attempting to join the BAN should wait until dedicated mode ends.

## S2P and FS-Spreader

S2P and FS-Spreader generates PHY Header (for RI method) and PSDU.  FS-Spreader is composed of Orthogonal coding and FSC as shown in Figure 8.

Figure 8 – S2P and FS-Spreader Block Diagram

The data to be transmitted is created by mapping 4 bits (a symbol) from FIFO to a 16-bit chip.  Table 8 shows the symbol-to-chip mapping.  The 16-bit chip is then spread by applying FSC.  The spreading factor of FSC used determines the final data rate.  Table 9 shows the number of chips per bit for each possible spreading factor.

Table 8 – Symbol-to-Chip Mapping

| Symbol | Data Bits | Chip | Symbol | Data Bits | Chip |
| --- | --- | --- | --- | --- | --- |
| 1 | 0000 | 1111 1111 1111 1111 | 9 | 1000 | 1111 1111 0000 0000 |
| 2 | 0001 | 1010 1010 1010 1010 | 10 | 1001 | 1010 1010 0101 0101 |
| 3 | 0010 | 1100 1100 1100 1100 | 11 | 1010 | 1100 1100 0011 0011 |
| 4 | 0011 | 1001 1001 1001 1001 | 12 | 1011 | 1001 1001 0110 0110 |
| 5 | 0100 | 1111 0000 1111 0000 | 13 | 1100 | 1111 0000 0000 1111 |
| 6 | 0101 | 1010 0101 1010 0101 | 14 | 1101 | 1010 0101 0101 1010 |
| 7 | 0110 | 1100 0011 1100 0011 | 15 | 1110 | 1100 0011 0011 1100 |
| 8 | 0111 | 1001 0110 1001 0110 | 16 | 1111 | 1001 0110 0110 1001 |

Table 9 – S2P and FS-Spreader Spreading Results

|  | Chip Length | Spreading Factor | Spreading Sequence/Bit |
| --- | --- | --- | --- |
| 2Mbps | 16 | 4 | 16×4/4  16 chip/bit |
| 1Mbps | 16 | 8 | 16×8/4  32 chip/bit |
| 500kbps | 16 | 16 | 16×16/4  64 chip/bit |
| 250kbps | 16 | 32 | 16×32/4  128 chip/bit |
| 125kbps | 16 | 64 | 16×64/4  256 chip/bit |

## Rate Indicator and Pilot Signal

### Rate Indicator using SFD

Besides the default traditional method using Data Rate Field (DRF) in PHY header, the SFD sequence can be used to indicate the data rate of the whole incoming packet, both header and payload.  This concept is called “Rate Indicator” (RI).

With RI, as shown in Figure 9, the transmitter can introduce varying time offset when sending the SFD sequence to indicate a fixed set of information.  By detecting this time offset, the receiver can figure out what particular information is being sent.  With RI, the information delivered is the whole packet’s data rate.

Figure 9 – Rate Indicator using SFD

A total of 24 garbage bits (all 1’s) are introduced to allow time offset in addition to 128 bit Gold code for SFD.  This sums to a total of 152 bits.  FSC with SF of 4 is applied to give the final SFD field length of 608 bits.

SFD Length = (128-bit gold code + 24 bits for time offset) × 4 = 608 bits	(1)

RI can indicate seven (7) different data rates as shown in Table 10.  RI allows both PHY header and PDSU to be transmitted at the same data rate which provides throughput efficiency, especially for high data rates.  To use RI, devices should negotiate using upper protocol any time during a session after initial handshaking.

Table 10 – SFD Time Offset and Data Rate Mapping for RI

| RI | Data Rate |
| --- | --- |
| Toffset1 | 125kbps |
| Toffset2 | 250kbps |
| Toffset3 | 500kbps |
| Toffset4 | 1Mbps |
| Toffset5 | 2Mbps |
| Toffset6 | Reserved |
| Toffset7 | Reserved |

### Pilot Signal

To prevent loosing synchronization due to clock drift, “Pilot” sequence can be inserted in PSDU as shown in Figure 10.  The same sequence used for SFD is used for pilot, and the pilot insertion interval is indicated in the “Pilot Info” field in PHY header.  There are three pilot insertion intervals (Table 11).

Figure 10 – Pilot Insertion in PSDU

Table 11 – Pilot Insertion Periods

| Pilot Info Field | Insertion Period |
| --- | --- |
| 000 | 32 byte |
| 001 | 32 byte |
| 010 | 64 byte |
| 011 | 128 byte |
| 100 | Reserved |
| 101 | Reserved |
| 110 | No pilot insertion |

## Transmitter Specifications

### Transmit Mask

A transmit spectrum mask shall be used to remove harmonics and possible interference in other bands, especially with 400MHz medical band.  The transmit power spectrum shall be less than 0 dBr (dB relative to the maximum spectral density of the signal) for f−fc<fBW, -3 dBr for f−fc=fBW, -0.5 dBr/MHz forfBW<f−fc<4fc as a function of the frequency, and -33 dBr forf−fc>4fc, where fc is channel center frequency and fBW is the channel bandwidth. The recommended transmit spectrum mask for fc=16MHz is shown in Figure 11 and the specification is given in Table 12.

Figure 11 – Transmit Spectrum Mask (for fc=16MHz)

Table 12 – Transmit Spectrum Mask Specification (for fc=16MHz)

| Frequency (MHz) | Mask Specification |
| --- | --- |
| f−fc<fBW | 0 dBr |
| f−fc=fBW | -3 dBr |
| fBW<f−fc<4fc | -0.5 dBr/MHz |
| f−fc>4fc | -33 dBr |

### Transmit Power

The transmit power of a transmitter should be considered with both the conduction and radiation conditions.  The conduction power can be measured with earth-grounded spectrum analyzer (SA) equipment.  The ground of the transmitter shares that of the SA.  However, the radiation power measurement can be conducted in a radiation emission (RE) chamber room where the ground of the device is separated far apart from the SA.  The capacitive coupling under this condition incurs very weak return path.  Hence, the electric-field strength at the output of the transmitter is considerably decreased when compared to the conduction power measurement.

The conduction power shall be more than -12 dBm as the overall power in the frequency band.  The radiation power shall be more than -39 dBm EIRP as the overall power in the frequency band.

### Clock Frequency Tolerance

Pilot symbol enables stable operation instead of variable clock frequency tolerance.  In the case of 64MHz clock, Table 13 shows allowed clock frequency tolerance values for 256 byte transmission according to variable data rates.

Table 13 – Clock Frequency Tolerance Example

(Variable Data Rates; 256-byte transmission)

| Pilot Info Field | Insertion Period | 1 Mbps | 500 kbps | 250 kbps | 125 kbps |
| --- | --- | --- | --- | --- | --- |
| 000 | 16 byte | ±320 ppm | ±160 ppm | ±80 ppm | ±40 ppm |
| 001 | 32byte | ±160 ppm | ±80 ppm | ±40 ppm | ±20 ppm |
| 010 | 64 byte | ±80 ppm | ±40 ppm | ±20 ppm | ±10 ppm |
| 011 | 128 byte | ±40 ppm | ±20 ppm | ±10 ppm | ±5 ppm |
| 100 | Reserved |  |  |  |  |
| 101 | Reserved |  |  |  |  |
| 110 | No pilot insertion | ±20 ppm | ±10 ppm | ±5 ppm | ±2.5 ppm |

### Transmit Timing Requirements

The timing information of the transmit signal is shown in Figure 12.  It has two timing requirements: duty cycle and rising/falling time.  Table 14 and Figure 12 show these requirements under the condition of 15pF capacitive load.

Table 14 – Transmit Timing Requirements

| Parameter | Conditions (CL=15pF) | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Output Duty Cycle | Ratio of twh over (twh+twl) | 48 | 52 | % |
| Rising/Falling Time | 10% to 90% of VDDTX | 2 | 4 | ns |

Figure 12 – Transmit Timing Information

## Receiver Specifications

### Receiver Sensitivity

The minimum receiver sensitivity levels shall be the levels listed in Table 15. The levels are obtained for a packet error rate (PER) of less than 1 % with a payload of 128 bytes. The measured device under the condition shows a noise figure of 10 dB and an implemented loss of 6 dB.

Table 15 – Minimum Receiver Sensitivity Level

| Data Rate (kbps) | Minimum Receiver Sensitivity (dBm) |
| --- | --- |
| 125 | -112 |
| 250 | -106 |
| 500 | -100 |
| 1000 | -94 |
| 2000 | -88 |