4f changes to IEEE 802.15.4 (2006) and IEEE 802.15.4a (2007) spec (433 MHz PHY Only:

### General Description

#### Introduction

Change the following items in the dashed list of 5.1 as shown:

- Carrier sense multiple access with collision avoidance (CSMA-CA) or ALOHA [ultra-wide band (UWB) and 433 MHz] channel access

- 16 channels in the 2450 MHz band, 30 channels in the 915 MHz band, 3 channels in the 868 MHz band, 14 overlapping chirp spread spectrum (CSS) channels in the 2450 MHz band, 16 channels in three UWB bands (500 MHz and 3.1 GHz to 10.6 GHz) and 1 channel in 433 MHz band.

### Physical Layer

#### Physical Layer (PHY)

Change the third paragraph in 5.4.1 as shown:

The features of the PHY are activation and deactivation of the radio transceiver, energy detection (ED), link quality indication (LQI), channel selection, clear channel assessment (CCA), and transmitting as well as receiving packets across the physical medium. The optional UWB PHY also has the optional feature of precision ranging. The radio operates at one or more of the following unlicensed bands:

- 868–868.6 MHz (e.g., Europe)

- 902–928 MHz (e.g., North America)

- 2400–2483.5 MHz (worldwide)

- 3100–10 600 MHz (UWB varies by region)

- 433.05 – 434.79 MHz (worldwide, varies by region)

Insert after 5.4.1.3 the following new subclause:

##### Advantages of 433 MHz PHY

433 MHz PHY intended and optimized primarily (but not restricted) for active RFID type applications within LR-WPAN space. 433 MHz PHY provides extended communication range when compared to higher frequency channels due to reduced signal attenuation, primarily in NLOS environment. Widespread availably of mature sub-gigahertz technology as well as global acceptability of 433 MHz frequency makes this PHY attractive for devices which move frequently from one location to another worldwide, as it is the case with active RFID devices.

One of critical advantages of 433 MHz PHY is its independence from any other 802 PHY allowing full coexistence and non-interference. This is important for active RFID systems where number of devices can be very high in very limited physical space.

## PHY Specification

### General Requirements and Definitions

Insert the following item at the end of the dashed list in 6.1:

- An 433 MHz MHz PHY employing minimum phase-shift keying (MSK) modulation

Insert the following item at the end of the 6.1:

In further additions to the rates supported in IEEE Std 802.15.4-2006 and IEEE Std 802.15.4a - 2007, additional, 433 MHz PHY has been added.

#### Operating Frequency Range

Insert the following row in “Table 1 – Frequency Bands and Data Rates”:

| PHY (MHz) | Frequency band (MHz) | Chip Rate (kchp/s) | Modulation | Bit Rate (kb/s | Symbol Rate (ksymbol/s) | Symbols |
| --- | --- | --- | --- | --- | --- | --- |
| 433 | 433.63 – 434.21 | ? | MSK | 250 | 250 | ? |

Insert the following rows at the end of 6.1.1:

Japan:

- Document ARIB STD T92

United States:

- Document: FCC CFR47 Section 15.231(e)

Canada:

- Document: IC RSS210

#### Channel Assignments

##### Channel Numbering

?????? Can unused Channels in existing Channel Pages 0,1 and 2 be used for 433 MHz channel. Currently channels 27-31 are unused in these 3 Channel Pages ??????

##### Channel Pages

Same as for 6.1.2.1

##### Minimum long interframe spacing (LIFS) and short interframe spacing (SIFS) periods

Insert the following new rows at the beginning of Table 3:

Table 7—PD-DATA.confirm parameters

| Channel Page (decimal) | macMinLIFSPeriod | macMinSIFSPeriod | Units |
| --- | --- | --- | --- |
| 433.63 – 434.21 MHz MSK | 40 | 12 | Symbols |

### PHY Service Specifications

##### PD-DATA.Indication

###### Semantics of Service Primitive

Insert the following new parameters at the end of the list in 6.2.1.3.1 (before the closing parenthesis):

ppduRSSI

Insert the following new rows at the end of Table 8:

Table 8—PD-DATA.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ppduRSSI | Integer | 0x00 - 0xFF | Received Signal Strength measured during reception of PPDU (see x.x.x) |

Sections from 6.2.2 to 6.3 not completed

### PPDU Format

#### Preamble Field

Insert the following new rows at the beginning of Table 19:

Table 19 — Preamble field length

| PHY | Length |  | Duration (us) |
| --- | --- | --- | --- |
| 433.63 – 434.21 MHz MSK | 4 octets | 32 symbols | 128 us |

#### SFD Field

Insert the following new rows at the beginning of Table 20:

Table 20 — SFD field length

| PHY | Length |  |
| --- | --- | --- |
| 433.63 – 434.21 MHz MSK | 2 octets | 16 symbols |

### 433 MHz PHY Specification

Note: This Chapter should probably be numbered 6.8b in order to preserve continuity of 4 /4f numbering.

#### 433 MHz Data Rate

The data rate of 433MHz MSK PHY shall be 250 kbps.

#### Modulation and Spreading

433 MHz PHY employs Minimum Shift Keying. This is special case of frequency shift keying where the transition between two bits occurs when the phase of frequencies is either 0° or 180° in order to minimize undesirable effects of phase discontinuation during frequency switching. This is also known as continuous phase frequency shift keying.

##### Reference Modulator Diagram

##### Bit-Top-Symbol Mapping

##### Symbol-To-Chip Mapping

##### MSK Modulation

###### Pulse Shape

###### Chip Transmission Order

Definitions

As of June 2009, please use the following introductory paragraph. Also, there is no longer a requirement to number definitions per the 2009 IEEE Standards Style Manual. Refer to the 2009 IEEE Standards Style Manual for updates.

For the purposes of this document, the following terms and definitions apply. The IEEE Standards Dictionary: Glossary of Terms & Definitions should be referenced for terms not defined in this clause.

# (informative)
Bibliography

The IEEE Standards Dictionary: Glossary of Terms & Definitions is available at http://shop.ieee.org/.