IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Necessary Changes to LL-Beacon Definition |  |
| Date Submitted | [15 March, 2010] |  |
| Source | [Michael Bahr] [Siemens AG] [Otto-Hahn-Ring, 80200 München, Germany] | Voice: [ ] Fax: [ ] E-mail: [ bahr et siemens dod com ] |
| Re: | [These are necessary text changes to the current version of the TG4e draft standard document 15-10/604r6. These changes have been accidentially removed during the editing process.] |  |
| Abstract | [Necessary changes to the description of the definition of the LL-Beacon frame. |  |
| Purpose | [To include the changes into the next version of the TG4e draft standard document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Instruction to Editor: Change clause 7.2.5.1.2 Beacon frame format, starting on page 77 (document 15-10-0604-06) as indicated by the winword change marks.

        - Beacon frame format

          - General

The Beacon frame with shortened frame control (1 octet MAC header) is sent during the beacon slot in every superframe. The structure of a Beacon frame depends on the current transmission mode (see Error: Reference source not found). The general structure of the beacon frame is shown in Figure 0.a.

- Figure 0.a—Format of the Shortened Beacon Frame

The beacon frame does have a very short MAC header (MHR) of one octet containing the frame type and sub frame type, followed by the beacon payload and the MAC footer (MFR). The beacon payload contains the transmission mode and several flags and information fields, those existences depend on the current transmission mode.

          - Beacon frame MHR fields

The beacon frame does have a very short MAC header (MHR) of one octet containing the Shortened Frame Control field.

In the Shortened Frame Control field, the Frame Type subfield shall contain the value that indicates a MAC frame with a shortened frame control, as shown in Table 120, and the Sub Frame Type subfield shall contain the value that indicates a beacon frame, as shown in Error: Reference source not found.

          - Beacon Payload fields

The beacon payload is of variable length. It contains flags, which include the transmission mode, actuator direction, and the length of the management timeslots, as well as the Gateway ID and configuration sequence number, the size of a base time slot, the number of base timeslots in the superframe, and a group acknowledgement. Some of the fields are omitted in some transmission modes. The structure of the beacon payload for beacon is depicted in Figure 0.b.

- Figure 0.b—Beacon payload

The Flags field contains several control information. The structure of the Flags field is shown in Figure 0.c.

- Figure 0.c—Structure of Flags field of Beacons with 1-octet MAC-Header

The Transmission Mode subfield defines the transmission modeThe values for the different transmission modes are specified in Table 0..

- Table 0.—Transmission Mode settings

| Bits 0-2 | Transmission Mode |
| --- | --- |
| 000 | Online Mode (see Error: Reference source not found) |
| 100 | Discovery Mode (see Error: Reference source not found) |
| 110 | Configuration Mode (see Error: Reference source not found) |
| 111 | Mode Reset: The devices reset their state of the discovery or configuration mode. no significance. |

The Actuator Direction subfield indicates the transmission direction of all actuator time slots. The bit defines the transmission direction of all actuator time slots during this superframe. If the Actuator Direction subfield is set to 0, the direction of all actuator time slots is uplink (from actuator to LL NW PAN coordinator). If the Actuator Direction subfield is set to 1, the direction of all actuator time slots is downlink (from LL NW PAN coordinator to actuator). The Actuator Direction subfield is only used in online mode.

The Number of Base Timeslots per Management Timeslot subfield contains the number of base time slots per management time slot. This value applies to both the downlink and the uplink management time slot. A value of 0 indicates that there are no management time slots available in the superframe.

The Gateway ID field contains the 1-octet short address of the LL_NW PAN coordinator.

The Configuration Sequence Number field contains an integer number that identifies together with the Gateway ID the current configuration of the LL-network.

The Timeslot Size field defines the length of a base timeslot through the number of bytes of the data payload of a data frame of frame type b100 (frame with 1-octet MAC header). The actual timeslot size in bytes is calculated as

tTS := ((p + (m + n)) * s + 12 {if m+n  18 octets} or 40 symbols {if m+n > 18 octets}) / v

with the description and values for the 2450 MHz PHY as an example as shown in Table x-1.

| Variable | Description | Value for 2450 MHz PHY |
| --- | --- | --- |
| p | Number of octets of PHY header | 6 octets |
| m | Number of octets of MAC overhead (MHR + MFR) | 3 octets for data frames of type b100 (1-octet MAC header) |
| n | Number of octets of data payload | Value of field Timeslot Size of beacon frame of type b100 (1-octet MAC header) |
| s | Number of symbols per octet | 2 symbols per octet |
| v | Symbol rate | 62,500 symbols / s |

Table x-1

The Number of Base Timeslots in Superframe field contains an integer number that represents the number of base timeslots for sensors and actuators immediately following the management timeslots of the superframe (corresponds to macFAnumTimeSlots). The Number of Base Timslots in Superframe field is only present in online mode.

The Group Acknowledgement field is a bitmap of length (macFAnumTimeSlots – macFAnumRetransmitTS) bits as shown in Error: Reference source not found and Figure 0.d to indicatd successful sensor and actuator transmissions from the previous superframe. In the separate group acknowledgment configuration, this field is not present in the beacon. The Group Acknowledgement field is only present in online mode. The Group Acknowledgement field contains a bit field where each bit corresponds to a time slot associated with a sensor device or an actuator device excluding retransmission time slots. Bit b0 of the Group Acknowledgement bitmap corresponds to the first time slot after the macFAnumRetransmitTS retransmission time slots, bit b1 of the Group Acknowledgement bitmap corresponds to the second time slot, and so on. Bit value 1 means the sensor transmission was successful, and bit value 0 means the sensor transmission in the previous superframe failed and the sensor is allocated a time slot for retransmission in the current superframe. Because concatenated time slots are multiples of base time slots, a concatenated time slot of length of n base time slots will have n bits in the group acknowledgement bitmap at the corresponding positions.

- Figure 0.d—Structure of Group Acknowledgement bitmap

If the LL NW PAN coordinator received a data frame successfully in a time slot associated with a sensor device or an actuator device during the previous superframe, it shall set the corresponding bit in the Group Acknowledgement field to 1, otherwise to 0 (corrupted transmission, no transmission). If the data frame has been received during a shared group time slot, all corresponding bits of this shared group time slot will be set accordingly in the Group Acknowledgement bitmap.