IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15.7 Task Group Visible-Light Communication (TG-VLC) |  |
| --- | --- | --- |
| Title | Color stabilization for CSK by use of visibility frames |  |
| Date Submitted | 17th May 2010 |  |
| Source | Joachim W. Walewski and Michael Bahr | Voice: +49-89-636-45850 Fax: E-mail: joachim.walewski@siemens.com |
| Re: |  |  |
| Abstract | We describe an extension of CSK for duplex links, so that the point of gravity of the CSK constellation diagram can be kept stable in case of changes in the output characteristics of a CSK transmitter. In contrast to 15-10-0089-02-0007 only the low-frequency dynamics of the transmitter is compensated for. Visibility frames are exploited for inferring the compensation coefficients in the Tx. Both PHY and MAC mechanisms are discussed in detail. We highlight the parts of this text that are of tutorial nature and that would not need to be included when porting this document into IEEE 802.1.57 D2 |  |
| Purpose | Contribution of standard-text material for the preparation of IEEE 802.15.7 D2 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Document # | 15-10-0262-00-0007 |  |

- Nomenclature

  - Symbols

| Symbol | Declaration |
| --- | --- |
| □(b) | Binary representation of □ |
| {.}A/D | A/D converted value of {.} |
| {.}D/A | D/A converted value of {.} |
| al | D/A scaling factor for band l |
| bl | A/D scaling factor for band l |
| a | One-dimensional quantity |
| a | Column vector |
| A | Matrix |
| elm | Sensitivity of photo detector for band m when receiving emission in band l [A/W] |
| i | Current [A] |
| p | Optical power [W] |
| Q | Diagonal matrix proporational to the quantum efficiencies of the LEDs in bands i, j, k [W/A] |
| s | Signal |
| sRx | Vector of received signals |
| sTx | Vector of transmitted signals |
| t | Time [s] |
| tlm | Transmittance from LED l to photodetector m [unit 1] |
| T | Transmittance matrix [unit 1] |
| Tcomp | Time between two color compensations [s] |

  - Abbreviations

| Abbreviation | Long form |
| --- | --- |
| CSK | Color-shift coding |
| D1 | IEEE 802.15.7 Draft 1, “IEEE Standard for Information technology — Telecommunications and information exchange between systems — Local and metropolitan area networks — Specific requirements — Part 15.7: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Visible Light Wireless Personal Area Networks (WPANs)”, February 2010 |
| LED | Light-emitting diode |
| MAC | Media-access control |
| MLME | MAC-layer management entity |
| PD | Photodetector |
| PHY | Physical layer (OSI layer 1) |
| Rx | Receiver |
| Tx | Transmitter |

- Description of use case

The quantum efficiency of LEDs is dependent on their operation time and age, and also on various environmental conditions. Among those, temperature is generally the most important factor. When using CSK modulation all these factors lead to a change of the center of gravity and a distortion of the CSK constellation diagram of the emitted signal. Clause D1-6.8.6.1 describes how the receiver adapts to such changes. In this Clause we describe how one can counteract the above change for low-frequency modulation of the transmitter side, i.e. the shift of the center of gravity of the CSK constellation diagram. By so doing one can stabilize the average color of the emitted light. Notice that this compensation scheme only is applicable to duplex links.

- Control loop

The control-loop model for the color-stabilization scheme is shown in Figure 1. Visibility patterns (see Clause D1-6.9.6) are sent from the tri-band Tx of an CSK-PHY (see Clause D1-6.8) to the tri-band Rx. The corresponding received signals feed back to the LEDs in the tri-band Tx. An optional back link is used to relay these signals back to the tri-band Tx, where they are used to correct the LED driving currents in such a way that the center of gravity of the constellation diagram is moved back to its initial position.

Figure 1 – Control loop for a color-stabilized CSK link. The calibration sequences are discussed in detail in Clause 4. The back link used for routing the compensation factors back to the tri-band Tx does not need to be a CSK link.

- PHY-layer mechanism

  - General approach

Here, we discuss the proposed compensation scheme for the general case of a nonlinear characteristic function (radiant power vs. driving current). However, in light of its high usability, we also briefly discuss the case of a linear characteristic function. Also notice, that we only attempt to apply this scheme for a visibility-pattern line rate that is slow enough to invoke thermal and other low-frequency relaxation effects in the transmitter. However, the scheme is also applicable to high-frequency modulation.

Figure 2 – Characteristic curve of an LED (radiant power as a function of driving current) for two quantum efficiencies and ac driving currents.

Figure 2 displays an example alteration of the transmitter characteristic function due to a change in quantum efficiency. For a driving current i, the initial radiant power drops from p0 to p’0. In order to regain the original radiant power p0 the driving current has to be changed to i’.

As shown in Figure 3, the signal to be transmitted over a CSK link, viz.sTx(b), is translated into the ac current vector iTx by aid of D/A convertors. The transmitted radiant power of the transmitter is, according to Figure 4, given by

pTx=f(iTx),

where f is a vector function consisting of the characteristic functions for each band. Examples for such vector functions are polynomial fits of the static characteristic function of the LEDs or Volterra-series approximations of the dynamic response of the LEDs.

For a simple linear relationship between radiant power and the driving current this Equation can be written as

pTx=QiTx,

where Q is a diagonal matrix, whose values are proportional to the quantum efficiencies of the individual LEDs.

Figure 3 – Definition of system vectors and matrices for a CSK link.

According to Figure 3 and Figure 4, the received radiant power is

pRx=TpTx,

where T is the transmission matrix. The pertinent current output of the photodetectors is

iRx=EpRx,

where E represents the spectral sensitivity of the photodetectors. The photodetector currents are translated into digital representations, viz.sRx(b), by aid of D/A convertors.

Figure 4 – Definition of further transfer matrices for a CSK link.

The relation between the transmitted and received signal is thus given by

sRx(b)=BETfAsTx(b)D/AA/D.

A change in the quantum efficiency of an LED in any of the three bands results in the change of f to f’. If the transmitter has access tosRx(b), the new vector characteristic function can be calculated from

f'AsTx=fAsTxfAsTxTsRxfAsTxT−1sRx,,

where sRx and sRx,are the receiver signals before and after the change in the transmitter efficiency, respectively.

The overall goal of this compensation strategy is to achieve a stabilization of the CSK color center of gravity by transforming the transmitted signals with a compensation function c so that

sRx(b)=BETf'AcsTx(b)D/A,

or, in a briefer version

fAsTx=f'AcsTx.

In general, this is a rather complex inverse problem, but there exist at least two practical cases, for which this problem can be solved in a closed form. In both cases one can write

csTx=CsTx,

where C is a diagonal matrix.

First, if only the magnitude of the characteristic curves change but not their shape (as is the case in Figure 2), the vector characteristic function can be written as

f(iTx)=QgiTx,

where Q is a diagonal matrix that can be understood as a generalized quantum efficiency. In contrast, the vector function g is independent of the transmitter efficiency. The compensation factors for this case can be calculated with

C=A−1g−1gAsTxgAsTxTsRx,gAsTxT−1sRxsTxsTxT−1.

Second, for the purely linear case [g(i) ~ i] the calculation of the compensation matrix C simplifies to

C=sTxsTxTsRx,sTxT−1sRxsTxTsTxsTxT−1.

In the following we will only consider schemes relying on the compensation matrix C.

  - Choice of data sent over the CSK channel

The CSK constellation maps to be used in IEEE 802.15.7 are shown in Figure 5. It is adamant not to change the radiant color of the emitter when observed by a human, i.e. the center of gravity of any symbol sequences used for color stabilization has to coincide with the center of gravity of the pertinent constellation diagram. There are two options: either (a) only submit the symbol coinciding with the center of gravity of the constellation diagram or (b) submit a sequence of alternating symbols that fulfill said condition. Since approach (a) is not feasible for 8 CSK (see Figure 5), approach (b) is more general and thus more feasible. The approach we recommend for IEEE 802.15.7 is as follows. Transmit alternating symbol sequences that cycle through the corner points of the CSK constellation diagrams, viz. ‘B’, ‘G’, and ‘R’ (see Figure 5).

Figure 5 – CSK constellation maps for IEEE 802.15.7 [Yokoi, 2008].

In order to measure the low-frequency thermal dynamics of the LED, which is of relevance for the bias-current stabilization of the CSK center of gravity, sending only on symbol for each corner is too short. Typical thermal response times of LEDs lie in the µs range, which is substantial longer than the length of an IEEE-802.15.7 CSK symbol (< 100 ns). Therefore, we propose to repeat each symbol so often that thermal equilibrium in the LED is reached before switching to the next symbol. The received signal of relevance for the afore-discussed signal vector sRxis then only acquired for the last sent repetitive symbol, or an average over a suitable number of last samples. For typical LEDs and the line rates in D1, this translates to more than 100 repetitions of each symbol before sending the next symbol in the ‘corner cycle’. One can send back the sRx for each corner symbol.

In order to simplify messaging between CSK Rx and CSK Tx (see Figure 1), we suggest that, especially in the case of a linear characteristic function, that instead of sending a vector sRxto the CSK Tx for each corner point, the CSK Rx sends the average of the signal vector acquired for all three corner points.

When choosing the number of symbol repetitions one also has to ensure that the aggregate time of cycling through all three corner symbols is (substantially) shorter than the time scale on which the human eye perceives this cycling as flicker (see Clause 5.5.4 in [IEEE 802.15.7]). For example, if the longest ‘non-flicker’ cycle period is limited to 5 ms, each corner has to be cycled through in 1.66 ms or less.

Since changes in the quantum efficiency of the LEDs rather occurs on a long time scale (seconds, minutes, hours), it is not necessary to send back sRx after each suitable set of visibility frames. Details of this message exchange are addressed in the MAC layer (see Clause 5.3).

- MAC-layer mechanisms

  - General guidelines for estimation of the compensation factors

In order for the proposed compensation scheme to function, reference received signal values are needed. They are relayed back to the CSK transmitter via an optional back channel (see Figure 6). In the following these reference values are referred to as SRx,0 and stored on the transmitter side. Recurring measurements of the received signals (for the same sent signal) results in the vectors SRx,s for time ts. If the elements of SRx,c differ significantly from those of the diagonal elements of SRx,0, the transmitted signals are corrected with the diagonal elements of C. A decision about whether an update can be made by aid of the value of the diagonal elements of C itself. If these values differ significantly from 1, an update of the current compensation factors is made. Whether this difference is significant can either be decided by a preset value (e.g., 5% difference), or the decision threshold can be estimated from the data itself. One example is to acquire a histogram of the diagonal values of C. A significant value can then be defined as lying outside a pre-defined confidence interval, e.g. the 95% confidence interval.

In order to avoid damage or saturation of the LEDs or a bit-level saturation of the A/D and D/A convertors, one can introduce a-priori maximum levels for the three values in sTx(b).

Figure 6 – Detailed implementation of a color-stabilized CSK link. The received signals are relayed back to the transmitter, and the CSK receiver a channel-estimation module in the receiver estimates compensation factors and sends them back to the CSK transmitter via an additional channel.

Changes that affect all channels similarly, e.g. a change in the link length, can be identified by comparing all diagonal values of C. If all elements experience the same relative change the compensation factors for the Tx driving currents shall not be updated.

If the off-diagonal elements of C significantly differ from zero this indicates other impairments of the CSK-modulated link than those hither discussed. An example for such impairment is the blocking of only one of the photodiodes against cross talk. E.g., photodiode i still receives light emitted from LED i, but not longer from the other bands. In such a case the Tx driving current shall not be compensated for. Instead one can use this information for, e.g., error messages created by the CSK Tx, the CSK Rx, or both. Another option is to update SRx,0 and to use the update for further compensation.

  - MAC sublayer specifications

    - Association primitives

The message sequences (both on the PHY layer and in the MLME) for the association of a (network) device to a coordinator is shown Figure 7. In the following we are detailing MLME primitives for the association of devices that offer color stabilization of CSK links. Hereby, the device possesses at least a CSK-enabled Rx, and the coordinator at least a CSK transmitter. Differences of the primitives to those in D1 are highlighted. Then necessary changes in the MLME fields are then discussed in Clause  and the consequent changes in the transmitted MAC frames in Clause . We conclude our discussion with a description of the MAC functions in 5.3.

Figure 7 – Message-sequence chart for association [IEEE 802.15.7,2009]

      - MLME-ASSOCIATE.request

        - Semantics of the service primitive

The semantics of the MLME-ASSOCIATE.request primitive in Figure 7 is as follows.

MLME-ASSOCIATE.request		(

LogicalChannel,

CoordAddrMode,

CoordWPANId,

CoordAddress,

CapabilityInformation,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

All parameters of the primitive except the CapabilityInformation (bitmap) are the same as detailed elsewhere [IEEE 802.15.7, 2009]. Changes made to the CapabilityInformation parameter, which specifies the operational capabilities of the associating device, are discussed in Clause .

      - MLME-ASSOCIATE.indication

        - Semantics of the service primitive

The semantics of the MLME-ASSOCIATE.indication primitive in Figure 7 is as follows.

MLME-ASSOCIATE.indication	(

DeviceAddress,

CapabilityInformation,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

All parameters of the primitive except the CapabilityInformation (bitmap) are the same as detailed elsewhere [IEEE 802.15.7, 2009]. Changes made to the CapabilityInformation parameter, which specifies the operational capabilities of the associating device, are discussed in Clause .

7.1.3.3 MLME-ASSOCIATE.response

Instruction to editor: Change clause 7.1.3.3.1 Semantics of the service primitive as indicated by the WinWord change marks

7.1.3.3.1 Semantics of the service primitive

The semantics of the MLME-ASSOCIATE.response primitive in Figure 7 is as follows.

MLME-ASSOCIATE.response	(

DeviceAddress,

AssocShortAddress,

status,

CapabilityNegotiationResponse,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

All parameters of the primitive save the CapabilityNegotiationResponse (bitmap) are the same as detailed elsewhere [IEEE 802.15.7, 2009]. Changes made to the CapabilityNegotiationResponse parameter, which specifies response of the coordinator to the association request of the joining device, are discussed in Clause .

7.1.3.4 MLME-ASSOCIATE.confirm

Instruction to editor: Change clause 7.1.3.4.1 Semantics of the service primitive as indicated by the WinWord change marks

7.1.3.4.1 Semantics of the service primitive

The semantics of the MLME-ASSOCIATE.confirm primitive in Figure 7 is as follows.

MLME-ASSOCIATE.confirm	(

AssocShortAddress,

status,

CapabilityNegotiationResponse,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

All parameters of the primitive save the CapabilityNegotiationResponse (bitmap) are the same as detailed elsewhere [IEEE 802.15.7, 2009]. Changes made to the CapabilityNegotiationResponse parameter, which specifies response of the coordinator to the association request of the joining device, are discussed in Clause .

7.2 MAC-frame formats

7.2.3 Information elements

Instruction to editor: Change clause 7.2.3.2 Capabilities Information Elements as indicated by the WinWord change marks

7.2.3.2 Capabilities Information Element

The capabilities information element is used to convey device MAC and PHY capabilities to peer devices. The capabilities IE (see Figure 8) consists of two fields: the capability information field (see Table 1), which indicates general capabilities of the device. The second field is the aggregation bitmap field (Figure 80), which facilitates the setup of point-to-multipoint networks (see 7.2.3.2.2).

| Octets: 9 | 1 |
| --- | --- |
| Capability Information Field | Aggregation Bitmap Field |

Figure 8 – Capabilities information element

7.2.3.2.1 Capability Information Field

|  | Bit | Function |
| --- | --- | --- |
| MAC-layer capabilities | 0 | Power Source |
|  | 1-2 | Battery information |
|  | 3 | Security capability |
|  | 4 | Coordinator capability |
|  | 5 | Traffic support |
|  | 6-7 | Topology support |
|  | 8-9 | Device type |
|  | 10 | Beacon support |
|  | 11 | Dimming support in MAC |
|  | 12 | Continuous visibility transmission (for infrastructure) |
|  | 13-23 | Reserved |
| PHY-layer capabilities | 24 | PHY-I support |
|  | 25 | PHY-II support |
|  | 26 | Alternate PHY (CSK) support |
|  | 27-28 | Color-stabilization capability (CSK) |
|  | 29 | Dimming support in PHY (VPM) |
|  | 30-32 | Max supported Tx clock |
|  | 33-35 | Max supported Rx clock |
|  | 35 | Explicit clock notification request |
|  | 36-48 | Reserved |
| Band capabilities | 8*n | Aggregate channels |
|  | 8*n | Guard channels |
|  | 49-56 | Bands used for CSK (any 3 bits of the bits set to 1 can be used) |
| Physical-device capabilities | 57-59 | Number of optical source types |
|  | 60-62 | Multiple direction support |
|  | 63-72 | Number of LEDs per optical source type |

Table 1 – Capability Information Field

The Alternate PHY (CSK) support subfield is one bit in length. It shall be set to 1 if the device supports CSK.

Instruction to editor: Move current (D1) clause 7.2.3.4 Data frame format to 7.2.2.4. Insert new clause 7.3.2.4 Capabilities-Negotiation-Response Information Elements as given below

7.3.2.4 Capabilities Negotiation Response information element

The Capabilities Negotiation Response information element is used to inform a device about MAC and PHY capabilities that will be used when communicating with the coordinator. The Capabilities Negotiation Response information element (see Figure 9) consists of two fields: the Capability Negotiation Response field (see Table 2), which indicates general capabilities of the device, and the Aggregation Bitmap field facilitating the setup of point-to-multipoint networks (see 7.2.3.2.2).

| Octets: 1 | 1 |
| --- | --- |
| Capability-Negotiation-Response Field | Aggregation Bitmap Field |

Figure 9 – Capability Negotiation Response element

The Capability Negotiation Response field indicates general capabilities of the device that will be used during communication with the coordinator. Its composition is detailed in Table 2

|  | Bit | Function |
| --- | --- | --- |
| Requested MAC-layer capabilities | 0-TBD | Reserved |
| Requested PHY-layer capabilities | TBD (+2) | Color stabilization scheme |
|  | TBD (+TBD) | Reserved |
| Band capabilities | TBD (+TBD) | Reserved |
| Physical-device capabilities | TBD (+TBD) | Reserved |

Table 2 – Capability Negotiation Response field

The color-stabilization-scheme function is detailed in Table 3

| Bits | Color-stabilization scheme |
| --- | --- |
| 00 | No color-stabilization |
| 01 | Color stabilization information to be sent from device to coordinator upon reception of visibility frames |
| 10 | Color stabilization information to be sent from coordinator to device upon reception of visibility frames |
| 11 | Color stabilization information to be sent from device to coordinator and from coordinator to device when either receives visibility frames |

Table 3 – Color-stabilization scheme

The dimming support in PHY capability subfield is one bit in length. It shall be set to 1 if the device is capable of supporting dimming in the PHY using VPM and idle patterns for receive. Otherwise, it shall be set to 0.

7.3 MAC command frames

The command frames defined by the MAC sublayer are listed in Table 4. A coordinator shall be capable of transmitting and receiving all command frame types, with the exception of the GTS request command, while the requirements for a device are indicated by an “X” in the table.

| Command-frame identifier | Command name | Device |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | Tx | Rx |  |
| 0x01 | Association request | X |  |  |
| 0x02 | Association response |  | X |  |
| 0x03 | Disassociation notification | X | X |  |
| 0x04 | Data request | X |  |  |
| 0x05 | WPAN ID conflict notification | X |  |  |
| 0x06 | Beacon request |  |  |  |
| 0x07 | Coordinator realignment |  | X |  |
| 0x08 | GTS request |  |  |  |
| 0x09 | Blinking notification |  |  |  |
| 0x0a | Dimming notification |  |  |  |
| 0x0b | Fast link recovery |  |  |  |
| 0x0c | Mobility notification |  |  |  |
| 0x0d | Information element exchange |  |  |  |
| 0x0e | GTS response |  |  |  |
| 0x0f | Clock-rate change notification |  |  |  |
| 0x10 | Multiple-channel assignment |  |  |  |
| 0x11 | Channel hopping |  |  |  |
| 0x12 | VLC cell and mobility |  |  |  |
| 0x13 | Visibility-frame transmission |  |  |  |
| 0x14 | Color-stabilization-timer notification | X | X | 5.2.2 |
| 0x15 | Color-stabilization information | X | X | 5.2.3 |
| 0x16-0xff | Reserved |  |  |  |

Table 4 – MAC command frames

7.3.2 Association-response command

The association response command allows the coordinator or a coordinator to communicate the results of an association attempt back to the device requesting association.

This command shall only be sent by the coordinator or coordinator to a device that is currently trying to associate.

All devices shall be capable of receiving this command, although a device is not required to be capable of transmitting it.

The association response command shall be formatted as illustrated in Figure 10.

| Octets: (see D1) | 1 | 2 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
| MHR fields | Command-frame identifier (see Table 4) | Short Address | Association Status | Capability-Negotiation Response | Codeword Information Element |

Figure 10 – Association-response command format

The individual fields in Figure 10 (save the capability-negotiation response) are explained elsewhere in the literature [IEEE 802.15.7, D1]. The capability-negotiation response is detailed in Clause .

    - Color-stabilization-timer notification

The color-stabilization-timer-notification command shall be formatted as illustrated in Figure 11. This command is used to inform a device or coordinator about the time between two colour-stabilization updates (upon reception of visibility frames).

| Octets: (see D1) | 1 | 2 | 2 |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier (see Table 4) | Short Address | Color-stabilization timer |

Figure 11 – Color-stabilization-timer-notification command format

The first three fields in Figure 11 are explained elsewhere in the literature [IEEE 802.15.7, D1]. The color-stabilization-timer field has the same format as macColorStabilizationTimer (see Clause 5.3).

    - Color-stabilization information

The color-stabilization-information command shall be formatted as illustrated in Figure 12. This command is used to inform a device or coordinator about the time between two colour-stabilization updates (upon reception of visibility frames).

| Octets: (see D1) | 1 | 2 | 6 |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier (see Table 4) | Short Address | Color-stabilization information |

Figure 12 – Color-stabilization-information command format

The first three fields in Figure 12 are explained elsewhere in the literature [IEEE 802.15.7, D1].

  - MAC constants and PIB attributes

Besides the MAC PIB attributes listed in D1, the PIB macColorStabilization and the macColorStabilizationTimer attributes have to be introduced. For the action implied for each setting of the attribute see Table 3.

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macColorStabilization | 0x5f | Binary Integer | 00-11 | The color-stabilization action entailed when receiving visibility frames | 00 |
| macColorStabilizationTimer | 0x60 | Integer | 0-TBD | Time between two stabilization measurements, i.e. sRx (see Clause 4.1) that are send back to the corresponding CSK Tx | TBD |

Table 5 – Extension of D1 MAC-PIB attributes

  - MAC functional description

    - Association and disassociation

      - Association

Besides the original text in D1 only the following paragraph needs to be intersected on page 185, line 52:

The MLME-ASSOCIATE.response and, consequently, the Association response (see Clause ) also contain information about what capabilities the device and the coordinator will and will not use during future communication. One example for such a capability is color stabilization information provided by, e.g., the device upon reception of visibility frames.

  - Color-stabilization  functional description

When a device joins a network (administrated by a coordinator) it advertises its capability of color stabilization in CSK links. In the following we assume that at least one link is a CSK point-to-point link. Otherwise, no color-stabilization functionality is invoked in the networked. Also, for the sake of simplicity, we assume that only the device will be requested to send color-stabilization updates.

The device and the coordinator go through the steps of association as depicted in Figure 7. Upon the issuance of a MLME-ASSOCIATE.request (see Clause 5.2.1.1) the device sends an Association request, among other advertising its capability for Rx-side CSK-color stabilization. Upon reception of this request the coordinator MLME creates an MLME-ASSOCIATE.indication (see Clause  5.2.1.2) to the next higher layer in the Coordinator. There a decision is cast, whether and where color stabilization will be invoked. (If the link to be established is a duplex CSK link, the coordinator can also choose to stabilize the color of the device Tx. As already mentioned we are here only describing the case of color stabilization of the coordinator, but the other possible cases can be inferred from the description here in a straight-forward manner). After this decision has been cast, the pertinent Capability-Negotiation-Response field in the MLME-ASSOCIATE.response is set according to Table 2 and the pertinent information is then translated by the coordinator MLME into the MAC Association-response message (see ). Upon reception of this message the device MLME creates and MLME-ASSOCIATE.confirm according to Clause  and sends it to the next higher layer in the device for further processing.

When the coordinator starts sending visibility frames to the device (identified by the pertinent MAC header; see Clause D1-7.6.13), the device sends color stabilization information (see Clause 4) back to the coordinator. The MAC command frame used for this can be found in Clause 5.2.3. After a time set in the variable macColorStabilizationTimer (see Clause 5.3) the current information is send again from the device to the coordinator. If the coordinator wants to change the time between two such updates it can send a Color-stabilization-timer notification (see Clause 5.2.2) to the device, upon which the device MLME sets the pertinent timer.

Upon dissociation the macColorStabilization variable (see Clause 5.3) is set back to its default value ‘00’.

References

IEEE 802.15.7 Draft 1, “IEEE Standard for Information technology — Telecommunications and information exchange between systems — Local and metropolitan area networks — Specific requirements — Part 15.7: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Visible Light Wireless Personal Area Networks (WPANs)”, February 2010

Atsuya Yokoi, “Color Multiplex Coding for VLC”, IEEE 802.15-08-0743-01, 2008

If not otherwise mentioned, an acknowledgment is sent after the reception of each message.