IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15.7 Task Group Visible-Light Communication (TG-VLC) |  |
| --- | --- | --- |
| Title | Color stabilization for CSK by use of visibility frames |  |
| Date Submitted | 17th May 2010 |  |
| Source | Joachim W. Walewski and Michael Bahr | Voice: +49-89-636-45850 Fax: E-mail: joachim.walewski curly a siemens.com |
| Re: |  |  |
| Abstract | We describe an extension of CSK for duplex links, so that the point of gravity of the CSK constellation diagram can be kept stable in case of changes in the output characteristics of a CSK transmitter. In contrast to 15-10-0089-02-0007 only the low-frequency dynamics of the transmitter is compensated for. Visibility frames are exploited for inferring the compensation coefficients in the Tx. Both PHY and MAC mechanisms are discussed in detail. |  |
| Purpose | Contribution of standard-text material for the preparation of IEEE 802.15.7 D2 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Document # | 15-10-0262-01-0007 |  |

Blue: new text

Pink: Editorial instructions

Crossed out: deleted

Orange: Replace with labels or hot links

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

4.

Acronyms and abbreviations, abbreviations, and symbols

4.1 Acronyms and abbreviations

Instruction to editor: Move the current list of acronyms and abbreviations to the new Clause 4.1. Port the below abbreviations into the D1 list of acronyms and abbreviation

| A/D | Analog-to-digital converter |  |  |
| --- | --- | --- | --- |
| D/A | Digital-to-analog converter |  |  |
| MAC | Media-access control |  |  |
| MLME | MAC-layer management entity |  |  |
| PHY | Physical layer |  |  |
| Rx | Receiver |  |  |
| Tx | Transmitter |  |  |

Instruction to editor: Introduce the below Clause + text after Clause 4.1.

4.2 Symbols

| cl | Compensation factor for band l |
| --- | --- |

Instruction to editor: Introduce the below Clause + text after Clause 6.9.6.3

6.9.6.4 Color stabilization for CSK links

The control-loop model for the color-stabilization scheme is shown in Figure XXFigure XX. Goal of this control mechanism is to stabilize the center of gravity of the CSK constellation diagram (see Clause 6.8.6). Visibility patterns (see Clause-6.9.6.1) are sent from the tri-band Tx of an CSK-PH) to the tri-band Rx. The corresponding received signals are eed back to the LEDs in the tri-band Tx. An optional back link is used to relay these signals back to the tri-band Tx, where they are used to correct the LED driving currents in such a way that the center of gravity of the constellation diagra  is moved back to its initial position.

Figure XX – Control loop for a color-stabilized CSK link.

Figure XY shows the details of the color stabilization mechanism. Upon transmission of visibility patterns the received signals after the D/As are relayed back to the CSK transmitter. In a color-stabilization module, which is not further described in this standard, compensation factors c for each band are calculated. Thereafter, all signal values outputted by the xy-to-(i,j,k) converter are multiplied by the respective compensation factors.

Figure XY – Detailed implementation of a color-stabilized CSK link.

In the color-stabilization modes the visibility patterns to be used are in-band OOK symbols with 100% visibility (see Clause 6.9.61) and for which the xy values of the emitted light coincides with the center of gravity of the constellation diagram previously used. The length of the visibility pattern in the visibility frame (see Clause 5.6.4.5) is chosen so, that

sTx(b)

pTx=f(iTx),

pTx=QiTx,

pRx=TpTx,

iRx=EpRx,

sRx(b)

sRx(b)=BETfAsTx(b)D/AA/D.

sRx(b)

f'AsTx=fAsTxfAsTxTsRxfAsTxT−1sRx,,

sRxsRx,

sRx(b)=BETf'AcsTx(b)D/A,

fAsTx=f'AcsTx.

csTx=CsTx,

f(iTx)=QgiTx,

Q

C=A−1g−1gAsTxgAsTxTsRx,gAsTxT−1sRxsTxsTxT−1.

C=sTxsTxTsRx,sTxT−1sRxsTxTsTxsTxT−1.

thermal equilibrium in all emitters is reached before switching to the next symbol. The received signal to be relayed back (see Figure XY) sRxis only acquired for the last sent visibility symbols or an average over a suitable number of last symbols. sRx

sRx

sRx

sTx(b)

7.1.3.3 MLME-ASSOCIATE.response

Instruction to editor: Change clause 7.1.3.3.1 Semantics of the service primitive as indicated by the WinWord change marks and also introduce the row in the table below after the “status” row in Table 38.

MLME-ASSOCIATE.response	(

DeviceAddress,

AssocShortAddress,

status,

CapabilityNegotiationResponse,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

| CapabilityNegotiationResponse | Integer | 00-11 | Coordinator indicates who will send color-compensation information (see Clause 7.2.3.4). |
| --- | --- | --- | --- |

7.1.3.4 MLME-ASSOCIATE.confirm

Instruction to editor: Change clause 7.1.3.4.1 Semantics of the service primitive as indicated by the WinWord change marks and also introduce the row in the table below after the “status” row in Table 39.

MLME-ASSOCIATE.confirm	(

AssocShortAddress,

status,

CapabilityNegotiationResponse,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

| CapabilityNegotiationResponse | Integer | 00-11 | Coordinator indicates who will send color-compensation information (see Clause 7.2.3.4). |
| --- | --- | --- | --- |

7.2.3.2 Capabilities IE

Instruction to editor: Change clause 7.2.3.2 Capabilities Information Elements as indicated by the WinWord change marks

| Octets: 9 | 1 |
| --- | --- |
| Capability Information Field | Aggregation Bitmap Field |

Figure 79  – Capabilities IE

7.2.3.2.1 Capability Information Field

Instruction to editor: Change Table 72 as indicated by the WinWord change marks

|  | Bit | Function |
| --- | --- | --- |
| MAC-layer capabilities | 0 | Power Source |
|  | 1-2 | Battery information |
|  | 3 | Security capability |
|  | 4 | Coordinator capability |
|  | 5 | Traffic support |
|  | 6-7 | Topology support |
|  | 8-9 | Device type |
|  | 10 | Beacon support |
|  | 11 | Dimming support in MAC |
|  | 12 | Continuous visibility transmission (for infrastructure) |
|  | 13-23 | Reserved |
| PHY-layer capabilities | 24 | PHY-I support |
|  | 25 | PHY-II support |
|  | 26 | Alternate PHY (CSK) support |
|  | 27-28 | Color-stabilization capability (CSK) |
|  | 29 | Dimming support in PHY (VPM) |
|  | 30-32 | Max supported Tx clock |
|  | 33-35 | Max supported Rx clock |
|  | 35 | Explicit clock notification request |
|  | 36-48 | Reserved |
| Band capabilities | 8*n | Aggregate channels |
|  | 8*n | Guard channels |
|  | 49-56 | Bands used for CSK (any 3 bits of the bits set to 1 can be used) |
| Physical-device capabilities | 57-59 | Number of optical source types |
|  | 60-62 | Multiple direction support |
|  | 63-72 | Number of LEDs per optical source type |

Table 72 – Capability Information Field

Instruction to editor: Introduce the following paragraph at line 30 on page 145

The color-stabilization capability subfield is 2 bits in length. It is set to 00 if the device does not support CSK color stabilization and if set to 01 if it does.

Instruction to editor: Move current (D1) clause 7.2.3.4 Data frame format to 7.2.3.5. Insert new clause 7.3.2.4 Capabilities-Negotiation-Response Information Elements as given below

7.2.3.4 Capabilities Negotiation Response information element

The Capabilities Negotiation Response information element is used to inform a device about MAC and PHY capabilities that will be used when communicating with the coordinator. The Capabilities Negotiation Response information element (see Figure Figure XZ) consists of two fields: the Capability Negotiation Response field (see Table Table YXYX), which indicates general capabilities of the device, and the Aggregation Bitmap field facilitating the setup of point-to-multipoint networks (see Clause 7.2.3.2.2).

| Octets: 1 | 1 |
| --- | --- |
| Capability-Negotiation-Response Field | Aggregation Bitmap Field |

Figure XZ – Capability Negotiation Response element

The Capability Negotiation Response field indicates general capabilities of the device that will be used during communication with the coordinator. Its composition is detailed in Table YXYX.

| Capability Negotiation Response |  | Bit | Function |
| --- | --- | --- | --- |
| Requested PHY-layer capabilities |  | 0-3 | Color stabilization scheme |

Table YX– Capability Negotiation Response field

The color-stabilization-scheme function is detailed in Table YY -Table YY.

| Bits | Color-stabilization scheme |
| --- | --- |
| 00 | No color-stabilization |
| 01 | Color stabilization information to be sent from device to coordinator upon reception of visibility frames |
| 10 | Color stabilization information to be sent from coordinator to device upon reception of visibility frames |
| 11 | Color stabilization information to be sent from device to coordinator and from coordinator to device when either receives visibility frames |

Table YY - Color-stabilization scheme

7.3 MAC command frames

Instruction to editor: Make the below indicated changes to Table 79

| Command-frame identifier | Command name | Device |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | Tx | Rx |  |
| 0x01 | Association request | X |  | 7.3.1 |
| 0x02 | Association response |  | X | 7.3.2 |
| 0x03 | Disassociation notification | X | X | 7.3.3 |
| 0x04 | Data request | X |  | 7.3.4 |
| 0x05 | WPAN ID conflict notification | X |  | 7.3.5 |
| 0x06 | Beacon request |  |  | 7.3.6 |
| 0x07 | Coordinator realignment |  | X | 7.3.7 |
| 0x08 | GTS request |  |  | 7.3.8 |
| 0x09 | Blinking notification |  |  | 7.3.9 |
| 0x0a | Dimming notification |  |  | 7.3.10 |
| 0x0b | Fast link recovery |  |  | 7.3.11 |
| 0x0c | Mobility notification |  |  | 7.3.12 |
| 0x0d | Information element exchange |  |  | 7.2.3 |
| 0x0e | GTS response |  |  | 7.3.13 |
| 0x0f | Clock-rate change notification |  |  | 7.3.14 |
| 0x10 | Multiple-channel assignment |  |  | 7.3.15 |
| 0x11 | Channel hopping |  |  | 7.3.16 |
| 0x12 | VLC cell and mobility |  |  | 7.6.12 |
| 0x13 | Visibility-frame transmission |  |  | 7.3.17 |
| 0x14 | Color-stabilization-timer notification | X | X | 7.3.18 |
| 0x15 | Color-stabilization information | X | X | 7.3.19 |
| 0x16-0xff | Reserved |  |  |  |

Table 79 – MAC command frames

7.3.2 Association-response command

Instruction to editor: Make the below indicated changes to Figure 82 and insert the below text directly after Figure 82

| Octets: (see 7.2.2.3) | 1 | 2 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
| MHR fields | Command-frame identifier (see Table ) | Short Address | Association Status | Capability-Negotiation Response | Codeword Information Element |

Figure 82 – Association-response command format

The capability-negotiation response is the same as that of the color-stabilization scheme in Table YY.

Instruction to editor: Introduce the below Clauses + text after Clause 7.3.17.

7.3.18 Color-stabilization-timer notification

The color-stabilization-timer-notification command shall be formatted as illustrated in Figure YZ. This command is used to inform a device or coordinator about the time between two colur-stabilization updates (upon reception of visibility frames).

| Octets: 7 | 1 | 2 | 2 |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier (seeTable Table 79) | Short Address | Color-stabilization timer |

Figure YZ – Color-stabilization-timer-notification command format

The color-stabilization-timer field has the same format as  the macColorStabilizationTimer (see Table 85).

7.3.19 Color-stabilization information

The color-stabilization-information command shall be formatted as illustrated in Figure 12Figure ZX. This command is used for relaying thercolor-stabilization updates (upon reception of visibility frames) back to the pertinent CSK transmitter (see Figure XX).

| Octets: 7 | 1 | 2 | 6 |
| --- | --- | --- | --- |
| MHR fields | Command-frame identifier (see Table ) | Short Address | Color-stabilization information |

Figure 12 – Color-stabilization-information command format

7.4.2 MAC PIB attributes

Instruction to editor: Introduce the below table rows at the end of table 85.

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macColorStabilization | 0x5f | Binary Integer | 00-11 | The color-stabilization action entailed when receiving visibility frames | 00 |
| macColorStabilizationTimer | 0x60 | Integer | 0x0-0xffffff | Time between two stabilization measurements (sRxsee Clause 1.2) that are send back to the corresponding CSK Tx. The time is measured in multiples of received color frames | 0x00400 |

7.6.3.1 Association

Instruction to editor: Introduce the following paragraph on page 185, line 52.

The MLME-ASSOCIATE.response and, consequently, the Association response (see Clause 7.3.2) also contain information about what capabilities the device and the coordinator will and will not use during future communication. One example for such a capability is color-stabilization information provided by, e.g., the device upon reception of visibility frames.

Instruction to editor: Introduce the following Clause after Clause 7.6.12 and increment all following Clauses by +1.

7.6.14 Color-stabilization

When a device joins a network (administrated by a coordinator) it advertises its capability of color stabilization in CSK links. In the following we assume that at least one link is a CSK point-to-point link. Otherwise, no color-stabilization functionality is invoked in the networked. Also, for the sake of simplicity, we assume that only the device will be requested to send color-stabilization updates.

The device and the coordinator go through the steps of associationFigure 7. Upon the issuance of a MLME-ASSOCIATE.request the device sends an Association request, among other advertising its capability for Rx-side CSK-color stabilization. Upon reception of this request the coordinator MLME creates an MLME-ASSOCIATE.indicatio) to the next higher layer in the Coordinator. There a decision is cast, whether and where color stabilization will be invoked. (If the link to be established is a duplex CSK link, the coordinator can also choose to stabilize the color of the device Tx. As already mentioned we are here only describing the case of color stabilization of the coordinator, but the other possible cases can be inferred from the description here in a straight-forward manner). After this decision has been cast, the pertinent Capability-Negotiation-Response field in the MLME-ASSOCIATE.response is set according toTable YX Table 38 and the pertinent information is then translated by the coordinator MLME into the MAC Association-response messag). Upon reception of this message the device MLME creates and MLME-ASSOCIATE.confirm accordin and sends it to the next higher layer in the device for further processing.

When the coordinator starts sending visibility frames to the device (identified by the pertinent MAC header; see Clause 7.6.13), the device sends color stabilization information back to the coordinator. The MAC command frame used for this can be found in Clause 7.3.19. After a time set in the variable macColorStabilizationTimer the current information is send again from the device to the coordinator. If the coordinator wants to change the time between two such updates it can send a Color-stabilization-timer notification (see Clause 7.3.18) to the device, upon which the device MLME sets the pertinent timer, which is not further described in this standard.

Upon dissociation the macColorStabilization variable is set back to its default value ‘00’.

If not otherwise mentioned, an acknowledgment is sent after the reception of each message.