IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Simplified Channel hopping for non-beacon PAN |  |
| Date Submitted | 12 May, 2010 |  |
| Source | Ben Rolfe, BCA Cristina Seibert, Silver Spring Networks Will San Filippo, Silver Spring Networks | [ben @ blindcreek.com] [cseibert @ silverspringnet.com] [wills @ silverspringnet.com] |
| Re: | Task Group 15.4e: d1P802-15-4e_Draft_Standard, LB 53 Comment Resolution; Task group 15.4g: d1P802-15-4g_Draft_Standard, LB51 Comment Resolution. |  |
| Abstract | Proposed comment resolution, providing a description of simplified channel hopping for non-beacon networks without global synchronization. |  |
| Purpose | Resolve comments from LB53 and LB51. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Simplified Channel hopping for non-beacon PAN

# Introduction and Purpose

This submission provides proposed text to resolve comments submitted during LB53 on d1P802-15-4e_Draft_Standard.  The purpose is to address channel hopping without global synchronization of the PAN in support of the draft PHY amendment P802.15.4g.

The channel hopping methods proposed in the P802.15.4e draft depend on global synchronization of the WPAN, either by use of periodic beacons, or by a means to ensure all devices share common time.  The current methods in P802.15.4e are optimized for PHYs with a small number of available channels compared to the PHYs defined in P802.15.4g, and are optimized for different topologies. In the wide area mesh architecture which is widely used for the SUN application, a method to support channel hopping that does not require global synchronization is preferred.

This submission provides a simplified channel hopping method that supports the requirements of the P802.15.4g PHYs operating in non-beacon enabled PANs, referred to as Non-Beacon Simplified Channel Hopping (NBSCH).  The proposed method provides the means for a next higher layer (NHL) entity to manage hopping in an efficient manner, and meet regulatory constraints as are required in some bands and regions. Note that regulatory compliance is not the responsibility of P802.15.4, and as such the standard does not assure that an implementation complies with local regulations. The methods here provide to the NHL the means by which known constraints can be complied with.

Editorial instructions are given in italics.

# Proposed Text for P802.15.4e Draft

3 Definitions

Insert in alphabetical order the following definitions:

Slot Time: for NBSCH, the nominal time a device stays on the channel before moving to the next channel in the hopping sequence. It includes the time to switch from one channel to another.

Insert the sub-clause after 5.5.6

5.5.7 Channel hopping in non-beacon PANs

Non-Beacon Simplified Channel Hopping (NBSCH) allows devices within a wide-area mesh network to use channel hopping without requiring global synchronization, a global hopping sequence, a global slot time or a fixed transmission time within the slot. This mode enables the deployment of large scale mesh networks such as for the Smart Grid, in a cost effective manner, and with low complexity. Additional information can be found in Annex M.8.

7.1.1.2 MCPS-DATA.confirm

Add to description of Timestamp in table 42:

| Timestamp | Integer | 0x000000–0xffffff | Optional. The time, in symbols, at which the data were transmitted (see 7.5.4.1). For devices implementing NBSCH, the Timestamp shall be implemented. The value of this parameter will be considered valid only if the value of the status parameter is SUCCESS; if the status parameter is not equal to SUCCESS, the value of the Timestamp parameter shall not be used for any other purpose. The symbol boundary is described by macSyncSymbolOffset (see Table 86 in 7.4.1). This is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. |
| --- | --- | --- | --- |

7.1.1.3 MCPS-DATA.indication

Add to description of Timestamp in table 43:

| Timestamp | Integer | 0x000000–0xffffff | Optional. The time, in symbols, at which the data were transmitted (see 7.5.4.1). For devices implementing NBSCH, the Timestamp shall be implemented. The value of this parameter will be considered valid only if the value of the status parameter is SUCCESS; if the status parameter is not equal to SUCCESS, the value of the Timestamp parameter shall not be used for any other purpose. The symbol boundary is described by macSyncSymbolOffset (see Table 86 in 7.4.1). This is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. |
| --- | --- | --- | --- |

7.4.2.8 NBSCH specific PIB attributes

Add attributes table containing:

- macNBSCHcapable, set to TRUE if a device supports NBSCH, and set to FALSE otherwise

- macNBSCHSwitchTime, the time it will take the implementation to switch from one channel to another (read only).

- macNBSCHClockAccuracy, the clock accuracy/stability (read only).

NOTE: Add to informative text in Annex “M” (new annex for 15.4e). The section numbering may change when integrated into the annex; the following is based on numbering in draft 1.

M.8 Non-Beacon Simplified Channel Hopping

This sub-clause describes a method for Non-Beacon Simplified Channel Hopping (NBSCH) which is optimized for a wide-area mesh network, where channel hopping is required and it is preferred to operate without requiring global synchronization.

M.8.1 Synchronization

When NBSCH is in use, global synchronization of the PAN is not necessary. Relative synchronization between peer devices is achieved without the use of beacons or synchronization to a global time source.

Two devices can communicate with relative synchronization between the devices forming the link. The timing information communicated is always “synchronized” to a known point in the frame. Given the frame reception time in the MCPS-Data.indication and the timing information in the frame, upper layers can calculate the offset between the local device and the neighbor from which the data packet was received. The reception time is always referenced to a known point in the incoming data packet (see macSyncSymbolOffset in sub-clause 7.4.2).

Devices using NBSCH will send to peer devices a packet that contains the offset into the hopping sequence, or sequence ‘tick’, which is synchronized to the known point in the transmitted packet.  This allows a receiving device to determine the sequence start time of device (relative to the receiving devices local clock) by taking the receive time of the frame (timestamp in the MCPS-Data.indication) and subtracting sequence offset time from it.

M.8.2 Channel Hopping in non-beacon PANs

The NBSCH is intended to operate with a large number of channels for channel hopping.  Each channel in a node’s hopping sequence is visited for a nominal amount of time called the slot time. If no reception is heard during the slot time, the node changes to the next channel in its hopping sequence. If a valid reception is heard (e.g. the start frame delimiter is received), channel hopping stops so that the reception can be processed.

When a frame is to be transmitted, channel hopping stops and the frame is sent on the channel that the receiver is monitoring at that particular time, or on some broadcast channel as specified by the NHL. Channel hopping resumes (from where it would have been had no transmit/reception occur) once the transaction ends.

M.8.2.1 Hopping sequence

Hopping sequences are assigned and managed by the NHL.

In order for two devices to communicate, they must know the other’s hopping sequence. One way for devices to know each other’s sequence is to exchange an ordered list of channel numbers and the band(s) to which each channel belongs.