IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Extensible MAC Frame |  |
| Date Submitted | [12 May, 2010] |  |
| Source | Ben Rolfe, BCA Cristina Seibert, Silver Spring Networks Will San Filippo, Silver Spring Networks | [ben @ blindcreek.com] [cseibert @ silverspringnet.com] [wills @ silverspringnet.com] |
| Re: | Task Group 15.4e: d1P802-15-4e_Draft_Standard, LB 53 Comment Resolution; Task group 15.4g: d1P802-15-4g_Draft_Standard, LB51 Comment Resolution. |  |
| Abstract | Proposed comment resolution, providing an extensible MAC frame. |  |
| Purpose | Resolve comments from LB53 and LB51 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Extensible MAC Frame

Currently 15.4e draft eliminates the ability to add frame types except for one particular combination of frame type and sFCF values [FT=00, sFCF = 1].  The 15.4g draft defines at least one new frame type, which does not use the sFCF. The combination of the two drafts results in no further additions to the standard without exiling legacy 15.4-2006 devices.

A solution is to use the remaining frame type combination (as specified by the combination of b0b1b2 = 0 0 1 in the frame control field) to introduce a flexible MAC frame which can accommodate future MLME frames. What are now MAC frame types for beacons, commands, etc. and future variations thereof can be represented by means of information elements (IEs) following the fixed part of the MHR.  The concept is borrowed from 802.11 and 802.15.3. An IE is a triplet (ID, length, value).  Borrowing from prior 802 standards:

| Octets: 1 | 1 | 0 to 255 octets |
| --- | --- | --- |
| Element ID | IE Payload Length | IE Payload |

Element ID allocation is as follows: 0x00 to 0x7f are defined for standard IEs, 0x80 to 0xFF reserved for vendor specific IEs.  The table below details the allocation of element IDs:

| Element ID Value | Description | Length of IE Payload |
| --- | --- | --- |
| 0x00 | Coexistence beacon (from 4g draft) | [see 7.2.2.4a of d1P802-15-4g] |
| 0x01 – 0x7E | Reserved | 0-255 octets |
| 0x7F | Tail IE | * |
| 0x80 – 0xFF | Vendor specific | 0-255 octets |

*Note: for the Tail IE (ID 0x7F) IE Payload Length and IE Payload fields are absent.

The Tail IE is used to terminate the IE list.  Vendor specific IEs allow for vendor specific extensions to the MAC header. The use of vendor specific IEs is outside the scope of the standard.

The general format of the MAC Frame is as follows:

| Octets: |  |  |  | Variable | Variable | 2 or 4 |
| --- | --- | --- | --- | --- | --- | --- |
| 1/2 | 0/1 | 0,2,4,8,10,16,18,20 0,5,6,10,14 |  |  |  |  |
| Frame Control | Sequence Number | Addressing Fields | Auxiliary Security | IEs | MAC Payload |  |
| MHR |  |  |  |  | MAC Payload | FCS |

Example IE list (two IEs + tail IE to signal end of list):

| Octets: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Variable | 1 | 1 | Variable | 1 |
| Element ID | IE Payload Length | IE Payload | Element ID | IE Payload Length | IE Payload | Element ID = 0x7F |