IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | PICS Updates for 802.15.4 |  |
| Date Submitted | 8 July 2010 |  |
| Source | Tim Godfrey | Voice: [ ] Fax: [ ] E-mail: |
| Re: | Task Group 15.4e: LB53 Comment Resolutions |  |
| Abstract | Updates to the 802.15.4 PICS (Annex D), providing specification of optional and mandatory features. |  |
| Purpose | Submission to resolve LB53 technical comments on the missing PICS updates. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

- Additions to the 802.15.4 PICS

To be appended to Table D.5—MAC sublayer functions

Note: the starting point of the MLF item numbers takes into account 802.15.4-2006 (MLF 1-14) as well as approved amendments 802.15.4a (MLF14), 802.15.4c (none), and 802.155.4d (none)

| Item Number | Item Description | Reference | Status | Support |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | N/A | Yes | No |
| MLF15 | TSCH Capability | 7.1.2.2 Table 46.a | O |  |  |  |
| MLF15.1 | TSCH-MAC Management Services | 7.1.2.2 7.1.18 | MLF14:M |  |  |  |
| MLF15.2 | TSCH-commands | 7.3.10 | MLF14:M |  |  |  |
| MLF15.2.1 | TSCH-commands (Join, Activate) | 7.3.10.2, 7.3.10.3 | FD1:M FD2:O |  |  |  |
| MLF15.3 | TSCH Channel Access | 7.5.1.4.2, 7.5.1.4.3, 7.5.1.5 | MLF14:M |  |  |  |
| MLF15.4 | TSCH-network formation | 7.5.2.6 | MLF14:M |  |  |  |
| MLF15.5 | Synchronization in TSCH-network | 7.5.4.4 | MLF14:M |  |  |  |
| MLF15.6 | TSCH-Retransmissions | 7.5.6.4.3.2 | MLF14:M |  |  |  |
| MLF16 | LL Capability | 7.1.2.3 Table 46.b | O |  |  |  |
| MLF16.1 | LL-MAC Management Services | 7.1.2.3 7.1.19 | MLF15:M |  |  |  |
| MLF16.2 | LL-commands. | 7.3.11 | MLF15:M |  |  |  |
| MLF16.2.1 | LL: Discover Response Command, Configuration Response, Configuration Request Frame | 7.3.11.1, 7.3.11.2, 7.3.11.3 | FD1:M FD2:O |  |  |  |
| MLF16.3 | LL Channel Access | 7.5.1.4.4 | MLF15:M |  |  |  |
| MLF16.4 | LL-Superframe structure | 7.5.1.6 | MLF15:M |  |  |  |
| MLF16.5 | LL-Transmission Modes | 7.5.9 | MLF15:M |  |  |  |
| MLF17 | DSME Capability | 7.1.2.4 Table 46.c | O |  |  |  |
| MLF17.1 | DSME-MAC management service | 7.1.2.4, 7.1.20 | MLF16:M |  |  |  |
| MLF17.1.1 | MLME-DSME-START primitive | 7.1.20.2 | FD1:M FD2:O |  |  |  |
| MLF17.2 | DSME-command | 7.3.12 | MLF16:M |  |  |  |
| MLF17.2.1 | DSME-Link status report command | 7.3.12.9 | FD1:M FD2:O |  |  |  |
| MLF17.3 | DSME-based Multi-superframe structure | 7.5.10 | MLF16:M |  |  |  |
| MLF18 | EBR (Enhanced Beacon Request) capability | 7.3.13 | O |  |  |  |
| MLF18.1 | EBR Commands | 7.3.13 | MLF17:O |  |  |  |
| MLF18.1.1 | EBR-Enhanced Beacon request command | 7.3.13.1 | FD1:M FD2:O |  |  |  |
| MLF19 | LE Capability | 7.3.14 | O |  |  |  |
| MLF19.1 | LE-specific MAC sublayer service specification | 7.1.21 | MLF18:M |  |  |  |
| MLF19.2 | RIT (Receiver Initiated Transmission) Capability | 7.4.2.2 Table 86.a | MLF18:O.1 |  |  |  |
| MLF19.3 | Coordinated Sampled Listening (CSL) capability | 7.5.11.1 | MLF18:O.1 |  |  |  |
| MLF19.4 | LE-commands | 7.3.14 | MLF18:M |  |  |  |
| MLF19.5 | LE-Functional description | 7.5.1.7, 7.5.2.1.5 | MLF18:M |  |  |  |
| MLF19.6 | LE-Transmission, reception and acknowledgement. | 7.5.11 | MLF18:M |  |  |  |
| MLF20 | RFID Capability | 7.3.15 | O |  |  |  |
| MLF21 | FastA-commands | 7.3.16 | O |  |  |  |
| MLF22 | MAC constants and PIB attributes | 7.4 | M |  |  |  |

To be appended to Table D.6—MAC frames

| Item Number | Item Description | Reference | Transmitter |  | Receiver |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Support N/A Yes No | Status | Support N/A Yes No |
| MF5.1 | LL-Frame Formats | 7.2.3 | MLF15:M |  | MLF15:M |  |
| MF5.2 | DSME-Frame Formats | 7.2.4 | MLF16:M |  | MLF16:M |  |
| MF5.3 | Blink Frame Formats | 7.2.5 | MLF19:M |  | MLF19:M |  |