- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802 |  |
| Date Submitted | [8th July, 2010] |  |
| Source | [Sang-Kyu Lim, Dae Ho Kim, Il Soon Jang, You Jin Kim, Tae-Gyu Kang] [ETRI] [Daejeon, Korea] | Voice: [ +82-42-860-1573] Fax: [+82-42-860-5218] E-mail: [sklim@etri.re.kr] |
| Re: | [Response to LB #50 comments] |  |
| Abstract | [The new text in clauses 5.5.3.1 has been implemented to merge 5.5.3.1 and 5.5.4.] |  |
| Purpose | [To resolve the LB comments related to 5.5.4 in D1 draft.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

5.5.3 Flicker compensation and dimming

5.5.3.1 Flicker compensation

Flicker is defined as unexpected and unpredictable brightness fluctuations that are perceptible by the human eye and can be injurious to human health; therefore, this standard supports flicker compensation. All devices shall be compliant to all applicable regulations in regards to flicker.

The maximum flickering time period (MFTP) is defined as the maximum time period over which the light intensity can be changing but the resulting flicker is not perceivable by the human eye [B37]. To avoid flickering in VLC, the brightness of each MFTP needs to be all equal.

The flicker in VLC is classified into two categories according to its generation mechanism: intra-frame flicker and inter-frame flicker. Intra-frame flicker is defined as bit pattern dependent brightness discrepancies within the data frame. Inter-frame flicker is defined as the average brightness discrepancy between the packet frame transmissions and the idle time between data transmissions.