- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802 |  |
| Date Submitted | [8th July, 2010] |  |
| Source | [Sang-Kyu Lim, Dae Ho Kim, Il Soon Jang, You Jin Kim, Tae-Gyu Kang] [ETRI] [Daejeon, Korea] | Voice: [ +82-42-860-1573] Fax: [+82-42-860-5218] E-mail: [sklim@etri.re.kr] |
| Re: | [Response to LB #50 comments] |  |
| Abstract | [The new texts of clauses 5.5.3.1.1 and 5.5.3.1.2 have been implemented.] |  |
| Purpose | [To resolve the LB comments related to 5.5.3.1.1 and 5.5.3.1.2] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

5.5.3.1.1 Intra-frame flicker compensation

Intra-frame flicker compensation can be accomplished by the use of one of the following mechanisms: Manchester encoding as specified in 6.6.4.2, 4B6B encoding as specified in 6.6.4.1, 8B10B encoding as specified in 6.7.3.2 or VPPM as specified in 6.9.6 and Annex F”.

5.5.3.1.2 Inter-frame flicker compensation

The compensation method used for inter-frame flicker can be the transmission of an idle pattern between data frames whose average brightness is equal to that of the data frames. The idle pattern is not specified and it is allowable to use an idle pattern symbol rate other than that used for data transmissions to avoid in-band modulation domain interference.